3GPP TSG RAN WG1 #121	R1-2503370

Malta, MT, 19th – 23rd May, 2025

Source:	vivo

Title:	Remaining issues on LP-WUS operation in IDLE/INACTIVE modes

Agenda Item:	9.6.2

Document for:	Discussion and Decision

- Introduction

In RAN 1#120bis meeting, the agreements and conclusion on LP-WUS operation in RRC idle/inactive modes are provided in [1]. Based on the last meeting’s progress, we provide our views on the procedure and configuration of LP-WUS monitoring in RRC idle/inactive mode in the following chapters.

- Subgrouping design of LP-WUS

  - Subgroup indication method

Agreement The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4. Agreement For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported. - For codepoint corresponding to more than one subgroups: - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO Agreement For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is - 7 for the case where 4 POs are mapped to one LO - 15 for the case where 2 POs are mapped to one LO TS: 38.213 for PEI design A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+i_smodNPOPEI is a paging occasion index, and N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion.

In the previous meeting, we have agreed that the subgroup indication of LP-WUS is based on codepoint. But, how LP-WUS subgroup ID maps with the codepoint and how UE determines its associated codepoints should be further discussed. Considering the case that a LP-WUS associated with more than one PO, as the same logic of the subgroup indication design of R17 PEI, there is no need for a LP-WUS to explicitly separate indicate the target PO index. The simplest mapping method is that for a specific subgroup, the corresponding codepoint values are sequentially mapped based on the index of associated PO and the index of the subgroup in the PO, as shown in Table 1. Besides, for a common codepoint per PO it can be mapped after the mapping of all specific subgroups as shown in the last raw of Table 1. To derive a formulation, let iPO denotes the relative PO index, with starting value of 0, among the POs associated with a LP-WUS; iSG represents the subgroup index for a UE within a PO; NSGPO is the configured number of LP-WUS subgroup per PO. Accordingly, a UE determines its subgroup specific codepoint value as “iPO*NSGPO+iSG”, which is also the same formula design as that of R17 PEI as specified in TS 38.213. And for the common codepoint value for a target PO, it can be derived as NPOLP-WUS*NSGPO+iPO.

Table 1 A mapping example between LP-WUS subgroups and codepoints, when NPOLP-WUS=4, NSGPO=7.

| Corresponding codepoint value | iPO=0 | iPO=1 | iPO=2 | iPO=3 |
| --- | --- | --- | --- | --- |
| iSG=0 | 0 | 7 | 14 | 21 |
| iSG=1 | 1 | 8 | 15 | 22 |
| iSG=2 | 2 | 9 | 16 | 23 |
| iSG=3 | 3 | 10 | 17 | 24 |
| iSG=4 | 4 | 11 | 18 | 25 |
| iSG=5 | 5 | 12 | 19 | 26 |
| iSG=6 | 6 | 13 | 20 | 27 |
| Common codepoint to wake up all subgroups for a target PO | 28 | 29 | 30 | 31 |

Observation 1: For the case that LO and PO to be 1: multiple mapping, there is no need to explicitly indicate the target PO index in a LP-WUS.

Proposal 1:

- UE identifies its subgroup specific codepoint value as follows:

- Let iPO denotes the PO index associated with a LP-WUS of the UE, with starting value of 0:

iPO=UE_IDmodN⋅NS+i_smodNPOLP-WUS,

where UE_ID, N, NS and i_s are defined in clause 7 of TS 38.304.

    - NPOLP-WUS denotes the configured number of PO associated with a LP-WUS.

- A UE determines its subgroup specific codepoint value as “iPO*NSGPO+iSG”, where

    - NSGPO denotes the configured number of LP-WUS subgroup per PO and 1≤NSGPO* NPOLP-WUS≤31;

    - iSG is the LP-WUS subgroup ID per PO for the UE and 0≤iSG<NSGPO.

- For a UE with PO index iPO, its common codepoint value is “NPOLP-WUS*NSGPO+iPO”.

- When a UE detects a codepoint value carried by a LP-WUS which is equal to “NPOLP-WUS*NSGPO+iPO” or “iPO*NSGPO+iSG”, the UE monitors its PO; otherwise, the UE is not required to monitor PO.

- LP-WUS monitoring configuration

  - LP-WUS MO configuration

Agreement UE determines whether a symbol is available for LP-WUS based on: - Alt 1: Time-domain pattern configured by the gNB - Alt 1A: Periodic time-domain pattern - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern - Alt 1B: Per-MO pattern, applicable for all Mos - Alt 2: Information from existing configurations available for idle/inactive UEs such as [SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc]. - Alt 3: Combination of Alt 1 and Alt 2 - Alt 4: NW ensures LP-WUS configuration without collision with existing signal(s) Agreement For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. - FFS additional configuration to control the subsequent MO locations, e.g., - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration - FFS restriction on MO locations, e.g. only on DL slots - FFS minimum gap is needed between two MOs to ensure LR processing time

- Determine whether a symbol is available for LP-WUS

On how to determine whether a symbol is available for LP-WUS transmission, we think alt2 (i.e., information from existing configurations) should be the basic method, which brings no explicitly signaling overhead. The information from existing configurations can be obtained by UE, including the following. So, UE can accordingly determine the symbols occupied by the following channels/signals as unavailable symbols for LP-WUS transmission.

- SSB symbols indicated by ssb-PositionsInBurst in SIB1 where SSB overlaps with LP-WUS bandwidth.

- CORESET 0/type0 CSS where CORESET 0 overlaps with LP-WUS bandwidth.

- UL slot/symbol

Observation 2: By applying alt 2 (i.e., information from existing configurations), the above NR channel/signals can be avoided to collide with LP-WUS transmission.

In addition to the channel/signal configurations known to UE based on alt 2 as mentioned above, there are still some NR signal/channel transmission for RRC connected mode UE unknow for RRC idle/inactive mode UE, e.g., CSS other than type-0 or USS, or reference signal, which may collide with LP-WUS resource. The network may explicitly configure a time-domain pattern for UE (i.e., alt 1) to avoid such unavailable symbols for LP-WUS transmission. Then, UE can determine whether a symbol is available for LP-WUS based on combination of alt 1 &2. Note that alt 4 is more like a configuration expectation rather than a specific method.

Observation 3: In addition to alt 2, alt1 can also be configured by the network to rate match e.g., the NR signal/channel transmission for RRC connected mode UE which is treated as unavailable symbol for LP-WUS transmission.

Besides, for the down-selection of alt 1A and alt1B, we think alt1A is more flexible than alt1B, since the length of time-domain pattern of alt1A is configurable while it is fixed to be the same as MO length for alt1B. For example, by using alt1A with X-slots periodic pattern, X can be configured to match with the periodicity of CSI-RS. But, if a unified per-MO pattern is adopted and the CSI-RS periodicity is different as the MO duration as shown in Figure 1 or not aligned with MO periodicity, then plenty of time resources will be reserved as unavailable symbols but without any actual CSI-RS transmission, resulting in poor spectrum efficiency. It is noted that, if the unified per-MO pattern also intends to avoid collision with SSB, UL symbols and CORESET 0, i.e., Alt 1B works standalone without Alt 2, the available symbol will be further reduced. Figure 2 gives an example that none available symbol exists within a unified per-MO pattern.

Figure 1 An example to derive a unified per-MO pattern for Alt 1B.

Figure 2 An example that none available symbol within a unified per-MO pattern by Alt 1B to avoid collision with SSB & UL symbols.

Observation 4: It would be quite challenge to find a unified per-MO pattern to avoid collision with existing NR signals/channels while efficiently provide sufficient resource for LP-WUS transmission in each MO by Alt 1B.

And for alt1A, lots of alternatives of periodic time-domain pattern are listed. In our view, as analyzed before, since the intention of configuring periodic pattern by alt1 is to rate match NR signal/channel transmission for RRC connected mode i.e., CSS other than type-0 or USS, or CSI-RS, it is reasonable to consider reuse the existing rate match pattern design for these NR channels/signals as cited below. Since no need to consider the rate matching in frequency domain, a rate matching pattern with the bitmaps including symbolsInResourceBlock and periodicityAndPattern as highlighted below are reused as a time-domain pattern of LP-WUS.

RateMatchPattern information element RateMatchPattern ::= SEQUENCE { rateMatchPatternId RateMatchPatternId, patternType CHOICE { bitmaps SEQUENCE { resourceBlocks BIT STRING (SIZE (275)), symbolsInResourceBlock CHOICE { oneSlot BIT STRING (SIZE (14)), twoSlots BIT STRING (SIZE (28)) }, periodicityAndPattern CHOICE { n2 BIT STRING (SIZE (2)), n4 BIT STRING (SIZE (4)), n5 BIT STRING (SIZE (5)), n8 BIT STRING (SIZE (8)), n10 BIT STRING (SIZE (10)), n20 BIT STRING (SIZE (20)), n40 BIT STRING (SIZE (40)) } OPTIONAL, -- Need S ... }, controlResourceSet ControlResourceSetId }, subcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond CellLevel dummy ENUMERATED { dynamic, semiStatic }, ..., [[ controlResourceSet-r16 ControlResourceSetId-r16 OPTIONAL -- Need R ]] }

Proposal 2: Support Alt3 by combining alt 1A and alt2.

- For Alt 1A, support to partially reuse the mechanism of rate matching pattern (i.e., reuse bitmaps including symbolsInResourceBlock and periodicityAndPattern). And the network can configure a rate matching pattern for LP-WUS transmission accordingly.

- For Alt 2, SSB indicated by ssb-PositionsInBurst, CORESET/Type-0 CSS and UL symbols indicated by TDD DL/UL configuration which are overlapped with LP-WUS transmission are considered as unavailable symbols.

- LP-WUS nominal MO duration and actual MO duration

Agreement Terminology definition - Nominal MO duration: this includes both the available and unavailable symbols - Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives: - Alt A: Nominal MO duration is configured. (e.g. in unit of slots) - Actual LP-WUS duration is the number of available symbols within the MO. - Actual LP-WUS duration can vary from one MO to another MO. - Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values) - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration. - FFS: Additional termination condition such as time window - Alt C: Both nominal MO duration and actual LP-WUS duration are configured. - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred. - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs. - Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

In the last meeting, four alternatives on how to determine LP-WUS nominal MO duration and/or actual MO duration are summarized as shown in Figure 3. But the selection of these alternatives highly depends on how to determine an available symbol for LP-WUS as analyzed in the previous section.

Figure 3 an example to help understand these four alternatives for MO design which is given in FL summary document.

For alt A, only nominal MO duration is configured and actual MO duration varies for different MOs. In some cases, there is insufficient time resource for a MO to do actual LP-WUS transmission, so the detection performance of LP-WUS cannot be guaranteed. If extend the nominal MO duration to promise the length of actual MO duration no less than a threshold, the overhead issue will arise according. Furthermore, variable MO duration increases LP-WUR reception complexity to support variable code block length after RM coding and rate matching.

Observation 5: For Alt A, the detection performance of LP-WUS cannot be guaranteed, since there is insufficient time resource for a MO to do actual LP-WUS transmission, and it increases RM decoding complexity for variable code block length.

For alt B, without the constraint of nominal MO duration length, there can be a case that the LP-WUS transmission span a long time period which exceeds the coherence length as shown in Figure 4, which could result in degrading of LP-WUS detection performance due to the change of the channel condition. Besides, alt B is more like a kind of floating MO design, that is, the start time of a MO is unfixed and depends on the ending of the last MOs if assuming MO and MO are located back-to-back. Once the time resource used to transmit a LP-WUS signal is not aligned between the network and UE, the subsequent LP-WUS reception will be impacted. And the floating MO start time also increases the complexity of scheduling management for the network and even makes LP-WUS scheduling more complex than the existing NR PDSCH/PUSCH scheduling which is restricted to within a time period e.g., within a TTI for single PDSCH/PUSCH transmission.

Figure 4 Example of an actual MO duration which spans a long unavailable duration by using alt B.

Observation 6: For Alt B without the constraint of nominal MO duration length, the following two drawbacks need to be considered:

- A LP-WUS transmission may span a long time period exceeding the coherence length, resulting in degradation of LP-WUS detection performance due to the change of the channel condition.

- The floating MO start time and the variable MO duration increase the complexity of scheduling management for the network.

For alt C, both nominal MO duration and actual MO duration are configured, so the start time of each nominal MO can be pre-calculated by the network without any LP-WUS scheduling complexity issue. And the detection performance of LP-WUS can be guaranteed by configuring a suitable length of actual MO duration. As shown in Figure 5, if the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, this MO is considered as invalid MO and defer the MO until it can be a valid MO. So, there will be no such a case that there is no valid MO for the beam(s). Actually, as denoted in the following Figure X, even dropping the invalid MO, there still has 2 or 3 valid MOs per beam in a typical slot-structure configurations i.e., DDDSUDDSUU (5ms pattern for 30KHz).

| D | D | D | S | U | D | D | S | U | U | D | D | D | S | U | D | D | S | U | U | D | D | D | S | U | D | D | S | U | U | D | D | D | S | U | D | D | S | U | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | invalid |  | valid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 nominal MOs for beam 1 |  |  |  |  |  |  |  | 4 nominal MOs for beam 2 |  |  |  |  |  |  |  | 4 nominal MOs for beam 3 |  |  |  |  |  |  |  | 4 nominal MOs for beam 4 |  |  |  |  |  |  |  | 4 nominal MOs for beam 1 |  |  |  |  |  |  |  |
| Note: nominal MO duration=2 slots, actual MO duration can be no less than 16symbols, 4 MOs per beam, 4 SSB beams are assumed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 5 Analysis of valid MOs per beam in a typical DDDSUDDSUU slot-structure config.

Observation 7: For Alt C, since both nominal MO duration and actual MO duration are configured, there are no issues of LP-WUS scheduling complexity and performance degradation of LP-WUS.

Observation 8: For Alt C, by deferring an invalid MO instead of dropping an invalid MO, there will be no such a case that no valid MO for a LP-WUS beam.

Observation 9: For Alt C, even dropping an invalid MO, there still can be 2 or 3 valid MOs per beam in a typical slot-structure configurations, assuming 4 LP-WUS MOs are configured per beam.

For alt D which is combinedly used with alt1B i.e., per-MO pattern, based our analysis above, it is challenge to find a unified per-MO pattern to avoid collision with existing NR signals/channels while efficiently provide sufficient resource for LP-WUS transmission in each MO without resource wasting by Alt 1B, as shown in Figure 2. Moreover, in fact, alt D has a condition that the actual LP-WUS duration is the same for all the MOs, which is naturally against with alt2 (i.e., information from existing configurations) i.e., causing various actual MO durations for different MOs. Without using alt2, it become quite more difficult to find a unified per-MO pattern as shown in Figure 3. Hence, alt D should not be recommended.

Observation 10: For Alt D which is combinedly used with alt1B, it is challenge to find a unified per-MO pattern for all MOs to avoid collision with existing NR signals/channels while efficiently provide sufficient resource for LP-WUS transmission in each MO.

Proposal 3: Support Alt C i.e., both nominal MO duration and actual MO duration are configured.

- If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped or deferred.

- How to determine the subsequent LP-WUS MOs

Agreement For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. - FFS additional configuration to control the subsequent MO locations, e.g., - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration - FFS restriction on MO locations, e.g. only on DL slots - FFS minimum gap is needed between two MOs to ensure LR processing time

In the last meeting, we agreed that the starting time location of the first LP-WUS MO in a LO is indicated by an offset i.e., slot-level or symbol-level offset. Here, the mentioned first LP-WUS MO refers to first nominal LP-WUS MO.

On how to determine the subsequent LP-WUS MOs, two alternatives are given as above. For alt 1, it needs to introduce a slot-level offset for each LP-WUS MOs of a LO. As calculated below, the upper boundary of the value range for a slot-level offset is:

N *M*DMO + TMO*(N*M-1),

wherein N is the number of LP-WUS beams, M is the number of MO per beam, DMO is the nominal LP-WUS MO duration, TMO is the time interval of adjacent MOs. Since there is N *M MOs, the total required bits for slot-level offsets are calculated as follows.

N *M*log2(N *M*DMO + TMO*(N*M-1))

For example, when N=8, M=4, DMO=6 slots, TMO= 2 slots (e.g., considering the maximum unavailable time gap caused by UL slot and SSB transmission), the maximum total required bits to indicate the slot-level offsets for a LO is 256bits. If the network configures two offsets to address different wake-up delays, then the number of required bits is doubled. And for Alt 2B, huge overhead will also be caused. For example, by using bitmap to achieve the effect of alt 2B, N *M*DMO + TMO*(N*M-1) e.g., 254 bits based on the mentioned assumptions will be needed. Signaling overhead for both Alt 1 and Alt 2B is too large to be carried by RRC signaling via SIB1, thus these alternatives are unacceptable.

Observation 11: Both Alt 1 and Alt 2B will cause huge signaling overhead e.g., hundreds of bits.

In addition to the signaling overhead consideration, we think the design for Alt2 that LP-WUS MOs placed back-to-back to each other is sufficient considering combining alt2 and alt1A for the available symbol determination and alt C for the design of nominal/actual MO duration. Hence, there is no need to consider other methods which may complex or overly optimize the derive for the subsequent MOs.

Furthermore, we believe that the LR processing time will be a value at the symbol level rather than at the millisecond level (which is too slow and will also cause around dozens of milliseconds (4*8=32ms) extend of a LO length). So, the concern to ensure LR processing time can be addressed by extend the nominal MO duration a little bit. And the same way can be applied to the LP-SS occasion design. Hence, a single time gap between adjacent MOs i.e., alt 2A is not needed.

Observation 12: The LR processing time will be a value at the symbol level rather than at the millisecond level and it can be covered by the nominal MO duration.

Proposal 4: Support Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly based on the previous LP-WUS MO i.e., the LP-WUS MOs are located back-to-back.

  - Configuration between LO and PO including offsets and association

- Modify the comparison between wake-up delay and LO-PO offset

Agreement For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure 1 or 2 offset values. - FFS whether gNB can configure 3 offset values - If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay. - Note: if a single offset value is configured, UE behaviour is according to Option 1-1. - All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

As we agreed for supporting option 2B-1 in RAN1 #120 meeting, it is mentioned that if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay. Here, the mentioned offset between LO and PO is not clear enough. According to the last meeting agreement, both the defined frame-level offset and slot/symbol-level offset point to the start of LO frame or first LP-WUS MO of a LO. But for the comparison between wake-up delay and LO-PO offset, using the offset between the start of LO or MO to the PO is not correct, resulting in the loss of paging monitoring in some cases. instead, the wake-up delay needs to compare with the gap between the end of the LO and the corresponding PO to determine which LO-PO offset needs to be applied. Hence, the modification for this part is provided as follows.

Proposal 5: Modify the previous agreement in red on the judgement condition of UE LP-WUS monitoring behavior as below.

Agreement For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure 1 or 2 offset values. - FFS whether gNB can configure 3 offset values - If multiple offset values are configured and if the gap between the end of the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the end of LO and the PO no less than the wake-up delay. - Note: if a single offset value is configured, UE behaviour is according to Option 1-1. - All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

- The value range of the frame-level offset

For the frame-level offset value range, it should consider both the UE wake-up delay capability and the LO length. According to our contributions [X], a MO length can be up to 6 slots. Hence, if assuming 8 beams and 4 MOs per beam, there is 32 MOs per LO and the length of LO as assumed in section 3.1 is up to 254 slots. Considering there will be 4 POs per PF, up to 4 LOs associated with a PF if LO and PO is 1:1 mapping. So, the upper bound of frame-level offset value should at least be equal to the maximum UE wake-up delay plus 4 maximum LO lengths. As the 3 candidate values for the wake-up delay capability report are {[70ms], [500ms] and [900ms]} for 20ms SSB periodicity, the frame-level offset range for different wake-up delays are {8,..,108}, {51,..,151}, {91,..,191}. If we consider the minimum step is one radio frame, then the entire value range of a frame-level offset is {8,..,191}. Besides, since we have sent LS to RAN4 to ask if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities. So, we can further study the frame-level offset value range for other SSB periodicity.

Proposal 6: {8,..,191} can be the starting point of the frame-level offset value range for 20ms SSB periodicity. FFS: the frame-level offset value range for other SSB periodicity.

- Discuss the case that the number of POs per LO i.e., PO-to-LO association is smaller than the number of POs per PF i.e., Ns.

Agreement Regarding whether there is any restriction on mapping multiple POs to one LO, no additional constraint for mapping multiple POs to one LO.

In the previous meeting, we have discussion on several potential restrictions on mapping multiple POs to one LO, and finally agreed that no additional constraint for mapping multiple POs to one LO. That is to say, there can be the case that the number of POs per LO i.e., PO-to-LO association is smaller or larger than the number of POs per PF i.e., Ns. Figure 6 accordingly depicts the case PO-to-LO association < Ns, in which multiple LOs corresponding to the same PF. To indicated these multiple LOs’ position and avoid the overlap between LOs, the network can configure a list of offset values. However, according to the last meeting’s discussion, companies have different values on configuring a list of frame-level offset values or a list of slot-level offset values for this case. From our view, the essential difference between these two configurations lies only in the causing signaling overhead. It is obvious that using frame-level offset list is more overhead-efficient than using slot-level offset list. Since the values of these two types of offsets differ by Nslotframe*μ  times (quoted in TS 38.211), the difference of the number of required bits is accordingly to be:

Ns/POtoLO association*log2Nslotframe*μ

When SCS is 30KHz, Ns=4, PO-to-LO association=1, a list of slot-level offsets will cause additional 18bits compared to frame-level offset list. And the larger SCS value, the more additional required bits for the list of slot-level offsets. Hence, it will be more reasonable for the network to configure a list of frame-level offsets to address the case PO-to-LO association < Ns. And in the list of frame-level offsets, each frame-level offset value will determine the start of a frame for each LO as shown in Figure 6.

Observation 13: To address the case that PO-to-LO association < Ns, configuring a list of slot-level offsets will cause more signaling overhead e.g., additional dozens of bits than configurating a list of frame-level offsets.

Proposal 7: For the case PO-to-LO association is smaller than Ns, a list of frame-level offsets needs to be configured, and each frame-level offset value determines the start of a frame for each LO.

On how UE can determine to apply which frame-level offset value in the list, an example is given in Figure 2. When PO-to-LO association =2 and Ns=4, there will be Ns/POtoLO association=2 LOs associated with all the 4 POs of a PF. The Network will configure a list of frame-level offset containing Ns/POtoLO association values, wherein each frame-level offset corresponding to each LO. By the given frame-level offset list, a UE needs to use its associated PO index (i.e., is is the associated PO index of a UE, given in TS 38.304) to determine to apply which frame-level offset and monitor which LO. To formula that, a UE applies the (flooris/POtoLO association+1)-th value out of Ns/POtoLO association values of the frame-level offset list as its frame-level offset to determine its associated LO, E.g., when PO-to-LO association =2, Ns=4 and a UE is associated with PO3 i.e., is=2, the UE will apply the second (derived by floor2/2+1=2) frame-level offset to determine its associated LO.

Figure 6: An illustration for frame-level and symbol/slot level offsets of LO for the case that the network configures one frame-level offset for each PO.

Proposal 8: A UE applies the (flooris/POtoLO association+1) -th value out of Ns/POtoLO association values of the frame-level offset list, wherein is is the associated PO index of the UE, POtoLO association is the configured number of PO per LO.

- The value range of Slot-level offset for the first LP-WUS MO of a LO

Agreement For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. - FFS additional configuration to control the subsequent MO locations, e.g., - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration - FFS restriction on MO locations, e.g. only on DL slots - FFS minimum gap is needed between two MOs to ensure LR processing time

As we agreed, the starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. On slot-level or symbol-level offset, we think slot-level offset is enough. And the actual transmission can start from the first available symbols based on the preconfigured time-domain (in)available symbol pattern as discussed in section 3.1. For the value range of slot-level offset, we think it can be within a frame, so the upper bound can be the same as the length of a radio frame i.e., 10ms.

Proposal 9: The value range for the slot-level offset can be {0,…,10*2^μ}.

- Other potential restriction on mapping multiple POs to one LO

Although we have agreed that no additional constraint for mapping multiple POs to one LO, the discussion on potential restrictions on mapping multiple POs to one LO in the last meeting was limited to the number of POs/PFs per LO. In our view, there still can be other reasonable restrictions on mapping multiple POs to one LO. For example, from network perspective, the POs associated with one LO should be consecutive POs. As such, there is no need to introduce additional spec impact on how to determine the associated POs for a LO when PO-to-LO association < Ns. For example, as shown in Figure 2, LO1 associates with PO1 and PO3 (non-consecutive POs) should be excluded.

Proposal 10: The POs associated with one LO should be consecutive POs from the network perspective.

- Determine the first PF of the PFs associated with a LO

Besides, when PO-to-LO association > Ns, i.e., the case that PO-to-LO association is multiple of Ns. If one LO is associated with POs of two or four PFs, how to determine the first PF of the PFs associated with a LO needed to be discussed. Since very similar functionality for LP-WUS and R17 PEI, we propose to reuse the PF determination method of R17 PEI for LP-WUS, as shown in the following TS 38.304. To facilitate understanding, the first PF determination is given as Figure 2.

TS 38.304 If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N. The first PF of the PFs associated with the PEI-O is provided by (SFN for PF) - floor (iPO/Ns)*T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4a in TS 38.213[4], T, Ns, and N are determined in clause 7.1.

Figure 3: An illustration of first PF determination for the case PO-to-LO association is multiple of Ns.

Proposal 11: If one LO is associated with POs of two or four PFs, these associated PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N. The first PF of the PFs associated with the LO is provided by (SFN for PF) - floor (iPO/Ns)*T/N, where SFN for PF is determined in clause 7.1 of TS 38.304, iPO is defined as the associated PO index for a LO, T, Ns, and N are determined in clause 7.1 of TS 38.304.

  - LP-SS MO configuration

Agreement Each LP-SS transmission for each beam always occupies consecutive OFDM symbols.

In the last meeting, we have agreed that a LP-SS transmission occupies consecutive OFDM symbols. And the periodicity (160ms or 320ms) and offset for LP-SS transmission are also needed to be configured. Additionally, the discussions of 1) determination of the available symbol, 2) actual/nominal duration and 3) determination of subsequent MOs which happens to the design of LP-WUS MO should also be considered for LP-SS transmission. In our view, since these designs for LP-WUS MO are also applicable to LP-SS transmission, maximizing the reuse of these designs in LP-SS transmission will reduce standard effort and implementation complexity, unless other specific design requirements for LP-SS are identified.

Observation 14: The designs of 1) determination of the available symbol, 2) actual/nominal duration and 3) determination of subsequent MOs for LP-WUS MO can be reused for LP-SS transmission.

In detail, LP-SS reuses the determination of available symbols as that for LP-WUS, including the configured time-domain (in)available symbol pattern. Besides, there will be multiple LP-SS transmissions considering multiple beam operation. The network only needs to indicate the start slot of the first LP-SS transmission by slot-level offset. As we also mentioned for LP-WUS, slot-level offset to determine the first occasion is enough and the actual transmission can start from the first available symbols based on the preconfigured time-domain (in)available symbol pattern. Considering the signalling overhead and the non-necessity of configuring a single gap between LP-SS transmissions, two adjacent nominal LP-SS transmission occasion can be placed back-to-back i.e., the gap is zero. Similar as LP-WUS MO configuration, actual transmission duration which is derived by LP-SS sequence length and nominal transmission duration is to be configured, with minor difference design for LP-SS that an occasion is considered as invalid and deferred if the number of consecutive OFDM symbols is less than the actual LP-SS transmission duration to ensure LP-SS transmission in consecutive OFDM symbols as agreed in the last meeting.

Figure 4: An illustration of LP-SS occasions

Proposal 12: For LP-SS transmission, support the following designs:

- LP-SS reuses the available symbol determination method defined for LP-WUS.

- Nominal LP-SS occasion needs to be configured.

- Actual LP-SS transmission duration is derived by the configured LP-SS sequence length.

  - If the number of consecutive available OFDM symbols within the nominal LP-SS occasion is less than the actual LP-SS transmission duration, the occasion is considered as invalid (no LP-SS monitoring in this occasion) and deferred it to the next valid occasion.

- Two adjacent nominal LP-SS occasions are placed back-to-back i.e., no gap between them.

- gNB configures LP-SS periodicity and a slot-level offset for the first nominal LP-SS occasion, e.g.,

  - slot160, INTEGER {0..159}

  - slot320, INTEGER {0..319}

- Remaining issues on Multiple beam operation for LP-SS and LP-WUS

Agreement The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO. Agreement At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams. Agreement The previous agreement in RAN1#119 is updated as follows: - Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB Agreement Confirm the following working assumption with the additional text in red: Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications. - The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement. - FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.

RAN1 agreed existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. However, whether all descriptions of existing SS-RSRP and SS-RSRP is applicable to OFDM-based LP-WUR needs further clarification. For example, according to description in TS 38.215 and TS 38.331 as cited below, the network can configure a subset of SSB used for measurement by MR, where the subset is provided by SSB-ToMeasure. SSB-ToMeasure contains the SSB for neighbour cell measurement which is typically different from ssb-PositionsInBurst in SIB1 for serving cell. However, for LP-SSS-RSRP and LP-SSS-RSRQ measurement by LP-WUR, it does not make sense to measure SSBs according to SSB-ToMeasure because LP-SSS-RSRP/RSRQ is only for serving cell. LP-SSS-RSRP/RSRQ should be performed based on ssb-PositionsInBurst in SIB1.

Proposal 13: The set of SSB for LP-SSS-RSRP/RSRQ measurement is determined by ssb-PositionsInBurst in SIB1 rather than SSB-ToMeasure.

TS 38.215 5.1.1 SS reference signal received power (SS-RSRP) If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s).

TS 38.331 #### –SSB-ToMeasure The IE SSB-ToMeasure is used to configure a pattern of SSBs. For operation with shared spectrum channel access in FR1, only mediumBitmap is used, and for FR2-2, longBitmap is used.

In the previous meeting, whether the number of LP-WUS/LP-SS beam cannot be same with SSB beam is still under discussion. For simplicity, the number of LP-WUS/LP-SS same as number of SSBs indicated by ssb-PositionsInBurst in SIB1 is preferred, though some of the SSB beams may not be useful for LP-WUS/LP-SS considering relatively limited coverage of LP-WUR. Regarding RRM measurement based on LP-SS, the number of LP-SS is also same as the number of SSBs indicated by ssb-PositionsInBurst. It can be seen that, the number of beams for measurement by LP-WUR is same for both LP-SSS-RSRQ/RSRQ and LP-SS-RSRP/RSRQ.

Proposal 14: The number of LP-WUS/LP-SS beams is the same as the number of SSB beams determined by ssb-PositionsInBurst in SIB1.

Agreement The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

Besides, we have agreed that the network can configure the EPRE ratio between LP-WUS/LP-SS and SSB. There is also an FFS for whether strive a common configuration for LP-WUS and LP-SS. Considering the transmission power of LP-WUS and LP-SS can be different, it is reasonable to provide the network flexibility to set different EPRE ratio for LP-WUS and LP-SS transmission.

Proposal 15: Support separate EPRE ratio configuration for LP-WUS and LP-SS to SSB.

- Suggestion on the terminologies used in CR 38.213

During the CR discussion phase, we think the following coresponding realstionship is assumed and adopted by editor, according to the text in CR 38.213 that “A WUS monitoring occasion includes N⋅M WUS receptions”:

- a WUS monitoring occasion  a LO;

- a WUS reception a WUS MO;

We can follow this mapping based on the understand of editor, but also suggest to change some of “WUS reception” to “WUS reception occasion” to make it clearer. Since we find the usage of “WUS reception” in the current CR 28.213 is still not aligned. For example, in some texts of CR 38.213 (e.g., “a number of OOK symbols per OFDM symbol, the first RB, and [one or more] overlaid OFDM sequences per OOK symbol for WUS reception”), a WUS reception is understanded as a reception of a WUS, while in other texts (e.g., “A WUS monitoring occasion includes N⋅M WUS receptions”) a WUS reception is more like a WUS reception attempt. Moreover, we also suggest to align the description based on this mapping between RRC idle/inactive paragraph and RRC connected paragraph, as the TP suggested below with modification in red.

Proposal 16: Adopt the TP given in section 5 with modification in red.

## 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE A UE configured with DRX mode operation and operating in the RRC_IDLE or RRC_INACTIVE state can be provided for LPSS/WUS reception - a number of OOK symbols per OFDM symbol, a first RB, and an overlaid OFDM sequence per OOK symbol for LPSS reception [4, TS 38.211], - a periodicity and time offset for LPSS receptions by LP-SS-periodicityoffset, - a number of OOK symbols per OFDM symbol, the first RB, and [one or more] overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and - a number of subgroups per paging occasion, by subgroupNumber-PO-LPWUS A UE determines to receive LPSS/WUS based on procedures defined in [17, TS 38.304]. A UE assumes that an SCS configuration for LPSS/WUS receptions is same as an SCS of the initial DL BWP and an SCS configuration of an SS/PBCH block the UE used to obtain SIB1.A UE monitors WUS monitoring occasions with a periodicity equal to the DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines paging occasions associated with a WUS monitoring occasion based on PO-to-LO association. Each WUS monitoring occasion starts a number of frames prior to the first of a number of paging frames associated with the one or more WUS monitoring occasions. Each number of frames is provided by LO-FrameOffsets. If multiple values for the number of frames provided by LO-FrameOffsets are larger than or equal to the value of XYZ, the UE monitors WUS starting at an WUS monitoring occasion corresponding to the smallest of the multiple values. If all values for the number of frames provided by LO-FrameOffsets are smaller than the value of XYZ, the UE monitors PDCCH according to Type2-PDCCH CSS sets associated with the paging occasion and does not monitor WUS. A WUS monitoring occasion includes N⋅M WUS reception occasions, where - N is the number of [transmitted] SS/PBCH blocks as provided by [ssb-PositionsInBurst in SIB1], M is a number of WUS reception occasions associated with each of the N [transmitted] SS/PBCH blocks provided by MONumperLO - a WUS reception occasion with index n⋅M+m+1, m=0,1,…,M-1 and n=0,1,…,N-1, corresponds to [transmitted] SS/PBCH block [with index n+1], and - a WUS reception occasion with index n⋅M+m+1 is quasi co-located with a n-th [transmitted] SS/PBCH block provided by [ssb-PositionsInBurst in SIB1], with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable A UE assumes a periodicity for LPSS receptions is TBD and time occasions for LPSS receptions within an LPSS period are TBD. An LPSS reception for a (n+1)-th time occasion is quasi co-located with a (n+1)-th transmitted SS/PBCH block provided by [ssb-PositionsInBurst in SIB1], with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable. When, from a WUS reception, a UE determines a subgroup associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion reception when a time from the end of the WUS reception to the start of the PDCCH monitoring occasion is not smaller than the value of XYZ; otherwise, the UE is not required to perform the PDCCH monitoring. The UE may also perform PDCCH monitoring for Type3-PDCCH CSS sets for DCI format 2_7 based on its implementation. ## 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED A UE configured with DRX mode operation and operating in the RRC_CONNECTED state can be provided for WUS reception on the primary cell of a cell group - a number of OOK symbols per OFDM symbol, a first RB, and overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and - a number of [codepoints] provided for the UE by the WUS [6, TS 38.212], by WUS-codepointCONNECTED A UE assumes that a WUS is quasi co-located with an SS/PBCH block or a CSI-RS with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable. A UE assumes that an SCS configuration for WUS reception is same as an SCS configuration for the active DL BWP. A UE does not monitor a WUS during Active Time [11, TS 38.321]. A UE does not monitor WUS during DTX inactive period for the primary cell. If a UE detects a [codepoint/WUS information bit sequence] in a WUS reception occasion, from the number of [codepoints/WUS information bit sequences], on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group. A UE can be provided by LPWUS-MOCONNECTED-Option1-1 a periodicity, by periodicityMO-Option 1-1, and a time offset, by offsetMO-Option 1-1, to determine WUS reception occasions. The UE starts to monitor WUS in a first WUS reception occasion, provided by numMO-Option 1-1, that is not earlier than a first slot that is prior to a second slot where the drx-onDurationTimer would start by a time provided by timeOffsetCONNECTEDOption1-1, and monitors WUS for a number of monitoring occasions provided by numMO-Option 1-1. The UE reports a number of slots [18, TS 38.306] where the UE is not required to monitor WUS prior to the slot where the drx-onDurationTimer would start, and is not required to monitor WUS within the reported number of slots prior to the slot where the drx-onDurationTimer would start. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts the drx-onDurationTimer [11, TS 38.321]. A UE can be provided by LPWUS-MOCONNECTED-Option1-2 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, to determine first WUS reception occasions from a number of WUS reception occasions per periodicity, provided by numMO-Option 1-2. The UE reports a number of slots [18, TS 38.306] and expects that a time gap, from a last WUS reception occasion from the number of WUS monitoring occasions per periodicity to the slot where the lpwus-PDCCHMonitoringTimer would start, is no smaller than the reported number of slots. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts lpwus-PDCCHMonitoringTimer [11, TS 38.321] after a time, provided by timeOffsetCONNECTEDOption1-2, with respect to the start of the first WUS reception occasion from the number of WUS reception occasions per periodicity. ## *** Unchanged parts are omitted ***

- Conclusion

In this contribution, the procedure of LP-WUS monitoring in RRC idle/inactive mode is discussion on the aspect of LP-WUS subgrouping and LP-WUS monitoring configuration etc., with the following observations and proposals:

Observation 1: For the case that LO and PO to be 1: multiple mapping, there is no need to explicitly indicate the target PO index in a LP-WUS.

Observation 2: By applying alt 2 (i.e., information from existing configurations), the above NR channel/signals can be avoided to collide with LP-WUS transmission.

Observation 3: In addition to alt 2, alt1 can also be configured by the network to rate match e.g., the NR signal/channel transmission for RRC connected mode UE which is treated as unavailable symbol for LP-WUS transmission.

Observation 4: It would be quite challenge to find a unified per-MO pattern to avoid collision with existing NR signals/channels while efficiently provide sufficient resource for LP-WUS transmission in each MO by Alt 1B.

Observation 5: For Alt A, the detection performance of LP-WUS cannot be guaranteed, since there is insufficient time resource for a MO to do actual LP-WUS transmission, and it increases RM decoding complexity for variable code block length.

Observation 6: For Alt B without the constraint of nominal MO duration length, the following two drawbacks need to be considered:

- A LP-WUS transmission may span a long time period exceeding the coherence length, resulting in degradation of LP-WUS detection performance due to the change of the channel condition.

- The floating MO start time and the variable MO duration increase the complexity of scheduling management for the network.

Observation 7: For Alt C, since both nominal MO duration and actual MO duration are configured, there are no issues of LP-WUS scheduling complexity and performance degradation of LP-WUS.

Observation 8: For Alt C, by deferring an invalid MO instead of dropping an invalid MO, there will be no such a case that no valid MO for a LP-WUS beam.

Observation 9: For Alt C, even dropping an invalid MO, there still can be 2 or 3 valid MOs per beam in a typical slot-structure configurations, assuming 4 LP-WUS MOs are configured per beam.

Observation 10: For Alt D which is combinedly used with alt1B, it is challenge to find a unified per-MO pattern for all MOs to avoid collision with existing NR signals/channels while efficiently provide sufficient resource for LP-WUS transmission in each MO.

Observation 11: Both Alt 1 and Alt 2B will cause huge signaling overhead e.g., hundreds of bits.

Observation 12: The LR processing time will be a value at the symbol level rather than at the millisecond level and it can be covered by the nominal MO duration.

Observation 13: To address the case that PO-to-LO association < Ns, configuring a list of slot-level offsets will cause more signaling overhead e.g., additional dozens of bits than configurating a list of frame-level offsets.

Observation 14: The designs of 1) determination of the available symbol, 2) actual/nominal duration and 3) determination of subsequent MOs for LP-WUS MO can be reused for LP-SS transmission.

Proposal 1:

- UE identifies its subgroup specific codepoint value as follows:

- Let iPO denotes the PO index associated with a LP-WUS of the UE, with starting value of 0:

iPO=UE_IDmodN⋅NS+i_smodNPOLP-WUS,

where UE_ID, N, NS and i_s are defined in clause 7 of TS 38.304.

    - NPOLP-WUS denotes the configured number of PO associated with a LP-WUS.

- A UE determines its subgroup specific codepoint value as “iPO*NSGPO+iSG”, where

    - NSGPO denotes the configured number of LP-WUS subgroup per PO and 1≤NSGPO* NPOLP-WUS≤31;

    - iSG is the LP-WUS subgroup ID per PO for the UE and 0≤iSG<NSGPO.

- For a UE with PO index iPO, its common codepoint value is “NPOLP-WUS*NSGPO+iPO”.

- When a UE detects a codepoint value carried by a LP-WUS which is equal to “NPOLP-WUS*NSGPO+iPO” or “iPO*NSGPO+iSG”, the UE monitors its PO; otherwise, the UE is not required to monitor PO.

Proposal 2: Support Alt3 by combining alt 1A and alt2.

- For Alt 1A, support to partially reuse the mechanism of rate matching pattern (i.e., reuse bitmaps including symbolsInResourceBlock and periodicityAndPattern). And the network can configure a rate matching pattern for LP-WUS transmission accordingly.

- For Alt 2, SSB indicated by ssb-PositionsInBurst, CORESET/Type-0 CSS and UL symbols indicated by TDD DL/UL configuration which are overlapped with LP-WUS transmission are considered as unavailable symbols.

Proposal 3: Support Alt C i.e., both nominal MO duration and actual MO duration are configured.

- If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped or deferred.

Proposal 4: Support Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly based on the previous LP-WUS MO i.e., the LP-WUS MOs are located back-to-back.

Proposal 5: Modify the previous agreement in red on the judgement condition of UE LP-WUS monitoring behavior as below.

Agreement For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure 1 or 2 offset values. - FFS whether gNB can configure 3 offset values - If multiple offset values are configured and if the gap between the end of the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the end of LO and the PO no less than the wake-up delay. - Note: if a single offset value is configured, UE behaviour is according to Option 1-1. - All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

Proposal 6: {8,..,191} can be the starting point of the frame-level offset value range for 20ms SSB periodicity. FFS: the frame-level offset value range for other SSB periodicity.

Proposal 7: For the case PO-to-LO association is smaller than Ns, a list of frame-level offsets needs to be configured, and each frame-level offset value determines the start of a frame for each LO.

Proposal 8: A UE applies the (flooris/POtoLO association+1) -th value out of Ns/POtoLO association values of the frame-level offset list, wherein is is the associated PO index of the UE, POtoLO association is the configured number of PO per LO.

Proposal 9: The value range for the slot-level offset can be {0,…,10*2^μ}.

Proposal 10: The POs associated with one LO should be consecutive POs from the network perspective.

Proposal 11: If one LO is associated with POs of two or four PFs, these associated PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N. The first PF of the PFs associated with the LO is provided by (SFN for PF) - floor (iPO/Ns)*T/N, where SFN for PF is determined in clause 7.1 of TS 38.304, iPO is defined as the associated PO index for a LO, T, Ns, and N are determined in clause 7.1 of TS 38.304.

Proposal 12: For LP-SS transmission, support the following designs:

- LP-SS reuses the available symbol determination method defined for LP-WUS.

- Nominal LP-SS occasion needs to be configured.

- Actual LP-SS transmission duration is derived by the configured LP-SS sequence length.

  - If the number of consecutive available OFDM symbols within the nominal LP-SS occasion is less than the actual LP-SS transmission duration, the occasion is considered as invalid (no LP-SS monitoring in this occasion) and deferred it to the next valid occasion.

- Two adjacent nominal LP-SS occasions are placed back-to-back i.e., no gap between them.

- gNB configures LP-SS periodicity and a slot-level offset for the first nominal LP-SS occasion, e.g.,

  - slot160, INTEGER {0..159}

  - slot320, INTEGER {0..319}

Proposal 13: The set of SSB for LP-SSS-RSRP/RSRQ measurement is determined by ssb-PositionsInBurst in SIB1 rather than SSB-ToMeasure.

Proposal 14: The number of LP-WUS/LP-SS beams is the same as the number of SSB beams determined by ssb-PositionsInBurst in SIB1.

Proposal 15: Support separate EPRE ratio configuration for LP-WUS and LP-SS to SSB.

Proposal 16: Adopt the TP given in section 5 with modification in red.

# References

- 3GPP RAN1#120bis meeting, Chairman notes.