3GPP TSG RAN WG1 #121									R1-2503389

St Julian’s, Malta, May 19th – 23th, 2025

Source:          vivo

Title:		 UE features for Rel-19 LP-WUS/WUR

Agenda Item:    9.15.5

Document for:   Discussion and Decision

- Introduction

In this contribution, we provide updates to UE features for Rel-19 LP-WUS/WUR based on the latest agreements made.

- NR_LPWUS

- Proposal with change mark (based on RAN1#120 endorsed version in R1-2501397 and RAN1#120bis further agreements in R1-2502966)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement with all M values (M=1,2,4) for LP-SS 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay 6. The supported M values for LP-WUS 7. The support of LP-WUS/LP-SS frequency resource within or outside the initial DL BWP where UE receives paging but within the same carrier FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: - For SSB periodicity 20ms, candidate values:[70]ms, [500]ms, [900]ms - FFS for other SSB periodicities. Component 6: - For 15KHz SCS: support all M values (M=1,2,4) - For 30KHz SCS: Support one of {M=1,2 or M=1,2,4} - For 60KHz SCS: FFS - For 120KHz SCS: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence(s) | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence(s), all M values (M=1,2,4) supported 2. FFS: The support of LP-SS based RRM measurement X. The support of SSB-based RRM measurement 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay 6.The support of LP-WUS/LP-SS frequency resource within or outside the initial DL BWP where UE receives paging but within the same carrier 7. The support of maximum 16, 8, 4 candidate overlaid sequences for M=1,2,4, respectively. FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence(s) is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: - For SSB periodicity 20ms, candidate values:[70]ms, [500]ms, [900]ms - FFS for other SSB periodicities. | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | - LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OOK signal，all M values (M=1,2,4) supported - Support of LP-WUS frequency resource within active DL BWP - The supported procedure(s) - Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode - The supported maximum number of codepoints monitored per LP-WUS MO FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: (Option 1-1, Option 1-2, both) Component 4 candidate values: {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3 Component 5: candidate value: any integer value in the range of 2~8 | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) | - LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OFDM overlaid sequence(s), all M values (M=1,2,4) supported - Support of LP-WUS frequency resource within active DL BWP - The supported procedure(s) - Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode - The supported maximum number of codepoints monitored per LP-WUS MO - The support of maximum 16, 8, 4 candidate overlaid sequences for M=1,2,4, respectively. FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: (Option 1-1, Option 1-2, both) Component 4 candidate values: {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3 Component 5: candidate value: any integer value in the range of 2~8 | Optional with capability signalling |
| 62. NR_LPWUS | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |

- Proposal in clean version.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement with all M values (M=1,2,4) for LP-SS 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay 6. The supported M values for LP-WUS 7. The support of LP-WUS/LP-SS frequency resource within or outside initial DL BWP where UE receives paging but within the same carrier FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: - For SSB periodicity 20ms, candidate values:[70]ms, [500]ms, [900]ms - FFS for other SSB periodicities. Component 6: - For 15KHz SCS: support all M values (M=1,2,4) - For 30KHz SCS: Support one of {M=1,2 or M=1,2,4} - For 60KHz SCS: FFS - For 120KHz SCS: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence(s) | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence(s), all M values (M=1,2,4) supported 2. FFS: The support of LP-SS based RRM measurement X. The support of SSB-based RRM measurement 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay 6. The support of LP-WUS/LP-SS frequency resource within or outside initial DL BWP where UE receives paging but within the same carrier 7. The support of maximum 16, 8, 4 candidate overlaid sequences for M=1,2,4, respectively. FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence(s) is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: - For SSB periodicity 20ms, candidate values:[70]ms, [500]ms, [900]ms - FFS for other SSB periodicities. | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | - LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OOK signal，all M values (M=1,2,4) supported - Support of LP-WUS frequency resource within active DL BWP - The supported procedure(s) - Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode - The supported maximum number of codepoints monitored per LP-WUS MO FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: (Option 1-1, Option 1-2, both) Component 4 candidate values: {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3 Component 5: candidate value: any integer value in the range of 2~8 | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) | - LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OFDM overlaid sequence(s), all M values (M=1,2,4) supported - Support of LP-WUS frequency resource within active DL BWP - The supported procedure(s) - Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode - The supported maximum number of codepoints monitored per LP-WUS MO - The support of maximum 16, 8, 4 candidate overlaid sequences for M=1,2,4, respectively. FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: (Option 1-1, Option 1-2, both) Component 4 candidate values: {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3 Component 5: candidate value: any integer value in the range of 2~8 | Optional with capability signalling |
| 62. NR_LPWUS | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |