3GPP TSG-RAN WG1 Meeting #RAN 121	R1-2503448

Malta, Malta, May 17 – 21, 2025

Agenda Item:	9.1.3

Source:	TCL

Title:	Discussion on CSI prediction

Document for:	Discussion and Decision

Introduction

Based on the discussion at the RAN#105 meeting [1], the AI-based CSI prediction using the UE-sided model has been transitioned from SID to WID. The main objectives are highlighted in the yellow section below:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In this contribution, we provide our views on AI-based CSI prediction using the UE-side model.

CSI prediction using UE-sided model

In this section, our views are provided from the following four aspects: data collection, model inference, performance monitoring, and consistency between training and inference for AI-based CSI prediction using the UE-side model.

Data collection

According to the process of the study on AI-based CSI prediction using the UE-side model, some specification impact on data collection has been proposed by companies as follows:

Data collection: In CSI prediction using UE sided model use case, at least the following aspects have been proposed by companies on data collection, including: - Signalling and procedures for the data collection - Data collection indicated by NW - Requested from UE for data collection - CSI-RS configuration - Assistance information for categorizing the data, if needed - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. Agreement For CSI prediction using UE-side model, for data collection for training, - For resource configuration, - For CSI-RS resource type for CMR, periodic and semi-persistent are supported. - FFS on support of aperiodic CSI-RS resource - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead. - FFS on CSI report configuration Agreement For CSI prediction using UE-side model, for training data collection, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report - FFS on how to indicate without CSI report in CSI-ReportConfig

For training data collection, the UE is not required to provide feedback on the predicted CSI to the NW. However, the UE needs to be aware that the CSI-ReportConfig is configured for data collection purposes without requiring a CSI report. In our opinion, both explicit and implicit methods can be employed to indicate the purpose of the CSI-ReportConfig.

For explicit methods, a new RRC parameter could be introduced in the CSI report configuration to explicitly indicate that the configuration is intended for data collection. Additionally, setting the reporting quantity to ‘none’ might offer a more straightforward approach.

For implicit methods, the absence of UL reporting resources implies that the CSI report configuration is designated for data collection. For example, if the ‘pucch-CSI-ResourceList’ is not configured for periodic or semi-persistent reporting on PUCCH, the ‘reportSlotOffsetList’ is not configured for semi-persistent reporting on PUSCH, and aperiodic reporting is not supported for training data collection.

However, based on the conclusion of the previous meeting, a new RRC parameter has been introduced to distinguish between the CSI reports for AI-CSI prediction and non-AI CSI prediction. Therefore, we believe that a unified design can be adopted to differentiate the CSI reporting configurations for data collection and inference.

: For training data collection, a new RRC parameter is introduced to distinguish between CSI reporting configurations for data collection and inference.

For training data collection, there are two approaches: 1) NW-initiated and 2) UE-requested. Based on RAN2's agreement for BM, both options are allowed for the UE-side model. In our opinion, when the UE sends a request for data collection, it is only intended to inform the NW that it is prepared for data collection. Therefore, we believe that reusing the SR mechanism is sufficient, such as using a specific SR.

: For CSI prediction, when a UE requests data collection for training, the reuse of the SR mechanism should be supported, such as using a specific SR.

Model inference

Based on previous research, the potential specification impacts for model inference have been proposed by companies as follows:

Inference: For CSI prediction using UE-sided model, legacy feedback mechanism using codebook type set to “typeII-Doppler-r18” is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects: - CSI processing criteria and timeline Agreement For CSI prediction using UE-side model, for CSI processing criteria and timeline, at least for inference further study on - Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting - Whether the Processing Unit should be shared or separately counted among AI/ML related features/functionalities. - Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates. - Whether legacy framework for active CSI-RS resource and port counting can be reused Note: Strive to study CSI processing criteria considering both BM and CSI case, and take the existing solutions as starting point. Agreement For CSI prediction using UE side model, for inference, consider following options for potential down selection - Option 1: only dedicated AI/ML PU (OAPU) is occupied - Option 2: only legacy CPU (OCPU) is occupied - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied - FFS whether OAPU and OCPU are based on defined rule and/or reported by UE - Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately. Agreement For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18') Agreement For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction. - Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion - Detailed parameter name is upto RAN2

In this section, we will present our views on model inference for CSI prediction using the UE-side model.

Update the number of predicted CSI dynamically

For the legacy Rel-18 Doppler codebook, the number of predicted CSIs is configured semi-statically by the RRC. Regardless of whether it is the legacy Rel-18 Doppler codebook or CSI prediction using the UE-side model, the performance of the predicted CSIs changes with variations in the channel environment. However, for CSI prediction using the UE-side model, the performance of the predicted CSIs can be monitored through performance evaluation. Intuitively, the further into the future the predicted CSI, the poorer the prediction performance becomes. Therefore, when the performance of the predicted CSI at longer time distances is detected to deteriorate, the model can reduce the number of predicted CSIs. Based on this, we propose that dynamically updating the number of predicted CSIs should be supported.

:For CSI prediction using UE-side model, dynamically update the number of predicted CSI should be supported.

CSI processing unit and timeline

In the last meeting, regarding the question of whether the CPU should be shared or separately allocated between legacy CSI reporting and AI/ML-based CSI reporting, three options were put forward, as mentioned above. In our opinion, this question depends on whether there are some hardware resources necessary and specific for AI/ML-based CSI reporting. For example, if the UE wants to realize some AI-based CSI reporting, the base-band system of  the UE requires to configure high performance CPU, GPU, or NPU. However, by leveraging model lightweight techniques such as pruning, quantization, and knowledge distillation, or through algorithm-hardware co-design approaches like binary neural networks, AI inference can be implemented on existing mobile hardware (CPU/DSP) without requiring additional hardware. For instance, lightweight CNNs or small-scale RNNs may not require NPU support.

- Based on the above analysis, we believe that whether the CPU is shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting is decided by the UE capability. Moreover, considering the AI-technical development, the AI process may not requires the dedicated resource, it depend on the hardware implementation of chip manufacturers or terminal manufacturers. In addition, the option 3 raises several questions, such as which process will occupy the dedicated AI/ML PU (OAPU) and for how long, as well as which processes will occupy the legacy CPU and the duration of their processing time. We believe it is very difficult to clearly define the processing time. If the processing time is not clearly defined, it could result in uncertainty in CSI reporting. Therefore, we prefer option 1 and option 2.

: For inference, we prefer option 1 and/or option 2. If multiple options are supported, the applied option will depends on the UE’s capability.

For CSI prediction using the UE-side model, the reporting quantity varies depending on the operating mode. For example, during model training, the UE does not report anything; during model inference, the UE compresses and reports the predicted CSIs; and during performance monitoring, the UE reports either the performance metrics or both the predicted CSIs and performance metrics together. The different reporting quantities correspond to varying levels of CSI computational complexity. Moreover, the structure of the AI/ML models may also affect the model inference time. Therefore, we suggest that the CSI processing criteria and timeline for CSI prediction should be further discussed for different stages of model operation, such as model training, model inference, and performance monitoring.

: For CSI prediction using a UE-side model, the CSI processing criteria and timeline should be further discussed for different stages of model operation.

CSI reporting priority

During the previous meeting, it was agreed that the inference report 'typeII-Doppler-r18' would be reused, meaning that the reporting quantity remains the same as that of 'typeII-Doppler-r18'. Consequently, we believe that the legacy CSI reporting priority calculation formula, Prii,CSI(y,k,c,s), can also be reused. Furthermore, the meeting concluded that a new RRC parameter should be introduced to distinguish between CSI reports generated by AI-CSI prediction and those from non-AI CSI prediction. Typically, AI-based methods for the same functionality outperform legacy methods. Therefore, regardless of the value of Prii,CSI(y,k,c,s), AI-based CSI reporting should have a higher priority than legacy CSI reporting, regardless of whether it carries L1-RSRP or L1-SINR.

: For AI-based CSI prediction, the priority value calculation for inference CSI reporting can reuse the legacy formula Prii,CSI(y,k,c,s).

: For the same functionality, AI-based CSI reporting has a higher priority than legacy CSI reporting regardless of the value of Prii,CSI(y,k,c,s).

Performance monitoring

Based on the previous meeting, the potential specification impacts for performance monitoring have been proposed by the companies as follows:

Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW; - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/deactivation/switching as defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW. For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action Agreement For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support - Based on intermediate KPI - Down select between SGCS and NMSE by RAN1#120bis - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation. Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism

In this part, our views on performance monitoring for CSI prediction using UE-sided model are presented.

CSI-RS configuration for performance monitoring

For the legacy Rel-18 Doppler codebook, when the channel measurement resource is a periodic or semi-persistent CSI-RS resource, the time-domain interval between two adjacent predicted CSIs is equal to the period of the measurement resource. We believe this mechanism will be reused for AI-based CSI prediction, and the periodic and semi-persistent CSI-RS resources used for model inference can also be utilized for performance monitoring.

However, for aperiodic CSI measurement and reporting, additional aperiodic CSI-RS resources are required to obtain the ground-truth CSI within the prediction window.

: For CSI prediction using the UE-side model, the periodic or semi-persistent CSI-RS resource can be utilized for both model inference and performance monitoring simultaneously.

:For aperiodic CSI measurement and reporting, additional aperiodic CSI-RS resources should be configured to enable performance monitoring.

Performance monitoring metric reporting

In the previous meeting, three alternatives for the reporting contents of UE-assisted performance monitoring were mentioned by some companies. For Alternative 1, we believe it is the most direct and effective method to evaluate the performance of the inference. For Alternative 2, we think having two SGCS with different calculation methods is unnecessary. For Alternative 3, we believe the reporting overhead for performance monitoring is acceptable, and the statistical information for performance monitoring can be processed at the base station. It is unnecessary for the UE to cache a large amount of monitoring data within the monitoring window. Based on the above analysis, we believe that Alternative 1 is sufficient for performance monitoring.

: For the reporting contents of the UE-assisted performance monitoring, Alternative 1 is sufficient.

- Moreover, from the perspective of monitoring signal integrity, we believe that reporting per prediction instance should be used as a baseline, as it helps the base station obtain more monitoring information.

: For reporting contents of UE-assisted performance monitoring, support reporting per prediction instance.

- For the reporting contents of UE-assisted performance monitoring, we believe that the frequency granularity of the reported SGCS value should be averaged over subbands.

: For the reporting contents of UE-assisted performance monitoring, support calculating the SGCS value based on the averaged over subbands.

- CSI reporting for both AI-based CSI prediction and legacy Rel-18 Doppler codebook is conducted per layer, and per-layer CSI reporting helps the base station determine the number of layers to schedule. Therefore, we believe that for performance monitoring metrics, at least per-layer reporting should be supported. From the perspective of reducing reporting overhead, averaged-over-layer reporting is also acceptable.

: For reporting contents of UE-assisted performance monitoring, at least per-layer reporting should be supported.

- For performance monitoring metric reporting, the range of SGCS values is [0,1], and these values are only used by the network to determine whether the inference result is reliable. Therefore, we believe that high precision is unnecessary, and 3-bit equal interval quantization should be sufficient. Moreover, differential quantization is unnecessary as it would increase reporting overhead due to the need to indicate the location of the maximum or minimum value.

: For performance monitoring metric reporting,  equal interval quantization is sufficient.

The CSI process unit for performance monitoring reporting

In the last meeting, it was agreed that SGCS would be used as the performance monitoring metric. It is clear that for performance monitoring metric reporting, the use of an AI model is not required. Therefore, we believe that performance monitoring metric reporting will only utilize the legacy CPU. Moreover, for AI-based beam management, it was agreed that performance monitoring result reporting would occupy one CPU. We believe that this conclusion can also be applied to AI-based CSI prediction.

:For performance monitoring metric reporting, only the legacy CPU is utilized, and Ocpu=1.

Performance monitoring metric reporting priority

For legacy CSI reporting, the priority of CSI reporting depends on the following three main factors: whether the CSI reporting carrier includes L1-RSRP or L1-SINR, the time-domain behavior of CSI reporting, and the container of CSI reporting.

As performance monitoring metric reporting is introduced as a new UCI type, we believe that the above three main factors should also be considered. Based on the conclusions of the previous meeting, performance monitoring metric reporting can support periodic, semi-persistent, and aperiodic reporting. Moreover, considering AI-based beam management, we propose that the priority of performance monitoring metric reporting for AI-based CSI prediction should be discussed together with AI-based beam management. Additionally, the priority of performance monitoring metric reporting should take into account whether the associated CSI reporting carrier includes L1-RSRP or L1-SINR.

:For the priority of performance monitoring metric reporting, both AI-based beam management and AI-based CSI prediction should be discussed together. The priority determination should take into account whether the associated CSI reporting carrier includes L1-RSRP or L1-SINR.

Consistency between training and inference

Based on the simulation results provided by some companies in the last meeting, we observed that for the CSI prediction use case, the performance degradation caused by the inconsistency between model training and model inference is very limited. Therefore, we believe that there is no need for specification enhancements to ensure consistency between model training and model inference.

: No specification enhancements are needed to ensure consistency between model training and model inference.

Conclusions

This contribution presents our views on AI-based CSI prediction. We have the following proposals:

- : For training data collection, a new RRC parameter is introduced to distinguish between CSI reporting configurations for data collection and inference.

: For CSI prediction, when a UE requests data collection for training, the reuse of the SR mechanism should be supported, such as using a specific SR.

:For CSI prediction using UE-side model, dynamically update the number of predicted CSI should be supported.

: For inference, we prefer option 1 and/or option 2. If multiple options are supported, the applied option will depends on the UE’s capability.

: For CSI prediction using a UE-side model, the CSI processing criteria and timeline should be further discussed for different stages of model operation.

: For AI-based CSI prediction, the priority value calculation for inference CSI reporting can reuse the legacy formula Prii,CSI(y,k,c,s).

: For the same functionality, AI-based CSI reporting has a higher priority than legacy CSI reporting regardless of the value of Prii,CSI(y,k,c,s).

: For CSI prediction using the UE-side model, the periodic or semi-persistent CSI-RS resource can be utilized for both model inference and performance monitoring simultaneously.

:For aperiodic CSI measurement and reporting, additional aperiodic CSI-RS resources should be configured to enable performance monitoring.

: For the reporting contents of the UE-assisted performance monitoring, Alternative 1 is sufficient.

: For reporting contents of UE-assisted performance monitoring, support reporting per prediction instance.

: For the reporting contents of UE-assisted performance monitoring, support calculating the SGCS value based on the averaged over subbands.

: For reporting contents of UE-assisted performance monitoring, at least per-layer reporting should be supported.

: For performance monitoring metric reporting,  equal interval quantization is sufficient.

:For performance monitoring metric reporting, only the legacy CPU is utilized, and Ocpu=1.

:For the priority of performance monitoring metric reporting, both AI-based beam management and AI-based CSI prediction should be discussed together. The priority determination should take into account whether the associated CSI reporting carrier includes L1-RSRP or L1-SINR.

: No specification enhancements are needed to ensure consistency between model training and model inference.

- References

- RP-242399, Revised WID for NR_AIML_air, Sep 9 – 12, 2024