3GPP TSG RAN WG1 #121		          R1-2503510

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:     9.2.2

Source:	Spreadtrum, UNISOC

Title:	Remaining issues on CSI enhancements

Document for:	Discussion and decision

Introduction

In RAN1#120b meeting, RAN1 discussed R19 CSI enhancements, and plenty of agreements have been achieved [1]. Based on the agreements, the feature design is almost finished, and there are only a few remaining issues to be discussed in this meeting.

In this contribution, we provide our views and suggestions on some of the remaining issues.

Discussion

Type-I and Type-II codebook refinement for up to 128 CSI-RS ports

During the last meeting, RAN1 discussed the triggering offset of the associated aperiodic CSI-IM for the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook. The following agreement is listed below.

Agreement For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot. - FFS: How to resolve this for Rel-19 Type-II Doppler

Regarding the Rel-19 Type-II Doppler codebook, similar issue needs to be resolved.

Based on the current spec, only one CSI-IM resource can be configured for the Rel-18 Type-II Doppler codebook. The UE reports a CSI report only if receiving at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or one CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource.

For Rel-19 Type-II Doppler codebook, the UE can be configured with KDOPP = {4, 8, 12} CSI-RS resource groups, and each CSI-RS resource group comprises K NZP CSI-RS resources. When the K>1 aggregated NZP aperiodic CSI-RS resources in each resource group for CMR are within two consecutive slots, it is simple and straightforward to define the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) in the first resource group for CMR in the first slot.

Proposal: For the Rel-19 Type-II Doppler codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources in each resource group for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) in the first resource group for CMR in the first slot.

Conclusion

In this contribution, we provided our views on the details of the remaining issues on CSI enhancements. The following proposals are made:

Type-I and Type-II codebook refinement for up to 128 CSI-RS ports

Proposal: For the Rel-19 Type-II Doppler codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources in each resource group for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) in the first resource group for CMR in the first slot.

References

[1] Draft_Minutes_report_RAN1#120b_v010