3GPP TSG RAN WG1 #121                                                                R1-2503515

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.4.1

Source:	Spreadtrum, UNISOC

Title:	Discussion on modulation aspects of physical channels design for Ambient IoT

Document for:	Discussion and decision

Introduction

The WID of solution for Ambient IoT (Internet of Things) in NR was updated in RAN#107 is as follows [1]:

General Scope … The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212. Applying or not applying the FEC to D2R is specified by ensuring it is under the reader control and applies to all devices targeted by the reader. - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission. …

In this contribution, we will discuss the modulation aspects of physical channels design.

Discussion

R2D Numerology

Issue: Alignment between the end of R2D transmission and OFDM symbol boundary

In RAN1#120b, the following agreement was made about the alignment between the end of R2D transmission and OFDM symbol boundary [2].

Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. - Padding is to be down-selected among the following alternatives: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. FFS how accurately the device can be aware. - Alt 1b: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the start of the padding. - Alt 2: Specify the detailed content of padding - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: the end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements.

Firstly, from our understanding, once an ambient IoT device has determined the end of a R2D transmission, e.g., the postamble is detected, the following transmission except another preamble will be omitted at the ambient IoT device side. Obviously, there is no need for device to know the content of padding and it can be transparent for the device. Hence, Alt 2 should not be supported.

Secondly, the difference between Alt 1a and Alt 1b is the reference point for determining all the timing relationship. Actually, in ambient IoT system, it is not expected that the R2D and D2R transmission in the same time. Hence, the comparison between the length of TR2D,min and padding should be further considered which will affect the down selection between Alt 1a and Alt 1b, and the minimum time gap between R=>T and T=>R in RFID system can be as a reference.

In RFID system [3], the minimum time gap (T1) between R=>T and T=>R is

max(RTcal, 10Tpri) * (1-FrT) – 2us

where the definition and value of parameters in RFID are given in the following Table.

Table 1: RFID parameters

| Parameter | Definition | Value |
| --- | --- | --- |
| RTcal | interrogator-to-tag calibration symbol | [2.5~3] Tari |
| Tari | Basic time unit | 6.25us, 25us |
| Tpri | T=>R link period | Tpri = 1/BLF |
| BLF | backscatter-link frequency | BLF = DR/TRcal |
| DR | divide ratio | 8, 64/3 |
| TRcal | tag-to-interrogator calibration symbol | 1.1 RTcal <= TRcal <= 3 * RTcal |
| FT | the frequency tolerance for backscatter operation. | ±15%, ±20%. ±10%, … |

Besides, for the T=>R transmission of RFID, Table 2 are used for determining the Tag-to-interrogator link frequencies.

Table 2: Tag-to-interrogator link frequencies

Leveraging the above two tables, the minimum value of T1 in RFID can be derived as:

T1min = Max(6.25, 10 * 33.3 * 3 / 64) * (1 – 22%) – 2 us = 10.1758 us.

Obviously, the minimum gap between R=>T and T=>R in RIFD is smaller than the length of an OFDM symbol. If ambient IoT share the same hardware capability with the typical RFID tag, the TR2D,min may be close to T1 of RIFD system. It means the padding may be larger than TR2D,min, and the R2D and D2R transmission may occur at the same time.

Hence, for avoiding R2D and D2R transmitting in the same time, we prefer to Alt 1a, where the reference point is at the end of the padding.

Proposal 1: Alt 1a for R2D padding should be supported, where the timeline determination of any timing relationship refers to the end of padding.

Another issue of Alt 1a is how to be aware of the duration of padding or the last OFDM symbol boundary from the device perspective. In our opinion, according to the local clock frequency and the number of sampling points in a chip, the device can infer the M value. Since the numerical interval between different supported M values is larger, the device can determine the M value accurately. Besides, the device knows the starting of the CAP, hence it can deduce the end of each OFDM symbol by counting the chip.

Observation 1: Ambient IoT device can be aware of the end of padding or the last OFDM symbol boundary by counting the chip, leveraging the inferred M value.

R2D Waveform

In the last meeting, the following agreement about how to capture the R2D waveform in the TS was achieved [2].

Agreement From reader perspective, for the needed certain specification of DFT-s-OFDM waveform generation for OOK-4 modulation: - An example is provided below, which does not presume any specific reader implementation: - Step 1: The time domain OOK signal is the M chips of one OFDM symbol - The specification only needs to reflect that one OFDM symbol contains M chips - Step 2: A chip is represented (e.g. upsampled) by L samples - The specification only needs to reflect that one chip contains L samples as the input to N’-points DFT - Step 3: An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal. - The specification only needs to reflect that there is an N’-points DFT operation, where N’ = M*L - Step 4: Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D - The specification only needs to reflect that N’ >= X, where X is corresponding to the Btx,R2D - Step 5: An N-points IDFT is performed to obtain the time domain signal. - The specification only needs to reflect that there is an N-points IDFT operation - Note: other examples were provided in contributions to RAN1#120bis, e.g. in annex 2 of R1-2502160 - From the example above, some normative specification related to at least step 1 and step 5 are needed. - Note: RAN1 to consider whether an information annex could describe other steps - Note: some normative RAN1 specification text about waveform is assumed to be needed for RAN4 requirements definition - Note: the specification also needs to reflect the timing of the CP insertion operation.

The current draft TS contains the text about step 1 and step 5, and whether any details of other steps captured in the TS should be further considered. From our understanding, the details of the above steps other than step 1 and step 5 is not needed. Because the exact value of L, N, and N’ should totally up to reader implementation, and the device is not aware of these parameters when it performs reception. Regarding some limitations for waveform generation, i.e., the sub-bullet of each step in the above agreement, it is necessary captured in the TS for generate a suitable baseband waveform.

Proposal 2: The limitations for waveform generation, i.e., the sub-bullet of each step in the above agreement, should be considered to be captured into TS, and the exact value of L, N, and N’ should totally up to reader implementation without any spec impact.

R2D CP handling

In the last meeting, the following agreements were made [2].

Agreement For R2D, at least for PRDCH, the set of M values is {2, 6, 12, 24} - FFS: whether/how CAP use same M values set as PRDCH Agreement For the below agreement, further update on the followings Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

According to the above agreements, at least Alt M1 will be used for any M values. For M value is 24, whether additionally adopting Candidate 3 of M2-1-1 (the last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’) will be further discussed.

From our understanding, when the M value is larger, the CP is comparable or larger than a chip. The CP will seriously affect the demodulating of the Ambient IoT device. Due to limited receiver sensitivity and large SFO, the reference point for determining a chip may drift and Alt M1 may not work well for M = 24. Hence, Option 1 in the above agreement should be supported.

Proposal 3: Candidate 3 of M2-1-1 should be supported for R2D CP handling with M = 24.

When M of CAP and PRDCH is 24,

- Based on our analysis in [4] of AI 9.4.3, the length of SIP should be fixed to one OFDM symbol and the ON-OFF with a ratio 1:3. Hence, the possible M value for SIP is 4, and no padding is needed for SIP part. The CP will copy the last chip of the OFDM symbol with low voltage. From the device’s perspective, the durations of ON and OFF voltage chips of the SIP are not changed. Hence, there is no impact of CP insertion for SIP detection.

- For the CAP of R-TAS, as shown in Figure 1, the device can use the duration of OFF to determine the chip duration. Hence, the impact for calibrating the chip length of CP inserted before the CAP can be omitted.

- For the OFDM symbol(s) carrying PRDCH, the padding chips and CP can be detected by counting the sample points.

To sum up, Candidate 3 can work well for the case with M = 24 for CAP and PRDCH.

Figure 1: CP handling for Preamble (take M = 24 as an example)

For high TBS case, the performance gap will be larger than that of low TBS case. While from device implementation and complexity perspective, if Alt M2-1-1 is supported with M = 24, it should be always to be supported no matter high or low TBS case.

Proposal 4: if Alt M2-1-1 is supported with M = 24, it should be always to be supported no matter high or low TBS case.

Considering that 2 padding bits will be inserted behind every 22 bits (after line coding), hence before step 1 of DFT-s-OFDM based OOK-4 signal generation, a pre-processing process is required for inserting the padding bits, which should be captured into TS.

Proposal 5: The pre-processing process for inserting the padding bits between every 22 information bits (after line coding) should be captured into TS.

R2D chip length

The definition of R2D chip length is related to the methods of CP handling. Based on the agreement achieved in the last meeting, i.e., at least Alt M1 should be supported, the chip length should be defined as

R2D chip length = 1 / (SCS * M).

Proposal 6: The R2D chip length in one OFDM symbol is defined as 1 / (SCS*M).

Summary

In the contribution, we provides our view on the ambient IoT device architectures, and propose that,

Proposal 1: Alt 1a for R2D padding should be supported, where the timeline determination of any timing relationship refers to the end of padding.

Observation 1: Ambient IoT device can be aware of the end of padding or the last OFDM symbol boundary by counting the chip, leveraging the inferred M value.

Proposal 2: The limitations for waveform generation, i.e., the sub-bullet of each step in the above agreement, should be considered to be captured into TS, and the exact value of L, N, and N’ should totally up to reader implementation without any spec impact.

Proposal 3: Candidate 3 of M2-1-1 should be supported for R2D CP handling with M = 24.

Proposal 4: if Alt M2-1-1 is supported with M = 24, it should be always to be supported no matter high or low TBS case.

Proposal 5: The pre-processing process for inserting the padding bits between every 22 information bits (after line coding) should be captured into TS.

Proposal 6: The R2D chip length in one OFDM symbol is defined as 1 / (SCS*M).

Reference

- RP-2507966, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, March, 2025

- R1-2503109, Final summary for Ambient IoT: “9.4.1 Physical channels design – modulation aspects”, Moderator(Huawei)

- EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID

- R1-2503517, Discussion on  timing acquisition and synchronization for Ambient IoT, Spreadtrum, Unisoc