3GPP TSG RAN WG1#121	                                       	 R1-2503526

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:     9.9.1

Source:	Spreadtrum, UNISOC

Title:	             Discussion on measurements related enhancements for LTM

Document for:	Discussion and decision

Introduction

The revised work item for Rel-19 NR mobility enhancements Phase 4 in RAN#105 [1] includes the objectives as follows. In this contribution, we will share our views on the issues for measurements related enhancements for purpose of supporting LTM, which include the event triggered L1 measurement reporting and CSI-RS measurements for LTM procedures.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Discussion on CSI acquisition for candidate cell(s)

Working Assumption As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified) Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement Agreement For the container of LTM CSI report to a target cell, UCI is used with a solution to avoid blind detection at gNB if valid CSI is not available - FFS: the definition of valid CSI Agreement For the solution to avoid blind detection for LTM CSI report if valid CSI is not available, - Option 1: CSI is reported when valid CSI is not available - Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available - Option 1-1-1: Codepoint with the lowest CQI index is reported Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability

In RAN#105 meeting, the work item for Rel-19 NR mobility enhancements Phase 4 was revised which includes CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch. The motivation for CSI acquisition is to obtain CSI of target cell and improve throughput on the target cell immediately after cell switch. In RAN1#120 meeting, the working assumption was confirmed, i.e. as baseline, CSI-RS measurement and CSI reporting operations are performed after the reception of LTM CSC MAC CE, while a UE capability for CSI-RS measurement starting before the reception of LTM CSC MAC CE is introduced.

The periodic CSI-RS resource and semi-persistent CSI-RS resource are supported for CSI measurement. The period to CSI measurement depends on the UE capability. For example, for UE supporting CSI measurement before the reception of LTM CSC, P-CSI-RS measurement can be started after the RRC configuration or SP-CSI-RS measurement can be started after the MAC CE activation. After the reception of LTM CSC, the UE can only perform CSI measurement on the target cell indicated by LTM CSC, and stop CSI measurements on other candidate cells that are not indicated. The CSI measurement on target cell can be stopped when the P-CSI-RS configuration is released or the SP-CSI-RS is deactivated in the target cell or the CSI report is transmitted on target cell.

For UE only supporting CSI measurement after the reception of LTM CSC, in order to preform CSI measurement on the target cell as early as possible, the UE can immediately start CSI measurement only for the target cell when the PDSCH carrying the LTM CSC is received or the PUCCH with HARQ-ACK corresponding to the PDSCH carrying LTM CSC is transmitted.

In RAN#120bis meeting, it was agreed that for a candidate cell only one CSI report configuration is configured and multiple CSI-RS resources for CMR can be associated with the CSI report configuration. The multiple CMRs can be configured with different QCL assumptions to facilitate CSI acquisition of the candidate cell. After the reception of LTM CSC, only the CSI-RS resource(s) of target cell need to be measured. Considering that the TCI state is indicated by LTM CSC, the CSI measurement in the direction indicated by the TCI state is more meaningful. So after the reception of LTM CSC, only the CSI-RS QCLed with the QCL RS in the indicated TCI state needs to be measured, or the QCL RS of the CSI-RS is updated based on the indicated TCI state.

In addition to periodic CSI-RS and semi-persistent CSI-RS, for UE only supporting CSI measurement after the reception of LTM CSC, AP-CSI-RS associated the CSI report configuration is more reasonable and also can be supported.

Proposal 1:

- For UE supporting CSI measurement before the reception of LTM CSC,

  - before the reception of LTM CSC, the UE shall perform CSI measurements on each candidate cells;

  - after the reception of LTM CSC, the UE only performs CSI measurement on the target cell indicated by LTM CSC,

  - the CSI measurement on the target cell can be stopped when the CSI report is transmitted on target cell.

Proposal 2:

- For UE only supporting CSI measurement after the reception of LTM CSC, when the PDSCH carrying the LTM CSC is received or the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC is transmitted, the UE starts measuring the CSI-RS resource of the target cell.

Proposal 3:

- In addition to P-CSI-RS and SP-CSI-RS, support AP-CSI-RS for UE only supporting CSI measurement after the reception of LTM CSC, where the AP-CSI-RS is associated with the CSI report.

Proposal 4:

- After the reception of LTM CSC, only the CSI-RS QCLed with the QCL RS in the TCI state indicated by LTM CSC needs to be measured, or the QCL RS of the CSI-RS is updated based on the indicated TCI state.

Due to the single CSI report configuration for a candidate cell, for early CSI acquisition and interruption time reduction, LTM CSC MAC CE can be used to indicate the CSI report configuration to target cell.  For example, a 1-bit field can be introduced in LTM CSC MAC CE to trigger a pre-configured CSI reporting. The reported measurement results can be derived based on the CSI-RS resource from the target cell in the CSI-RS resource set under the CSI report configuration.

Proposal 5:

- Support triggering CSI reporting by LTM CSC MAC CE and the reported measurement results are derived based on the CSI-RS resource from the target cell in the resource set under the CSI report configuration.

Regarding the uplink resource carrying CSI report in target cell, if the UL resource scheduled by target cell after cell switching is used to carry CSI report, the reporting latency is increased. For early CSI reporting reception at the target cell, the CSI report can be included on the Msg3 for RACH-based LTM or the first UL channel to the target cell for RACH-less LTM.

Proposal 6:

- The CSI report can be included on the Msg3 for RACH-based LTM or the first UL channel for RACH-less LTM.

The restriction of CSI report configuration is introduced in RAN#120bis to reduce measurement overhead at UE. Furthermore, for UE supporting CSI measurement before the reception of LTM CSC, the power assumption and complexity at UE are increased for multiple candidate cells measurement. So the number of candidate cells to be measured can be limited, e.g. no more than N (e.g. 1 or 2) for CSI-RS measurement before the reception of LTM CSC MAC CE, where N value is up to UE capability.

Proposal 7:

- For further UE complexity reduction and power saving, the number of candidate cells to be measured not exceeds N for CSI-RS measurement before the reception of LTM CSC MAC CE.

  - The supported N value is up to separate UE capability.

In 38.214, the active CSI-RS port/resource are defined to ensure that the number of the active CSI-RS ports or active CSI-RS resources in active BWPs does not exceed the reported number as UE capability. The periodic CSI-RS resource is active in a duration of time defined as follows: starting when the periodic CSI-RS is configured by higher layer signaling, and ending when the periodic CSI-RS configuration is released. The semi-persistent CSI-RS resource is active in a duration of time defined as follows: starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied.

For UE supporting CSI measurement before the reception of LTM CSC, for the candidate cell other than target cell, the duration of time of the active P-CSI-RS port/resource can be defined as: starting when the periodic CSI-RS is configured by higher layer signaling, and ending when the PDSCH carrying the LTM CSC is received or the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC is transmitted; the duration of time of the active SP-CSI-RS port/resource can be defined as: starting from the end of when the activation command is applied, and ending at the end of when the PDSCH carrying the LTM CSC is received or the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC is transmitted. As for the target cell, the ending time the active P-/SP-CSI-RS port/resource can be modified as: ending at the end of the uplink channel resource containing the CSI report.

For UE only supporting CSI measurement after the reception of LTM CSC, the definition mentioned above can be modified for the P-/SP-CSI-RS in LTM CSI acquisition procedure. For example, for CSI measurement after the reception of LTM CSC, the duration of time of the active P-/SP-CSI-RS resources is defined as: starting from the end of the PDSCH carrying the LTM CSC or the end of the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC, and ending at the end of the uplink channel resource containing the CSI report.

Proposal 8:

- For LTM CSI acquisition, the CSI-RS resource is active in a duration of time defined as follows:

  - for UE supporting CSI measurement before the reception of LTM CSC:

    - reuse the starting time as legacy definition for P-/SP-CSI-RS,

    - the ending time for target cell is: at the end of the uplink channel resource containing the CSI report,

    - the ending time for the candidate cell other than target cell is: when the PDSCH carrying the LTM CSC is received or the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC is transmitted,

  - for UE only supporting CSI measurement after the reception of LTM CSC:

    - the starting time is: the end of the PDSCH carrying the LTM CSC or the end of the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC,

    - the ending time is: at the end of the uplink channel resource containing the CSI report.

In legacy spec, the CSI processing criteria and CSI computation time are defined to facilitate the gNB controlled CSI acquisition. For CSI acquisition in LTM procedure, the supported periodic CSI-RS resources need to be measured at UE to calculate PMI/RI/CQI for candidate cells, where the CPU needs also to be occupied. If the CSI processing criteria is not defined, the gNB cannot accurately determine the CPU occupation at UE and cannot accurately schedule the UE to perform CSI processing. It is necessary to discuss whether to define the CSI processing criteria for LTM CSI acquisition.

In legacy spec, for an aperiodic CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. In Rel-19 LTM, the CSI report only can be sent to target cell after the reception of LTM CSC MAC CE. Considering that the LTM CSI report is similar to a gNB controlled aperiodic CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the similar principle can be used for CSI computation time. That is to say, the LTM CSI report occupies CPU(s) from the first symbol after the trigger signaling of the LTM CSI report, e.g. LTM CSC MAC CE, until the last symbol of the uplink channel resource carrying the report in target cell.

Proposal 9:

- Define the CSI computation time for LTM CSI acquisition as follows:

  - the LTM CSI report occupies CPU(s) from the first symbol after the trigger signaling of the report, e.g. LTM CSC MAC CE, until the last symbol of the uplink channel resource carrying the report in target cell.

Conclusion

In this contribution, we provide our proposals on Rel-19 NR mobility enhancements Phase 4:

Proposal 1:

- For UE supporting CSI measurement before the reception of LTM CSC,

  - before the reception of LTM CSC, the UE shall perform CSI measurements on each candidate cells;

  - after the reception of LTM CSC, the UE only performs CSI measurement on the target cell indicated by LTM CSC,

  - the CSI measurement on the target cell can be stopped when the CSI report is transmitted on target cell.

Proposal 2:

- For UE only supporting CSI measurement after the reception of LTM CSC, when the PDSCH carrying the LTM CSC is received or the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC is transmitted, the UE starts measuring the CSI-RS resource of the target cell.

Proposal 3:

- In addition to P-CSI-RS and SP-CSI-RS, support AP-CSI-RS for UE only supporting CSI measurement after the reception of LTM CSC, where the AP-CSI-RS is associated with the CSI report.

Proposal 4:

- After the reception of LTM CSC, only the CSI-RS QCLed with the QCL RS in the TCI state indicated by LTM CSC needs to be measured, or the QCL RS of the CSI-RS is updated based on the indicated TCI state.

Proposal 5:

- Support triggering CSI reporting by LTM CSC MAC CE and the reported measurement results are derived based on the CSI-RS resource from the target cell in the resource set under the CSI report configuration.

Proposal 6:

- The CSI report can be included on the Msg3 for RACH-based LTM or the first UL channel for RACH-less LTM.

Proposal 7:

- For further UE complexity reduction and power saving, the number of candidate cells to be measured not exceeds N for CSI-RS measurement before the reception of LTM CSC MAC CE.

  - The supported N value is up to separate UE capability.

Proposal 8:

- For LTM CSI acquisition, the CSI-RS resource is active in a duration of time defined as follows:

  - for UE supporting CSI measurement before the reception of LTM CSC:

    - reuse the starting time as legacy definition for P-/SP-CSI-RS,

    - the ending time for target cell is: at the end of the uplink channel resource containing the CSI report,

    - the ending time for the candidate cell other than target cell is: when the PDSCH carrying the LTM CSC is received or the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC is transmitted,

  - for UE only supporting CSI measurement after the reception of LTM CSC:

    - the starting time is: the end of the PDSCH carrying the LTM CSC or the end of the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC,

    - the ending time is: at the end of the uplink channel resource containing the CSI report.

Proposal 9:

- Define the CSI computation time for LTM CSI acquisition as follows:

  - the LTM CSI report occupies CPU(s) from the first symbol after the trigger signaling of the report, e.g. LTM CSC MAC CE, until the last symbol of the uplink channel resource carrying the report in target cell.

References

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4.