3GPP TSG RAN WG1 #121                                                      R1-2503527

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.11.1

Source:	Spreadtrum, UNISOC

Title:	Discussion on NR-NTN downlink coverage enhancement

Document for:	Discussion and decision

Introduction

The revised WID on Non-Terrestrial Networks (NTN) for NR Phase 3 is approved in [1]. The objectives of the work item on DL coverage enhancements are the following

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

In this paper, we share our views on NR-NTN downlink coverage enhancement.

Link level enhancements

In RAN1#118, the following agreement on the NTN DL coverage enhancements at both system level and link level had been achieved [2].

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

In this section, we share our views on the NTN DL coverage enhancements at link level.

PDCCH

Type0-PDCCH CSS

Solution for searchSpaceZero configured to Type0 PDCCH CSS

In RAN1#120b, the following agreement on enabling PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero had been achieved [3].

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS.

The bits of aA+6, aA+7 are reserved in PBCH payloads for FR1-NTN. One of them can be used for enabling PDCCH repletion for Type-0 PDCCH CSS of searchSpaceZero. According to the draft CR, aA+7 is used to indicate the enabling of PDCCH repetition. We can directly agree the formulation in draft CR and without any change.

Proposal 1: The reserved 1 bit of aA+7 in PBCH payload can be used for enabling PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero, which aligns with the draft CR.

In RAN1#120b, the following agreement on PDCCH repetition scheme for Type0 PDCCH CSS of searchSpaceZero had been achieved [3].

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2.

Considering the limited number of active satellite beams, when M=1 and M=1/2, intra beam repetition maybe not feasible, due to two PDCCH repetitions associated with a SSB overlap with the PDCCH repetitions with next SSB. But this issue can be resolved by gNB. If there are a sufficient number of beams serving the cell, the value of M can be configured as any one of 1/2, 1 or 2. If the number of beams serving the cell is limited, the value of M can be configured to 2. Therefore, repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) for all M values should be supported.

Figure 2: Locations of Type-0 PDCCH CSS when M equals 1

Figure 3: Locations of Type-0 PDCCH CSS when M equals 1/2

Proposal 2: Repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) for all M values should be supported.

Regarding BD counting, a UE can indicate by numBD-twoPDCCH-r17 a capability for counting PDCCH candidates msi,nCI(L) and msj,nCI(L) either as 2 PDCCH candidates or as 3 PDCCH candidates. We propose 2 or 3 can also applicable for Rel-19 PDCCH repetition. However, there is no capability report for Type-0 PDCCH CSS in RRC_IDLE/INACTIVE mode. So it is up to gNB guarantee the BD limits and non-CCE limits with PDCCH repetitions are still satisfied. Since there is clear requirements that no dropping for CSS. When 3 BDs are counted for two linked candidates, the third BD is counted in the later slot PDCCH repetition.

Proposal 3: For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition of Type0-PDCCH CSS, 2 or 3 BDs for the two PDCCH candidates are supported.

- When 3 BDs are counted for two linked candidates, the third BD is counted in the later slot PDCCH repetition.

Other non-zero value search space configured to searchSpaceSIB1

It was agreed that only inter-slot repetition for Type0 PDCCH CSS is supported. The case of other non-zero value search space configured to searchSpaceSIB1 was discussed in last meeting. Most companies support that if searchSpaceSIB1 is set to a search space other than zero, meaning it does not overlap with the initial bandwidth part (BWP), the Type-0 CSS PDCCH repetition is disabled. We also support that when non-zero value search space is configured to Type0-PDCCH CSS by UE dedicated signaling, no PDCCH repetition applied.

Proposal 4: If searchSpaceSIB1 is set to a search space other than zero, meaning it does not overlap with the initial bandwidth part (BWP), the Type-0 CSS PDCCH repetition is disabled.

Type0A/1/2-PDCCH CSS

In RAN1#120b, the following working assumption had been achieved [3].

Working assumption For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported. RAN1 to down select between option 1 and option 2: Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2) - Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17) - FFS: Blind decoding limit Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X - FFS: Blind decoding limit - FFS: details configuration and signalling

Firstly, the above working assumption can be confirmed.

Proposal 5: The above working assumption can be confirmed

In our views, search space linkage based PDCCH repetition in R17 can be as a start point for PDCCH CSS (except Type-3) link level enhancements except searchSpaceZero. Search space linkage based PDCCH repetition can be reused for PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero (i.e., Option 1). In addition, Option 2 can be achieved by Option 1. There is no need to introducing a new scheme of PDCCH repetition configuration.

Proposal 6: Search space linkage based PDCCH repetition in R17 can be reused for PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero (i.e., Option 1)

Regarding BD counting, same as Rel-17 PDCCH repetition, 2 PDCCH candidates or 3 PDCCH candidates are supported. The requirement of no dropping for CSS is still applicable even with PDCCH repetitions. When 3 BDs are counted for two linked candidates, the third BD is counted in the later span for inter-span PDCCH repetition.

Proposal 7: For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition of Type0A/1/2-PDCCH CSS, 2 or 3 BDs for the two PDCCH candidates are supported.

- When 3 BDs are counted for two linked candidates, the third BD is counted in the later span for inter-span PDCCH repetition, which is same as Rel-17.

PDSCH with Msg4

In previous RAN1 meeting, the following agreement on PDSCH with Msg4 link level enhancement had been achieved.

Agreement For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots: - The same resource allocation is assumed for all repetitions - The supported repetition factors are: 2 and 4 - The network configures a single value between 2 and 4 at a given time - The RV cycling is used for each repetition - If N=4: Starting RV; RV applicable to the nth repetition/transmission. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3. Agreement For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: - Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 - FFS: indication details. - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) - Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. - Option 3: The enabling/disabling is indicated by SIB1 configuration - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) FFS: Whether UE reports its capability

For enabling/disabling Msg4 PDSCH repetition, the most straightforward and flexible way is to indicate the Msg4 PDSCH repetition factor by a field in DCI scheduling Msg4, i.e., Option 1. In addition, bits of the existing field (e.g., MCS field) can be repurposed for enabling/disabling Msg4 PDSCH repetition. The legacy MSG3 PUSCH repetition number indicated by MCS field can be as a start point. When DL coverage is bottleneck, lower MCS for PDSCH may be used, and some bits of MCS field can be repurposed for enabling/disabling Msg4 PDSCH repetition. It should notice MSG4 can be UE specific, so it is worth to provide flexibility to enable or disable MSG4 PDSCH repetition dynamically.

For UE capability of support PDSCH repetition, we think UE can report its request/capability of Msg4 PDSCH repetition by MAC-CE in Msg3 PUSCH. PRACH based report lead to more PRACH fragment. But MAC-CE report is more flexible.

Proposal 8: For PDSCH with Msg4 Link level enhancement, the Msg4 PDSCH repetition can be indicated by DCI scheduling Msg4, Option 1.

Proposal 9: For UE capability of support PDSCH repetition, UE can report its request/capability of Msg4 PDSCH repetition via MAC-CE in Msg3 PUSCH.

PDSCH with SIB1

In RAN1#120b, the following agreement on PDSCH with SIB1 link level enhancement had been achieved [3].

Agreement When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination. Agreement For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

Considering backward compatibility, SIB1 PDSCH repetition should always bundle with SIB1 PDCCH repetition in Type-0 PDCCH CSS. It means if PDCCH repetition is enabled for Type-0 PDCCH CSS, SIB1 PDSCH is also enabled. No separate configurations for SIB1 PDCCH and PDSCH. In our views, the case of type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed should not be supported. The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition (i.e., Option 3).

Proposal 10: The case of type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed should not be supported.

Proposal 11: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition (i.e., Option 3).

Conclusion

In this contribution, we provided views on Redcap positioning. In summary, we have following observations and proposals:

Proposal 1: The reserved 1 bit of aA+7 in PBCH payload can be used for enabling PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero, which aligns with the draft CR.

Proposal 2: Repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) for all M values should be supported.

Proposal 3: For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition of Type0-PDCCH CSS, 2 or 3 BDs for the two PDCCH candidates are supported.

- When 3 BDs are counted for two linked candidates, the third BD is counted in the later slot PDCCH repetition.

Proposal 4: If searchSpaceSIB1 is set to a search space other than zero, meaning it does not overlap with the initial bandwidth part (BWP), the Type-0 CSS PDCCH repetition is disabled.

Proposal 5: The following working assumption can be confirmed

Working assumption For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported. RAN1 to down select between option 1 and option 2: Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2) - Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17) - FFS: Blind decoding limit Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X - FFS: Blind decoding limit FFS: details configuration and signalling

Proposal 6: Search space linkage based PDCCH repetition in R17 can be reused for PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero (i.e., Option 1)

Proposal 7: For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition of Type0A/1/2-PDCCH CSS, 2 or 3 BDs for the two PDCCH candidates are supported.

- When 3 BDs are counted for two linked candidates, the third BD is counted in the later span for inter-span PDCCH repetition, which is same as Rel-17.

Proposal 8: For PDSCH with Msg4 Link level enhancement, the Msg4 PDSCH repetition can be indicated by DCI scheduling Msg4, Option 1.

Proposal 9: For UE capability of support PDSCH repetition, UE can report its request/capability of Msg4 PDSCH repetition via MAC-CE in Msg3 PUSCH.

Proposal 10: The case of type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed should not be supported.

Proposal 11: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition (i.e., Option 3).

Reference

- RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Thales, CATT.

- 3GPP RAN1#118 Chairman’s Notes, August 19th – 23rd, 2024.

- 3GPP RAN1#120b Chairman’s Notes, April 07th – 11th, 2025.