3GPP TSG RAN WG1 #121                                                                 R1- 2503529

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.11.3

Source:	Spreadtrum, UNISOC, SGITG

Title:	Discussion on NR-NTN uplink capacity and throughput enhancement

Document for:	Discussion and decision

Introduction

The WID on Non-Terrestrial Networks (NTN) for NR Phase 3 is approved in [1].

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this paper, we share our views on NR-NTN uplink capacity and throughput enhancement.

Potential specification aspects on OCC technique

In RAN1#119, the following agreement on OCC schemes for PUSCH had been achieved [2].

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

In RAN1#117, the following agreement on the potential specification aspects on OCC techniques had been achieved [3].

Agreement RAN1 to at least further study the potential specification aspects on OCC techniques: - TBS calculation / Rate matching - UCI multiplexing - RV cycling across repetitions - Frequency hopping, e.g. intra /inter slot - OCC indication/configuration - Power control - FFS others aspects

In this section, we share our views on the potential specification aspects on inter-slot time-domain OCC with PUSCH repetition Type A.

Maintain phase continuity and power consistency

It was agreed UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH, and FFS which conditions. Phase continuity/events applied for DMRS bundling can be starting point.

Agreement RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling

Among all the events defined for DMRS bundling, only other uplink transmissions are scheduled between the two consecutive PUSCH transmissions matters phase continuity and power consistency. To simplify the discussion, we can go with other uplink transmission between any two PUSCH repetitions within an OCC period can be avoided by gNB implementation.

Second, there are additional events for redcap HD UE, including DL cancel UL or DL reception between two consecutive PUSCH in an OCC groups. We can easily go with redcap HD UE does not support PUSCH repetitions with inter-slot OCC to save time. Due to too many collision cases, it is very hard to finish them in one left RAN1 meeting.

Proposal 1: For maintaining phase continuity and power consistency for the duration of one OCC group with PUSCH, other uplink transmission between any two PUSCH repetitions within an OCC period can be avoided by gNB implementation.

Proposal 2: Redcap HD UE does not support PUSCH repetitions with inter-slot OCC in Rel-19 NTN.

UCI multiplexing

In RAN1#120b, the following agreement on UCI multiplexing had been achieved [4].

Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group. Note: how to capture the above agreement is up to the spec editor. Agreement If PUCCH without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group with a same priority index for PUCCH/PUSCH, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - FFS: PUSCH repetition with A-CSI reports - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain. - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group. - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH Note 1: If the UCI on the PUCCH is dropped according to legacy rules and [updated] timeline conditions for UCI dropping are satisfied, there is no [additional] spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified. Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

Timeline

The first PUSCH repetition of an OCC group agreed to determine the timeline conditions for UCI multiplexing or prioritization for PUSCH/PUCCH. One open issue is whether PUCCH dropping should be considered in the updated timeline conditions.

First, in legacy rule, a dropped PUCCH is taken into account even it is dropped. As shown in the following spec. Although only multiplexes is mentioned, dropping also covered by the timeline. For example, “UE does not transmit…” exits in many places. Figure 1 gives an example, SR on PUCCH format 1, 1bit HARQ-ACK on PUCCH format 0, and DCI scheduled PUSCH are overlapping. Overlapped PUCCHs result SR resource to carrier SR and HARQ-ACK. When handling PUCCH and PUSCH, only HARQ-ACK is piggyback on PUSCH, SR is dropped finally. Obviously, the dropped PUCCH resource still used to determine the first symbol, due to it may affect the resultant PUCCH among the overlapped PUCCHs. Similarly, it is same for PUSCH repetitions with inter-slot OCC, dropped PUCCH should be involved.

Figure 1: PUCCH and PUSCH overlapping

If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot and, when applicable as described in clauses 9.2.5.1, 9.2.5.2, 9.2.5.3 and 18, the UE is configured to multiplex different UCI types or UCI of different priority indexes in one PUCCH, and at least one of the multiple overlapping PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE multiplexes all corresponding UCI types or UCI of different priority indexes if the following conditions are met. If one of the PUCCH transmissions or PUSCH transmissions is in response to a DCI format detection by the UE, the UE expects that the first symbol S0 of the earliest PUCCH or PUSCH, among a group overlapping PUCCHs and PUSCHs in the slot, satisfies the following timeline conditions …

Proposal 3: For PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, it can confirm dropped PUCCH is involved in the timeline determination, as same as legacy.

CSI reference resource

Another related timeline issue is CSI processing timeline referring as CSI reference resource.

38.214 5.2.2.5 CSI reference resource definition - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6, TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, … - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot.

For resolving the overlapping PUSCH repetitions with inter-slot OCC and P/SP CSI on PUCCH with/without repetitions, the legacy definition of CSI reference resource is impacted, following alternatives can be considered:

Alt1: For CSI reference resource determination, the first PUSCH repetition of an OCC group is used instead of uplink slot of CSI report. As shown in the following figure.

Alt2: P/SP-CSI on PUCCH does not expect overlapping over PUSCH in other than the first PUSCH repetition in an OCC group.

Figure 2: CSI processing timeline

Proposal 4: For resolving the overlapping PUSCH repetitions with inter-slot OCC and P/SP CSI on PUCCH with/without repetitions, further discuss impacts to CSI reference resource.

- Alt1: For CSI reference resource determination, the first PUSCH repetition of an OCC group is used instead of uplink slot of CSI report.

- Alt2: P/SP-CSI on PUCCH does not expect overlapping over PUSCH in other than the first PUSCH repetition in an OCC group.

PUCCH without repetition with same priority

For inter-slot time-domain OCC with PUSCH repetition type A, there are three open issues for UCI multiplexing.

First, some companies wanted to consider PUCCH overlapping with PUSCH repetition in both time and frequency domain. The intention is to avoid the survived PUCCH interference PUSCH of other paired UEs. However, PUCCH resource can be located at lower or higher part of BWP, to reduce the segmentation of UL resources. Then there is less chance PUCCH is overlapping with PUSCH in both time and frequency domain. Moreover, it is unreasonable to scheduling multiple UE to transmit PUSCH within the region of PUCCH resources. Thus, it is good enough up to gNB implementation to avoid PUCCH is overlap with PUSCH repetition in both time and frequency domain. No need to further study this issue.

Proposal 5: it is good enough up to gNB implemantion to avoid PUCCH is overlap with PUSCH repetition in both time and frequency domain.

Second, for multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group, there was an FFS. In draft CR, the following sentence has cover the intention of FFS if PUCCH and PUSCH are with same priority. Therefore, FFS can be closed for same priority case and same SCS case. However, there would be issue for different priority or different SCS in TN, we provide our view in the following section.

- does not expect to transmit more than one PUCCH without repetitions providing HARQ-ACK information or CSI reports in different slots that would overlap with the PUSCH transmission

Proposal 6: For PUCCH without repetition, FFS point of whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH is aligned with draft CR.

PUCCH with repetition

In the current specification, when a PUSCH w/o repetition overlapping with a PUCCH with repetitions in at least one slot, PUSCH is dropped in the slot.

When PUCCH with repetitions overlapping with PUSCH repetitions, PUSCH repetitions should be dropped according to legacy rule. One example is if CSI/HARQ-ACK on PUCCH without repetition overlapping with PUSCH repetitions, and SR/CSI/HARQ-ACK PUCCH repetitions overlapping with PUSCH on another slots, it needs further discussion whether or not it allows. The result is HARQ-ACK is multiplexed on PUSCH, and then PUCCH repetition require to drop PUSCH, which make a high complexity for UCI multiplexing.

Figure 3: PUCCH with repetitions overlapping with PUSCH repetitions

In order to reduce the complexity of UCI multiplexing, for PUCCH with repetition, we suggest:

Proposal 7: UE does not expect to transmit one PUCCH without repetitions providing HARQ-ACK information or CSI reports in a slot and another PUCCH with repetitions in a later slot that would overlap with the PUSCH transmission.

The other parts in the previous agreement of PUCCH without repetition and same priority can apply to PUCCH with repetitions. We support to confirm the working assumption with the update.

Proposal 8: Confirm the following working assumption together with the proposal 7 to replace the third bullet.

Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

Different priority

Different priority case is more complex. One case given needs more discussion.

LP PUSCH, LP HARQ-ACK/CSI PUCCH in a first slot and HP SR PUCCH in a later slot. According to the third bullet, SR is excluded. However, if SR PUCCH with HP, it should be involved.

Figure 5: Case 1

In order to reduce the complexity of UCI multiplexing, for PUCCH and PUSCH with different priority, we suggest:

Proposal 9: UE does not expect to transmit more than one PUCCH without repetitions providing HARQ-ACK information or CSI reports or SR in different slots that would overlap with the PUSCH transmission

A-CSI report on PUSCH

Another issue is A-CSI triggered by DCI on PUSCH. In the current spec, the A-CSI is only multiplexed on the first repetition. A-CSI can be multiplexed on all repetitions of the first PUSCH OCC group, instead of the first one.

Proposal 10: When a DCI triggers A-CSI report on PUSCH, the A-CSI multiplex on all repetitions of the first PUSCH OCC group.

RV

RV cycling per OCC group was agreed for DG-PUSCH and CG-PUSCH.

Agreement For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0]. - Note: no new RRC parameter is needed for the above - FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case

When OCC group dropping, it has no impact on RV determination. RV cycling also include the dropped PUSCH OCC groups. Because OCC group dropping may due to dynamic DCI, it would be very dynamic to determine the RV and complex.

Proposal 11: When OCC group dropping, it has no impact on RV determination.

Frequency hopping

For inter-slot time-domain OCC with PUSCH repetition Type A, since the phase continuity may not be guaranteed after frequency hopping, it cannot support intra-slot frequency hopping or inter-slot frequency hopping, only inter-slot frequency hopping with enhancement is possible. Such the inter-slot FH with DMRS bundling. The frequency hopping only hops across different intervals. There is no hop among slots in one interval, the following starting RB of two hops are defined in 38.214. It can also used as a baseline.

For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group. Furthermore, instead of additional frequency hopping interval, OCC length can be directly applied.

In case of inter-slot frequency hopping and when pusch-DMRS-Bundling is enabled, and when a PUSCH is not scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: RBstartnsμ=RBstartRBstart+RBoffsetmodNBWPsizensμNFH mod 2=0nsμNFH mod 2=1 where nsμ is the current slot number within a system radio frame, NFH is the value of the higher layer parameter pusch-FrequencyHopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Proposal 12: For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group.

- The RB start index in inter-slot frequency hopping with DMRS bundling can be used as baseline.

OCC indication/configuration

In RAN1#120b, the following agreements on OCC indication and signalling had been achieved.

Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2. Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

Considering the flexibility of network configuration, the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2 can be flexibly indicated by the DCI. Therefore, we prefer that the candidates values of OCC length are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2 (i.e., Option 3).

Proposal 13: The candidates values of OCC length are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2 (i.e., Option 3).

In last meeting, it was agreed that for the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI and the DCI filed design is FFS. Considering DMRS antenna ports are designed to be orthogonal among different UEs, the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2 can be associated with DMRS antenna port and implicitly indicated during scheduling to reduce signaling overhead.

Proposal 14: The OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2 can be associated with DMRS antenna port and implicitly indicated during scheduling.

Conclusion

In this contribution, we provided views on Redcap positioning. In summary, we have following proposals:

Proposal 1: For maintaining phase continuity and power consistency for the duration of one OCC group with PUSCH, other uplink transmission between any two PUSCH repetitions within an OCC period can be avoided by gNB implementation.

Proposal 2: Redcap HD UE does not support PUSCH repetitions with inter-slot OCC in Rel-19 NTN.

Proposal 3: For PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, it can confirm dropped PUCCH is involved in the timeline determination, as same as legacy.

Proposal 4: For resolving the overlapping PUSCH repetitions with inter-slot OCC and P/SP CSI on PUCCH with/without repetitions, further discuss impacts to CSI reference resource.

- Alt1: For CSI reference resource determination, the first PUSCH repetition of an OCC group is used instead of uplink slot of CSI report.

- Alt2: P/SP-CSI on PUCCH does not expect overlapping over PUSCH in other than the first PUSCH repetition in an OCC group.

Proposal 5: it is good enough up to gNB implemantion to avoid PUCCH is overlap with PUSCH repetition in both time and frequency domain.

Proposal 6: For PUCCH without repetition, FFS point of whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH is aligned with draft CR.

Proposal 7: UE does not expect to transmit one PUCCH without repetitions providing HARQ-ACK information or CSI reports in a slot and another PUCCH with repetitions in a later slot that would overlap with the PUSCH transmission.

Proposal 8: Confirm the following working assumption together with the proposal 7 to replace the third bullet.

Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

Proposal 9: UE does not expect to transmit more than one PUCCH without repetitions providing HARQ-ACK information or CSI reports or SR in different slots that would overlap with the PUSCH transmission

Proposal 10: When a DCI triggers A-CSI report on PUSCH, the A-CSI multiplex on all repetitions of the first PUSCH OCC group.

Proposal 11: When OCC group dropping, it has no impact on RV determination.

Proposal 12: For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group.

- The RB start index in inter-slot frequency hopping with DMRS bundling can be used as baseline.

Proposal 13: The candidates values of OCC length are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2 (i.e., Option 3).

Proposal 14: The OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2 can be associated with DMRS antenna port and implicitly indicated during scheduling.

Reference

- RP-243300  Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

- 3GPP RAN1#119 Chairman’s Notes, November 18 – 22, 2024.

- 3GPP RAN1#117 Chairman’s Notes, May 20th – 24th, 2024.

- 3GPP RAN1#120b Chairman’s Notes, April 07th – 11th, 2025.