3GPP TSG RAN WG1 #121			R1-2503530

Saint Julian, Malta, May 19th – 23th, 2025

Agenda Item:	9.11.4

Source:	Spreadtrum, UNISOC

Title: 	Discussion on IoT-NTN uplink capacity/throughput enhancement

Document for:	Discussion and decision

- Introduction

In RAN#107 meeting, the WID of ‘Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3’ was revised [1]. The corresponding objective is copied below.

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1[RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

We will share our view on UL capacity enhancement of IoT NTN.

- Single-tone

  - 3.75kHz TDM DMRS pattern

In RAN1#120bis, there are two candidate options for 3.75kHz DMRS pattern [2].

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots

For option 1, as shown in figure 1, since it changes legacy DMRS sequence, it may have interference for neighbour cell.

Figure 1 TDM DMRS pattern option 1

For option 2, follow original DMRS sequence and drop DMRS symbols of blanked slots. Option 2 doesn’t have interference for neighbour cell.

Figure 2 TDM DMRS pattern option 2

- Support option 2 for TDM DMRS pattern.

Option 2: Dropping of samples of the original DMRS sequence in blanked slots

  - Indication of OCC sequence and dynamic activation

In RAN1#120bis, it was agreed using DCI format N0 for indication of OCC sequence index and activation / deactivation [2].

Agreement The OCC sequence index is signalled using DCI format N0. Agreement For indicating OCC sequence index and activation / deactivation, the following fields may be repurposed or constrained to be not present in the DCI: - Modulation and coding scheme - Repetition number - Redundancy version - Number of scheduled TB for Unicast - Subcarrier indication - Resource reservation

For connected mode NPUSCH, the maximum OCC length is 2 which means we need 1 bit to indicate OCC sequence index and 1 bit for dynamic activation/ deactivation.

For subcarrier indication field, is reserved for 3.75kHz SCS. Although there are some reserved codepoints, there is not 1 bit reserved can be used to indicate OCC-related feature.

TS 36.213 For NPUSCH transmission with subcarrier spacing, where is the subcarrier indication field and is reserved, or nsc is configured by higher layers parameter npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources.

For redundancy version filed, there is only one bit to indicate RV, if using it to indicate OCC-related feature, it can’t indicate RV. Similar as redundancy version field, there is only one bit for Number of scheduled TB for Unicast field and Resource reservation field respectively, if we use those bits, legacy function can’t work.

TS 36.212 - Subcarrier indication – 6 bits as defined in clause 16.5.1.1 of [3] - Modulation and coding scheme – 4 bits as defined in clause 16.5.1.2 of [3]. This field is not present if format N0 CRC is scrambled by PUR-RNTI. If npusch-16QAM-Config is configured and the value is '1111', it functions as 16QAM indicator. - Redundancy version – 1 bit as defined in clause 16.5.1.2 of [3] - Repetition number – 3 bits as defined in clause 16.5.1.1 of [3]. If 16QAM is indicated, it functions as Modulation and coding scheme for 16QAM as defined in 16.5.1.2 of [3]. - Number of scheduled TB for Unicast – 1 bit, where value 0 indicates a single TB is scheduled and value 1 indicates multiple TB are scheduled. This field is only present if higher layer parameter npusch-MultiTB-Config is enabled and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3]. The field is set to 0 if the CRC of the DCI is scrambled by SPS C-RNTI. - Resource reservation – 1 bit as defined in clause 16.5 of [3]. This field is only present if higher layer parameter resourceReservationConfigUL is configured and the DCI is mapped onto the UE-specific search space given by C-RNTI as defined in [3].

Considering the NTN scenario, the MCS order is not too high and the number of candidate values of PUSCH repetition can be limited. Therefore, we think repurposing some bits from MCS field and repetition number field. For example, repurposing one bit from MCS field to indicate OCC sequence index and one bit from repetition number field to indicate activation/deactivation and vice versa.

- Repurposing one bit from MCS field to indicate OCC sequence index and one bit from repetition number field to indicate activation/deactivation and vice versa.

  - Gaps in current spec

In RAN1#118, an agreement about UL transmission gap is achieved as follow [3].

Agreement RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity - UL gaps for synchronization (from Rel-13) - Gaps around NPRACH occasions - UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) - TDM DMRS that are muted - Guard periods for 3.75kHz UL transmissions

(1) Case 1: UL gaps for synchronization (from Rel-13)

For the NPUSCH, if the transmission duration surpasses 256ms, a 40ms gap is introduced after each 256ms and the remaining data will be sent after the gap. UE can correct UL synchronization in the gap.

For NPRACH, if the repetitions surpass 64 for frame structure type 1, a 40ms gap is introduced after 64repetitions and the remaining NPRACH will be sent after the gap.

(2) Case 2: Gaps around NPRACH occasion

If NPUSCH transmission is collision with NPRACH occasion, NPUSCH transmission will postpone after NPRACH occasion.

(3) Case 3: UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

In IoT NTN, segment transmission is introduced for NPUSCH and NPRACH due to larger repetitions. The length of segment and gap are configured by network. UE can adjust TA in gap between two adjacent segments.

When using OCC, one or more symbols are dropped as part of a segmented transmission in a gap which would affect orthogonality between UEs. For example, UE0 drops one symbol but UE1 drops one slot, which may cause eNB can’t demodulate data.

The issue is similar for above 3 cases. When using OCC, the orthogonality of OCC cross the gap may be broken. In RAN1#120bis, dynamic activation and deactivation is introduced so we think network can deactivation OCC to avoid collision with above gap.

- For case 1, 2 and 3, up to network implement to avoid these cases.

Case 1: UL gaps for synchronization (from Rel-13)

Case 2: Gaps around NPRACH occasion

Case 3: UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

(4) Case 4:TDM DMRS that are muted

When using TDM DMRS, there is a symbol-length gap in some slots which may not affect orthogonality around the gap because of the length is small. Density of DMRS decreases for TMD DMRS which may affect the performance of channel estimation.

(5) Case 5: Guard periods for 3.75kHz UL transmissions

For 3.75 kHz, one slot contains 7 OFDM symbols and one guard period and the length of symbol and guard period is shown in Figure 3. The length of guard period is much smaller than symbol. The orthogonality of OCC code isn’t a problem for this issue.

Figure 3: 3.75 kHz slot structure

- Case 4 and 5 have no impact on OCC.

Case 4: TDM DMRS that are muted

Case 5: Guard periods for 3.75kHz UL transmissions

- Multi-tone

  - OCC scheme

Since we almost have no time to discuss multi-tone OCC, we can reuse OCC design for single-tone as much as possible. For multi-tone, repetition is based on Nslot slots. Similar as slot-level OCC for 15kHz single-tone, the elements of an OCC sequence can be multiplied on every Nslot slots as shown in Figure 4 (Nslot = 2). For OCC length, we can follow single-tone, e.g., support OCC 2 for multi-tone.

Figure 4: Slot-level OCC of 15kHz multi-tone without resource mapping change (Nslot = 2)

- Repetition is based on Nslot slots for 15kHz multi-tone and Nslot=2.

- Support Nslot slot-level OCC 2 for multi-tone and Nslot=2.

  - DMRS scheme

For multi-tone, the reference signal sequences are defined in TS 36.211.

The reference signal sequences for is defined by a cyclic shift of a base sequence according to , where is given by Table 10.1.4.1.2-1 for , Table 10.1.4.1.2-2 for and Table 5.5.1.2-1 for . If group hopping is not enabled, the base sequence index is given by higher layer parameters threeTone-BaseSequence, sixTone-BaseSequence, and twelveTone-BaseSequence for , , and , respectively. If not signalled by higher layers, the base sequence is given by If group hopping is enabled, the base sequence index is given by clause 10.1.4.1.3. The cyclic shift for and is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 10.1.4.1.2-3. For , if npusch-CyclicShift in PUR-Config-NB is configured for NPUSCH (re)transmission corresponding to preconfigured uplink resource it provides the value of ncs and the cyclic shift α in a slot is given as α=2πncs/12, otherwise . Table 10.1.4.1.2-1: Definition of for u; ϕ0, ϕ1, ϕ2. 0; 1; -3; -3. 1; 1; -3; -1. 2; 1; -3; 3. 3; 1; -1; -1. 4; 1; -1; 1. 5; 1; -1; 3. 6; 1; 1; -3. 7; 1; 1; -1. 8; 1; 1; 3. 9; 1; 3; -1. 10; 1; 3; 1. 11; 1; 3; 3. Table 10.1.4.1.2-2: Definition of for u; ϕ0, …, ϕ5. 0; 1; 1; 1; 1; 3; -3. 1; 1; 1; 3; 1; -3; 3. 2; 1; -1; -1; -1; 1; -3. 3; 1; -1; 3; -3; -1; -1. 4; 1; 3; 1; -1; -1; 3. 5; 1; -3; -3; 1; 3; 1. 6; -1; -1; 1; -3; -3; -1. 7; -1; -1; -1; 3; -3; -1. 8; 3; -1; 1; -3; -3; 3. 9; 3; -1; 3; -3; -1; 1. 10; 3; -3; 3; -1; 3; 3. 11; -3; 1; 3; 1; -3; -1. 12; -3; 1; -3; 3; -3; -1. 13; -3; 3; -3; 1; 1; -3. Table 10.1.4.1.2-3: Definition of threeTone-CyclicShift; sixTone-CyclicShift. 0; 0. 1; 1. 2; 2. 3.

For one base sequence, there are different cyclic shift . The reference signals generated by different cyclic shifts are orthogonal. Current spec can be reused to distinguish different UE’s DMRS. Another solution is to reuse CDM DMRS pattern similar as 15kHz single-tone.

- For multi-tone NPUSCHs multiplexed with OCC, DMRS sequences with different cyclic shifts or CDM DMRS (following 15kHz single-tone DMRS pattern) can be considered.

  - Indication of OCC sequence and dynamic activation

For indication of OCC sequence and activation/deactivation, reuse single-tone OCC design.

- Reuse single-tone OCC design for indication of OCC sequence and activation/deactivation for multi-tone.

- Conclusion

In this contribution, we provided our views on the details of UL capacity enhancement. The following observations and proposals are made:

- Repetition is based on Nslot slots for 15kHz multi-tone and Nslot=2.

- Support option 2 for TDM DMRS pattern.

Option 2: Dropping of samples of the original DMRS sequence in blanked slots

- Repurposing one bit from MCS field and repetition number field to indicate OCC sequence index and activation/deactivation.

- For case 1, 2 and 3, up to network implement to avoid these cases.

Case 1: UL gaps for synchronization (from Rel-13)

Case 2: Gaps around NPRACH occasion

Case 3: UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

- Case 4 and 5 have no impact on OCC.

Case 4: TDM DMRS that are muted

Case 5: Guard periods for 3.75kHz UL transmissions

- Support Nslot slot-level OCC for multi-tone and Nslot=2.

- For multi-tone NPUSCHs multiplexed with OCC, DMRS sequences with different cyclic shifts or CDM DMRS (following 15kHz single-tone DMRS pattern) can be considered.

- Reuse single-tone OCC design for indication of OCC sequence and activation/deactivation for multi-tone.

- Reference

- RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Incheon, Korea, March 12-14, 2025.

- 3GPP, Chairman’s Notes, RAN1 #120bis.

- 3GPP, Chairman’s Notes, RAN1 #118.