3GPP TSG RAN WG1 #121   		                           R1-2503541

St Julian's, Malta, May 19th – 23th, 2025

Agenda item:		5

Source:			Samsung

Title:			Discussion on RAN4 reply LS for LP-WUS UE RF

Document for:		Discussion and decision

- Introduction

In [1], RAN4 provided the information for deriving REFSENS and asked to consider it for designing the LP-WUS/LP-SS for sufficient coverage. In addition, RAN4 asks the further details on signal configuration of LP-WUS and LP-SS as the follow:

To RAN1: ACTION: RAN4 respectfully asks RAN1 to consider the above information while designing the LP-WUS/LP-SS for sufficient coverage and provide feedback on the information which could be used to derive SNR for LP-WUR Rx requirements.

In this contribution, it discusses on how to reply LS.

- Discussion

According in [1], to facilitate deriving SNR with sufficient coverage, RAN1 has been requested to provide responses to the following information from RAN4:

- Chip rate and length of information bits (the number of ON/OFF symbols for the single LP-WUS transmission) of OOK sequence - Coding, if any - Whether CRC will be configured? If CRC is configured, what length of CRC is used? - Any other information relevant to detection sensitivity.

For the response to RAN4, the information can be classified for three parts: OOK-based LP-WUS, OFDM-based LP-WUS (overlaid OFDM sequences), and OOK-based LP-SS.

For OOK-based LP-WUS, chip rate can be provided based on agreed M value corresponding to each SCSs. However, M values for some SCS haven’t agreed yet. We can just provide the equation to calculate the chip rate and the set of M and SCS, which is the same as 2μ×15kHz where μ is numerology in NR. To provide the information about coding, further discussion is necessary. Including the coding scheme corresponding to each information bit range, the candidates of length of LP-WUS can be provided after the discussion on the MO length or coded block length after rate matching in units of OOK symbols or OFDM symbols. Since it was agreed to support codepoint-based LP-WUS for all RRC states, CRC will not be configured. Based on the above discussion, Table 1 is suggested for the response related to OOK-based LP-WUS. Note that FFS can be further decided in RAN1#121.

Table 1. Draft LS response for OOK-based LP-WUS part

OOK-based LP-WUS - Chip rate=M×14×2μ - where M is the number of OOK chip within the OFDM symbol and μ is numerology related to the SCS. - The following (M, μ) are supported for LP-WUS: - (1, 0), (2, 0), (4, 0) for μ=0 (15kHz SCS) - (1, 1), (2, 1), [(4, 1) as Working assumption] for μ=1 (30kHz SCS) - FFS for 60kHz and 120kHz SCSs - Length of information bits (before applying Reed Muller coding) - maximum 5 bits for both RRC IDLE/INACTIVE and RRC CONNECTED state - Coding design for LP-WUS - RM coding with rate matching is applied for more than 2 bits information for LP-WUS - FFS: candidates of the length of LP-WUS (number of ON/OFF symbols or number of OFDM symbols) - [Working assumption] For 1 and 2 information bits for LP-WUS, the coding is done as descripted in the first row in the table in section 5.3.3.1 and 5.3.3.2 of 38.212 - FFS: candidates of the length of LP-WUS (number of ON/OFF symbols or number of OFDM symbols) - CRC is not configured

For OFDM-based LP-WUS, the overlaid OFDM sequence length and maximum number of overlaid OFDM sequences per ON symbol can be provided for the further information. The overlaid OFDM sequence length is that the OFDM sequence in time domain, which is mapped before the DFT/LS processing at the gNB side. Therefore, it can be also explained based on M value and the number of REs for LP-WUS. If necessary, how to generate the overlaid sequence can be also provided referring to the agreement. The maximum number of overlaid OFDM sequence also depends on M value. After the further discussion, other aspects also can be provided further if it is beneficial for RAN4.

Table 2. Draft LS response for OFDM-based LP-WUS part

OFDM-based LP-WUS (overlaid OFDM sequence) - The length of sequences=12XM - where M is the number of OOK chip within the OFDM symbol and X is the number of RBs . - X=11 for 15/30/60/120kHz SCSs - The following (M, μ) are supported for LP-WUS: - (1, 0), (2, 0), (4, 0) for μ=0 (15kHz SCS) - (1, 1), (2, 1), [(4, 1) as Working assumption] for μ=1 (30kHz SCS) - FFS for 60kHz and 120kHz SCSs - The overlaid sequences are the sequence in the time domain before DFT/LS processing at the gNB - The maximum number of overlaid OFDM sequences - 4, 8, 16 overlaid OFDM sequences for M=4, 2, 1, respectively. - The information bit is carried by the overlaid OFDM sequences without coding scheme - FFS: whether to support repetition of information bit carried by overlaid OFDM sequences

For OOK-based LP-SS, because it doesn’t have any coding scheme, providing the chip rate, the supported sequence set and the periodicity seems enough. Except for the periodicity of LP-SS for the case when M value of LP-WUS is 4, the remaining aspects have been agreed.

Table 3. Draft LS response for OOK-based LP-SS part

OOK-based LP-SS - Chip rate=M×14×2μ - where M is the number of OOK chip within the OFDM symbol and μ is numerology related to the SCS. - The SCS for LP-SS should be same with the SCS for LP-WUS. - The supported M values are 1, 2, 4. - The length of LP-SS binary sequence (L) corresponding to each M and the binary sequence set - M=1, L= {6,8} For M=1, L=6, the set of LP-SS sequence is: [1 0 1 0 1 0] [0 1 0 1 0 1] [1 0 0 1 0 1] [1 0 1 0 0 1]; For M=1, L=8, the set of LP-SS sequence is: [1 0 1 0 0 1 0 1] [1 0 1 0 1 0 0 1] [1 0 0 1 0 1 0 1] [0 1 0 1 0 1 0 1]. - M=2, L= {12,16} For M=2, L=12, the set of LP-SS sequence is: [1 0 0 1 1 0 0 1 1 0 0 1] [0 1 1 0 1 0 0 1 1 0 0 1] [0 1 1 0 0 1 1 0 1 0 0 1] [0 1 1 0 0 1 0 1 1 0 0 1]; For M=2, L=16, the set of LP-SS sequence is: [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1] [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1] [1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1] [1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]. - M=4, L= {16,32} For M=4, L=16, the set of LP-SS sequence is: [0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0] [0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0] [1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1] [1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]; For M=4, L=32, the set of LP-SS sequence is: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1]. - Note: For each value of M, UE supports both L values – no UE capability. - Note: There is no consensus in RAN1 that SNR of -3dB can be fulfilled with the smaller value of L for each M. Definition of the proper requirements for different values of L is up to RAN4. - Periodicity of LP-SS - 320ms and 160ms (for LP-WUS with M=1 and M=2, FFS: M=4)

References

[1] R1-2501703, LS on LP-WUS UE RF.