3GPP TSG RAN WG1 #121							R1-2503555

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	9.2.2

Source: 	Moderator (Samsung)

Title: 	Moderator Summary#1 on Rel-19 CSI enhancements: Round 1

Document for:	Discussion and Decision

## Introduction

The scope given in the Rel-19 NR MIMO Phase 5 WID pertaining to CSI enhancement is as follows (2d added in [1]):

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

## Summary of companies’ proposals and views

| 3.1 | Proposal 3.A: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-Dd’ (delay offset), add the following definition of the measurement quantity in TS38.215 as follows (analogous to the Rel-18 TDCP): - For FR1, the reference point shall be the antenna connector of the UE. FL assessment: The above was already captured in TS38.215 but put in brackets awaiting RAN1 decision (agreement or conclusion of no support). The text was copied from Rel-18 TDCP definition. Whether this is needed or not for CJTC report is unclear especially for FR2. | Support/fine: Google, Samsung, Spreadtrum, OPPO, CATT, ZTE, Sanechips, Xiaomi, Apple, Qualcomm, vivo, ETRI, Fujitsu, Huawei/HiSi, KDDI, Rakuten, Ericsson, Lenovo/MotM, Not support: |  |
| --- | --- | --- | --- |
| 1.5 | [118] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the port subset indication (from the Rel-18 SD NES Type-1), extend the bitmap (i.e., port-subsetIndicator) by adding p48, p64, and p128 along with their appropriate lengths - Note: Per previous agreements, K is the number of aggregated NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128 Proposal 1.E: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when configured with port subset indication (from the Rel-18 SD NES Type-1), the actual number of ports (the number of 1s in the port-subsetIndicator bitmap) can correspond to a supported value of PCSI-RS for the Rel-19 Type-I SP codebook (i.e. either 48, 64, or 128), or to a supported value of PCSI-RS for the Rel-15 Type-I SP codebook (i.e., either 2, 4, 8, 12, 16, 24, 32), respectively - The supported number of ports for port subset of the full PCSI-RS ports, is according to UE capability FL assessment: Since Rel-19 Type-I SP can be used with SD NES Type-1, and in Rel-18 SD NES Type-1 necessitates that the actual number of ports (after port selection) should be aligned with a supported PCSI-RS value for Type-I codebook, this principle should extend to Rel-19 Type-I SP - Extension of Rel-19 Type-I SP to <=32 ports was proposed before and wasn’t agreeable even after discussed for 4 meetings | Support/fine: Google, NTT DOCOMO, Nokia, Samsung, MediaTek, Spreadtrum, HONOR, OPPO, CATT, ZTE, Sanechips, CMCC, Xiaomi, Apple, NEC, Qualcomm, vivo, Sharp, ETRI, Fujitsu, Huawei/HiSi, Rakuten, Ericsson, Fraunhofer IIS/HHI, Sony, Lenovo/MotM (conclusion) Not support: |  |
| 1.2.2 | [120bis] Agreement For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot. - FFS: How to resolve this for Rel-19 Type-II Doppler Proposal 1.B.2: For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, when the KDOPP CSI-RS resource groups are configured for aperiodic CMR, where each CSI-RS resource group comprises K>1 aggregated NZP aperiodic CSI-RS resources, the triggering offset of an associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot FL assessment: Alt1 (“first”) strictly follows the previous agreement on non-Doppler case and should be considered as the baseline. Alt2 (“last”) was argued by Samsung to relax UE buffering requirement and potentially more accurate IM for future PDSCH transmission especially in higher speed scenarios (use case for Doppler) [5]. Alt3 (lowest ID) was argued by CATT following some other Doppler solutions in the spec [7]. - Either Alt2, Alt2, or Alt3 is valid and works. - It’s unclear if Alt4 actually works since some of the K resources within the 1st resource group can be configured in different slots. - Alt2 may have slight advantage over the other two for high-speed UE although the extent is unclear. - Either Alt1 or Alt2 can be perceived as a simple extension of the previous agreement (but not Alt3). - This is a very minor issue and here over-creativity/over-optimization isn’t necessary. | Support/fine: IDC, Spreadtrum, UNISOC, ZTE, Sanechips, CMCC, Xiaomi, Nokia, Fujitsu, ETRI, HONOR, OPPO, NTT DOCOMO, Google, Samsung (ok), MediaTek, CATT, Apple, NEC, Qualcomm, vivo, Sharp, ETRI, Fujitsu, Huawei/HiSi, Rakuten, Ericsson, Fraunhofer IIS/HHI, Tejas, Sony, Lenovo/MotM, Not support: |  |
| 1.4 | [118] Agreement For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, - For Capability 1 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8 - Active resource counting follows the legacy Rel-18 Type-II Doppler - For Capability 2 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS - Active resource counting follows the legacy Rel-18 Type-II Doppler Proposal 1.D: For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, for Capability 1, further clarify the previous agreement that OCPU=min⁡(Y1⋅KDOPP⋅P/32,8) FL assessment (revised): After further checking, it seems the current agreement was not properly captured by the editor in the latest CR for TS 38.214 (the editor didn’t apply the min(.,8) operation as correctly pointed out by NEC. Perhaps the phrase “while still not exceeding 8” was misinterpreted that this requires {Y1, KDOPP} combinations to achieve OCPU<=8. Note that other proposals from Nokia [17] such as setting OCPU=8 or following Capability 2 would revert previous agreement. | Support/fine: NEC, NTT DOCOMO, IDC, MediaTek, Spreadtrum, HONOR, OPPO, ZTE, Sanechips, CMCC, Xiaomi, Apple, Qualcomm, vivo, Sharp, Fujitsu, Huawei/HiSi, Nokia, Samsung, Ericsson, ETRI, Tejas (ok), Not support (NW implementation): Google, |  |
| 1.1 | [116bis] Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation, support NW to configure UE with one of the following mapping methods via higher-layer (RRC) signaling, - Mapping method 1: Sequential ordering/indexing within (1st resource, 1st polarization), then (2nd resource, 1st polarization), …, then (Kth resource, 1st polarization), then (1st resource, 2nd polarization), then (2nd resource, 2nd polarization), …, then (Kth resource, 2nd polarization) - Mapping method 2: Sequential ordering/indexing within (where K*n2 = N2): [117] Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding port mapping, - Following legacy principle, “sequential ordering/indexing within” a group of Q indices {i0, i1, …, iQ-1} is a linearly increasing sequence such that iq < iq+1 (where q=0, 1, …, Q-2 is the port index within a CSI-RS resource, and iq or iq+1 ∈ {0, 1,…, KQ-1}) is the port index for the codebook, across the K>1 CSI-RS resources). - After resource aggregation, P (=48, 64, or 128) ports are numbered in accordance to Table 7.4.1.5.3-1 from TS 38.211 Proposal 1.A: For the Rel-19 Type-I SP and Type-II codebook refinement for P=48, 64, and 128 CSI-RS ports with K>1 aggregated NZP aperiodic CSI-RS resources for CMR, to implement the previous agreements on the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation: - In TS38.211, extend the enumeration of antenna port p to 3000+P–1, and directly specify the mapping relationship from CSI-RS sequences in each CDM group of each aggregated CSI-RS resources to antenna ports. - In TS38.214, remove the term ‘port index for CSI/PMI calculation’ FL assessment: The above proposal attempts to amend the implementation of the previous agreements in the current editor CRs (for 38.211 and 38.214). An example TP is given in [6]. This proposal will admittedly result in a clearer specification. But it can also be argued that the implementation in the current editor CRs correctly may already capture the agreements. |  | Support/fine: ZTE, Sanechips, Google, NTT DOCOMO, Samsung, MediaTek, Spreadtrum, HONOR, OPPO, CATT, CMCC, Xiaomi, Apple, NEC, Qualcomm, Sharp, ETRI, Fujitsu, Huawei/HiSi, Rakuten, Ericsson, Tejas (ok), Not support: Nokia, vivo, Fraunhofer IIS/HHI, |

Please read:

- With 1 more meeting left for the normative phase, per Chairman’s guidance, the focus should be on essential issues.

  - Non-essential proposals from RAN1#120bis that were not agreed are now removed in RAN1#121. Proposals 2.B and 2.C are still kept since the main proponent (Tejas) couldn’t attend RAN1#120bis due to visa issues.

  - New non-essential proposals will not be discussed.

- Proposals strictly related to editor CRs (such as TPs) will not be addressed here since we are still in the normative phase. Please share them during the email discussion for editor CR review, or (for editorial issues) contact the editors directly.

  - TP proposals that can’t be handled by editor CR review but are clearly maintenance-level issues will be handled during maintenance (starting from RAN1#122 August 2025)

- Proposals related to UE capability will be handled in UE feature session.

### Issue 1 (WID objective 2a and 2b): Type-I and Type-II codebook refinement for up to 128 CSI-RS ports

Table 1A Summary: issue 1

| # | Issue/proposal | Companies’ views |
| --- | --- | --- |
| New proposals |  |  |
| 1.1 | [116bis] Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation, support NW to configure UE with one of the following mapping methods via higher-layer (RRC) signaling, - Mapping method 1: Sequential ordering/indexing within (1st resource, 1st polarization), then (2nd resource, 1st polarization), …, then (Kth resource, 1st polarization), then (1st resource, 2nd polarization), then (2nd resource, 2nd polarization), …, then (Kth resource, 2nd polarization) - Mapping method 2: Sequential ordering/indexing within (where K*n2 = N2): [117] Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding port mapping, - Following legacy principle, “sequential ordering/indexing within” a group of Q indices {i0, i1, …, iQ-1} is a linearly increasing sequence such that iq < iq+1 (where q=0, 1, …, Q-2 is the port index within a CSI-RS resource, and iq or iq+1 ∈ {0, 1,…, KQ-1}) is the port index for the codebook, across the K>1 CSI-RS resources). - After resource aggregation, P (=48, 64, or 128) ports are numbered in accordance to Table 7.4.1.5.3-1 from TS 38.211 Proposal 1.A: For the Rel-19 Type-I SP and Type-II codebook refinement for P=48, 64, and 128 CSI-RS ports with K>1 aggregated NZP aperiodic CSI-RS resources for CMR, to implement the previous agreements on the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation: - In TS38.211, extend the enumeration of antenna port p to 3000+P–1, and directly specify the mapping relationship from CSI-RS sequences in each CDM group of each aggregated CSI-RS resources to antenna ports. - In TS38.214, remove the term ‘port index for CSI/PMI calculation’ FL assessment: The above proposal attempts to amend the implementation of the previous agreements in the current editor CRs (for 38.211 and 38.214). An example TP is given in [6]. This proposal will admittedly result in a clearer specification. But it can also be argued that the implementation in the current editor CRs correctly may already capture the agreements. | Support/fine: ZTE, Sanechips, Google, NTT DOCOMO, Samsung, MediaTek, Spreadtrum, HONOR, OPPO, CATT, CMCC, Xiaomi, Apple, NEC, Qualcomm, Sharp, ETRI, Fujitsu, Huawei/HiSi, Rakuten, Ericsson, Tejas (ok), Not support: Nokia, vivo, Fraunhofer IIS/HHI, |
| 1.2.1 | [120bis] Agreement For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48 , 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot. - FFS: How to resolve this for Rel-19 Type-II Doppler Proposal 1.B.1: For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset configured with the NZP CSI-RS resource set including an associated aperiodic NZP CSI-RS resource for interference measurement is the same as that of the associated NZP CSI-RS resource(s) for CMR in the first slot. FL assessment: This proposal is technically sound and a simple extension of the previous agreement for AP CSI-IM to NZP CSI-RS for IM except for Type-II Doppler. However, Nokia argues that this can be handled via NW implementation since NZP CSI-RS for IM (unlike CSI-IM) has its own triggering offset. Nokia’s argument is valid. | Support/fine: Samsung, Google, NTT DOCOMO, MediaTek, Spreadtrum, HONOR, CATT, Apple, NEC, Qualcomm, vivo, Sharp, Huawei/HiSi (open), Rakuten, Fraunhofer IIS/HHI, Tejas, Lenovo/MotM, Not support (NW implementation): Nokia, OPPO, ZTE, CMCC, ETRI, Fujitsu, Ericsson, Sony, |
| 1.2.2 | [120bis] Agreement For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot. - FFS: How to resolve this for Rel-19 Type-II Doppler Proposal 1.B.2: For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, when the KDOPP CSI-RS resource groups are configured for aperiodic CMR, where each CSI-RS resource group comprises K>1 aggregated NZP aperiodic CSI-RS resources, the triggering offset of an associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot FL assessment: Alt1 (“first”) strictly follows the previous agreement on non-Doppler case and should be considered as the baseline. Alt2 (“last”) was argued by Samsung to relax UE buffering requirement and potentially more accurate IM for future PDSCH transmission especially in higher speed scenarios (use case for Doppler) [5]. Alt3 (lowest ID) was argued by CATT following some other Doppler solutions in the spec [7]. - Either Alt2, Alt2, or Alt3 is valid and works. - It’s unclear if Alt4 actually works since some of the K resources within the 1st resource group can be configured in different slots. - Alt2 may have slight advantage over the other two for high-speed UE although the extent is unclear. - Either Alt1 or Alt2 can be perceived as a simple extension of the previous agreement (but not Alt3). - This is a very minor issue and here over-creativity/over-optimization isn’t necessary. Alt1 (baseline). Following the associated NZP CSI-RS resource(s) for CMR in the first slot - Support/fine: IDC, Spreadtrum, UNISOC, ZTE, Sanechips, CMCC, Xiaomi, Nokia, Fujitsu, ETRI, HONOR, OPPO, NTT DOCOMO, Google, Samsung (2nd), MediaTek, Spreadtrum, HONOR, - Concern: Alt2. Following the associated NZP CSI-RS resource(s) for CMR in the last slot - Support/fine: Samsung, - Concern: Alt3. Following the associated NZP CSI-RS resource for CMR with the lowest CSI-RS resource ID - Support/fine: CATT, Xiaomi, - Concern: Alt4. Following the first resource group without any additional offset - Support/fine: Qualcomm, - Concern: | Support/fine: IDC, Spreadtrum, UNISOC, ZTE, Sanechips, CMCC, Xiaomi, Nokia, Fujitsu, ETRI, HONOR, OPPO, NTT DOCOMO, Google, Samsung (ok), MediaTek, CATT, Apple, NEC, Qualcomm, vivo, Sharp, ETRI, Fujitsu, Huawei/HiSi, Rakuten, Ericsson, Fraunhofer IIS/HHI, Tejas, Sony, Lenovo/MotM, Not support: |
| 1.3 | Proposal 1.C: For Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, CSI-RS ports (within a given CSI-RS resource group) mapped to the same port index for CSI/PMI calculation across the K aperiodic CSI-RS resources share the same antenna port. FL assessment: This proposal is technically sound. It was argued by vivo that for “KDOPP = {4, 8, 12} CSI-RS resource groups are introduced for Type-II Doppler CSI. This means that within a CMR group, there may be multiple CSI-RS ports with the same CSI-RS port index mapped to different antenna ports.” | Support/fine: vivo, NTT DOCOMO, Nokia, IDC, Spreadtrum, Sharp, Rakuten, Not support (NW implementation): Google, Samsung, HONOR, OPPO, ZTE (resolve 1.A first), Sanechips, CMCC, Xiaomi, Apple (wait 1.A), ETRI, Fujitsu, Ericsson (wait 1.A), Fraunhofer IIS/HHI, Tejas, Lenovo/MotM, |
| 1.4 | [118] Agreement For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, - For Capability 1 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8 - Active resource counting follows the legacy Rel-18 Type-II Doppler - For Capability 2 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS - Active resource counting follows the legacy Rel-18 Type-II Doppler Proposal 1.D: For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, for Capability 1, further clarify the previous agreement that OCPU=min⁡(Y1⋅KDOPP⋅P/32,8) FL assessment (revised): After further checking, it seems the current agreement was not properly captured by the editor in the latest CR for TS 38.214 (the editor didn’t apply the min(.,8) operation as correctly pointed out by NEC. Perhaps the phrase “while still not exceeding 8” was misinterpreted that this requires {Y1, KDOPP} combinations to achieve OCPU<=8. Note that other proposals from Nokia [17] such as setting OCPU=8 or following Capability 2 would revert previous agreement. | Support/fine: NEC, NTT DOCOMO, IDC, MediaTek, Spreadtrum, HONOR, OPPO, ZTE, Sanechips, CMCC, Xiaomi, Apple, Qualcomm, vivo, Sharp, Fujitsu, Huawei/HiSi, Nokia, Samsung, Ericsson, ETRI, Tejas (ok), Not support (NW implementation): Google, |
| 1.5 | [118] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the port subset indication (from the Rel-18 SD NES Type-1), extend the bitmap (i.e., port-subsetIndicator) by adding p48, p64, and p128 along with their appropriate lengths - Note: Per previous agreements, K is the number of aggregated NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128 Proposal 1.E: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when configured with port subset indication (from the Rel-18 SD NES Type-1), the actual number of ports (the number of 1s in the port-subsetIndicator bitmap) can correspond to a supported value of PCSI-RS for the Rel-19 Type-I SP codebook (i.e. either 48, 64, or 128), or to a supported value of PCSI-RS for the Rel-15 Type-I SP codebook (i.e., either 2, 4, 8, 12, 16, 24, 32), respectively - The supported number of ports for port subset of the full PCSI-RS ports, is according to UE capability FL assessment: Since Rel-19 Type-I SP can be used with SD NES Type-1, and in Rel-18 SD NES Type-1 necessitates that the actual number of ports (after port selection) should be aligned with a supported PCSI-RS value for Type-I codebook, this principle should extend to Rel-19 Type-I SP - Extension of Rel-19 Type-I SP to <=32 ports was proposed before and wasn’t agreeable even after discussed for 4 meetings | Support/fine: Google, NTT DOCOMO, Nokia, Samsung, MediaTek, Spreadtrum, HONOR, OPPO, CATT, ZTE, Sanechips, CMCC, Xiaomi, Apple, NEC, Qualcomm, vivo, Sharp, ETRI, Fujitsu, Huawei/HiSi, Rakuten, Ericsson, Fraunhofer IIS/HHI, Sony, Lenovo/MotM (conclusion) Not support: |
| Proposals from previous more recent meeting(s) and/or round(s) |  |  |
|  | -- |  |

Table 1B SLS results: issue 1

--

Table 1C Additional inputs: issue 1

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 1A. |
| Google | Proposal 1.A: OK Proposal 1.B.1: OK with the change. Question 1.B.2: We support Alt1 Proposal 1.C: We failed to see the necessity. Probably we can discuss the TP directly. Proposal 1.D: We failed to see the necessity. Probably we can discuss the TP directly. Conclusion 1.E: OK |
| NTT DOCOMO | Proposal 1.A: No strong view. Either way works for us. Proposal 1.B.1: Support Question 1.B.2: We support Alt1 Proposal 1.C: Support Proposal 1.D: Support Conclusion 1.E: For gNB with larger port number number, NES operation is important. And it is a promising case that gNB configures a sub-configuration with <=32 ports for a UE for NES purpose. Thus, we think following configuration should be supported. - CSI-ReportConfig (parent configuration) with 48, 64, 128 ports - Sub-configuration: actual enabled port number <= 32 ports - For this sub-configuration, Rel-15 single-panel codebook is configured And we’d like to clarify whether the latest CR has supported the above case or not. |
| Nokia | Proposal 1.A This is not needed. The current drafts in 211 and 214 are well aligned and correctly implement the agreements. There doesn’t seem to be any problem that needs fixing Proposal 1.B.1 Ok to discuss. However, in our view, the specs already state the aperiodicTriggeringOffset configured for the NZP IMR resource set is the same as that configured for the NZP CMR resource set. The NZP IMR resource cannot be configured with additionalOneSlotOffset, so the NZP IMR resource is always in the first of the two slots Question 1.B.2 In my understanding both Alt1 and Alt4 mean the same thing, i.e., “the aperiodic triggering offset of the CSI-IM follows the offset of the associated NZP CSI-RS resource(s) in the first resource group configured without additionalOneSlotOffsetDopp”. In practice, this is “the first slot” Proposal 1.C We are ok to clarify the UE assumption on the antenna port across the KDOPP resource groups in a similar way as was done in Rel-18 Type-II-Doppler. Because the CSI-RS resources are ordered ascendingly by the resource ID and group index, this can be done more simply without referring to the port index for CSI/PMI calculation, for example as follows: “The UE shall assume that the antenna port with the same port index of the KDOPP aperiodic CSI-RS resources {k,k+K,…,k+KDOPP-1K} is the same, for each k=0,1,…,K-1” Proposal 1.D The issue in our view is that for capability 1, there is no value of Y1 that a UE can indicate for KDOPP=8 such that OCPU≤8 for any P∈{48,64,128}. For KDOPP=4, only Y1=1 can be indicated for P∈{48,64}, which yields OCPU=8, and no value for P=128. Conclusion 1.E My understanding is the same as DOCOMO, i.e. we don’t have any restriction for the number of enabled ports to be ≤32, and for these subconfigurations, Rel-15 Type-I SP is configured in the subconfiguration. This is implied by supporting SD NES Type-1 for 48 ports, for which the number of enabled ports is necessarily ≤32. If we introduce the restriction in the conclusion, we also need to remove p48 from the value range of the RRC parameter port-subsetIndicator Agreement(RAN1#118) For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the port subset indication (from the Rel-18 SD NES Type-1), extend the bitmap (i.e., port-subsetIndicator) by adding p48, p64, and p128 along with their appropriate lengths |
| Samsung | Proposal 1.A We couldn’t find any issue of the current implementation in the specs 211/214 (i.e., correctly reflecting the agreements), so we don’t think this proposal is needed. However, we agree that the proposal makes it somewhat clearer, so we are ok if the majority supports. Proposal 1.B.1 @Nokia, we can discuss. But it looks like some clarification is needed, since in the spec we have the following text: 5.2.1.5.1 of TS38.214: If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.. Here, it is stated that a different aperiodic triggering offset of the NZP CSI-RS for IMR from the associated NZP CSI-RS for CMR is not configurable, but now we have multiple NZP CSI-RS resources for CMR in Rel-19, where the multiple NZP CSI-RS resources can have different aperiodic triggering offsets within two consecutive slots. As we have discussed the same issue for CSI-IM, it is unclear which triggering offset of the NZP CSI-RS resources is ‘the reference triggering offset’ for the NZP CSI-RS for IMR. We think it’d be better to clarify this. Question 1.B.2 Our second preference is Alt1. Proposal 1.C We don’t think it is needed since it is an obvious assumption that the NW/UE needs to follow, unless an erroneous case is considered. Proposal 1.D. We think it is a valid issue and are OK with this proposal. For example, when Y1=2 is reported as UE cap, there is no KDOPP∈{4,8} value satisfying Y1⋅KDOPP.P32≤8, meaning NW can’t configure aperiodic CMR for Rel-19 Doppler CSI. As another example, when Y1=1 is reported as UE cap, KDOPP=4 is the only possible candidate for configuring aperiodic CMR for Rel-19 Doppler CSI. Conclusion 1.E Our understanding is the same as Nokia/DCM, so we suggest to revise it as follows: Conclusion 1.E: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when configured with port subset indication (from the Rel-18 SD NES Type-1), the actual number of ports (the number of 1s in the port-subsetIndicator bitmap) corresponds to a supported value of PCSI-RS for the Rel-19 Type-I SP codebook (i.e. either 48, 96, or 128) and to a supported value of PCSI-RS for the Rel-15 Type-I SP codebook (i.e., either 2, 4, 8, 12, 16, 24, 32), respectively. - Note: This is analogous to the legacy Rel-18 SD NES Type-1. There is no need for introducing additional restriction(s) for the actual number of ports in the specification |
| InterDigital | Proposal 1.C: Support Proposal 1.D: Support |
| Mod V7 | Revision to proposals 1.C and 1.E per comments from Nokia and DOCOMO/Nokia/Samsung, respectively (1.E is no longer conclusion). @Nokia: please check Samsung’s response to your question on 1.B.1 |
| MediaTek | Proposal 1.A: We are fine to support, as it makes spec more clear. Proposal 1.B.1: Support Question 1.B.2: Support Alt1 Proposal 1.D: We support this proposal. Proposal 1.E: Support |
| Spreadtrum | Proposal 1.A: Support this clarification. Proposal 1.B.1: Support. Question 1.B.2: Support Alt1. We agree with FL that this is a minor issue, a unified solution for all Type-I/II codebooks is preferred. Proposal 1.C: We think this assumption is correct. Maybe we can achieve a conclusion instead of an agreement. Proposal 1.D: We can support this proposal. Proposal 1.E: Support. |
| Nokia | Proposal 1.B.1 @Samsung, in my understanding the multiple NZP CSI-RS resources for channel measurement in Rel-19 have the same aperiodic triggering offset (the RRC parameter with that name, aperiodicTriggeringOffset, is common across the resources), and each resource can be configured with additionalOneSlotOffset. So, if the NZP-IMR resource set is configured with the same aperiodicTriggeringOffset as the NZP-CMR resource set, that’s sufficient to ensure that the NZP-IMR is in the “first slot”. For ZP-CSI-IM, the problem was different because the resource set is not configured with its own aperiodic triggering offset, so we have to define one, and the legacy definition was ambiguous for Rel19 because it “follows the offset of the associated CMR”, and this offset can be interpreted as the resource-specific offset. Proposal 1.E Ok |
| HONOR | Proposal 1.A: No strong view. Fine with the clarification. Proposal 1.B.1: Support. Question 1.B.2: Support Alt1. It should be the first slot in the first resource group. Proposal 1.C: Not necessary. It’s an obvious network implementation. Proposal 1.D: Support. Proposal 1.E: Support |
| Mod V12 | Changed question 1.B.2 to proposal 1.B.2 based on Alt1 given the super-majority situation. - The clause on NZP IMR is put in brackets as 1.B.1 is still pending @Samsung: please check Nokia’s response re 1.B.1, which I tend to agree (that it can be handled via NW implementation as NZP IMR has itw own slot offset) |
| OPPO | Proposal 1.A: We are fine with the proposal. The proposal can achieve a unified design for >32 ports and legacy ports, and is easy to be extended to 256 ports in 6G. Proposal 1.B.1: We intend to agree with Nokia that current spec. is sufficient without ambiguity. Proposal 1.B.2: Support. Proposal 1.C: Not support. The proposal is unclear to us. Why the CSI-RS ports across the K aperiodic CSI-RS resources within a group should be mapped to the same port index for CSI/PMI calculation? Should it be the CSI-RS ports across the KDOPP aperiodic CSI-RS resource group which needs to be mapped to the same port index for CSI/PMI calculation? Proposal 1.D: Support Proposal 1.E: Support |
| CATT | Proposal 1.A: OK Proposal 1.B.1: OK Proposal 1.B.2: We are also fine with the first slot. In our understanding, Alt1 (follows the associated NZP CSI-RS resource(s) for CMR in the first slot) and Alt3 (follows the associated NZP CSI-RS resource for CMR with the lowest CSI-RS resource ID) are equivalent. Since following resource ordering was agreed in #117： Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, - Ordering the KDOPPK CSI-RS resources ascendingly by the CSI-RS resource ID and kDOPP ={0,1,…, KDOPP –1}, CSI-RS resources { kDOPPK, kDOPPK +1, …, (kDOPP+1)K –1} are associated with the kDOPP-th CSI-RS resource group - FFS: If the CSI-RS resources in a resource group span two consecutive slots, m is 2. - FFS: If the CSI-RS resources in a resource group are located in one slot, m can be configured from {1, 2}. Therefore，resource with the lowest resource ID is indeed in the first slot. Our understanding is that a description such as “CMR in the first slot” is not very clear in the spec while the description “CMR with the lowest CSI-RS resource ID” is more accurate. However, we understand that how to capture “CMR in the first slot” is up to the spec editor. Proposal 1.E: support |
| ZTE | Proposal 1.A: Support. - Based on current standards, each aggregated CSI-RS resource is mapped to same antenna port indices. For instance, when 4 32-port CSI-RS resources are aggregated to support 128-port CSI-RS, each 32-port CSI-RS is mapped to antenna port 3000 to 3031. In legacy, if one CSI-RS resource is mapped to a set of antenna ports, this CSI-RS should be transmitted using this set of antenna ports. Then the aggregated CSI-RS resources fail to support up-to-128-port CSI-RS. - Besides, the new introduced concept ‘port index for CSI/PMI calculation’ is very ambiguous, which is never defined in legacy or other places of the standards. With such a new concept, the interpretation of antenna ports in CSI/PMI calculation is misaligned. More importantly, RAN1 never agreed to introduce such a new concept. Proposal 1.B.1: We tend to agree with Nokia that, this proposal is NOT needed. As described in current standards, AP NZP CSI-RS for interference measurement is expected to be configured with a same triggering offset as AP NZP CSI-RS for channel measurement. So, the triggering offset of AP NZP CSI-RS for interference measurement is naturally same as the triggering offset of AP NZP CSI-RS for channel measurement in the first slot (if more than one CSI-RS are aggregated over two consecutive slots.). 5.2.1.5.1 of TS38.214: If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.. However, it is better to have a conclusion to clarify that, the triggering offset of AP NZP CSI-RS for interference measurement is same as the resource-set-level triggering offset of AP NZP CSI-RS for channel measurement, like: Conclusion 1.B.1: For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset configured with the NZP CSI-RS resource set including an associated aperiodic NZP CSI-RS resource for interference measurement is the same as the resource-set-level triggering offset configured with that of the associated NZP CSI-RS resource(s) for CMR in the first slot. [Mod: After more comments, it is quite clear to me that the proposal is not needed since it can be achieved by NW implementation. Likewise, I don’t think we need to conclude anything since the slot offset for NZP IMR is already supported in the spec – there is no ambiguity at all. We don’t need to conclude on every single possible NW implementation facilitated by the spec. It is understood that NW vendors understand the spec and how to use it.] Proposal 1.B.2: Support. Proposal 1.C: To our understanding, this proposal is to solve the ambiguity caused by the new introduced ‘port index for CSI/PMI calculation’. As suggested in Proposal 1.A, the mapping between sequences in each aggregated CSI-RS resources to antenna ports 3000+P (P = 0, 1, …, 47/63/127) should be directly specified. Then Proposal 1.C is no further needed. So, we suggest to discuss Proposal 1.A first. Proposal 1.D: OK. Proposal 1.E: OK. |
| CMCC | Proposal 1.A: Support. Proposal 1.B.1: Share same view as Nokia/OPPO/ZTE, current spec is enough to make sure the NZP IMR share the same slot offset as CMR. Proposal 1.B.2: Support this proposal without the wording in the blanket. Proposal 1.C: The proposal is unclear to us. We think it should be the CSI-RS ports across the KDOPP aperiodic CSI-RS resource group share the same antenna port. Proposal 1.D: Support. Proposal 1.E: Support. |
| Xiaomi | Proposal 1.A: OK Proposal 1.B.2: We are fine. Proposal 1.C: We think the proposal is not needed. If mapping relationship from CSI-RS sequences in each CDM group of each aggregated CSI-RS resources to antenna ports is same for KDopp resource groups, the same CSI-RS port index denotes the same antenna port. Proposal 1.D: OK Proposal 1.E: OK |
| Samsung | Proposal 1.B.2 @Nokia/ZTE: Thanks for the comments. We understood your view and actually started from your understanding. But we have realized that the current text of the spec became not clear whether the different aperiodic triggering offset in the spec text (highlighted in red) is associated with either NZP-CSI-RS resource level IE or NZP-CSI-RS resource-set level IE, as we’ve introduced resource-specific additional slot offset. The spec text only states the associated NZP CSI-RS for channel measurement (highlighted in blue). In other words, it is unclear whether the associated NZP CSI-RS is either an NZP-CSI-RS resource set or an NZP CSI-RS resource. 5.2.1.5.1 of TS38.214: If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.. At the same time, we admit that what we suggested as a proposal is actually the same result of Nokia’s (some other companies’) understanding. So, as ZTE suggested, we also think that at least the conclusion is needed and can be sufficient to align companies’ understanding. [Mod: Please check my comment to ZTE. I don’t think conclusion is needed.] |
| Apple | Proposal 1.A: okay Proposal 1.B.1: okay Proposal 1.B.2: okay Proposal 1.C: don’t we need to progress Proposal 1.A first? Proposal 1.D: okay Proposal 1.E: okay |
| Mod V20 | Revised proposal 1.B.2 per CATT comment, and removed the bracketed text for NZP IMR since 1.B.1 is clearly not needed. Based on the current situation, proposals 1.A, 1.B.2, 1.D, and 1.E may be promising for endorsement. I won’t discuss proposals 1.B.1 and 1.C. |
| Samsung | Proposal 1.D: After further checking, we agree with Google that the proposal is not an essential issue and have inclined to not support. Although Y1=1 and KDOPP=4 are the only combo that satisfies OCPU≤8, that doesn’t necessarily mean that the proposal is needed. It just basically says NW can configure only KDOPP=4, only if Y1=1 is reported as UE cap. Given that the spec draft has been endorsed with capturing the previous agreement correctly and there is no issue, we can’t support the proposal. |
| NEC | Proposal 1.A: Support. Proposal 1.B.1/1.B.2: Fine. Proposal 1.D: Support. @ Google and Samsung, the issue is actually the agreement was not accurately captured, we have the agreement to say “not exceeding 8”, and we didn’t distinguish different solutions for different values of KDOPP (4, 8) or 12. Agreement@RAN1#118 For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, - For Capability 1 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8 - Active resource counting follows the legacy Rel-18 Type-II Doppler - For Capability 2 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS - Active resource counting follows the legacy Rel-18 Type-II Doppler. While the current TS CR distinguishes the cases, i.e. for KDOPP =12, OCPU is directly defined as 8, (i.e. max(,8) was added), but for KDOPP =4 or 8, not exceeding 8 is not mentioned (actually, whether support or not depends on NW configuration and UE capability). So in our understanding, we need to align/unify the cases with different values of KDOPP . it’s not correct to interpret the agreement differently. And there is no agreement for the current TS draft (no agreement saying OCPU=8 only for KDOPP=12) In other words, we can have two interpretations for the agreement. - First interpretation: all cases relying on NW configuration, such as (the cons for this interpretation will lead to no configuration can be supported for 128 ports) - if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to ‘cri-RI-PMI-CQI’ and with codebookType set to ‘typeII-Doppler-r19’, - if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with KDOPP CSI-RS resource groups of K CSI-RS resources, with total number of CSI-RS ports P∈{48,64,128}, - for capability 1, OCPU=8 for KDOPP=12 and OCPU=Y1⋅KDOPP⋅P/32 for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication, - for capability 2, OCPU=8 for KDOPP=12 and OCPU=Y1⋅KDOPP for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication, - if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with K CSI-RS resources, OCPU=4 for N4=1 and OCPU=max(Y2⋅N4,4) for N4>1, where the value of N4 is configured by the higher layer parameter N4, and Y2∈{1, 2, 3} is reported by UE capability indication, for both capability 1 and 2 indication,. - Second interpretation: adding restriction of not exceeding 8 to all cases, such as - if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to ‘cri-RI-PMI-CQI’ and with codebookType set to ‘typeII-Doppler-r19’, - if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with KDOPP CSI-RS resource groups of K CSI-RS resources, with total number of CSI-RS ports P∈{48,64,128}, - for capability 1, OCPU=8 for KDOPP=12 and OCPU=min⁡(Y1⋅KDOPP⋅P/32,8) for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication, - for capability 2, OCPU=8 for KDOPP=12 and OCPU=Y1⋅KDOPP for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication, - if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with K CSI-RS resources, OCPU=4 for N4=1 and OCPU=max(Y2⋅N4,4) for N4>1, where the value of N4 is configured by the higher layer parameter N4, and Y2∈{1, 2, 3} is reported by UE capability indication, for both capability 1 and 2 indication,. We prefer the second interpretation, which is aligned with the agreement, and can support 128 ports. In addition, if we go with current TS CR wording, it means KDOPP=12 can be supported with OCPU=8, but KDOPP=4 or 8 can not be supported, it’s really a weird design. Proposal 1.E: Fine |
| NEC2 | Proposal 1.D: To clarify more, our intention is not to revert the agreement to define OCPU=8 for any case, but to reflect the agreement “not exceeding 8” correctly with min(,8) , as the current TS CR didn’t capture the agreement correctly. Agreement@RAN1#118 For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, - For Capability 1 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8 - Active resource counting follows the legacy Rel-18 Type-II Doppler - For Capability 2 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS - Active resource counting follows the legacy Rel-18 Type-II Doppler. Draft TS 38.214 - if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to ‘cri-RI-PMI-CQI’ and with codebookType set to ‘typeII-Doppler-r19’, - if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with KDOPP CSI-RS resource groups of K CSI-RS resources, with total number of CSI-RS ports P∈{48,64,128}, - for capability 1, OCPU=8 for KDOPP=12 and OCPU=Y1⋅KDOPP⋅P/32 for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication, - for capability 2, OCPU=8 for KDOPP=12 and OCPU=Y1⋅KDOPP for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication, - if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with K CSI-RS resources, OCPU=4 for N4=1 and OCPU=max(Y2⋅N4,4) for N4>1, where the value of N4 is configured by the higher layer parameter N4, and Y2∈{1, 2, 3} is reported by UE capability indication, for both capability 1 and 2 indication,.. So we propose: Updated Proposal 1.D: For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, for Capability 1, OCPU is set to OCPU=min⁡(Y1⋅KDOPP⋅P/32,8) for KDOPP=4, 8, 12 |
| Qualcomm | Proposal 1.A: OK with majority view. We think it is cleaner to have less terminologies – the new term p' (port index for PMI) and legacy term p (port index of CSI-RS resource) look redundant, and can be unified as one terminology. - In our understanding, p' basically is the row index of Equaltion yk0(3000+p(0))(i)yk1(3000+p(1))(i)⋮ykP-1(3000+p(P-1))(i)=W(i)x0(i)⋯xυ-1(i) in brackets: kp', and pp'. - If we reuse legacy simpler Equation since Rel-15: , we don’t need to introduce p'. Therefore, we slightly prefer Proposal 1.A, over the current spec draft for Rel-19 (which has no ambiguity – but it may take me 10 more minutes every time reading here, than Proposal 1.A). Proposal 1.B.1: OK Proposal 1.B.2: OK Proposal 1.D: OK Proposal 1.E: OK in general. But the issue is, if all full-sub combinations are supported, it is heavy for NES IODT test. Therefore, we’d like to add a bullet on UE capability. - The supported number of ports for port subset of the full PCSI-RS ports, is according to UE capability.. Besides, there is a typo: 9664 |
| vivo | - A We don’t agree with it. The current endorsed CR has correctly captured agreements. We don’t see any reason to revisit this. Further, regarding the issue mentioned by ZTE, we don’t think it is a real issue. We cannot say ports with same port index but in different resources are same ports. There are multiple examples in the current spec showing they are not same ports. For codebook or NCB based UL, SRS ports with same port index in different resources are not same ports. For CRI based reporting, we cannot say CSI-RS ports with same port index in different resource are same ports either. Hence, it is not a real issue. 1.B.1 OK. This is to reuse legacy restriction that NZP IMR has the same triggering offset as CMR. 1.B.2 OK - C We think this is needed. In legacy specification (Rel-18 Type II Doppler), the corresponding CSI-RS ports in K resources (a triggered burst) are same ports. This is to reuse the same principle and to ensure correct UE behaviour. - D OK - E OK |
| Sharp | Proposal 1.A: We are fine with the clarification. Proposal 1.B.1: Support. Since the condition of “it is not expected that the triggering offset of NZP CSI-RS for IM is different from that of NZP CSI-RS for CM” cannot be met due to consecutive two slots for NZP CSI-RS for CM, the proposal is needed. Proposal 1.B.2: Support. We agree with FL assessment. Proposal 1.C: Support. We think this assumption is needed for UE. Proposal 1.D: Support. Proposal 1.E: Support. |
| ETRI | Proposal 1.A: Support. We have similar concerns raised by ZTE. Also, we don’t see any necessity to introduce the term ‘port index for CSI/PMI calculation’ in TS 38.214 since the legacy equation can be applied for the extended port number without any change. Proposal 1.B.1: Although Mod has decided not to discuss this proposal, we would like to share our opinion for your reference. We think that whether to configure additionalSlotOffset is up to NW implementation, by following the legacy rule where aperiodicTriggeringOffset is applied as a resource-set level slot offset, like CMR. If the working group’s understanding is that additionalSlotOffset is not permitted for NZP CSI-RS for IM, then we recommend that Mod conclude accordingly, in order to ensure proper implementation through aligned understanding. [Mod: I was instructed not to conclude on things that are not ambiguous from spec perspective. So I will not do so, sorry.] Proposal 1.B.2: Support. Proposal 1.C: Not support. Although Mod has decided not to discuss this proposal, we would like to share our opinion for your reference. This can be handled via NW/UE implementation. We believe that both NW and UE are implemented to maintain the mapping between the ports and the port indices across KDOPP CSI-RS resource groups to guarantee the best performance. We don’t see any necessity to change it at NW and UE for any purpose. Proposal 1.D: Not support. We have the same understanding as Google and Samsung. If we support the proposal, then UE capability indication on Y1 may no longer be required, which could have an impact on the specification. Proposal 1.E: Support. In our understanding, for the actual number of ports (after port selection) Ps <= 32, one of the legacy codebooks can be configured with codebookSubConfig in CSI-ReportSubConfig-r18, while codebookConfig in the root of CSI-ReportConfig is configured with Rel-19 Type-I SP. |
| Fujitsu | Proposal 1.A: Support. Proposal 1.B.1: Support the FL’s assessment. Proposal 1.B.2: Support. Proposal 1.C: We guess the intention of this proposal is that all CSI-RS ports (within one CMR group) in different AP CMR groups can share the same antenna port. Not for all CSI-RS ports across the different AP CMR within one group? [Mod: Also my understanding] If this understanding is correct, we think it can be up to UE implement, which is similar as R18 Doppler Type II. Proposal 1.D: Support. Proposal 1.E: Support. |
| Huawei, HiSilicon | Proposal 1.A: Support it, the proposal makes spec clearer, removing any potential ambiguity. Proposal 1.B.1: Open for further discussion. Proposal 1.B.2: fine with the proposal. Proposal 1.D: OK. Proposal 1.E: OK. |
| Rakuten | Proposal 1.A: Support Proposal 1.B.1: Support Proposal 1.B.2: Accept Proposal 1.C: Support Proposal 1.E: okay |
| Samsung | On Proposal 1.D Regarding the updated proposal from NEC, if this interpretation is correct, the current spec draft didn’t capture the agreement correctly, and needs to be changed as NEC suggested. We admit the NEC interpretation can be valid (if the majority has the same understanding), since it is purely based on the previous agreement. Regardless, the current form of Proposal 1.D is not needed, as we provided input previously. |
| Mod V35 | Proposals 1.D, 1.E: revised per inputs from NEC and Qualcomm, respectively Proposals 1.B.2, 1.C: I was instructed to remove the Note re capturing up to editor since this obviously holds for every agreement – and not to do this anymore for future agreements. |
| Ericsson | Proposal 1.A: Support Proposal 1.B.1: We agree with previous comments that current spec may be sufficient and the proposal may not be needed Proposal 1.B.2: Support Proposal 1.C: As suggested by ZTE and Apple, it is better to resolve Proposal 1.A first before discussing this issue. Proposal 1.D: Support Proposal 1.E: Support |
| Fraunhofer IIS/HHI | Proposal 1.A: Not support. The current spec is in line with the agreements. Proposal 1.B.1: Fine Proposal 1.B.2: Fine Proposal 1.C: Not needed in our opinion. Proposal 1.E: Support |
| Mod V38 | No revision |
| Tejas | Proposal 1.A: We fail to see a misalignment between currently endorsed CRs and related agreements. As this is not a major issue, we do not support the proposal. Proposal 1.B.1: Open to discuss further. Proposal 1.B.2: We are ok to support the proposal for spec completion. We can be fine with Alt 1 or Alt 2, Alt 1 being our first preference due to its alignment with previous agreement and Alt 2 being second preference as it is more reasonably relevant. Proposal 1.C: Not support. Not needed in our opinion as it can be taken care of through network implementation. Proposal 1.D: We fail to see the necessity for the proposal and hence not ok. In our view, the CR captures the agreed relation and a text that additionally clarifies related part in the currently endorsed CR could be adequate for network to handle the configuration. |
| ETRI | Proposal 1.D: We are now fine with the updated proposal. |
| Sony | Proposal 1.B.1: We also believe that the current spec is sufficient and that this proposal is not needed. Proposal 1.B.2: Ok. Proposal 1.E: Support. |
| Lenovo/ MotM | Proposal 1.B.1/2: Support in principle the need to align the triggering offsets, whether with the first or second slot slightly prefer last slot for smaller interference measurement latency) Proposal 1.B.2: We are ok to support the proposal for spec completion. We can be fine with Alt 1 or Alt 2, Alt 1 being our first preference due to its alignment with previous agreement and Alt 2 being second preference as it is more reasonably relevant. Proposal 1.C: Prefer Nokia’s alternative Proposal 1.C in comments. Better to discuss in offline session first. Proposal 1.E: Seems like a conclusion to me and the spec impact of that is not clear. Would the moderator mind to clarify? [Mod: The fallback to Rel-15 Type-I when the number of ports <48 is not yet agreed.] |
| Final | No revision |

### Issue 2 (WID objective 2c): CRI-based CSI for hybrid beamforming (HBF)

Table 2A Summary: issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| New proposals |  |  |
|  | -- |  |
| Proposals from previous more recent meeting(s) and/or round(s) |  |  |
| 2.2 | Proposal 2.B: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order of NRep CSI reports, - For reports with non-M CRI based CSI reports (Legacy CSI reports till Rel-18) should have higher priority over M CRIs based CSI reports. - For reports configured with MR resources should have higher priority over reports with non-MR based CSI reports. - For reports with lower M values should have higher priority over higher M CRIs based CSI reports. PriiCSIy,k,c,s,m,M, MR=2∙Ncells∙Ms∙Mm∙y+Ncells∙Ms∙Mm∙k +Ms∙Mm∙c+s+Ms∙(M-MR)∙m where - m = 0 for non-M CRI based CSI reports. Mm= 1 (legacy CSI reports up to Rel-18), - m = 1 for M CRI based CSI reports. Mm is the maximum number of CRIs configured for multi-CRI CSI reports not carrying L1-RSRP or L1-SINR Note: It is up to the editor how this is captured in the specification FL assessment: This proposal is from RAN1#120 round-1. But since the main proponent was not present in Wuhan due to visa issues, the final decision for this proposal is deferred to RAN1#121 Malta. This proposal seems aligned with the legacy principle and is technically sound. | Support/fine: Tejas, Samsung, Huawei/HiSi (open), MediaTek (open), Spreadtrum (open), Apple (open), IDC, CMCC, Not support: Google, Apple, ZTE, ETRI, Fujitsu, Lenovo/MotM, OPPO, Ericsson (concern), Nokia/NSB (concern), NTT DOCOMO, NTT CORP, TCL, HONOR, NEC, |
| 2.3 | [116bis] Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, the M CRIs (each with log2KS bits) are separated indicated [118] Agreement At least for type 1 single panel, for the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-2, other than priority 0 (G0), the UCI packing order and the 2M consecutive priority levels are defined w.r.t the M CRIs (including the non-reported MR CRIs) as follows: - Consecutive priority levels (higher to lower): - Even sub-bands (G1) associated with the first configured CMR among the non-reported MR CRIs, - Odd sub-bands (G2) associated with the first configured CMR among the non-reported MR CRIs, - …, - Even sub-bands (G1) associated with the last configured CMR among the non-reported MR CRIs, - Odd sub-bands (G2) associated with the last configured CMR among the non-reported MR CRIs, - Even sub-bands (G1) of the 1st reported CRI, - Odd sub-bands (G2) of the 1st reported CRI, - …, - Even sub-bands (G1) of the (M-MR)th reported CRI, - Odd sub-bands (G2) of the (M-MR)th reported CRI. Note: For UCI fields in wideband (G0), even sub-bands (G1), and odd sub-bands (G2), legacy design is fully reused. {Similar agreement for Type-II} Proposal 2.C: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-2, support the following method for assigning priority levels (from higher to lower) to multiple CRIs in a single CSI instance. - For the non-reported MR CRIs, the priority order (from higher to lower) is assigned based on the RRC configured order of MR. - For the reported M-MR CRIs (or M CRIs if MR is not configured), the priority order (from higher to lower) is assigned based on the beam quality measure and not based on the RRC configured order of resource config IDs. Note: It is up to the editor how this is captured in the specification FL assessment: This proposal is from RAN1#120 round-1. But since the main proponent was not present in Wuhan due to visa issues, the final decision for this proposal is deferred to RAN1#121 Malta. It is unclear why this proposal is necessary with the existing agreement as well as the already agreed priority order. Based on the priority order, the UE can determine the UCI packing order across resources. In addition, what “beam quality measure” entails is ambiguous. | Support/fine: Tejas, Huawei/HiSi (open), IDC, Fujitsu (open), Spreadtrum, Not support: Samsung, Google, Apple, ZTE, ETRI, CMCC, Lenovo/MotM, Ericsson, OPPO, Nokia/NSB, NTT DOCOMO, NTT CORP, TCL, HONOR, NEC, MediaTek, |

Table 2B SLS results: issue 2

--

Table 2C Additional inputs: issue 2

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 1A |
| Google | Proposal 2.B/C: We failed to see the necessity for the two proposals. |
| NTT DOCOMO | Proposal 2.B/C: Our view remains unchanged. |
| Samsung | Our views have not been changed. |
| InterDigital | Proposal 2.B Support Proposal 2.C Support |
| Mod V7 | No revision |
| MediaTek | Proposal 2.C: No essential |
| Mod V12 | No revision |
| ZTE | Proposal 2.B/2.C: Our views are not changed. |
| Mod V20 | No revision |
| ETRI | Proposal 2.B/2.C: Our position remains unchanged. |
| Mod V35 | No revision |
| Tejas | Proposal 2.B We think this enhancement is needed. The enhancement of M CRI reporting was introduced in view of the performance gain it offers, yet being aware of the large overhead it creates. We should not disregard the possibility of such a large M, performance enhancing report congesting single (or small) CRI reports that the network essentially configures for regular and minimal beam management. As UCI omission is designed for emergency purposes, when UL RA is not sufficient, in our view, the new rule can efficiently manage this with minimal n/w effort and enhances the whole multi-CRI implementation. We prefer that the issue be taken care in spec using the priority rule for Rel 19 multi-CRI reporting, instead of leaving it to networks discretion through additional book-keeping to manage the report configuration IDs. A few reasons are; - Network taking care of prioritizing could create an overhead on gNBs with heavy traffic load and dynamic channel learning requirements. - Also, Network taking care of prioritizing could have been employed in the legacy systems as well for various cases. i,e., for AP, SP and P CSI reports’ reportconfigid assignment, for primary and secondary cells' reportconfigid assignment or for L1-RSRP, L1-SINR and non- L1-RSRP, L1-SINR reports' reportconfigid assignment. Yet, the current priority rule helps to handle these aspects very efficiently, irrespective of the reportconfigids (with minimal network intervention). By efficiently modifying the current rule in Rel 19 to apply for multi-CRI reports, we need not entrust this additional responsibility on gNB. Also, because legacy does not leave it to network implementation now. We support the modifications as the proposal is an incremental change, even not requiring too many complex conditions and can still stay with Rel 18 priority being backward compatible. Proposal 2.H: We are open to discuss the proposal for progress. |
| Lenovo/ MotM | Proposal 2.B/2.C: Our position remains unchanged |
| Final | No revision |

### Issue 3 (WID objective 3): CJT calibration reporting for non-ideal synchronization and backhaul

Table 3A Summary: issue 3

| # | Issue | Companies’ views |
| --- | --- | --- |
| New proposals |  |  |
| 3.1 | Proposal 3.A: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-Dd’ (delay offset), add the following definition of the measurement quantity in TS38.215 as follows (analogous to the Rel-18 TDCP): - For FR1, the reference point shall be the antenna connector of the UE. FL assessment: The above was already captured in TS38.215 but put in brackets awaiting RAN1 decision (agreement or conclusion of no support). The text was copied from Rel-18 TDCP definition. Whether this is needed or not for CJTC report is unclear especially for FR2. | Support/fine: Google, Samsung, Spreadtrum, OPPO, CATT, ZTE, Sanechips, Xiaomi, Apple, Qualcomm, vivo, ETRI, Fujitsu, Huawei/HiSi, KDDI, Rakuten, Ericsson, Lenovo/MotM, Not support: |
| Proposals from previous more recent meeting(s) and/or round(s) |  |  |
|  | -- |  |

Table 3B LLS/SLS results: issue 3

--

Table 3C Additional inputs: issue 3

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 3A |
| Google | Support. This is similar to other metrics. Without this, the reference point for the measurement is unclear, and it would cause issues for test. If we agree the CJT is an FR1 only feature, we can remove FR2 part. But the enhanced TCI indication for CJT already implies this is also applicable for FR2. [Mod: There wont e any restriction in RAN1 spec on the use of the TCI enhancements for FR2] In addition, it seems one of our TPs is not captured. We pasted it below: Proposal 2: Endorse the following TP for 38.214 - Reason for change: UE should not receive the CSI-RS for PO report during cell DTX inactive period, since it is a CSI-RS for CSI acquisition - Summary of change: Clarify that UE does not receive the CSI-RS for PO report during cell DTX inactive period - Consequence without the change: UE will receive the CSI-RS for PO report during cell DTX inactive period by mistake ------------------------------ start of TP for section 5.1.6.1 -------------------------- During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least ‘RI’ or ‘cjtc-P’. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least ‘RI’ or ‘cjtc-P’. ------------------------------ start of TP for section 5.1.6.1 -------------------------- [Mod: Per Chairman’s instruction, MIMO FLs are not allowed to discuss maintenance TPs in this meeting. Please bring this up in RAN1#122] |
| Samsung | Proposal 3.A As clearly stated in WID: - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH. we are targeting CJTC for only FR1. Hence, the text for FR2 should be removed. We can be OK with that the text for FR1 is reused for CJTC report quantity. |
| Mod V7 | Removed FR2 part from proposal 3.A |
| Spreadtrum | Proposal 3.A: Support the FR1 part. |
| Mod V12 | No revision |
| OPPO | Proposal 3.A: Fine. |
| CATT | Proposal 3.A: OK |
| ZTE | Proposal 3.A: Although only FR1 is mentioned in the WID, the CJT-C report could be also beneficial for FR2. So, we suggest to keep both the FR1 and FR2 parts. [Mod: The FR2 part is not agreeable to companies so I will not add it here. If you think it is important, please raise it in RAN4 since for TDCP this text actually came from RAN4] |
| Xiaomi | Proposal 3.A: support the definition of reference point for FR1. |
| Apple | Proposal 3.A: okay |
| Mod V20 | No revision |
| Qualcomm | Proposal 3.A: OK |
| vivo | - A OK |
| ETRI | Proposal 3.A: Fine |
| Fujitsu | Proposal 3.A: OK |
| Huawei, HiSilicon | Proposal 3.A: OK. |
| KDDI | Proposal 3.A: OK. |
| Rakuten | Proposal 3.A: Support |
| Mod V35 | No revision |
| Ericsson | Proposal 3.A: Ok |
| Mod V38 | No revision |
| Sony | Proposal 3.A: Ok |
| Lenovo/ MotM | Proposal 3.A: Fine |
| Final | No revision |

# References

| 1 | R1-2503276 | On 128 CSI-RS ports and UE reporting enhancement | Huawei, HiSilicon |
| --- | --- | --- | --- |
| 2 | R1-2503352 | Remaining issues on Rel-19 CSI enhancements | vivo |
| 3 | R1-2503434 | Remaining Details of Rel-19 Enhancements of CSI | InterDigital, Inc. |
| 4 | R1-2503510 | Remaining issues on CSI enhancements | Spreadtrum, UNISOC |
| 5 | R1-2503556 | Views on Rel-19 CSI enhancements | Samsung |
| 6 | R1-2503669 | Discussion on CSI enhancements | ZTE Corporation, Sanechips |
| 7 | R1-2503711 | Discussion on Rel-19 CSI Enhancements | Tejas Network Limited |
| 8 | R1-2503787 | Remaining issues on Rel-19 CSI enhancements | CATT |
| 9 | R1-2503825 | Discussion on CSI enhancements | CMCC |
| 10 | R1-2503860 | Discussion on the Remaining Issues of CSI Enhancement | Rakuten Mobile, Inc |
| 11 | R1-2503877 | Further discussion on the remained issues for CSI enhancement | Xiaomi |
| 12 | R1-2503910 | CSI enhancements for Rel. 19 MIMO | Fraunhofer IIS, Fraunhofer HHI |
| 13 | R1-2503913 | Discussion on CSI enhancements | Lenovo |
| 14 | R1-2503917 | Discussion on CSI enhancements | NEC |
| 15 | R1-2504028 | CSI Enhancement for NR MIMO | Google |
| 16 | R1-2504062 | Discussion of CSI enhancements | Sony |
| 17 | R1-2504077 | CSI enhancement for NR MIMO Phase 5 | Nokia |
| 18 | R1-2504085 | Remaining issues on Rel-19 CSI enhancements | Fujitsu |
| 19 | R1-2504096 | Discussion on CSI enhancements | HONOR |
| 20 | R1-2504134 | Discussion on CSI enhancements for NR MIMO Phase 5 | ETRI |
| 21 | R1-2504229 | CSI enhancements for Rel-19 MIMO | OPPO |
| 22 | R1-2504389 | Maintenance on CSI for >32 ports and UE-assisted CJT | Qualcomm Incorporated |
| 23 | R1-2504445 | CSI enhancements | Sharp |
| 24 | R1-2504451 | Maintenance on CSI enhancements for large antenna arrays and CJT | Ericsson |
| 25 | R1-2504496 | Discussion on CSI enhancements | NTT DOCOMO, INC. |
| 26 | R1-2504549 | Discussion on CSI enhancements for NR MIMO Phase 5 | KDDI Corporation |