3GPP TSG RAN WG1 #121                                         R1-2503557

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.2.3

Source: 	Samsung

Title: 	Views on Rel-19 3-antenna-port codebook-based transmissions

Document for:	Discussion and Decision

Introduction

The Rel-19 WID which has been finally revised in RAN#105 [1] includes the following objectives:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

This contribution provides Samsung’s view regarding the Rel-19 3TX highlighted above.

Discussion

  - Text proposal on TS38.212

In RAN1#120bis [2], we agreed to re-use an RRC parameter, which was already defined for disabling port 1003 of 4-port SRS resource(s) in an SRS resource set with usage of ‘codebook’ or ‘antennaSwitching’, in order to indicate 3-port non-codebook based PUSCH transmission. This is because, such RRC parameter is needed at least for distinguishing UE behavior for PTRS-DMRS association between Rel-18/19, which is already captured in the agreement as follows:

Agreement in RAN1#120bis For a 3TX UE, when configured to ‘nonCodebook’, to distinguish UE behavior for PTRS-DMRS association between the Rel-19 and Rel-18 behavior, - Do not introduce a new RRC parameter, but enhance the definition of the RRC parameter 4portSRS_3Tx, for example, - ‘1-bit RRC parameter to indicate that port 1003 is disabled for all the 4-port resources in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, or to indicate 3-port transmission is enabled for a configured SRS resource set with usage ‘nonCodebook’.

Hence, as for the purpose of the re-used RRC parameter, we think that the corresponding description for the RRC parameter shall be captured in the relevant RAN1 specification, especially for UE behavior on PTRS-DMRS association. In that sense, we would like to add “when 4portSRS_3Tx is configured”, because the RRC parameter can be used for both of codebook and non-codebook based PUSCH, as suggested in the following text proposal in TS38.212.

Proposal 1. Adopt the following text proposal expressed by yellow highlight in Clause 7.3.1.1.2 in TS38.212.

< Unchanged parts are omitted > - PTRS-DMRS association - number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxMIMO-Layers=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxRank=1 and maxRankSFN=1, or if maxMIMO-Layers=1 and maxMIMO-LayersforSFN=1, or if maxRank=1 and maxRankSDM=1 when two PTRS ports are configured by maxNrofPorts-SDM, or if maxMIMO-Layers=1 and maxMIMO-LayersforSDM=1 when two PTRS ports are configured by maxNrofPorts-SDM; - 1 or 2 or 4 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26/7.3.1.1.2-26A/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", and maxRank<=4 or maxMIMO-Layers<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or 4 or maxMIMO-Layers=3 or 4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank=2 or maxMIMO-Layers=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRank<=3 or maxMIMO-Layers<=3, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25. - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRank<=3 or maxMIMO-Layers<=3, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or maxMIMO-Layers=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRank=2 or maxMIMO-Layers=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port and DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRank<=3 or maxMIMO-Layers<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B. - 2 bits when two PTRS ports are configured by maxNrofPorts-SDM in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 2 bits when one PTRS port is configured by maxNrofPorts-SDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 or maxMIMO-Layers>4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port(s) corresponding to the selected codeword according to Table 7.3.1.1.2-25B, where the selected codeword is the codeword with higher MCS for the initial PUSCH if the MCS indices of the two codewords are different for the initial PUSCH, or codeword 0 otherwise. - 4 bits when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 or maxMIMO-Layers>4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-26A. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. When the Transform precoder indicator field is present, if the bit width of PTRS-DMRS association field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the PTRS-DMRS association field for the case with smaller bit width until the bit width of the PTRS-DMRS association field for the two cases are the same. - Second PTRS-DMRS association - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank>2 or maxMIMO-Layers>2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or maxMIMO-Layers=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank<=3 or maxMIMO-Layers<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 0 bit otherwise. Tables 7.3.1.1.2-25 and 7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. < Unchanged parts are omitted >

Proposal 2. Adopt the following text proposal expressed by yellow highlight in Clause 7.3.1.1.3 in TS38.212.

< Unchanged parts are omitted > - PTRS-DMRS association - number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxMIMO-LayersDCI-0-2=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxRankDCI-0-2=1 and maxRankSFN-DCI-0-2=1, or if maxMIMO-LayersDCI-0-2=1 and maxMIMO-LayersforSFN-DCI-0-2=1, or if maxRankDCI-0-2=1 and maxRankSDM-DCI-0-2=1 when two PTRS ports are configured by maxNrofPorts-SDM, or if maxMIMO-LayersDCI-0-2=1 and maxMIMO-LayersforSDM-DCI-0-2=1 when two PTRS ports are configured by maxNrofPorts-SDM; - 1 or 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or “01” and maxRankDCI-0-2<=4 or maxMIMO-LayersDCI-0-2<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or “11”, maxRankDCI-0-2=3 or 4 or maxMIMO-LayersDCI-0-2=3 or 4 and neither multipanelSchemeSDM nor multipanelSchemeSFN is not configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11" and maxRankDCI-0-2=2 or maxMIMO-LayersDCI-0-2=2 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25. - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=3 or maxMIMO-LayersDCI-0-2=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=2 or maxMIMO-LayersDCI-0-2=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port and DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B. - 2 bits when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 2 bits when one PTRS port is configured by maxNrofPorts-SDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. When the Transform precoder indicator field is present, if the bit width of PTRS-DMRS association field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the PTRS-DMRS association field for the case with smaller bit width until the bit width of the PTRS-DMRS association field for the two cases are the same. - Second PTRS-DMRS association - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2>2 or maxMIMO-LayersDCI-0-2>2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=3 or maxMIMO-LayersDCI-0-2=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 0 bit otherwise. Tables 7.3.1.1.2-25 and 7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. < Unchanged parts are omitted >

Conclusion

In this contribution, the following observation and proposals are given:

Proposal 1. Adopt the following text proposal expressed by yellow highlight in Clause 7.3.1.1.2 in TS38.212.

< Unchanged parts are omitted > - PTRS-DMRS association - number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxMIMO-Layers=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxRank=1 and maxRankSFN=1, or if maxMIMO-Layers=1 and maxMIMO-LayersforSFN=1, or if maxRank=1 and maxRankSDM=1 when two PTRS ports are configured by maxNrofPorts-SDM, or if maxMIMO-Layers=1 and maxMIMO-LayersforSDM=1 when two PTRS ports are configured by maxNrofPorts-SDM; - 1 or 2 or 4 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26/7.3.1.1.2-26A/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", and maxRank<=4 or maxMIMO-Layers<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or 4 or maxMIMO-Layers=3 or 4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank=2 or maxMIMO-Layers=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRank<=3 or maxMIMO-Layers<=3, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25. - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRank<=3 or maxMIMO-Layers<=3, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or maxMIMO-Layers=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRank=2 or maxMIMO-Layers=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port and DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRank<=3 or maxMIMO-Layers<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B. - 2 bits when two PTRS ports are configured by maxNrofPorts-SDM in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 2 bits when one PTRS port is configured by maxNrofPorts-SDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 or maxMIMO-Layers>4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port(s) corresponding to the selected codeword according to Table 7.3.1.1.2-25B, where the selected codeword is the codeword with higher MCS for the initial PUSCH if the MCS indices of the two codewords are different for the initial PUSCH, or codeword 0 otherwise. - 4 bits when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 or maxMIMO-Layers>4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-26A. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. When the Transform precoder indicator field is present, if the bit width of PTRS-DMRS association field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the PTRS-DMRS association field for the case with smaller bit width until the bit width of the PTRS-DMRS association field for the two cases are the same. - Second PTRS-DMRS association - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank>2 or maxMIMO-Layers>2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or maxMIMO-Layers=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank<=3 or maxMIMO-Layers<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 0 bit otherwise. Tables 7.3.1.1.2-25 and 7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. < Unchanged parts are omitted >

Proposal 2. Adopt the following text proposal expressed by yellow highlight in Clause 7.3.1.1.3 in TS38.212.

< Unchanged parts are omitted > - PTRS-DMRS association - number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxMIMO-LayersDCI-0-2=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxRankDCI-0-2=1 and maxRankSFN-DCI-0-2=1, or if maxMIMO-LayersDCI-0-2=1 and maxMIMO-LayersforSFN-DCI-0-2=1, or if maxRankDCI-0-2=1 and maxRankSDM-DCI-0-2=1 when two PTRS ports are configured by maxNrofPorts-SDM, or if maxMIMO-LayersDCI-0-2=1 and maxMIMO-LayersforSDM-DCI-0-2=1 when two PTRS ports are configured by maxNrofPorts-SDM; - 1 or 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or “01” and maxRankDCI-0-2<=4 or maxMIMO-LayersDCI-0-2<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or “11”, maxRankDCI-0-2=3 or 4 or maxMIMO-LayersDCI-0-2=3 or 4 and neither multipanelSchemeSDM nor multipanelSchemeSFN is not configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11" and maxRankDCI-0-2=2 or maxMIMO-LayersDCI-0-2=2 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25. - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=3 or maxMIMO-LayersDCI-0-2=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25. - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=2 or maxMIMO-LayersDCI-0-2=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port and DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B. - 2 bits when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. - 2 bits when one PTRS port is configured by maxNrofPorts-SDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25. - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. When the Transform precoder indicator field is present, if the bit width of PTRS-DMRS association field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the PTRS-DMRS association field for the case with smaller bit width until the bit width of the PTRS-DMRS association field for the two cases are the same. - Second PTRS-DMRS association - 2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2>2 or maxMIMO-LayersDCI-0-2>2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=3 or maxMIMO-LayersDCI-0-2=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports when 4portSRS_3Tx is configured, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; - 0 bit otherwise. Tables 7.3.1.1.2-25 and 7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. < Unchanged parts are omitted >

References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105.

- Chair’s note, RAN1#120bis.