3GPP TSG RAN WG1 #121			R1-2503562

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.2

Source: 	Samsung

Title: 	Draft Stage-2 CR for Rel-19 NR MIMO Ph5 in TS38.300

Document for:	Discussion and Decision

Introduction

This contribution provides Samsung’s view on reflecting Rel-19 MIMO functionality into stage-2 specification (TS38.300) based on the outcome of normative work until RAN1#120bis and reflecting the initial discussion on the draft CR [1].

Discussion

The scope of Rel-19 MIMO is described in [2] including UE-initiated/event-driven beam management (UEI-ED/BM), CSI enhancement up to 128 CSI-RS ports and coherent joint transmission (CJT) calibration reporting, UL 3TX enhancement, and enhancement on asymmetric DL single-TRP and UL multi-TRP scenario. Considering these aspects, we would like to suggest the following draft CR on TS38.300.

Proposal. Support the following CR for describing an enhancement on UEI-ED/BM, asymmetric DL single-TRP and UL multi-TRP scenario, and CJT calibration reporting adopted in Rel-19 MIMO.

6.12 Multiple Transmit/Receive Point Operation In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from two TRPs, providing better coverage, reliability and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH. There are two different operation modes to schedule multi-TRP PDSCH transmissions: single-DCI and multi-DCI. For both modes, control of uplink and downlink operation can be done by physical layer and MAC layer, within the configuration provided by the RRC layer. In single-DCI mode, the UE is scheduled by the same DCI for both TRPs and in multi-DCI mode, the UE is scheduled by independent DCIs from each TRP. There are two different operation modes for multi-TRP PDCCH: PDCCH repetition as in Clause 5.2.3 and Single Frequency Network (SFN) based PDCCH transmission. In both modes, the UE can receive two PDCCH transmissions, one from each TRP, carrying the same DCI. In PDCCH repetition mode, the UE can receive the two PDCCH transmissions carrying the same DCI from two linked search spaces each associated with a different CORESET. In SFN based PDCCH transmission mode, the UE can receive the two PDCCH transmissions carrying the same DCI from a single search space/CORESET using different TCI states. For multi-TRP PUSCH repetition, according to indications in a single DCI or in a semi-static configured grant provided over RRC, the UE performs PUSCH transmission of the same contents toward two TRPs with corresponding beam directions associated with different spatial relations. For multi-TRP PUCCH repetition, the UE performs PUCCH transmission of the same contents toward two TRPs with corresponding beam directions associated with different spatial relations. For inter-cell multi-TRP operation, for multi-DCI PDSCH transmission, one or more TCI states can be associated with SSB with a PCI different from the serving cell PCI. The activated TCI states can be associated with at most one PCI different from the serving cell PCI at a time. For inter-cell and intra-cell multi-DCI multi-TRP operation, up to two TAGs with associated TAG IDs can be configured per serving cell. Each UL/Joint TCI state is associated with a TAG ID and the UE applies the timing advance of the TAG ID associated with the UL/joint TCI state utilized for UL transmission. For single-DCI multi-TRP Simultaneous Transmission with Multi-Panel (STxMP) Spatial Domain Multiplexing (SDM) PUSCH transmission, different layers of one PUSCH are separately transmitted towards two TRPs. For single-DCI multi-TRP STxMP SFN PUSCH transmission, same layers of one PUSCH are transmitted towards two TRPs. For multi-DCI based multi-TRP STxMP PUSCH+PUSCH transmission, two PUSCHs are transmitted towards two TRPs. For single-DCI multi-TRP STxMP SFN PUCCH transmission, one PUCCH is transmitted towards two TRPs. For inter-TRP delay, frequency, and phase calibration for coherent joint transmission (CJT), CSI-RS per TRP is transmitted to the UE to measure the CSI-RS and estimate the reporting quantity including inter-TRP delay offset, frequency offset, and phase offset. A CSI report associated with the reporting quantity is carried on a PUSCH. For asymmetric DL single-TRP and UL multi-TRP scenario where UL TRP may reduce or even turn off DL transmission, pathloss offset can be indicated by TCI state for PUSCH, PUCCH, and SRS transmission, and can be indicated by a PDCCH order for the PDCCH order triggered PRACH toward UL TRP facilitating pathloss calculation. In addition, up to two closed loop power control adjustment states can be supported for SRS separate with PUSCH. Also, up to two TAGs can be configured in the serving cell, and a TCI state for PUSCH, PUCCH, and SRS transmission can be associated with any one of two TAGs. <unchanged parts are omitted> 9.2.3 Mobility in RRC_CONNECTED 9.2.3.1 Overview <unchanged parts are omitted> Beam Level Mobility does not require explicit RRC signalling to be triggered. Beam level mobility can be within a cell, or between cells, the latter is referred to as inter-cell beam management (ICBM). For ICBM, a UE can receive or transmit UE dedicated channels/signals via a TRP associated with a PCI different from the PCI of a serving cell, while non-UE-dedicated channels/signals can only be received via a TRP associated with a PCI of the serving cell. The gNB provides via RRC signalling the UE with measurement configuration containing configurations of SSB/CSI resources and resource sets, reports and trigger states for triggering channel and interference measurements and reports. In case of ICBM, a measurement configuration includes SSB resources associated with PCIs different from the PCI of a serving cell. Beam Level Mobility is then dealt with at lower layers by means of physical layer and MAC layer control signalling, and RRC is not required to know which beam is being used at a given point in time. SSB-based Beam Level Mobility is based on the CD-SSB associated to the initial DL BWP and can be configured for the initial DL BWPs, for DL BWPs containing the CD-SSB associated to the initial DL BWP, and if supported, for DL BWPs not containing the CD-SSB associated to the initial DL BWP. SSB-based Beam Level Mobility can be also performed based on an NCD-SSB, if configured for the active DL BWP. Beam Level Mobility can be also performed based on CSI-RS, if configured for the active DL BWP. For UE-initiated beam reporting, both Mode A and Mode B are supported, where upon determination of an event, they transmit a one-bit indication in a PUCCH resource to indicate a beam report. In Mode A, the UE transmits the beam report on a PUSCH indicated by a DCI in response to the one-bit indication, while in Mode B, the UE transmits the beam report on a type1 CG PUSCH associated to the PUCCH resource.

Conclusion

In this contribution, the following proposal is given:

Proposal. Support the following CR for describing an enhancement on UEI-ED/BM, asymmetric DL single-TRP and UL multi-TRP scenario, and CJT calibration reporting adopted in Rel-19 MIMO.

6.12 Multiple Transmit/Receive Point Operation In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from two TRPs, providing better coverage, reliability and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH. There are two different operation modes to schedule multi-TRP PDSCH transmissions: single-DCI and multi-DCI. For both modes, control of uplink and downlink operation can be done by physical layer and MAC layer, within the configuration provided by the RRC layer. In single-DCI mode, the UE is scheduled by the same DCI for both TRPs and in multi-DCI mode, the UE is scheduled by independent DCIs from each TRP. There are two different operation modes for multi-TRP PDCCH: PDCCH repetition as in Clause 5.2.3 and Single Frequency Network (SFN) based PDCCH transmission. In both modes, the UE can receive two PDCCH transmissions, one from each TRP, carrying the same DCI. In PDCCH repetition mode, the UE can receive the two PDCCH transmissions carrying the same DCI from two linked search spaces each associated with a different CORESET. In SFN based PDCCH transmission mode, the UE can receive the two PDCCH transmissions carrying the same DCI from a single search space/CORESET using different TCI states. For multi-TRP PUSCH repetition, according to indications in a single DCI or in a semi-static configured grant provided over RRC, the UE performs PUSCH transmission of the same contents toward two TRPs with corresponding beam directions associated with different spatial relations. For multi-TRP PUCCH repetition, the UE performs PUCCH transmission of the same contents toward two TRPs with corresponding beam directions associated with different spatial relations. For inter-cell multi-TRP operation, for multi-DCI PDSCH transmission, one or more TCI states can be associated with SSB with a PCI different from the serving cell PCI. The activated TCI states can be associated with at most one PCI different from the serving cell PCI at a time. For inter-cell and intra-cell multi-DCI multi-TRP operation, up to two TAGs with associated TAG IDs can be configured per serving cell. Each UL/Joint TCI state is associated with a TAG ID and the UE applies the timing advance of the TAG ID associated with the UL/joint TCI state utilized for UL transmission. For single-DCI multi-TRP Simultaneous Transmission with Multi-Panel (STxMP) Spatial Domain Multiplexing (SDM) PUSCH transmission, different layers of one PUSCH are separately transmitted towards two TRPs. For single-DCI multi-TRP STxMP SFN PUSCH transmission, same layers of one PUSCH are transmitted towards two TRPs. For multi-DCI based multi-TRP STxMP PUSCH+PUSCH transmission, two PUSCHs are transmitted towards two TRPs. For single-DCI multi-TRP STxMP SFN PUCCH transmission, one PUCCH is transmitted towards two TRPs. For inter-TRP delay, frequency, and phase calibration for coherent joint transmission (CJT), CSI-RS per TRP is transmitted to the UE to measure the CSI-RS and estimate the reporting quantity including inter-TRP delay offset, frequency offset, and phase offset. A CSI report associated with the reporting quantity is carried on a PUSCH. For asymmetric DL single-TRP and UL multi-TRP scenario where UL TRP may reduce or even turn off DL transmission, pathloss offset can be indicated by TCI state for PUSCH, PUCCH, and SRS transmission, and can be indicated by a PDCCH order for the PDCCH order triggered PRACH toward UL TRP facilitating pathloss calculation. In addition, up to two closed loop power control adjustment states can be supported for SRS separate with PUSCH. Also, up to two TAGs can be configured in the serving cell, and a TCI state for PUSCH, PUCCH, and SRS transmission can be associated with any one of two TAGs. <unchanged parts are omitted> 9.2.3 Mobility in RRC_CONNECTED 9.2.3.1 Overview <unchanged parts are omitted> Beam Level Mobility does not require explicit RRC signalling to be triggered. Beam level mobility can be within a cell, or between cells, the latter is referred to as inter-cell beam management (ICBM). For ICBM, a UE can receive or transmit UE dedicated channels/signals via a TRP associated with a PCI different from the PCI of a serving cell, while non-UE-dedicated channels/signals can only be received via a TRP associated with a PCI of the serving cell. The gNB provides via RRC signalling the UE with measurement configuration containing configurations of SSB/CSI resources and resource sets, reports and trigger states for triggering channel and interference measurements and reports. In case of ICBM, a measurement configuration includes SSB resources associated with PCIs different from the PCI of a serving cell. Beam Level Mobility is then dealt with at lower layers by means of physical layer and MAC layer control signalling, and RRC is not required to know which beam is being used at a given point in time. SSB-based Beam Level Mobility is based on the CD-SSB associated to the initial DL BWP and can be configured for the initial DL BWPs, for DL BWPs containing the CD-SSB associated to the initial DL BWP, and if supported, for DL BWPs not containing the CD-SSB associated to the initial DL BWP. SSB-based Beam Level Mobility can be also performed based on an NCD-SSB, if configured for the active DL BWP. Beam Level Mobility can be also performed based on CSI-RS, if configured for the active DL BWP. For UE-initiated beam reporting, both Mode A and Mode B are supported, where upon determination of an event, they transmit a one-bit indication in a PUCCH resource to indicate a beam report. In Mode A, the UE transmits the beam report on a PUSCH indicated by a DCI in response to the one-bit indication, while in Mode B, the UE transmits the beam report on a type1 CG PUSCH associated to the PUCCH resource.

References

- R1-2503093, Summary of discussion on Draft CR on TS38.300 for Rel-19 MIMO, Samsung, RAN1#120bis.

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105.