3GPP TSG RAN WG1 #121	R1-2503564

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.3.2

Source:	Samsung

Title:	Random access on SBFD resources

Document for:	Discussion

# Introduction

The Rel-19 WID Evolution of Duplex operation introduces NR specification support for gNB-side SBFD within a TDD carrier, enhancements to inter-gNB and inter-UE CLI handling, and BS RF and RRM requirements [1].

In the Rel-19 WID, it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands.

SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or F symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

The revised Rel-19 WID objectives include the introduction of specification support for random access in SBFD symbols for RRC_CONNECTED mode and for RRC_IDLE/INACTIVE mode [1].

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

Several feature-level RAN1 agreements and conclusions for random access in SBFD symbols were made starting from RAN1#116. It was concluded that no new PRACH format are introduced in the Rel-19 SBFD WI. It was agreed that CBRA and CFRA in SBFD symbols for SBFD-aware UEs in RRC_CONNECTED are supported. It was agreed that for SBFD aware UEs in RRC CONNECTED mode, the Type-1 random access procedure is supported in SBFD symbols. It was agreed that the Rel-18 PRACH repetition feature is supported for random access in SBFD symbols at least within additional ROs. RAN1#120 concluded that there is no RAN1 consensus to support the Type-2 random access procedure (2-step RACH) in SBFD symbols.

In Section 2, we discuss some remaining details for RACH Msg.1 (preamble) transmission and the related L1 processing to specify support for random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode. In Section 3 we discuss some remaining details for RACH Msg.2 reception and RACH Msg.3 transmission for SBFD-aware UEs.

# RACH Msg.1 transmission

In this section, we discuss some remaining details for RACH Msg.1 (preamble) transmission for RACH configuration Option 2 and the related L1 processing to specify support for random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode.

In RAN1#120bis, the following agreement was made.

Agreement (RAN1#120bis)

For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure.

- FFS: Whether specification change is necessary

For the SBFD-aware UE, the selection of the RO type is performed before the selection of the set of random access resources, and the gNB can indicate via explicit signalling whether the RO is selected when the SSB RSRP are “below” or “above” the configured threshold, and an RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold, and the RO type can be signalled in cases such as CFRA as by the RAN2#129bis agreements and WA.

The principle is that any change to the implemented and tested UE behavior on legacy random access should be avoided. Therefore, there should not be any resulting specification impact.

No specification impact is necessary when for CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure.

For RACH configuration Option 2, the following agreements and conclusions were made.

Agreement (RAN1#118bis)

Confirm the following Working Assumption:

Working Assumption

For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration.

- FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs

Agreement (RAN1#119)

For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives:

- Alt-1: do not optimize this case

- Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

Conclusion (RAN1#120)

For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt-1:

- Alt-1: do not optimize for this case

An open design question for RACH configuration Option 2 is the association period and association pattern period for the additional ROs configured by the additional RACH configuration.

The following agreement was made in RAN1#119.

Agreement (RAN1#119)

For RACH configuration Option 2, the association period and association pattern period of additional-ROs are determined separately from legacy-ROs.

- The legacy rules for determining association period and association pattern period are reused in principle for additional-ROs.

In our view, the use of a different association period and association pattern period for the additional RACH configuration is clearly motivated in the case of the RACH configuration Option 2, i.e., more so than in the case of RACH configuration Option 1 where separate association period/pattern period were already agreed.

In Rel-15 NR, the association period is determined such that the UE maps the set of SSBs at least once to the valid ROs within a period given as integer multiple of the PRACH configuration period. For RACH configuration option 2, separate configuration of the association periods and association period patterns for the additional ROs and the legacy ROs are then required. The additional RACH configuration and the legacy PRACH configuration are typically expected to result in different PRACH periodicities due to need to configure different preamble formats. One motivation for RACH configuration Option 2 is use case (larger cell size/better UL coverage) and a long PRACH format such as F0/B4 is typically configured for the additional ROs. The number of additional ROs and their mapping to the UL subband of SBFD slots must then be chosen such that the SSB-RO mapping sequence results in a longer association period / association pattern period when compared to the legacy ROs.

One desirable property for the separately configured association period and association pattern period of the additional RACH configuration is that the configuration should allow for an integer multiple of the association period and association pattern period of the legacy RACH configuration. This simplifies the UE implementation because the SSB-RO mapping sequence can then be pre-computed and corresponds to the longer period of the additional RO.

Proposal 2: For RACH configuration Option 2, the association period and association pattern period of additional ROs are configured an integer multiple of the association period and association pattern period of the legacy ROs.

# Additional specification impacts

In this section, we discuss additional specification impacts related to random access on SBFD symbols such as RACH Msg.2 reception and RACH Msg.3 transmission for SBFD-aware UEs.

## RACH Msg.2 monitoring

Upon successful reception of a preamble, the gNB transmits RACH Msg.2 (RAR). A PDSCH carrying RAR can include information for one or more UEs, wherein some of the information is common to UEs and remaining information is UE-specific. The RAR payload depends on the RA cause. For example, during Initial Access in RRC_IDLE, the gNB applies the TC-RNTI and scheduled UL resources for RACH Msg.3. The RAR contains the RA-preamble identifier, TA, initial UL grant, and the temporary C-RNTI.

After the UE sends a preamble, it monitors the PDCCH and waits for an RA response within an RA response window. If the UE receives a response containing an RA-preamble identifier which is the same as the identifier contained in the transmitted RA preamble, the response is successful. The UE then transmits RACH Msg.3 using the FDRA from the RAR. If the UE does not receive a response within the RA response window or fails to verify the response, the response fails. In this case, if the number of RA attempts is smaller than the upper limit, the UE retries RA. Otherwise, RA fails

Monitoring and reception of the RACH Msg.2 (RAR) requires a PDCCH and PDSCH reception. The UE receives the RAR in the initial DL BWP for the case of initial access in RRC_IDLE or in the active DL BWP for other RA triggers. The SCS for a PDCCH reception scheduling a PDSCH with a RAR message is the SCS of a Type1-PDCCH common search space (CSS) set. The SCS for any subsequent PDCCH and PDSCH reception is also same as the SCS for the PDCCH/PDSCH providing the RAR unless the UE is configured a different SCS.

The UE monitors PDCCH for detection of a DCI format 1_0 scheduling a PDSCH providing a RAR during a configured time window according to the Type1-PDCCH CSS set of the PCell/SpCell identified by a RA-RNTI or, in the case of beam failure recovery (BFR) with CFRA in the search space indicated by recoverySearchSpaceId of the PCell/SpCell identified by the C-RNTI.

RAR can include a 4-bit backoff indicator (BI) that indicates a maximum back-off time needed before a next PRACH transmission attempt by a UE. The UE selects an actual back-off time uniformly at random between zero and the value indicated by the BI field. The BI is typically used to control loading of PRACH preamble transmissions on the serving cell. In response to an SI on-demand request from the UE, RAR can also include a RAPID that corresponds to a preamble ID on which the UE transmitted.

Which CORESET and search space configuration can be supported for random access in SBFD symbols will depend on the RA cause. For CBRA, RAR monitoring in response to preamble transmission in SBFD symbols can in principle share the common ra-searchspace configuration for random access in UL symbols. It is also possible to consider the use of UE-specific search spaces when possible such as in the case of CFRA. Similar considerations apply to the RNTI, i.e., RA-RNTI/msgB-RNTI and/or C-RNTI where possible should be mostly reused unless some necessary change is found.

In our view, legacy RACH Msg.2 reception procedures can be reused as is.

- Proposal 3: The existing RACH Msg.2 (RAR) reception procedure is reused for random access in SBFD symbols.

## RACH Msg.3 PUSCH and PUCCH for Msg.4

The following agreements or conclusions were made in RAN1#120 and RAN1#120bis.

Conclusion (RAN1#120)

For SBFD-aware UEs, there is no restriction on the combination of RO type of the latest PRACH transmission and the symbol type of following Msg3 PUSCH transmission or PUCCH carrying HARQ ACK for Msg4.

Agreement (RAN1#120)

For an SBFD aware UE, if additional-RO is selected for PRACH transmission, for Msg3 PUSCH repetition Configuration 1,

- In case the valid symbol type is SBFD symbol, a slot is determined as available if the symbols allocated for Msg3 PUSCH are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

- In case the valid symbol type is non-SBFD symbol, a slot is determined as available if the symbols allocated for Msg3 PUSCH are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

### Interpretation of FDRA for Msg.3

The following agreement was made in RAN1#120bis.

Agreement (RAN1#120bis)

Update the following agreement made in RAN1#118-bis in red:

Agreement

For SBFD aware UEs, for Msg3 PUSCH transmission in SBFD symbols, reuse Table 8.3-1 in TS 38.213 for determination of the frequency offset for the 2nd hop, with the update that the frequency offset for 2nd hop is based on the size of UL usable PRBs.

- FFS the case when the value of NUL,hop= 11

- FFS the definition of UL usable PRBs for Msg3 PUSCH transmission

From Section 8.3 of TS38.213, a UE defines a set of RBs for determining the FDRA interpretation for Msg.3 PUSCH. The definition of the set of RBs are shown below:

If useInterlacePUCCH-PUSCHis notprovided byBWP-UplinkCommonandBWP-UplinkDedicated, for determining the frequency domain resource allocation for the PUSCH transmission within the active UL BWP - If the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the initial UL BWP is used - else, the RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

In short, the set of RBs can be same as the initial UL BWP if the condition is met, otherwise {RB X, RB X+1,… RB X+NBWPsize-1} where RB X is the first RB of the active UL BWP and NBWPsize is the number of RBs in the initial UL BWP. Note that, both cases have NBWPsize RBs within the set of RBs. After that the UE interprets 14-bit FDRA in RAR UL grant based on the NBWPsize RBs.

In RAN1#119, RAN1 has been discussed on how to determine the RB numbering and the number of RBs for the interpretation of the FDRA for Msg.3 PUSCH when Msg.3 PUSCH is scheduled in the SBFD symbol. First, the existing rule does not work. This is because in some configurations, the set of RBs cannot include any or only a very limited number of RBs of the UL subband.

To address this issue, the following options were listed and discussed. No conclusion was reached in RAN1#119.

#### Updated proposal 3-1-2a: For determining the frequency domain resource allocation for Msg3 PUSCH transmission in SBFD symbols, consider the following alternatives: - Alt 1: the RB numbering starts from the first RB of the UL usable PRBs for Msg3 PUSCH transmission and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP. - Alt 2: the RB numbering starts from the first RB of the UL usable PRBs for Msg3 PUSCH transmission and the maximum number of RBs for frequency domain resource allocation equals the min {size of UL usable PRBs, number of RBs in the initial UL BWP}.

Based on the FL proposal 3-1-2a, the set of RBs can be (Alt 1) {RB Y, RB Y+1,… RB Y+NBWPsize-1} or (Alt 2) {RB Y, RB Y+1,…, RB Y+min⁡{NBWPsize,NUL_usabble_PRBssize}-1, where RB Y is the first RB of the UL usable BWP, NBWPsize is the number of RBs in the initial UL BWP and NUL_usabble_PRBssize is the number of RBs in the UL usable PRBs.

First of all, both alternatives can work well since both include all the RBs within the UL usable PRBs. The only difference is the number of RBs within the set of RBs.

We think that Alt 1 is more aligned with the existing rule (where the set of RBs always includes NBWPsize RBs and FDRA interpretation is based on the initial UL BWP of NBWPsize RBs).

If Alt 2 would be adopted, RAN1 would need to discuss the interpretation of the FDRA based on min⁡{NBWPsize,NUL_usabble_PRBssize} RBs which is results in additional specification effort. Also, the gNB only indicates the RBs within the UL usable RBs even for Alt 1. That is, even though the set of RBs {RB Y, RB Y+1,… RB Y+NBWPsize-1} for Alt 1 includes RBs outside of the UL usable PRBs { RB Y+min⁡{NBWPsize,NUL_usabble_PRBssize}, … RB Y+NBWPsize-1}, gNB will not indicate those of RBs.

- Proposal 4: For determining the frequency domain resource allocation for Msg.3 PUSCH transmission in SBFD symbols, the RB numbering starts from the first RB of the UL usable PRBs and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP.

### Power control for Msg.3 PUSCH

The following agreements were made in RAN1#120bis.

Agreement (RAN1#120bis)

For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2:

- preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols;

- preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols;

- FFS whether/how to support separate msg3-DeltaPreamble for non-SBFD symbols and SBFD-symbols.

Agreement (RAN1#120bis)

For SBFD-aware UEs, support separate configuration of msg3-Alpha for Msg3 PUSCH transmission on SBFD symbols. When separate msg3-Alpha for Msg3 PUSCH transmission on SBFD symbols is configured:

- msg3-Alpha configured for Msg3 PUSCH transmission on non-SBFD symbols is used if Msg3 PUSCH is transmitted in non-SBFD symbols;

- msg3-Alpha configured for Msg3 PUSCH transmission on SBFD symbols is used if Msg3 PUSCH is transmitted in SBFD symbols.

Agreement (RAN1#120bis)

For SBFD-aware UEs, support separate configuration of p0-nominal for PUCCH on SBFD symbols. When separate p0-nominal for PUCCH on SBFD symbols is configured:

- p0-nominal configured for PUCCH on non-SBFD symbols is used if PUCCH is transmitted in non-SBFD symbols;

p0-nominal configured for PUCCH on SBFD symbols is used if PUCCH is transmitted in SBFD symbols

The PUSCH transmission power is determined by

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ci,PO_PUSCH,b,f,cj+10log102μMRB,b,f,cPUSCHi+αb,f,cjPLb,f,cqd+ΔTF,b,f,ci+fb,f,c(i,l)

In case of Msg.3 PUSCH (re-)transmission corresponding to RAR UL grant, j=0 and l=0. The values of the parameters are shown in Table 1.

Table 1. Parameters and values applied for Msg.3 PUSCH transmission

| PO_PUSCH,b,f,c0 | =PO_NOMINAL_PUSCH,f,c(0)+PO_UE_PUSCH,b,f,c(0) - POUEPUSCH,b,f,c0=0 - PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 - PO_PRE = preambleReceivedTargetPower in RACH-ConfigGeneric - ΔPREAMBLE_Msg3 = msg3-DeltaPreamble, (-1..6), 0 by default |
| --- | --- |
| αb,f,c0 | =1 if not configured |
| PLb,f,cqd | referenceSignalPower – higher layer filtered RSRP - referenceSignalPower = ss-PBCH-BlockPower - For higher layer filtered RSRP computation, SSB used to obtain MIB |
| ΔTF,b,f,ci | =0 |
| fb,f,c(i,0) | fb,f,c0,0=ΔPrampup,b,f,c+δmsg2,b,f,c - δmsg2,b,f,c: a TPC command value indicated in RAR grant - ΔPrampup,b,f,c=minmax0,PCMAX,f,c-10log10(2μMRB,b,f,cPUSCH0)+PO_PUSCH,b,f,c0+αb,f,c0PLc+ΔTF,b,f,c0+δmsg2,b,f,c,ΔPrampuprequested,b,f,c - ΔPrampuprequested,b,f,c: the total power ramp-up requested by higher layer of ΔPrampup,b,f,c, ΔPrampuprequested,b,f,c= (PREAMBLE_POWER_RAMPING_COUNTER-1)× PREAMBLE_POWER_RAMPING_STEP |

When a UE determines the initial closed loop power control value for Msg.3 PUSCH, fb,f,c0,0, the PRACH preamble power ramping, ΔPrampup,b,f,c, must be further considered.

More specifically, the UE can use up to ΔPrampuprequested,b,f,c dB, which is the total power ramp-up requested by higher layer. In the last RAN1#120bis meeting, RAN1 has agreed that ΔPrampuprequested,b,f,c is the amount of power ramping applied to the latest Random Access Preamble transmission, regardless of the used RO types and symbol type of Msg3 PUSCH. For example, a UE first transmits a PRACH by using SBFD RO and switches to legacy RO. According to the agreement, ΔPrampuprequested,b,f,c is the total power ramping across two RO types. The value is applicable to msg3 PUSCH on non-SBFD symbol and SBFD symbol.

The following agreement was made in RAN1#120bis.

Agreement (RAN1#120bis)

- For determination of Msg3 PUSCH transmission power, ∆Prampup_requested,b,f,c is provided by MAC layer and is the amount of power ramping applied to the latest Random Access Preamble transmission, regardless of the used RO types and symbol type of Msg3 PUSCH.

- FFS: For RACH configuration Option 2, interpretation of ∆Prampup_requested,b,f,c when separate PREAMBLE_POWER_RAMPING_STEP configurations for different RO types when UE switches from additional-ROs to legacy-ROs.

  - This does not imply that separate PREAMBLE_POWER_RAMPING_STEP configurations are supported

One remaining issue is therefore the interpretation of ΔPrampuprequested,b,f,c when different values of PREAMBLE_POWER_RAMPING_STEP are configured for RACH configuration Option 2. One proposal was to use PREAMBLE_POWER_RAMPING_STEP according to symbol type. That is, if Msg.3 3 PUSCH is scheduling on non-SBFD symbol, use PREAMBLE_POWER_RAMPING_STEP for legacy PRACH configuration to calculate ΔPrampuprequested,b,f,c. That is, ΔPrampuprequested,b,f,c is dependent on the symbol type where Msg.3 PUSCH is scheduled. It is complicated especially when Msg.3 PUSCH is retransmitted in different symbol type. Also, the TPC command in RAR UL grant or DCI format 0_0 scrambled with TC-RNTI can indicate a proper closed power offset value.

- Proposal 5: For RACH configuration Option 2, interpretation of ΔPrampuprequested,b,f,c is based on the latest RO type, regardless of symbol types for Msg.3 PUSCH.

# Conclusion

In this contribution, we made the following proposals:

- No specification impact is necessary when for CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure.

For RACH configuration Option 2, the association period and association pattern period of additional ROs are configured an integer multiple of the association period and association pattern period of the legacy ROs.

The existing RACH Msg.2 (RAR) reception procedure is reused for random access in SBFD symbols.

- Proposal 4: For determining the frequency domain resource allocation for Msg.3 PUSCH transmission in SBFD symbols, the RB numbering starts from the first RB of the UL usable PRBs and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP.

- Proposal 5: For RACH configuration Option 2, interpretation of ΔPrampuprequested,b,f,c is based on the latest RO type, regardless of symbol types for Msg.3 PUSCH.

# References

RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”; RAN#104

TR38.858 v2.0.0 “Study on Evolution of NR Duplex Operation (Rel-18)”; RAN#102

R1-2503543 “Discussion on RAN4 LS on the transient period for SBFD operation”; Samsung

R1-2504003 “Discussion on RAN2 LS on simultaneous configuration of SBFD and DC”; Samsung

R1-2503563 “SBFD operation and procedures”; Samsung

R1-2502366 “Random access on SBFD resources”; Samsung

R1-2503565 “Cross-link interference handling for SBFD”; Samsung

R1-2503593 “UE features for Rel-19 Duplex Evolution for NR”; Samsung