3GPP TSG RAN WG1 #121		R1-2503569

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.4.4

Source:	Samsung

Title:	Views on aspects including multiplexing/multiple access, scheduling information, and timing relationships

Document for:	Discussion and decision

# 1	Introduction

In this contribution, we share our views on various other aspects for A-IoT according to the WID [1]. The discussion involves the following aspects:

- Control information: R2D control information for PRDCH reception, PDRCH scheduling, and D2R control information.

- Timing relationship.

- Physical layer aspects of random access, including multiplexing and multiple access.

- Other aspects of physical channels such as PRDCH for group signaling, and chip length determination.

- Direction 1 for extending device operation time.

- Solution 1 for proximity determination.

# 2	Control information

  - R2D control information

For R2D reception

During the SI, it was agreed that the following information can be explicitly/implicitly indicated to the device for R2D reception via the corresponding PRDCH:

- ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported).

The IDs associated with devices can be a particular device ID, a list of device IDs, a particular device group ID, or all devices by indicating NULL. A device group ID can be particularly useful for many A-IoT use cases, such as inventorying a certain product type or category.

In addition, we think that the following information needs to be further indicated to the device for R2D reception via the corresponding PRDCH:

- TBS/payload related information to determine the end of the PRDCH, which may be implicit or explicit depending on the message type. For instance, for a message type with a fixed message size, a device can derive the end of PRDCH transmission based on message type indication without an explicit TBS indication. For a message type with a variable message size, a device can derive the end of PRDCH transmission based on TBS indication in the R2D control information.

The following R2D control information for PRDCH reception is indicated in the corresponding PRDCH

- ID associated with device(s) intended for the reception of R2D

- TBS/payload related information, either explicitly or implicitly based on message type indication for messages having a fixed size.

Figure 1 PRDCH with separate CRC attachment for R2D control information and data

For R2D control information, it can be further discussed whether it needd to be transmitted via L1 signaling with a separate CRC or via higher-layer signaling by multiplexing in the R2D data. In our view, providing R2D scheduling information via L1 control information has an advantage as the device can early detect whether the corresponding R2D transmission is addressed to the device or not. This will also save device’s energy by avoiding unnecessary processing of the R2D payload, which is not addressed to the device. Therefore, the above R2D control information needs to be signaled to the device via L1 R2D control information. The L1 R2D control information should have a fixed size and attached with a separate CRC.

- Providing R2D control information for PRDCH reception via L1 control information has an advantage of early detecting whether the transmission is addressed to the device or not.

Provide R2D control information for PRDCH reception via L1 control information in the corresponding PRDCH, wherein the L1 control information is attached with separate CRC-6.

Otherwise, for a PRDCH, the device has to determine the end of PRDCH by detecting a postamble and, only after reading the entire payload and checking CRC, the device can identify whether the PRDCH is destined to the device or not.

If L1 R2D control information is provided, it is straightforward to indicate the TBS/payload related information in the L1 R2D control information for a device to determine the end of PRDCH.

The end of PRDCH is determined by a device based on TBS/payload related information provided in the L1 R2D control information.

Figure 2 Issues with data chip length indication via control

The above figure illustrates a situation where R2D data chip length is different from R2D control chip length, which has several issues as discussed below.

First of all, there is no clear motivation for having different chip lengths between R2D control and data as both control and data parts go through the same channel condition. Also, it will increase the device complexity for switching chip rates quickly between control and data without clear motivation, which can be challenging for such low complexity devices. For receiving the data part, the device has to deduce the chip synchronization from that obtained from the clock-acquisition part rather than it can assume that the chip synchronization is directly conveyed over. This will require additional complexity as well as lose precision.

- Having different chip lengths for R2D control and R2D data has no clear motivation as well as it will increase device complexity while the chip synchronization accuracy will be impacted.

The PRDCH chip length is obtained from the preceding R2D preamble signal, i.e., R2D control information does not provide information related to the PRDCH chip length.

For D2R scheduling

In RAN1 120b, the following agreements were made related to D2R scheduling [2].

Agreement For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. Working assumption For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size: - 7 bits for byte-level D2R payload size indication

So far, it was agreed that the following information can be explicitly/implicitly indicated to the device for D2R scheduling via the corresponding PRDCH:

- Time domain resources

- Frequency domain resources

- MCS-like information

- Chip duration

- ID associated with device(s)

- Repetitions

- Midamble (if supported) related information

- TBS/payload related information for the PDRCH transmission

Among the above information, some information had been agreed on how to indicate them. The following list of information requires further details.

- Starting time indication for TDMA resources with X=1 and X=2, i.e., Toffset1 and Toffset2.

- Midamble related information, which will be discussed in AI 9.4.3.

- Chip duration and small frequency shift factors, which will be discussed in AI 9.4.2.

- Repetitions, including maximum repetition factors and a number of bits for indication. This may be also discussed in AI 9.4.2.

We will discuss the indication for starting time indication for TDMA resources in Section 4.1 of this contribution, while the rest of above listed issues will be discussed in the corresponding contributions.

In RAN1 120 [3], it was agreed that the payload size (i.e., TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information. There are a couple of remaining issues regarding the payload size indication.

- Payload size (i.e. TBS-like) indication for PDRCH transmission with fixed size.

- Signaling details for indicating payload size (i.e., TBS-like) with variable size.

If the PDRCH conveys a message with a known and fixed payload size, the payload size determination can be based on the known payload size without an explicit indication.

For PDRCH transmission with a fixed and known payload (i.e., TBS-like) size, an explicit indication for the payload size is not provided.

In the previous meeting, it was made as a working assumption that, for indicating the payload size (i.e., TBS-like) for PDRCH transmission with variable size, 7 bits for byte-level D2R payload size indication is used. It is proposed to confirm the working assumption with the following clarification.

Confirm the working assumption: For indicating the payload size (i.e., TBS-like) for PDRCH transmission with variable size, 7 bits for byte-level D2R payload size indication. The 7 bits indicates payload size of 1 to 128 bytes.

Figure 3 D2R transmission timing indication

When D2R transmission timing is determined based on R2D control information, e.g., by providing a timing parameter TDelay in the R2D control information, the time unit for the indication needs to be defined. Indicating TDelay using a shortest chip length supported in R2D provides the most efficiency in indicating the interval.

If TDelay is indicated using the chip length of the scheduled PDRCH or scheduling PRDCH, the time granularity for the indicated interval can be too coarse when the M value of PDRCH/PRDCH is too small. Note that TDelay itself has no relation with the used M value of either PDRCH or PRDCH.

- Indicating a time interval between the scheduling PRDCH and the scheduled PDRCH using the shortest chip length is the most efficient.

Therefore, our view is that the time unit for the indication shall be the minimum chip length supported in R2D, which corresponds to the largest OOK-4 M value defined. If the largest value of M is denoted by Mmax, which is 24, then the time unit for the D2R transmission timing indication is (1/Mmax)∙Tsymbol, where Tsymbol is an underlying CP-OFDM symbol duration.

The time unit for indicating the time interval between a R2D transmission and the corresponding D2R transmission is the minimum chip length supported for R2D, which corresponds to M=24.

  - D2R control information

In RAN2 129b [4], for CBRA, it was agreed that NACK based mechanism is applied only to the Msg3 as a baseline. It is understood that the NACK message is transmitted via higher-layer signaling as it was agreed in RAN2. Therefore, there is no further need to discuss the support of ACK/NACK in RAN1.

On the other hand, as copied earlier, it was agreed in RAN1 120b [2] that the payload size (i.e., TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information. However, the reader does not have a precise knowledge on the current buffered data size at a device. Therefore, it is highly likely that there is a mismatch between the current buffered data size at the device and the indicated payload size, further given that there is no BSR like mechanism supported for A-IoT.

- It is highly likely that there is a mismatch between the current buffered data size at a device and the indicated payload size for PDRCH transmission.

Therefore, there is a need to provide actually transmitted payload size by the device to the reader.

If the available data size at a device for D2R transmission is smaller than the payload size (i.e., TBS-like) indicated by the reader, the actual transmitted payload size is indicated in the D2R control information.

# Timing relationship

In RAN1 120b, the following agreements were made related to this section [2].

Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2

Based on the progress made so far in RAN1 and RAN2, the following figure describes timing parameters that have been confirmed or potentially needs to be defined.

Figure 4 Timing relationships to be defined

In RAN2 129b [4], the following agreement was made regarding the support of random access triggering message.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Given the above RAN2 agreement, it is understood that there will be an R2D random access triggering message immediately following the R2D paging message as described in the figure above.

- It has been identified that there is a case where an R2D transmission immediately follows a preceding R2D transmission, i.e., R2D paging and R2D triggering messages.

Therefore, there is a need to define TR2D_R2D_min considering that there should be a sufficient processing time provisioned for a device to process back-to-back R2D transmissions such as a PRDCH providing a paging message followed by another PRDCH providing a triggering message, or a PRDCH providing a command followed by another PRDCH providing a command, etc. However, there is no need to define TR2D_R2D_max given that R2D transmission can occur at any time.

TR2D_R2D_min is defined such that a device is not require to monitor a R2D transmission earlier than TR2D_R2D_min after the end of a previous R2D transmission to the device.

For TR2D, there are two options:

- TR2D is always indicated by a reader in the preceding PRDCH and do not define any fixed timing relationship in RAN1. There may be still a certain timing requirement defined in RAN4 such that a device is not required to transmit D2R transmission earlier than the requirement. Accordingly, the indicated timing by the reader would ensure that it is valid and can be handled by the device.

- TR2D_nominal is defined without considering the impact of SFO, while defining timing requirements due to SFO is up to RAN4. TR2D_nominal may be defined for different D2R message types or different PHY processing requirements such as the use of FEC or not.

For TR2D, down-select from the following two options:

- Option 1:  TR2D is always indicated by a reader in the preceding PRDCH and do not define any fixed timing relationship in RAN1. The reader ensures that the indicated timing can be handled by the device.

- Option 2: TR2D_nominal is defined for different D2R message types or different PHY processing requirements such as the use of FEC or not. TR2D (≥ TR2D_nominal) can be still indicated by the reader.

It is noted that TD2R_min was already agreed to be defined in the previous meeting.

Lastly, there is a question on whether RAN1 has sufficient expertise to define minimum timing requirements, and it is feasible to define within one meeting. The decision should also be informed and confirmed by RAN4 during this meeting. Given that the minimum timing requirements include RF turn-around time, and base band processing time, which is highly dependent on implementation, it might be suitable to leave these parameters to be defined by RAN4.

- Given that minimum timing requirements (e.g., TD2R_min and any additional parameters to be agreed in this meeting) involves RF turn-around time, and base band modulation/demodulation time, which is highly dependent on implementation and within RAN4’s expertise, as well as considering that this is the last RAN1 meeting, it might be appropriate to request RAN4 to define these parameters.

D2R transmission timing

For the time interval between a R2D transmission and corresponding D2R transmission following it, there can be two options, which are not mutually exclusive:

- Option 1: D2R transmission follows R2D transmission after a predefined TR2D_nominal. TR2D_nominal may be defined for different D2R message types or different PHY processing requirements such as the use of FEC or not.

- Option 2: D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_nominal.

When D2R transmission timing is determined based on R2D control information, a guard time between time-multiplexed devices may be considered by the reader for a possible timing drift or processing delay for PDRCH transmission at the devices.

Figure 5 Scheduling multiple D2R transmissions using delay parameter

The figure above describes R2D transmission scheduling one or more D2R transmissions via a timing delay indication. Multiple D2R transmissions do not need to be in a TDMA manner following a notion of time slots. It can be in an asynchronous manner based on indicated timing parameters.

The time interval between a R2D transmission and corresponding D2R transmission following it is determined by one of the following options:

- Option 1: D2R transmission follows R2D transmission after a predefined TR2D_nominal.

- Option 2: D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the preceding R2D transmission, where TR2D ≥ TR2D_nominal.

# Physical layer aspects of random access

The figure below describes the overall three-step random access procedure for inventory. In this section, we discuss in more details and share our views on each of the transmissions involved in the procedure, namely the paging/trigger message, Msg1, Msg2, and Msg3.

Figure 6 Overall 3-step random access procedure

  - PRDCH triggering random access & PDRCH providing Msg1

In RAN1 120b, the following agreements were made related to this section [2].

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. Agreement At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions.

One remaining issue is regarding the FFS on whether Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access, and detailed value(s) considering device processing time including FEC impact.

The first time slot starts a certain time interval, denoted by Toffset1, after the reception of the PRDCH triggering random access. Toffset1 may be indicated in the paging message, in the triggering message, or predefined in the specification.

Indicating Toffset1 is more reasonable with the following reasons:

- It is more flexible to indicate Toffset1 in the R2D message rather than predefining the interval to a single fixed value given that processing time for the following Msg1 D2R transmission can vary depending on the use of FEC, use of repetition, or different chip lengths.

- Also, RAN1 will not define TR2D_min and TR2D_max. The discussion on defining Toffset1 without based TR2D_min and TR2D_max would require more specification effort given that this is the last meeting.

- Given the required specification effort and flexibility, it is preferable to indicate Toffset1 in the R2D message rather than predefining it to a single fixed value.

On the other hand, it is understood from RAN2 discussion that the triggering message is intended to be compact and minimal. Also, if Toffset1 is indicated, it can be applied to all the random access rounds triggered by one or more triggering messages in a given paging session, i.e., it doesn’t need to be repeatedly indicated in each triggering message.

- Toffset1 can be applied to all the random access rounds triggered by one or more triggering messages in a given paging session, and it doesn’t need to be repeatedly indicated in each triggering message.

Therefore, it is reasonable to indicate Toffset1 in a paging message.

Toffset1 is indicated in a PRDCH providing a paging message.

Another remaining issue is to confirm a working assumption on a reference timing for indicating Toffset2.

Figure 7 Starting time indication for time slots with TDMA X=2

First of all, it is proposed to confirm the working assumption on the reference timing for indicating Toffset2 as it is the most logical approach given the efficiency of signaling. In other words, indicating Toffset2 from the end of the corresponding R2D transmission triggering random access would waste bits for indicating Toffset2.

Confirm the working assumption that, for X=2, the starting time for the second Msg1 time domain resource is derived based on the starting time of the first Msg1 time domain resource plus Toffset2.

Another remaining issue is whether Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

The length of time slot Toffset2 depends on various factors such as:

- The chip length indicated for Msg1 transmission. The longer the total transmission duration, the longer the guard time needed between slots. A device may be able to calculate the transmission duration for Msg1 based on indicated transmission parameters. However, the device cannot and should not determine the guard time by itself.

- The target coverage and characteristics on device SFO as it affects the required guard time between slots. Reader can only determine the guard time as it has knowledge on the target coverage and device SFO.

- The length of time slot Toffset2 depends on various factors such as scheduled transmission parameters for Msg1 as well as guard time required between slots for a given specific situation.

Therefore, it is the most reasonable to explicitly indicate the Toffset2 in the preceding PRDCH providing the paging message. Also, considering that this is the last meeting, defining an implicit rule would be daunting.

Toffset2 is explicitly indicated in the preceding PRDCH providing the paging message.

PDRCH providing Msg1

In this step, a random number of devices attempt to access the medium by randomly selecting a time and/or frequency resource from a set of resources according to the parameters provided in the PRDCH providing the triggering message. The PDRCH via random access delivers the transmitting device’s ID to the reader. The ID used at this stage is a temporary ID, i.e., RN16, per RAN2 agreement.

The random access may involve a process to determine whether to access the medium or not based on the provisioned access probability through which a congestion control can be achieved.

For a multiplexed random access for Msg1 transmission, a device first randomly selects a random access round from a total number of random access rounds. Denote by Nfreq a number of frequency domain resources, i.e., a number of frequency shifts. Then, for a chosen random access round, the device randomly selects one resource from a total of Nslot∙Nfreq number of TDMA/FDMA resources. For a chosen random access resource, the device transmits Msg1 including randomly drawn RN16.

For a multiplexed random access for Msg1 transmission, a device first randomly selects a random access round from a number of random access rounds and, then, randomly selects one resource from a number of TDMA/FDMA resources.

  - PRDCH providing Msg2

In this step, the reader transmits PRDCH providing an ACK for a successfully received PDRCH random access transmission. There may be a certain time interval that a device waits for the possible ACK response from the reader, which may be predefined or indicated to the device in the paging message. If a device does not receive a following ACK message within the time interval, the device considers that the random access is failed.

In the previous meeting, it was agreed to support the following two options for Msg2 transmission in response to multiple Msg1 transmissions:

- Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device

- Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

There are further aspects for monitoring Msg2 in response to multiple Msg1 transmissions such as the window duration to be 1) predefined, or 2) indicated in the R2D transmission triggering random access. Also, a reference time for the starting time for monitoring Msg2 needs to be defined.

Figure 8 Reception timing for a Msg2 corresponding to multiple Msg1

For Msg2 corresponding to one or more Msg1, a device starts monitoring the PRDCH providing Msg2 TD2R_min after the end of X time domain resources for Msg1 transmission. The monitoring window can be Δ = TD2R_max - TD2R_min such that the start of PRDCH falls within the Δ time interval. If Msg2 is not received within the Δ time interval, the device preceives that the current inventory round is unsuccessful and restarts the inventory round, e.g., retransmit Msg1 in a later round.

It is noted that the timing of the end of X time domain resources is the timing perceived by the device with the timing imperfection such as due to SFO, and it can deviate from the precise timing perceived by the reader. Given that there is no timing adjustment for a device in A-IoT, the reader accomodates such timing imperfection by accounting a certain timing margin considering possible timing drift by devices.

For a Msg2 corresponds to multiple Msg1s received from different devices, the PRDCH providing Msg2 can be received no earlier than TD2R_min from the end of the last time domain resource for Msg1 transmission.

Figure 9 Reception timing for a Msg2 corresponding to a Msg1

For a Msg2 corresponding to a Msg1, the reference timing for a device to start monitoring the PRDCH providing Msg2 can be still the same as before, i.e., TD2R_min after the end of X time domain resources for Msg1 transmission. In this case, given that more than one Msg2s can be transmitted in a staggered manner, there can be an additional offset to start monitoring the Msg2.

Assume that the time/frequency resources for Msg1 transmission are indexed by frequency, then time. As an example, in the figure above, it can be indexed in the order of (x=1, y=1), (x=1, y=2), (x=2, y=1), (x=2, y=2) wherein x is the time slot index and y is the frequency resource index, e.g., frequency shifts. For Msg1 transmission resource index 1, there is no additional offset and the monitoring window Δ time interval starts TD2R_min after the end of X time domain resources for Msg1 transmission. For Msg1 transmission resource index i, there is (i-1)×Toffset additional timing offset to start monitoring Δ time interval TD2R_min after the end of X time domain resources for Msg1 transmission. Toffset can be defined to account the transmission duration of PRDCH providing single Msg2 and any additional guard time between PRDCH transmissions providing Msg2.

For a Msg2 corresponds to a Msg1 received from a device, the PRDCH providing Msg2 can be received no earlier than TD2R_min+TΔ from the end of the last time domain resource for Msg1 transmission, where TΔ is a timing offset determined by the time/frequency resource used for Msg1 transmission.

The Msg2 needs to provide information related to transmit Msg3 as follows:

- Scheduling information to transmit PDRCH providing Msg3 including time/frequency resources.

- Any data request to be included in the Msg3 transmission.

- A command for subsequent operation following device identification, if applicable.

The PRDCH providing Msg2 indicates 1) scheduling information to transmit PDRCH providing Msg3, 2) additional data request to be included in the Msg3, and 3) a subsequent command, if applicable.

  - PDRCH providing Msg3

In RAN1 120, the following agreements were made related to Msg3 transmission frequency domain resource [2].

Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

As copied above, it was agreed that the frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. Regarding the FFS, having an additional default mode of operation will make the device operation more complicated as it involves a decision making on whether there is an explicit indication provided or not. If provided, always providing the corresponding control information will make the device operation simpler.

The frequency domain resource for Msg3 is always determined based on explicit indication in the PRDCH providing Msg2.

# Physical channels – other aspects

The figure below describes PRDCH structure providing a signaling for a group of devices. The PRDCH providing a group signaling can be beneficial to improve the overall system efficiency.

Figure 10 PRDCH structure providing a group signaling

RAN1 agreed the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access:

- Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device

- Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

The PRDCH providing a group signaling enables Option 2 above, i.e., a PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices. The header field of the PRDCH, i.e., L1 control, providing a group signaling may indicate that the corresponding PRDCH is for group-specific signaling addressed to a group of devices. The PRDCH providing a group signaling may include one or more information blocks for one or more devices. Each block may provide a specific device ID within the group and may be attached with a separate CRC for the block.

A PRDCH providing a group signaling for multiple devices is supported.

# Direction 1 for extending device operation time

To handle potential impact of device unavailability due to charging by energy harvesting, the following two directions were identified during the SI:

- Direction 1:	Reader does not provide information to a device regarding when the device may become available/unavailable.

- Direction 2:	Reader can provide information to a device based on which the device may become available/unavailable.

Based on device implementation or standard defined definitions, a device may have the following operation states:

- OFF state: A device is unable to transmit or receive a signal or a channel from a reader. A volatile memory is not maintained, while non-volatile memory is maintained. The device can perform energy harvesting.

- ON state: A device is able to receive a signal or a channel from a reader, and the device is able to transmit in response to a reception from the reader. Depending on the device implementation, it may or may not be able to recharge via energy harvesting, while the device is monitoring or receiving a signal or a channel from the reader.

- Power Saving (PS) state: Both volatile and non-volatile memory are maintained. Therefore, counters or information received from a reader can be kept. During the PS state, the device is not expected to be able to continuously receive a signal or a channel from a reader. However, the device is able to transition into the ON state in a periodic manner or when a timer expires.

The state transition between ON and OFF states can be based on energy level of the device. As an example, a device can transition OFF  ON when the energy level exceeds a certain threshold and, therefore, the device is sufficiently charged. Similarly, a device can transition ON  OFF when the energy level goes below a certain threshold. The energy threshold values may be an implementation choice.

Similarly, the state transition between PS and OFF states can be also based on energy level of the device. A device transitions PS  OFF when the energy level goes below a certain threshold. This may be when the charging rate in the PS state is slower than the discharging rate in the state for maintaining basic functional blocks such as memories and counting operations. The energy level triggering a transition from the PS state to the OFF state can be an implementation choice.

On the other hand, the state transitions between ON and PS states can be based on Direction 1 for extending device operation time.

- Define a minimum and a maximum time between two PRDCHs providing random access triggering messages, denoted by Ttrigger_max and Ttrigger_min, such that the device is not required to monitor a potential R2D transmission during the minimum time, if the device does not participate the corresponding random-access round. During the minimum time, the device may transition into the PS state. After the minimum time, the device is required to monitor a potential R2D transmission at least until the maximum time from the preceding PRDCH reception.

- Define a maximum time for a device, which successfully finished the random-access procedure, such that the device is not required to monitor a potential R2D transmission during the defined maximum time, denoted by Tsleep_max. After the expiration of the maximum time, the device is required to monitor a potential R2D transmission. The maximum time duration may be dependent on the count of the current random access round. In this case, if the current random access round is n, the maximum sleep time is denoted as Tsleep_max(n), which is a function of n.

- Define a minimum time, denoted by TON_min, for a device to continuously monitor a potential R2D transmission, whenever the device transitions into the ON state, such as from OFF state, to guarantee a reachability from a reader to the device.

The figure below describes Ttrigger_max and Ttrigger_min between two consecutive PRDCHs providing triggering messages.

Figure 11 A minimum and a maximum time between two PRDCHs providing triggering messages

As illustrated in the figure, a PRDCH transmission providing a trigger indicates a start of a random access round. The time interval between two consecutive triggers may be variable, e.g., a reader may early terminate the random access round, if no Msg1 is successfully received, by transmitting the next PRDCH providing a trigger early. For a given random access round initiated by a trigger message, a device may or may not perform random access based on random determination. If the device decides to not to perform random access for the given random access round, the device can transition into the PS state with a timer value of Ttrigger_min. After the expiration of the timer, the device transitions back to the ON state to monitor a possible trigger message from the reader. After transitioning into an active state, the device monitors a trigger during [0, Ttrigger_max - Ttrigger_min]. If no trigger is detected, the device assumes that the inventory process has ended and performs normal state transition operation as discussed earlier.

If a device is successfully inventoried, the device does not need to participate in the remaining random access rounds of the current inventory process. Therefore, a device may transition into PS state with a certain timer denoted by Tsleep. Tsleep may be predefined in a specification of system operations, or calculated by a device based on the current round count, n, announced in the trigger messages and other parameters such as Ttrigger_max and Ttrigger_min.

Define parameters to facilitate device dormancy including:

- A minimum and a maximum time between two consecutive PRDCHs providing triggering messages.

- A maximum time for a device, during which a device successfully finished inventory is not required to monitor a potential R2D transmission.

- A minimum time required for a device to continuously monitor a potential R2D transmission, when the device transitions into the ON state to guarantee a reachability from a reader to the device.

For direction 1, where the reader does not provide information the device regarding when it may become available or unavailable, the device sending its availability status can be helpful. The reader can refer to this information when scheduling devices. This availability information can be the energy status report as specified in TR 38.769:

- The energy status report from the A-IoT device to the reader, which indicates that the A-IoT device, if the A-IoT device can determine this, does not have sufficient energy to perform the remaining/follow-up procedure.

The energy status report (ESR) should be specified from the RAN1 perspective, as energy availability inherently reflects the device’s real-time energy state. In particular, if segmentation is performed at the MAC layer, the ESR may be included in only one of transport blocks. In addition, if repetition is applied at the PHY layer, the same ESR may be included in all TBs and transmitted. In these cases, the information in the ESR may be outdated by the time the corresponding TB is actually transmitted. Therefore, it is appropriate to consider ESR multiplexing at the A-IoT PHY layer.

Specify the energy status report multiplexing at the A-IoT PHY layer.

Figure 12 Protocol stack for the Ambient-IoT

Figure 13 Energy status report in A-IoT MAC segmentation and PHY repetition for D2R transmission

# Solution 1 for proximity determination

During the SI, proximity determination for a device was studied and the following solution is chosen for specification work for the WI phase:

- Solution 1:	 If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

In this section, we share our views on the specification impact of Solution 1 for proximity determination. The figure below illustrates proximity determination based on a D2R reception at a reader in response to R2D transmission according to Solution 1.

Figure 14 Proximity determination based on a reader R2D power control (Solution 1)

The general principle of Solution 1 includes a reader transmitting a message with a reduced power, which can be only heard by a proximity device, and based on a reception of a response message from the proximity device. The main advantage of Option 1 is that it can work with any type of device in any topology, and proximity determination can be based on the random access or command protocols.

The proximity determination may be for one or more targeted or untargeted devices. Therefore, the REQ message in Figure 12 may include a device ID or device group ID, if the proximity determination is for targeted devices. For the case of the proximity determination of the targeted devices, the REQ message may further indicate a dedicated resource for the device to respond with the RES message, i.e., a contention free random access. The REQ message transmission power control is a reader implementation issue, i.e., no separate study is needed. The RES message may include the responding device’s ID for the identification of the message for proximity determination. The REQ and RES messages are not necessarily newly designed messages dedicated for the proximity determination. They can be those to be defined for the inventory process. If the topology includes two separate readers for transmission and reception, respectively, the receiving reader sends the proximity determination report to the transmitting reader.

- No specification work is needed to support Solution 1 for proximity determination.

# 8	Conclusion

In the previous sections we made the following observation:

Observation 1	Providing R2D control information for PRDCH reception via L1 control information has an advantage of early detecting whether the transmission is addressed to the device or not.

Observation 2	Having different chip lengths for R2D control and R2D data has no clear motivation as well as it will increase device complexity while the chip synchronization accuracy will be impacted.

Observation 3	Indicating a time interval between the scheduling PRDCH and the scheduled PDRCH using the shortest chip length is the most efficient.

Observation 4	It is highly likely that there is a mismatch between the current buffered data size at a device and the indicated payload size for PDRCH transmission.

Observation 5	It has been identified that there is a case where an R2D transmission immediately follows a preceding R2D transmission, i.e., R2D paging and R2D triggering messages.

Observation 6	Given that minimum timing requirements (e.g., TD2R_min and any additional parameters to be agreed in this meeting) involves RF turn-around time, and base band modulation/demodulation time, which is highly dependent on implementation and within RAN4’s expertise, as well as considering that this is the last RAN1 meeting, it might be appropriate to request RAN4 to define these parameters.

Observation 7	Given the required specification effort and flexibility, it is preferable to indicate Toffset1 in the R2D message rather than predefining it to a single fixed value.

Observation 8	Toffset1 can be applied to all the random access rounds triggered by one or more triggering messages in a given paging session, and it doesn’t need to be repeatedly indicated in each triggering message.

Observation 9	The length of time slot Toffset2 depends on various factors such as scheduled transmission parameters for Msg1 as well as guard time required between slots for a given specific situation.

Observation 10	No specification work is needed to support Solution 1 for proximity determination.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The following R2D control information for PRDCH reception is indicated in the corresponding PRDCH

	ID associated with device(s) intended for the reception of R2D

	TBS/payload related information, either explicitly or implicitly based on message type indication for messages having a fixed size.

Proposal 2	Provide R2D control information for PRDCH reception via L1 control information in the corresponding PRDCH, wherein the L1 control information is attached with separate CRC-6.

Proposal 3	The end of PRDCH is determined by a device based on TBS/payload related information provided in the L1 R2D control information.

Proposal 4	The PRDCH chip length is obtained from the preceding R2D preamble signal, i.e., R2D control information does not provide information related to the PRDCH chip length.

Proposal 5	For PDRCH transmission with a fixed and known payload (i.e., TBS-like) size, an explicit indication for the payload size is not provided.

Proposal 6	Confirm the working assumption: For indicating the payload size (i.e., TBS-like) for PDRCH transmission with variable size, 7 bits for byte-level D2R payload size indication. The 7 bits indicates payload size of 1 to 128 bytes.

Proposal 7	The time unit for indicating the time interval between a R2D transmission and the corresponding D2R transmission is the minimum chip length supported for R2D, which corresponds to M=24.

Proposal 8	If the available data size at a device for D2R transmission is smaller than the payload size (i.e., TBS-like) indicated by the reader, the actual transmitted payload size is indicated in the D2R control information.

Proposal 9	TR2D_R2D_min is defined such that a device is not require to monitor a R2D transmission earlier than TR2D_R2D_min after the end of a previous R2D transmission to the device.

Proposal 10	For TR2D, down-select from the following two options:

	Option 1:  TR2D is always indicated by a reader in the preceding PRDCH and do not define any fixed timing relationship in RAN1. The reader ensures that the indicated timing can be handled by the device.

	Option 2: TR2D_nominal is defined for different D2R message types or different PHY processing requirements such as the use of FEC or not. TR2D (≥ TR2D_nominal) can be still indicated by the reader.

Proposal 11	The time interval between a R2D transmission and corresponding D2R transmission following it is determined by one of the following options:

	Option 1: D2R transmission follows R2D transmission after a predefined TR2D_nominal.

	Option 2: D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the preceding R2D transmission, where TR2D ≥ TR2D_nominal.

Proposal 12	Toffset1 is indicated in a PRDCH providing a paging message.

Proposal 13	Confirm the working assumption that, for X=2, the starting time for the second Msg1 time domain resource is derived based on the starting time of the first Msg1 time domain resource plus Toffset2.

Proposal 14	Toffset2 is explicitly indicated in the preceding PRDCH providing the paging message.

Proposal 15	For a multiplexed random access for Msg1 transmission, a device first randomly selects a random access round from a number of random access rounds and, then, randomly selects one resource from a number of TDMA/FDMA resources.

Proposal 16	For a Msg2 corresponds to multiple Msg1s received from different devices, the PRDCH providing Msg2 can be received no earlier than TD2R_min from the end of the last time domain resource for Msg1 transmission.

Proposal 17	For a Msg2 corresponds to a Msg1 received from a device, the PRDCH providing Msg2 can be received no earlier than TD2R_min+TΔ from the end of the last time domain resource for Msg1 transmission, where TΔ is a timing offset determined by the time/frequency resource used for Msg1 transmission.

Proposal 18	The PRDCH providing Msg2 indicates 1) scheduling information to transmit PDRCH providing Msg3, 2) additional data request to be included in the Msg3, and 3) a subsequent command, if applicable.

Proposal 19	The frequency domain resource for Msg3 is always determined based on explicit indication in the PRDCH providing Msg2.

Proposal 20	A PRDCH providing a group signaling for multiple devices is supported.

Proposal 21	Define parameters to facilitate device dormancy including:

	A minimum and a maximum time between two consecutive PRDCHs providing triggering messages.

	A maximum time for a device, during which a device successfully finished inventory is not required to monitor a potential R2D transmission.

	A minimum time required for a device to continuously monitor a potential R2D transmission, when the device transitions into the ON state to guarantee a reachability from a reader to the device.

Proposal 22	Specify the energy status report multiplexing at the A-IoT PHY layer.

# References

RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120b, Wuhan, China, April 7th – 11th, 2025.

RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120, Athens, Greece, February 17th – 21st, 2025.

RAN2 Chair’s Notes, 3GPP TSG RAN WG2 #129b, Wuhan, China, April 7th – 11th, 2025.