3GPP TSG RAN WG1 #121		R1-2503572

St Julian's, Malta, 19th – 23rd May, 2025

Agenda item:	9.5.3

Source:	Samsung

Title:	Adaptation of common signal/channel transmissions

Document for:	Discussion and decision

# Introduction

A WI on enhancements of network energy saving for NR was endorsed in [1], and revised in [2]. One objective in the revised WID is to specify adaptation of common signal/channel transmissions as following.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution discusses how to support adaptation of common signal/channel transmissions, including the adaptation of SSB in time domain, and adaptation of PRACH in time domain.

# Adaptation of SSB transmissions in time domain

## Timeline for applying the DCI format

In RAN1#120bis [11], the following working assumption was made regarding the timeline on using DCI format to trigger adaptation of SSB transmissions:

Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst

The remaining issue is on how to determine the first possible SSB burst, after the adaptation. So far, RAN1 only agreed to support a nesting structure of SSB transmission occasions before and after adaptation – transmission occasions for a longer periodicity is a subset of transmission occasions for a shorter periodicity. For example, as illustrated in Figure 1, assuming the periodicity before adaptation is 10 ms (e.g., P1 = 10 ms) with associated SFN offset as 0 and half frame index as 0, and the periodicity after adaptation is 20 ms (e.g., P2 = 20 ms). If without any further explicit signalling, there can be two cases on the half frames for SSB transmission (i.e., possible SSB burst) after adaptation: Case 1 SFN offset = 1 with HF index = 0; Case 2 SFN offset = 0 with HF index = 0. After receiving the DCI indicating the adaptation, a UE may need to blind detect from the two cases and determine which from two cases is applicable and further determine the first possible SSB burst accordingly. It is noted that the number of blind detections equals to P2/P1, which can be significantly large when P2 is remarkably larger than P1. To avoid such unnecessary UE complexity, RAN1 can further introduce explicit signalling of the SFN offset and HF index (e.g., similar to on-demand SSB) per configured periodicity, then a UE can apply the associated SFN offset and HF index to determine the first possible SSB burst after adaptation, without blind detections.

Figure 1 Illustration of first possible SSB burst after adaptation.

Proposal 1: Support explicit signalling of a SFN offset and a half frame index per configured periodicity for SSB adaptation, by higher layer parameters.

- A UE can determine the first possible SSB burst in the half frames determined by the SFN offset and half frame index.

Moreover, there is another further discussion point on the value of the processing delay d. A serving cell configured with SSB adaptation is activated and a UE can receive and transmit channels or signals on the serving cell, this case is different from SCell dormancy and the application delay for SSB adaptation can be much smaller than the application delay for SCell dormancy. In addition, a small value of d can help UE stay longer in the sleep mode and thus is beneficial for network energy saving. For example, d =1 can be considered.

Proposal 2: For the application delay of SSB adaptation indicated by a DCI format, support d =1.

## UE behaviour on missing the DCI format

There was a discussion on whether to support UE behaviour on missing the DCI format, and the consequence of misunderstanding of the SSB transmission due to the missed DCI reception. Our view is, similar to other group common DCI formats, especially the DCI format 2_9 for cell DTX/DRX purpose, the probability on misdetection is already very small, and a UE can always deal with the issue of SSB transmission alignment by implementation (e.g., trying to decode data channel using an alternative hypothesis), although such UE treatment may not be even necessary since there will be another group common DCI for reception soon.

Proposal 3: No need to introduce specification support for handling missed DCI for SSB adaptation.

# Adaptation of PRACH transmissions in time domain

## Configuration related

In RAN1#120bis, following agreement on the subset configuration is reached:

Agreement from RAN1#120bis For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by: - Option 1-2: Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods) - For K: one from up to four candidate values {2,4,8, [1 or 16]} Mask index; Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP). 0; 1st half of the APs in K APPs. 1; 1st quarter of the APs in K APPs. 2; 1st eighth of the APs in K APPs. 3; 1st sixteenth of the APs in K APPs.

One remaining issue is the K value, considering the flexibility of the RACH configuration already existed at network side, we think there is no much need to support a lot of values. To us, the dense RACH configuration will easily lead the AP and APP to be same as 10ms, so having 16 as one candidate value is also reasonable.

Proposal 4: One from up to four K candidate values {2, 4, 8, 16}, K=1 if not configured.

## Adaption related

With following agreements, the remaining details of DCI based PRACH adaptation is discussed.

Agreement from RAN1#120b - For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI - The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})

There is one more issue to be discussed for the RACH adaption design,

Figure 2 illustration of UE missing the activated RACH resource

Given an exemplary configuration like in above figure, network is able to transmit the activation DCI in any modification period. That naturally creates two types of UE: one is like UE1 who is able to detect the paging DCI and be aware of the activated RACH resource in validity period (e.g., 4 modification periods); the other is like UE2 who is unable to detect and not aware of the activated resource even from network perspective it’s available to use. This drawback makes the design so far is not beneficial to all the UE who support the feature and willing to use the resource when the resource is actually activated. This also against the design intention to allow the NES UE to access the system as soon as possible by utilizing the additional NES RACH resource so that these activated RACH resource can be also deactivated ASAP..

Observation 1: Some UE might not be able to use the resource even it’s activated.

To solve this issue, SIB1 directly indicates whether the additional RACH resource is activated or not, e.g., “0” means in current modification period, the additional RACH resource is not activated, “1” means in current modification period, the additional RACH resource is activated, and from current modification period by considering the configured validity period. Such validity period needs to be controlled by network, e.g., revise accordingly to the remaining validity periodicities. This should be depending on the network already send paging DCI before. Another way is to keep the DCI activation while asking UE to read the SIB 1 to know the update of validity periods. This might require more effort (e.g., DCI reading from UE, PDCCH Tx from network) so seems not quite energy saving per say.

Proposal 5: RAN1 supports 1 bit activation status indication in SIB1 and validates from current SI modification, and network ensures the configured validity period to be aligned with remaining validity period from the DCI activation.

One remaining detail is the determination of the value sets for “Multiple of SI modification period”. Given the fact that the value of SI modification period is already in terms of power of 2, e.g., 160ms, 320ms. The choice is either kept as small value set, e.g., no need of very large value. Originally, the value “1” is not attractive since the starting time is current SI modification period, but after a second thought, it can be up to network to decide whether “1” is useful, e.g., if the RACH resource is all later than the last paging occasion.

Proposal 6: RAN1 supports the value set for “Multiple of SI modification period” to be {1, 2, 3, 4}.

RAN1 has agreed that for DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. This DCI is paging DCI and transmitted in PO and addressed to P-RNTI.

In order to receive this, UE needs to monitor PO in RRC_CONNECTED state. However, PO is monitored only if paging search space is configured in active DL BWP. Paging search space configuration is not mandatory in every BWP. In case paging search space is not configured in active DL BWP, UE cannot receive the DCI format 1_0. So other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources may be needed.

Proposal 7: Discuss and adopt one of the following for RRC_CONNECTED:

- Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that paging search space is configured in active DL BWP.

- Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

UE can be configured with multiple BWPs in RRC_CONNECTED. Multiple BWPs can be configured with additional PRACH resources. The issue is which BWPs additional PRACH resources are activated when the DCI is received. In our view, for DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, DCI activates the additional PRACH resources configured in the active UL BWP. This is a simple approach and does not require additional changes to DCI.

Proposal 8: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, DCI activates the additional PRACH resources configured in the active UL BWP.

In addition, since the network might also prepare the SUL in the system, there might be a chance the either or both of SUL and NUL are configured with additional PRACH resource. Given the SUL and NUL are in different carrier and for different purpose, e.g., SUL for more coverage limited case, so the expectation on the UE in SUL and NUL are different. Then it’s more gNB’s choice on which one(s) in the SUL and/or NUL to be actually activated for UE to use. So we suggest to include an indication of the NUL and/or SUL in the Paging DCI.

Proposal 9: Paging DCI also indicate SUL and/or NUL in addition to 1 bit for activation.

## PDCCH order related

For following agreement

Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication

Several remaining aspects are to be decided:

- The applicable resource of the 1 bit indication of whether additional PRACH resource is available. E.g., it applies to both additional PRACH and legacy PRACH or applies to only additional PRACH. The Pros of the former explanation could choose the earliest PRACH resource as much as possible for contention free RACH. But it will mix the legacy resource and additional resource together. Given the association pattern period is separately determined by legacy and additional PRACH resource, thus, it’s difficult for network to always align these two resource configurations so the indication of PRACH mask index could easily applied. Thus the clean way is to apply to additional PRACH resource only if such 1 bit indication enables the additional PRACH resource. However, if the PDCCH order triggered the CBRA, there is no need to differentiate the additional and legacy resource, we can follow the normal procedure of CBRA that both resources can be selected by UE.

- The timeline of applying the additional resource. If network triggers the CFRA, to fit the general requirement of the CFRA, which is to allow UE conduct the RACH quicker. The indicated additional resource becomes available a certain time after the PDCCH order, e.g., it doesn’t need to wait for the paging cycle for the availability. Since this is CFRA and network can ensure the suitable PRACH resource for reception without impact the possible deep sleep. In addition, there should be also a limit on the time window for such CFRA resource availability, so that network no needs to keep waiting, as shown in following figure. on the other side for CBRA, it can follow the rules in previous section for the general timeline of the availability.

Figure 3. Illustration of applicable time window for PDCCH order triggered CFRA.

Proposal 10:

- For PDCCH triggered CFRA:

  - The PRACH mask indication in the PDCCH order only applies to the additional PRACH resource when the 1 bit indication enables the additional PRACH resource;

  - The additional PRACH resource is available starting from a certain time after the PDCCH order and only available within a configured applicable time window.

- For PDCCH triggered CBRA, the availability and applicable timeline follows the design for CBRA in idle state.

## RA-RNTI related

During last meeting discussion, there is one potential point on the impact of RA-RNTI change in case of FDMed RO.

Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

Given the agreement already limited the FDMed valid RO is no more than 8, e.g., msg1_FDM_legacy + msg1_FDM_additional <=8, thus the calculation of RA-RNTI only needs to ensure the starting is counting from the first RO from legacy RO then to the additional RO. By doing this, there is no impact on the RA-RNTI calculation and also no impact on the legacy UE calculating the RA-RNTI. As shown in following figure, no matter what is the relative position of legacy RO and additional RO in F domain, if we start legacy first, their f_id can be kept as aligned with legacy without changing the calculation methods.

Figure 4. Illustration of applicable time window for PDCCH order triggered CFRA.

Proposal 11: The f_id in the RA-RNTI calculation for additional RO is starting from legacy FDMed RO to additionally FDMed RO, i.e., f_id for additional NES PRACH is:

- From 0 to msg1_FDMed_additional if there is no FDMed legacy RO

- From msg1_FDMed_legacy to msg1_FDMed_legacy + msg1_FDMed_additional if there is some FDMed legacy RO.

## 2step RACH related

There was some discussion on whether allow 2step RACH in the NES situation. In legacy, the 2step RACH was used for two purposes mainly. One is for quick access originally, e.g., make msg1+msg3 as msgA, the other is in the SDT discussion, e.g., in the RA-SDT, which allows UE to transmit “small data” without transferring to RRC connected model. In that sense, it seems beneficial to support 2step RACH in NES as well, so that network can have more sleep time in case of supporting SDT. However, we need to minimize the impact to specification given the remaining time is limited.

Observation 2: The 2step RACH is beneficial to support but the spec impact must be minimized.

Given the existing framework of 2step RACH, there are two aspects to look into:

- From resource perspective, in 2step RACH, the msgA PUSCH is configured refereeing to each RACH slot. In NES discussion, this can be inherited but only applied to the masked additional PRACH resource. Unless if applied to legacy resource it will be too wasted if the non-masked resource is also needed to be padded with PUSCH resource while spec doesn’t transmit PUSCH if there is no corresponding msgA PRACH transmission. The mapping ratio can be the similar way since the mask unit is SSB-RACH association pattern period, which is also the resource unit used in the mapping ratio calculation in legacy.

- From procedure perspective, it can also reuse the existing 2step RACH procedure, e.g., using RSRP to select 2step RACH and fallback to 4step RACH if failed a certain time. Only one additional thing is, if the resource for 2step RACH is indicated as unavailable before UE had tried enough attempts, the UE has to fallback to legacy RACH resource. But given the NES UE can use legacy and additional 4step RACH resource anyway, this is not an issue.

Proposal 12: In order to support 2step RACH, the msgA PUSCH is only configured for the masked additional PRACH resource.

# Conclusion

The contribution discusses adaptation of common signal/channel transmissions, and the proposals and observations are summarized below:

Proposal 1: Support explicit signalling of a SFN offset and a half frame index per configured periodicity for SSB adaptation, by higher layer parameters.

- A UE can determine the first possible SSB burst in the half frames determined by the SFN offset and half frame index.

Proposal 2: For the application delay of SSB adaptation indicated by a DCI format, support d =1.

Proposal 3: No need to introduce specification support for handling missed DCI for SSB adaptation.

Proposal 4: One from up to four K candidate values {2, 4, 8, 16}, K=1 if not configured.

Observation 1: Some UE might not be able to use the resource even it’s activated.

Proposal 5: RAN1 supports 1 bit activation status indication in SIB1 and validates from current SI modification, and network ensures the configured validity period to be aligned with remaining validity period from the DCI activation.

Proposal 6: RAN1 supports the value set for “Multiple of SI modification period” to be {1, 2, 3, 4}.

Proposal 7: Discuss and adopt one of the following for RRC_CONNECTED:

- Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that paging search space is configured in active DL BWP.

- Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

Proposal 8: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, DCI activates the additional PRACH resources configured in the active UL BWP.

Proposal 9: Paging DCI also indicate SUL and/or NUL in addition to 1 bit for activation.

Proposal 10:

- For PDCCH triggered CFRA:

  - The PRACH mask indication in the PDCCH order only applies to the additional PRACH resource when the 1 bit indication enables the additional PRACH resource;

  - The additional PRACH resource is available starting from a certain time after the PDCCH order and only available within a configured applicable time window.

- For PDCCH triggered CBRA, the availability and applicable timeline follows the design for CBRA in idle state.

Proposal 11: The f_id in the RA-RNTI calculation for additional RO is starting from legacy FDMed RO to additionally FDMed RO, i.e., f_id for additional NES PRACH is:

- From 0 to msg1_FDMed_additional if there is no FDMed legacy RO

- From msg1_FDMed_legacy to msg1_FDMed_legacy + msg1_FDMed_additional if there is some FDMed legacy RO.

Observation 2: The 2step RACH is beneficial to support but the spec impact must be minimized.

Proposal 12: In order to support 2step RACH, the msgA PUSCH is only configured for the masked additional PRACH resource.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”

- RAN1#116 Chair’s notes

- RAN1#116bis Chair’s notes

- RAN1#117 Chair’s notes

- RAN1#118 Chair’s notes

- RAN1#118bis Chair’s notes

- RAN1#119 Chair’s notes

- R4-2416913, LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN4.

- RAN1#120 Chair’s notes

- RAN1#120bis Chair’s notes

# Appendix

## RAN1#116 agreements

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration.

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

## RAN1#116-bis agreements

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

## RAN1#117 agreements

Agreement

For the study of adaptation of PRACH in spatial domain, following network energy savings gains were reported by sources based on the evaluation framework agreed in RAN1#116bis:

- Two sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs A1-1, zero load [R1-2404409, R1-2405107]

  - -4% ~ -45%

- Seven sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1/A1-2, zero load [R1-2404225, R1-2404185, R1-2404334, R1-2404123, R1-2404562, R1-2405107, R1-2405163]

  - 0% ~ 31%

  - Note: Five sources assumed that case B1 has same PRACH resources as case A1-2. Remaining two sources evaluated only A1-2.

  - Note: Three sources showed NES gains 0% ~ 10% [R1-2404225, R1-2404185, R1-2404334]

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 1.0%~8.8%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes.

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -48.41%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, CAT1 BS power model, for case C1 vs A1-2, zero load [R1-2404626]

  - 4.59%~38.04%

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Four sources showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1/A1-2, zero load [R1-2404562, R1-2404225, R1-2403943, R1-2404626]

  - 0% ~ 3.5%

  - Note: Three sources assumed that case B1 has same PRACH resources as case A1-2. One source evaluated only A1-2.

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 0%~0.2%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -1.19%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Two sources showed following NES gain for TDD, CAT1 or CAT2 BS power model, case C2 vs B2, zero load [R1-2403943, R1-2405107]

  - Less than 0.2%

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs A1-2 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 13.7%/8.7%/4.9%/2.6% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs B1 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 8.03%/5.1%/3.06%/1.74% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, C1 vs B1/A1-2, different loads [R1-2404562]

  - 16%/4.78% for light/medium cell load for CAT1 BS power model

  - 0.65%/0.29% for light/medium cell load for CAT2 BS power model

- One source showed following NES gain for TDD, C1 vs B1, different loads [R1-2404626]

  - -18.57%~0%/-2.52%~0% for low /medium cell load for CAT1 BS power model

  - -0.81%~0%/-0.42%~0% for low /medium cell load for CAT2 BS power model

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, C1 vs A1-2, different loads [R1-2404626]

  - 3.67%~19.88%/2.29%~5.22% for low /medium cell load for CAT1 BS power model

  - 0.67%~1.75%/0.39%~0.91% for low /medium cell load for CAT2 BS power model

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed NES gain for FDD, C1 vs B1, zero load [R1-2404464]

  - 1.4%~7% for CAT1 BS power model

  - 0%~0.3% for CAT2 BS power model

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed NES gain for FR2, CAT1 BS power model, spatial domain adaptation of PRACH configuration index 75 vs a time domain adaptation of PRACH configuration index 75, zero load [R1-2405163]

  - 4%~7%

- Note 1: About possibility of scenarios with non-uniform distribution of UEs in different beams

  - Several companies indicated (and three companies showed data/analysis) that there can be scenarios with non-uniform distribution of UEs in different beams.

  - Several companies mentioned that for non-uniform UE distribution, it can be addressed by gNB implementation e.g. by adjusting SSB beamwidth, etc. Several companies also mentioned that it is not clear how gNB can predict the distribution of UEs in different beams, especially for Idle/Inactive UEs.

- Note 2: Most sources that showed the NES gains (if any) for adaptation of PRACH in spatial domain compared to A1-2/B1 observed that the gain would be due to reduction in the number of overall ROs in time domain in their evaluations. Most of these companies only accounted for ROs in time domain.

- Note 3: The evaluation results assumed the non-uniform distribution of UE is static during the evaluation time period.

Conclusion

There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain

Agreement

For adaptation of PRACH in time-domain, support at least the following case(s)

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

- FFS: whether additional conditions are needed to support the above cases

- FFS: Additional case whether full/partial overlap in both time and frequency is allowed

- Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Note: Using Option 2 to realize Option 1 is not precluded

  - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

    - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities

- FFS: Details including applicable scenarios

- FFS: Support of Cell DTX for connected mode UEs for SSB

Agreement

For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least:

- a PRACH configuration index

- FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources

Study further the following

- When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource,

  - Additional parameter(s) for determining the additional PRACH resources e.g.

    - Scaled/adjusted PRACH configuration period

    - Additional timing offset

    - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Muting/masking ROs

- When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource

  - Additional mechanisms (if any) for determining the additional PRACH resources e.g.

    - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

- Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Agreement

For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

  - Strive to re-use existing DCI format(s)

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

  - FFS: details

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation

  - Strive to re-use existing DCI format(s)

- Option 5: Enhanced cell DRX

## RAN1#118 agreements

Agreement

- For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 1-1: Scaled/adjusted PRACH configuration period

    - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Opt 1-3: Muting/masking ROs

    - Opt 1-4: additional timing offset(s)

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources,

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

    - Opt 2-2: Additional timing offset(s)

- FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

- FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

Agreement

- Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

- For SSB-RO mapping rule for additional PRACH resources for Case 2.

- Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

Agreement

For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE),

- At least DCI based adaptation is supported. No introduction of new DCI format.

Agreement

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Agreement

For DCI-based adaptation for additional PRACH resources,

- Select from the following DCI format(s) to carry the adaptation indication.

  - DCI format 1_0

  - DCI format 2_7

  - DCI format 2_9

- FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options

- Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period

- Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period

- Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

## RAN1#118-bis agreements

Agreement

- For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Agreement

- For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Conclusion

There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).

- No spec impact

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

## RAN1#119 agreements

Agreement

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Agreement

Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

Agreement

Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Further update to be made based on RAN1#119 progress if any.

Agreement

At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

    - Alt 2-6: Network configured time period

Conclusion

There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Working Assumption

For DCI-based adaptation for additional PRACH resources,

- Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI

- Option 1: Use reserved bits in the DCI format

  - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format

- Option 2: Use Bits 5-8 within the Short Message (from upper layers)

  - Note: Availability should be confirmed by checking with RAN2.

- Option 3: Use bits available for both Option 1 and Option 2

FFS: Payload size for adaptation for additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - [DCI payload size = 1 bit]

  - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources

  - FFS: details

Agreement

Confirm the following working assumption from RAN1#118bis.

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

## RAN1#120 agreements

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

Conclusion

There is no RAN1 consensus to support the following in Rel-19

- New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

- New UE trigger

- Adapting the transmitted number of SSBs within a SSB burst

- Adaptation of the time domain positions of SSBs within a burst

Conclusion

There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell

Agreement

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Agreement

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources

- The mask is applicable at unit of

  - Alt 1: PRACH association period

  - Alt 2: PRACH association pattern period

  - Alt 3: SFN level

- The PRACH association period is determined based on valid additional ROs only.

- The mask is applied after valid RO determination and SSB-RO mapping.

- Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask.

- This is applicable at least for adaptation for DCI 1_0 with P-RNTI

- The DCI does not indicate PRACH mask selection

- FFS: how the mask is identified

  - Option 1: The PRACH mask is from a PRACH mask table

    - Pre-defined table with N=[4 or 8 or 16] rows

    - The semi-static signalling indicates a PRACH mask index

  - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

Agreement

Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE,

- Option 1: SFN of the first PF from the next I-DRX cycle

- Option 2: SFN of the first PF from the current I-DRX cycle

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period

- Option 5: From the first frame of the next SI modification period

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

## RAN1#120bis agreements

Agreement

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

Agreement

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

Agreement

For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI

- the validity duration is configured via higher layer signalling.

Agreement

For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of

- PRACH association period

- This PRACH mask applies to every [configured] K SSB RO association pattern period(s)

Agreement

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled a new RNTI,

  - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell

- the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter

- the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell

Agreement

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax=2

Agreement

Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported

- CB-PreamblesPerSSB

Agreement

LS on DCI-based PRACH adaptation endorsed with the ACTION part modified compared to draft LS in R1-2503085 as follows:

- ACTION: RAN1 respectfully asks RAN2 to confirm whether the use of above bit is feasible.

Final LS in R1-2503086.

Agreement

For adaptation of PRACH in time-domain, for a connected mode UE,

- One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

Working Assumption

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213].

- FFS: how to determine the first [possible] SSB burst

Agreement

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods)

  - For K: one from up to four candidate values {2,4,8, [1 or 16]}

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 1st quarter of the APs in K APPs |
| 2 | 1st eighth of the APs in K APPs |
| 3 | 1st sixteenth of the APs in K APPs |

Agreement

- For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI

- The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

  - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})