3GPP TSG RAN WG1 #121		R1-2503573

St Julian's, Malta, May 19th – 23th, 2025

Agenda item:	9.6.1

Source:	Samsung

Title:	Discussion on LP-WUS and LP-SS design

Document for:	Discussion and Decision

- Introduction

In WID for Rel-19 LP-WUS [1], objectives for LP-WUS and LP-SS design have been agreed as follow:

Objective for LP-WUS design - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported

Objective for LP-SS design - For IDLE/INACTIVE modes - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

In addition, there are notes for the target coverage and priority of LP-WUS signal design as follow:

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

This contribution discusses the potential issues for the design of LP-WUS/LP-SS based on the objectives.

- Common aspects

Configuration of frequency resource for LP-WUS/LP-SS

Regarding LP-WUS/LP-SS’s frequency resource configuration, the number of PRBs, SCS and the frequency location in relation to the initial DL BWP or active DL BWP were agreed. The remaining part to be determined is how to configure the frequency position of LP-WUS/LP-SS.

The following agreements have been made regarding the frequency resource configuration of LP-WUS and LP-SS.

Agreement For RRC idle/inactive, support Alt 1 for both Redcap and non-redcap UEs: - Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier. - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB Agreement For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP - Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP

In RRC IDLE/INACTIVE states, LP-WUS and LP-SS may be located outside the initial DL BWP. In RRC CONNECTED state, LP-WUS may be configured outside the active DL BWP depending on the UE capability. However, even if LP-WUS and LP-SS are placed outside the DL BWP used for triggered PDCCH monitoring, they are still located within the same carrier as the DL BWP. Therefore, when the network configures the frequency positions of LP-WUS and LP-SS, it can specify their locations based on the carrier boundary using the offset value. This offset can be set in units of PRBs, and the PRB offset can be determined relative to the carrier boundary defined by offsetToCarrier, regardless of the RRC state.

Meanwhile, unlike in RRC CONNECTED state, LP-SS may also be transmitted in RRC IDLE/INACTIVE states. In this case, it needs to be discussed whether LP-SS must always be transmitted at the same frequency location as LP-WUS, or whether different frequency locations should be allowed. Since reception of signals transmitted on different frequency positions may require frequency tuning or additional band-pass filters, it is preferable to align the frequency positions of the signals received by LP-WUR, particularly to minimize ON power for OOK-based LP-WUR which implies low power consumption. Therefore, if LP-SS transmission is configured, it would be reasonable to configure both LP-WUS and LP-SS frequency positions using a same PRB offset parameter.

Proposal 1: For LP-WUS/LP-SS frequency resource configuration, PRB offset with reference to carrier boundary determined by offsetToCarrier corresponding to SCS of LP-WUS/LP-SS is configured by the gNB.

- If LP-SS transmission is configured, frequency position for LP-WUS and LP-SS is configured by the same PRB offset parameter.

M value configuration

Another remaining aspect to transmit OOK waveform is to decide the M value, which is the number of OOK chips per 1 OFDM symbol. An issue raised in RAN1#119 is whether the M value for LP-WUS and LP-SS are always same or not, and the following agreement was made for M values larger than 1.

Agreement For the M value for LP-WUS and LP-SS, supports: - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - At least for M values larger than 1 (applies to both LP-WUS and LP-SS) - FFS: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both

When the M values of LP-WUS and LP-SS are larger than 1, even if the M values differ between the two signals as the agreement, the average power of the ON symbols can be maintained at the same level as long as they share the same EPRE. This allows for consistent average ON power between LP-WUS and LP-SS, which may offer advantages in terms of AGC adjustment and RRM measurements.

Figure 1. Relationship between EPRE configuration and M value for LP-WUS and LP-SS

However, as illustrated in Figure 1, under the assumption of the same EPRE, there is a 2x difference in average ON power between the case where M = 1 and the cases where M = 2 or 4. Specifically, when LP-WUS uses M = 1 while LP-SS uses a larger M value, even with the same EPRE, the average ON power differs by a factor of two. This discrepancy may negatively affect the accuracy of AGC and RRM-related procedures mentioned above.

One potential solution to ensure equal average ON power when LP-WUS uses M = 1 is to force LP-SS to use the same M value (i.e., M = 1) while maintaining the same EPRE for both LP-WUS and LP-SS. However, in this case, a UE would not be able to take advantage of better synchronization performance achievable with larger M values, such as M = 2, which would normally reduce overhead.

Alternatively, when LP-WUS uses M = 1, configuring LP-SS with a higher M value (e.g., 2 or 4) while still aligning their average ON power is possible by configuring different EPRE values for LP-WUS and LP-SS. For instance, as shown in Figure 1, if the EPRE of LP-WUS is set 3 dB higher than that of LP-SS, the average ON power can be equalized. Furthermore, when M = 1, a single OOK OFF symbol entirely mapped to one OFDM symbol would result in half the overall power compared to cases with M > 1. This inherently degrades performance.

Therefore, in order to prevent performance degradation when LP-WUS operates with M = 1 while still enabling enhanced LP-SS synchronization performance, it is desirable to allow flexible configuration of LP-SS with either the same or different M values relative to LP-WUS. In such cases, support for different EPRE configurations for LP-WUS and LP-SS is essential to maintain balanced average ON power and proposed in our companion paper for 9.6.2 [2].

Proposal 2: For LP-WUS with M=1, support configuring M values for LP-SS to be same or different.

In RAN1#118 meeting, the agreement was made related to the supported M values for OOK-4 with M>1 for 15kHz SCS as the follow:

Agreement Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4.

In addition, during RAN#105, the following conclusion was made:

Endorsed the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible No additional guidance in RAN#105 for RAN4 work and TU allocation

Since the supported M values and SCS are related to the OOK chip duration, the supported M values were discussed with SCS considering the performance of OOK waveform. As shown in the agreement, supported M value for 15kHz SCS and 30kHz SCS have been mainly discussed so far. However, 15kHz SCS and 30kHz SCS are only used for FR1.

According to the conclusion in RAN#105, FR2 band is also considered to design Rel-19 LP-WUS/WUR as well. In addition, the bandwidth for 60kHz/120kHz SCS was already agreed as 11PRBs. To support operation with 60kHz/120kHz SCS, the supported M values for each SCS used should be considered as well. In this case, to have the same LP-WUS/LP-SS design and performance as much as possible, the same maximum OOK chip rate can be considered for all SCSs. For example, if M=4 is only supported for 15kHz SCS, the maximum OOK chip rate for FR1 band can be 56kcps. Then, 60kHz SCS can support only for OOK-4 with M=1 or OOK-1. Otherwise, when M=4 is supported for 30kHz SCS, the maximum OOK chip rate for FR1 band can be 112kcps. M=1 (for 120kHz SCS), and M=1 and M=2 (for 60kHz SCS) can be considered as the supported M values. Because the performance based on higher pulse rate over 112kcps was not evaluated in study phase, applying same M values for 60kHz/120kHz SCSs compared to that for 15kHz/30kHz SCSs should be avoided.

Proposal 3: Consider limiting M value for 60kHz/120kHz to have the same maximum OOK chip rate with 15kHz/30kHz SCSs case (i.e., M =1, 2 for 60kHz SCS, M = 1 for 120kHz SCS).

- How to carry information of LP-WUS

LP-WUS information in RRC CONNECTED

Regarding the LP-WUS information, for RRC IDLE/INACIVE UEs, it was agreed to support codepoint based subgroup wake-up indication by LP-WUS as follow:

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.

For the common design on both RRC IDLE/INACTIVE and RRC CONNECTED, the working assumption for codepoint based LP-WUS transmission was also made:

Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8

According to the working assumption, multiple-to-one mapping for UE-to-codepoint has been proposed to minimize the drawbacks of codepoints. This approach helps reducing the resource overhead and the false alarm rate (FAR) caused by a one-to-one mapping for UE-to-codepoint. Additionally, to simplify the codepoint detection, the number of codepoints a single UE needs to check per MO has been limited to 8.

When a UE detects a codepoint, even if coding is applied, the length of the codeword corresponding to a single codepoint remains significantly shorter than the number of samples required for overlaid OFDM sequence detection. As a result, unless a UE is required to detect dozens or hundreds of codewords, the complexity is unlikely to increase significantly. Therefore, there is no strong need to consider a UE capability that checks fewer than eight codepoints. In fact, introducing different UE capabilities may complicate the scheduling for the gNB when mapping the waking UEs to their respective codepoints.

Meanwhile, the number of UEs waking up simultaneously depends on the LP-WUS MO periodicity, traffic characteristics, and the number of UEs per LP-WUS. Given these factors, rather than using a predefined rule to determine the mapping relationship between codepoints and waking UEs, a more straightforward approach would be allowing the gNB to configure up to 8 codepoint values per UE.

In addition, different words have been used such as codepoint, information bit, and raw information during the discussion. From our understanding, the codepoint value itself can be regarded as the (raw) information bit, which determines the overlaid OFDM sequences for OFDM-based LP-WUS and is further processed through RM coding to generate the codeword for OOK-based LP-WUS to indicate the specific wake-up information.

Proposal 4: Confirm the working assumption related to codepoint-based LP-WUS for RRC CONNECTED mode with the following modification:

- Delete “Depending on UE capability, a UE may support less than 8.”

- Add “Codepoint value set to be checked by a UE can be configured by the gNB.”

Proposal 5: Codepoint value is defined as the information bits/raw information bits to generate OOK-based and OFDM-based LP-WUS.

Code block length for OOK-based LP-WUS

In the last meeting, coding schemes for OOK-based LP-WUS to meet the requirement was agreed and the remaining part is to decide the candidate code block length after rate matching as shown in the following agreement:

Agreement For > 2 information bits with RM coding, support candidate code block (coded bits) length after rate matching within the below range: - Alt 1a: code block (coded bits) length: [3,4]~[32,64] - FFS whether all or a subset of lengths is selected. - FFS whether same range applies to all M values - FFS: above lengths do not consider repetition (if supported) after rate matching

The first issue concerns whether to support all code block lengths within the defined range or only a subset of code block lengths. Supporting the full range would only require specifying the minimum and maximum values; however, for the case where LP-WUS uses M = 4, additional considerations arise. Specifically, when the number of OFDM symbols required to transmit a single LP-WUS after applying Manchester coding does not result in an integer value, additional handling is necessary. For example, if the code block length after rate matching is set to 15, Manchester coding will produce 30 ON/OFF symbols. When M = 4 is used, this does not map evenly into OFDM symbols. Therefore, discussions are required on how to handle the remaining ON/OFF symbols in the last OFDM symbol.

To avoid this issue, defining a subset of code block lengths that offers sufficient flexibility for the gNB to configure code block lengths may be a more practical approach.

Proposal 6: The subset of code block lengths after rate matching should be supported considering the following:

- The number of OFDM symbols for a LP-WUS transmission after rate matching and Manchester coding should be the integer value (i.e., the number of OOK symbols after Manchester coding mod M should be zero).

The second issue is whether different subsets of code block lengths should be defined based on the M value or a single subset should be applied regardless of the M value. According to the current agreement, the minimum and maximum values for the code block length after rate matching are discussed to be 3 or 4 and 32 or 64, respectively.

Code block length affects both detection performance and resource overhead, and the M value, which determines the ON/OFF symbol duration, also significantly impacts these factors. When M is smaller, acceptable performance can be achieved without requiring long code block lengths, and longer lengths may only introduce unnecessary overhead. For example, when M = 1 and the code block length is 64, 128 OFDM symbols are required to transmit a single LP-WUS. In contrast, with M = 4, shorter ON/OFF symbol durations may degrade performance, which can be compensated by using longer code block lengths. Therefore, since the appropriate code block length depends on the M value, defining distinct subsets for each M value and selecting a code block length from each subset may be a reasonable option.

Alternatively, for the sake of simplicity, it may be preferable to define a single subset that applies regardless of the M value. As discussed, detection performance (i.e., coverage) and resource overhead have a trade-off relationship determined by the combination of M and the code block length. The total number of OFDM symbols required for LP-WUS transmission is ultimately derived from this combination. As the number of OFDM symbols increases, coverage improves while overhead decreases. Conversely, reducing the number of OFDM symbols lowers overhead but limits coverage. This relationship applies regardless of the M value. Therefore, if a subset is defined based on the number of OFDM symbols used for LP-WUS transmission rather than on the code block length itself, a single subset can be applied irrespective of the M value. Furthermore, based on the configured number of OFDM symbols provided by the gNB, the code block length after rate matching can be implicitly determined.

While both approaches are valid, the single subset configuration based on the number of OFDM symbols is preferred to simplify the discussion and reduce the time required to finalize the subset definition.

Proposal 7: To determine the code block length after rate matching for each M value, the following alternatives can be considered:

- Alt. 1: The different subset for the code block length is explicitly specified for each M value.

- Alt. 2: The same number of OFDM symbols are configured for a LP-WUS transmission regardless of M values. The code block length after rate matching can be implicitly derived from the number of OFDM symbols.

The last issue is whether to support additional repetition. If sufficient resources are allocated per MO, rate matching naturally provides a repetition effect. Thus, adding repetition on top of rate matching lacks a clear motivation.

Proposal 8: Do not support the additional repetition after rate matching.

- Repetition by rate matching is sufficient for intra-MO repetition.

- Overlaid OFDM sequence over OOK symbol

As specified in [1], OOK (On-Off Keying) with overlaid OFDM sequences over OOK symbol was decided as the waveform of LP-WUS. The intention to consider the LP-WUR type in [1] is that various LP-WUR type can be implemented according to use cases. According to the observation during SI in [3], OOK-based LP-WUS and OFDM-based LP-WUS had the trade-off relationship in terms of the benefits between power saving gain and coverage/resource overhead aspects. Therefore, if UEs can get the information by reception of harmonized LP-WUS which can be detected regardless the type of LP-WUR, the UEs can implement LP-WUR considering the performance of each LP-WUR type as illustrated in Figure 2.

Figure 2. Diagram for transmission and reception of OOK-based LP-WUS with overlaid OFDM sequences

For UEs who want to achieve better coverage at the expense of the power consumption, the information related to the overlaid OFDM sequence should be provided to the UEs. LP-WUR with I/Q branches detects whether ON symbol is transmitted based on the cross correlation between the received signal and the provided OFDM sequence. Otherwise, LP-WUR with the envelope detector detects whether ON symbol is transmitted based on the envelope of received signal. Thus, LP-WUR with the envelope detector and LP-WUR with I/Q branches are referred as OOK-based LP-WUR and OFDM-based LP-WUR in our tdoc, respectively. In addition, OFDM-based LP-WUR can detect existing PSS/SSS because it is assumed to have PLL and I/Q branches described in [3].

Observation 1: If overlaid OFDM sequences over OOK symbol are specified, OOK-based LP-WUS can be detected either by envelope detection or correlation with OFDM sequence according to the type of LP-WUR.

Compared to OOK-based LP-WUR, OFDM-based LP-WUR can detect one from multiple OFDM sequences within a single ON symbol. Therefore, it can detect multiple information bits according to the number of OFDM sequence candidates for each ON symbol. If multiple candidates of OFDM sequences are specified, OFDM-based LP-WUR can get whole information carried by OOK-based LP-WUS by only detecting some of OOK symbols. Therefore, it is not required for UE with OFDM-based LP-WUR to receive whole OOK-based LP-WUS, it leads to reduce the power consumption of LP-WUS monitoring for OFDM-based LP-WUR.

Observation 2: If multiple candidates of OFDM sequences are specified to be overlaid by ON pulse of OOK symbol, it is beneficial for OFDM-based LP-WUR to reduce the power consumption of LP-WUS monitoring.

To give the benefit for UEs with OFDM-based LP-WUR, the case of overlaid OFDM sequence carrying information as well as the case of overlaid OFDM sequence not carrying information.

Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

In addition, the number of candidates overlaid sequences and the sequence configuration were determined by the following agreements.

Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. Agreement For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips

Agreement For the root(s) used for overlaid OFDM sequences, gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC, where LZC is the overlaid OFDM sequence length. Working Assumption At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for each M value). - Different roots can be picked for different M values.

The remaining part is how to configure root and cyclic shift values for overlaid OFDM sequence candidates. To configure the root values and cyclic shift values used for LP-WUS, the gNB must first configure the number of overlaid OFDM sequences to be applied to each ON symbol. The agreement described above defines the maximum number of candidates overlaid OFDM sequences per M value. Based on this, the gNB may configure either the maximum number or a lower value to indicate how many candidates for overlaid OFDM sequences can be transmitted for each ON symbol.

Agreement For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1 and Alt2: - Alt 1: Raw information bits are mapped to sequence(s) - N raw information bits are divided into K segments from MSB to LSB, where K= ceil (N/log2L), L is the number of candidate overlaid OFDM sequences for one OOK ON chip. - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2. - In case N/log2L is not an integer, Bit 0 as MSB is used for padding

However, according to the agreement, log2(L) should be an integer to enable natural mapping between segments of information bits and overlaid OFDM sequences. This means that the number of candidates overlaid OFDM sequences (L) must be configured as a power of two. As a result, rather than configuring the L value directly, the gNB can instead configure the number of bits per segment (N1), which represents the number of information bits to be carried by each overlaid OFDM sequence.

In addition to N1, the gNB must configure the number of roots (N2) and their corresponding root values to enable the UE to determine the actual overlaid OFDM sequences being transmitted. While the cyclic shift values are also required to identify the candidates, they do not need to be explicitly configured by the gNB. Instead, they can be derived from the configured N1 and N2 values. Specifically, since the total number of overlaid OFDM sequences is 2N1, the number of cyclic shifts required per root can be calculated as (2N1)/N2. Furthermore, the actual cyclic shift values can be determined by dividing the length of ZC sequence (Bzc) evenly to ensure a wide sliding detection window. Accordingly, the cyclic shift spacing can be calculated as floor(Bzc×(N2 /2N1)). As shown, these values can be implicitly determined based on the other configured parameters.

There is also a discussion point regarding whether to apply the same candidate set to all ON symbols or allow different candidate sets per symbol. If different candidate sets are applied per ON symbol, this would increase UE detection complexity and require the gNB to generate more sequence sets. Furthermore, even when transmitting the same segmented information, different mappings across ON symbols would add further complexity. Therefore, it is considered reasonable to apply a single candidate set, configured by the gNB, consistently across all ON symbols.

According to the previous agreement, both cases (where the overlaid OFDM sequences are carrying information and not carrying information) are covered. Whether the gNB supports the information-carrying case can be determined by the UE based on the configured number of overlaid OFDM sequences, specifically the value of N1. When N1 is set to 0, only a single root (N2 = 1) is used and the number of cyclic shift is equal to 1, meaning that only one sequence is configured. This can be interpreted as a non-information-carrying case.

Proposal 9: The number of information bits per OOK ON symbol (N1), the number of roots(N2), and root index(s) are configured by the gNB.

- The number of cyclic shift ((2N1)/N2) and corresponding cyclic shift values can be derived implicitly.

- The single candidate set for overlaid OFDM sequence is applied across OOK ON symbol for a LP-WUS.

- If the number of information bits per OOK ON symbol is 0 (N1=0), the single overlaid OFDM sequence is configured for each OOK ON symbol (not carrying information case).

On the other hand, UEs with OFDM-based LP-WUR benefit from superior detection performance, which allows raw information bits to be directly mapped onto overlaid OFDM sequences. Unlike OOK-based LP-WUR, which requires detection of the entire LP-WUS transmission, OFDM-based LP-WUR may enable early termination after detecting only a portion of the ON symbols, allowing the receiver to return to sleep mode sooner.

While OFDM-based LP-WUR does not necessarily require coding schemes such as RM coding, as used for OOK-based LP-WUS, its performance may vary depending on implementation factors, such as receiver noise figure (NF). To accommodate different implementations, repetition of raw information transmission can be considered as an alternative approach.

In this scheme, raw information bits can be repeated across multiple ON symbols, and each OFDM-based LP-WUR can determine whether to utilize repetition based on its own NF. For example, assuming a 3 dB performance difference between two types of OFDM-based LP-WUR, the gNB may configure a repetition factor of 2. In this case, a high-performance LP-WUR with lower NF may decode the information after receiving the raw bits only once, while a lower-performance LP-WUR may rely on two receptions to successfully decode the information.

If repetition is not explicitly configured and the gNB is forced to repeat raw information bits across all ON symbols, aligning the rate-matched bit size of OFDM-based LP-WUS with that of OOK-based LP-WUS, it would be possible for UEs to implement partial detection. However, from the gNB perspective, this would require transmitting all ON symbols even in scenarios where only OFDM-based LP-WUR is targeted for wake-up, resulting in inefficient resource usage. Therefore, this approach should be avoided.

Instead, the gNB should be able to configure the repetition factor (R) for the overlaid OFDM sequences. Based on the configured R and the number of segments (K), UEs can derive how many ON symbols are actually used to carry information for OFDM-based LP-WUS and determine the detection duration accordingly.

Proposal 10: Support repetition of overlaid OFDM sequence.

- The repetition factor (R) is configured by the gNB.

- The monitoring window for OFDM-based LP-WUS can be determined based on the configured repetition factor (R) and the number of segments (K).

- Design of LP-SS

During the previous discussion, it was decided to support multiple binary sequences to differentiate LP-SS from different cells. To specify the multiple binary sequence for LP-SS, the following agreements for LP-SS were made earlier:

Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration

Regarding LP-SS transmission from a cell, because a single binary sequence from the multiple binary sequence candidates will be used for a cell, how to determine the binary sequence for the cell was discussed. At least, explicitl configuration about the information for determining the sequence is already supported and the remaining FFS is whether to support additional predefined rule without configuration. According to the RAN2 discussion, it was already agreed that at least LP-SS related configuration is included in SIB. Therefore, the LP-SS binary sequence for the cell also can be included in that SIB and UEs in RRC IDLE/INACTIVE state should obtain the LP-WUS/LP-SS related SIB before entering LP-WUS operation. Thus, there is no clear reason to support predefined rule without configuration in terms of determining LP-SS binary sequence.

Proposal 11: For the LP-SS binary sequence used in a cell, additional support of sequence determination by predefined rule is not needed.

In the last meeting, the supported sets for binary sequences were determined. Additionally, the supported periodicity of LP-SS for LP-WUS with M=1,2 was agreed as the follow:

Agreement Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support LP-SS periodicity of 160ms

The reason why no agreement has been reached for the case where LP-WUS uses M = 4 is due to the tighter timing accuracy required for LP-WUS with M = 4 compared to the performance achievable with LP-SS using M = 4. According to the [4], when the LP-WUS M value is 1 or 2, and the LP-SS periodicity is set to 320ms or 160ms, the LP-WUR can meet the time error tolerances defined in [3] (i.e., 5 µs for M = 1 and 3 µs for M = 2), even with an RTC-based frequency error of 10ppm. However, when LP-WUS uses M = 4, the required 1 µs time error tolerance cannot be met under any conditions, even with the frequency error of 5ppm.

A straightforward way to reduce the total timing error for M=4 is to introduce a shorter LP-SS periodicity. However, as shown in [4], even adopting an 80ms LP-SS periodicity fails to meet the required tolerance. Therefore, to achieve comparable performance with other M values, the LP-SS period would need to be reduced to below 80ms. Moreover, since LP-SS must always be transmitted by the gNB, this would significantly increase the overhead burden on the gNB. To avoid such a straightforward but inefficient approach, alternative solutions should be considered to compensate for the performance gap of LP-WUS with M = 4 without simply reducing the LP-SS periodicity.

For instance, the gNB can configure the longer LP-WUS code block length after rate matching for M=4 to compensate the performance degradation due to timing error. Since more OOK symbols can be transmitted per OFDM symbol in this case, the overhead burden resulting from the longer code block length can be considerably reduced compared to cases with smaller M values of LP-WUS. Notably, unlike LP-SS, which must always be transmitted, LP-WUS is only sent when paging occurs. As a result, the additional overhead introduced for performance compensation can be minimized based on the paging probability per UE and the number of UEs in the cell. Therefore, rather than introducing the smaller periodicity than 160ms or designing an additional synchronization signal, it is preferable to compensate for excess timing errors by leveraging LP-WUS coding and rate matching under the same LP-SS periodicities with M=1 and M=2 cases.

Proposal 12: For LP-WUS M=4 case, support Option 1A with additional support for 160ms LP-SS periodicity (same option with M=1 and M=2 LP-WUS cases).

- Conclusion

The proposals and observation made in this contribution are summarized below:

Observation 1: If overlaid OFDM sequences over OOK symbol are specified, OOK-based LP-WUS can be detected either by envelope detection or correlation with OFDM sequence according to the type of LP-WUR.

Observation 2: If multiple candidates of OFDM sequences are specified to be overlaid by ON pulse of OOK symbol, it is beneficial for OFDM-based LP-WUR to reduce the power consumption of LP-WUS monitoring.

Proposal 1: For LP-WUS/LP-SS frequency resource configuration, PRB offset with reference to carrier boundary determined by offsetToCarrier corresponding to SCS of LP-WUS/LP-SS is configured by the gNB.

- If LP-SS transmission is configured, frequency position for LP-WUS and LP-SS is configured by the same PRB offset parameter.

Proposal 2: For LP-WUS with M=1, support configuring M values for LP-SS to be same or different.

Proposal 3: Consider limiting M value for 60kHz/120kHz to have the same maximum OOK chip rate with 15kHz/30kHz SCSs case (i.e., M =1, 2 for 60kHz SCS, M = 1 for 120kHz SCS).

Proposal 4: Confirm the working assumption related to codepoint-based LP-WUS for RRC CONNECTED mode with the following modification:

- Delete “Depending on UE capability, a UE may support less than 8.”

- Add “Codepoint value set to be checked by a UE can be configured by the gNB.”

Proposal 5: Codepoint value is defined as the information bits/raw information bits to generate OOK-based and OFDM-based LP-WUS.

Proposal 6: The subset of code block lengths after rate matching should be supported considering the following:

- The number of OFDM symbols for a LP-WUS transmission after rate matching and Manchester coding should be the integer value (i.e., the number of OOK symbols after Manchester coding mod M should be zero).

Proposal 7: To determine the code block length after rate matching for each M value, the following alternatives can be considered:

- Alt. 1: The different subset for the code block length is explicitly specified for each M value.

- Alt. 2: The same number of OFDM symbols are configured for a LP-WUS transmission regardless of M values. The code block length after rate matching can be implicitly derived from the number of OFDM symbols.

Proposal 8: Do not support the additional repetition after rate matching.

- Repetition by rate matching is sufficient for intra-MO repetition.

Proposal 9: The number of information bits per OOK ON symbol (N1), the number of roots(N2), and root index(s) are configured by the gNB.

- The number of cyclic shift ((2N1)/N2) and corresponding cyclic shift values can be derived implicitly.

- The single candidate set for overlaid OFDM sequence is applied across OOK ON symbol for a LP-WUS.

- If the number of information bits per OOK ON symbol is 0 (N1=0), the single overlaid OFDM sequence is configured for each OOK ON symbol (not carrying information case).

Proposal 10: Support repetition of overlaid OFDM sequence.

- The repetition factor (R) is configured by the gNB.

- The monitoring window for OFDM-based LP-WUS can be determined based on the configured repetition factor (R) and the number of segments (K).

Proposal 11: For the LP-SS binary sequence used in a cell, additional support of sequence determination by predefined rule is not needed.

Proposal 12: For LP-WUS M=4 case, support Option 1A with additional support for 160ms LP-SS periodicity (same option with M=1 and M=2 LP-WUS cases).

Reference

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

[2] R1-2503574, Discussion on LP-WUS operation in IDLE/INACTIVE modes (RAN1#121), Samsung

[3] 3GPP TR 38.869, Study on low-power wake-up signal and receiver for NR.

[4] R1-2503099, Final summary of discussion on LP-WUS and LP-SS design (RAN1#120bis), vivo