3GPP TSG RAN WG1 #121		R1-2503575

St Julian's, Malta, 19th – 23rd May, 2025

Agenda item:	9.6.3

Source:	Samsung

Title:	Discussion on LP-WUS operation in CONNECTED modes

Document for:	Discussion and Decision

# Introduction

Agreements and conclusions made in RAN1#121 [1] regarding the LP-WUS operation for RRC_CONNECTED mode were summarized in the Appendix. This contribution discusses the further details on supporting LP-WUS operation in RRC_CONNECTED mode.

# Configuration for LP-WUS Monitoring Occasion(s)

In RAN1#120bis, configurations for LP-WUS MO(s) for both Option 1-1 and Option 1-2 were further discussed, and there is one remaining issue for further discussion in Option 1-2.

Figure 1 Illustration of configurations for LP-WUS MOs in Option 1-2.

For Option 1-2, as shown in Figure 1, one or multiple MOs can be supported per periodicity, and there is a FFS on how to determine the time domain occasions when multiple MOs are supported. Similar to MO configuration in Option 1-1 and MO configuration in RRC_IDLE/INACTIVE mode, supporting a configurable window would improve the flexibility on the time domain locations of the MOs, and may also save the UE power by properly configuring the relationship between the periodicity and window duration for monitoring MOs. In this sense, we prefer to support a configurable time domain interval between neighboring MOs in the multiple MOs, wherein the time domain interval is provided by an additional RRC parameter.

Proposal 1: For MO configuration in Option 1-2, support a configurable time interval between two consecutive MOs by RRC, when multiple MOs are configured per periodicity.

# LP-WUS Monitoring

For Option 1-2, the UE behavior on LP-WUS monitoring after the reception of the wake-up indication needs to be further clarified. For example, as shown in Figure 2, after a UE receives a wake-up indication (e.g., in the blue MOs in the figure), the UE may start the new timer after the time offset4, wherein within the offset4, there can be additional MOs (e.g., the red MOs in the figure) associated with other new timer. If the UE continues to monitor the MOs within the offset4, the UE behaviour on the time duration for PDCCH monitoring after wake-up would be confusing. Hence, we propose to stop LP-WUS monitoring in the MOs within the time offset4.

Figure 2 Illustration of LP-WUS monitoring in Option 1-2.

Proposal 2: For Option 1-2, after the reception of wake-up indication, a UE does not need to monitor LP-WUS in the remaining MOs within the time offset4.

# Gap between LP-WUS and PDCCH monitoring

In RAN1#120bis, further agreement on the gap between LP-WUS and PDCCH monitoring was made, wherein 3 candidate values were agreed to be reported from and the exact value are up to further discussion. In particular, advanced LP-WURs that are capable of overlaid OFDM sequence detection will be able to detect the LP-WUS at an earlier point in time, comparing to the LP-WURs that are only capable of receiving OOK waveform. In this sense, different set of candidate values shall be considered for different types of LP-WURs.

Meanwhile, for UE assistant information, the preferred value of the gap shall also be selected from the same set of candidate values for the UE capability, and the preferred value shall be larger than the reported UE capability.

Proposal 3:

- For the time gap between the last symbol of LP-WUS and MR to start PDCCH monitoring, support a set of 3 candidate values per LP-WUR type:

  - For OOK-based LP-WUR, the candidate values are {6, 20, 42} ms;

  - For OFDM-based LP-WUR, the candidate values are {3, 10, 33} ms;

- Candidate values for the indicated preferred gap value by UAI shall be same as the candidate values for the UE capability reporting.

# QCL source for LP-WUS

In RAN1#120bis, further agreements on the QCL source for LP-WUS were made, wherein two alternatives are considered for down-selection when Rel-15 non-unified TCI framework is used, e.g., when a UE is not capable of Rel-17 unified TCI framework or capable but not configured with Rel-17 unified TCI framework.

For Rel-15 non-unified TCI framework, the UE can be provided with a CORESET ID, wherein the CORESET is associated with a configured TCI state. This type of semi-static configuration of the TCI source could be sufficient for LP-WUS, since a dynamic one can be supported already by applying the Rel-17 TCI framework.

Proposal 4: When a UE is not capable of Rel-17 unified TCI framework or capable but not configured with Rel-17 unified TCI framework, support RRC providing the CORESET ID that UE shall derive the active TCI state for LP-WUS (Alt 1).

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: For MO configuration in Option 1-2, support a configurable time interval between two consecutive MOs by RRC, when multiple MOs are configured per periodicity.

Proposal 2: For Option 1-2, after the reception of wake-up indication, a UE does not need to monitor LP-WUS in the remaining MOs within the time offset4.

Proposal 3:

- For the time gap between the last symbol of LP-WUS and MR to start PDCCH monitoring, support a set of 3 candidate values per LP-WUR type:

  - For OOK-based LP-WUR, the candidate values are {6, 20, 42} ms;

  - For OFDM-based LP-WUR, the candidate values are {3, 10, 33} ms;

- Candidate values for the indicated preferred gap value by UAI shall be same as the candidate values for the UE capability reporting.

Proposal 4: When a UE is not capable of Rel-17 unified TCI framework or capable but not configured with Rel-17 unified TCI framework, support RRC providing the CORESET ID that UE shall derive the active TCI state for LP-WUS (Alt 1).

# References

[1] 3GPP RAN1#120bis chairman notes, April, 2025.

# Appendix

Agreement

- For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS can be configured only on PCell

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group

Conclusion

From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers PDCCH monitoring according to the legacy UE behaviour

Conclusion

For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19.

- Alt1

  - Basic capability

    - LP-WUS can be configured only on PCell

    - LP-WUS indication is applicable to all serving cells of all DRX groups.

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Advanced capability

    - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group

    - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

    - This capability has prerequisite of the above basic capability

- Alt2

  - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group

  - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

    - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

Agreement

Update previous agreement in red:

Agreement

For LP-WUS MOs in connected mode for Option 1-1, support Approach 1:

- LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

- Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start

  - UE monitors the first Z LP-WUS MO(s) at or after Time offset2.

    - FFS: How to determine the value of Z is RRC configured

  - Time offset2 is RRC configured

UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined given based on the minimum time gap reported by UE capability.

Agreement

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-2, support Approach 1:

  - LP-WUS MOs, including a periodicity and a time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - FFS one or multiple MOs per periodicity

    - UE is configured with a number of MOs for the periodicity by RRC

  - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS

    - FFS: Definition of time offset4

    - Alt1:

      - TheA time offset4 configured by the network indicating a time from the first LP-WUS MO per periodicity, after which the UE starts PDCCH monitoring via starting the new timer.

      - The time gap from the last LP-WUS MO per periodicity and the start of new timer is not smaller than the minimum time gap given based on UE capability

Agreement

For the UE capability report on the minimum time gap is between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, support X=3 candidate values corresponding to {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3.

- FFS: Whether to support different set of values for different receiver types

Conclusion

There is no consensus in RAN1 on whether to introduce any enhancements of PDCCH skipping indication for the interaction with LP-WUS in RRC connected mode in Rel-19.

Agreement

For RRC CONNECTED mode when LP-WUS is configured with Cell DTX, during Cell DTX inactive time, the UE is not expected to monitor LP-WUS both for Option 1-1 and 1-2

Agreement

The case where a UE does NOT supports Rel-17 unified TCI framework but supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode, select one of the following in RAN1#121 for this case:

- Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS

- Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them

The case where a UE supports Rel-17 unified TCI framework and supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode,

- When Rel-17 unified TCI framework is configured, LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals (no change to DCI format)

- When Rel-17 unified TCI framework is NOT configured, select one of the following in RAN1#121 for this case

  - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS

  - Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them

Same alternative to be chosen for both cases.

Agreement

Update previous agreement in red:

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable