3GPP TSG RAN WG1 #121			R1-2503576

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.7.1

Title:	Discussion on ISAC Deployment Scenarios

Source:	Samsung

Document for:	Discussion and Decision

- Introduction

The Rel-19 study items on “Study on channel modelling for integrated sensing and communications (ISAC) for NR” is endorsed in RAN#103 [1]. The objectives for this SI are shown below:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In this contribution, we discuss deployment scenario related aspects for Rel-19 ISAC channel model SI and agreements in the last RAN1 meeting.

- Remaining considerations

2.1 Evaluation parameters

In last meetings, RAN1 had reached agreement on discussion regarding evaluation parameters for each sensing target. However, some parameter remains optional. For instance, 3D distribution, 3D mobility, minimum distance between targets, etc. On the other hand, calibration requires aggregation and verification of statistical results of calibration metrics from companies. Since different options can yield different statistical results, RAN1 needs to decide on one unified option for each parameter to facilitate consistent calibration.

In Metrics for full calibration, the coupling loss get results the complex values even though the calibration assumed the single dual-pol isotropic antenna. Therefore, in order to get drawn the CDF, the coupling loss should be absolute values (magnitude) such as

Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pT CPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pFtx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p

The coupling loss get results the complex values even though the calibration assumed the single dual-pol isotropic antenna

RAN1 consider the absolute value of coupling loss to derive CDF

2.2 RCS application for vehicle, UAV with large size and human type 2 in full calibration

In ISAC study, RCS relevant parameter σRCS of a SPST for a pair of incident/scattered angles is composed of a first component σM which is included in the large-scale parameters, and a second component σD and third component σS which are both included in the small-scale parameters, i.e., σRCS=σMσDσS. During calibration, when calculating coupling loss, large-scale calibration considers σM within the power scaling factor. However, for large-sized objects such as vehicles, UAVs, and human types, full calibration involves generating the angular-dependent RCS component within the channel coefficient, which is represented as σMσD, and this is applied along with σS to the path power. In this scenario, if the total coupling loss is calculated, σM may be inadvertently applied twice. Therefore, RAN1 needs to clarify whether σMshould be applied to either the large-scale or small-scale elements exclusively, or if it can be applied to both without restriction.

When calculating coupling loss, large-scale calibration considers σM within the power scaling factor. However, for large-sized objects such as vehicles, UAVs, and human types, full calibration involves generating the angular-dependent RCS component within the channel coefficient, which is represented as σMσD, and this is applied along with σS to the path power

RAN1 clarify whether σMshould be applied to either the large-scale or small-scale elements exclusively, or if it can be applied to both without restriction

2.3 EO type 2 modelling per scenario

In the last meeting, a table for evaluation parameters were agreed for all sensing targets which is in the study scope. These evaluation parameters will serve as important guidelines for feasibility/performance evaluation and channel calibration. Moreover, it is crucial to consider what types of EOs can exist in each deployment scenario for each target.

EOs can be categorized into two main types: EO type-1 with physical characteristics similar to those of sensing targets or same with sensing targets, and EO type-2 which are extremely large in size.

As a starting point of discussion, EO type-1 could be surrounding targets specified for each sensing target. However, further discussion on the types and sizes of EO type-2 are necessary. Fig. 1 shows the length and width of a hexagonal cell for both UMa and UMi. As an example, for EO type-2, major objects could be buildings. The shape of buildings defined by 3GPP represents the cell layout of an Urban Grid scenario, considering the inter-site distance (ISD) of an UMa, resulting in a 2D size of 433 m x 250 m. However, EO type-2 should not be limited to simply being a cell layout element but rather play the role of deterministic clutters within channels. For instance, EO type-2s may take the form of buildings. Nonetheless, these forms of EO type-2 should no vary across different scenarios but rather have standardized shapes applicable to all scenarios. Therefore, the width and length of EO type-2 can be determined based on UMi, which has the smallest cell layout among outdoor scenarios. Considering examples such as a width of ISD*13*14 and length of ISD*13*32*14. Alternatively, typical building sizes might be suggested. The key point here is that EO type-2 should be distributed within cells so they can function as deterministic clutters.

| (a) UMa | (b) UMi |
| --- | --- |

Fig. 1 - Example for cell layout

For EO type-2, major objects could be buildings. The shape of buildings defined by 3GPP represents the cell layout of an Urban Grid scenario, considering the inter-site distance (ISD) of an UMa, resulting in a 2D size of 433 m x 250 m, However, EO type-2 should not be limited to simply being a cell layout element but rather play the role of deterministic clutters within channels.

EO type-2s may take the form of buildings. Nonetheless, these forms of EO type-2 should no vary across different scenarios but rather have standardized shapes applicable to all scenarios.

RAN1 consider the sensing target as EO type-1 for each sensing target scenario in unintended/environment object category

RAN1 discuss the detailed shape and size for EO type-2 in unintended/environment object category

- Calibration Results

3.1 UAV

Based on the agreed UAV-related parameters, large scale and full calibration of the target channel using TRP monostatic / TRP-TRP bistatic sensing modes were performed in the FR1.

Large scale calibration

| TRP monostatic: Coupling loss | TRP-TRP bistatic: Coupling loss |
| --- | --- |

Full calibration

| TRP monostatic: Coupling loss | TRP-TRP bistatic: Coupling loss |
| --- | --- |

| TRP monostatic: Delay spread | TRP-TRP bistatic: Delay spread |
| --- | --- |

| TRP monostatic: ASD | TRP-TRP bistatic: ASD |
| --- | --- |

| TRP monostatic: ZSD | TRP-TRP bistatic: ZSD |
| --- | --- |

| TRP monostatic: ASA | TRP-TRP bistatic: ASA |
| --- | --- |

| TRP monostatic: ZSA | TRP-TRP bistatic: ZSA |
| --- | --- |

3.2 Vehicle

Based on the agreed Automotive-related parameters, large scale and full calibration of the target channel using TRP monostatic / TRP-TRP bistatic sensing modes were performed on Urban Grid scenario in the FR1. In here, we assumed the vehicle as one scattering point.

Large scale calibration

| TRP monostatic: Coupling loss | TRP-TRP bistatic: Coupling loss |
| --- | --- |

Full calibration

| TRP monostatic: Coupling loss | TRP-TRP bistatic: Coupling loss |
| --- | --- |

| TRP monostatic: Delay spread | TRP-TRP bistatic: Delay spread |
| --- | --- |

| TRP monostatic: ASD | TRP-TRP bistatic: ASD |
| --- | --- |

| TRP monostatic: ZSD | TRP-TRP bistatic: ZSD |
| --- | --- |

| TRP monostatic: ASA | TRP-TRP bistatic: ASA |
| --- | --- |

| TRP monostatic: ZSA | TRP-TRP bistatic: ZSA |
| --- | --- |

- Conclusion

- The coupling loss get results the complex values even though the calibration assumed the single dual-pol isotropic antenna

- RAN1 consider the absolute value of coupling loss to derive CDF

- When calculating coupling loss, large-scale calibration considers σM within the power scaling factor. However, for large-sized objects such as vehicles, UAVs, and human types, full calibration involves generating the angular-dependent RCS component within the channel coefficient, which is represented as σMσD, and this is applied along with σS to the path power

- RAN1 clarify whether σMshould be applied to either the large-scale or small-scale elements exclusively, or if it can be applied to both without restriction

For EO type-2, major objects could be buildings. The shape of buildings defined by 3GPP represents the cell layout of an Urban Grid scenario, considering the inter-site distance (ISD) of an UMa, resulting in a 2D size of 433 m x 250 m, However, EO type-2 should not be limited to simply being a cell layout element but rather play the role of deterministic clutters within channels.

EO type-2s may take the form of buildings. Nonetheless, these forms of EO type-2 should no vary across different scenarios but rather have standardized shapes applicable to all scenarios.

RAN1 consider the sensing target as EO type-1 for each sensing target scenario in unintended/environment object category

RAN1 discuss the detailed shape and size for EO type-2 in unintended/environment object category

Appendix. Simulation assumption for calibration

Table 1. Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm |
| Bandwidth | FR1: 100MHz |
| BS noise figure | FR1: 5dB |
| UT antenna configurations | - |
| UT noise figure | - |
| UT height | - |
| UT Tx power | - |
| UT Distribution | - |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | -12.81 dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component A included): LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss for target channel Note: CDFs can be separately generated for target channel, background channel |

Table 2. Simulation assumptions for full calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm |
| Bandwidth | FR1: 100MHz |
| BS noise figure | FR1: 5dB |
| UT antenna configurations | - |
| UT noise figure | - |
| UT height | - |
| UT Tx power | - |
| UT Distribution | - |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| RCS for each scattering point | Component A: -12.81 dBsm Component B1: 0 dB Component B2: 3.74 dB for standard deviation The same values are used for monostatic RCS and bistatic RCS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Fast fading model | TR 36.777 Annex B.1.3 |
| (u, std) for XPR of target | Mean 13.75 dB, deviation 7.07 dB |
| Coupling loss for target channel | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Sensing Tx/Rx selection | Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel. |
| Absolute delay | The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref] is reused for UMa-AV for all sensing modes. |
| Metrics | Coupling loss for target channel Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996. |

Table 3. Simulation assumptions for large scale calibration for Automotive sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | For FR1: Urban Grid (ISD=500m, BS height=25m) |
| Sensing mode | TRP monostatic, TRP-TRP bistatic |
| Target type | Vehicle type 2 [TR37.885] |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm |
| Bandwidth | FR1: 100MHz |
| BS noise figure | FR1: 5dB |
| UT antenna configurations | - |
| UT noise figure | - |
| UT height | - |
| UT Tx power | - |
| UT Distribution | - |
| Sensing target distribution | Per TR37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline. NOTE: vehicle is dropped with 5 scattering points (front/left/right/back/roof) and each point has one location, or vehicle is dropped with 1 scattering points |
| Component A of the RCS for each scattering point | -20dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | As defined in urban grid/highway scenario |
| Coupling loss for target channel | Power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N= Tx-Rx pairs to be selected for the target. For urban grid N = 4 For Highway N = 4 NOTE: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss for target channel Note: CDFs can be separately generated for target channel, background channel |

Table 4. Simulation assumptions for full calibration for Automotive sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | For FR1: Urban Grid (ISD=500m, BS height=25m) |
| Sensing mode | TRP monostatic, TRP-TRP bistatic |
| Target type | Vehicle type 2 [TR37.885] |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm |
| Bandwidth | FR1: 100MHz |
| BS noise figure | FR1: 5dB |
| UT antenna configurations | - |
| UT noise figure | - |
| UT height | - |
| UT Tx power | - |
| UT Distribution | - |
| Sensing target distribution | Per TR37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline. NOTE: vehicle is dropped with 5 scattering points (front/left/right/back/roof) and each point has one location, or vehicle is dropped with 1 scattering points. In the case of vehicle with 5 scattering points, spatial consistency is enabled with the following assumptions: - The correlation for LOS/NLOS condition of the 5 points is assumed equal to 1. LOS/NLOS condition can be calculated based on the distance of the Tx/Rx to the centroid of the ST, then apply the LOS/NLOS condition to each of the 5 points. - The correlation for stochastic cluster paths of the 5 points is assumed equal to 1. The stochastic cluster paths can be calculated between the Tx/Rx and the centroid of the ST, then the stochastic cluster paths are added to each of the 5 points. |
| RCS for each scattering point | Based on the agreed RCS. |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | As defined in urban grid/highway scenario |
| Fast fading model (full calibration) | Procedures based on 37.885 |
| (u, std) for XPR of target | (21.12, 6.88) dB. |
| Coupling loss for target channel | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222 ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Sensing Tx/Rx selection | Best N= Tx-Rx pairs to be selected for the target. For urban grid N = 4 For Highway N = 4 NOTE: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. |
| Absolute delay | The model of UMa scenario defined in TR 38.901 7-24GHz channel modelling [Table 7.6.9-1] is reused for Urban Grid for all sensing modes. |
| Metrics | Coupling loss for target channel Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996. |

References

RP-240799 “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR,” RAN#103

TR38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”

TR36.777 “Study on enhanced LTE support for aerial vehicles (Release 15)”

TR37.885 “Study on evaluation methodology of new vehicle-to-everything (V2X) use cases for LTE and NR (Release 15)”

TR22.137 “Service requirement for integrated sensing and communication; Stage 1 (Release 19)”

TR22.837 “Feasibility study on integrated sensing and communication (Release 19)”

R1-2400746 “Discussion on ISAC Deployment Scenarios”, Samsung

R1-2402478 “Discussion on ISAC Deployment Scenarios”, Samsung

R1-2404127 “Discussion on ISAC Deployment Scenarios”, Samsung

R1-2406664 “Discussion on ISAC Deployment Scenarios”, Samsung

R1-2408657 “Discussion on ISAC Deployment Scenarios”, Samsung

R1-2409608 “Discussion on ISAC Deployment Scenarios”, Samsung

R1-2500860 “Discussion on ISAC Deployment Scenarios”, Samsung

R1-2502378 “Discussion on ISAC Deployment Scenarios”, Samsung