3GPP TSG-RAN WG1 Meeting #121		R1-2503579

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.8.2

Title:	Discussion on channel model adaptation/extension of TR38.901 for 7-24GHz

Source:	Samsung

Document for:	Discussion and Decision

- Introduction

The Rel-19 study items on “Study on channel modelling enhancements for 7 – 24 GHz for NR” is endorsed in RAN#102 [1]. The objectives for this SI are shown below:

The objectives of this study are: - Validate using measurements the channel model of TR38.901 at least for 7-24 GHz - Note: Only stochastic channel model is considered for the validation. - Note: The validation may consider all existing scenarios: UMi-street canyon, UMa, Indoor-Office, RMa and Indoor-Factory. - Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios: - Near-field propagation (with consideration being given to consistency between near-field and far-field) - Spatial non-stationarity Note 1: Continuity of the channel model in the frequency domain below 7 GHz and above 24 GHz shall be ensured. Note 2: Mathematical and/or theoretical aspects (if any) may be studied before results of measurement campaigns are available. While measurement results may be available and submitted at any time, the study of measurement results may start later (e.g., Q3 2024).

In this contribution, we discuss channel model adaptation/extension of TR 38.901 [2] related aspects for Rel-19 channel model SI.

- Near-field channel

2.1 Calibration

In the last meeting, RAN1 decided on the calibration assumptions for the near-field channel model. Among these, the phase term is fundamentally considered to reflect the influence of the near-field. Specifically, the impact of the phase term can be compared in two scenarios: (1) only the LOS ray and (2) both LOS and NLOS rays. This comparison allows for an analysis of the characteristics of the near-field channel. To this end, RAN1 agreed to use a metric based on the ratio of singular values, comparing the 1st (strongest) PRB singular value to the singular values of others. Below is a figure 1 and 2 showing the CDF of singular value ratios as a function of distance in the UMi environment.

- Phase term application for LOS ray only

Firstly, when the phase term is modified antenna element-wise for only the LOS ray, it is observed that Config 1B (4 ports on a handheld antenna model) at a distance of 10m shows better orthogonality compared to the far-field model. This improvement is attributed to the diversification of paths caused by the antenna arrangement within the UE. However, as the distance increases, the difference between the near-field and far-field models becomes negligible.

| - 10 m | - 30 m | - 50 m |
| --- | --- | --- |

- (a)

| - 10 m | - 30 m | - 50 m |
| --- | --- | --- |

- (b)

- Figure 1. Comparison CDF of singular value ratio between near-field and far-field, when updates on phase term for LOS ray only are applied: (a) Config 1A and (b) Config 1B

- Phase term application for both LOS ray and NLOS ray

Next, the scenario where the phase term is modified antenna element-wise for both LOS and NLOS rays is shown. Similar to the case of applying only the LOS ray, Config 1B (4 ports on a handheld antenna model) at a distance of 10m shows better orthogonality in the near-field model compared to the far-field model. Becoming the distance is increased, the non-planar wave characteristics of the near-field do not show significant differences from the planar wave characteristics of the far-field.

- Overall, when the distance between the BS and UE is very close or when antennas are placed at various positions within the UE, the application of near-field modeling results in a smaller difference in singular values between the 1st (strongest) stream and the 2nd, 3rd, and 4th streams. Since the calibration assumptions is in ideal environment (UMi with large BS antenna array, LOS environment and 10 m distances), the near-field characteristics may be valid. However, replicating such conditions in real-life scenarios can be limited.

| - 10 m | - 30 m | - 50 m |
| --- | --- | --- |

- (a)

| - 10 m | - 30 m | - 50 m |
| --- | --- | --- |

- (b)

- Figure 2. Comparison CDF of singular value ratio between near-field and far-field, when updates on phase term for both LOS and NLOS ray are applied: (a) Config 1A and (b) Config 1B

Config 1B (4 ports on a handheld antenna model) at a distance of 10m with phase term updates on both LOS and NLOS ray shows better orthogonality in the near-field model compared to the far-field model

Becoming the distance is increased, the non-planar wave characteristics of the near-field do not show significant differences from the planar wave characteristics of the far-field

Although near-field effects can have positive potentials compared to far-field environment, whether such calibration assumptions can be applied to real-life scenarios requires detailed review.

2.2 Other considerations

2.2.1 Methodology of channel realization

Agreement For near-field channel model, RAN1 strives to design a unified model to explicitly reflect the new properties of near- and existing properties of far-field under the structure of existing stochastic model TR 38.901. - FFS: whether the same or different implementations, e.g., procedures/equations, are used for near- and far-field channel realization

RAN1 has agreed to design a unified model that explicitly reflect newly introduced near-field properties and existing far-field properties based on the structure of TR38.901. Accordingly, there is a need for discussion on whether the implementation of near-field and far-field should be handled in the same or different manner (procedure or equation).

The same implementation method will employ a single procedure/equation that integrates non-planar wavefront effects, application across all distances between BS and UE. The primary advantage is its consistency. With one procedure/equation handling both near-field and far-field scenarios, the model remains uniform. This consistency makes that the model can be universally applied without the need for transition points. However, this comes at the complexity for computation. Non-planar wavefronts are significant in near-field conditions, while far-field condition is adequately modelled using plane wavefront. So, by using a more complex equation across all field condition, the model may introduce unnecessary computational overhead in far-field conditions.

Alternatively, a different implementation method divides the channel model into near-field and far-field regions, each with its own tailored equation. For example, the model in near-field region captures the properties of non-planar wavefronts, while the model in far-field regions simplifies calculations by assuming a planar wavefront. This method enhances computational efficiency by applying the appropriate model to each region. However, it introduces challenges in managing the transition between near-field and far-field regions.

Therefore, the decision needs to be made taking into account trade-off between channel consistency and computational complexity.

For same implementation method, the consistency makes that the model can be universally applied without the need for transition points. However, this comes at the complexity for computation. Non-planar wavefronts are significant in near-field conditions, while far-field condition is adequately modelled using plane wavefront. So, by using a more complex equation across all field condition, the model may introduce unnecessary computational overhead in far-field conditions

For different implementation method, it enhances computational efficiency by applying the appropriate model to each region. However, it introduces challenges in managing the transition between near- and far-field regions

RAN1 discuss same/different implementation method taking into account channel consistency and computational complexity

2.2.2 Other channel parameter

Another discussion point of interest is the field pattern of antenna arrays. During the last meeting, RAN1 came to a consensus not to factor in the near-field properties of separate antenna elements. Nevertheless, exploring the subject of analogue beams in arrays continues to hold significance. When considering an antenna array, it is necessary to confirm phase control for tilting analogue beams. In general, the propagation characteristics of an antenna array in the far-field region is a pattern multiplication form that include a weighting term called an array factor for each antenna element’s phase control to the propagation characteristics of an antenna element [10], [11]. Unlike the far-field region’s plane wave approximation, spherical waves in the near field region require consideration of both angle and distance. This is because not all antenna elements maintain the same angle with respect to the wave front in the near-field area. Therefore, it is necessary to validate the array factor for array operation along with the propagation characteristics of an antenna element.

However, the existing channel models lack terms representing distinct phases of field patterns for each antenna element. Therefore, referring to [10], RAN1 needs to explicitly indicate the weighting vectors of each antenna element and consider modeling incorporating these factors.

RAN1 consider the updates of channel parameter with weighting vector for antenna array

| Configuration | Multiple columns (NV x NH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | the steering matrix components are given by the weighting factor is given by wm,n=1NHNVexpi∙2πn-1dVλsinθetilt-m-1dHλcosθetiltsinφescan M=1,2,…NH; n=1,2,…NV;_ |

- Spatial non-stationarity

In the last RAN1 meeting, two options were agreed upon as candidates for modelling spatial non-stationarity. However, prior to full-fledged discussions on modelling, it is necessary to clarify when this phenomenon occurs. This is because clarifying the purpose of channel modelling can make it easier to determine the direction of the modelling.

Spatial non-stationarity’ means that the antenna elements or subarrays within the BS's whole antenna array experience different propagation environments. For instance, one end of an antenna element and/or subarray in whole antenna array may be in LOS status with the UE, while the opposite end of the whole antenna array is shadowed by obstacles and becomes NLOS status.

However, it seems this phenomenon occurs when a large-scale antenna array is introduced or the UE is close to the BS. Therefore, the occurrence condition of spatial non-stationarity depends on the configuration of the BS's antenna array and the distance between the BS and UE.

Based on this, we consider that it is important to clarify the conditions under when spatial non-stationarity occurs. In addition, discussions on how each antenna element within the BS whole antenna array determines its LOS state with the UE are needed. Finally, methods for generating and/or determining multipath clusters according to this phenomenon should also be discussed.

Spatial non-stationarity occurs when a large-scale antenna array in BS is introduced or the UE is very close to the BS.

RAN1 discuss the condition of occurrence for spatial non-stationarity

3.1 Modeling methodology

Discussions are ongoing regarding options for modeling methodologies for spatial non-stationarity. From our understanding, the main discussion point for spatial non-stationarity is based on the perspective of BS equipped with large antenna array. Hence, considering the presence of geographical blockers between the BS and UE, it is crucial to examine power attenuation per antenna element within the BS. To maintain this perspective and channel modeling continuity, continuing discussion is prioritized using existing Blockage model B of blocker-based approach as the starting point.

Considering the presence of geographical blockers between the BS and UE, it is crucial to examine power attenuation per antenna element within the BS

RAN1 consider the blockage model B of blocker-based approach for the modelling of spatial non-stationarity

- Conclusion

- Config 1B (4 ports on a handheld antenna model) at a distance of 10m with phase term updates on both LOS and NLOS ray shows better orthogonality in the near-field model compared to the far-field model

Becoming the distance is increased, the non-planar wave characteristics of the near-field do not show significant differences from the planar wave characteristics of the far-field

Although near-field effects can have positive potentials compared to far-field environment, whether such calibration assumptions can be applied to real-life scenarios requires detailed review.

For same implementation method, the consistency makes that the model can be universally applied without the need for transition points. However, this comes at the complexity for computation. Non-planar wavefronts are significant in near-field conditions, while far-field condition is adequately modelled using plane wavefront. So, by using a more complex equation across all field condition, the model may introduce unnecessary computational overhead in far-field conditions

For different implementation method, it enhances computational efficiency by applying the appropriate model to each region. However, it introduces challenges in managing the transition between near- and far-field regions

- RAN1 discuss same/different implementation method taking into account channel consistency and computational complexity

- RAN1 consider the updates of channel parameter with weighting vector for antenna array

Spatial non-stationarity occurs when a large-scale antenna array in BS is introduced or the UE is very close to the BS.

- RAN1 discuss the condition of occurrence for spatial non-stationarity

Considering the presence of geographical blockers between the BS and UE, it is crucial to examine power attenuation per antenna element within the BS

- RAN1 consider the blockage model B of blocker-based approach for the modelling of spatial non-stationarity

Appendix. Simulation assumptions for calibration

| Parameter |  | Values |
| --- | --- | --- |
| Scenarios |  | UMi-street Canyon |
| Carrier Frequency |  | 7 GHz |
| Bandwidth |  | 20 MHz |
| BS antenna height |  | UMi-street Canyon: 10m |
| BS antenna downtilt |  | 102 degrees for UMi-street Canyon |
| BS Tx power |  | 44 dBm for UMi-Street Canyon |
| BS antenna configurations and port mapping |  | For UMi scenario: Config 1 for 7 GHz (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling |  | Model-2 in Clause 7.3.2 |
| UE Location | Outdoor/indoor | UMi-street Canyon: 100% outdoor |
|  | LOS/NLOS | 100% LOS |
|  | UE antenna height | UMi-street Canyon: 1.5m |
| UT antenna configurations |  | Config A: Mg = Ng = 1, M = 2, N = 1, P = 2, dH = dV = 0.5λ Config B: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern |  | Isotropic |
| UT Polarized antenna modelling |  | For UT Config A: Model-2 in Clause 7.3.2 For UT Config B: following the clause 7.3 |
| UT orientation |  | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 45 degree, ΩUT, = 0 degree |
| Applicability of Nearfield Modeling |  | Component of NF channel to be considered: - Phase of Direct path - Phase of Non-direct path, applied only at the BS side |
| Calibration method |  | Drop multiple users in the multiple cells at the fixed horizontal distance Z from the BS, and collect the metric for a each distance Z For UMi-street Canyon: Z = 10, 30, 50, 100 m (optional) Z = 20, 40, 80 m |
| Metric |  | CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest, see note 1) PRB singular value and the 1st PRB (largest) singular value (see note 2) of the serving cell at t=0 plotted in 10*log10 scale. |
| NOTE 1: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) NOTE 2: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. |  |  |

References

RP-234018 “New SID: Study on channel modelling enhancements for 7-24GHz for NR,” RAN#102

TR38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”

R1-2402481 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2404130 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2406667 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2408660 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2409611 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2500863 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2502381 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

TR37.840 “Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station (Release 12)”

Balanis, C. A. (2016). Antenna theory: analysis and design. John wiley & sons.