3GPP TSG RAN WG1 #121	        		               R1-2503581

St Julian's, Malta, May 19th – 23rd, 2025

Agenda item:		9.10.1

Source:			Samsung

Title:			Remaining topics on enabling TX/RX during RRM measurements for XR

Document for:		Discussion and decision

# Introduction

This contribution considers one remaining issue on enabling Tx/Rx during RRM measurements for XR [1, 2].

# Discussion

One remaining issue relates to the case where a UE is provided multiple MG configurations that could overlap (in time). The interaction between the MG collision resolution procedures defined in TS 38.133 and the DCI-based signaling for XR to enable Tx/Rx during RRM measurements needs to be clarified.

In NR, RRC-indications are first considered prior to considering DCI-indications. For example, a PUCCH resource indicated by DCI is considered valid based on the indication of UL/flexible symbols by tdd-UL-DL-ConfigurationCommon. Similar, for the applicability of the DCI field for an MG to be skipped for RRM measurements, the applicable MGs are the ones determined after the existing procedures for overlapping resolution are performed first. It is noted that the ordering of the DCI formats (e.g. “last” DCI format) is as in legacy specifications.

Proposal: The procedures defined in TS 38.133 of resolving MG overlapping remain applicable before determining the MG occasions for the applicability of the “Measurement Gap Cancellation” field in the DCI format.

# Conclusions

This contribution considered an outstanding discussion point on enabling Tx/Rx during RRM measurements for XR and proposes the following.

Proposal: The procedures defined in TS 38.133 of resolving MG overlapping remain applicable before determining the MG occasions for the applicability of the “Measurement Gap Cancellation” field in the DCI format.

# References

- RAN1#120 Chairman notes

- R1-2501531, “Moderator summary #2 - Enabling TX/RX for XR during RRM measurements”