3GPP TSG RAN WG1 #121	 R1-2503587

# Malta, MT, May 19th – 23rd, 2025

Agenda item:	9.12.1

Source:	Samsung

Title:	Enhancements for multi-cell PUSCH/PDSCH scheduling

Document for:	Discussion and decision

# Introduction

A WI on “Multi-carrier enhancements for NR Phase 2” was endorsed in RAN#105 with the following objectives [1]:

1. Specify the support of the followings for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs per scheduled cell is [4 or 8]. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

This contribution considers specification requirements for the above two objectives.

# MC-DCI with different SCS and carrier types

To support MC-DCI with different SCS, the only remaining issue is whether the support of multiple SCS among the co-scheduled cells can have an impact on the ‘minimum scheduling offset indicator’ field. This field is a Type-1A field in Rel-18 MCE and indicates a K0min/K2min value that depends on the SCS of the respective scheduled cell. With different SCS for the set of co-scheduled cells, a same indicated value can be interpreted differently for different cells based on the respective configured values and the respective SCS for each cell. However, such fixed linkage can be restrictive for the case of different SCS. To improve the scheduling flexibility in case of mixed SCS, this field can be updated to be a Type-1B field with 2 bits.

Proposal 1: Update the field type of the ‘minimum scheduling offset indicator’ field from Type-1A to Type-1B.

# MC-DCI with multi-PDSCH/PUSCH scheduling

The Rel-19 WI on MCE aims at providing support for multi-PDSCH/PUSCH (multi-PxSCH) scheduling using MC-DCI formats. In the following, remaining issues related to RRC parameters, DCI fields and HARQ-ACK codebook are discussed.

## RRC parameter

RRC parameters for Rel-19 MC-DCI with multi-PxSCH scheduling were endorsed in RAN1#120bis [2], except for an FFS point for the range of the multi-cell multi-PDxCH scheduling TDRA tables TDRA-FieldIndexListDCI-1-3-r19 and TDRA-FieldIndexListDCI-0-3-r19. The multi-cell single-PxSCH TDRA tables were agreed in Rel-18 to include up to 32 rows for DL DCI format 1_3 and up to 64 rows for the UL DCI format 0_3 (and even that range was a compromise).

In order to accommodate the same configuration/scheduling flexibility as in Rel-18 for the case with multi-PDSCH scheduling, it is beneficial / necessary to increase the TDRA field size, e.g., 64 for DL and 128 for UL.

Proposal 2: To resolve the FFS on the range of RRC parameters TDRA-FieldIndexListDCI-1-3-r19 and TDRA-FieldIndexListDCI-0-3-r19:

- support increasing the number of multi-cell TDRA combinations, e.g., 64 for DL and 128 for UL, to ensure same flexibility as in Rel-18.

## DCI fields

One remaining clarification for NDI and RV fields of DCI format 0_3/1_3 is on determination of the yellow-highlighted wording “Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell” in the following agreements: whether the maximum is taken based only on the multi-PxSCH TDRA table for the active BWP of the corresponding cell, or taken across different multi-PxSCH TDRA tables for all configured BWPs of the corresponding cell. The former option (active BWP) is more reasonable, and was also the RAN1 conclusion in Rel-17, as shown below.

Agreement (RAN1#119) - For NDI indication in DCI format 0_3/1_3 for TB1, further study - Option 1: For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3. - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell. - Option 2a: … [Not fully reproduced for brevity] - Other options are not precluded. - Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of NDI field for TB1, same as in Rel-18. Agreement (RAN1#119) - For RV indication in DCI format 0_3/1_3 for TB1, further study - Option 1: For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell. - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell. - Option 2a: … [Not fully reproduced for brevity] - Other options are not precluded. - Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of RV field for TB1, same as in Rel-18. Agreement (RAN1#120) - For NDI indication in DCI format 0_3/1_3 for a TB, - Option 1 is adopted. Agreement (RAN1#120) - For RV indication in DCI format 0_3/1_3 for a TB, - Option 1 is adopted.

Conclusion (RAN1#114bis) - It is clarified that the maximum DCI field sizes of NDI/RV for PDSCH and NDI/RV for PUSCH are determined based on the maximum number of schedulable PUSCH or PDSCH among all entries in TDRA table configured for the active BWP. - If a bandwidth part indicator field … [Not fully reproduced for brevity]

Proposal 3: For the number of bits of NDI/RV fields in DCI format 0_3/1_3 with multi-PUSCH/PDSCH scheduling:

- the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the active BWP of the cell.

## HARQ-ACK aspects

The specifications of HARQ-ACK information for multi-PDSCH scheduling using MC-DCI is mostly complete.

A first remaining issue is about handling the UE capability FG 18-9 for reception of multiple DCI formats 1_3 with multi-PDSCH scheduling within a same PDCCH monitoring occasion (MO), when a reference cell (with smallest cell index) among the multiple DCI formats is the same. Such UE capability was considered in Rel-18 MCE in the context of multi-cell single-PDSCH scheduling, and it was agreed to count (and run the Type-2 HARQ-ACK pseudo-code for) the serving cell a number of times that is equal to the number of scheduled PDSCHs on the serving cell, which is consistent with the agreed principle for DAI counting per DCI format.

However, with multi-PDSCH scheduling, the number of DCI formats can be different from the number of scheduled PDSCHs, and to follow the DAI counting principle per DCI format, the statement about counting the serving cell NPDSCH, cm times (i.e., equal to the number of scheduled PDSCHs) cannot be directly reused. Instead, the counting should be based on the number of NPDCCH, cm DCI formats that are received for the same serving cell (with the smallest cell index) and in the same PDCCH MO. TP #1 is presented in the Appendix to address this issue. It is noted that a similar discussion / TP is also provided for the Rel-17 single-cell multi-PDSCH scheduling [3, 4].

Proposal 4: For the application of FG 18-9 (type2-HARQ-ACK-Codebook) to Rel-19 multi-cell multi-PDSCH scheduling:

- The UE counts a serving cell c a number NPDCCH, cm times in the Type-2 HARQ-ACK pseudo-code, when the UE receives NPDCCH, cm>1 DCI formats 1_3 in the same PDCCH MO that schedule NPDSCH, cm>1 PDSCH receptions, with NPDSCH, cm≥NPDCCH, cm, and the serving cell c is a serving cell with smallest cell index across the NPDCCH, cm DCI formats;

- Adopt TP #1 in the Appendix for TS 38.213.

A second remaining issue is on the determination of the number of HARQ-ACK bits for PUCCH power control. In Rel-18 MCE, it was agreed that, for UCI bits fewer than 11 bits, the UE determines nHARQ-ACK=nHARQ-ACK,0+nHARQ-ACK,1, where nHARQ-ACK,0 is the value of nHARQ-ACK for the first Type-2 HARQ-ACK sub-codebook and nHARQ-ACK,1 is the value of nHARQ-ACK for the second Type-2 HARQ-ACK sub-codebook that is determined as:

nHARQ-ACK=VDAI,mlastDL-s=0NsetsDL-1UDAI,smodTDNTB,maxDL,MC+s=0NsetsDL-1m=0M-1Nm,sreceived.

The same formula applies to Rel-19 MCE with multi-PDSCH MC-DCI with the following clarifications:

-	NTB,maxDL,MC=NsetsHARQ-ACK,max as defined in the draft CR for TS 38.213 [5], that already captures different cases with or without configuration of harq-ACK-SpatialBundlingPUCCH and with or without configuration of nrofHARQ-BundlingGroups;

-	Nm,sreceived is based on the number of TBs / PDSCH receptions when nrofHARQ-BundlingGroups is not configured, or the number of transport block groups (TBGs) or PDSCH reception groups when nrofHARQ-BundlingGroups is configured, along with legacy handling for overlap with UL symbols of the TDD UL/DL configuration.

Two TPs have been prepared in the Appendix, where TP #2A captures the change as part of the “modifications” for Rel-19 MCE that were recently endorsed in the draft CR [5, R1-2503173], while TP #2B captures the change as a direct update of the corresponding Rel-18 specifications for PUCCH power control. TP #2A is compatible with the Rel-18 assumption of single PDSCH per cell for PUCCH power control, while TP #2B is simpler and with less specification change.

Proposal 5: For determination of the number of HARQ-ACK bits for PUCCH power control, reuse the Rel-18 MCE formula with the following modifications:

- NTB,maxDL,MC is the number NsetsHARQ-ACK,max of HARQ-ACK information bits per DCI format 1_3, as per draft CR for TS 38.213 [5, R1-2503173];

- Nm,sreceived is based on the number of TB / PDSCH receptions when nrofHARQ-BundlingGroups is not configured, or the number of transport block groups (TBGs) or PDSCH reception groups when nrofHARQ-BundlingGroups is configured, along with legacy handling for overlap with UL symbols of the TDD UL/DL configuration;

- Adopt one of TP #2A or TP #2B in the Appendix for TS 38.213.

A last clarification is about the number of HARQ-ACK information bits per cell. During the RAN1 email discussion [6] for endorsing the draft CR [5] for Rel-19 MCE, there was a suggestion to additionally specify such per-cell number of HARQ-ACK bits as part of the Rel-19 “modifications” for the Rel-18 pseudo-code for MC-DCI. However, such change is not necessary, since the endorsed draft CR [5] already includes the corresponding Rel-17 specifications text for the number of per-cell HARQ-ACK bits along with an update to include the corresponding Rel-19 RRC parameter. The per-cell number of HARQ-ACK bits was captured as a separate text in Rel-17 single-cell multi-PDSCH scheduling, and was not included as part of the “modifications” to the legacy pseudo-code for Rel-17; the same procedure is followed in Rel-19 MCE. It is noted that, the list of “modifications” for Rel-19 already includes the per-DCI number of HARQ-ACK information bits.

Proposal 6: Further clarification of the per-cell number of HARQ-ACK information bits, e.g., as part of the “modifications” to the pseudo-code for the Rel-19 MCE is not necessary – such text already exists in the endorsed draft CR for TS 38.213 [5, R1-2503173].

# Conclusions

This contribution considered the Rel-19 enhancements for multi-cell scheduling of PUSCHs/PDSCHs using DCI format 0_3/1_3 and provided the following proposals.

Proposal 1: Update the field type of the ‘minimum scheduling offset indicator’ field from Type-1A to Type-1B.

Proposal 2: To resolve the FFS on the range of RRC parameters TDRA-FieldIndexListDCI-1-3-r19 and TDRA-FieldIndexListDCI-0-3-r19:

- support increasing the number of multi-cell TDRA combinations, e.g., 64 for DL and 128 for UL, to ensure same flexibility as in Rel-18.

Proposal 3: For the number of bits of NDI/RV fields in DCI format 0_3/1_3 with multi-PUSCH/PDSCH scheduling:

- the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the active BWP of the cell.

Proposal 4: For the application of FG 18-9 (type2-HARQ-ACK-Codebook) to Rel-19 multi-cell multi-PDSCH scheduling:

- The UE counts a serving cell c a number NPDCCH, cm times in the Type-2 HARQ-ACK pseudo-code, when the UE receives NPDCCH, cm>1 DCI formats 1_3 in the same PDCCH MO that schedule NPDSCH, cm>1 PDSCH receptions, with NPDSCH, cm≥NPDCCH, cm, and the serving cell c is a serving cell with smallest cell index across the NPDCCH, cm DCI formats;

- Adopt TP #1 in the Appendix for TS 38.213.

Proposal 5: For determination of the number of HARQ-ACK bits for PUCCH power control, reuse the Rel-18 MCE formula with the following modifications:

- NTB,maxDL,MC is the number NsetsHARQ-ACK,max of HARQ-ACK information bits per DCI format 1_3, as per draft CR for TS 38.213 [5, R1-2503173];

- Nm,sreceived is based on the number of TB / PDSCH receptions when nrofHARQ-BundlingGroups is not configured, or the number of transport block groups (TBGs) or PDSCH reception groups when nrofHARQ-BundlingGroups is configured, along with legacy handling for overlap with UL symbols of the TDD UL/DL configuration;

- Adopt one of TP #2A or TP #2B in the Appendix for TS 38.213.

Proposal 6: Further clarification of the per-cell number of HARQ-ACK information bits, e.g., as part of the “modifications” to the pseudo-code for the Rel-19 MCE is not necessary – such text already exists in the endorsed draft CR for TS 38.213 [5, R1-2503173].

# Appendix: Suggested TPs

## TP #1 for Proposal 4 (against TS 38.213 v18.6.0) // Application of FG 18-9 to multi-cell multi-PDSCH scheduling

*** Unchanged parts are omitted ***

The UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK, for a total number of OACK HARQ-ACK information bits in the second Type-2 HARQ-ACK sub-codebook according to the following pseudo-code.

Set Ncells,setDL,max to the maximum number of serving cells in ScheduledCell-ListDCI-1-3 of a set of serving cells provided by MC-DCI-SetofCells, across the number of sets of serving cells, that can be scheduled PDSCH receptions by DCI format 1_3

Set NsetsTB,max to the maximum total number of TBs in PDSCH receptions that can be scheduled by a DCI format 1_3 over more than one serving cells in a set of serving cells across the number of sets of serving cells

Set NsetsDL to the number of sets of serving cells MC-DCI-SetofCells in a PUCCH group

Set NcellsDL to the number of serving cells, across NsetsDL sets of serving cells in the PUCCH group

Set c to the index of serving cells, c=0,…, NcellsDL-1, a lower index corresponds to a lower RRC index of a corresponding serving cell

-	if the UE indicates type2-HARQ-ACK-Codebook and receives NPDSCH, cm>1 PDSCHs on a serving cell c that are scheduled by NPDSCH, cm NPDCCH, cm DCI formats 1_3, where NPDCCH, cm≤NPDSCH, cm, in PDCCH receptions at a same PDCCH monitoring occasion m, where

-	each of the DCI formats 1_3 schedules more than one PDSCH receptions on respective more than one serving cells,

-	c is the smallest cell index among the respective more than one serving cells that are scheduled by each of the DCI formats 1_3, and

-	c is same across the NPDSCH, cm NPDCCH, cm DCI formats 1_3

the serving cell c is counted NPDSCH, cm NPDCCH, cm times for PDCCH monitoring occasion m in increasing order of the respective earliest PDSCH reception starting time, for each of the DCI formats 1_3, among the NPDSCH, cm PDSCH receptions

Set mc to the index of a serving cell, in a set of indexes of serving cells arranged in ascending order, from the set of Ncells,setDL,max serving cells, mc=0,…, Ncells,setDL,max-1

*** Unchanged parts are omitted ***

## TP #2A for Proposal 5 (against the endorsed draft CR [5, R1-2503173]) // PUCCH power control with multi-cell multi-PDSCH scheduling – Alt-1: as part of the “modifications” for Rel-19 MCE

if harq-ACK-SpatialBundlingPUCCH is not provided,

OACK=NsetsTB,max⋅4⋅j+Vtemp2

else

OACK=Ncells,setDL,max⋅4⋅j+Vtemp2

end if

oiACK=NACK for any i∈0,1,⋯,OACK-1\Vs .

If

- OACK+OSR+OCSI≤11 and NsetsDL>0, and

- a UE is not provided pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for any serving cell in a PUCCH group, or provided nrofHARQ-BundlingGroups with value of 1 for any serving cell that is provided pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, or

- for PDSCH receptions scheduled by a DCI format 1_3 that does not schedule more than one PDSCH reception on each serving cell, or

- for PDSCH receptions scheduled by a DCI format 1_3 on a serving cell when the UE is provided nrofHARQ-BundlingGroups with value of 1

for obtaining a PUCCH transmission power as described in clause 7.2.1, the UE determines nHARQ-ACK=nHARQ-ACK,0+nHARQ-ACK,1, where nHARQ-ACK,0 is the value of nHARQ-ACK for the first Type-2 HARQ-ACK sub-codebook and nHARQ-ACK,1 is the value of nHARQ-ACK for the second Type-2 HARQ-ACK sub-codebook that is determined as

nHARQ-ACK=VDAI,mlastDL-s=0NsetsDL-1UDAI,smodTDNTB,maxDL,MC+s=0NsetsDL-1m=0M-1Nm,sreceived

where

-	in the following

-	a DCI format 1_3 schedules more than one PDSCH receptions providing transport blocks with enabled HARQ-ACK information

-	a dormancy indication is considered as a PDSCH reception providing a single transport block with enabled HARQ-ACK information

-	VDAI,mlastDL is the value of the total DAI field in a last DCI format 1_3 the UE detects in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format 1_3. VDAI,mlastDL=0 if the UE does not detect any DCI format 1_3 in any of the M PDCCH monitoring occasions.

-	UDAI,s is the total number of DCI format 1_3 that the UE detects within the M PDCCH monitoring occasions for the set s of serving cells. UDAI,s=0 if the UE does not detect any DCI format 1_3 associated with scheduling on set s of serving cells in any of the M PDCCH monitoring occasions.

-	NTB,maxDL,MC=NsetsTB,max if harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL,MC=Ncells,setDL,max.

-	Nm,sreceived is

-	the number of transport blocks, in PDSCH receptions not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided

-	the number of more than one PDSCHs, not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, scheduled by a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided

If a UE is provided pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for a serving cell in a set of serving cells provided by MC-DCI-SetofCells in a PUCCH group, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-codes with the following modifications

-	the first HARQ-ACK sub-codebook is associated with

-	unicast SPS PDSCH receptions

-	any unicast DCI format scheduling a single PDSCH reception on a serving cell

-	any unicast DCI format having associated HARQ-ACK information without scheduling a PDSCH reception

-	any DCI format 1_3 scheduling more than one PDSCH receptions on a serving cell c for which the UE is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max=1

-	the second HARQ-ACK sub-codebook is associated with DCI format 1_3 that

-	schedules PDSCH receptions on more than one serving cells from the set of serving cells, or

-	schedules more than one PDSCH receptions on a serving cell c, from the set of serving cells, for which the UE is not provided nrofHARQ-BundlingGroups or is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max>1, or

-	does not include a SCell dormancy indication field or the SCell dormancy indication field is reserved, indicates SCell dormancy, and schedules PDSCH receptions on one or more serving cells from the set of serving cells

-	in the above, and for the purpose of providing HARQ-ACK information corresponding to SCell dormancy indication by the DCI format 1_3

-	the UE assumes that the UE receives one PDSCH, with first SLIV in the row of pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that the PDSCH provides one transport block that the UE correctly decodes, if the UE is not provided nrofHARQ-BundlingGroups for the serving cell

-	the UE assumes that the UE receives one or more PDSCHs associated with a TBG with smallest index, or a PDSCH reception group with smallest index, among one or more PDSCH receptions scheduled by the DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that each of the one or more PDSCHs provides one transport block that the UE correctly decodes if the UE is provided nrofHARQ-BundlingGroups for the serving cell

-	the UE assumes incorrect decoding for transport blocks in each of the remaining PDSCH receptions scheduled by the DCI format 1_3 on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication

-	instead of generating NsetsTB,max HARQ-ACK information bits when harq-ACK-SpatialBundlingPUCCH is not provided, or generating Ncells,setDL,max HARQ-ACK information bits when harq-ACK-SpatialBundlingPUCCH is provided, for the PDSCH receptions scheduled by a DCI format 1_3, the UE generates

-	NsetsHARQ-ACK,max HARQ-ACK information bits for the PDSCH receptions scheduled by the DCI format 1_3, where NsetsHARQ-ACK,max is the maximum number, over the number NsetsDL sets of serving cells provided by MC-DCI-SetofCells in the PUCCH group, of the sum of

-	NTB,cDL⋅NPDSCH,cmax if nrofHARQ-BundlingGroups is not provided for a serving cell c, or

-	NTB,cDL⋅NHARQ-ACK,cTBG,max if nrofHARQ-BundlingGroups is provided for the serving cell c,

across serving cells of a respective set of serving cells MC-DCI-SetofCells that can be scheduled PDSCH reception by a DCI format 1_3,

where

-	NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1;

-	NPDSCH,cmax is the maximum number of SLIVs over all rows of the TDRA table provided by pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for serving cell c; and

-	NHARQ-ACK,cTBG,max is provided by nrofHARQ-BundlingGroups for serving cell c.

-	If for a set s of serving cells provided by MC-DCI-SetofCells, NsetsHARQ-ACK,s is equal to the sum of

-	NTB,cDL⋅NPDSCH,cmax if nrofHARQ-BundlingGroups is not provided for a serving cell c, or

-	NTB,cDL⋅NHARQ-ACK,cTBG,max if nrofHARQ-BundlingGroups is provided for the serving cell c,

across the serving cells of the set s that are scheduled PDSCH receptions by a DCI format 1_3, and NsetsHARQ-ACK,s <NsetsHARQ-ACK,max, the UE generates NACK values for the last NsetsHARQ-ACK,max-NsetsHARQ-ACK,s HARQ-ACK information bits corresponding to the DCI format 1_3.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

If OACK+OSR+OCSI≤11 and NsetsDL>0, for obtaining a PUCCH transmission power as described in clause 7.2.1, the UE determines nHARQ-ACK=nHARQ-ACK,0+nHARQ-ACK,1, where nHARQ-ACK,0 is the value of nHARQ-ACK for the first Type-2 HARQ-ACK sub-codebook and nHARQ-ACK,1 is the value of nHARQ-ACK for the second Type-2 HARQ-ACK sub-codebook that is determined as

nHARQ-ACK=VDAI,mlastDL-s=0NsetsDL-1UDAI,smodTDNTB,maxDL,MC+s=0NsetsDL-1m=0M-1Nm,sreceived

where

-	in the following

-	a DCI format 1_3 schedules more than one PDSCH receptions, providing transport blocks with enabled HARQ-ACK information, on more than one serving cells, or

-	a DCI format 1_3 schedules more than one PDSCH receptions, providing transport blocks with enabled HARQ-ACK information, on a single serving cell c, for which the UE is not provided nrofHARQ-BundlingGroups or is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max>1

-	a dormancy indication is considered as a PDSCH reception, providing a single transport block with enabled HARQ-ACK information, corresponding to the first SLIV in the row of pdsch-TimeDomainAllocationList or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, if the UE is not provided nrofHARQ-BundlingGroups for a corresponding serving cell

-	a dormancy indication is considered as one or more PDSCH receptions providing respective one or more transport blocks with enabled HARQ-ACK information, corresponding to a TBG with smallest index or a PDSCH reception group with smallest index, if the UE is provided nrofHARQ-BundlingGroups for a corresponding serving cell

-	VDAI,mlastDL is the value of the total DAI field in a last DCI format 1_3 the UE detects in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format 1_3. VDAI,mlastDL=0 if the UE does not detect any DCI format 1_3 in any of the M PDCCH monitoring occasions.

-	UDAI,s is the total number of DCI format 1_3 that the UE detects within the M PDCCH monitoring occasions for the set s of serving cells. UDAI,s=0 if the UE does not detect any DCI format 1_3 associated with scheduling on set s of serving cells in any of the M PDCCH monitoring occasions.

-	NTB,maxDL,MC is

-	NTB,maxDL,MC=NsetsHARQ-ACK,max if pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided for at least one serving cell in the PUCCH group;

-	NTB,maxDL,MC=NsetsTB,max if pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is not provided for any serving cell in the PUCCH group and harq-ACK-SpatialBundlingPUCCH is not provided;

-	otherwise, NTB,maxDL,MC=Ncells,setDL,max.

-	Nm,sreceived is

-	the number of transport blocks, in PDSCH receptions not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided and nrofHARQ-BundlingGroups is not provided

-	the number of more than one PDSCHs, not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, scheduled by a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided and nrofHARQ-BundlingGroups is not provided

-	the number of transport block groups (TBGs) with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, from/based on PDSCH receptions associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided and nrofHARQ-BundlingGroups is provided

-	the number of PDSCH groups, with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, from/based on PDSCH receptions associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided and nrofHARQ-BundlingGroups is provided.

Table 9.1.3-1: Value of counter DAI for NC-DAIDL=2 and of total DAI

*** Unchanged parts are omitted ***

## TP #2B for Proposal 5 (against TS 38.213 v18.6.0) // PUCCH power control with multi-cell multi-PDSCH scheduling – Alt-2: directly modify the Rel-18 text to make it also applicable to Rel-19 MCE

*** Unchanged parts are omitted ***

If OACK+OSR+OCSI≤11 and NsetsDL>0, for obtaining a PUCCH transmission power as described in clause 7.2.1, the UE determines nHARQ-ACK=nHARQ-ACK,0+nHARQ-ACK,1, where nHARQ-ACK,0 is the value of nHARQ-ACK for the first Type-2 HARQ-ACK sub-codebook and nHARQ-ACK,1 is the value of nHARQ-ACK for the second Type-2 HARQ-ACK sub-codebook that is determined as

nHARQ-ACK=VDAI,mlastDL-s=0NsetsDL-1UDAI,smodTDNTB,maxDL,MC+s=0NsetsDL-1m=0M-1Nm,sreceived

where

-	in the following

-	a DCI format 1_3 schedules more than one PDSCH receptions, providing transport blocks with enabled HARQ-ACK information, on more than one serving cells, or

-	a DCI format 1_3 schedules more than one PDSCH receptions, providing transport blocks with enabled HARQ-ACK information, on a single serving cell c, for which the UE is not provided nrofHARQ-BundlingGroups or is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max>1

-	a dormancy indication is considered as a PDSCH reception, providing a single transport block with enabled HARQ-ACK information, corresponding to the first SLIV in the row of pdsch-TimeDomainAllocationList or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, if the UE is not provided nrofHARQ-BundlingGroups for a corresponding serving cell

-	a dormancy indication is considered as one or more PDSCH receptions providing respective one or more transport blocks with enabled HARQ-ACK information, corresponding to a TBG with smallest index or a PDSCH reception group with smallest index, if the UE is provided nrofHARQ-BundlingGroups for a corresponding serving cell

-	VDAI,mlastDL is the value of the total DAI field in a last DCI format 1_3 the UE detects in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format 1_3. VDAI,mlastDL=0 if the UE does not detect any DCI format 1_3 in any of the M PDCCH monitoring occasions.

-	UDAI,s is the total number of DCI format 1_3 that the UE detects within the M PDCCH monitoring occasions for the set s of serving cells. UDAI,s=0 if the UE does not detect any DCI format 1_3 associated with scheduling on set s of serving cells in any of the M PDCCH monitoring occasions.

-	NTB,maxDL,MC is

-	NTB,maxDL,MC=NsetsHARQ-ACK,max if pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided for at least one serving cell in the PUCCH group;

-	NTB,maxDL,MC=NsetsTB,max if pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is not provided for any serving cell in the PUCCH group and harq-ACK-SpatialBundlingPUCCH is not provided;

-	otherwise, NTB,maxDL,MC=Ncells,setDL,max.

-	Nm,sreceived is

-	the number of transport blocks, in PDSCH receptions not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided and nrofHARQ-BundlingGroups is not provided

-	the number of more than one PDSCHs, not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, scheduled by a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided and nrofHARQ-BundlingGroups is not provided

-	the number of transport block groups (TBGs) with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, from/based on PDSCH receptions associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided and nrofHARQ-BundlingGroups is provided

-	the number of PDSCH groups, with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, from/based on PDSCH receptions associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided and nrofHARQ-BundlingGroups is provided.

Table 9.1.3-1: Value of counter DAI for NC-DAIDL=2 and of total DAI

*** Unchanged parts are omitted ***

# References:

- RP-242408, New WID: Multi-carrier enhancements for NR Phase 2, RAN#105, Lenovo.

- R1-2503155, Consolidated Rel-19 higher layers parameters list Post RAN1#120bis, Moderator (Ericsson).

- R1-2503547, Discussion for HARQ-ACK aspects of Rel-17 multi-PDSCH scheduling, RAN1#121, Samsung.

- R1-2503980, Draft CR for HARQ-ACK aspects of Rel-17 multi-PDSCH scheduling, RAN1#121, Samsung.

- R1-2503173, Introduction of multi-carrier enhancements for NR Phase 3 [in TS 38.213], Samsung.

- R1-2503174, Summary of email discussions [120bis-R19-38.213-NR_MC_enh2], Moderator (Samsung).