3GPP TSG-RAN WG1 Meeting #121	R1-2503589

St. Julian’s, Malta, May 19th - 23rd, 2025

Agenda item:		9.13.1

Source:				Samsung

Title:					Further details on TFI for 5G Broadcast phase-2

Document for:		Discussion & Decision

# 1 Introduction

RAN1 has reached many broad agremeents in previous meetings and also, identified a set of open issues. In this paper, we discuss the open issues and address further details on Time and Frequency Interleaving for 5G Broadcast Phase 2.

# 2 Discussion

## 2.1. Time Interleaver design – Configuration aspects

Co-existance for legacy and R19 transmission and UEs:

As Time interleaving is being introduced in R19, while 5G Broadcast has been specified since R15, it is essential that coexistence of legacy and R19 transmissions and UEs is ensured in the new system.

Defining separate R19 PMCHs in addition to legacy PMCHs can ensure co-existance for legacy and new set of UEs which differ in their reception capabilities with respect to TFI based transmission, configurations and scheduling requirements. Table 1 depicts an extension list for R19 PMCHs (PMCH-InfoExt-r19) that provides a clean co-existance approach as the PMCH-InfoExt-r19 ASN configurations are not accessible to legacy UEs while legacy configurations can still be accessed, whereas new set of UEs can access both legacy configurations (w/o TFI) and R19 configurations (w/ TFI). The system can, thus, be judiciously configured to meet a diversity of UEs and transmission requirements on need basis.

Proposal 1: Coexistance for legacy and R19 transmission and UEs is achieved with defining R19 PMCHs to cater to Time interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs.

Configuration for Time interleaver:

In R1#120bis, the following options were shortlisted for configuration of timer interleaver

Open Issue: - Options to configure TI: - Option 1: Per PMCH configuration, e.g., via pmch-Config - Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE, with: - Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching. - Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g., that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

At the outset, we think a configuration signaling through MSI MAC CE is an overkill as TFI configuration is not expected to be changing dynamically. Time interleaver configuration is a typical MBSFN physical channel design attribute such as MCS and likewise, is better suited for RRC signaling based configuration.

Proposal 2: RRC signaling is used for configuring the values M and N

Next question concerns the choice between per PMCH configuration or per MBMS session based configuration for time interleaver parameters (e.g. configurable values of M and N).

We understand the motivation for per session approach (Option 2) is more flexibility for Time Interleaving parameters M/N configuration per service. However, service requirements (mobility, coverage and reliability) and diversity of M/N configurations should be achieved by mapping service MTCH to appropriate PMCH (specifically, SCS/FFT size is configured per MBSFN area and data MCS is configured per PMCH). Moreover, broadcast system can provide a couple of PMCHs (up to 15 PMCHs allowed per MBSFN area) that can fulfil diverse transmission charateristerics.

There are the serious drawbacks with TI configuration per session approach:

- dataMCS is configured per PMCH and fundamentally limits the range of M and N values possible for the associated MTCHs. Conversely, if one service has large M and N, it will reduce dataMCS applicable for all other MTCHs, which is a big drawback.

- Tproc is not a desirable interruption and we can avoid this with per PMCH approach

- Excessive MCCH signalling burden with per session configuration. For example, a theoretical maximum 1024 sessions requiring a large increase in the size of MCCH information message with per session TI configuration. This may be drastic with requiring potential segmentation and spreading of MCCH message over more than one Transport Block, especially in case of smaller carrier bandwidths.

- Scheduler operation and reception operation is difficult with many different M, N based MTCHs in same MSI periodicity (e.g. if UE has to share HARQ resources between broadcast and unicast, it is difficult to pursue w/o prior knowledge of M and N in MSI period)

Therefore, RAN1 should prefer option 1 (Per PMCH configuration for TI parameters, e.g., via pmch-Config). Table 1 illustrates how per PMCH based Time interleaving configuration can be achieved.

Proposal 3: Configuration for Time interleaving (e.g. M and N values) is per PMCH configuration i.e. via pmch-Config.

Table 1: Configuration for legacy and R19 PMCHs

PMCH-Config-r12 ::=					SEQUENCE {

sf-AllocEnd-r12						INTEGER (0..1535),

dataMCS-r12							CHOICE {

normal-r12							INTEGER (0..28),

higerOrder-r12						INTEGER (0..27)

},

mch-SchedulingPeriod-r12		ENUMERATED {

rf4, rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024},

...,

[[	mch-SchedulingPeriod-v1430		ENUMERATED {rf1, rf2}			OPTIONAL	-- Need OR

]]

}

PMCH-InfoListExt-r19 ::=		SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-InfoExt-r19

PMCH-InfoExt-r19 ::=				SEQUENCE {

pmch-Config-r19						PMCH-Config-r19,

mbms-SessionInfoList-r19			MBMS-SessionInfoList-r9,

...

}

PMCH-Config-r19 ::=					SEQUENCE {

sf-AllocEnd-r12						INTEGER (0..1535),

dataMCS-r12							CHOICE {

normal-r12							INTEGER (0..28),

higerOrder-r12						INTEGER (0..27)

},

mch-SchedulingPeriod-r19		Extended set of scheduling periods

pmch-TimeInterleavingConfig-r19   SEQUENCE {

pmch-TimeInterleaving-M          ENUMERATED {4,8,16,32,..}

pmch-TimeInterleaving-N          ENUMERATED (4,8,16,..}

}

}

MBMS-SessionInfo-r9 ::=			SEQUENCE {

tmgi-r9								TMGI-r9,

sessionId-r9						OCTET STRING (SIZE (1))		OPTIONAL,	-- Need OR

logicalChannelIdentity-r9			INTEGER (0..maxSessionPerPMCH-1),

...

}

Alternate periodicities for mch-SchedulingPeriod:

Open Issue: - Values of the new periodicities aimed at reducing resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern.

As shown in Table 2, the resource wastage performance can be significantly improved with introducing some suitable intermediate values as compared to what we would achieve with legacy MSI periodicities. Also we can notice the resource wastage can be about 6% with smaller periodicities and can be almost avoided when larger periodicities are considered. Further, note that as the PMCH would cater multiple MTCHs, the MSI periodicities (or MCH-SchedulingPeriods) used would typically be large.

Further we impose a condition that alternate MSI periodicities must be an integral multiple of 40 ms so as to properly allow CAS sub-frame occurence. This will also facilitate the scenario when CAS muting is deployed for the Tel-19 5G Broadcast, and the ON-period for CAS muting is also configured in terms of (K • 40) ms.

Table 2: Resource wastage for legacy and alternate MSI periodicities

| MSI periodicity TMSI (ms) | N | M | P= (#(M×N)-units) =TMSIM×N | MTCH duration P×M×N (ms) | CAS+MSI Overhead (ms) | Total Duration with CAS Tuseful (ms) | Percent Resource wastage (TMSI-Tuseful)/TMSI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | 4 | 16 | 1 | 64 | 1+1 | 66 | 17.5 |
| 160 | 8 | 16 | 1 | 128 | 3+1 | 132 | 17.5 |
| 280 | 8 | 16 | 2 | 256 | 6+1 | 263 | 6.07 |
| 320 | 8 | 16 | 2 | 256 | 6+1 | 263 | 17.81 |
| 560 | 16 | 32 | 1 | 512 | 12+1 | 525 | 6.25 |
| 640 | 16 | 32 | 1 | 512 | 12+1 | 525 | 17.97 |
| 1080 | 16 | 32 | 2 | 1024 | 25+1 | 1050 | 2.78 |
| 1280 | 16 | 32 | 2 | 1024 | 25+1 | 1050 | 17.96 |
| 2120 | 32 | 32 | 2 | 2048 | 51+1 | 2100 | 0.09 |
| 2560 | 32 | 32 | 2 | 2048 | 51+1 | 2100 | 17.97 |
| 4240 | 32 | 64 | 2 | 4096 | 102+1 | 4199 | 0.009 |
| 5120 | 32 | 64 | 2 | 4096 | 102+1 | 4199 | 17.99 |

Proposal 4: RAN1 to adopt alternate values for MSI scheduling periods to reduce resource wastage when using time-interleaved transmission for (M x N) sub-frames. The set of alternate value for MSI scheduling periods include at least values of {280, 560, 1080, 2120, 4240}.

Larger values for M and N:

Open Issue: - Larger values of M and/or N, including M=64, N={32,64}

In our view, with agreed values of M=32 and N=16, we have already achieved an interleaving depth of 512 ms, which is quite comparable with other broadcasting systems. Any larger values for M and N seem not reasonable, given the impact it has on the UE implementation requirements, processing capabilities and latency performance.

Proposal 5: RAN1 not support further larger values of M and/or N, inclusing M = 64, N = {32, 64}

UE behaviour when configurations exceed UE’s capability:

Open Issue: - UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI.

It is not reasonable to expect the UE implementation to take chances in terms of attempting decode for the configuration parameters that are beyond the UE’s reported capabilities, as this would make the performance unreliable for the UE receiver and affects the user satisfaction. UE would be well aware from the configuration parameters (e.g. M, N values and associated soft buffer size and TBS) that are being used for the Rel-19 5G Broadcast services and need not receive if the configurations exceed the UE capabilities

Proposal 6: UE behaviour should not be required to receive 5G Broadcast services when the associated configuration parameters (e.g. M, N values) exceed the UE capabilities (e.g. in terms of soft buffer size, maximum TBS supportable in a TTI).

## 2.2. Time Interleaver design – Scheduling aspects

UE behaviour for handling residual mismatch for scheduling:

Open Issue: - Need and details to specify UE behaviour in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity, considering: - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity - Option 2: Drop the time-interleaved TBs which could not be successfully decoded. - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour) - Other Options not precluded

For the case of residual mismatch, we understand following cases can be considered:

Figure 1: Padding of the residual mismatch part (< M x N subframes) in the MCH Scheduling Period

Case 1: Network scheduler only transmits those number of MTCHs for which M/N transmission pattern can be fully accommodated within the MCH-SchedulingPeriod and remaining (residual mismatch) part of the MCH-SchedulingPeriod is unused / padded.

Figure 2: Transmission of last MTCH Z in the residual mismatch part (< M x N subframes) in the MCH Scheduling Period

Case 2: Network scheduler utilizes the remaining part to still schedule the last MTCH with partial M/N transmission pattern until the end of the MCH-SchedulingPeriod. UE drops TBs which could not be successfully decoded (successful decoding need not require all RVs) and processes the successfully decoded TBs. Performance may vary based on the amount of the mismatch and loss perceived by UE

We think for case 2, a satisfactory reception performance needs to be ensured and is dependent on amount of remaining part (residual mismatch) available in the MCH-SchedulingPeriod. In our view, the decision whether to schedule or not schedule MTCH transmission in the remaining part of the MCH-SchedulingPeriod can be left to network scheduler and if scheduled, UE needs to decode until the end of the MCH-SchedulingPeriod as per MSI (legacy behaviour).

That is, option 2 is sutiable for the UE behaviour. Option 1 is not reasonable as UE should not drop time-interleavedTBs just on basis of not all the N RVs transmitted within the MSI periodicity, as successful decoding not necessarily require all RVs and UEs cannot pre-judge decoding result.

Proposal 7: RAN1 adopt option 2 to specify UE behaviour in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity, considering:

  - Drop the time-interleaved TBs which could not be successfully decoded.

    - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour)

MCH Scheduling Information signalling for R19 PMCHs:

Open Issue: - Signalling aspects, such as RRC parameters, including: - In case TI is configured per MBMS session, study whether to be done via MBMS-SessionInfo or via MAC CE. - Whether signaling is needed to indicate not allowing multiplexing of two MTCHs in same subframe - Whether signaling is needed to indicate not applying Time interleaving to subframe carrying MSI/eMSI/MCCH - Whether signaling is needed to indicate not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference - Whether signaling is needed to indicate inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios - Other aspects

We think there is no need for explcitily signalling these aspects and rather, they need to be incorporated in the MAC specification.

TS 36.321 5.12 MCH reception ... The MAC entity shall assume that the first scheduled MTCH starts immediately after the MCCH or the MCH Scheduling Information MAC control element or the Extended MCH Scheduling Information MAC control element if the MCCH is not present, and the other scheduled MTCH(s) start immediately after the previous MTCH, at the earliest in the subframe where the previous MTCH stops. …

In order to support basic transmission pattern of MxN subframes for Time interleaver, it seems essential to bring a few enhacements for MSI as compared to legacy specifications (shown in spec snippet), including

- Not allowing multiplexing of two MTCHs in same subframe

- Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

- Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to TI difference

- Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

As these enhancements are not backward compatible with legacy specifications, it is required that such enhancements are kept as minimal as possible and are applied only for scheduling of R19 PMCHs based MTCHs. Further, these enhancements need to be captured in the (RAN2) MAC specification.

Proposal 8: RAN1 asks RAN2 to adopt minimal multiplexing enhancements for MCH reception are applied only for scheduling of R19 PMCHs based MTCHs and include:

  - Not allowing multiplexing of two MTCHs in same subframe

  - Not applying Time insterleaving to subframe carrying MSI/eMSI/MCCH

  - Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference

  - Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

## 2.3. Time Interleaver design – Rate-matching aspects

Starting pointer for the circular buffer:

Open Issue: - Value of the starting pointer for the circular buffer, including considering the following proposals: - Proposal1: For the TB transmitted in N subframes, the starting pointer for the circular buffer is set as k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe and mapped sequentially in the N subframes (i.e., rvn to subframen pattern is [0,1,2,…N-1]). - Proposal 2: For the TB transmitted in N subframes, the starting pointer for the circular buffer is set as k0n=2RsubblockTC+nEmin, where Emin=NLQmG'/C (as per TS 36.212), and n∈{0…N-1} is mapped sequentially in the N subframes.

As per proposal 1, RVs are placed sequentially in the circular buffer to allow large portion of systematic bits to be transitted by each redundancy version and to facilitate a greater number of RVs in the circular buffer for higher time interleaver depth. Unlike legacy, proposal 1 also avoid puncturing of systematic bits to make RV version sequential i.e. coded bits for rvi+1 start right after rvi.

While proposal 2, essentially maps circular buffer in n sequential transmitting buffers for TBs to be transmitted in N sub frames.

In our view, if RV versions or transmitting buffer per nth subframe are transmitted sequentially for N subframes and number of bits in each RV version remains same then both proposals will result in same outcome with respect to functionality. Therefore, we don’t have a strong preference for either option but to stay consistent with legacy, proposal -1 is preferred.

Proposal 9: As both proposals would have same outcome if number of bits in each RV and transmitting buffer for nth subframe are same, however, to stay consistent with legacy, proposal-1 is preferred.

Consistent LBRM for diversity of UEs:

Open Issue: - Mechanisms so that the calculation of LBRM parameters (e.g. NIR and NCB) is consistent between the broadcast network and the (multiple) UEs that are receiving the broadcast transmission.

Based on the device type and device category, LBRM paramers (e.g., NIR and NCB) could be different for among multitude of UEs. NW has no information about LBRM params of UEs unless there is a capability signalling from the UEs. Moreover, network is still required to select one among many of the reported LBRM capabilities reported. To bring a consistent and coordinated mechanism, NW should provision signaling for UEs to indicate the required LBRM paramters required for the broadcast reception.

We need more discussion on how to indicate to the UEs and possible use of signaling from the NW.

Proposal 10: RAN1 to consider a signalling approach to indicate to UEs, the common LBRM parameters for broadcast reception.

Derivation of maximum TBS:

Open Issue: - Derivation of the maximum TBS a UE supports for the scaled TB when the UE category has multiple maximum TBS

Derivation of maximum TB size that UE can support for scaled TB could be infered from TS 36.306 Table 4.1-4, wherein maximum TB size for different category of UEs are specified. For some UE category more than one TB Size has been specified based on the choice of modulation coding scheme (MCS). For broadcast, TB size scaling will have direct bearing on LBRM paramters and Nsoft bits requirement.  Note that MCS adaptation is dynamic in unicast communication, while MCS could be adaptated on semi-static basis as per PMCH configuration.

In our view, choice of maximum TB size should be choosen based on max TB size value within a category for a higher modulation order. Since UE can always be configured on smaller TB size for a PMCH configuration and re-adjust the value of LBRM paramters.

Proposal 11: Select maximum TB size for a category for higher modulation order, in case if more than one TB size has been specified for different modulation orders.

Codeblock Cyclic shift:

Open Issue: - Adoption of cyclic shift to ensure that different RVs of the same codeblock are evenly distributed to fully exploit the frequency diversity. - Circular Shift the RVs of different codeblocks within i -th Subframe in the following codeblock index order: CB[0+∆ mod C] , CB[1+∆ mod C] , ...,CB[j] , ...,CB[C-1+∆ mod C] , where circular offset ∆=-i, CB[j] denotes the codeblock index, and C is the number of codeblock. Note: the circular granularity is per RV. - Other options

The proposed cyclic shift of the various codeblocks, as part of code block concatenation, is expected to distribute the channel burst errors across the codeblocks, rather than limiting them to just one or two codeblocks. This provides a form of codeblock diversity and helps improve the performance. However, care must be taken to study this method in the context of the number of columns of the frequency interleaver. In what follows we depict the dependency of this method on the number of columns of the frequency interleaver.

As a brief description on this dependency, the channel burst errors that are contiguous are effectively placed R symbols (length of a row in frequency interleaver) apart after the frequency de-interleaver at the receiver. If the number of columns C1 (not to be confused with C the number of codeblocks) is the same as C, then R becomes equal to the length of a codeblock. In this case, at the output of the frequency deinterleaver, the channel burst errors are distributed R symbols apart or equally between all codeblocks. Cyclic shift of code blocks is not expected to provide any advantages. This can be seen from Figure 3 and Figure 4. If the the number of columns is C2, then R is twice the length of a codeblock. This implies that the channel burst errors are distributed across two codeblocks. Two of the four codeblocks in this case will have errors and the other two codeblocks will not have any errors. So, a cyclic shift of the codeblocks will help distribute the errors evenly across all codeblocks and there is indeed a gain, as can be seen from Figure 5 and Figure 6.

Figure 3: Depiction of channel burst errors after frequency de-interleaver at the receiver. Number of codeblocks is four. Number of columns of frequency interleaver is four. Codeblocks are not cyclic shifted.

Figure 4: Depiction of channel burst errors after frequency de-interleaver at the receiver. Number of codeblocks is four. Number of columns of frequency interleaver is four. Codeblocks are cyclic shifted.

Figure 5: Depiction of channel burst errors after frequency de-interleaver at the receiver. Number of codeblocks is four. Number of columns of frequency interleaver is two. Codeblocks are not cyclic shifted.

Figure 6: Depiction of channel burst errors after frequency de-interleaver at the receiver. Number of codeblocks is four. Number of columns of frequency interleaver is two. Codeblocks are cyclic shifted.

We now discuss how fractional cyclic shifts of the codeblock can be important. (By fractional we mean fraction of the rate matching output codeblock length). Turbo decoders are designed to correct random occurrences of errors. Burst errors are handled by the interleaver of the turbo decoder and are spread apart. The distance between burst errors can affect the interleavers ability to spread out the errors. The distance between the burst errors can impact the convergence of the decoder itself. So, one can conclude that the distance between the burst errors has an impact on the performance, inspite of the inerent decoder’s ability to spread out, to an extent, the burst errors with the use of it’s interleavers. In Figure 7(A), it can be seen that cyclic shift by integer codeblock values, the burst errors due to the frequency interleaver are about two codeblocks away, but with intelligent fractional cyclic shifts in Figure 7(B), the burst errors are three codeblocks away, thereby increasing the distance between the burst errors by more than a codeblock.

Figure 7: Depiction of channel burst errors after frequency deinterleaver at the receiver. Number of codeblocks is four. Number of columns of frequency interleaver is two. Codeblocks are cyclic shifted (integer values of rate matching output length of codeblock) in (A). Codeblocks are cyclic shifted (fractional values of rate matching output length of codeblock) in (B). The distance between erors is roughly twice codeblock length when integer cyclic shift values are used in (A), while when fractional cyclic shift values are used, as in (B) the distance between errors increases to thrice the codeblock length.

Observation 1: The performance of cyclically shifting the codeblocks is dependent on the number of columns of the frequency interleaver.

Observation 2: The codeblock concatenation is benefited from fractional (fraction of the length of a codeblock) cyclic shifts of codeblocks in the construction of codeblock concatenation.

Proposal 12: The output of codeblock concatenation be CB[0+∆ mod C](P:Q[0+∆ mod C]) , CB[1+∆ mod C] , ...,CB[j] , ...,CB[C-1+∆ mod C] , CB[0+∆ mod C] (1:P-1)  where CBf(a:b) denotes elements a to b of the CBfand Q[f] is the number of elements of CBf, where CBf is the fth codeblock.

## 2.4. Frequency Interleaver design – Open issues

Cyclic shift offset for the frequency interleaver output:

Open Issue: - Application of a cyclic shift offset for the concatenated frequency interleaver outputs of all OFDM symbols before the mapping to REs of a subframe, once the rate matching details regarding k0n are agreed.

One can see that the codebook cyclic shift and cyclic shift offset of the frequency interleaver attain the same effect of distributing burst errors across codeblocks. The benefits of both methods are therefore similar. The fractional cyclic shifts of the codebook cyclic shift method give a granularity in terms of bits whereas this method gives a granularity in terms of constellation symbols. Furthermore, based on discussions pertaining to Figure 3 - Figure 6 and in Section 2.3 on codeblock cyclic shift, this method has a dependency on the number of columns of frequency interleaver and has to be designed in conjunction with that.

Proposal 13: For the frequency interleaver output, instead of start reading the output row-wise from (0, 0) element of the row-column interleaver, propose to start the read up from some other starting point (a, b) of the row-column interleaver, where at least one of a, b is non-zero and configurable. Furthermore, it is proposed that the number of columns belesser than the number of codeblocks when this method is used.

Cyclic shift offset for the frequency interleaver output:

Agreement The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH, - Working assumption: - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.

Based on the discussions in Section 2.3, and  Figure 3 - Figure 6, it is important that if codebook cyclic shift and/or cyclic shift offsets of frequency interleaver outputs are adopted, then the number of columns should be selected appropriately. In general, a lower value of C is preferred, as that ensures a higher value of R and effect of channel bursts at the receiver after the deinterleaver is that two consecutive errors are separated by R symbols.

Proposal 14: Determination of the number of columns of the frequency interleaver to be studied in conjunction with codebook cyclic shift and cyclic shift offset of frequency interleaver output methods. A value of C1 (the number of columns) lower than the number of codeblocks is proposed.

# 3 Conclusion

Based on the discussion above, request RAN1 to consider the following observations and proposals:

Time Interleaver design – Configuration aspects

Proposal 1: Coexistance for legacy and R19 transmission and UEs is achieved with defining R19 PMCHs to cater to Time interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs.

Proposal 2: RRC signaling is used for configuring the values M and N

Proposal 3: Configuration for Time interleaving (e.g. M and N values) is per PMCH configuration i.e. via pmch-Config.

Time Interleaver design – Scheduling aspects

Proposal 4: RAN1 to adopt alternate values for MSI scheduling periods to reduce resource wastage when using time-interleaved transmission for (M x N) sub-frames. The set of alternate value for MSI scheduling periods include at least values of {280, 560, 1080, 2120, 4240}.

Proposal 5: RAN1 not support further larger values of M and/or N, inclusing M = 64, N = {32, 64}

Proposal 6: UE behaviour should not be required to receive 5G Broadcast services when the associated configuration parameters (e.g. M, N values) exceed the UE capabilities (e.g. in terms of soft buffer size, maximum TBS supportable in a TTI).

Proposal 7: RAN1 adopt option 2 to specify UE behaviour in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity, considering:

  - Drop the time-interleaved TBs which could not be successfully decoded.

    - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour)

Proposal 8: RAN1 asks RAN2 to adopt minimal multiplexing enhancements for MCH reception are applied only for scheduling of R19 PMCHs based MTCHs and include:

- Not allowing multiplexing of two MTCHs in same subframe

- Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

- Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference

- Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

Time Interleaver design – Rate-matching aspects

Proposal 9: As both proposals would have same outcome if number of bits in each RV and transmitting buffer for nth subframe are same, however, to stay consistent with legacy, proposal-1 is preferred.

Proposal 10: RAN1 to consider a signalling approach to indicate to UEs, the common LBRM parameters for broadcast reception.

Proposal 11: Select maximum TB size for a category for higher modulation order, in case if more than one TB size has been specified for different modulation orders.

Observation 1: The performance of cyclically shifting the codeblocks is dependent on the number of columns of the frequency interleaver.

Observation 2: The codeblock concatenation is benefited from fractional (fraction of the length of a codeblock) cyclic shifts of codeblocks in the construction of codeblock concatenation.

Proposal 12: The output of codeblock concatenation be CB[0+∆ mod C](P:Q[0+∆ mod C]) , CB[1+∆ mod C] , ...,CB[j] , ...,CB[C-1+∆ mod C] , CB[0+∆ mod C] (1:P-1)  where CBf(a:b) denotes elements a to b of the CBfand Q[f] is the number of elements of CBf, where CBf is the fth codeblock.

Frequency Interleaver design

Proposal 13: For the frequency interleaver output, instead of start reading the output row-wise from (0, 0) element of the row-column interleaver, propose to start the read up from some other starting point (a, b) of the row-column interleaver, where at least one of a, b is non-zero and configurable. Furthermore, it is proposed that the number of columns belesser than the number of codeblocks when this method is used.

Proposal 14: Determination of the number of columns of the frequency interleaver to be studied in conjunction with codebook cyclic shift and cyclic shift offset of frequency interleaver output methods. A value of C1 (the number of columns) lower than the number of codeblocks is proposed.