3GPP TSG RAN WG1 Meeting #121                                                                             R1-2503627

Malta, Malta, May 19th – 23rd, 2025

Source:	TCL

Title:	Discussion on AI/ML beam management

Agenda Item:	9.1.1

Document for:	Discussion and Decision

- Introduction

In RP-234039[1], the following WIDs on AI/ML Beam management have been agreed.

Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”). - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”). - Specify necessary signaling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any. - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE. NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2.

In NR, beam pairing and refinement are managed by the P1/P2/P3 processes in the DL, which are dedicated to beam management (BM). Specifically, P1 handles initial beam pairing, while P2 and P3 respectively handle Tx and Rx beam refinement. With the incorporation of AI/ML into conventional beam management, the specification must consider the implications of this integration and provide adequate enhancements to signaling and procedures to support AI/ML-based beam management.

Besides the beam pairing and refinement, the beam failure detection and recovery are also essential component of the beam management. The conventional framework of BFD and BFR resolves the beam failure problem by periodically perform the measurement and maintain a list of candidate beams in case of beam failure. This requires periodical RS transmission and measurement, which is neither resource nor energy efficient. Furthermore, the BFR may not be effective in many cases, such as in case of poor candidate beam quality.

- AI/ML Beam Prediction and Selection

  - General Framework of AI/ML Beam Management

In cross phase AI/ML BM, the beam prediction is used to reduce the overhead associated with beam sweeping when selecting gNB Tx beams in P1 and P2. Specifically, it uses a subset or all of the SSB beams to predict the optimal beam(s) selected from the CSI-RS beams, instead of a two-stage exhaustive search over all beams in P1/P2/P3. As a result, the necessary configurations and reports related to AI/ML beam prediction must be signaled between the gNB and UE.

- Cross Phase AI BP (NW-sided Model)

| Process | Functionality | Description |
| --- | --- | --- |
| P1 | Tx Beam Selection | NW sweeps DL-Tx beam, and UE sweeps DL-Rx beam and obtains a set of measurements, UE reports the measurements to NW as model inputs. NW uses an AI/ML model to predict the best DL-Tx beam. |
| P2 | Rx Beam Selection | NW transmits with the best DL-Tx beam and UE refines its Rx beam by AI/ML BP or beam sweeping. |

- Cross Phase AI BP (UE-sided Model)

| Process | Functionality | Description |
| --- | --- | --- |
| P1 | Tx Beam Selection | NW sweeps DL-Tx beam, and UE sweeps DL-Rx beam and obtains a set of measurements, UE uses an AI/ML model to predict the optimal DL-Tx beam(s) and report to NW. |
| P2 | Rx Beam Selection | NW transmits with the best DL-Tx beam and UE refines its Rx beam by AI/ML BP or beam sweeping. |

Based on above analysis, we have the following proposal.

Proposal 1: Conventional P1/P2/P3 approach to beam pairing/refinement can be simplified by AI/ML beam prediction to two phases.

- FFS: If the AI/ML model in P1 is able to predict the DL-Tx-Rx beam pairs, only one phase is required for beam pairing as the Rx beam selection is also acquired in P1.

The performance of AI/ML beam prediction depends on both the model and the input measurements. To generate the labels and AI/ML inputs, new RSRP and SSBRI/CRI report behaviors may need to be collected. Therefore, the pattern of sparse beam measurements needs to be discussed.

The beam pattern goes beyond deciding the relationship between beam set B and beam set A. Set B comprises sparse samplings in the spatial domain through RS measurements, and the beam measurements alone constitute a pattern. The challenge in deciding the pattern of set B is how to capture the spatial features to the fullest extent with sparse samplings. Generally, samples near the connected beam will have higher correlations for low-speed UEs, while samples far from the connected beams will have lower correlations for low-speed UEs. Thus, the patterns of set B can be designed following the distribution of correlation in the spatial domain. The spatial correlation between the samples depends on the physical channel geometry in the long term, while being affected by UE mobility in the short term. Therefore, both fixed regular patterns, where beams are spatially sampled with equal density while repeating across time, and random patterns, where beams are sampled with different densities generated randomly in different time durations, should be considered.

Figure 1 Fixed Pattern for AI/ML BM Measurement

Figure 5 illustrate a fixed regular pattern that the four dark blocks denote the measured beams in spatial domain. Alternatively, the random pattern for measurement can be another option for further discussion. We have the following proposal based on above analysis.

Proposal 2: Measurement patterns for AI/ML beam management should be designed for spatial domain beam prediction, and should consider both the fixed regular and random patterns.

  - AI/ML Beam Management Based on Beam Prediction

In RAN1 #118bis, the following agreement has been achieved:

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring),

- At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring

  - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation

  - FFS: on other details including how to configure the resource set/resources for monitoring, including

    - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric

FFS other alternatives

We note that the prediction accuracy metric relies how the prediction results is reported, and the agreement of the inference report is provided in RAN1 #116 as follows,

Agreement

For UE-sided model, at least for BM-Case1, for content in the report of inference results, support

- Opt 1: Beam information on predicted Top K beam(s) among a set of beams

- Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

  - At least K=1 and more, FFS on max value

  - FFS on beam information

  - FFS on the definition of predicted Top K beam(s)

  - FFS on definition of reported RSRP when applicable

  - FFS on other information in the report with potential down selection among the following options

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

  - FFS on the quantization method of probability information

  - Probability information is the probability of the beam to be the Top 1 or Top K beam

- Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

  - FFS on definition of reported RSRP

  - FFS on the definition and quantization method of confidence information

- Other options are not precluded.

where the set of beams is Set A, i.e., the beams for UE prediction.

The agreement of RAN1 #116 rise a problem on how to choose one beam out of the K for data transmission. The solution proposed by most of the companies is to perform a second-round beam sweeping upon the Top-K beams, i.e., the Top-K beam sweeping. An evaluation from one source [2] shows that with top K beam sweeping, the beam prediction accuracy will be improved from 80% to more than 90% at additional cost. Many companies support to perform the Top-K beam sweeping at least for BM-Case2 as the prediction result may be less reliable in feature time slots. However, the Top-K beam sweeping introduces extra configuration and measurement overhead.

The procedure of Top-K beam sweeping in BM-Case1 and BM-Case2 are illustrated in Figure 1, where BM-Case1 can be considered as a special case of BM-Case2 when n=1.

Figure 2 Model inference and Top-K beam sweeping

It is straightforward for BM-Case1 that the performance monitoring can be directly carried out after model inference. However, the situation is complicated for BM-Case2, as performance monitoring should verify all predicted beams in future N slots, which may require multiple beams sweeping procedures that causes considerable overhead.

In general, the Top-K beam sweeping is similar to legacy P2 beam sweeping that the UE fixed to one Rx beam to calculate the RSRP measured from different Tx beams selected from the Top-K beams, hence the CSI report configuration framework for legacy beam management can be reused. However, if the Top-K beam sweeping is adopted in the model inference stage, then the ground-truth measurements of the Top-K beams should be measured from may be multiple aperiodic CSI resource sets and reported to the gNB. Meanwhile, according to the agreement of RAN1 #118bis, the model monitoring also requires these ground-truth measurements.

Observation 1: The Top-K beam sweeping introduces extra measurement overhead especially for BM-Case2.

To alleviated the burden of beam sweeping, some improvements could be made on the CSI report of Top-K beams by integrating performance-monitoring related information to facilitate the performance monitoring. Different form existing solutions that based on dedicated measurement for monitoring, we propose to reuse the Top-K beam sweeping results in model monitoring to reduce signaling overhead. This will lead to a different configuration and content in the CSI report.

Observation 2: It is straightforward to consider merging of the measurement of Top-K beam sweeping and model monitoring for overhead/complexity reduction purposes.

In case that the model monitoring and inference processes is assumed to be independent, they are still possible to be connected by the Top-K beam sweeping. In general, the model monitoring can be an always on process or a configured/triggered process, which will implement the measurement on periodic and aperiodic CSI resource sets, respectively. The role of Top-K beam sweeping in the two cases are analyzed as follows.

- Case 1: For periodic performance monitoring, the always on monitoring provides the timely reaction towards model failure. However, it causes considerable overhead for measurements on periodic CSI resource sets. Moreover, additional CSI report overhead for monitoring is introduced when the NW-side model monitoring is adopted.

  - The Top-K beam sweeping have no impact on the periodic monitoring.

- Case 2: For aperiodic performance monitoring, the main benefits of configured/triggered monitoring is the small overhead comparing to the always on one. However, it sacrifices the capability of rapid response to model failure. Therefore, how to efficiently trigger the aperiodic monitoring process to balance the overhead and performance become a crucial problem.

  - The Top-K beam sweeping could trigger an aperiodic monitoring.

  - Alleviating the data collection overhead of monitoring by reusing the Top-K beam sweeping results.

In another case where the model monitoring relies on the inference, i.e., the model monitoring process only start running after a model is activated for inference, the two processes can be integrated for overhead reduction purposes. In an integrated inference and monitoring processes, the Top-K beam sweeping is considered as an instantaneous or short-term measurements from the model monitoring’s perspective, i.e., it can be expanded to a ‘reduced-capability’ model monitoring. Therefore, upon receiving the Top-K beam sweeping results, the NW needs to decide on next steps, e.g., fallback, another ‘full-step’ monitoring or something else? In consequence of the above UE behaviors, the design of the current CSI report configuration and content should be revisited for integrating the Top-K beam sweeping and the performance monitoring.  We suggest RAN1 to study the impact of introducing the Top-K beam sweeping on model inference and monitoring procedures for overhead reduction purposes and we have the following proposal.

Proposal 3: The measurement results collected during Top-K beam sweeping could be used in the performance monitoring.

- Option 1: The results obtained from Top-K beam sweeping could be used to trigger the performance monitoring.

- Option 2: The results obtained from Top-K beam sweeping could be used to calculate the metrics of the performance monitoring.

- CSI processing Criteria

  - AI/ML processing unit

In RAN1 #120b, the following agreement has been achieved.

Agreement For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection: - Option 1: only dedicated AI/ML PU is occupied, OAPU=N1 is reported by UE. - And OCPU=0 - Option 2: only legacy CPU is occupied, OCPU=M it is reported by UE. - Option 3: both dedicated AI/ML PU and legacy CPU are occupied, OAPU=N2 is reported by UE. - And OCPU=1 - Note: The supported option by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

The AI-related CSI report requires an AI auxiliary processor to handle the AI/ML model inference, while the legacy CSI report requires only the legacy baseband processor. Furthermore, the non-AI part in AI-related CSI processing, such as the data collection, PUCCH generation, etc., are also handled by the legacy baseband processor. Therefore, it is straightforward to consider that the AI-related CSI processing take up both the AI/ML PU and legacy CPU, where the AI portion of the AI report is calculated using the APU, and the non-AI portion of the AI report is calculated using the CPU. We have the following proposal.

Proposal 4: Both dedicated AI/ML PU and legacy CPU are occupied for AI-related CSI report for inference.

- FFS if the CPU occupied by the AI-related CSI report and the legacy CSI report share the same amount of CPU.

- FFS if data collection for usage of AI/ML model is counted by AI/ML PU or legacy CPU.

If we consider both the CPU and AI/ML PU occupation of the CSI report, the number of supported CSI reports are constrained by both the available CPU and AI/ML PU. Assume that the total number of CPU and AI/ML PU are NCPU and NAPU, and the CPU or AI/ML PU occupied by the mth CSI report are recorded as OCPU and OAPU, respectively. The CPU and AI/ML PU that are currently occupied or reserved are LCPU and LAPU.

We consider firstly the APU constrained case, m=1MAOAPUm≤NAPU-LAPU, where MA is the number of AI-related CSI reports selected. It is also necessary to consider the CPU-constrained case, when the traditional CSI report shares the resource pool with the non-AI portion of the AI-related CSI report we have m'=1MCOCPUm'≤NCPU-LCPU-i=1MC'OCPUi, where MC is the number of selected AI-related CSI reports and MC' is the number of selected legacy CSI reports.

Note that the number of AI-related CSI reports that can actually be supported is less than the minimum between MC and MA. For example, assume that the set of IDs of CSI reports corresponding to MA selected AI-related CSI reports is RA, and the set of IDs of CSI reports corresponding to MC selected AI-related CSI reports is RC then M=|RA∩RC|

Proposal 5: Study the selection rule if the AI-related CSI report to be reported is jointly determined by the legacy CPU and AI/ML PU.

- Option 1: Selection of CSI report prioritizes the AI/ML PU constraint.

- Option 2: Selection of CSI report prioritizes the legacy CPU constraint.

- Option 3: Selection of CSI report jointly considers the legacy CPU and AI/ML PU constraint.

  - AI/ML processing timeline

If Option 3 is adopted, the PU occupation time should separately consider the legacy CPU and AI/ML PU. During the data collection stage, the observation window is not necessarily to be fully occupied and the CPU occupation duration could be configured as X slots from the first or last symbol of each reference signal resource within the observation window. After data collection, i.e., measurement and model input calculation, the AI/ML PU will be occupied during the model inference stage, and the starting of AI/ML PU occupation during could be Y slots away from the first or last symbol of the last reference signal resource within the observation window.

Figure 3 The occupation time of CPU and AI/ML PU

Proposal 6: The occupation time of CPU and A/ML PU should be considered separately. The duration of the PU occupation time could be configured by the NW.

For BM-Case2, the beam prediction result applies to several future time instance. When the CSI report time is located after some of the predicted time instances, the last prediction results before the CSI report are risk to be out of date. In this case, we need to define a valid duration T of the prediction results in the CSI report, which is the basis for determining whether this part of the CSI report is available or not.

Figure 4 Valid time of prediction result result.

Proposal 7: For BM-Case2,  we need to define he valid duration of the prediction result to determine if a prediction result is valid or not.

- The invalid prediction could be omitted in the CSI report for overhead reduction purpose.

- Consistency Between Training and Inference

In RAN1 #120, the following agreement has been achieved.

Agreement

For UE-sided model, for configuring the resource for data collection purpose, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

  - One CSI-ResourceConfigId is configured for Set A.

  - One CSI-ResourceConfigId is configured for Set B.

  - Note: UE performs measurement on all resources

  - One or two associated IDs can be configured in CSI-ReportConfig

    - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

    - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: whether/how to support 'aperiodic' CSI RS

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Agreement

For UE-sided model, in CSI-ReportConfig for inference

- One or two associated IDs can be configured in CSI-ReportConfig

  - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

  - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

The purpose of introducing associated ID is to ensure the consistency of network additional conditions through model training and inference. The basic function of the associated ID is to identify if the wireless transmission environments changes. However, if the associated ID applied to different cells, then the meaning may be ambiguous. Hence before defining the mapping between associated ID and NW additional conditions, we should first clarify the application area of the associated ID, e.g., it is a global ID or cell-wise ID.

Proposal 8: If the associated ID is globally or cross-cell defined, an explicit mapping between the associated ID and the NW additional conditions should be defined.

In AI/ML beam management use cases, the key parameter of network additional conditions is the beam related information. The beam related information could be categorized into two types.

- The first type is the beam index related information, which assumes there is a consensus on the mapping of beam index between the NW and the UE. Such kind of information includes the beam index and beam pointing order.

- The second type is the beam shape related information, which does not require the consensus on the beam index. Such kind of information includes the TX/RX beam codebook, beam shape, beam weight.

In AI/ML beam management, the data collection may across different resource sets and time occasions. Therefore, the mapping of the beam index to the beam shape may vary among different data collection occasions. In this case, it is preferable to contain the second type of the beam related information in the network additional conditions, otherwise the NW and UE should find a way to update the consensus on the mapping of beam index.

Observation 3: The beam shape information is more stationary comparing to the beam index information during data collection.

Proposal 9: The associated ID in AI/ML beam management at least ensures the beam consistency.

FFS: the details of the beam consistency.

- Option 1: The beam consistency means same beam index or beam order in both training and inference.

- Option 2: The beam consistency means same beam shape or beam codebook in both training and inference.

In realistic, the wireless channel environment has a time varying pattern especially in urban area. Therefore, the time information also has non-negligible impacts on the consistency of data collection. If no explicit time stamps are included in the collected data, the associated ID should consider to encode the time information to provide reference of the similarity of the wireless channel environment during data collection.

Proposal 10: The associated ID may contain the time information about the data collection if no explicit time stamps come with the collected data.

In realistic, the wireless channel environment has a time varying pattern especially in urban area. Therefore, the time information also has non-negligible impacts on the consistency of data collection. If no explicit time stamps are included in the collected data, the associated ID should consider to encode the time information to provide reference of the similarity of the wireless channel environment during data collection.

Proposal 11: The associated ID could be indicated using the TCI framework FFS the details of corresponding TCI enhancement.

- AI/ML for Beam Failure Detection and Recovery

As the AI/ML BP is introduced to enhance the beam management in 3GPP R19. Alternative ways to perform BFD and BFR using the AI/ML based approach needs to be discussed. Different from the legacy approach, the BFD and BFR procedure are potentially to be integrated into a unified framework, which will significantly improve the efficiency and performance of BFR.

There are at least two benefits to study the AI/ML for BFD and BFR.

Firstly, the legacy BFD and BFR relies on measurement to the RSs configured dedicatedly, which is quite energy and resource consumption. The AI/ML BM enables UE acquire the beams based on prediction rather than measurement, therefore it may release the burden of frequent RS transmission and measurement.

Secondly, the legacy system falls to the RLF procedure if BFR fails to maintains the data link. By introducing the AI/ML based beam prediction, better beams could be predicted in case of lacking of high quality measurements, therefore AI/ML model helps to reduce the risk of RLF and improves both the latency and overall throughput.

Observation 4: Introducing he AI/ML based beam prediction into the BFD and BFR procedure is potentially reduce the overhead of reference signals and improve the efficiency of BFR, that is beneficial to the system performance.

Furthermore, the AI/ML tools can cooperate with the conventional BFD/BFR in different levels. In the remaining of this section, we take the PCell/PSCell scenario as an example to illustrate different levels of cooperation between the AI/ML model and BFD/BFR.

Level 1: AI/ML model helps to improve the quality of candidate beam

In this level of cooperation between AI/ML and BFR, the legacy BFD and BFR procedure are not affected, which is illustrated in Figure 2. However, since the candidate beams for BFR can be found based on both AI/ML model prediction and legacy measurement, some aspects in legacy BFR need to be enhanced or modified to fulfil the requirements for NW decision. For example, the PRACH/UCI may contain two kinds of candidate beams to report, one is from measurement and the other is from prediction. As a consequence, the content in the BFR request may be updated to adapt to the AI/ML based report. Furthermore, the mapping policy between SSB and PRACH frequency occasion may be modified to leave space to AI-aided BFR. This approach can flexibly fallback to legacy scheme that minimize the standard impact.

Figure 5 AI/ML is used to find the candidate beam in BFR

Proposal 12: Study to use AI/ML based beam prediction to maintain the candidate beam list for BFR.

Level 2:  Unified framework for BFD and BFR based on AI/ML model

In this level of cooperation between AI/ML and BFR, one set of RSs are transmitted for both BFD and BFR purposes, which is illustrated in Figure 3. The measurement burden of finding candidate beams can be alleviated by AI/ML based prediction. For example, previously measurement in both BFD and BFR procedure could acts as the model input to determine the beams to be updated on candidate list, thus the time window for measurement could be sparser than the legacy one. In some cases, when the quality of measured beams is not good enough, the AI/ML model could help to improve the candidate beam quality by prediction.

Figure 6 BFD and BFR share the same RS configuration.

Observation 5: In the AI-aided BFR procedure, the candidate beams can be predict based on the measurement results from both BFD and BFR procedure, which enables a common RS configuration for both BFD and BFR.

Proposal 13: Study to integrate the RS configuration and measurement configuration of BFD and BFR under a unified AI/ML based framework.

Issue 3: AI/ML based prediction for both BF event and candidate beams

Besides the beam prediction, the AI/ML model may be capable to predict the BFR event similar to the RLF and HO event prediction discussed in RAN2, which is illustrated in Figure 4. In this case, the legacy BFD can be omitted or at least simplified. Moreover, the measured beams are more effectively utilized since dual functionalities, i.e., the candidate beam prediction for BFR and the BF event prediction for BFD, are achieved by only one-shot measurement.

Figure 7 BF event is predicted by AI/ML model

Observation 6: The legacy BFD is potentially to be replaced by AI/ML based BF event prediction, or at least being simplified by only measuring a few beam samples.

In this case the beam failure is prevented in advance rather than being recovered when happened, that improves the robustness of the radio link. As the BF event is predicted with a timestamp, the AI/ML model could also suggest the best occasions to apply those predicted beams rather than simply choose the best one in the consecutive slots/symbols.

Observation 7: The prediction on both BF event and candidate beams for BFR will fully utilize the capability of AI/ML model.

Proposal 14: Study to evolve the BFD to BF event prediction while using temporal beam prediction to find the candidate beams for BFR.

- Configuration and Report Enhancement

  - TCI Framework Enhancement for AI/ML BM

In the TCI framework, the QCL relationship is defined to indicate the spatial parameter reuse between different reference signals. It is straightforward that we may extend the concept of QCL to represent the inferred relations between RS sets.  Aided by the AI/ML model, the QCL concept can be extended from guiding the reuse of parameters to providing guidance to inference of parameters. To this end, we may introduce the extended QCL Types for the configuration of AI/ML BP, as well as the corresponding signaling design.

For example, if we want to indicate the beams of a particular CSI-RS Set, we assign the qcl-InfoPeriodicCSI-RS in NZP-CSI-RS-Resource IE with a particular TCI state ID.  Then the beam reuse policy is found by indexing the activated TCI-States using the indicated TCI-StateId. Similar approaches are applied to other kind of RSs, such as the SSB and DM-RS. Hence the critical problem to enable the AI/ML BP in the current protocol with minimum standard impact relies on sufficient enhancement of the TCI-States in order to support AI/ML.

As the AI/ML BP may incur different measurement and report behaviors, the AI/ML specific configurations should be introduced in the TCI framework. In order to ensure the backward compatibility with the legacy UE, we propose additional TCI state ID pool for AI/ML purpose to distinguish them from the legacy one for non-AI/ML purpose. Therefore, the maximum number of TCI states should be increased to a number larger than 128. Since there is a separate TCI state definition for PDCCH associated with maxNrofTCI-StatesPDCCH, it may also be increased to a number larger than 64.

In consequence, the TCI state should be enhanced to support both the legacy QCL and the extended QCL. In previous release the TCI state supports two legacy QCL types, namely the qcl-Type1 and the qcl-Type2, to enable the RS be QCLed with two different RSs. In the enhanced TCI state additional QCL types, for example the qcl-Type3 and qcl-Type4 should be added for AI/ML purpose and named differently to distinguish them from the legacy ones.

In legacy TCI framework, the QCL-Type D characterize the beam reuse relationship between two RS sets and the valid time of the reused beams are configured by higher layer signaling in timeDurationForQCL IE. For the BM-Case1, the similar configuration of valid time can be used and the maximum time duration of the beam validity may be enlarged as the AI model gives more preciously estimation than the legacy one. For the BM-Case2, conventional configuration is not able to figure out the exact time instances each one of the N predicted beams take effort, therefore necessary configuration to associate multiple future time instances to multiple predicted beams is demanded.

Through defining new TCI state and corresponding QCL types, the legacy and the AI/ML beam management works under the same framework. As illustrated in Figure 6, the legacy TCI-state ID nj indicates a legacy QCL relationship between current RS/channel with respect to another reference signal, i.e., SSB#(i2), while the new TCI-state ID ni indicates the mapping between the current RS/channel corresponding to Set A, with respect to other reference signals corresponding to Set B. This approach reuses the concurrent signaling framework that adapt to both legacy and AI/ML based BM with only slight changes on configured parameters.

Figure 8 New TCI-states for AI/ML BM

Based on the above analysis, we propose the following:

Proposal 15: The following enhancement on the TCI framework should be considered to support the AI/ML BM.

- Additional TCI state ID dedicated for AI/ML BP should be introduced.

- New QCL types is indicated in TCI state to associate the RS sets corresponding to Set A and Set B beams.

- At least for BM-Case2, timing information related to different predicted beams should be configured to the UE using RRC signaling, e.g., included in the TCI state information.

  - Impact of AI/ML to Report Information

The following agreements are made during the RAN WG1 meeting #117. Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range Agreement Following Working Assumption is confirmed. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

The protocol should consider from two perspectives to reduce the report overhead. Firstly, the report quantities should be down selected based on functionalities, e.g., by model type and reporting purposes, to avoid the redundant report. Secondly, the samples within the selected report quantities should be down selected based on the beam qualities, e.g., by evaluating the beam quality based on RSRP threshold, to avoid the low-quality report which may mislead the inference result.

For NW-sided model, if the NW use a classification-based model, the input should be the L1-RSRPs of Set B beams and the corresponding beam information, where the beam information is mandatory for training while optional for inference. If the NW use a regression-based model, the input should be the L1-RSRPs of Set B beams and the corresponding measured L1-RSRPs of Set A beams, where L1-RSRPs of Set A beams is mandatory for training.

For UE-sided model, if the UE use a classification-based model, the output should be the beam information of Set A beams and the corresponding probability, where the probability is optional for inference. If the UE use a regression-based model, the output should be the beam information of Set A beams and the corresponding predicted L1-RSRPs, where the confidential level of the L1-RSRP is optionally provided for inference.

Suppose different model types are supported by the AI/ML BP, intuitively the report quantities are already implicitly indicated by the model types as each of the model types include proprietary model input and output quantities. Moreover, for particular model type, different report quantities may be demanded for different report purposes, for example, the data collection report for model training should include the L1-RSRP and beam information of both Set A and Set B beams, while for inference only the L1-RSRP and beam information of Set B beams are demanded. Hence for particular model type, down selection from the model input can be configured to reduce the report overhead.

Basically, the report content relies on the model type. Hence firstly the NW should indicate to the UE the model type, then indicate to the UE the report contents if multiple options there. For example, whether to send the report quantities can be expressed by a bitmap, and the bitmap use “0” and “1” to represent the absence and presence of one report quantity respectively, then a K report quantities selection can be resolved by a K-bit bitmap. Another alternation is to drive a look-up table.

Proposal 16: The following enhancement on the beam management report should be considered.

- Alt. 1 Indicating the model type and/or a bitmap to indicate the selected report quantities.

- Alt. 2 Indicating the model type and/or the report type to indicate the selected report quantities.

Since a single UCI may not have enough space for reporting multiple quantities for BM Case-2, the PUSCH may help transmit the AI/ML BP report. For example, the report message can be divided into two parts, the first part is carried by the UCI in the PUCCH and the second part is carried by the data payload in the PUSCH.

Proposal 17: RAN1 should consider the following enhancement on the report of AI/ML beam management.

- Study the two-stage report mechanism using both PUCCH and PUSCH.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

For BM-Case2, the reporting of beam prediction for multiple future time instances imposes a significant overhead. Therefore, grouping or segmentation-based approach may be applied to reduces the overhead.

For example, we may split Set A beams into multiple subsets, indicate whether each subset contains predicted beams or not by 1bit respectively. If there are predicted beams then the bitmap information corresponding to that beam set subset is reported, if not then the bitmap information corresponding to that beam set subset does not need to be reported.

Based on the assumption that the optimal beams are consecutive in Set A, we may set the strongest beam in Set A as a reference beam, and the report includes the reference beam and the neighboring 2L beams. The report quantities include the identification and RSRP of the reference beam, the neighboring area L, as well as a 2L-bitmap indicating the appearance of predicted beams within the neighboring area.

Figure 8 Group Based Report of Predicted Beams

Proposal 18: For BM-Case2, the following overhead reduction approach can be considered.

- The report may be split into multiple groups for latency and overhead reduction, FFS the splitting rule and collision control mechanism.

- The selection of predicted beams in the report can be indicated by a reference beam plus a bitmap indicating the appearance of predicted beams within the neighbourhood.

- Conclusion

In this contribution, we make the following observations.

Observation 1: The Top-K beam sweeping introduces extra measurement overhead especially for BM-Case2.

Observation 2: It is straightforward to consider merging of the measurement of Top-K beam sweeping and model monitoring for overhead/complexity reduction purposes.

Observation 3: The beam shape information is more stationary comparing to the beam index information during data collection.

Observation 4: Introducing he AI/ML based beam prediction into the BFD and BFR procedure is potentially reduce the overhead of reference signals and improve the efficiency of BFR, that is beneficial to the system performance.

Observation 5: In the AI-aided BFR procedure, the candidate beams can be predict based on the measurement results from both BFD and BFR procedure, which enables a common RS configuration for both BFD and BFR.

Observation 6: The legacy BFD is potentially to be replaced by AI/ML based BF event prediction, or at least being simplified by only measuring a few beam samples.

Observation 7: The prediction on both BF event and candidate beams for BFR will fully utilize the capability of AI/ML model.

In this contribution, we have the following proposals.

Proposal 1: Conventional P1/P2/P3 approach to beam pairing/refinement can be simplified by AI/ML beam prediction to two phases.

- FFS: If the AI/ML model in P1 is able to predict the DL-Tx-Rx beam pairs, only one phase is required for beam pairing as the Rx beam selection is also acquired in P1.

Proposal 2: Measurement patterns for AI/ML beam management should be designed for spatial domain beam prediction, and should consider both the fixed regular and random patterns.

Proposal 3: The measurement results collected during Top-K beam sweeping could be used in the performance monitoring.

- Option 1: The results obtained from Top-K beam sweeping could be used to trigger the performance monitoring.

- Option 2: The results obtained from Top-K beam sweeping could be used to calculate the metrics of the performance monitoring.

Proposal 4: Both dedicated AI/ML PU and legacy CPU are occupied for AI-related CSI report for inference.

- FFS if the CPU occupied by the AI-related CSI report and the legacy CSI report share the same amount of CPU.

- FFS if data collection for usage of AI/ML model is counted by AI/ML PU or legacy CPU.

Proposal 5: Study the selection rule if the AI-related CSI report to be reported is jointly determined by the legacy CPU and AI/ML PU.

- Option 1: Selection of CSI report prioritizes the AI/ML PU constraint.

- Option 2: Selection of CSI report prioritizes the legacy CPU constraint.

- Option 3: Selection of CSI report jointly considers the legacy CPU and AI/ML PU constraint.

Proposal 6: The occupation time of CPU and A/ML PU should be considered separately. The duration of the PU occupation time could be configured by the NW.

Proposal 7: For BM-Case2,  we need to define he valid duration of the prediction result to determine if a prediction result is valid or not.

- The invalid prediction could be omitted in the CSI report for overhead reduction purpose.

Proposal 8: If the associated ID is globally or cross-cell defined, an explicit mapping between the associated ID and the NW additional conditions should be defined.

Proposal 9: The associated ID in AI/ML beam management at least ensures the beam consistency.

FFS: the details of the beam consistency.

- Option 1: The beam consistency means same beam index or beam order in both training and inference.

- Option 2: The beam consistency means same beam shape or beam codebook in both training and inference.

Proposal 10: The associated ID may contain the time information about the data collection if no explicit time stamps come with the collected data.

Proposal 11: The associated ID could be indicated using the TCI framework FFS the details of corresponding TCI enhancement.

Proposal 12: Study to use AI/ML based beam prediction to maintain the candidate beam list for BFR.

Proposal 13: Study to integrate the RS configuration and measurement configuration of BFD and BFR under a unified AI/ML based framework.

Proposal 14: Study to evolve the BFD to BF event prediction while using temporal beam prediction to find the candidate beams for BFR.

Proposal 15: The following enhancement on the TCI framework should be considered to support the AI/ML BM.

- Additional TCI state ID dedicated for AI/ML BP should be introduced.

- New QCL types is indicated in TCI state to associate the RS sets corresponding to Set A and Set B beams.

- At least for BM-Case2, timing information related to different predicted beams should be configured to the UE using RRC signaling, e.g., included in the TCI state information.

Proposal 16: The following enhancement on the beam management report should be considered.

- Alt. 1 Indicating the model type and/or a bitmap to indicate the selected report quantities.

- Alt. 2 Indicating the model type and/or the report type to indicate the selected report quantities.

Proposal 17: RAN1 should consider the following enhancement on the report of AI/ML beam management.

- For overhead reduction purpose, study the quantization of report quantities, starting from the enhancement on the RSRP quantization.

- Study the two-stage report mechanism using both PUCCH and PUSCH.

Proposal 18: For BM-Case2, the following overhead reduction approach can be considered.

- The report may be split into multiple groups for latency and overhead reduction, FFS the splitting rule and collision control mechanism.

- The selection of predicted beams in the report can be indicated by a reference beam plus a bitmap indicating the appearance of predicted beams within the neighbourhood.

- Reference

- RP-243244 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- R1-2407892 Discussion on specification support for beam management, RAN1 118bis Meeting, CMCC.