3GPP TSG RAN WG1#121	   R1-2503702

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.11

Source:	WI rapporteur (Thales)

Title:	RAN1 agreements for NR NTN Phase 3 up to RAN1#120-bis

Document for:	Information

# Content

Content	1

Introduction	2

1	WID Objective	2

2	3GPP TSG RAN WG1 Meeting #116	4

2.1	NR-NTN downlink coverage enhancement	4

2.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	8

2.3	NR-NTN uplink capacity/throughput enhancement	9

3	3GPP TSG RAN WG1 Meeting #116bis	10

3.1	NR-NTN downlink coverage enhancement	10

3.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	13

3.3	NR-NTN uplink capacity/throughput enhancement	13

4	3GPP TSG RAN WG1 Meeting #117	14

4.1	NR-NTN downlink coverage enhancement	14

4.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	17

4.3	NR-NTN uplink capacity/throughput enhancement	17

5	3GPP TSG RAN WG1 Meeting #118	17

5.1	NR-NTN downlink coverage enhancement	17

5.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	18

5.3	NR-NTN uplink capacity/throughput enhancement	18

6	3GPP TSG RAN WG1 Meeting #118bis	19

6.1	NR-NTN downlink coverage enhancement	19

6.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	19

6.3	NR-NTN uplink capacity/throughput enhancement	20

7	3GPP TSG RAN WG1 Meeting #119	20

7.1	NR-NTN downlink coverage enhancement	20

7.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	21

7.3	NR-NTN uplink capacity/throughput enhancement	21

8	3GPP TSG RAN WG1 Meeting #120	22

8.1	NR-NTN downlink coverage enhancement	22

8.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	24

8.3	NR-NTN uplink capacity/throughput enhancement	24

9	3GPP TSG RAN WG1 Meeting #120-bis	25

9.1	NR-NTN downlink coverage enhancement	25

9.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	26

9.3	NR-NTN uplink capacity/throughput enhancement	27

Reference	29

# Introduction

This document summaries RAN1 agreements made for the Rel-19 WI on NR NTN enhancements phase 3.

The Agreements, Working Assumptions and Conclusions achieved in RAN1 meetings up to RAN1#120 are listed in sections 3 to 10.

The WI objective from the WID [1] is inserted below for convenience.

# WID Objective

The work item aims at specifying further enhancements for NG-RAN based NTN (Non-Terrestrial Networks) with the following assumptions: - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS (Global Navigation Satellite Systems) capabilities - In frequency band above 10 GHz, both Terminal Type 1 (Electronic steering antenna) and Type 2 (Mechanical steering antenna) to be considered for GSO and NGSO - Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant Note 1: In Rel-19 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion). The objectives of the work item are the following: - Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it. - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 3GPP TSG RAN WG1 Meeting #116

RAN1#116 made the following agreements:

## NR-NTN downlink coverage enhancement

RAN1#116 made the following agreements:

Agreement

For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1:

| LEO600km Set1-1 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size(Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set1-2 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |
| --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Agreement

Adopt the following phased array antenna parameters for LEO 600km in FR1:

| Satellite phased array antenna Characteristics | LEO-600 |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | [65] for H [65] for V |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | [400 elements (20 x 20)] |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | [4dB] |

Agreement

RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level:

At least:

- CDF of the received SINR

- The dwell time and revisit time interval for each beam illumination across the coverage

- Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio

Other metrics may be reported such as

- CDF of the cell throughput

- CDF of user perceived throughput (UPT)

- CDF of Latency

- Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889)

For system level study based on analytical evaluation:

- N1 beam footprints are in state “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints are also served the necessary information for cell discovery and initial access

- N1 + N2 + N3 = “Total number of beam footprints “

- N1, N2, N3, X are to be reported by companies.

- Resource utilization obtained under the assumptions above is to be reported by companies.

- Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Agreement

For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following:

- -5.5 dBi antenna gain is assumed

- at least 2Rx are considered at the UE

  - 4Rx can be optionally considered and reported

Note: Redcap device is not considered in the scope of DL coverage study

Agreement

The following traffic models are considered for system level evaluation of DL coverage:

- FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time

- FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time

- VoIP can be considered in the evaluation.

It is up to company report which traffic model is used among the discussed traffic models in their evaluations.

- Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in Rel-18 NTN CE. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

Agreement

For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused.

- Using other beam layouts is not precluded, and should be reported by companies

Agreement

For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed.

Values different from 0 can be optionally reported

Agreement

DL coverage is evaluated at link level with the following considerations:

- NGSO at LEO-600 operating in FR1 is considered in priority

- Additional satellite payload parameters defined for system level evaluation are used

- FFS: Antenna gain reduction due to steering loss can be considered

Agreement

For the evaluation of NTN downlink coverage at link level, reuse the target data rate from Rel-18 NTN Coverage enhancements:

- For VoIP: AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval

- For data rate service: both 3 kbps and 1Mbps can be considered

  - Companies can also use the data rates corresponding to the traffic types used for system level evaluations

Agreement

For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps.

Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821

Step 2: Required SNR of target service is evaluated by LLS

Step 3: The CNR and the required SNR are compared

Agreement

For link-level study, for NR NTN DL coverage enhancement, the following channels/signals can be considered for evaluations:

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB, e.g., SIB1, SIB 19

- PDSCH for paging

- PDCCH

- Broadcast PDCCH (e.g. PDCCH of Msg.2, paging)

- SSB

Note: RAN1 will aim to identify necessary link-level enhancements for these channels in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential link-level enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread:

  - Channel model as in Table 6.1.2-4 of TR38.821, NTN-TDL-C (LOS)

- Evaluation scenario:

  - Rural (LOS)

- Channel estimation: Realistic estimation:

  - Companies are encouraged to report channel estimation method.

- SCS:

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: TBD

- Frequency offset: 0.1 ppm

Agreement

For link budget calculation, parameters in the following table are assumed:

| Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz for DL (S-band) |
| Satellite altitude | 600 km |
| Target elevation angle | 30° (LEO) |
| Atmospheric loss | Equation (6.6-8) in [38.811] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| UE antenna gain | -5.5dBi |
| Free space path loss | Equation (6.6-2) in [38.811] |
| Polarization loss | 3dB |
| Outcome | CNR |

## Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Work in RAN1 is limited to checking whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024.

Agreement

Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN:

- Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report:

  - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission

  - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission

  - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission

  - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

  - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission

  - Case 6: Dynamic or semi-static DL collides with valid RO

  - Case 7: Collision due to direction switching

- At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs

  - Error cases in case 3 and case 4

  - SIB19 reception collides with UL transmission

  - Slot counting for UL repetition transmission colliding with SSB reception

  - Invalid symbol determination for PUSCH repetition type B

  - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling

  - CPU occupation due to omitted DL reception or UL transmission

Note: Both GSO and Non-GSO should be considered.

## NR-NTN uplink capacity/throughput enhancement

Agreement

Adopt the table below for assumptions for Evaluation parameters for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| Channel model | - NTN-TDL-C Rural, 30° elevation angle |
| Carrier frequency | - 2 GHz |
| Subcarrier spacing | - 15 kHz |
| UE speed | - 3 km/h |
| Frequency hopping | - No frequency hopping |
| PUSCH mapping type A with | - 14 OS- for OCC across slots including DMRS |
| HARQ configuration | - No HARQ |
| Channel coding | - LDPC |
| TBS | Reported by companies, e.g. - ≈184 bits payload @AMR 4.75kbps96 bits @Low data rate |
| DMRS configuration / port / bundling | 1 port per UE Reported by companies - DMRS positions for single-symbol DMRS and optional double-symbol DMRS for PUSCH mapping type A defined in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 respectively with ld=14, l0=2 and pos1 in [38.211]. - up to 8 DMRS Ports Optional DMRS Bundling |
| PRBs/MCS | Reported by companies, e.g. - 1 PRB, 2 PRBs - MCS in Table 6.1.4.1-2 in [TS 38.214] |
| Max repetition number | - Reported by companies – up to 20 for VoIP, up to 32 for low data rates |
| OCC length | Reported by companies, e.g. - Up to 8 |
| OCC sequence | Reported by companies, e.g. - Walsh sequences in Table 6.3.2.6.3-1 in TS38.211 - DFT sequence in Table 6.3.2.6.3-2 in TS38.211 |
| Antenna configuration at Satellite | - 1Rx |
| Antenna configuration at UE | - 1Tx |

Agreement

Adopt the table below for assumptions for modelling impairments for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| TO | Reported by companies - With TO: Uniform selection from [-0.94us, 0.94us], where 0.94us=29Ts - Optional without TO |
| FO | Reported by companies - Uniform selection from [-0.1 ppm, +0.1 ppm], Variation of frequency error is negligible. - Optional: with lower maximum residual FO, to be reported by companies |
| Timing drift | Optional |
| Receiver algorithm | To be reported by companies, e.g. - MMSE |
| Channel estimation | - Real channel estimation |

Agreement

Adopt the table below for assumptions for KPIs for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| Number of code-division multiplexed users | Reported by companies (up to 8) |
| KPI – SNR for a target BLER per UE | As in Rel-18 (otherwise reported by companies) - VoIP: SNR @2% BLER - For other cases: SNR @10% BLER |
| KPI - Aggregated throughput | Reported by companies Total throughput according to number of code-division multiplexed users (up to 8) Note: companies should also report the throughput for the case without OCC |

# 3GPP TSG RAN WG1 Meeting #116bis

## NR-NTN downlink coverage enhancement

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 8 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 24 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. 24 PRBs for SIB1 and SIB19 |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.2 | 72 bits |
| Payload size for PDSCH of SIB1 | FFS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Payload size for PDSCH of SIB19 | FFS |
| Other parameters | Reported by companies. |

Agreement

Antenna gain reduction due to steering loss is not considered in the link level evaluation.

Note: This is aligned with the assumptions made in Rel-18 UL coverage enhancement

Observation

The CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively.

Agreement

Confirm the Satellite phased-array antenna parameters for LEO 600km in FR1 defined in RAN1#116.

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65 for H 65 for V |
| Antenna element spacing | 0.667 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 400 elements (20 x 20) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | 4 dB |

Al least the above model is considered for SLS to ease the alignment between evaluation results. The model below can be optionally considered:

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | TR38.820 section 7.2.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| Antenna element spacing | 0.5 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 676 elements (26 x 26) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi (Note 1) |
| Steering loss at 30° elevation angle | 2.5 dB |

Note 1: The maximum antenna gain is determined by considering an overall array efficiency [of 50%.]

Agreement

For coverage evaluation of PDSCH in NR NTN, the following payload sizes for PDSCH are assumed:

| Payload | value |
| --- | --- |
| Payload size for PDSCH of SIB1 | Option 1: 800 bits Option 2: 1280 bits |
| Payload size for PDSCH of SIB19 | 616 bits |

Note: At least the above values are simulated and reported. Other values can be considered.

Note: the values above are not the TBS.

Agreement

For DL coverage study at system level, it is up to companies to report the following parameters for LEO600km Set1-1 FR2:

Beam size

Satellite EIRP density /beam (dBW/MHz)

Payload Total DL power level (dBW)

Aggregated EIRP (Total) (dBW)

Satellite Tx max Gain

EIRP per Satellite beam (dBW)

Agreement

For coverage performance evaluation of DL channels/signals before the SIB19 acquisition, the maximum Doppler frequency drift is assumed to be equal to 0.27 ppm/s based on TR 38.821.

## Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Work in RAN1 is limited to checking whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024.

Observation

To avoid the occurrence of error cases 3 and 4 through network scheduling, there are less resources available for a scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB.

Observation

For collision cases 1, 2, 5 and 6, when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, there might be less resources available for the scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN if gNB attempts to avoid the collision or there is a loss of DL/UL transmissions due to collision.

Observation

When there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, there may be a BLER performance degradation for the reception of UL transmissions at the gNB for the scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN if gNB does not attempt to avoid the collision at least in the following cases:

- UL transmission with repetitions due to different available slot counting at UE and gNB when colliding with SSB reception

- PUSCH repetition type B due to different invalid symbol determination at gNB and UE when colliding with DL transmissions

- UL transmission with DMRS bundling due to the different actual TDW determination at gNB and UE when colliding with DL transmissions

Note: the above cases happen at least with one of collision cases 1, 2, 5, 6, and 7.

## NR-NTN uplink capacity/throughput enhancement

- Agreement

Support OCC for PUSCH in Rel-19 NR NTN:

- At least PUSCH with Type A repetition

  - FFS PUSCH without Type A repetition for intra-symbol and/or inter-symbol cases

- At least code length 2 or 4, FFS code length 8

- FFS: number of RBs

- Potential OCC techniques listed below are for further down-selection:

  - Inter-slot time-domain OCC with PUSCH repetition Type A

  - Inter-symbol(s) time domain OCC

  - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4)

  - Combinations of OCC techniques

- TBoMS for OCC techniques is FFS

Agreement

RAN1 to at least further study the potential specification aspects on OCC techniques:

- TBS calculation / Rate matching

- UCI multiplexing

- RV cycling across repetitions

- Frequency hopping, e.g. intra /inter slot

- OCC indication/configuration

- Power control

- FFS others aspects

# 3GPP TSG RAN WG1 Meeting #117

## NR-NTN downlink coverage enhancement

Observation

Based on LLS results on PDCCH coverage evaluation collected from different sources:

- It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1.

      - The coverage margin is around 4 dB compared to CNR of -1.9 dB.

  - With parameter LEO600km Set1-3 FR1:

    - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average compared to CNR of -9.9 dB.

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDCCH.

Observation

Based on LLS results on PDSCH Msg2 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg2 is in average equal to – 10.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 9 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 12 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 1 dB on average compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg2.

Observation

Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg4:

      - The coverage margin is around 3.3 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with Msg4:

    - The coverage gap is around 4.7 dB on average compared to CNR of -9.9 dB

  - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg4.

Observation

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2FR1:

  - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is around 3.9 dB on average compared to CNR of -1.9 dB

  - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

      - The coverage margin is around 1.5 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

      - The coverage gap is around 4.1 dB on average compared to CNR of -9.9 dB

  - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is 3.4 dB compared to CNR of -9.9 dB

  - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

      - The coverage gap is around 6.5 dB on average compared to CNR of -9.9 dB

- Note: some results assumed SIB1 combination (where SIB1 is repeated within 160 ms) and some results assumed no SIB1 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1.

Observation

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is around 4.2 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

      - The coverage gap is around 3.5 dB on average compared to CNR of -9.9 dB

- Note: all the results above assumed no SIB19 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB19.

Observation

Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km:

- For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied.

- Note: coverage ratio is N2+N3/ total beam footprints

- Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint:

- 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%.

- 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively.

  - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams.

- 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%.

  - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth.

Note: RAN1 will further investigate the impact of SSB periodicity extension

- Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary

- Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...)

Observation

Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources)

- With parameter LEO600km Set1-1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 9.1 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 2.3 dB on average, compared to CNR of -9.9 dB

  - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH for VoIP.

Observation

Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 9.1 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 1.6 dB on average, compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH 3kbps.

Observation

Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps:

  - The coverage margin is around 2.2 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps:

  - The coverage gap is around 5.5 dB on average, compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH 1Mbps.

## Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Work in RAN1 is limited to checking whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024.

Conclusion

For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4.

- Note: further discussion on other cases is not precluded

Conclusion

For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.

Observation

TA reporting is beneficial to mitigate the TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap/eRedCap UE in NTN from RAN1 perspective.

- Note: complexity, power consumption and signaling overhead impact of TA reporting for (e)redcap UEs was not investigated in this work item

## NR-NTN uplink capacity/throughput enhancement

Agreement

For the normative phase, at least one of the OCC techniques will be specified:

- Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4

- Inter-symbol(s) time domain OCC with OCC length 2 or 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4

- FFS Combination of OCC techniques including multiplexing of 8 UEs

- FFS Use of OCC techniques with TBoMS

- FFS Backward compatibility with non-Rel-19 UEs

Conclusion

OCC with PUSCH can support at least multiplexing of 2 or 4 UEs and achieve up to 2 or 4 times capacity gains respectively, when repetitions are used.

Note: the actual gain may be less due to e.g. intra/inter cell interference.

# 3GPP TSG RAN WG1 Meeting #118

## NR-NTN downlink coverage enhancement

Observation

Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints:

- With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used.

- With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used.

- Note: the overhead of SIB19 was included in some of the results

- Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation

Agreement

As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider:

- Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

  - Default value[s] with extended periodicity assumed by NTN UE for initial access can be:

    - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}

- Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

- Link-level enhancements for the following channels:

- PDCCH

- PDSCH with Msg 4

- PDSCH with SIB1/SIB19.

  - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact.

- Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

## Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Agreement

The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported.

- Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling

  - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped

  - Note: it is not precluded that the same direction is prioritized for all different use cases

- Option B: network indicates the priority configuration to the UE

  - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped

  - It is not precluded to define default rules, if necessary

Agreement

To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied:

- Finer TA report granularity

- Smaller TA offset threshold

- Trigger a TA report when the collision is detected at the UE

- TA reporting with a new triggering mechanism from gNB

- TA drifting rate reporting

Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated.

Note: other solutions can also be studied

## NR-NTN uplink capacity/throughput enhancement

Agreement

At least one of the OCC techniques when PUSCH repetitions are used will be specified:

- Inter-slot time-domain OCC with OCC length 2

- Inter-slot time-domain OCC with OCC length 2 and 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

- Note: combination of techniques is not precluded

- PUSCH repetition Type B is not considered

Conclusion

Multiplexing of 8 UEs with PUSCH OCC is not discussed in RAN1 until the work for multiplexing of less than 8 UEs has been completed.

# 3GPP TSG RAN WG1 Meeting #118bis

## NR-NTN downlink coverage enhancement

Agreement

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

- The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms.

  - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

Agreement

Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3.

- The following techniques are for further study:

  - PDCCH repetition, including:

    - Option 1: Intra-slot PDCCH repetition

    - Option 2: Inter-slot PDCCH repetition

  - CORESET length (i.e. number of OFDM symbols) extension

  - DCI format optimization (e.g. size reduction, etc)

- Note: the same technique is intended to apply to all search space types targeted for link level enhancements

- For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques:

  - Configuration

  - Backward compatibility and UE behaviour of legacy UE

  - Linked Search Space

  - Blocking probability

  - DCI format size budget

  - Resource overhead

  - Impact on CORESET0

- Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

- FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS

Agreement

For PDSCH with Msg4 Link level enhancement:

- Continue studying PDSCH repetition

  - Further discuss the specification impact for at least the following:

    - Procedure and signaling

    - Repetition factor

  - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

## Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Conclusion

The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among:

- Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS]

- Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS

## NR-NTN uplink capacity/throughput enhancement

Working assumption

For the normative phase,

- Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs.

- Support OCC length 4 with one of the following OCC techniques

  - Option 1: Inter-slot with OCC length 4 to multiplex up to 4 UEs.

  - Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs.

  - Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs.

  - Note 1:

    - At least consider 8 slots, 16 slots, and 20 slots for VoIP with BLER 2% target, with 1 RB, 2 RBs when comparing Option 1, Option 2, and Option 3. Companies can additionally report on 4 slots at least for 2 RBs.

    - Option 2 assumes TBoMS, FFS Option 3 assumes TBoMS

Note 2: as part of the working assumption, it is assumed that there would be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Conclusion

For TBS calculation and rate matching for OCC with PUSCH, for inter-slot OCC in the working assumption of RAN1#118bis:

- for inter-slot OCC for OCC length 2 and for inter-slot OCC for OCC length 4 in option 1 in the working assumption of RAN1#118bis

  - No change in determination of TBS

  - No change for rate matching

Agreement

For RV cycling for OCC with PUSCH

- For inter-slot OCC for OCC length 2 and for inter-slot OCC for OCC length 4 in option 1 in the working assumption of RAN1#118bis

  - Same RV value is used in one OCC group (i.e., OCC length applied to N slots).

  - FFS: RV cycling can be additionally used across OCC groups

Agreement

For OCC sequence for OCC with PUSCH:

- For OCC length 2, re-use orthogonal sequence [1 1; 1 -1]

# 3GPP TSG RAN WG1 Meeting #119

## NR-NTN downlink coverage enhancement

Agreement

For PDSCH with Msg4 Link level enhancement:

- Support PDSCH repetition

  - FFS: signalling design including number of repetitions

  - FFS: impact on UE capability

- Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB

- Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

Observation

Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan.

Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

Agreement

For link level enhancement of PDSCH with SIB1:

- Support PDSCH repetitions within 20 ms duration

  - The number of repetitions is fixed to 2 repetitions

  - Further discuss the specification impact for at least the following:

    - Procedure and signaling (enabling repetitions, associated time resource determination, etc.)

- Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size.

- Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

- Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern.

Agreement

For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN.

- FFS: intra-slot and/or inter-slot

## Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Agreement

For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3,

- Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all the other use cases above

    - This is signaled by RRC configuration

Note: UE shall comply to the following existing procedure in 38.331:

UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Agreement

The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode.

- FFS whether to prioritize UL or prioritize DL

## NR-NTN uplink capacity/throughput enhancement

Agreement

RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows:

- Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs.

- Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences

- RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs.

- RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs.

- Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact.

- RAN1 assumes no specification impact for CFO grouping

- RAN1 does not pursue closed-loop frequency adjustment commands.

- RAN1 assumes that RAN4 does not define new UE requirements for CFO.

Observation:

Option 1 Inter-slot OCC with OCC length 4 to multiplex up to 4 UEs with 2 PRBs can meet VoIP 2% BLER within 1 dB of single UE baseline for 8 slots or larger with UE grouping with similar CFO (e.g. maximum differential CFO of 200 Hz).

Observation:

Option 2 Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs with TBoMS can meet VoIP 2% BLER target within 1 dB of single UE baseline with 2 PRBs and 4 repetitions or larger; and with 1 PRB and 8 repetitions or larger, without need for CFO grouping.

Agreement

For RV cycling for OCC with DG-PUSCH, the following are considered:

- Option 1: RV cycling is used across OCC groups

  - Note 1: RV cycling is applied when the number of repetitions is greater than the OCC length

- Option 2: Fixed RV is used across OCC groups

- Option 3: For OCC length 2, fixed RV is used across two OCC groups, RV cycling is used across groups of two OCC groups

Agreement

If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options to be considered:

- Option 1: UCI is dropped

  - FFS: whether all UCI is dropped

- Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped.

- Option 3: UCI is multiplexed on PUSCH with inter-slot OCC

  - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC

    - FFS: which OCC group

  - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group

  - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions

Note: combination of the above can be considered

FFS Details timeline of PUCCH and PUSCH

FFS: handling of PUCCH with repetition

FFS: handling of different UCI types

# 3GPP TSG RAN WG1 Meeting #120

## NR-NTN downlink coverage enhancement

Agreement

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

- The additional default value assumed by UE during initial access (apart from the existing 20ms value) is 160 ms.

Agreement

Support only inter-slot repetition for Type0 PDCCH CSS.

Agreement

For Msg4 PDSCH repetition support, RAN1 to consider:

- Option 1: UE specific repetition indication via DCI

- Option 2: Msg4 repetition is configured by SIB1

- Option 3: Msg4 PDSCH repetition is implicitly determined by SIB1 PDSCH repetition

Agreement

At least for enabling PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, RAN1 to consider the following options

- Option 1: Using the spare 1 bit in MIB

- Option 2: Using reserved bit(s) in PBCH payload

- Option 3: Using codepoint(s) in PBCH payload

- Option 4: UE blind decoding without signaling from the network during initial access

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, consider the following:

- Option 1: Support repeated PDCCH candidates in the two consecutive slots  n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213)

  - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index

  - FFS: Details including how the two PDCCH candidates are counted toward the BD limits

  - Note: with option 1, if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot.

  - FFS: whether/how option 1 can be applicable for M=1 and M= ½

- Option 2: Support repeated PDCCH candidates in the two slots  n0 and n0+X [or n0+1 and n0+1+X ] associated with the same SSB index ( n0 as defined in section 13 of TS 38.213)

  - Value of X>1, predefined or configured

    - FFS: Value of X

  - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index

  - FFS: Backward compatibility to legacy UE

- Option 3:

  - The PDCCH candidates in slots n0 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index

    - For M=1/2 and M=1, the repeated PDCCH candidates in two consecutive slots associated with different SSB indexes;

    - For M=2, the PDCCH candidates in slots n0+1 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index

- Option 4: Option 2 with cross SSB beam repetition support

  - The PDCCH candidates in slots n0 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index

Agreement

For SIB1 link level enhancement, RAN1 to consider the following options:

- Option 1: PDSCH repetition of SIB1 is transmitted within the same slot as the type0-CSS PDCCH repetition.

  - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted.

  - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI

  - The two associated PDSCHs have the same RV

- Option 2: Option 1 and an additional PDSCH with SIB1 repetition can occur after the slot of type0-PDCCH CSS repetition.

  - FFS: How to schedule the SIB1 repetition

- Option 3: The repetition of PDSCH with SIB1 is indicated by the scheduling PDCCH

  - PDSCH is repeated in two slots

Note: Backward compatibility should be maintained

## Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Agreement

When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS.

Conclusion

For Rel-19 HD-FDD (e)Redcap UE with CG-SDT procedure ongoing in RRC-inactive mode, for collision case 3,

- Handling of collision with PDCCH CSS in RRC-inactive mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20].

- UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

- ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion every DRX cycle.

- ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle.

Working assumption

For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized.

  - Network is allowed to indicate [UL or DL] overriding [DL or UL] for all cases

    - This is signaled by one UE specific RRC parameter

  - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

## NR-NTN uplink capacity/throughput enhancement

Conclusion

For OCC time synchronization / alignment of multiplexed UEs to maintain orthogonality of the codes used for OCC, OCC group alignment is handled by network scheduling without specification impact.

Conclusion

OCC with Msg3 PUSCH is not in scope of Release 19 NR NTN Ph3.

Agreement

For RV cycling for OCC with DG-PUSCH, support Option 1

- Option 1: RV cycling is used across OCC groups.

  - Note: when option 1 is applied, RV cycling is applied when the number of repetitions is greater than the OCC length

No optimization in Rel-19 for pairing UEs with OCC2 and UEs with OCC4.

# 3GPP TSG RAN WG1 Meeting #120-bis

## NR-NTN downlink coverage enhancement

Agreement

When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition.

- UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted.

- Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI

- The two associated PDSCHs have the same RV

FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination.

Agreement

For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options:

- Option 1: Using reserved bit(s) in PBCH payload.

- Option 2: Using scheduling PDCCH.

- Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1:

- Enabling/disabling using reserved bit(s) in PBCH payload

  - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS.

R1-2501726	FL Summary #2: NR-NTN downlink coverage enhancements	Moderator (THALES)

Agreement

For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots:

- The same resource allocation is assumed for all repetitions

- The supported repetition factors are: 2 and 4

  - The network configures a single value between 2 and 4 at a given time

- The RV cycling is used for each repetition

  - If N=4:

| Starting RV | RV applicable to the nth repetition/transmission |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2.

- Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index

    - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot.

Note: further discuss the potential solution for M=1 and M=1/2.

Working assumption

For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported.

RAN1 to down select between option 1 and option 2:

Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2)

- Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17)

- FFS: Blind decoding limit

Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X

- FFS: Blind decoding limit

- FFS: details configuration and signalling

Nokia expressed the concern on the above working assumption that this will take physical resources away from intra-slot scheduling for legacy PDSCH.

Agreement

For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options:

- Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0

  - FFS: indication details.

  - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB)

- Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition.

- Option 3: The enabling/disabling is indicated by SIB1 configuration

  - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB)

FFS: Whether UE reports its capability

## Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Agreement

Update the working assumption in RAN1 #120 with the following updates:

For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all cases

    - This is signaled by one UE specific RRC parameter

  - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note2: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

## NR-NTN uplink capacity/throughput enhancement

Agreement

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

- FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling

Agreement

Send LS to RAN4 on requirements for the phase continuity and power consistency:

RAN1 has agreed the following:

- RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

  - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling

RAN1 ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

Agreement

The draft LS in R1-2503070 is endorsed. Final LS is in R1-2503071.

Agreement

OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers.

- Up to RAN2 whether to signal this with one or two RRC parameters

- Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur

Agreement

For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0].

- Note: no new RRC parameter is needed for the above

- FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case

Agreement

For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update:

- “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group

Note: how to capture the above agreement is up to the spec editor.

Agreement

For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options:

- Option 1: Configured by RRC higher-layer parameter.

  - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration

- Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number.

- Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2.

  - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration

- Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values).

- Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2.

Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2.

Agreement

If PUCCH without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group with a same priority index for PUCCH/PUSCH, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates:

- If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a)

  - FFS: PUSCH repetition with A-CSI reports

- If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2)

  - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain.

- UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group.

  - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH

Note 1: If the UCI on the PUCCH is dropped according to legacy rules and [updated] timeline conditions for UCI dropping are satisfied, there is no [additional] spec impact. (Option 1)

Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications

Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified.

Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

Agreement

For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI

- FFS: with a new field or reusing an existing field

# Reference

| - RP-243300 Rel-19 WID NR NTN phase 3, Thales, CATT, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024 - RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #116, Athens, Greece, February 26th – March 1st, 2024 - RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024 - RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024 - RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118, Maastricht, NL, August 19th – 23rd, 2024 - RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118bis, Hefei, China, October 14th – 18th, 2024 - RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #119, Orlando, US, November 18th – 22nd, 2024 - RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120, Athens, Greece, February 17th – 21st, 2025 - R1-2401844 FL Summary #2: NR-NTN downlink coverage enhancements Moderator (Thales) - R1-2401847 Summary for 9.11.2 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands Moderator (CATT) - R1-2401543 - R1-2401993 FL Summary #3: NR-NTN downlink coverage enhancements, THALES, 3GPP TSG RAN WG1 #116-bis - R1-2403743 Summary #2 for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands Moderator (CATT), 3GPP TSG RAN WG1 #116-bis - R1-2403423 Feature lead summary #2 of AI 9.11.3 on NR-NTN uplink capacity and throughput Moderator (MediaTek), 3GPP TSG RAN WG1 #116-bis - R1-2404204 FL Summary #3: NR-NTN downlink coverage enhancements Moderator (THALES), 3GPP TSG RAN WG1 #117 - R1-2405517 Summary #2 for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands Moderator (CATT), 3GPP TSG RAN WG1 #117 - R1-2405648 Feature lead summary #4 of AI 9.11.3 on NR-NTN uplink capacity and throughput Moderator (MediaTek), 3GPP TSG RAN WG1 #117 - R1-2406437 FL Summary #3: NR-NTN downlink coverage enhancements THALES, 3GPP TSG RAN WG1 #118 - R1-2407307 Summary #2 for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands Moderator (CATT), 3GPP TSG RAN WG1 #118 - R1-2407208 Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput enhancements Moderator (MediaTek), 3GPP TSG RAN WG1 #118 - R1-2407770 FL Summary #4: NR-NTN downlink coverage enhancements THALES, 3GPP TSG RAN WG1 #118bis - R1-2409103 Summary #3 for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands Moderator (CATT), 3GPP TSG RAN WG1 #118bis - R1-2409020 Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput enhancements Moderator (MediaTek), 3GPP TSG RAN WG1 #118bis - R1-2409438 FL Summary #4: NR-NTN downlink coverage enhancements THALES, 3GPP TSG RAN WG1 #119 - R1-2410807 Summary #2 for 9.11.2 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands Moderator (CATT), 3GPP TSG RAN WG1 #119 - R1-2410642 Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput enhancements Moderator (MediaTek), 3GPP TSG RAN WG1 #119 - R1-2500042 FL Summary #4: NR-NTN downlink coverage enhancements THALES, 3GPP TSG RAN WG1 #120 - R1-2501467 Summary #3 for 9.11.2 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands Moderator (CATT), 3GPP TSG RAN WG1 #120 - R1-2501402 Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput enhancements Moderator (MediaTek), 3GPP TSG RAN WG1 #120 - R1-2501728 FL Summary #4: NR-NTN downlink coverage enhancements Moderator (THALES) - R1-2503051 Summary #3 for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands Moderator (CATT) - R1-2502985 Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput enhancements Moderator (MediaTek) |  |
| --- | --- |