3GPP TSG RAN WG1 #121		R1-2503757

Malta, May 19th – May 23rd, 2024

Agenda Item:	9.1.1

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Discussion on Specification Support of AI/ML for Beam Management

Document for:	Discussion

# Introduction

This document includes our discussions, observations and proposals on specification support for AI/ML-based beam management.

# Background

We present some of the agreements and working assumptions made during RAN1#120 bis [1] and RAN1#120 [2] meetings that address issues related to AI/ML based beam management (BM-Case 1, BM-Case 2) with an UE-sided model.

Agreement

For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as:

- At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling)

  - Where K is the number of predicted beam(s) in the corresponding inference report per time instance

  - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring

  - M is configured by NW in CSI report configuration for monitoring

    - M= 1, 2

  - FFS: detailed rule or signalling

Working Assumption

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

Working Assumption

For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

- wherein N (N>=1) is configured in CSI-ReportConfig

- FFS on additional rule for counting N linked pair

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Working Assumption

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- FFS on whether to configure more than one time instance

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring

- N (N>=1) is configured in the CSI-ReportConfig

- FFS on additional rule for counting N linked pair

- measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

# Discussion

Performance metric for beam monitoring for BM-Case 1

1) FL Proposal on Beam Accuracy Indicator:

Introduce a new quantity (Beam Accuracy Indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, where BAI is  Np (0 Np  N) and the corresponding beam prediction accuracy is

NpN (0     NpN  N).

- The size of CSI field associated with the BAI is  log2(N+1)

- Where Np is the number of linked transmission occasions for monitoring that meet the metric among N linked pair(s).

We support the metric, Beam Accuracy Indicator (BAI) proposed by the FL and we  propose to use such a metric to be considered for LCM of AI/ML model for beam management particularly model update, model switching and fallback.

##### 2) FL Proposal (definition of Top K predicted beam(s)):

The predicted Top K beam(s) are the best K beam(s) based on the predicted RSRP values, or the probability of each beam in Set A to be the Top-1 beam.

We support this definition of the predicted beam for the beams in set A of  model inference.

3) FL Proposal on consistency of additional conditions between training and inference (associated ID is absent and ordering of as set for collection and inference)

- If the associated ID for Set B/Set A is absent, UE may assume similar properties over the collected Set B/Set A with absent associated ID in the same cell, respectively.

- With the same associated ID, the n-th resource in the Set A/SetB in data collection is linked to the n-th resource in Set A/Set B in inference.

We support the proposal by FL to achieve consistency regarding additional conditions in case of no associated ID.

##### 4) FL Proposal on NW-sided model (proposal of M values)

For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, the value of M of reported beam in one report is configurable, and can be 4, 6, 8, 12, 16

- The max value of M is subject to UE capability.

We support the proposal and suggest the minimum value to be greater than 4 for Rel 19 and beyond.

# Conclusion

Observation 1: For Performance monitoring of AI/ML based beam management, the monitoring metric is defined for a single instance of monitoring, i.e., one monitoring resource set.

Observation 2: For UE-assisted performance monitoring of a UE-sided AI/ML model, the Top 1/K beam prediction accuracy is defined as the percentage of actual Top M beams (based on the L1-RSRP measurements) being predicted as the Top 1/K beams.

Proposal 1: At least for the monitoring Type 1 Option 2 of UE-side model monitoring, if M is configured to be 1, the value of K for top predicted beams should be less than or equal to 2. If M is configured to 1,2 the value of K for top predicted beams could be less than or equal to 4.

Proposal 2: Model management procedures  of AI/ML LCM, such as model update, model switch and fall back of AI/ML model for beam management should consider the performance of the model (Beam Prediction accuracy) for multiple monitoring instances (at least N=5).

Proposal 3: For BM-Case 1 and BM-Case 2, introduce a threshold for minimum time offset between transmission of CSI-RS/SSB resources for monitoring and the CSI reference resource signal transmission for Inference in order to link the CSI resources for monitoring and an inference report.

Proposal 4: We recommend the threshold value for minimum time offset for linking monitoring CSI-RS/SSB resources and inference report to be in the order of a few milli-seconds say 1ms, 2ms, 3ms, 5ms.

Proposal 5: For BM-Case 2, which is beam prediction for future time instances, the beam prediction accuracy should be configured for each future prediction time instance.

Proposal 6: For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), support the size of the set for monitoring to be integer multiple of  K (Top K predicted beams of the linked inference report).

Proposal 7: For NW-sided model, for inference, in a beam report initiated by network based on one measurement resource set, the number of reported beams in one report, M, is suggested to be greater than 4.

# References

[1] Chair Notes, RAN1 #120-bis Wuhan, April 2025

[2] Chair Notes, RAN1 #120 Greece, February 2025

[3] 3GPP TR 38.843 Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)