3GPP TSG RAN WG1 #121  	                              R1-2503982

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		9.1.3

Source:		LG Electronics

Title:			Discussions on CSI prediction

Document for:	Discussion and Decision

Introduction

In RAN #105, revised WID was agreed as captured below [1]:

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

This contribution discusses on issues related to functionality-based LCM and consistency between training and inference for AI/ML based CSI prediction.

Discussion

Data collection

Agreement For CSI prediction using UE-side model, for data collection for training, - For resource configuration, - For CSI-RS resource type for CMR, periodic and semi-persistent are supported. - FFS on support of aperiodic CSI-RS resource - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signaling overhead. - FFS on CSI report configuration Agreement For CSI prediction using UE-side model, for training data collection, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report - FFS on how to indicate without CSI report in CSI-ReportConfig Conclusion For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported.

In the previous meetings, above were agreed regarding on data collection for training. One remaining issue is whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI. In our view, separate configuration seems useful when two A-CSI-RSs are triggered for model input and ground-truth CSI, however, in the last meeting, it was concluded that A-CSI-RS is not supported for the training data collection purpose. Since P/SP CSI-RS resources are transmitted periodically, the measurements collected for model input are naturally aligned with ground-truth CSI. Due to scheduling delay and/or processing delay, there may be some misalignment between model input and ground-truth CSI. In that case, it could be handled by UE side implementation (e.g., interpolation). Besides, if separate configuration is supported, whether/how to link between CSI-RSs for input and CSI-RS resources for ground-truth CSI needs to be discussed.
Proposal #1: Conclude that separate configuration of CSI-RS resources for model input and ground truth CSI is not supported.

To configure CSI-RS resources without CSI report using CSI-reportConfig, there could be two alternatives as below.

- Alt 1: set report quantity as “none” for training data collection purpose

- Alt 2: introduce new reportConfigType-r19 with “none”

For alternative 1, it reuses report quantity to implicitly indicate no CSI report for CSI configuration. However, report quantity is the parameter to indicate CSI contents to be reported. So, without report quantity, UE has no idea on what to collect or log as a dataset. For alternative 2, reportConfigType is used to indicate the time domain behavior of CSI report, e.g., periodic, semi-persistent or aperiodic. Since this parameter is mandatory, new reportConfigType-r19 with “none” can be introduced to ignore the legacy reportConfigType without suffix (mandatory config). Since Alt 2 is simpler and future-proof design compared to Alt 1, Alt 2 is preferred for configuring the resource for training data collection.

Proposal #2: For configuring the resource for training data collection, CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

- Introduce new reportConfigType-r19 with “none”

CSI processing criteria and timeline

In the previous meeting, regarding processing unit, following was agreed.

Agreement For CSI prediction using UE side model, for inference, consider following options for potential down selection - Option 1: only dedicated AI/ML PU (OAPU) is occupied - Option 2: only legacy CPU (OCPU) is occupied - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied - FFS whether OAPU and OCPU are based on defined rule and/or reported by UE - Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Regarding the CSI processing criteria, three options were listed with potential down-selection. Option 1 ensures that AI/ML processing is fully isolated from legacy CPU operations, providing optimized AI/ML inference operation. Option 2 relies solely on legacy CPUs, leveraging existing computational resources without additional AI/ML-specific hardware. Option 3 adopts a hybrid approach, utilizing both APUs and CPUs, potentially balancing resource availability and flexibility.

When using only legacy CPUs (Option 2), the complex operations required for AI/ML inference, including high-dimensional tensor computations and iterative neural network processing, can be a burden for UE implementation and may result in high computational latency and potential processing bottlenecks. For shared resource usage as in Option 3, since both CPU and APU are occupied for one CSI report, CPU/APU occupancy time can be complicated. Also, in option 3, CPU may take process of measurements and/or CSI calculation, and APU may take only inference operation, so this may result in inefficiently CPU usages.

Meanwhile, using only APUs (Option 1) provides optimized computational efficiency dedicated for AI/ML tasks, resulting in better inference operation compared to Option 2. Also, independent APU usage simplifies standardization effort by clearly separating AI/ML-related processing from legacy mechanisms. Based on above discussion, Option 1 is preferred.

Proposal #3: Support Option 1, where only dedicated AI/ML PU(OAPU) is occupied for CSI prediction inference operations.

The definition of CPU occupancy time corresponding to each CSI processing configuration option (Option 1/2/3) for AI/ML-based CSI prediction is also an issue that needs to be addressed. For Options 1 and 2, since they only employ a single processing unit (either legacy CPU or dedicated APU), legacy CPU occupancy time can be a starting point of discussion.

For Option 3, since both the CPU and APU are simultaneously occupied for one CSI report, enhancements seem to be needed to ensure clear and efficient UE behavior of CSI processing. One simple way is to have the same occupancy time for both CPU and APU. However, this may lead inefficient usage of processing unit. Alternatively, different occupancy time for CPU and APU according to their role can be considered. As mentioned earlier, CPU may take process of measurements and/or CSI calculation, and APU may take only inference operation, so based on this assumption, occupancy time can be defined. For example, CPU occupancy time can be the same as legacy, but APU occupancy time can be from the latest CSI-RS occasion before reference resource to corresponding CSI report. Other alternative can be NW can control of APU occupancy time via dedicated signaling.

Another issue is CPU occupancy for performance monitoring, which involves calculation of performance metrics such as SGCS based on inference results and measurement from monitoring RS. Compared to inference report or other CSI report, this operation is relatively simple. Also, it could be related to the number of monitoring RS occasions, so that OCPU is multiple of the number of monitoring RS occasions. For CPU occupancy time, legacy occupancy time can be reused.

Proposal #4: For performance monitoring, OCPU=K where K is the number of monitoring RS occasions.

Regarding active resource/port counting for AI/ML based CSI prediction, our view is reusing legacy active resource/port counting for Rel-18 CSI (higher layer parameter codebookType set to 'typeII-Doppler-r18') at least for inference. One issue can be how to handle performance monitoring since calculation of monitoring metric requires measurement from monitoring RS and inference results. To resolve this issue, we can define active resource counting time and how many resources are counted for performance monitoring. For active resource counting time, legacy rule can be reused with some restriction that monitoring resource should be triggered before associated inference report. For issues on how many resources are counted for performance monitoring, CSI-RS ports within the CSI-RS resource (α⋅X) times where α is a scaling factor that considers buffering of inference results and X is the number of monitoring RS.

Inference related aspects

One remaining issue for inference operation is how to calculate the CQI for inference report. In the current specification (TS 38.214), the method for determining the CQI for Rel-18 CSI (Time Domain CQI (tdCQI)) has been clearly defined as captured below. The tdCQI was introduced to align CQI reporting based on Rel-18 Doppler codebook, allowing flexibility to accurately reflect temporal channel variations.

TS 38.214 5.2.1.4.2 Report quantity configurations

The tdCQI supports different reporting modes, enabling both simple and more detailed CQI reporting depending. For N₄ = 1, the UE reports one CQI associated with predicted PMI corresponding to the slot interval [l, l+d-1]. For N₄ > 1, the following tdCQI defines three reporting approaches:

- tdCQI 1-1 : The UE reports one CQI reflecting channel quality corresponding to the first precoder matrices over the interval [l, l+d-1].

- tdCQI 1-2 : The UE reports one CQI, calculated based on both the first precoder matrices (slot interval [l, l+d-1]) and the N4th precoder matrices (slot interval [l+(N4-1)∙d , l+N4∙d-1])

- tdCQI 2 : The UE reports two CQIs, the 1st CQI calculated based on the first precoder matrices (slot interval [l, l+d-1]) and the 2nd CQI calculated based on N42th precoder matrices (slot interval [l+(N42-1)∙d , l+N42∙d-1])

These tdCQI parameter sets were specifically designed to handle the wide range of channel dynamics observed in NR, from static to highly variable environments. These tdCQI reporting schemes are not only effective for traditional CSI reporting but also naturally align with the characteristics of AI/ML-based CSI prediction. AI/ML predictions inherently cover multiple time instances. As such, the existing tdCQI structure can be applied to AI/ML-based CSI reports without modification, therefore, it is reasonable to reuse Release 18 tdCQI configurations for AI/ML based CSI reporting.

Proposal #5: For CQI calculation in inference report, reuse the mechanism of Rel-18 tdCQI.

Performance monitoring

During the RAN1 #120bis meeting, for performance monitoring for CSI prediction, following three alternatives were agreed as captured below.

Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signaling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism

Based on the agreement above, three alternatives are considered and it needs to be down-selected in RAN1#121. Alt 1 is reporting a single SGCS value calculated directly from predicted CSI and corresponding ground-truth CSI. On the other hand, Alt 2 is reporting two separate SGCS values—one comparing predicted CSI with ground-truth CSI, and another comparing latest historical measurement with ground-truth CSI (SGCS of sample and hold). Alt 3 is statistical reporting (e.g., mean, x% CDF of SGCS values) of SGCS over a configured monitoring window.

Alt 1 offers clear advantages due to its straightforward mechanism and minimal signaling overhead. It simplifies UE implementation and reduces signaling overhead, providing immediate insight into AI/ML prediction performance. In contrast, Alt 2 increases computational complexity since it requires two different SGCS values. Alt 2 may have a benefit to gNB that reference SGCS can be used to judge whether prediction performance is due to good channel condition or AI/ML performance. However, this can also be done by gNB implementation by configuring another CSI reporting corresponding to latest CSI-RS occasion no later than CSI reference resource. In addition, the definition of additional SGCS needs to be discussed. Alt 3 may have benefit to reduce reporting overhead of performance monitoring, and it may reflect relatively long-term channel variation. However, it may be a burden for UE implementation as it needs to buffer the monitoring results during the monitoring window. Besides, it is required to decide the details of monitoring window. Thus, based on above discussion, our view is to support Alt 1 for report contents of UE-assisted performance monitoring.

Proposal #6: For report contents of UE-assisted performance monitoring, support Alt 1 where a single SGCS value is calculated based on predicted CSI and ground-truth CSI.

The second issue is how to quantize of SGCS values reported by the UE. Considering SGCS quantization, it is essential to effectively balance precision and reporting overhead. Typically, SGCS ranges between 0 and 1, with values close to 1 indicating excellent correlation between predicted and ground-truth CSI. However, when SGCS is substantially low, providing exact values adds unnecessary signaling overhead without meaningful benefit. Therefore, a configurable quantization scheme would effectively balance reporting accuracy and efficiency. In this scheme, the network explicitly configures a lower bound (X) and step size (Y) for SGCS reporting.

Proposal #7: For SGCS reporting, the reported value range is X to 1 with Y step size.

- FFS on X, Y values

The third issue relates to the granularity at which SGCS values are calculated and reported. Typically, CSI prediction performance can vary significantly across prediction instances, layers, and/or subbands. At least for regarding prediction instance, reporting averaged value may not be a proper way to capture performance variations across these dimensions, limiting the network's ability to perform precise assessment. Also, for performance monitoring, it would be NW’s choice that which prediction instances are monitored since NW has controllability of functionality of fallback. Whether to report per layer is one discussion point. In general, to accurately predict CSI per layer, per layer AI/ML model would be required which is a burden for UE. Otherwise, layer common model can be used. The purpose of performance monitoring is to verify whether the prediction performs well or not. In that sense, considering only rank 1 seems sufficient. For frequency granularity, it is preferred SGCS averaged over subband compared to SGCS per subband due to reporting overhead. Based on above discussion, following is proposed.

Proposal #8: Consider following SGCS calculation

- per prediction instance with NW configured prediction instance(s),

- averaged over subband,

- the first layer only.

One open issue is reporting mechanism for performance monitoring. Monitoring operation requires low latency, therefore relying on L3 mechanism is not preferred. Also, considering that we have only one meeting left, it is hard to support event-based reporting since we need to discuss on how to define event first. Besides, BM agenda already agreed to use UCI for monitoring reporting. To have unified framework, it is preferred to use UCI for monitoring reporting. In this case, similar to BM Case 2, dedicated monitoring resource set can be configured for performance monitoring, and monitoring report configuration can be configured with an id of an inference report configuration. For monitoring resource type, all types of CSI-RS can be supported, i.e., periodic CSI-RS, semi-persistent CSI-RS, and aperiodic CSI-RS. Also, for the combination of inference report and monitoring report, the same combinations agreed in BM agenda can be supported.

Proposal #9: For monitoring reporting, support to reuse CSI framework for the configuration for monitoring result report in L1 signaling.

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

Proposal #10: For monitoring resource type, periodic CSI-RS, semi-persistent CSI-RS, and aperiodic CSI-RS are supported.

Proposal #11: For monitoring resource type, following combination of inference report and monitoring report are supported.

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |
| P report | Support | Support | Support |

Consistency between training and inference

In order to ensure AI/ML performance, consistency between training and inference is an important issue. In this context, other use cases using one-sided model (e.g., BM and positioning) is currently under discussion on how to handle consistency issue. For BM use case, two candidates (i.e., 1) associated id based solution and 2) monitoring based solution) were discussed, and associated id based solution is down-selected to support where UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID. Note that performance monitoring should be supported for LCM purpose, therefore the consistency between training and inference also can be achieved via an implementation manner.

For CSI prediction, in the RAN1#118bis meeting, followings were agreed to identify whether and which potential NW-side additional condition may impact on the performance of CSI prediction. During the intensive discussion in RAN1 #119, it was not able to conclude whether any NW-side additional condition for CSI prediction are needed. In this subsection, we further discuss on this issue. If any NW-side additional condition for CSI prediction is identified, associated id can be introduced similar to BM agenda.

Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered. Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

As shown above, gNB antenna tilt angle and TxRU mapping were listed as potential NW-side additional conditions. To verify those factors, evaluation of generalization performance can be considered. Simulation assumptions are listed in Table 1.

Table 1. Simulation assumptions

| Parameters | Values |
| --- | --- |
| AI/ML model backbone | 2D-FC |
| FLOPs/M for model | 2.74 |
| Parameters/M | 1.36 |
| Model input/output type | Raw channel |
| UE distribution | 100% outdoor |
| UE speed | 30km/h |
| Dataset size of training | 9k |
| Dataset size of test | 1k |
| Whether/how channel estimation error is modelled | Realistic channel estimation error |
| Whether/how to adopt spatial consistency | No |

Impact on various tilt angles

For evaluation with respect to various tilt angles, tilt angles of 102 and 110 degrees are considered.

Table 2. Generalization performance over various gNB tilt angles

| Generalization case 1 | Train/k | 102 degrees / 9 | 110 degrees / 9 |
| --- | --- | --- | --- |
|  | Test/k | 102 degrees /1 | 110 degrees / 1 |
|  | SGCS gain (%) | 100% | 100%, |
| Generalization case 2 | Train/k | 102 degrees / 9 | 110 degrees / 9 |
|  | Test/k | 110 degrees / 1 | 102 degrees /1 |
|  | SGCS gain (%) | 98.6% | 97.4% |

As shown above Table 2, generalization Case 2 provides marginal loss (i.e., 1.4 %, 2.6 % loss) compared to generalization Case 1. Thus, it can be concluded that specification support is not needed to ensure consistency between training and inference with respect to the gNB antenna tilt angle for CSI prediction using UE-sided model.

Observation #1: Regarding gNB antenna tilt angles, for generalization Case 2, generalization performance is achieved.

Proposal #12: For CSI prediction using UE-sided model, conclude that specification support is not needed to ensure consistency between training and inference with respect to gNB antenna tilt angles.

Impact on TXRU mappings

For evaluation with respect to various TXRU mappings, [N1, N2, P] = [2, 8, 2] and [4,4,2] with (8x8x2) antenna elements are considered.

Table 3. Generalization performance over various TXRU mappings

| Generalization case 1 | Train/k | [2, 8, 2] / 9 |
| --- | --- | --- |
|  | Test/k | [2, 8, 2] /1 |
|  | SGCS gain (%) | 100% |
| Generalization case 2 | Train/k | [2, 8, 2] / 9 |
|  | Test/k | [4, 4, 2] /1 |
|  | SGCS gain (%) | 95.8% |

As shown above Table 3, generalization Case 2 provides marginal loss (i.e., 4.2% loss) compared to generalization Case 1.

Observation #2: Regarding TXRU mapping, for generalization Case 2, marginal loss is observed.

Conclusion

In this contribution, it is discussed on issues related to functionality-based LCM and consistency between training and inference for AI/ML based CSI prediction. Based on the discussion, we propose followings.

Observation #1: Regarding gNB antenna tilt angles, for generalization Case 2, generalization performance is achieved.

Observation #2: Regarding TXRU mapping, for generalization Case 2, marginal loss is observed.

Proposal #1: Conclude that separate configuration of CSI-RS resources for model input and ground truth CSI is not supported.

Proposal #2: For configuring the resource for training data collection, CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

- Introduce new reportConfigType-r19 with “none”

Proposal #3: Support Option 1, where only dedicated AI/ML PU(OAPU) is occupied for CSI prediction inference operations.

Proposal #4: For performance monitoring, OCPU=K where K is the number of monitoring RS occasions.

Proposal #5: For CQI calculation in inference report, reuse the mechanism of Rel-18 tdCQI.

Proposal #6: For report contents of UE-assisted performance monitoring, support Alt 1 where a single SGCS value is calculated based on predicted CSI and ground-truth CSI.

Proposal #7: For SGCS reporting, the reported value range is X to 1 with Y step size.

- FFS on X, Y values

Proposal #8: Consider following SGCS calculation

- per prediction instance with NW configured prediction instance(s),

- averaged over subband,

- the first layer only.

Proposal #9: For monitoring reporting, support to reuse CSI framework for the configuration for monitoring result report in L1 signaling.

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

Proposal #10: For monitoring resource type, periodic CSI-RS, semi-persistent CSI-RS, and aperiodic CSI-RS are supported.

Proposal #11: For monitoring resource type, following combination of inference report and monitoring report are supported.

Proposal #12: For CSI prediction using UE-sided model, conclude that specification support is not needed to ensure consistency between training and inference with respect to gNB antenna tilt angles.

- Reference

RP-242399, “Revised WID on AI/ML for NR Air Interface”, Qualcomm