3GPP TSG RAN WG1 Meeting #121	R1-2504001

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	Fujitsu

Title:	Discussion on downlink coverage enhancements

Agenda Item:	9.11.1

Document for:	Discussion

Introduction

In the WID for Rel-19 NR NTN [1], the detailed objectives for downlink coverage enhancements for FR1-NTN are described as follows:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

In this contribution, we provide our views on the details of DL link level coverage enhancements.

Link level enhancements

In RAN1#119 [2], the following agreements were made.

Agreement in RAN1#119 For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot

Agreement in RAN1#119 For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern.

In the above agreements, PDCCH repetition and SIB1 PDSCH repetition for Rel-19 NTN were agreed to enhance corresponding channels. In the following sub-sections, we discuss type-0 PDCCH repetition, PDCCH repetition (except type-0 and type-3) and SIB1 PDSCH repetition from the perspectives of specification impact, search space (SS) configuration and UE implementation.

Type-0 PDCCH repetition

In RAN1#120-bis [3], following agreement was made for type-0 PDCCH repetition.

Agreement in RAN1#120-bis For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2.

The potential solution for M=1 and M=1/2 is still FFS. In RAN1#120-bis, RAN1 agreed that type-0 PDCCH repetition can be performed in the case of M=2 without degradation of coverage ratio. For the M=1 and M=1/2 cases, the number of simultaneous satellite beams may be limited, and some companies have proposed to apply cross-SSB beam operation or time-separated type-0 PDCCH repetition to avoid the degradation of coverage ratio due to this limitation. However, cross-SSB beam operation is not preferable because a UE should combine two PDCCH monitored in the SS associated with different SSB index. On the other hand, time-separated type-0 PDCCH repetition takes additional time resources to transmit whole of PDCCH repetition compared to using consecutive slots. Considering these point, it is fine to discuss for the case of M=1 and 1/2 other than cross-SSB beam operation and time-separated type-0 PDCCH repetition because the SS configuration would become more flexible by defining type-0 PDCCH repetition for the M=1 and M=1/2 cases.

Observation 1: For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, the potential solution for the M=1 and M=1/2 cases other than cross-SSB beam and time-separated type-0 PDCCH repetition can be further discussed.

Fig 1. An example of type-0 PDCCH repetition with repetition factor of 2

In the Fig 1, we provide an example of type-0 PDCCH repetition with repetition factor of 2 for clarification. For PDCCH repetition mechanism, BD behaviour should be defined in the specification and the BD limits to maintain UE power consumption. The issue here is how to count the number of BDs of combined PDCCH candidates (i.e., which slot is used to count the number of BD of combined PDCCH candidates). In our opinion, the BD of combined PDCCH candidates is counted in the slot n0+1 because the UE does not need to perform legacy BD operation in the slot n0+1. Therefore, there is no need to change the BD limits.

Proposal 1: For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, the UE only decode combined PDCCH in the slot n0+1 and the number of BD to decode combined PDCCH is counted within the BD limits in the slot n0+1.

PDCCH repetition (except type-0 and type-3)

In RAN1#120-bis [3], the following working assumption was made for PDCCH repetition (except for type-0 and type-3).

Working assumption in RAN1#120-bis For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported. RAN1 to down select between option 1 and option 2: Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2) - Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17) - FFS: Blind decoding limit Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X - FFS: Blind decoding limit - FFS: details configuration and signalling Nokia expressed the concern on the above working assumption that this will take physical resources away from intra-slot scheduling for legacy PDSCH.

For the Nokia’s concern, in our opinion, when physical resources for legacy PDSCH is reduced by intra-slot PDCCH repetition, the gNB can increase frequence resources to achieve the same performance as without intra-slot PDCCH repetition. In addition, there is a concern on linking mechanism of two SS for inter-slot PDCCH repetition (i.e., UE cannot recognize which PDCCH candidates are linked). Therefore, we support to confirm the working assumption.

Proposal 2: For PDCCH repetition (except type-0 and type-3), confirm the working assumption.

For the down-selection in the above working assumption, Option 2 is a better way to transmit repeated PDCCH. In the Rel-17 specification, SearchSpaceLinkingId is used for SS linking. This parameter can associate two SS sets and the maximum number of linking SS is 3. However, this parameter allows to transmit both PDCCH in the same resource because mTRP is assumed for Rel-17 PDCCH repetition. For Rel-19 NTN, as PDCCH repetition assumes sTRP case, overlapping is not permissible. Hence, a non-overlapping mechanism is needed and can be easily achieved by Option 2. Moreover, only Option 2 enables combined PDCCH monitoring from the beginning of BD behaviour without monitoring the PDCCH with each CORESET (i.e., Option 2 can ignore BD limits by defining BD behaviour within one slot). This is because if SearchSpaceLinkingId is reused with Option 1, the UE would monitor the PDCCH with each CORESET based on the current specification (see TS 38.213 clause 10 [4]).

If RAN1 goes with Option 2, an RRC signalling of symbol offset X is needed for the configuration flexibility of repeated CORESET in the time domain, where repeated CORESET including 2nd PDCCH is located X symbol away from the last symbol of reference CORESET.

Proposal 3: For Rel-19 PDCCH repetition (except type-0 and type-3), support Option 2 to configure non-overlapped repeated CORESET and to define BD behaviour towards BD limits.

SIB1 PDSCH repetition

In RAN1#120-bis [3], following agreements were made for SIB1 PDSCH repetition.

Agreement in RAN1#120-bis For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

Agreement in RAN1#120-bis When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination.

Regarding the FFS in the agreement, the concerning case should be supported because PDSCH has different required SINR depends on the SIB1 payload size even if the PDCCH is the same.

Proposal 4: For SIB1 PDSCH repetition, the following case should be supported: type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed.

Regarding how to handle the slot determination, we think SIB1 PDSCH in the slot n0 and n0+1 should be combined because these two SIB1 PDSCH are transmitted from gNB regardless of whether type-0 PDCCH repetition is performed. The concern here is how to handle the out-of-order processing for PDSCH when PDCCH is decoded in the slot n0+1 and SIB1 PDSCH repetition is performed. In this case, a simple solution is that UE assumes SIB1 PDSCH repetition in slot n0 and slot n0+1 only when PDCCH is detected in the slot n0. While there is an issue of out-of-order processing, we think the SIB1 PDSCH repetition is generally preferable to perform within slot n0 and n0+1 from resource utilization perspective.

Observation 2: For SIB1 PDSCH repetition, whether the out-of-order processing for PDSCH can be implemented or may not need to be clarified.

For the down-selection of enabling/disabling SIB1 PDSCH repetition, Option 1 or 2 can be adopted to cope with the flexibility to indicate the repetition factor for SIB1 PDSCH repetition. Option 3 should be excluded because fixed repetition factor cannot address the case where SIB1 PDSCH repetition is needed while type-0 PDCCH repetition is not needed (proposal 5). Then, comparing Option 1 and Option 2, Option 1 has 1 available reserved bit (the other 1 reserved bit is used for type-0 PDCCH repetition) whereas Option 2 has 15 available bits. To indicate the repetition factor of {1, 2, 4} for SIB1 PDSCH repetition, 1 bit is not enough. On the other hand, to indicate the repetition factor of {1, 2}, 1 bit is enough. In summary, Option 2 should be supported if repetition factor of 4 is needed for SIB1 PDSCH repetition. Otherwise, Option 1 should be supported to align with that for type-0 PDCCH repetition.

Proposal 5: For enabling/disabling SIB1 PDSCH repetition, Option 2 should be supported if repetition factor of 4 is needed for SIB1 PDSCH repetition. Otherwise, Option 1 should be supported to align with that for type-0 PDCCH repetition.

Conclusion

In this contribution, we provided our views as below.

Observation 1: For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, the potential solution for the M=1 and M=1/2 cases other than cross-SSB beam and time-separated type-0 PDCCH repetition can be further discussed.

Observation 2: For SIB1 PDSCH repetition, whether the out-of-order processing for PDSCH can be implemented or may not need to be clarified.

Proposal 1: For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, the UE only decode combined PDCCH in the slot n0+1 and the number of BD to decode combined PDCCH is counted within the BD limits in the slot n0+1.

Proposal 2: For PDCCH repetition (except type-0 and type-3), confirm the working assumption.

Proposal 3: For Rel-19 PDCCH repetition (except type-0 and type-3), support Option 2 to configure non-overlapped repeated CORESET and to define BD behaviour towards BD limits.

Proposal 4: For SIB1 PDSCH repetition, the following case should be supported: type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed.

Proposal 5: For enabling/disabling SIB1 PDSCH repetition, Option 2 should be supported if repetition factor of 4 is needed for SIB1 PDSCH repetition. Otherwise, Option 1 should be supported to align with that for type-0 PDCCH repetition.

References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- Chair’s notes RAN1#119.

- Chair’s notes RAN1#120-bis.

- TS 38.213, Physical layer procedures for control, V18.5.0, Jan, 2025.