3GPP TSG RAN WG1 #121 	R1- 2504036

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		5

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on LS from RAN2 on CB-Msg3-EDT

Document for:		Discussion and Decision

# Introduction

In RAN2 #129bis meeting, RAN2 send one LS [1] to RAN1 on CB-Msg3-EDT of Rel19. RAN2 agreements and 8 questions are included in the LS for RAN1 to confirm or discuss.

We will provide our view on the questions and related CB-Msg3 matters in this contribution.

# Discussion

## Question 1

Q1: For both eMTC and NB-IoT, RAN2 assumes power ramping should be supported for CB-msg3-EDT. RAN2 would like to confirm with RAN1 on whether to have power ramping on this Msg3 (re-)transmission. If RAN1 confirms, please provide the control parameters for power ramping.

We consider power ramping a useful feature for handling CB-Msg3 transmission issues.

In our view, the power ramping shall only be applied between CB-Msg3 windows and not per CB-Msg3 replica within a window. In other words, the UE shall only apply power ramping after the Msg4 monitoring has resolved in unsuccessful contention and a CB-Msg3 reattempt in a new CB-Msg3 window starts.

Proposal 1: The UE shall only apply power ramping between CB-Msg3 windows in case of unsuccessful contention resolution or, that is, CB-Msg3 reattempt.

It is also worth noting that change of CE level within the CB-Msg3 window is difficult because 1) the half-duplex UE has limited downlink measurement opportunities within the window and 2) it will complicate the Msg4 monitoring because the PDCCH configuration is per CE level. Therefore, if the CE level needs to be changed it is preferable to start using the CB-Msg3 transmission window of the appropriate CE level. This may be applicable for CB-Msg3 reattempt, in addition to or instead of power ramping, for example.

Regarding the control parameters for the power ramping we suggest to use the legacy specification as a baseline. For Msg3 of 4-step RA the target power is defined as:

MSG3_RECEIVED_TARGET_POWER =preambleInitialReceivedTargetPower + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

Since there is no preamble in CB-Msg3, the PREAMBLE_TRANSMISSION_COUNTER_CE would need to be updated to CB_MSG3_ATTEMPTS_COUNTER_CE. As noted above, the UE shall only update the counter between CB-Msg3 windows and not per replica transmission.

We think RAN1 can discuss whether the preambleInitialReceivedTargetPower needs to be updated to CBmsg3InitialReceivedTargetPower or can be reused. In our view, a different powerRampingStep is not needed.

Proposal 2: For CB-Msg3 power ramping a new counter is needed for the number of CB-Msg3 attempts per CE level, while the powerRampingStep can be reused. RAN1 may discuss whether a new Initial Received Target Power parameter is needed.

## Question 2

Q2: For eMTC, RAN2 agrees to have MPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR MPDCCH configuration (in IE PUR-MPDCCH-Config-r16, as below) as baseline. RAN2 think TDD parameter is not needed and maybe narrowband configuration should be defined as a set, not one single narrow band. RAN2 would like to check with RAN1 on how to define the detail MPDCCH configuration.

It is OK to use the PUR MPDCCH configuration as a baseline, but we have the following proposals to further adjust the configuration:

- The MPDCCH configuration shall be for the common search space and not the UE specific search space as is the case for the PUR MPDCCH configuration. The reason is that the RRC Idle UEs may not have the USS configured. In our view, the CSS Type 2 is suitable, because it is also used for RAR and Msg4 monitoring in the legacy operation.

- The MPDCCH configuration shall be per CE level. This enables UEs in various coverage conditions to reliably receive the DCI.

- The MPDCCH narrowband shall be configured as a set. This provides network control channel capacity and flexibility and is the legacy approach for the RAR in 4-step RA.

Proposal 3: The PUR MPDCCH configuration can be used as a baseline for Msg4 monitoring, but it shall be defined for the common search space type 2, be per CE level, and provide narrowband as a set.

One aspect of using a set of MPDCCH narrowbands is that the UE needs to determine which narrowband to monitor. In the legacy 36.213 specification for the RAR this is based on the UE determining “mod (preamble index,2)”. However, in the case of CB-Msg3 there is no preamble index and therefore RAN1 needs to discuss an alternative solution.

Observation 1: Legacy specification defines the UE determines the MPDCCH narrowband to monitor based on the Msg1 preamble index, but an alternative is needed for CB-Msg3.

In our view, it is important that the metric used to determine the narrowband is constant across the CB-Msg3 replicas to ensure the UE will not be required to monitor more than one narrowband, which would for example be the case if the determination is based on CB-Msg3 repliaca transmission time. Since the transmitted CB-Msg3s are replicas of the first CB-Msg3 the CCCH SDU and the corresponding CRC are examples of such constant content.

Proposal 4: RAN1 to discuss how the UE determines which MPDCCH narrowband to monitor.

## Question 3

Q3: For eMTC, RAN2 agrees to have PUSCH configuration for shared resource configuration and decides to reuse the parameters from PUR PUSCH configuration (in IE PUR-PUSCH-Config-r16, as below) as baseline. RAN2 has some questions on some parameters: - Whether pusch-CyclicShift-r16 and pusch-NB-MaxTBS-r16 are needed - Whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources Note that: • CE mode B related parameters will not be included as RAN2 agreed to preclude CE mode B. • The power related parameters (p0-UE-PUSCH-r16 and alpha-r16) should be updated based on the result of Q1. RAN2 would like to check with RAN1 on how to define the detail PUSCH configuration.

Regarding the PUSCH cyclic shift we are fine to use the legacy approach for the DMRS sequence.

The pusch-NB-MaxTBS-r16 is needed to activate another TBS table and thus it is good to have for network flexibility. In addition, we think the edt-TBS (defining the TBS for the Msg3 in EDT) can also be relevant for CB-Msg3.

Proposal 5: pusch-CyclicShift-r16 and pusch-NB-MaxTBS-r16 are supported for CB-Msg3.

Regarding the prb-AllocationInfo we agree it can be beneficial for network flexibility and capacity to enable the prb-AllocationInfo to be a set of 6 PRBs.

Proposal 6: prb-AllocationInfo can be configured as a set for CB-Msg3.

## Question 4

Q4: For eMTC, RAN2 has discussed the PDSCH configuration for shared resource configuration. Some companies think PUR PDSCH configuration is configured according to UE capability, which is not the case for CB-Msg3/Msg4 configuration. So, RAN2 does not know whether the parameters of PUR PDSCH configuration (in pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS within IE PUR-Config-r16) is needed. RAN2 would like to check with RAN1 on whether and how to define the detail PDSCH configuration.

The pur-PDSCH-maxTBS is used to activate another TBS table so it is needed for network flexibility.

The pur-PDSCH-FreqHopping pattern depends on cell-specific parameters and can be reused as it is supported in legacy.

In the legacy PUR operation, the network is aware of the UE capability in terms of supporting the larger TBS and the frequency hopping. However, when using the CB-Msg3 procedure the UE may not have reported any capabilities before the CB-Msg3 transmission. Therefore, if UE supports CB-Msg3 (and thus perform such CB-Msg3 transmission) the UE shall also support the PDSCH TBS and frequency hopping enhancements for Msg4.

Proposal 7: the pur-PDSCH-maxTBS and pur-PDSCH-FreqHopping are adopted to be supported for CB-Msg3/4.

Proposal 8: if the UE supports the CB-Msg3 procedure the UE shall also support the CB-Msg3/4 counterpart of pur-PDSCH-maxTBS and pur-PDSCH-FreqHopping features.

## Question 5

Q5: For eMTC, RAN2 agrees to have PUCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PUCCH configuration (in IE PUR-PUCCH-Config-r16, as below) as baseline. RAN2 would like to check with RAN1 on how to define the detailed PUCCH configuration.

The PUR-PUCCH-Config-r16 contains the following elements:

PUR-PUCCH-Config-r16 ::=			SEQUENCE {

n1PUCCH-AN-r16						INTEGER (0..2047)			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r16	ENUMERATED {n1, n2, n4, n8}	OPTIONAL	-- Need ON

}

The pucch-NumRepetitionCE-Format1-r16 defines the number of PUCCH repetitions for PUCCH format 1/1a, while the n1PUCCH-AN-r16 is the UE-specific PUCCH AN resource offset. Both parameters are needed to transmit PUCCH, but the challenge is that the PUCCH AN resource offset cannot be signaled as a UE specific-value in the CB-Msg3 configuration. This may cause PUCCH collisions in the frequency domain since multiple UEs will use the same resource.

Observation 2: the n1PUCCH-AN cannot be signaled as a UE-specific value in the CB-Msg3 configuration and may therefore result in PUCCH collisions.

Likewise, the DCI scheduling the CB-Msg4 cannot provide UE-specific HARQ time domain allocations, because all UEs will receive the same DCI scrambled by the CB-RNTI.

We think one approach can be to adopt the legacy approach of 2-step RA, where the successRAR contains a “HARQ Feedback Timing Indicator”. For CB-Msg4 this would imply such a field is included in the MAC PDU for the UEs receiving their CR ID and that the indicator can allocate the UEs to different PUCCH time-domain resources and thereby alleviate the issue of the common n1PUCCH-AN.

Proposal 9: RAN1 to discuss how to ensure there are no PUCCH transmission collisions when UEs respond to CB-Msg4, e.g. by successRAR containing a “HARQ Feedback Timing Indicator to allocate the UEs to different PUCCH time domain resource.

## Question 6

Q6: For NB-IoT, RAN2 agrees to reuse the physical layer parameters pur-PhysicalConfig-r16 in PUR-Config-NB as baseline. RAN2 assumes to have below parameters: • Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16) • Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16) • Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set. • MCS configuration for NPUSCH (as in npusch-MCS-r16). Note that the power parameters (p0-UE-NPUSCH-r16 and alpha-r16) should be updated according to the result of Q1. RAN2 would like check with RAN1 on how to define the detail physical layer parameters.

In our view, the parameters related to the number of resource units, number of repetitions, and MCS configuration can be adopted from the pur-PhysicalConfig to a corresponding CB-Msg3 configuration for NB-IoT.

Regarding the indication of the NPUSCH subcarriers we agree it may be provided as a set. For network flexibility it is OK that the sets are not contiguous.

Proposal 10: The parameters related to the number of resource units, number of repetitions, and MCS configuration can be adopted from the pur-PhysicalConfig to a corresponding CB-Msg3 configuration for NB-IoT.

Proposal 11: The NPUSCH subcarriers for CB-Msg3 can be provided as a non-contiguous set.

## Question 7

Q7: For NB-IoT, RAN2 agrees to have NPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PDCCH configuration (in IE NPDCCH-ConfigDedicated-NB-r13, as below) as baseline. RAN2 would like to check with RAN1 on how to define the detail NPDCCH configuration.

NPDCCH-ConfigDedicated-NB-r13 ::=	SEQUENCE {

npdcch-NumRepetitions-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-USS-r13				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-USS-r13				ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

}

NPDCCH-ConfigDedicated-NB-v1530 ::=	SEQUENCE {

npdcch-StartSF-USS-v1530			ENUMERATED {v96, v128}

}

As also discussed for question 2, the NPDCCH configuration shall be for the common search space and not the UE specific search space as is the case for the PUR NPDCCH configuration. The reason is that the RRC Idle UEs may not have the USS configured. In our view, the CSS Type 2 is suitable, because it is also used for RAR and Msg4 monitoring in the legacy operation.

Furthermore, the NPDCCH configuration shall be per CE level. This enables UEs in various coverage conditions to reliably receive the DCI.

Proposal 12: The PUR NPDCCH configuration can be used as a baseline for Msg4 monitoring, but it shall be defined for the common search space type 2 and be per CE level.

## Question 8

Q8: For both eMTC and NB-IoT, any other L1 parameters are needed for CB-Msg3-EDT procedure in additional to previous discussion?

It may be beneficial to define a TBS per CE level for CB-Msg3 as is the case for the legacy EDT procedure.

Proposal 13: The CB-Msg3 configuration shall support a TBS per CE level.

## Additional question

In addition, RAN2 agrees that: Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1 RAN2 would like to inform RAN1 on the new RNTI for CB-Msg4 monitoring and CB-Msg3 scrambling. RAN2 has not yet made agreements on how to determine the RNTI, but RAN2 assumes there may be some potential RAN1 specification impact on this.

The introduction of a new CB-RNTI will impact the RAN1 specification defining the scrambling for the (N)PUSCH when transmitting CB-Msg3. We think scrambling of the CB-Msg3 by use of CB-RNTI is simple and similar to the legacy Msg3 scrambling, which relies on the temporary C-RNTI assigned by the RAR. As long as the Msg3 transmission windows do not overlap there is no issue with this solution.

Observation 3: RAN1 specification needs to reflect the CB-RNTI is used for scrambling the CB-Msg3.

For CB-Msg3-EDT, the UE will send a CB-Msg3 on the configured resource within the resource window by use of the contention based transmission. At the eNB receiver, there may be one or more than one UE’s CB-Msg3 received in the same Msg3 transmission resource window, with or without collision, but all these CB-Msg3 will be transmitted with same RNTI, i.e. CB-RNTI.

If the eNB receives a CB-Msg3 successfully, the eNB will transmit a Msg4 to the corresponding UE. If the eNB  receives more than one UE’s Msg3 successfully the eNB will transmit Msg4 related PDCCH/PDSCH for all these UEs in the Msg4 window. Due to Msg4 size constraints the network may have to transmit multiple Msg4s in order to address a subset of the UEs (with CR ID MAC CE) in each Msg4.

Then there will be an issue, because one UE may detect the PDCCH based on the CB-RNTI but then the UE may determine that the PDSCH MAC PDU (the Msg4) is not for that UE, based on decoding the PDSCH and finding it contains a CR ID of another UE. This will cause power waste for the UE, especially for the case that the UE finds or did not find its Msg4 after UE detects/decodes multiple PDCCH/PDCSH within the Msg4 monitoring window.

Observation 4: During the Ms4 monitoring window a UE which transmitted CB-Msg3-EDT may need to detect PDCCH and decode PDSCH multiple times with the same CB-RNTI and corresponding to the same CB-Msg3 resource window.

Observation 5: There will be much power waste on Msg4 detection as UE may only find its own Msg4 or even not find its own Msg4 after multiple times detection of PDCCH/PDSCH with other UEs’ CR ID.

RAN1 should discuss how to avoid this unnecessary power waste for IoT UE. Actually, it is clear that not all the Msg4 (PDCCH/PDSCH), within the Msg4 monitoring window, are for the same UE even they are using the same CB-RNTI. One simple way is to divide the Msg4 monitoring window corresponding to the same Msg3 transmission window into groups corresponding to different groups of UE and UE only need to decode the Msg4 for the corresponding group. Another approach can be that the DCI scheduling the Msg4 PDSCH provides information on which CB-Msg3 it is addressing, for example based on a constant parameter of the CB-Msg3 replicas such as the  CRC.

Proposal 14: The network may provide assistance information to help the UE determine whether/how to monitor for CB-Msg4 during the Msg4 monitoring window.

# Conclusion

In this contribution, we discussed our view on CB-Msg3-EDT. Our observations and proposals are as follows:

Observation 1: Legacy specification defines the UE determines the MPDCCH narrowband to monitor based on the Msg1 preamble index, but an alternative is needed for CB-Msg3.

Observation 2: the n1PUCCH-AN cannot be signaled as a UE-specific value in the CB-Msg3 configuration and may therefore result in PUCCH collisions.

Observation 3: RAN1 specification needs to reflect the CB-RNTI is used for scrambling the CB-Msg3.

Observation 4: During the Ms4 monitoring window a UE which transmitted CB-Msg3-EDT may need to detect PDCCH and decode PDSCH multiple times with the same CB-RNTI and corresponding to the same CB-Msg3 resource window.

Observation 5: There will be much power waste on Msg4 detection as UE may only find its own Msg4 or even not find its own Msg4 after multiple times detection of PDCCH/PDSCH with other UEs’ CR ID.

Proposal 1: The UE shall only apply power ramping between CB-Msg3 windows in case of unsuccessful contention resolution or, that is, CB-Msg3 reattempt.

Proposal 2: For CB-Msg3 power ramping a new counter is needed for the number of CB-Msg3 attempts per CE level, while the powerRampingStep can be reused. RAN1 may discuss whether a new Initial Received Target Power parameter is needed.

Proposal 3: The PUR MPDCCH configuration can be used as a baseline for Msg4 monitoring, but it shall be defined for the common search space type 2, be per CE level, and provide narrowband as a set.

Proposal 4: RAN1 to discuss how the UE determines which MPDCCH narrowband to monitor.

Proposal 5: pusch-CyclicShift-r16 and pusch-NB-MaxTBS-r16 are supported for CB-Msg3.

Proposal 6: prb-AllocationInfo can be configured as a set for CB-Msg3.

Proposal 7: the pur-PDSCH-maxTBS and pur-PDSCH-FreqHopping are adopted to be supported for CB-Msg3/4.

Proposal 8: if the UE supports the CB-Msg3 procedure the UE shall also support the CB-Msg3/4 counterpart of pur-PDSCH-maxTBS and pur-PDSCH-FreqHopping features.

Proposal 9: RAN1 to discuss how to ensure there are no PUCCH transmission collisions when UEs respond to CB-Msg4, e.g. by successRAR containing a “HARQ Feedback Timing Indicator to allocate the UEs to different PUCCH time domain resource.

Proposal 10: The parameters related to the number of resource units, number of repetitions, and MCS configuration can be adopted from the pur-PhysicalConfig to a corresponding CB-Msg3 configuration for NB-IoT.

Proposal 11: The NPUSCH subcarriers for CB-Msg3 can be provided as a non-contiguous set.

Proposal 12: The PUR NPDCCH configuration can be used as a baseline for Msg4 monitoring, but it shall be defined for the common search space type 2 and be per CE level.

Proposal 13: The CB-Msg3 configuration shall support a TBS per CE level.

Proposal 14: The network may provide assistance information to help the UE determine whether/how to monitor for CB-Msg4 during the Msg4 monitoring window.

# References

- R2-2503175, “LS on CB-msg3-EDT”, Apr 2025