3GPP TSG RAN WG1 #121								R1-2504071

St Julians, Malta. 19th – 23rd May 2025

Agenda Item 	:	9.11.4

Source 	:	Sony

Title 	:	On IoT-NTN uplink capacity enhancements

Document for 	:	Discussion and decision

# Introduction

In RAN1#120 Athens, RAN1 made a working assumption on the support of TDM DMRS for NPUSCH format 1 with a 3.75kHz SCS:

Working assumption For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT. Send LS to RAN4 asking whether there would be any issue (e.g. phase continuity) for supporting such TDM DMRS for IoT NTN.

RAN1 sent the following LS [2] to RAN4 in order to see whether RAN4 had any concerns on the working assumption:

Overall description RAN1 has made the following working assumption on the DMRS pattern for OCC for 3.75kHz SCS OCC for NPUSCH format 1: For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT. Question: 1. From a RAN4 perspective, is it feasible to introduce support of TDM DMRS, as per the description above, in Rel-19?

In RAN1#121 Malta, RAN4 provided the following LS response to RAN1 [3]:

# 1. Overall Description: RAN4 would like to thank RAN1 on the LS on TDM DMRS for 3.75kHz SCS OCC [1], and RAN4 has studied the feasibility of TDM DMRS. - RAN4 has concluded that it is feasible for the UE to maintain the phase continuity across the blank DMRS symbol in TDM DMRS scheme. - RAN4 will further discuss potential need for requirements if TDM DMRS would be specified (e.g. the measurement time for frequency error requirement). Therefore, from RAN4 side, no feasibility issue is identified and RAN4 respectfully asks RAN1 to take into account the information above when determining the final feasibility of TDM DMRS. # 2. Actions: # To RAN1: # RAN4 respectfully asks RAN1 to take into account the information above when determining the final design on TDM DMRS.

At RAN1#120bis Wuhan, RAN1 made the following agreement related to down selection of mapping options for TDM DMRS:

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots

This document considers the support of TDM DMRS for 3.75kHz SCS NPUSCH format 1.

# TDM DMRS for 3.75kHz SCS NPUSCH

## 2.1 Confirmation of working assumption

In RAN1#120 Athens, RAN1 made a working assumption on the support of TDM DMRS for NPUSCH format 1 with a 3.75kHz SCS:

Working assumption For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT. Send LS to RAN4 asking whether there would be any issue (e.g. phase continuity) for supporting such TDM DMRS for IoT NTN.

In RAN1#121 Malta, RAN4 provided the following LS response to RAN1 [3]:

# 1. Overall Description: RAN4 would like to thank RAN1 on the LS on TDM DMRS for 3.75kHz SCS OCC [1], and RAN4 has studied the feasibility of TDM DMRS. - RAN4 has concluded that it is feasible for the UE to maintain the phase continuity across the blank DMRS symbol in TDM DMRS scheme. - RAN4 will further discuss potential need for requirements if TDM DMRS would be specified (e.g. the measurement time for frequency error requirement). Therefore, from RAN4 side, no feasibility issue is identified and RAN4 respectfully asks RAN1 to take into account the information above when determining the final feasibility of TDM DMRS. # 2. Actions: # To RAN1: # RAN4 respectfully asks RAN1 to take into account the information above when determining the final design on TDM DMRS.

The TDM DMRS scheme allows for orthogonal multiplexing of two UEs for OCC. While there is a small performance loss due to channel estimation loss associated with the smaller DMRS density, this small performance loss was considered acceptable, based on simulations in RAN1 from various companies. RAN4 have provided an LS reply indicating that they do not have a concern with the adoption of TDM DMRS and that phase continuity can be maintained across the blanked TDM DMRS.

It is hence concluded that it is feasible to support TDM DMRS for 3.75kHz SCS for NPUSCH format 1.

Proposal 1: Confirm the following working assumption from RAN1#120 Athens:

“For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.”

### 2.2 Mapping between DMRS sequence samples and active TDM DMRS slots

At RAN1#120bis Wuhan, RAN1 made the following agreement related to down selection of mapping options for TDM DMRS:

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots

Option 1 changes or loses the orthogonality properties of the DMRS sample sequence. Hence, this is not the preferred mapping between DMRS sequence samples and active TDM DMRS slots.

Option 2 preserves the orthogonality properties of the DMRS sample sequence. We understand that this was the original proposal when the TDM DMRS scheme was introduced.

Proposal 2: For the mapping between DMRS sequence samples and active TDM DMRS slots, the following option is adopted:

Option 2: Dropping of samples of the original DMRS sequence in blanked slots

### 2.3 Scheduling of TDM DMRS pattern to UE

This section considers different potential methods of scheduling and aligning OCC and TDM DMRS transmissions for NPUSCH format 1 at 3.75kHz SCS.

Option 1: Transmissions can start at every 4 slots; association between OCC codeword and TDM DMRS pattern

The TDM DMRS pattern for multiplexing 2 UEs with TDM DMRS for OCC with this option is shown in Figure 1.

Figure 1 - Basic pattern for TDM DMRS multiplexing of UEs

This pattern can be achieved by constraining OCC transmissions to start at slot boundaries such that n mod 4 = 0. UE1 can then be assigned with OCC codeword [1 1] and TDM pattern 1100 and UEs can be assigned with codeword [1 -1] and TDM pattern 0011.

This approach leads to a level of scheduling inflexibility and can be wasteful of resource if the eNB needs to wait to schedule a UE in order to align it with a transmission from another UE.

Option 2: DMRS pattern starts at beginning of NPUSCH and scheduler orthogonalizes DMRS pattern

In this option, only a single TDM DMRS pattern is specified, such as 1100, and the scheduler ensures orthogonality of the DMRS of the UEs by deferring the start time of a UE’s NPUSCH transmission to ensure orthogonality, for example as shown in  Figure 2.

Figure 2 – Scheduler orthogonalization of TDM DMRS patterns

This approach can be wasteful of NPUSCH resource since paired OCC transmissions need to be deferred to provide compatibility of the TDM DRMS locations between UEs.

Option 3: TDM DMRS pattern is a function of slot number and OCC codeword. NPUSCH can start at any slot boundary

In this option, the TDM DMRS pattern (or phase of the pattern) depends on the slot number and the OCC codeword, but not on the start time of the NPUSCH transmission. This would allow the NPUSCH transmission to start at any slot and decouple the timing of the TDM DMRS pattern from the timing of the start of the NPUSCH transmission. In an example, the following mapping between OCC codeword and slots for DMRS transmission could be applied:

| OCC codeword | Slots n for DMRS transmission |
| --- | --- |
| [1 1] | N mod 4 = 0 N mod 4 = 1 |
| [1 -1] | N mod 4 = 2 N mod 4 = 3 |

This mapping would allow the following example allocations and multiplexing of UEs (Figure 3):

Figure 3 – TDM DMRS transmission timing based on slot number and OCC codeword

The allocations in this figure show:

- Top: UE1 is scheduled with OCC codeword [1 1] and NPUSCH starting in slot 0.

- Middle: UE1 is alternatively scheduled with OCC codeword [1 1] and NPUSCH starting in slot 1. Note that the TDM DMRS locations are the same as for the top row since the TDM DMRS location is not a function of the NPUSCH start slot.

- Bottom: UE2 is scheduled with OCC codeword [1 -1] and NPUSCH starting in slot 0. The TDM DMRS locations are different to those of the top row since the TDM DMRS locations are a function of the OCC codeword.

Option 4: TDM DMRS pattern statically or semi-statically associated with a UE. Scheduler pairs UEs with compatible TDM DMRS patterns

In this option, the TDM DMRS pattern is associated with the UE either statically (e.g. as a function of the UE’s RNTI) or semi-statically (e.g. the UE is RRC configured with a TDM DMRS pattern). In order to OCC multiplex UEs, the scheduler would need to pair UEs in terms of their TDM DMRS pattern (as well as based on other characteristics, such as power level, traffic activity etc). This option would hence seem to make the scheduling and pairing task more difficult for the scheduler.

Proposal 3: RAN1 down-selects between the following options for determining the TDM DMRS pattern applied to a UE:

- Option 1: NPUSCH transmission can start at every 4 slots; association between OCC codeword and TDM DMRS pattern

- Option 2: DMRS pattern starts at beginning of NPUSCH and scheduler orthogonalizes DMRS pattern

- Option 3: TDM DMRS pattern is a function of slot number and OCC codeword. NPUSCH can start at any slot boundary

- Option 4: TDM DMRS pattern statically or semi-statically associated with a UE. Scheduler pairs UEs with compatible TDM DMRS patterns

Based on the analysis of the options above, with the goal of maximising scheduler flexibility and providing the greatest possible amount of resource for NPUSCH in order to maximise capacity gains, our preference is that option 3 is applied:

Proposal 4: TDM DMRS pattern is a function of slot number and OCC codeword. NPUSCH can start at any slot boundary.

# Conclusion

This document has considered issues related to the TDM DMRS pattern applied for NPUSCH format 1 with a 3.75kHz SCS. The following proposals are made:

Proposal 1: Confirm the following working assumption from RAN1#120 Athens:

“For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.”

Proposal 2: For the mapping between DMRS sequence samples and active TDM DMRS slots, the following option is adopted:

Option 2: Dropping of samples of the original DMRS sequence in blanked slots

Proposal 3: RAN1 down-selects between the following options for determining the TDM DMRS pattern applied to a UE:

- Option 1: NPUSCH transmission can start at every 4 slots; association between OCC codeword and TDM DMRS pattern

- Option 2: DMRS pattern starts at beginning of NPUSCH and scheduler orthogonalizes DMRS pattern

- Option 3: TDM DMRS pattern is a function of slot number and OCC codeword. NPUSCH can start at any slot boundary

- Option 4: TDM DMRS pattern statically or semi-statically associated with a UE. Scheduler pairs UEs with compatible TDM DMRS patterns

Proposal 4: TDM DMRS pattern is a function of slot number and OCC codeword. NPUSCH can start at any slot boundary.

References

[1]	R1-2502333. “Final FL summary for IoT-NTN”. Moderator (Sony). RAN1#120bis. Wuhan, China. April 2025.

[2]	R1-2501622. “LS on TDM DMRS for 3.75kHz SCS OCC”. RAN1, Sony. RAN1#120. Athens, Greece. February 2025.

[3]	R1-2503211. “LS reply on TDM DMRS”. RAN4, Sony. RAN1#121. Malta. May 2025.