3GPP TSG RAN WG1 #121		2504089

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.4.2

Source:	Fujitsu

Title:	Discussion on coding aspects

Document for:	Discussion

Introduction

Following the suggestion given in [1], the open issues on 1/2 code rate for FEC and the number of repetitions are discussed in this contribution.

FEC for D2R

[H] Proposal 4.2-1-v2 If FEC is applied for a D2R transmission, for the code rate of FEC, down-select from the following options: - Option 1: 1/3. - Option 2: {1/2, 1/3}. For the code rate of 1/2, it is achieved by bypassing the third parity chain with polynomial (165). If FEC is applied for a D2R transmission, the code rate of FEC additionally supports values 1/2. - The code rate of 1/2 is achieved according to TS 36.212.

In RAN1#120 bis, the discussions on whether to support 1/2 code rate was quite controversial. In the end, the issue is kept open, and the above proposal was left for a quick check in RAN1#121.

The potential concerns to the support of 1/2 code rate might be as follows:

- Really think that the support of 1/2 code rate doesn’t provide enough gain.

  - The gain of 1/2 code rate does not obviously over that of double repetition.

  - The working SNR of 1/2 code rate is not at the intermediate point in between those of 1/3 code rate and no FEC. The working SNR of 1/2 code rate is near the working SNR of 1/3 code rate and little far from the working SNR of no FEC.

- Don’t understand how to achieve 1/2 code rate at transmitter and/or how to decode the coded bits with 1/2 code at receiver, and don’t want to understand it.

Regarding concern 1-1, this kind of comparison and conclusion is ridiculous. The FEC and the repetition can be deployed together, but they are usually applied to resolve different issues.

- The FEC can provide coding gain which means that, assuming the transmitting energy is constant, the FEC can provide additional gain by adopting well-designed check-parity relationships among coded bits.

- The repetition can only provide accumulation gain and is usually adopted when the gain is expected to be very large, e.g., 3dB and 6dB, such as for the purpose of coverage enhancement. Such large gain requires the transmitting energy gets doubled and redoubled. Repetition is the way to obtain doubled and redoubled energy in time domain which is widely applied in mobile communication because the energy boosting in power domain and in frequency domain is often restricted due to the high requirement for devices and/or regulations.

Regarding concern 1-2, it is true that the choice of code rate 1/2 is not perfect. The working SNR gaps between no FEC, 1/2 code rate and 1/3 code rate are not averaged. This may impact the gain which can be obtained by MCS adaptation. Usually, when MCS levels are designed for scheduling adaptation, the working SNRs of MCS levels are expected to averagely distribute in the whole working SNR region of a system to pursue the biggest potential gain. Although 1/2 code may not be satisfying enough, supporting one more code rate in between no FEC and 1/3 code rate does be worthy to further discuss, if this item has time. But unfortunately, it does not.

Regarding concern 2, there is only one thing clear that this kind of issue is not remediable. If delegates/companies having this concern are the majority in the AIoT session, the only way we can go is to give 1/2 code rate up.

AIoT is somehow a special item. There is no precise requirement for spectrum efficiency, capacity, latency or reliability. What we can do and what we are doing is try to reach a ‘best’ design, while the definition of the ‘best’ design is unclear and almost impossible to reach common sense on it among companies which is a question without its answer.

Now, time is nearly up for the AIoT item. If the support of the 1/2 code is still a minority view, let’s give it up and move on to complete the whole item.

Observation 1:

- Supporting at least one intermediate code rate between no FEC and 1/3 code rate can provide more flexibility to readers and bigger probability to achieve better network spectrum efficiency.

- The 1/2 code rate is not perfect. If the AIoT item had time, it would be a good direction to further study which code rate is better than 1/2 as the intermediate code rate in between no FEC and 1/3 code rate.

Repetition

[H] Proposal 5.1-v2 For the number of block-level repetitions, down-select from the following options: - Option 1: {1, 2}. - Huawei, - Option 2: {1, 2, 4}. - LGE, CMCC, ZTE, Panasonic, OPPO, MediaTek, NEC, - Note: When the number of block-level repetition is 1, it indicates no repetition.

Another open issue is about the repetition number to be supported for D2R. We support Option2. The reason is that some margin should be reserved to guarantee the stability and reliability of Rel-19 AIoT system. This is the first release for AIoT. AIoT is a quite unique item compared with all other items completed or being completed in 3GPP. Some margin is necessary to make sure the Rel-19 AIoT is robust enough for commercial deployment.

Proposal 1: Support {1, 2, 4} for D2R block-level repetitions.

Conclusion

In this contribution, we have the following proposal and observation:

Observation 1:

- Supporting at least one intermediate code rate between no FEC and 1/3 code rate can provide more flexibility to readers and bigger probability to achieve better network spectrum efficiency.

- The 1/2 code rate is not perfect. If the AIoT item had time, it would be a good direction to further study which code rate is better than 1/2 as the intermediate code rate in between no FEC and 1/3 code rate.

Proposal 1: Support {1, 2, 4} for D2R block-level repetitions.

- Reference

- R1-2503107, “Summary #6 for coding aspects of physical channel design”, CMCC, RAN1#120bis, Wuhan, China, April 7-11, 2025.