3GPP TSG RAN WG1 #121	 R1-2504100

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.4.4

Source:	HONOR

Title:	Discussion on L1 control information and L1 procedural aspects

Document for:	Discussion and Decision

Introduction

In RAN#106, a new Rel-19 WID “Solutions for Ambient IoT (Internet of Things) in NR” has been approved with the following objectives [1]:

RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this contribution, we will discuss the following aspects: multiplexing/multiple access and scheduling information.

Discussion

  - Multiplexing/multiple access

    - Resource allocation for Msg1

The following agreements were reached at the last RAN1 meeting [2].

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. Agreement At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions. Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2

Regarding Toffset1, it is used to determine the starting position of the first time domain of Msg1. Considering forward compatibility, a certain degree of flexibility needs to be provided. Therefore, we recommend indicated explicitly by the R2D transmission triggering random access.

Proposal 1: Toffset1 is explicitly indicated by the R2D transmission triggering random access.

Regarding Toffset2, it is used to determine the starting position of the second time domain of Msg1. To save the cost of indication, it is recommended to obtain this value through predefined rules. Since the size of the Msg1 message is known and the starting position is also known, it is sufficient to indicate only the GAP value after the first Msg1. Considering simplified indications and overhead, we suggest directly reusing the value of Toffset1 as the value for GAP.

Proposal 2: Toffset2 is obtained based on the start time of the first Msg1 time domain resource, the time domain length of Msg1, and the GAP between Msg1. Additionally, the time domain length of GAP reuses the value of Toffset1.

    - Msg2 related aspects

Msg2 transmission(s)

The following agreements have been reached at the RAN1 meeting [3].

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

Since Msg2 already supports two transmission schemes, the first issue to address is how the device knows which specific transmission scheme is used. We will discuss the following solutions.

The first and simplest solution is to configure it through Msg0, and if it is a one-to-many transmission method, the association between multiple Msg1 and Msg2 messages must also be specified. Since there is a high possibility of conflicts with Msg1, pre-configuring the relationship between Msg2 and Msg1 would waste a lot of resources. Therefore, this solution is not recommended.

The second solution is to determine the corresponding Msg2 message of the device through the control information of Msg2 and some pre-configured rules, but the relationship between Msg1 and Msg2 is not specified in advance. For example, the control information can indicate the message of non-conflicting Msg1, the maximum value in the RN_16 of the response message, etc. Based on this indicated information, the device can pre-determine whether its response information is included in a certain Msg2, thereby deciding in advance whether it needs to demodulate the corresponding Msg2 message. Therefore, this solution not only can flexibly determine the corresponding Msg2 message but also can save the device's energy consumption.

Observation 1: Indicating relevant information in the control information of Msg2 not only can flexibly determine the corresponding Msg2 message but also can save the device's demodulation energy consumption.

Proposal 3: The control information of Msg2 indicates the relevant information for the device to determine the Msg2 response message.

Starting time for A-IoT Msg2 monitoring

Regarding the selection of the reference point for the start time of Msg2, the following schemes have been identified in TR 38.769 and can serve as the starting point for discussion.

Option 1: End of time domain resource where the device transmitted the Msg1 Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s) Option 3: End of the R2D transmission triggering random access Option 4: End of a R2D transmission carrying Msg2 Note: The reference time is used to determine the starting time for Msg2 monitoring, it does not mean that the starting time always equals the reference time.

Considering that multiple TDMA and FDMA resources can be allocated simultaneously, Option 2 is more reasonable than Option 1. Option 3 needs to introduce additional time domain indication compared to Option 2, so that the device can calculate the actual sending start point of Msg2 based on this reference point. Option 4 has already detected Msg2, so it is meaningless to calculate the starting reference point of Msg2. Therefore, we recommend using Options 2 as the method for selecting the starting reference point of Msg2.

Proposal 4: Support option 2 as the selection scheme for the starting reference point of Msg2.

Time duration for A-IoT Msg2 monitoring

Regarding the time duration for A-IoT Msg2 monitoring, the following schemes have been identified in TR 38.769 and can serve as the starting point for discussion.

To determine the time duration for monitoring Msg2 in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: Option 1: Predefined Option 2: Indicated in the R2D transmission triggering random access Note: Option 1 and Option 2 may not be mutually exclusive.

Option 1 is relatively simple and can save on indication overhead. However, option 1 may require a longer duration and cannot be flexibly adapted to different scenarios. Option 2 is more flexible and can adapt to different scenarios of Msg1 resources and Msg2 response methods. Therefore, it is recommended to support option 2.

Proposal 5: Support Option 2 for the time duration for A-IoT Msg2 monitoring.

    - Resource allocation for Msg3

The following agreements have been reached at the RAN1 meeting.

Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

If option 1 is supported, then additional rules are needed to determine whether the indication field exists. This will increase the complexity of control information detection, so it is not recommended to support Option 1.

Proposal 6: Do not support option 1 for the allocation of Msg3 frequency domain resources.

For the explicit indication, considering the mutual exclusivity of the device's frequency domain resources, a bitmap can be used to indicate all frequency resource allocations for responding users. As shown in the figure below, each bit represents a frequency resource position. When the bit is 1, it indicates that the resource is allocated for use by a specific device; otherwise, it indicates that the resource is not allocated.

Proposal 7: Use the same bitmap to indicate the frequency resources for Msg3 of all responding users.

Figure 2. Frequency resource explicit indication for Msg3

  - Scheduling information

    - Scheduling of R2D

For R2D reception

Compared with eMBB, the design principles of A-IoT control information should focus more on how to reduce the complexity and energy consumption of the device. Below we will discuss what control information needs to be included based on this principle.

First, the message type needs to be carried in the control information, such as paging, command messages, etc. The purpose is to let the device know whether the message is a broadcast or unicast message type. If it is a multicast type, it is necessary to detect the following information. If it is a unicast, it is necessary to determine whether it is information for the device based on further information. If not, no further detection is required. Otherwise, it is necessary to detect the following information.

First, the message type needs to be carried in the control information, such as paging, command messages, etc. The purpose is to let the device know whether the message is a broadcast or unicast message type. If it is a multicast type message, the device needs to detect the following information. If it is a unicast, it is necessary to determine whether it is information for the device based on further information. If not, no further detection is required. Otherwise, the device needs to detect the following information.

Observation 2: The message type allows the device to know as early as possible whether the following information needs to be processed.

Proposal 8: The control information includes the message type.

When the message type is unicast, it is necessary to further determine whether it is a message that the device needs to receive based on the device ID. If the device ID matches, the message is received and processed. Otherwise, no subsequent processing is required to save power.

Proposal 9: The control information of the unicast message contains the device ID.

Control information

There are two control information carrying schemes in TR 38.769: L1-control signaling and Higher-layer signaling [4]. The pros and cons of the two solutions are compared as follows:

Table 1. Comparison of the pros and cons of L1 control signaling and higher layer signaling

|  | Pros | Cons |
| --- | --- | --- |
| L1-control signaling | - The device can obtain information in advance and save power. - High reliability. | - Increase design complexity. - High overhead. |
| Higher-layer signaling | - Simple design. - Low overhead. | - Blind detection may be required. - High power consumption; |

Although both have their own pros and cons, power consumption and complexity are critical for device 1. Therefore, we recommend using L1-control signaling.

Proposal 10: Support L1-control signal to carry control information.

- Conclusions

In this contribution, we discuss on L1 control information and L1 procedural aspects with the following observations and proposals.

Observation 1: Indicating relevant information in the control information of Msg2 not only can flexibly determine the corresponding Msg2 message but also can save the device's demodulation energy consumption.

Observation 2: The message type allows the device to know as early as possible whether the following information needs to be processed.

Proposal 1: Toffset1 is explicitly indicated by the R2D transmission triggering random access.

Proposal 2: Toffset2 is obtained based on the start time of the first Msg1 time domain resource, the time domain length of Msg1, and the GAP between Msg1. Additionally, the time domain length of GAP reuses the value of Toffset1.

Proposal 3: The control information of Msg2 indicates the relevant information for the device to determine the Msg2 response message.

Proposal 4: Support option 2 as the selection scheme for the starting reference point of Msg2.

Proposal 5: Support Option 2 for the time duration for A-IoT Msg2 monitoring.

Proposal 6: Do not support option 1 for the allocation of Msg3 frequency domain resources.

Proposal 7: Use the same bitmap to indicate the frequency resources for Msg3 of all responding users.

Proposal 8: The control information includes the message type.

Proposal 9: The control information of the unicast message contains the device ID.

Proposal 10: Support L1-control signal to carry control information.

References

- RP-243326, “Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106, Dec. 9 - 12, 2024.

- Draft_Minutes_report_RAN1#120b_v010

- Draft_Minutes_report_RAN1#120_v030

- 3GPP TR 38.769 V2.0.1 (2024-12)