3GPP TSG RAN WG1 #121	 R1-2504101

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.5.3

Source:	HONOR

Title:	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and Decision

Introduction

In RAN#102, a new Rel-19 WID “Enhancements of network energy savings for NR” has been approved with the following objective as one of the objectives [1]:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we will mainly discuss two aspects: adaptation of SSB and adaptation of PRACH in time domain.

Discussion

  - Adaptation of SSB in time domain

RAN1 has reached the following agreement at the last meeting [2].

Agreement For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell, - The DCI is scrambled a new RNTI, - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell - the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter - the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell Agreement For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell. - Xmax=2 Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst

Regarding the determination of the first SSB burst, it can be determined by combining the candidate positions of the SSB burst within 20ms periodicity. The purpose is that, compared to legacy SSB transmission, it does not introduce additional candidate positions for SSB transmission. As shown in the figure below, after the SSB periodicity switch, the position of the first SSB burst is the first candidate transmission position of the 20ms SSB periodicity that satisfies the interval of d slots. The first chosen sending position is because the interval is 8ms, meeting the 5ms interval requirement. The second chosen sending position is because the first interval is 3ms, therefore, the second candidate position was chosen. Therefore, we recommend that the candidate position for the first SSB burst be jointly determined based on the 20ms periodicity SSB candidate position and interval time.

Proposal 1: The first SSB burst is the first candidate position for SSB transmission with a periodicity of 20ms after d slots.

Fig 1. Determine the position of the first SSB burst

  - Adaptation of PRACH in time domain

RAN1 has reached the following agreement at the last meeting.

Agreement For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon Agreement For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs Agreement For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling. Agreement For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of - PRACH association period - This PRACH mask applies to every [configured] K SSB RO association pattern period(s) Agreement Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported - CB-PreamblesPerSSB Agreement For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by: - Option 1-2: Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods) - For K: one from up to four candidate values {2,4,8, [1 or 16]} Mask index; Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP). 0; 1st half of the APs in K APPs. 1; 1st quarter of the APs in K APPs. 2; 1st eighth of the APs in K APPs. 3; 1st sixteenth of the APs in K APPs. Agreement - For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI - The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..}) Agreement For adaptation of PRACH in time-domain, for a connected mode UE, - One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

Regarding the issue of RA-RNTI, no additional special handling is required. The reason is that for users supporting PRACH time domain adaptation, it is possible to distinguish between legacy PRACH resources and additional PRACH resources. Therefore, for NES users, it is necessary to unify the handling of legacy PRACH resources and additional PRACH resources, that is, to unify the numbering of legacy ROs and additional ROs configured in the frequency domain. This way, there will be no issues when calculating RA-RNTI.

Proposal 2: Unify the numbering of legacy frequency domain ROs and additional frequency domain ROs according to their frequency domain positions to resolve the RA-RNTI ambiguity issue.

- Conclusions

In this contribution, we discuss on adaptation of common signal/channel transmission with the following proposals.

Proposal 1: The first SSB burst is the first candidate position for SSB transmission with a periodicity of 20ms after d slots.

Proposal 2: Unify the numbering of legacy frequency domain ROs and additional frequency domain ROs according to their frequency domain positions to resolve the RA-RNTI ambiguity issue.

References

- RP-234065, “Enhancements of network energy savings for NR”, RAN#102, Dec. 11 - 15, 2023.

- Draft_Minutes_report_RAN1#120b_v010