3GPP TSG RAN WG1 #121		                                           R1-2504191

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	5

Source: 	OPPO

Title:	Draft reply LS on LP-WUS in RRC_CONNECTED

Document for:	Discussion & Decision

Introduction

In RAN2 LS [1], 3 issues are listed for RAN1 input: Short DRX configuration restriction with Option 1-1, LP-WUS monitoring, and UE time offset preference.

In this contribution we give out views for replying the LS.

Discussion

Short DRX configuration restriction

RAN2 discussed the Short DRX cycle applicability with LP-WUS in RRC_CONNECTED mode and made the following agreement and a working assumption: For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

RAN1 had agreements making the Option 1-1 and 1-2 work independently. Thus, it seems the configuration restriction is feasible only for Option 1-1. In case of Option 1-2, RAN1 have understanding that the short DRX can also define time outside of the Active time. Thus, the working assumption is also feasible.

Proposal1: RAN1 can reply the Working Assumption for Option 1-2 is feasible in RAN1 perspective.

LP-WUS monitoring

RAN2 discussed whether there are any scenarios on when the UE would not be able to monitor LP-WUS and made the following working assumption: Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Based on this working assumption, RAN2 would like to ask RAN1: 1. Whether there are any cases/scenarios on when the UE is not able to monitor LP-WUS? - RAN2 would like to note that for DCP, such cases/scenarios included collision with Active Time, measurement gap, interruption caused by BWP switching, and RAR window monitoring for BFR. 2. Is there any case when the UE is not able to monitor LR and MR simultaneously?

It was also discussed in the earlier RAN1 meeting that when the UE had a positive indication by LP-WUS, the UE should be only turned off LR when the MR switched on. And when the MR monitoring is over, the LR should be back. The motivation of that assumption is lower UE complexity and power consumption.

That is reasonable motivation. However, it is more related to how the LR and MR is implemented and how exact power consumption goes during different phases. Further, it is unclear that the LR and MR will not work in the same time even for measurement.

Thus, RAN1 did not reached agreement on whether and how to restrict that LR and MR should not be turned on in same time. Thus, we consider this can be up to RAN2 decision.

Proposal 2: For the LP-WUS, the LR can be able to monitor in any time without restriction. It is up to RAN2 to allow UE to monitor LR and MR simultaneously.

UE time offset preference

RAN2 also discussed and agreed the following: If configured, the UE can signal a preferred time offset via UAI signalling. RAN2 would like to know whether there were any further conclusions in RAN1 on what information the preferred time offset via UAI should provide (e.g. in relation to the minimum time offset provided as a UE capability)?

RAN1 discussed 3 minimal values for time gap corresponding to deep sleep, light sleep or micro sleep. Considering those margins, we can extend them into 33, 13 and 3 ms as candidates for UE capability report. We don’t see needs for different times for different SCSs, since the MR ramp-up time is not so depending on SCS. The UAI is also related to the candidate values.

Regarding the UE Assistant Information of the minimum gap, we understand the UE could report another smaller gap than its capability report. The UAI report should be optional. To give RAN2 more information, the UAI should indicate to a longer sleeping time, but still in the 3 candidate values. Considering the following information to be reply to RAN2, if can be agreed.

Proposal 3: In RRC CONNECTED mode, support UE capability report one value among 3 candidate values for minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

3 ms, 13ms and 33 ms are the candidature values.

UE could additionally report as UAI with another smaller gap than its capability report and among the candidature values.

Conclusion

We have following proposals regarding the questions in the LS:

Proposal1: RAN1 can reply the Working Assumption for Option 1-2 is feasible in RAN1 perspective.

Proposal 2: For the LP-WUS, the LR can be able to monitor in any time without restriction. It is up to RAN2 to allow UE to monitor LR and MR simultaneously.

Proposal 3: In RRC CONNECTED mode, support UE capability report one value among 3 candidate values for minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

3 ms, 13ms and 33 ms are the candidature values.

UE could additionally report as UAI with another smaller gap than its capability report and among the candidature values.

Reference

[1] R2-2503202	LS on LP-WUS in RRC_CONNECTED, RAN WG2