3GPP TSG RAN WG1 #121												R1- 2504253

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.10.1

Source: 	LG Electronics

Title: 	Discussion on XR during RRM measurements

Document for:	Discussion and decision

- Introduction

In the RAN1#120, followings were agreed for XR traffic during RRM measurements[1]:

Agreement Agreement from RAN1#119 is updated (red part) For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells). - Alt1: If skipping field is configured to DCI format 0_3/1_3, only cells that are intra-band (and have same SCS) can be co-scheduled. In other words, if co-scheduled cells could be inter-band, skipping indication field cannot be configured. Agreement When measurement gap/restriction is indicated to be skipped by explicit indication by DCI format 1_1/1_2 that does not schedule PDSCH and - if CIF field is present, the scheduled cell for the skipping indication is determined by CIF field. - if CIF field is not present, the scheduled cell for the skipping indication is the scheduling cell.

In this contribution, we discuss and provide views on enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements.

- Discussion

Configuration of the gap/restriction skipping indicator

While the previous agreement clarified that the one-bit indication in DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell in cross carrier scheduling scenarios, which/how the RRC configuration/parameter enables the inclusion of this one-bit indication in the DCI remains undefined. Further clarification is required to ensure unambiguous UE behavior, especially for cross carrier scheduling where the DCI is transmitted on a scheduling cell but applies to a scheduled cell.

For cross carrier scheduling scenarios, the following options can be considered to determine when the one-bit indication should be included in the DCI:

- Option 1 (Scheduling Cell Configuration): The inclusion of the one-bit indication is determined by the configuration of the scheduling cell. When the scheduling cell is configured with the parameter that enables this indication for a DCI format, the one-bit indication is included in the DCI scheduling any possible scheduled cells. It enables gap/restriction cancellation for the corresponding scheduled cells as agreed.

- Option 2 (Scheduled Cell Configuration): The inclusion of the one-bit indication is determined by the configuration of the scheduled cell. The one-bit indication is only included in the DCI format scheduling cells that are configured with the parameter that enables this indication. It enables gap/restriction cancellation for those specific scheduled cells as agreed..

We propose that the inclusion of this indication in the DCI is determined by the RRC configuration for the DCI format on the scheduled cell. This aligns with the existing principle that DCI contents and their interpretation for the case of cross-carrier scheduling

Proposal 1: In case of cross carrier scheduling, consider the following Options for configuring gap skip/cancel indication in a DCI format. The one-bit indication is included by the configuration for the DCI format of the scheduling cell.

For DCI formats 0_3/1_3, which are designed for multi-carrier scheduling, the configuration of the 1-bit gap/restriction skipping indication requires specific attention. It has been agreed that this 1-bit indication, when present, corresponds to the first gap/restriction occasion among the co-scheduled cells (acting as a Type-1C field). This clearly defines the target of the indication. However, the RRC mechanism determining the inclusion of this 1-bit field within the DCI format 0_3/1_3 itself is not yet clearly defined and is crucial for unambiguous UE operation.

If the RRC parameter enabling the 1-bit field in DCI format 0_3/1_3 is configured for the DCI format in the scheduling cell (Option 1 in Proposal 1), the logic is straightforward: the bit is included if DCI 0_3/1_3 is configured to support it, and it then applies to the cell with the first gap. However, if its inclusion were to depend on the individual RRC configurations of the multiple potentially co-scheduled cells (e.g., if the bit is included when 'at least one' co-scheduled cell is configured for skipping), this could lead to conflicts with the 'first gap' application rule. For instance, the cell with the 'first gap' might not be the one configured for skipping, creating an inconsistency. To prevent such ambiguities and ensure predictable behavior, a clear and simple RRC configuration source for this bit in DCI format 0_3/1_3 is essential.

For DCI formats 0_3/1_3, to determine the RRC configuration for the inclusion of the 1-bit gap/restriction skipping indication, consider the following options:

- Option 1 (Scheduling Cell Parameter): The inclusion of the 1-bit indication is determined by an RRC parameter configured for the DCI format 0_3/1_3 on the scheduling cell. This parameter applies irrespective of the specific set of co-scheduled cells or their individual configurations.

- Option 2 (Co-scheduling Cell Set Parameter): The inclusion of the 1-bit indication is determined by an RRC parameter configured per set of co-scheduling cells (e.g., within an IE such as `MC-DCI-SetOfCells-r18` in `ServingCellConfig`). The 1-bit field is included in the DCI if the currently scheduled set of cells has this indication enabled via its set-specific configuration.

- Option 3 (Individual Scheduled Cell Parameter Trigger): The inclusion of the 1-bit indication is determined based on the RRC configurations of the individual cells within the co-scheduling set. The 1-bit field is included in DCI format 0_3/1_3 if at least one of or all of the cells being co-scheduled by that DCI is configured with a cell-specific RRC parameter (e.g., within an IE like `PDSCH-ConfigDCI-1-3` in `PDSCH-config`) enabling gap skipping. The application of the skipping indication, when included, still follows the agreed behavior.

Proposal 2: 7: For DCI formats 0_3/1_3, to determine the RRC configuration for the inclusion of the 1-bit gap/restriction skipping indication, consider the following options:

- Option 1: A RRC parameter configured on the scheduling cell determines the inclusion of the 1-bit indication

- Option 2 A RRC parameter configured for a 'set' of co-scheduled cells determines the inclusion of the 1-bit indication.

- Option 3: The 1-bit indication is included in the DCI if 'at least one' or ‘all’ of the co-scheduled cells has a RRC parameter configured.

minimum time offset(s) X

Based on R4-2420198, Alt 1-1 is feasible from the RAN4 perspective with X=5ms as the baseline, and X=3ms with optional UE capability. Given that the positions of the PDCCH and MG are fixed, it is more beneficial for the gNB to set this parameter rather than always using the smallest value (3 or 5 ms). To achieve this flexibility and ensure the configurability of the base station, an RRC parameter can be introduced to set X. This parameter would allow network operators to dynamically adjust the time offset based on practical circumstance and specific scenarios, with network optimization and ensuring compatibility with various UE capabilities.

Proposal 3: Introduce RRC parameter to set minimum time offset(s) X, which indicates the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding measurement gap to be skipped.

It has been agreed that one bit indication can be included in DCI format 0_3/1_3. The one bit indication works as Type 1C field, which means it only applies to one cell. However, the way to apply this one bit indication is still ambiguous in terms of minimum time offset X.

For DCI format 0_3/1_3, this ambiguity comes from the process of determining which cell the one bit indication applies to, it is necessary to check the gap/restriction of all cells. This means that even though the one bit indication is intended to apply to only one cell, all co-scheduled cells need to involves determination considering the gap/restriction of multiple cells. In this case, minimum time offset X needs to be determined for each cells.

If the value of X varies according to UE capability for each cell/FR/SCS, it may be ambiguous how to handle this in DCI format 0_3/1_3. Additionally, in the case of cross-carrier scheduling, it needs to be clarified whether the X of the scheduling cell or the X of the scheduled cell applies. In such cases, the following approaches can be considered:

- Option 1: Apply the X of the scheduling cell.

- Option 2: Apply the X of the scheduled cell.

  - Option 2-1: In the case of multi-cell scheduling, apply the largest value among those cells.

  - Option 2-2: In the case of multi-cell scheduling, apply the X of each respective cell.

For simplicity, we prefer Option 1. However, since X can vary depending on the UE's PDCCH processing and the handling of measurement gaps, it may differ based on the SCS of the scheduling cell or the FR where the MG is located.

Proposal 4: For the minimum time offset(s) X, down-select from the following options for both multi-carrier and cross-carrier scheduling scenarios:

- Option 1: Apply the X value of the scheduling cell.

- Option 2-1: Apply the largest X value among scheduled cells.

- Option 2-2: Apply individual X values for each scheduled cell.

Conclusion

In this contribution, we discussed and provided our views on the potential capacity enhancement techniques for Rel-19 XR, and the followings are proposed:

Proposal 1: In case of cross carrier scheduling, consider the following Options for configuring gap skip/cancel indication in a DCI format. The one-bit indication is included by the configuration for the DCI format of the scheduling cell.

Proposal 2: 7: For DCI formats 0_3/1_3, to determine the RRC configuration for the inclusion of the 1-bit gap/restriction skipping indication, consider the following options:

- Option 1: A RRC parameter configured on the scheduling cell determines the inclusion of the 1-bit indication

- Option 2 A RRC parameter configured for a 'set' of co-scheduled cells determines the inclusion of the 1-bit indication.

- Option 3: The 1-bit indication is included in the DCI if 'at least one' or ‘all’ of the co-scheduled cells has a RRC parameter configured.

Proposal 3: Introduce RRC parameter to set minimum time offset(s) X, which indicates the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding measurement gap to be skipped.

Proposal 4: For the minimum time offset(s) X, down-select from the following options for both multi-carrier and cross-carrier scheduling scenarios:

- Option 1: Apply the X value of the scheduling cell.

- Option 2-1: Apply the largest X value among scheduled cells.

- Option 2-2: Apply individual X values for each scheduled cell.

- References

RAN1#120 Chairman’s notes