3GPP TSG RAN WG1 #121                                                                     R1- 2504315

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.3.1

Source:	Apple Inc.

Title:	Remaining issues on SBFD TX/RX/measurement procedures

Document for:	Discussion/Decision

# Introduction

In RAN#102, a new work item was endorsed in RP- 234035, later revised in RP-241614, to specify Evolution of NR duplex operation: Sub-band full duplex (SBFD) in Rel-19, where one of the objectives is related SBFD TX/RX/measurement procedures as follows (with some arrangements):

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

In this contribution, we share our views on objectives related to SBFD TX/RX/measurement procedures.

# Remaining issues for SBFD operation at gNB

## On transient period

In Rel-19, some mechanisms (like L1-based CLI measurement and report, separate open loop uplink power control, etc) were specified aiming to identify aggressor and victim UEs, and to manage UE-to-UE CLI. However, still some additional/separate processing may be needed at UE side when operating in SBFD symbols (in comparison with operating in legacy non-SBFD symbols). Such additional processing (e.g. to suppress CLI from uplink sub-band) needs additional transient time when UE switches from non-SBFD symbols (no CLI) to SBFD symbols. Some companies suggested this transient time to be discussed under UE feature. Although eventually this transient time is to be captured as a UE capability, with some candidate values like {2, 3} symbols, but there are still some aspects that must be discussed under this agenda for RAN1 agreement. More precisely, we need to discuss whether the transient period shall be defined in non-SBFD symbols or in SBFD symbols. In our view, given that such additional processing is needed for operation in SBFD symbols, the transient period must locate at the beginning of SBFD symbols. In the example shown in Fig.1, SBFD symbols start from slot n+2, and the transient time from non-SBFD to SBFD is shown by yellow. The candidate values can be {1, 2, 3} symbols in the SCS of TDD band.

Proposal 1: Support transient time at the beginning of SBFD duration, with the candidate values for transient time are {1, 2, 3} symbols in TDD band numerology.

Figure 1: Transient time between non-SBFD and SBFD symbols.

## UL power control

In RAN1#120b, there was some discussion to support separate configurations of PCMAX, or p-max for SBFD and non-SBFD symbols. The idea is mainly to manage UE-to-UE CLI by limiting the maximum output power in SBFD symbols. However, we should note that for uplink transmissions (PUSCH/PUCCH/SRS/PRACH) in SBFD and non-SBFD symbols, different open-loop power control parameters were already agreed.

Proposal 2: Do not support separate configurations of PCMAX, or p-max for SBFD and non-SBFD symbols

In RAN1#120, there was some discussion to support separate PHR report for SBFD and non-SBFD, in a single MAC-CE report. Multiple reports in a single MAC-CE for the case of SBFD and non-SBFD is motivated similar to multiple reports for CA, and mTRP operation, where justification is due to different uplink power control parameters associated to different reports.

Proposal 3: Support separate PHR reports associated to SBFD and non-SBFD, when separate UL power control parameters for SBFD symbols and non-SBFD symbol is indicated through the same unified TCI state.

## More on resource configurations for SBFD operation

For uplink transmission (or downlink reception) with Configuration 2, where different occasions may span different symbol types, some agreements were achieved mainly on how to determine frequency domain resource allocation. In general, for a CG-PUSCH in Configuration 2, where different repetitions span different slot types (non-SBFD vs SBFD), different CG-PUSCH parameters may be desired. This is due to the facts that UE may follow different power control parameters due to different interference profile in SBFD and non-SBFD slots, and/or gNB may experience self-interference, wider beam indication etc for SBFD slots vs non-SBFD slots. In Fig. 2, an example is shown where CG PUSCH is configured with RRC parameter periodicity as 2 slots (sym2x14). In this example, MCS corresponding to those occasions within legacy non-SBFD slots (CG_PUSCH 1 and CG_PUSCH_3) is different from the MCS for occasions that happen in a SBFD slot (CG_PUSCH_2).

Figure 2: CG-PUSCH configuration with different MCS configurations

Proposal 4: For a type-1 CG-PUSCH and Configuration 2, ConfiguredGrantConfig may include a pair of parameters for MCS, (mcsAndTBS, mcsAndTBS2) where the second value indicates the parameter for CG-PUSCH transmission in SBFD slot.

Similarly, for Configuration 2 and type-2 CG PUSCH or SPS-PDSCH, or multi-grant PUSCH/PDSCH, the MCS in the activation DCI can be applied to non-SBFD occasions, while a RRC configured offset MCS in respect to the indicated MCS by the activation DCI can be applied to grants in SBFD occasions.

Proposal 5: For Configuration 2 and type-2 CG PUSCH or SPS-PDSCH, or multi-grant PUSCH/PDSCH, DCI indicates a MCS which is applied to non-SBFD occasions, while a RRC configured offset to the MCS indicated by DCI is applied to SBFD occasions.

In RAN1#119, there has been discussions about interaction between cell-specific indication of SBFD symbol configuration, and legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated. More precisely, if a D of F symbol in TDD-UL-DL-ConfigCommon is turned to SBFD symbol, what should be UE behavior on that symbol indication via UE specific TDD-UL-DL-ConfigDedicated.

Proposal 6: A SBFD-aware UE does not expect a SBFD symbol to be configured as UL or DL symbol in TDD slot configuration via TDD-UL-DL-ConfigDedicated.

# Conclusion

In this contribution, we provided our views on remaining issues related to UE TX/RX/measurement procedures for SBFD operation at the gNB. Based on what we discussed, the following proposals are made:

Proposal 1: Support transient time at the beginning of SBFD duration, with the candidate values for transient time are {1, 2, 3} symbols in TDD band numerology.

Proposal 2: Do not support separate configurations of PCMAX, or p-max for SBFD and non-SBFD symbols

Proposal 3: Support separate PHR reports associated to SBFD and non-SBFD, when separate UL power control parameters for SBFD symbols and non-SBFD symbol is indicated through the same unified TCI state.

Proposal 4: For a type-1 CG-PUSCH and Configuration 2, ConfiguredGrantConfig may include a pair of parameters for MCS, (mcsAndTBS, mcsAndTBS2) where the second value indicates the parameter for CG-PUSCH transmission in SBFD slot.

Proposal 5: For Configuration 2 and type-2 CG PUSCH or SPS-PDSCH, or multi-grant PUSCH/PDSCH, DCI indicates a MCS which is applied to non-SBFD occasions, while a RRC configured offset to the MCS indicated by DCI is applied to SBFD occasions.

Proposal 6: A SBFD-aware UE does not expect a SBFD symbol to be configured as UL or DL symbol in TDD slot configuration via TDD-UL-DL-ConfigDedicated.