3GPP TSG RAN WG1 #121                                                                                   R1-2504337

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.7.2

Source:	Apple Inc.

Title:	Discussion on ISAC channel modelling

Document for:	Discussion/Decision

# Introduction

A new study item on Channel Modelling for Integrated Sensing and Communications was initiated in RAN #112 with the following objectives and update from RAN #105 [5].

In this contribution, we discuss details on ISAC channel modelling for this SI.

# Discussion

## Physical Object Modeling

For target modeling, issues such as Radar Cross Section, Channel Polarization Matrix, and single/multiple scattering point modeling are addressed.

### Radar Cross Section (RCS) considerations:

RCS needs to be characterized for the relevant sensing targets/scenarios. Different sensing targets such as humans, vehicles, or AGV/AMR (smart factory) may have different RCS values.

#### Monostatic RCS based on Measurements and Electromagnetic Simulations

We show the RCS based on actual measurements of a human phantom model as shown in Figure 1. In this case, we show the result for both the HH and HV polarizations in Figure 2.

Figure 1: RCS Measurement Setup

Figure 2: Cross Polarization Measurement Results

The diagrams in Figure 3 and Figure 4 show the measurements across all incident angles and frequencies illustrates the dependency on the monostatic incident angle.

| Frontside/Backside: Mean = 3.53 | Armside: Mean = -3.2 |
| --- | --- |

Figure 3: HH Measurements across incident angle and frequency for mono-static sensing

Figure 4: HV Measurements across incident angle and frequency

As can be seen from the diagrams, the front-side and back-side of the dummy may have a slightly higher RCS. This may be accounted for in the model.

The figure below shows the RCS for the human averaged across frequency in 15-degree increments and an associate model with angular continuity across the azimuth angles.

Figure 5: HH Measurements across incident angle averaged over frequency for Monostatic RCS

Based on measurements, the values of A and B for co-polarized and cross-polarized RCS may be selected from the following

Omni-directional Measurement Results

| Frequency | Mean = A | Std = B2 |
| --- | --- | --- |
| HH | -3.10 | 4.7 |
| HV | -22.84 | 4.42 |

Directional Measurement Results

| Frequency | Mean = A |
| --- | --- |
| HH (frontside/back-side) | 3.53 |
| HH (Armside) | -3.2 |

Note that the results are averaged across all incident angles at all frequencies

On the monostatic RCS of a human with a single scattering point,

- The monostatic RCS for a scattering point of the target is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

- The values/pattern of component A*B1 are generated by the following parameters:

|  | φcenter | φ3dB,k | θcenter | θ3dB,k | Gmax | σmax | Applicable Range of φ | Applicable Range of θ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Back | 90° | 60 | 90° | - | 3.53 | 3.53 + 3.2 | 90° | (135°,225°) |
| Front | 270° | 60 | 90° | - | 3.53 | 3.53 + 3.2 | 90° | (-45°, 45°) |

Based on offline discussions and averaging with multiple companies, the following is our harmonized proposal:

Proposal 1: The Monostatic RCS for human with RCS model 2 is dependent on both elevation angle and horizontal angle

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

- The values/pattern of component A*B1 are generated by the following parameters :

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [-90, 90] |
| Back | 180 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [90,270] |

- The standard deviation of component B2 is 3.94 dB

#### Bi-static RCS based on Electromagnetic Simulations

In RAN1 #119, the following agreement was made [16]:

Agreement Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction.

In RAN1 #120-bis, the following working assumption was made:

Working assumption For vehicle with single/multiple scattering points, the bistatic RCS is generated by - The values/pattern of A*B1 of bistatic RCS is given by: rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs where σVdBθ=-min12θ-θcenterθ3dB2, σmax σHdB φ=-min12φ-φcenterφ3dB2, σmax - Attenuatefactor=k1×sin(k2×β2) is applied to the β within 0~180 degrees. k1= 6 and k2=1.65. β is the absolute bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs). - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively. - FFS: RCS value when β is 180 degrees - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19 - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs - FFS: how to avoid angular discontinuity - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity. - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle - the following formula can be a reference for the study Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax)) Note: the working assumption agreed on Thursday was updated on Friday as follows: k1= 6 and k2=1.65

We characterize the RCS of a human in the bi-static sensing scenario using an electromagnetic simulation. We assume the bi-static angle is  about the axis of the human as shown in Figure 6 below:

Figure 6: Bi-static RCS angle

This results in RCS values as shown in Figure 7 and Figure 8 below:

Figure 7: Mean Value of RCS for Bi-static Sensing from Electromagnetic Simulations

Figure 8: Standard Deviation of RCS for bi-static case

Figure 9 below shows the equivalent results for a measurement campaign for angles [0,10, 20, 90]. The results show the same trends as the electromagnetic simulation.

Figure 9: Mean Value of RCS for Bi-static Sensing from actual measurements

The following observations ana proposals on the models can be made:

Observation 1:  With bistatic scattering RCS decreases as angular separation increases, until TX and RX approach the LOS path in which case we see an increase due to forward scattering. This is seen in both electromagnetic simulations and in actual measurements.

Observation 2: The Forward scattering/diffraction region starts at about 70 degrees. Note that in R19, it has been agreed not to model the forward scattering region.

Proposal 2: For the model for the RCS in the bi-static case:

- For an incident direction, the bistatic RCS in the backscatter direction is equal to monostatic RCS

- For human with RCS model 1, the value of component A of the bistatic RCS is the same as that of the monostatic RCS

  - A is the mean of monostatic RCS values in all 3D incident/scattered angles, and is applied to both monostatic and bistatic RCS

Proposal 3: The bistatic RCS of Human with RCS model 1 is modelled as

- The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxA-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

  - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1

  - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

  - The AttenuateFactor can be modelled as a linear or approximately linear function

Proposal 4: The bistatic RCS of Human with RCS model 2 is modelled using the bi-static RCS for vehicle formula with the following updates:

    - k1=0.5714 and k2=0.1

  - The values of k1 and k2 map to an approximately linear function

### Single or Multiple Scattering Points:

In RAN1 #116-bis, the following agreement was made [11]:

Agreement - In the target channel between Tx and Rx, scattering of a sensing target can be modelled as single scattering point or multiple scattering points - FFS one or multiple incoming/output rays corresponding to a scattering point - FFS how to select single or multiple scattering points for the target, e.g. depending on the distance between target and Tx/Rx, size/shape of target, etc. - Note: the sensing target can be assumed in far field of sensing Tx/Rx. - FFS details to model the single or multiple scattering points

In RAN1 #120 the following agreements were made [17]:

Agreement If a target is modelled with multiple scattering points, - The number of scattering points of the target is generated in the beginning of the simulation and kept unchanged in the whole simulation - The number and locations of the scattering points of the target (if it is a vehicle) are common to each pair of sensing Tx/Rx - RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target - RCS values of the multiple scattering points are individually determined Agreement To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point. Agreement - The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated - The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated Agreement For a target with single/multiple scattering points, the 3D location of each scattering point is defined in the evaluation assumptions.

#### Selection of single or multiple scattering points

To decide between modeling a target with single or multiple scattering points depends on the target type, the use case and deployment scenario and evaluation methodology. To limit the complexity during a SLS, a single scattering point, set with appropriate parameters may be used to represent a target. This can be used as the default model. For use cases and scenarios where there may be a need to differentiate between target types, to track the movement of a target, or in LLS, the target may be modeled as a multiple scattering points e.g. in the shape of the target.

If a target is modelled with multiple scattering points, RAN1 assumes the relative locations of the multiple scattering points of a target are not changed. Translational motion of the target can be modelled but rotational motion of the target should not be considered in Rel-19. Also, we should assume that there is no interaction between the scattering points within a target.

Note the multiple scattering points for a target should also have the same speed/direction for a mobile target.

Proposal 5: To decide between modeling a target with single or multiple scattering points depends on the target type/size, the distance between sensing Tx/Rx and the target, the use case and deployment scenario and evaluation methodology

Proposal 6:

- A single scattering point with appropriate parameters may be used to represent a target in the SLS as default.

- Multiple scattering points may be used in an LLS or for use cases and scenarios where there may be a need to differentiate between target types or to track the movement of a target

  - FFS: the abstraction method from the multi-point model to the single point model.

Proposal 7: If a target is modelled with multiple scattering points,

- RAN1 assumes the relative locations of the multiple scattering points of a target are not changed

  - Translational motion of the target can be modelled while rotational motion of the target should not be considered.

- RAN1 assumes no rays are scattered from one scattering point to another scattering point of the same target

## Target Channel Modeling

### General Target Channel Modeling

#### Multiple targets in a pair of Tx/Rx

In RAN1 #117, the following agreement was made [13]:

#### Agreement - Multiple sensing targets can be modelled in the ISAC channel of a pair of sensing Tx and sensing Rx - FFS whether to model a propagation path from Tx to Rx interacting with more than one sensing target - The same sensing target can be modelled in the ISAC channels of multiple pairs of sensing Tx and Rx

To address the FFS, RAN1 should limit interaction between more than one sensing target to limit the complexity of system level simulations subject to validation by measurements.

Proposal 8: RAN1 should limit interaction between more than one sensing target to limit the complexity of system level simulations subject to validation by measurements.

### Path Dropping

In RAN1 119, the following agreement was made [16]:

Agreement - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

To study the power threshold for path dropping, we use the following assumptions:

- Human scenario Outdoor

- Channel (UMi)

  - Tx-Target channel: Umi

  - Target-Rx channel: 38.858, Umi based

- Mode: TRP-UE bi-static

- Mobility: 0km/h

- Carrier frequency: 3.5GHz

- FFT size: 4096

- SCS: 30KHz

- RCS value: - 2 dBsm

To investigate the power threshold for path dropping we plot the CDFs of the delay spread (Figure 12), the RSRP (Figure 13) and the channel modeling complexity captured by the average number of channel taps (Figure 14) for different thresholds compared with the full convolution option.

Observation 3: A power threshold of 30 dB shows little change in delay spread and RSRP when compared with full concatenation with a substantial drop in complexity (68.997 taps vs. 123 taps on average) vs 20 dB.

Observation 4: A power threshold of 30 dB shows little RSRP difference with and without normalization.

Proposal 9: Any indirect path with power metric less than 30 dB is dropped

Figure 12: Effect of dropping threshold on delay spread

Figure 13: Effect of Dropping Threshold on RSRP

Figure 14: Effect of Dropping Threshold on Complexity (Number of Taps)

## Spatial Consistency

Some use cases may require spatial/temporal consistency modeling. For example, let us assume a case where multiple devices cooperate to sense a single target. If the sensing channels of the two devices are generated independently, the cooperative sensing performance may be incorrectly estimated. If two devices are physically close together or a device or a target moves in limited space, their channel generation may also correlate. It is necessary to discuss how we can make a correlation in LSPs and/or SSPs generation between spatial/temporal displacements.

Proposal 10: For the component B2,

- The component B2 of two different targets are generated independently.

- If Tx, target, Rx positions do not change during a simulation, the RCS component B2 of different incident/scattered angles for a target are generated independently

Proposal 11: The initial random phase  and XPR of two different targets are generated independently.

Proposal 12: For evaluations where there is movement, model 2 (angular dependent RCS) is used to maintain the spatial consistency.

- B1 captures the changes based on any bi-static angle change in a pre-determined manner.

- Parameter B2 can either be kept constant or changed based on a coherent time/distance.

## LLS

For the link level simulation, we have the following proposals:

Proposal 13: The LLS is needed in the case of studies on signal processing methods e.g. waveform

- It should be based on the existing CDL channel model

- To model an ISAC channel, we add one or more clusters representing the target(s). The RCS is added to the sensing target cluster.

- Note there may need to be a discussion on the CDL parameters for the monostatic background channel

## Miscellaneous Issues

### Doppler

Agreement - Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula, rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft Where, - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point - rtx,n,mT is the spherical unit vector at transmitter for the link from Tx to the scattering point - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0 - ft is only applicable for indirect path - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray - Support two terms of 2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray - Doppler is separately determined for each of the multiple scattering points of a target - vspt can include macro-Doppler and/or micro-Doppler motion, vspt=vmacrot+vmicro,pt - FFS: maximum speed of moving scatterers - FFS: ratio of moving scatterers among all scatterers

Proposal 14. To address the issues that are FFS:

- Dual mobility model in 7.6.10, TR 38.901 is reused to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

- FFS1: the maximum speed of moving scatterers should not exceed maximum speed of target, Tx or Rx

- FFS2: the ratio of moving scatterers among all scatterers depends on evaluation goal.

# Conclusion

In the contribution, the following observations and proposals are made:

Proposal 1: The Monostatic RCS for human with RCS model 2 is dependent on both elevation angle and horizontal angle

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

- The values/pattern of component A*B1 are generated by the following parameters :

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [-90, 90] |
| Back | 180 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [90,270] |

- The standard deviation of component B2 is 3.94 dB

Observation 1:  With bistatic scattering RCS decreases as angular separation increases, until TX and RX approach the LOS path in which case we see an increase due to forward scattering. This is seen in both electromagnetic simulations and in actual measurements.

Observation 2: The Forward scattering/diffraction region starts at about 70 degrees. Note that in R19, it has been agreed not to model the forward scattering region.

Proposal 2: For the model for the RCS in the bi-static case:

- For an incident direction, the bistatic RCS in the backscatter direction is equal to monostatic RCS

- For human with RCS model 1, the value of component A of the bistatic RCS is the same as that of the monostatic RCS

  - A is the mean of monostatic RCS values in all 3D incident/scattered angles, and is applied to both monostatic and bistatic RCS

Proposal 3: The bistatic RCS of Human with RCS model 1 is modelled as

- The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxA-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

  - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1

  - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

  - The AttenuateFactor can be modelled as a linear or approximately linear function

Proposal 4: The bistatic RCS of Human with RCS model 2 is modelled using the bi-static RCS for vehicle formula with the following updates:

    - k1=0.5714 and k2=0.1

  - The values of k1 and k2 map to an approximately linear function

Proposal 5: To decide between modeling a target with single or multiple scattering points depends on the target type/size, the distance between sensing Tx/Rx and the target, the use case and deployment scenario and evaluation methodology

Proposal 6:

- A single scattering point with appropriate parameters may be used to represent a target in the SLS as default.

- Multiple scattering points may be used in an LLS or for use cases and scenarios where there may be a need to differentiate between target types or to track the movement of a target

  - FFS: the abstraction method from the multi-point model to the single point model.

Proposal 7: If a target is modelled with multiple scattering points,

- RAN1 assumes the relative locations of the multiple scattering points of a target are not changed

  - Translational motion of the target can be modelled while rotational motion of the target should not be considered.

- RAN1 assumes no rays are scattered from one scattering point to another scattering point of the same target

Proposal 8: RAN1 should limit interaction between more than one sensing target to limit the complexity of system level simulations subject to validation by measurements.

Observation 3: A power threshold of 30 dB shows little change in delay spread and RSRP when compared with full concatenation with a substantial drop in complexity (68.997 taps vs. 123 taps on average) vs 20 dB.

Observation 4: A power threshold of 30 dB shows little RSRP difference with and without normalization.

Proposal 9: Any indirect path with power metric less than 30 dB is dropped

Proposal 10: For the component B2,

- The component B2 of two different targets are generated independently.

- If Tx, target, Rx positions do not change during a simulation, the RCS component B2 of different incident/scattered angles for a target are generated independently

Proposal 11: The initial random phase  and XPR of two different targets are generated independently.

Proposal 12: For evaluations where there is movement, model 2 (angular dependent RCS) is used to maintain the spatial consistency.

- B1 captures the changes based on any bi-static angle change in a pre-determined manner.

- Parameter B2 can either be kept constant or changed based on a coherent time/distance.

Proposal 13: The LLS is needed in the case of studies on signal processing methods e.g. waveform

- It should be based on the existing CDL channel model

- To model an ISAC channel, we add one or more clusters representing the target(s). The RCS is added to the sensing target cluster.

- Note there may need to be a discussion on the CDL parameters for the monostatic background channel

Proposal 14. To address the issues that are FFS:

- Dual mobility model in 7.6.10, TR 38.901 is reused to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

- FFS1: the maximum speed of moving scatterers should not exceed maximum speed of target, Tx or Rx

- FFS2: the ratio of moving scatterers among all scatterers depends on evaluation goal.

# References

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)

- 3GPP TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles (Release 15)

- 3GPP TR 38.855, Study on NR positioning support (Release 16)

- 3GPP TR 38.802, Study on New Radio Access Technology, Physical Layer Aspects, (Release 14)

- RP-242348, Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Xiaomi, AT&T

- 3GPP TS 22.137: Service requirements for Integrated Sensing and Communication, Stage 1, (Release 19)

- 3GPP TR 22.837: Feasibility Study on Integrated Sensing and Communication (Release 19)

- R1-2401495, Summary #1 on ISAC channel modelling, RAN1 #116, Xiaomi

- R1-xxxx, Summary on ISAC channel modeling, RAN1 #116-bis, Xiaomi

- Chairman’s Notes, RAN1 #116, February 2024

- Chairmans’s Notes, RAN1 #116-bis, April 2024

- Skolnick, M.I., Introduction to Radar Systems, McGraw-Hill, 1980

- Chairman’s Notes, RAN1 #117, May 2024

- Chairman’s Notes, RAN1 #118, August 2024

- Chairman’s Notes, RAN1 #118-bis, October 2024

- Chairman’s Notes, RAN1 #119, November 2024

- Chairman’s Notes, RAN1 #120, February 2024

- Chairman’s Notes, RAN1 #120-bis, February 2024