3GPP TSG RAN WG1 #121		                                                                                                                                                                                                 	R1- 2504354

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.3

Source: 	Apple Inc.

Title: 	Views on UE features for Rel-19 SBFD

Document for:	Discussion/Decision

- Views on UE features for Rel-19 NR duplex operation: SBFD

In this contribution, we present our high-level views on UE features for Rel-19 SBFD, based on the progress achieved in previous meeting. The FGs are organized in the following way to handle different objectives/agendas in NR Rel-19 NR SBFD.

- FG 60-1x covers SBFD TX/RX/measurement procedures

- FG 60-2x covers SBFD random access operation

- FG 60-3x covers CLI handling

## SBFD TX/RX/measurement procedures

## Discussion

Proposal 1: For the Basic feature for SBFD-aware UE, the following components shall be included

- Support of DUD and/or DU

  - Candidate values can be {DUD, DU, both}

- Additional transient time from non-SBFD symbol to SBFD symbol for DL reception

  - Candidate values can be {1, 2, 3} symbols in TDD SCS

- A starting symbol index for SBFD symbols

  - Candidate values can be {0, 2, 7}

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | [Per Band] | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1a | Support of SBFD ware UE in RRC connected | - Support of single shot SBFD Tx in SBFD slot - Support of single shot SBFD Rx in SBFD - Support of SBFD measurement in SBFD slot |  | Yes | n/a | SBFD Tx/Rx/measurement is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-1 | Transition gap between non-SBFD symbols and SBFD symbols and vice versa | 1. number of symbols to switch from non-SBFD symbol to SBFD symbol 2. number of symbols to switch from SBFD symbol to non-SBFD symbol 3. number of symbols to switch from UL sub-band to DL sub-band 4. number of symbols to switch from DL sub-band to UL sub-band |  | Yes | n/a |  | Per band | n/a | n/a | n/a | Component 1 candidate values in number of symbols {0, 2, 7, 14, 28} symbols Component 2 candidate values in number of symbols {0, 2, 7, 14, 28} symbols | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-2 | Number of DL sub-bands in SBFD slot | 1. number of DL sub-bands in SBFD slot |  | Yes | n/a | UE cannot be scheduled/configured to receive across two DL sub-bands | Per band | n/a | n/a | n/a | Component 1 candidate values are {1, 2} | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-3 | FDRA Type 0 in SBFD slot | 1. FDRA Type 0 for PDSCH 2. FDRA Type 0 for PUSCH | 5-1 | Yes | n/a | UE cannot be scheduled with Type 0 PXSCH in SBFD | Per band | n/a | n/a | n/a | Note: SBFD aware UE must support at least one FDRA type | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-3a | # of partial PRG in SBFD | 1. Number of partial PRG for DL reception in SBFD slot |  | Yes | n/a | UE cannot be scheduled more than 2 partial PRG | Per band | n/a | n/a | n/a | Candidate values are {2,3,4} | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-4 | FDRA Type 1 in SBFD slot | 1. FDRA Type 1 for PDSCH 2. FDRA Type 1 for PUSCH | 5-1 | Yes | n/a | UE cannot be scheduled with Type 1 PXSCH in SBFD | Per band | n/a | n/a | n/a | Note: SBFD aware UE must support at least one FDRA type | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-5 | Frequency hopping in SBFD slot | 1. intra-slot frequency hopping in SBFD slot 2. Inter-slot frequency hopping in SBFD slots 3. Inter-slot frequency hopping between SBFD and non-SBFD slots |  | Yes | n/a | UE ignores FH flag for PUSCHs in SBFD | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-5a | RB-Offset for PUSCH in SBFD | 1. Applying RB-Offset for PUSCH in SBFD symbols |  | Yes | n/a | UE assumes RB-Offset =0 | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-6 | Separate freq. config for PUCCH-Resource | 1. Support separate configurations of startingPRB 2. Support separate configurations of secondHopPRB |  | Yes | n/a | No separate PUCCH-Resource config is supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-7 | Separate SRS-ResourceSets | 1. Support two SRS-Resource set with usage Codebook for SBFD and non-SBFD transmissions 2. Support two SRS-Resource set with usage nonCodebook for SBFD and non-SBFD transmissions |  | Yes | n/a | No separate SRS-ResourceSets is supported | Per band | n/a | n/a | n/a | Note: if UE supports separate SRS-ResourceSets for Codebook, the two sets have same number of SRS resources Note: if UE supports separate SRS-ResourceSets for nonCodebook, the two sets have same number of SRS antenna ports | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-8 | Support of Configuration 2 for DL reception | 1. Support Configuration 2 for PDSCH with repetitions 2. Support Configuration 2 for SPS PDSCH 3. Support Configuration 2 for multi-grant PDSCH 4. Support Configuration 2 for CSI-RS measurement |  | Yes | n/a | No support of Configuration 2 for DL reception | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-9 | Support of Configuration 2 for UL reception | 1. Support Configuration 2 for PUSCH with repetitions Type A 2. Support Configuration 2 for PUSCH with repetitions Type B 3. Support Configuration 2 for CG-PUSCH (Type1 or 2) 4. Support Configuration 2 for multi-grant PUSCH 5. Support Configuration 2 for P/SP-PUCCH |  | Yes | n/a | No support of Configuration 2 for UL transmission | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-10 | CSI-RS processing across two DL subbands | 1. CSI processing timeline and CPU counting for CSI processing across DL sub-bands | 60-2 | Yes | n/a | CSI processing timeline Z/Z’ is scaled by factor of 2 | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-11 | Separate UL power control for SBFD and non-SBFD | 1. Support new P0AlphaSets in Uplink-powerControl for PUSCH 2. Support new P0AlphaSets in Uplink-powerControl for PUCCH 3. Support new P0AlphaSets in Uplink-powerControl for SRS |  | Yes | n/a | Same/single P0AlphaSets is applied to both SBFD and non-SBFD | Per band | n/a | n/a | n/a |  | Optional with capability signalling |

## SBFD random access operation

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-3 | RACH configuration Option 1 (Use one single RACH configuration and based on the existing parameters of the single RACH configuration) | - Support additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon - Support of reinterpretation of frequency offset of lowest RO - Support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period - Support PRACH transmission in additional-ROs for SBFD-aware UEs in RRC_CONNECTED state and RRC_IDLE/INACTIVE state | 60-1 | Yes | n/a | Preamble repetition for SBFD RA operation with RACH configuration Option 1 is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-4 | RACH configuration Option 2 (Use two separate RACH configurations, including one legacy and one additional RACH configuration) | - Support additional-ROs in non-SBFD symbols configured by additional RACH configuration - Support a valid RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period - Support PRACH transmission in additional-ROs for SBFD-aware UEs in RRC_CONNECTED state and RRC_IDLE/INACTIVE state | 60-1 | Yes | n/a | Preamble repetition for SBFD RA operation with RACH configuration Option 2 is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-5 | Preamble repetition for SBFD RA operation with RACH configuration Option1 | - Determination of the set of valid additional-ROs and the time period of the set(s) of valid additional-ROs - Support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 54-1, 60-3 | Yes | n/a |  | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-5a | Preamble repetition for SBFD RA operation with RACH configuration Option1 | - Determination of the set of valid additional-ROs and the time period of the set(s) of valid additional-ROs - Support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs | 54-1, 60-4 | Yes | n/a |  | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-6 | Msg.3 repetition in SBFD symbols | - Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols | 30-6, 60-3 or 60-4 | Yes | n/a | Msg.3 repetition in SBFD symbols is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |

## CLI handling

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-3-1 | Support of L1 based CLI measurement method within the active DL BWP | - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband |  | Yes | n/a | L1 based CLI measurement and report is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-3-2 | Support of CLI-RSSI measurement resource type | - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands |  | Yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-3-3 | Support of L1 CLI-RSSI and L1-SRS-RSRP measurement | - The max number of L1-CLI-RSSI/L1-SRS-RSRP resources (sum of aperiodic/periodic/semipersistent) across all CCs configured to measure L1-CLI-RSSI or L1-SRS-RSRP within a slot shall not exceed M - The max number of aperiodic L1-CLI-RSSI resources across all CCs configured to measure CLI-RSSI shall not exceed MA_1 - The max number of aperiodic L1-SRS-RSRP resources across all CCs configured to measure SRS-RSRP shall not exceed MB_1 |  | Yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-3-4 | Support of non-transparent UL resource muting | - CP-OFDM - DFT-S-OFDM |  | Yes | n/a | Non-transparent UL resource muting is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |