3GPP TSG RAN WG1 #121                       		R1-2504367

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.7.1

Source:	AT&T, FirstNet

Title:	ISAC scenarios and 7-24GHz alignment

Document for:	Discussion/Decision

# Introduction

A study item on channel modeling for Integrated Sensing and Communication (ISAC) was initially agreed in RAN#102 [1] and later updated at RAN#105 [2]. This study item will solely focus on defining a common channel modeling framework to enable wireless sensing in 5G-Advanced and will aim to support object detection and tracking use cases defined in SA1 [3][4]. For those use cases, the following objectives are identified for this study:

To develop a common framework for channel modeling, including target modeling, it is essential to further clarify the corresponding scenarios that are beneficial from a wireless network deployment perspective. In this contribution, we discuss the further updates to the deployment scenarios that are of importance for enabling ISAC in 5G-Advanced (Rel-19) for some essential sensing targets for object detection and/or tracking including UAVs, Automotive targets, and humans to meet the needs of commercial, public safety, and other verticals.

# ISAC deployment scenarios

As a starting point, below are the current RAN1 agreements for the ISAC deployment scenarios for each of the sensing targets from RAN1#119 [5] and prior:

RAN1#119 Agreements

Agreement

For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118 as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-UAV

Details on ISAC-UAV scenarios are listed in Table x.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. NOTE1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. NOTE 3: time-varying velocity may be considered for future evaluations. |
|  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions N = {1, 2, 3, 4, 5} NOTE4: N=0 may be considered for the evaluation of false alarm Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. NOTE5: target(s) are outside the minimum distance to the Tx/Rx |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s) |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. NOTE5: the sensing target is assumed in the far field of sensing Tx/Rx Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  | FFS |

NOTE: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Automotive sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Automotive

Details on ISAC-Automotive scenarios are listed in Table x.

Table x. Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Values |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid. NOTE1 |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Additional option: ISD between TRPs of Urban Grid is 250m |
| Sensing target | LOS/NLOS | LOS and NLOS (including NLOSv) |
|  | Outdoor/indoor | Outdoor |
|  | Mobility (horizontal plane only) | Based on TR37.885 mobility for urban grid or highway scenario |
|  | Distribution (horizontal) | Based on dropping in TR37.885 per urban grid or highway communication scenario |
|  | Orientation | Lane direction in horizontal plane |
|  | Physical characteristics (e.g., size) | Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1: calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Human (indoor and outdoor) sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Human

Details on ISAC-Human scenarios are listed in Table x.

Table x. Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Indoor office, indoor factory [TR38.901] Indoor room [TR38.808] | UMi, Uma, RMa [TR38.901] |
| Sensing transmitters and receivers properties | Rx/Tx Locations NOTE 2 | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario |
|  | Rx/Tx Mobility for UEs | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr |
| Sensing target | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory) | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory) |
|  | 3D distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment FFS: Value of N NOTE1: N=0 may be considered for the evaluation of false alarm | Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. Uniform in horizontal plane NOTE1: N=0 may be considered for the evaluation of false alarm |
|  | Orientation | Random over the horizontal area | Random over the horizontal area |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859as a starting point NOTE2: the sensing target is assumed in the far field of sensing Tx/Rx | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859 as a starting point NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS, based on outcome for AI 9.7.2 | FFS, based on outcome for AI 9.7.2 |

NOTE1: For the human (indoor and outdoor) sensing targets, additional communication scenarios can be considered for future evaluations. Channel model calibration for Urban Grid with outdoor humans is expected to be performed from Objects creating hazards on the road/railway sensing scenarios.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For AGV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-AGV

Details on ISAC-AGV are listed in Table x.

Table x. Evaluation parameters for Automated Guided Vehicles

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | InF (TR38.901 including Table 7.8-7) |
| Sensing transmitters and receivers properties NOTE2 |  | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities [3, 10] km/h |
|  | 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane |
|  | Orientation | Horizontal plane only |
|  | Physical characteristics (e.g., size) | Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: Min distances based on min. TRP/UE distances defined in TR38.901 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option A: At least larger than the physical size of a target Option B: Fixed value, [x] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS |

NOTE1: For the AGV sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

NOTE3: RAN1 can further discuss narrowing down the number of sub-scenarios of InF

Agreement

For Objects creating hazards sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Hazards

Details on ISAC-Hazards are listed in Table x.

Table x. Evaluation parameters for objects creating hazards

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Highway, Urban grid, HST (High Speed Train) |
| Sensing transmitters and receivers properties NOTE2 | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. FFS: Option 2 Additional option ISD between TRPs of Urban Grid is 250 m |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: up to [10] km/h for humans and animals FFS: Additional velocities, trajectory |
|  | 3D distribution | Uniformly distributed in horizontal plane |
|  | Orientation | Random distribution in horizontal plane |
|  | Physical characteristics (e.g., size) | For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m FFS: other types/sizes of targets may be considered for future evaluations |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: based on min TRP/UE distances defined in TR37.885 and TR38.802 and TR36.843 and TR38.859 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1: For the objects creating hazards sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

At RAN1#119 and prior RAN1 meetings, good progress was made with the agreements endorsed in the ISAC session report [5] regarding specific scenarios and evaluation parameters for the UAV, Automotive, Human, AGV and Objects creating hazards sensing targets scenarios. However, there remain some open issues with the deployment scenarios for ISAC for some of the specific parameters for various sensing targets.

One example of this is “applicable communication scenarios” where updates have been agreed to be included in the 7-24GHz channel modeling study item in Rel. 19. The Rel. 19 7-24GHz channel modeling item, in addition to validating the channel models in TR38.901 for the frequency range 7-24GHz, agreed to introduce a new channel model for suburban macro scenarios (SMa), reflective of deployments in north America in particular. The SMa scenario captures unique deployment parameters that are not modeled by UMa and RMa such as ISD, BS heights, penetration loss, LoS probability, etc. For some of the outdoor sensing target scenarios, the SMa scenario should be reflected in the evaluation parameter tables, as an applicable communication scenario.

Additionally, the 38.901 running CR should reflect the TR section number as references rather than simply a reference to itself (38.901). As such, specification section numbers are proposed to be added as references where applicable.

Observation 1: Applicable communications scenarios for ISAC channel modelling does not account for new communications scenarios defined in the 7-24GHz agenda item.  Additionally, some references for the scenarios are incomplete.

Proposal 1: For ISAC evaluations, RAN1 considers the updates made in Rel. 19 SI on channel modeling for 7-24GHz as needed and applicable.

Proposal 2: RAN1 includes the following updates to the applicable communication scenario rows for Evaluation parameters for UAV sensing scenarios as shown below in Table x:

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters | Value |
| --- | --- |
| Applicable communication scenarios | UMi, UMa, RMa, SMa (see clause 7.2) UMi-AV, UMa-AV, RMa-AV [36.777] |

Proposal 3: RAN1 includes the following updates to the applicable communication scenario rows for Evaluation parameters for Human (indoor and outdoor) sensing scenarios as shown below in Table y:

Table y. Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters | Indoor Values | Outdoor Values |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 | Indoor office, indoor factory (see clause 7.2) Indoor room [TR38.808] | UMi, UMa, RMa, SMa (see clause 7.2) |

# Conclusion

In this contribution, we discussed the updating the deployment scenarios for the ISAC channel model. We made the following observations and proposals:

Observation 1: Applicable communications scenarios for ISAC channel modelling does not account for new communications scenarios defined in the 7-24GHz agenda item.  Additionally, some references for the scenarios are incomplete.

Proposal 1: For ISAC evaluations, RAN1 considers the updates made in Rel. 19 SI on channel modeling for 7-24GHz as needed and applicable.

Proposal 2: RAN1 includes the following updates to the applicable communication scenario rows for Evaluation parameters for UAV sensing scenarios as shown below in Table x:

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters | Value |
| --- | --- |
| Applicable communication scenarios | UMi, UMa, RMa, SMa (see clause 7.2) UMi-AV, UMa-AV, RMa-AV [36.777] |

Proposal 3: RAN1 includes the following updates to the applicable communication scenario rows for Evaluation parameters for Human (indoor and outdoor) sensing scenarios as shown below in Table y:

Table y. Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters | Indoor Values | Outdoor Values |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 | Indoor office, indoor factory (see clause 7.2) Indoor room [TR38.808] | UMi, UMa, RMa, SMa (see clause 7.2) |

# References

[1] RP-234069 New SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR

[2] RP-242348 Revised SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR

[3] 3GPP TR22.837 Feasibility Study on Integrated Sensing and Communication, Rel. 19

[4] 3GPP TS 22.137 Service Requirements for Integrated Sensing and Communication, Stage 1, Rel. 19

[5] R1-2410846	Session notes for 9.7 (Study on channel modelling for Integrated Sensing And Communication for NR), Ad-Hoc Chair (Huawei). RAN1#119.