3GPP TSG RAN WG1 #121			R1-2504402

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.6.2

Source: 	Qualcomm Incorporated

Title: 	LP-WUR Operation in Idle and Inactive Modes

Document for:	Discussion and Decision

# Introduction

NR Rel-19 work item (WI) is presented in [1] for the low-power (LP) wakeup signal (WUS) and wakeup receiver (WUR) design for NR with objectives below. The RAN1 related scope includes three main topics:

- LP-WUS and LP-SS design

- LP-WUR operation in UE idle and inactive modes

- LP-WUR operation in UE connected mode.

In this paper, we discuss the design aspects for the LP-WUR operation in idle and inactive modes. The general LP-WUS/LP-SS design and connected mode procedures are discussed in our companion papers in [2] and [3].

4.1 Objective of SI or Core part WI or Testing part WI The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Specify the demodulation performance and test cases for LP-WUS/WUR[RAN4]. - Specify RRM measurement performance requirement and corresponding test cases for the LP-WUS/WUR[RAN4].

# Offset between LO and reference point

In RAN1 #120bis, it was agreed that a 3rd offset value is not supported for the offset between an LO and the reference PO/PF and at most 2 values can be configured by network. There are a few FFS’s to be resolved in RAN1 #121. The first one is whether slot-level or symbol-level offset is used to indicate the starting time location of the first LP-WUS MO in the LO w.r.t. the reference frame. The symbol-level offset is similar to the Rel-17 PEI symbol-level offset from the reference point. The difference between LP-WUS and PEI is that due to the non-coherent OOK modulation, a LP-WUS MO duration (e.g., a few slots) can be much longer than the PEI MO (e.g., 3 symbols). In this case, it may not be necessary to have the symbol-level granularity.

Proposal 1: Support slot-level offset w.r.t. the reference frame to determine the starting time location of the first LP-WUS MO in the LO. The starting symbol of LP-WUS MO is the first available symbol in the determined slot.

As for the subsequent LP-WUS MOs after the first MO in the LO, we prefer Alt 1 that indicates offset for each of the subsequent LP-WUS MOs. This is related to the discussion of available symbols for LP-WUS in section 4. For that, we prefer to have a fixed nominal LP-WUS MO duration (i.e., Alt C) as a simpler design compared to other alternative solutions. Alt 1 for offset configuration helps avoid the issue that no MO contains enough available symbols for LP-WUS transmission in a beam.

Observation 1: Alt 1 for offset configuration for subsequent LP-WUS MOs provides more configuration flexibility to avoid the situation that no MO contains enough available symbols for LP-WUS transmission in a beam.

Proposal 2: Support Alt 1 to indicate an offset for each subsequent LP-WUS MO.

RAN1 #120bis Conclusion For the offset value(s) between an LO and a reference PO/PF, do not support the configuration of 3 offset values. Agreement For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. - FFS additional configuration to control the subsequent MO locations, e.g., - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration - FFS restriction on MO locations, e.g. only on DL slots - FFS minimum gap is needed between two MOs to ensure LR processing time

Another FFS is related to the processing time for LP-WUS detection after LP-WUS waveform is received by the LP-WUR. RAN4 is also discussing the processing time as part of the MR wake-up time after LP-WUS detection. The LP-WUR may or may not support parallel waveform reception and processing for LP-WUS detection. Figure 1-(a) shows an example that LP-WUR can not receive LP-WUS from one MO and process LP-WUS from the last MO simultaneously. Figure 1-(b) shows an example that LP-WUR can receive and process LP-WUS for different MOs simultaneously. Depending on which of (a) and (b) is implemented, the gap between the end of a MO and start of the next MO can be different for the same LP-WUS detection processing time.

Proposal 3: LP-WUS detection/processing time is determined by RAN4.

- RAN1 asks RAN4 for LP-WUS processing time and whether LP-WUR can process the LP-WUS in one MO and receive LP-WUS in the next MO simultaneously.

Alternatively, the UE capability for the gap between adjacent MOs can be defined with at least two candidate values corresponding to the case that LP-WUR can or cannot receive and process LP-WUS from different MOs simultaneously.

Proposal 4: Define UE capability for the time gap between MOs with at least two candidate values. One value corresponds to the concurrent reception and processing of LP-WUS from different MOs and the other value corresponds to the sequential reception and processing of LP-WUS from different MOs.

(a) Sequential LP-WUS reception and processing

(b) Sequential LP-WUS reception and processing

Figure 1: Gap between MOs in the LO.

# Available symbols for LP-WUS

In the following agreement, 4 alternative solutions are captured for the determination of available symbols for LP-WUS. As discussed in agenda 9.6.1, the OOK LP-WUS may have a quite long duration, e.g., 16 to 64 OFDM symbols. It is less likely the entire LP-WUS can be transmitted in consecutive symbols in the same slot. Certain symbols may not be suitable for LP-WUS transmission. For example, gNB may not be able to use UL symbols in TDD for LP-WUS transmission. Otherwise, it may require the gNB to support full duplex with concurrent transmission and reception. In another example, gNB may want to reserve certain symbols configured for important PDCCHs and does not use these symbols for LP-WUS transmission. For OOK LP-WUR, determination of the available symbol is more about the implementation restriction for gNB. For OFDM LP-WUR, determination of the available symbol may also depend on the UE capability to simultaneously receive SSB and LP-WUS when they are FDM’ed.

Observation 2: Determination of available symbols for LP-WUS depends on both gNB implementation and UE implementation capabilities

- For gNB, whether concurrent transmission and reception is supported, whether symbols are reserved for important PDCCHs such as paging PDCCH, and whether it can simultaneously transmit two signals in different beams, etc.

- For UE, whether FDM’ed reception of LP-WUS/LP-SS and SSB is supported.

Network can choose to send either LP-WUS or PDCCH if MOs for the two signals collide. Whether FDM’ed LP-WUS and SSB in the same symbol can be received by UE may depend on UE capability. For example, an OFDM LP-WUR may not be able to receive both LP-WUS and SSB in the same symbol if frequency gap between them is larger than the LP-WUR RF bandwidth. Ideally, all these conditions should be discussed to determine available symbols for LP-WUS and new UE capabilities can be defined. However, due to the limited meeting time for Rel-19 WI, we prefer to have a simple design rule to be specified.

Proposal 5: Determination of available symbols for LP-WUS transmission should be based on simple rules, e.g., if an existing NR signal is considered as higher priority than LP-WUS, LP-WUS is not transmitted in symbols configured for the existing signal irrespective of the frequency gap between the existing signal and LP-WUS.

Among all these alternatives in the agreement, Alt 3 is the best solution which considers both configuration flexibility and signaling overhead efficiency. Alt 3 includes both Alt 2 which minimizes signaling overhead based on existing configurations and maintains flexibility by defining the time-domain pattern for cases that are not fully covered by existing configuration.

Proposal 6: Support Alt 3 for the determination of available symbols for LP-WUS

- The periodic time-domain pattern is based on mechanism of NR rate matching pattern.

One last thing we want to discuss is whether the available symbols are determined within a beam or across beams. E.g., if symbols for SSB are not available for LP-WUS, does this rule only apply to SSB in the same beam as the LP-WUS transmission or apply to SSBs in all beams? We slightly prefer the latter, i.e., available symbols are determined between LP-WUS and NR signals across beams because this is a simpler design. Note that this problem does not exist for paging PDCCH as for paging PDCCH only TDD UL symbols are reserved in the determination of available symbols.

Proposal 7: Clarify whether available symbols are determined between LP-WUS and NR signals within the same beam or across beams

- For Rel-19, we prefer “across beams” for a simpler design.

RAN1 #120bis Agreement Support maximum length for a LP-WUS (actual OFDM symbols used) which is larger than one slot - Non-consecutive OFDM symbols at least across multiple slots can be used for a LP-WUS transmission Agreement UE determines whether a symbol is available for LP-WUS based on: - Alt 1: Time-domain pattern configured by the gNB - Alt 1A: Periodic time-domain pattern - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern - Alt 1B: Per-MO pattern, applicable for all Mos - Alt 2: Information from existing configurations available for idle/inactive UEs such as [SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc]. - Alt 3: Combination of Alt 1 and Alt 2 - Alt 4: NW ensures LP-WUS configuration without collision with existing signal(s)

# LP-SS design

Maximum duration of adopted LP-SS sequences is always smaller than the slot duration. Because of this, it was agreed in RAN1 #120bis the LP-SS transmission always occupies consecutive OFDM symbols. We think it is reasonable to add a further restriction that the LP-SS transmission in each beam is confined within a slot. Available symbol was already discussed for LP-WUS design. There can be a similar discussion for LP-SS to determine the starting symbol in a slot.

Proposal 8: LP-SS transmission for each beam is confined in the same slot

- Discuss how to determine the starting symbol of LP-SS transmission in the slot.

RAN1 #120bis Agreement Each LP-SS transmission for each beam always occupies consecutive OFDM symbols.

# Nominal MO duration and actual LP-WUS duration

Discussion of available symbols for LP-WUS transmission leads to two concepts in the following RAN1 #120bis agreement, i.e., the nominal MO duration and actual LP-WUS duration. There are 4 alternative solutions. For Alt A, the actual LP-WUS duration is not fixed. Hence, the LP-WUS detection performance, e.g., false alarm from noise, is not consistent. For Alt B, the nominal MO duration is not fixed. Problem with Alt B is that even the actual LP-WUS duration contains multiple very short segments, e.g., segments containing a single symbol, it is still considered as a valid actual LP-WUS duration. In general, we think for Alt B, the nominal MO should be not valid if it contains segment of a single or at most N (N<=2) consecutive available symbols, nominal MO is not valid if it contains too many (e.g., > 2) segments of consecutive available symbols, or if the nominal MO duration exceeds a threshold, etc.

For Alt C, both the nominal MO duration and the actual LP-WUS duration are fixed. A potential issue is that there may not be a single valid actual LP-WUS duration in a beam for LP-WUS transmission. Multiple techniques can be considered to resolve this problem, such as network ensures there is at least one valid MO per beam or it falls back to Alt B so that UE can autonomously extend the nominal MO duration if no actual LP-WUS duration exist in the beam. For Alt D, to support a same pattern of available symbols, network may have to exclude the union of unavailable symbols for all MOs. This may make many symbols that can be used for LP-WUS transmission unavailable.

Among all 4 alternatives, we think Alt C is the neatest design which can manage the fragmentation issue of actual LP-WUS duration by configuring a proper nominal MO duration, e.g., nominal MO duration is close to actual LP-WUS duration.

Proposal 9: Support Alt C - Both nominal MO duration and actual LP-WUS duration are configured

- Network ensures that there is at least one valid MO for each beam based on separately configured offsets w.r.t. the reference point for all MOs in the LO.

RAN1 #120bis Agreement Terminology definition - Nominal MO duration: this includes both the available and unavailable symbols - Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives: - Alt A: Nominal MO duration is configured. (e.g. in unit of slots) - Actual LP-WUS duration is the number of available symbols within the MO. - Actual LP-WUS duration can vary from one MO to another MO. - Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values) - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration. - FFS: Additional termination condition such as time window - Alt C: Both nominal MO duration and actual LP-WUS duration are configured. - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred. - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs. - Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

# RRM measurement

In RAN1 #120bis, there is an FFS on whether OFDM LP-WUR can measure LP-SS if overlaid OFDM sequence is not configured for M=1. Technically, OFDM LP-WUR can measure LP-SS based LP-RSRP/LP-RSRQ when overlaid OFDM sequence is not configured for any M. This can be done either by frequency domain filtering to extract signal components for OOK On and Off chips and calculate the corresponding RSSI and RSRP/RSRQ accordingly or in time domain direct accumulation of energy from OOK On and Off chips. For M=1, the frequency domain operation is even simpler as no filtering is needed.

Proposal 10: At least for M=1, OFDM receiver can measure LP-SS based LP-RSRP/LP-RSRQ if overlaid OFDM sequence is not configured.

RAN1 #120bis Agreement For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR. FFS: Whether OFDM receiver can measure LP-SS if overlaid OFDM sequence is not configured (M=1).

# Codepoint mapping to UE subgroups

In RAN1 #120bis, the following agreements were made for the maximum number of UE subgroups per PO. The common codepoint is always configured per PO. In RAN1 #120, the maximum number of UE subgroups per PO is agreed to be 31. Then, maximum 7, 15 or 31 UE subgroup specific codepoints are supported for 4, 2 and 1 POs per LO. Maximum number of codepoints for LP-WUS is 32. Based on these agreements, we have the following proposal.

Proposal 11: The number of codepoints configured for LP-WUS is equal to

- (1 + number of UE subgroups per PO) * number of POs mapped to a LO

- Codepoint value is from 0 to number of codepoints configured for LP-WUS - 1

When multiple POs are mapped to the LO, codepoint values are sequentially allocated for the common codepoint followed by codepoints for individual UE subgroups for each PO and across POs (see Figure 2). The same allocation pattern is repeated for all MOs in the LO.

Proposal 12: Within a LP-WUS MO, the codepoint values are sequentially allocated for the common codepoint for each PO followed by codepoints for individual UE subgroups for each PO of all POs associated with the LO.

Figure 2: Codepoint allocation for multiple POs, SGi,j is subgroup #j of PO #i.

RAN1 #120bis Agreement From specification design perspective, for both RRC idle/inactive and RRC connected mode, support up to 5 information bits and up to 32 codepoints for a LP-WUS. Agreement For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported. - For codepoint corresponding to more than one subgroups: - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO Agreement For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is - 7 for the case where 4 POs are mapped to one LO - 15 for the case where 2 POs are mapped to one LO RAN1 #120 Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32.

# EPRE configuration

In RAN1 #120, it was agreed that the EPRE ratio w.r.t. SSB can be configured for LP-WUS/LP-SS. The FFS is whether it is a common configuration or separate configuration for LP-WUS and LP-SS. To ease the AGC design for LP-WUR, it is preferrable that both the EPRE (i.e., average power in OFDM symbol duration) and the average power in OOK ON chips are the same for LP-WUS and LP-SS. Then, irrespective of the time domain receiver and frequency domain receiver, the LP-WUR AGC gain can be maintained between LP-WUS and LP-SS and the AGC gain only needs to be slowly updated to track the channel.

Observation 3: Considering different LP-WUR implementations, it is preferrable that LP-WUS and LP-SS have the same EPRE (i.e., average power in OFDM symbol duration) and the same average power in OOK ON chips.

Hence, we propose the same EPRE ratio w.r.t. SSB is configured for LP-WUS and LP-SS.

Proposal 13: The same EPRE ratio is configured for LP-WUS and LP-SS.

More importantly the LP-SS should be always transmitted at the configured EPRE because the LP-WUR needs a reliable estimate of the received signal power information to determine the detection threshold for LP-WUS. If this can be not guaranteed, network should transmit preamble before LP-WUS.

Proposal 14: LP-SS should be always transmitted at the configured EPRE so that detection threshold can be reliably determined for the LP-WUS.

- If this cannot be guaranteed, network needs to transmits preamble before LP-WUS.

There is another FFS on whether and how to support different number of OOK ON symbols in different OFDM symbols. For M=4, when the EPRE (i.e., average power in OFDM symbol duration) is fixed, different number of OOK ON chips transmitted in the OFDM symbol leads to different average power in the OOK On chip. This may happen for LP-WUS when the signal does not completely occupy all the used OFDM symbols, e.g., for the last symbol. This complicates the AGC design and operation for LP-WUR. Hence, we prefer not to support. This problem can be avoided by zero padding to the LP-WUS so that the signal always fully occupies integer number of OFDM symbols. Note that length of the agreed LP-SS sequences does not cause this problem.

Observation 4: For M=4, when the EPRE (i.e., average power in OFDM symbol duration) is fixed, more OOK ON chips in the OFDM symbol results in lower average power in the OOK On chip, and vice versa.

Proposal 15: Do not support different number of OOK ON symbols in different OFDM symbols

- LP-SS does not have this problem

- Only support a code block length for LP-WUS that does not have this problem.

A similar AGC issue exists if M=1 is configured for LP-WUS and M>1 is configured for LP-SS. In this case, when same EPRE is configured for LP-WUS and LP-SS, the average power in OOK On chip is different by 3dB between LP-WUS and LP-SS. We also prefer to avoid this issue based on the rule that if M=1 is configured for one of LP-WUS and LP-SS, M=1 is configured for both. Note that this issue is also discussed in agenda 9.6.1.

Observation 5: An issue similar to the different number of OOK ON symbols in different OFDM symbols for M=4 also exists for M=1 for LP-WUS and M>1 for LP-SS.

Proposal 16: If M=1 is configured for one of LP-WUS and LP-SS, M=1 is configured for both

- This issue is also discussed in agenda 9.6.1.

RAN1 #120 Agreement The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

# PEI reception after receiving LP-WUS

PEI and LP-WUS are two power saving features for the idle/inactive modes. The following RAN1 #116 agreement allows the two features to be simultaneously configured by network.

RAN1 #116 Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

The joint indication based on LP-WUS and PEI can be beneficial by further reducing the false paging probability that a UE monitors the paging PDCCH when it is not paged or there is no short message. This can be achieved by independently splitting UEs into subgroups with different sets of UEs for PEI and LP-WUS. This improves the subgrouping granularity without additional signaling cost.

In RAN2, the following agreement was made for UE-ID based subgrouping for PEI subgrouping [4].

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

In TS 38.304, UE ID based subgrouping for PEI is described by the following specification text.

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 Ns: number of paging occasions for a PF UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

In Figure 3, we plot the varying parts (i.e., yellow highlighted parts in the formulas) for the PEI subgroup ID and LP-WUS subgroup ID for N = 4, Ns = 4, Np = 8, subgroupsNumForUEID for LP-WUS = 8 and subgroupsNumForUEID for PEI = 4. It clearly shows that subgroup ID for LP-WUS and subgroup ID for PEI independently changes as the UE ID varies. This proves that joint subgrouping of LP-WUS and PEI improves the subgrouping granularity and reduce the UE false paging probability.

Observation 6: Joint subgrouping of LP-WUS and PEI improves the subgrouping granularity and reduces UE false paging probability.

Proposal 17: Support UE monitoring PEI after receiving LP-WUS to enhance the UE subgrouping granularity.

Figure 3: Different subgroup splitting for LP-WUS and PEI

# Conclusions

In this contribution, we have provided the following observations and proposals:

Proposal 1: Support slot-level offset w.r.t. the reference frame to determine the starting time location of the first LP-WUS MO in the LO. The starting symbol of LP-WUS MO is the first available symbol in the determined slot.

Observation 1: Alt 1 for offset configuration for subsequent LP-WUS MOs provides more configuration flexibility to avoid the situation that no MO contains enough available symbols for LP-WUS transmission in a beam.

Proposal 2: Support Alt 1 to indicate an offset for each subsequent LP-WUS MO.

Proposal 3: LP-WUS detection/processing time is determined by RAN4.

- RAN1 asks RAN4 for LP-WUS processing time and whether LP-WUR can process the LP-WUS in one MO and receive LP-WUS in the next MO simultaneously.

Proposal 4: Define UE capability for the time gap between MOs with at least two candidate values. One value corresponds to the concurrent reception and processing of LP-WUS from different MOs and the other value corresponds to the sequential reception and processing of LP-WUS from different MOs.

Observation 2: Determination of available symbols for LP-WUS depends on both gNB implementation and UE implementation capabilities

- For gNB, whether concurrent transmission and reception is supported, whether symbols are reserved for important PDCCHs such as paging PDCCH, and whether it can simultaneously transmit two signals in different beams, etc.

- For UE, whether FDM’ed reception of LP-WUS/LP-SS and SSB is supported.

Proposal 5: Determination of available symbols for LP-WUS transmission should be based on simple rules, e.g., if an existing NR signal is considered as higher priority than LP-WUS, LP-WUS is not transmitted in symbols configured for the existing signal irrespective of the frequency gap between the existing signal and LP-WUS.

Proposal 6: Support Alt 3 for the determination of available symbols for LP-WUS

- The periodic time-domain pattern is based on mechanism of NR rate matching pattern.

Proposal 7: Clarify whether available symbols are determined between LP-WUS and NR signals within the same beam or across beams

- For Rel-19, we prefer “across beams” for a simpler design.

Proposal 8: LP-SS transmission for each beam is confined in the same slot

- Discuss how to determine the starting symbol of LP-SS transmission in the slot.

Proposal 9: Support Alt C - Both nominal MO duration and actual LP-WUS duration are configured

- Network ensures that there is at least one valid MO for each beam based on separately configured offsets w.r.t. the reference point for all MOs in the LO.

Proposal 10: At least for M=1, OFDM receiver can measure LP-SS based LP-RSRP/LP-RSRQ if overlaid OFDM sequence is not configured.

Proposal 11: The number of codepoints configured for LP-WUS is equal to

- (1 + number of UE subgroups per PO) * number of POs mapped to a LO

- Codepoint value is from 0 to number of codepoints configured for LP-WUS - 1

Proposal 12: Within a LP-WUS MO, the codepoint values are sequentially allocated for the common codepoint for each PO followed by codepoints for individual UE subgroups for each PO of all POs associated with the LO.

Observation 3: Considering different LP-WUR implementations, it is preferrable that LP-WUS and LP-SS have the same EPRE (i.e., average power in OFDM symbol duration) and the same average power in OOK ON chips.

Proposal 13: The same EPRE ratio is configured for LP-WUS and LP-SS.

Proposal 14: LP-SS should be always transmitted at the configured EPRE so that detection threshold can be reliably determined for the LP-WUS.

- If this cannot be guaranteed, network needs to transmits preamble before LP-WUS.

Observation 4: For M=4, when the EPRE (i.e., average power in OFDM symbol duration) is fixed, more OOK ON chips in the OFDM symbol results in lower average power in the OOK On chip, and vice versa.

Proposal 15: Do not support different number of OOK ON symbols in different OFDM symbols

- LP-SS does not have this problem

- Only support a code block length for LP-WUS that does not have this problem.

Observation 5: An issue similar to the different number of OOK ON symbols in different OFDM symbols for M=4 also exists for M=1 for LP-WUS and M>1 for LP-SS.

Proposal 16: If M=1 is configured for one of LP-WUS and LP-SS, M=1 is configured for both

- This issue is also discussed in agenda 9.6.1.

Observation 6: Joint subgrouping of LP-WUS and PEI improves the subgrouping granularity and reduces UE false paging probability.

Proposal 17: Support UE monitoring PEI after receiving LP-WUS to enhance the UE subgrouping granularity.

# References

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- R1-2504401, LP-WUS and LP-SS design, Qualcomm, May 2025

- R1-2504403, LP-WUR operation in connected mode, Qualcomm, May 2025

- RP-250194, SR for LP-WUS WI for RAN# 107, vivo, NTT DOCOMO, March 2025