3GPP TSG RAN WG1 #121			      R1-2504497

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	9.2.4

Document for: 	Discussion and Decision

- Introduction

In RAN#105 meeting, Rel-19 WID of ‘NR MIMO Phase 5’ was revised [1]. The detailed objectives for RAN1 are as follows. In this contribution, we discuss the enhancements for asymmetric DL sTRP/UL mTRP scenarios.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

- Discussion

2.1 Two TA for single DCI based multi-TRP/panel or single-TRP

Closed loop power control adjustment state

In RAN1#120bis, the following proposal was discussed.

Companies are encouraged to consider the following for RAN1#121: Regarding whether/how to reset the CLPC of PUSCH/PUCCH/SRS, further study and down-select one from the following alternatives: - Updated Alt1: For a UE configured with 2 TAGs, when the UE receives a RAR of a PDCCH-order PRACH: - If PL offset is configured in joint/UL TCI state(s), according to the new 1-bit DCI field PL offset indication in DCI format 1_0: - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI: - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state. - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state. - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI: - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state; - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state - If PL offset is not configured in joint/UL TCI state(s) and the UE is configured with SSB-MTC-AddtionalPCI, according to the PRACH association indicator in DCI format 1_0: - If the indicated joint/UL TCI state is associated with the same PCI as indicated by the PRACH association indicator, the UE resets the closed loop power control adjustment state associated with the indicated joint/UL TCI state. - If the indicated joint/UL TCI state is associated with a different PCI from the one indicated by the PRACH association indicator, the UE resets the closed loop power control adjustment state that is different from the closed loop power control adjustment state associated with the indicated joint/UL TCI state. - Alt2: For a UE configured with two TAGs, one TAG ID is indicated in the RAR - The UE resets the closed loop power control adjustment state associated with a joint/UL TCI state associated with the indicated TAG. - Alt3: Not introduce any enhancement.

For the case of two TAGs, PDCCH order PRACH can be transmitted to either one of the two TRPs. If the PRACH is transmitted to the 2nd TRP (l = 1), it is preferable to reset the closed loop power control adjustment state for l = 1 (the second closed loop). However, in the legacy specification, the closed loop power control adjustment state for l = 0 (the first closed loop) for PUSCH/PUCCH/SRS is reset any time after receiving a RAR.

This issue has been discussed in Rel.18 two TA for multi DCI multi TRP, however it was not agreed because it was discussed in CR phase of Rel.18 two TA. We believe it is beneficial to fix the issue for Rel.19 two TA for other than multi DCI multi TRP.

To indicate to which TRP the PRACH is transmitted, we believe either Alt.1 or Alt.2 works. Hence, either option is acceptable for us.

Proposal 1

- Support to reset the closed loop power control adjustment state for PUSCH/PUCCH/SRS.

2.2 PL-offset

MPE report enhancement

In RAN1#120bis, the following was proposed in [2].

Proposal 9. For asymmetric MTRP scenario in FR2, support SRS as one of MPE resource for beam specific MPE report.

In Rel-17 unified TCI framework, beam specific MPE report has been introduced in FR2. Resource(s) represented by DL RS (either SSB or CSI-RS) for certain beam(s) and corresponding MPE value(s) can be reported via Enhanced Single Entry PHR MAC CE or Enhanced Multiple Entry PHR MAC CE.

In TS38.331, RRC IE for MPE resource pool has been defined as follows. So far, since beam management is mainly based on DL RS, only either SSB or CSI-RS is considered as MPE resource when beam specific MPE report has been introduced in FR2.

MPE-Resource-r17 ::= SEQUENCE { mpe-ResourceId-r17 MPE-ResourceId-r17, cell-r17 ServCellIndex OPTIONAL, -- Need R additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R mpe-ReferenceSignal-r17 CHOICE { csi-RS-Resource-r17 NZP-CSI-RS-ResourceId, ssb-Resource-r17 SSB-Index } }

In Rel-19 asymmetric DL sTRP/UL mTRP scenarios, UL TRP does not transmit SSB when PL offset is configured. The corresponding MPE value is not identified for UL transmission toward UL TRP from gNB side due to lack of DL RS which can be only MPE resource. Hence, we think it is beneficial to additionally introduce UL RS (i.e., SRS) as one of MPE resource for beam specific MPE report.

Proposal 2

- For asymmetric DL sTRP/UL mTRP scenarios, support SRS as one of MPE resource for beam specific MPE report.

- Conclusion

In this contribution, we discussed the enhancements for asymmetric DL sTRP/UL mTRP scenarios. Based on the discussion, we made following observations and proposals.

Proposal 1

- Support to reset the closed loop power control adjustment state for PUSCH/PUCCH/SRS.

Proposal 2

- For asymmetric DL sTRP/UL mTRP scenarios, support SRS as one of MPE resource for beam specific MPE report.

- Reference

- 3GPP RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105, Sep. 2024.

- Samsung R1-2502360, “Views on Rel-19 asymmetric DL sTRP/UL mTRP scenarios,” RAN1#120bis, Apr. 2025.