3GPP TSG RAN WG1 #121                                  R1-2504545

Malta, MT, May 19th – 23rd, 2025

Agenda item:	9.11.1

Source: 	CSCN

Title:	Discussion on downlink coverage enhancement for NR-NTN

Document for:	Discussion & Decision

- Introduction

In RAN1#120-bis meeting, several agreements about NR-NTN downlink coverage enhancements were obtained as following [1],

Agreement When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination. Agreement For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS. Agreement For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots: - The same resource allocation is assumed for all repetitions - The supported repetition factors are: 2 and 4 - The network configures a single value between 2 and 4 at a given time - The RV cycling is used for each repetition - If N=4: Starting RV; RV applicable to the nth repetition/transmission. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2. Working assumption For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported. RAN1 to down select between option 1 and option 2: Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2) - Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17) - FFS: Blind decoding limit Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X - FFS: Blind decoding limit - FFS: details configuration and signalling Nokia expressed the concern on the above working assumption that this will take physical resources away from intra-slot scheduling for legacy PDSCH. Agreement For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: - Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 - FFS: indication details. - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) - Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. - Option 3: The enabling/disabling is indicated by SIB1 configuration - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) FFS: Whether UE reports its capability

In this contribution, we discuss the remaining issues focusing on the downlink coverage enhancements as per the revised WID [2] where the extended default SSB periodicity is supported.

- Discussion

  - 2.1 System level coverage enhancement

### 2.1.1 Beam hopping

As mentioned in [3], since a single beam from a satellite can only cover several hundred or thousand square kilometers on the earth, the service area within a satellite footprint normally consists of up to thousands of beam footprints. While the proportion of the simultaneously active beams out of total beam footprints is as low as 1% around [4] due to limited satellite payload capability, such as power consumption, RF/antenna structure and beam forming capability. To improve the downlink coverage performance at system level, beam hopping is a necessary method that can efficiently cover the whole service area with simultaneously active beams and illuminate each beam footprint.

Taking Set 1-2 as an example, the network comprises 1058 footprints with 16 simultaneously active beams, where each cell utilizes 4 SSB beams (SSB indices #0–#3). During initial access, a 160ms SSB periodicity is adopted alongside a beam hopping mechanism to enhance coverage efficiency, wherein an active beam dynamically transmits SSB bursts to different geographic areas at varying time intervals within the 160ms period, as depicted in Fig. 1.

Fig. 1 SSB configuration

It is unclear whether the SSB index is associated with the ground beam footprint or with the beam. From our perspective, the SSB index should be associated with the beam footprint to simplify synchronization and beam switching, as the SSB index remains stable within a specific geographical area in despite of satellite motion. Instead, if the SSB index is associated with the beam, beam switching could become more complicated obviously.

Proposal 1: SSB index should be associated with the ground beam footprint to ensure consistent synchronization and simplify mobility management for UEs.

Regarding Set 1-2, there are two options for transmitting SSB, SIB1, and SIB19, where four SSBs are supported. For Option 1, a satellite sequentially transmits four SSBs across four beam footprints in a sweeping manner through different beams within 5ms SSB burst duration. Following the SSB transmission, the same approach is used to transmit SIB1 and SIB19 sequentially across the four beam footprints correspondingly. In Option 1, the SSB pattern aligns with the existing specs. However, a long gap between an SSB and relevant SIB1/SIB19 means that frequent beam switching is inevitable and increases system complexity.

The dwell time and revisit time of a beam can be described by beam hopping pattern for serving all footprints. For Option 1, the dwell time is at least 20ms to ensure sufficient time for transmitting the synchronization information through four beams. While, the revisit time should not exceed SSB periodicity to maintain the consistent synchronization for UEs. After the SSB, SIB1, and SIB19 transmission, which takes at least 10ms across four footprints, the remaining time could be utilized for additional DL data transmissions and UL PRACH transmissions.

For Option 2, one SSB and relevant SIB1/SIB19 associated with certain beam footprint could be sequentially transmitted by one beam before the beam hops to the next beam footprint. The same procedure is repeated across four footprints until all SSBs, SIB1, and SIB19 are transmitted. The dwell time of each beam can be as short as 5ms. The revisit time is not larger than SSB periodicity to ensure that the beam returns to the same footprint to provide synchronization information.

Option 2 minimizes beam switching by arranging SIB1/SIB19 adjacent to the SSB, meanwhile, it also modifies the SSB pattern and deviates from the existing specs. Without impact of SSB pattern on specs, the 16 simultaneously active beams in Set 1-2 could be grouped into four sets, each containing four beams. For each group, the first beam is used to transmit one SSB to the first beam footprint, and the second beam follows to transmit another SSB immediately to the second beam footprint, and similarly for the subsequent third and fourth beams to respective footprints. This sequential group transmission strategy could ensure that the SSB pattern remains consistent with existing standards. However, neither Option 1 nor Option 2 can achieve full coverage for Set 1-2 when the SSB periodicity is 160ms.

Considering the efficient payload power consumption of satellite, the imbalanced user distribution among different beam footprints and corresponding traffic demands will lead to an unequal beam/dwell time resource allocation or configuration. The imbalanced user distribution of different beam footprints should be considered for the beam hopping pattern design.

The UL beams are used for PRACH/Msg3 transmission to initiate the random access, HARQ feedback, etc. If beam hopping includes UL beams as well, the UL beam hopping pattern should be considered in conjunction with the DL beam hopping pattern. Additionally, the beam hopping patterns for both UL and DL should take the DL/UL inter-beam interference reduction into consideration.

Proposal 2: RAN1 to consider the beam hopping pattern, including dwell time and revisit time, and the UL beam hopping pattern should be considered in conjunction with the DL beam hopping pattern.

Proposal 3: The imbalanced user distribution of different beam footprints and DL/UL inter-beam interference should be considered for the beam hopping pattern design.

### 2.1.2 Impact of extended SSB periodicity

In RAN1#120 meeting, 160ms is supported as the maximum of the additional default value (apart from the existing 20ms value) for SSB periodicity extension.

The potential impacts of larger SSB periodicity are analyzed and summarized in the following.

1. Impact on backward compatibility

For the legacy UEs in Rel-17/Rel-18 with default 20ms SSB periodicity, a longer periodicity could cause fewer opportunities to detect SSB during the initial access. As a result, there may be issues on connection establishment, handovers, or signal detection for legacy UEs, posing challenges for backward compatibility.

Considering these potential problems, different SSB periodicities could be applied. The legacy UEs can still operate on the default 20ms SSB periodicity, while the Rel-19 NTN UEs can take advantage of the extended SSB periodicity, which reduces the common control channel overhead and increases the coverage ratio. Whether legacy UEs can access the network depends on the UE capability.

2. Impact on time and frequency synchronization

In NTN, the time/frequency drift mainly comes from satellite motion, and the maximum Doppler rate is about 0.27 ppm/s. UEs rely on SSB to achieve DL synchronization. But, with a larger SSB periodicity, time and frequency deviations may increase because of the satellite motion and propagation delay.

Based on the satellite ephemeris and position information, UEs can compensate the timing offset and frequency shift in advance. Since the satellite ephemeris is not available to UE until it receives SIB19, the SIB1/SIB19 reception shortly after SSB detection may ensure the time/frequency deviations are within acceptable range even extended SSB periodicity applied. Therefore, it is feasible to transmit SIB1/SIB19 as close as possible to the SSB transmission, which minimizes time and frequency impairments before the ephemeris is obtained.

3. Impact on cell search complexity, cell selection, and success rate

Regarding the cell search complexity, extended SSB periodicity induces more time needed for detecting SSB on a certain frequency grid, thus increases the time-domain cell search complexity, which may also decrease the success rate. But this is highly dependent on UE implementation, and some acquisition schemes/algorithms, e.g., parallel detection, could be adopted to mitigate the impact. Since NR has already employed sparse synchronization grid for SSB detection, the increased time-domain detection complexity is somewhat affordable. Furthermore, if NTN is deployed on a certain frequency band different from TN, sufficiently sparser grids could be employed to further decrease the frequency-domain detection complexity, while offsetting the increased time-domain complexity, thereby mitigate the impact on cell search and cell selection in general and potentially expedite the cell search procedure when the SSB periodicity is set to 160ms [5].

Proposal 4: Sufficiently sparser synchronization grids could be employed to decrease the cell search complexity.

4. Type0-PDCCH CSS

In RAN1#120-bis meeting, the following proposal was discussed with no consensus [6].

Proposal 6-1-v1 For NTN with extended SSB periodicity, one of the following options regarding type0-PDCCH CSS set periodicity is: - Option 1: is equal to the periodicity of SSB for CORESET multiplexing pattern 1.

For different SSB and CORESET multiplexing patterns, the periodicity of PDCCH monitor occasion (MO) is highly related to SSB periodicity. Particularly, for multiplexing pattern 1, a UE monitors Type0-PDCCH in the odd or even frames, which obviously matches the 20ms periodicity of SSB. When the SSB periodicity is extended to 160ms, if we follow the existing MO configuration, such frequent monitoring will result in a waste of power and resources. Thus, to adapt the extended SSB periodicity and reduce unnecessary resource consumption, the Type0-PDCCH CSS MO configuration need to be adjusted. Moreover, with beam hopping scheme, when a certain footprint is not expected to be illuminated by a beam, it is unnecessary for UEs to monitor PDCCH transmitted to this footprint. Therefore, when modifying the Type0-PDCCH CSS MO, the beam hopping pattern, i.e., dwell time and revisit time, should also be considered.

Proposal 5: The existing Type0-PDCCH CSS MO needs to be adjusted to adapt the extended SSB periodicity and beam hopping pattern, i.e., dwell time and revisit time.

5. Random access

After decoding SIB1, UE needs to initiate random access and transmit Msg1 on the RACH occasion (RO) resources indicated by SIB1. While there is a clear mapping relationship between ROs and SSBs, which is indicated by parameters including association period for mapping SSB to RO, the number of SSB associated with one RO, etc. However, existing specs restrict that ROs can only be placed on the first or second radio frames of the PRACH configuration period. Moreover, the association period starting point, determined by the PRACH configuration period, is always aligned with Frame#0. When all cells share the same SFN, this results in RO collisions among all cells served by a single active beam. Additionally, if the PRACH configuration period is shorter than 160ms, frequent collisions between neighboring cells will occur, as shown in Fig. 2.

Fig. 2 RO configuration when RO association period is 20ms

To mitigate RO collisions, a mechanism should be introduced to configure ROs for different cells within the coverage of the same active beam. Then, the gNB should adaptively adjust the uplink active beam to monitor potential PRACH transmissions on configured ROs.

In RAN1#120-bis meeting, the following proposals about RO configuration were proposed/discussed [5] [6].

Proposal 6-2-v0 A validity window is introduced to indicate a subset of RACH occasions, where only the RACH occasions within the window are considered to be valid for PRACH transmission: - The length and an offset to configure the window location are introduced. Proposal 6-2-v1 For NTN with extended SSB periodicity, a subset of RACH occasions is configured to support valid ROs for PRACH transmission. - The subset of ROs can be configured with bitmap setting.

The validity window mechanism mentioned in the Proposal 6-2-v0 is illustrated in Fig. 3. By introducing a time offset (offset) and a window length (Twin), valid ROs are explicitly indicated for its associated UEs without changing the existing SSB-to-RO mapping relationship. Besides, the same window length for all cells can be configured to reduce the complexity of on-board processing. However, only continuous ROs in time domain can be indicated in one window for one cell (e.g., the ROs in SFN1 and SFN3, as shown in Fig. 3, can be allocated to Cell 0), which highly restricts the flexibility. When discontinuous ROs are configured to align with the beam hopping pattern (e.g., configuring the ROs in SFN1 and SFN5 for Cell 0, during which Cell 0 is illuminated by active beam, while ROs in SFN3 and SFN7 are configured for Cell 1), multiple windows should be configured for one cell, resulting in increased complexity and signaling overhead.

Observation 1: If the validity window mechanism is adopted:

- If the same window length is configured for all cells, the complexity of on-board processing can be reduced.

- Only continuous ROs can be configured for one cell in one validity window, which highly restricts the flexibility of RO configuration and beam hopping.

Fig. 3 The configuration of RO window, where Cell 0 and Cell 1 sharing the same active beam

Moreover, the time-segmented bitmap indication mechanism proposed in proposal 6-2-v1 is illustrated in Fig. 4. A dedicated bit in the bitmap indicates the validity of ROs for the serving cell while preserving existing SSB-to-RO mapping relationship. This mechanism needs different bitmap indications for different cells, which brings some complexity to on-board processing. However, the continuous or discontinuous ROs can be configured by the bitmap indication mechanism with greater flexibility. Besides, the indication signaling overhead is related to the segmentation granularity of the 160ms duration, and the duration of each time segment could be aligned with the dwell time of the beam hopping pattern defined in section 2.1.1.

Observation 2: If the bitmap indication mechanism is adopted:

- This mechanism needs different bitmap indications for different cells, which brings some complexity to on-board processing.

- The continuous or discontinuous ROs can be configured by bitmap indication mechanism with greater flexibility, and the signaling overhead is related to the segmentation granularity.

Fig. 4 Bitmap indication, where Cell 0 and Cell 1 sharing the same active beam

Based on above analysis, a mechanism should be introduced to configure ROs for different cells within the coverage of the same active beam, thus better adapting to the extended SSB periodicity and beam hopping pattern.

Proposal 6: One of the following mechanisms should be introduced to configure ROs for different cells within the coverage of the same active beam:

- Option 1: A validity window configured by time offset and window length,

- Option 2: Time-segmented bitmap indication.

6. Paging

Similarly, the satellite beam will not cover all beam footprints in each radio frame with beam hopping scheme. If the UEs in idle or inactive state still wake up at each paging occasion (PO) configured in existing specification to monitor PDCCH for paging message scheduling, additional power and resource consumption will be introduced. Thus, the configuration of PO also needs to be modified to adapt the DL beam hopping pattern.

Proposal 7: The existing PO needs to be adjusted to adapt the extended SSB periodicity and beam hopping pattern.

### 2.1.3 Wide and narrow beam approach

RAN1#117 made the following observation and noted that extending the SSB periodicity and using the wide beam can significantly enhance coverage ratio across all three satellite parameter sets. According to simulation results for wide beams, the transmit power of one beam would be reduced with 6dB, and further link level enhancements may be required. However, using a wider beam for transmitting SSB and common signals/channels can achieve 100% coverage ratio with reduced overhead.

In RAN1#120 meeting, the maximum SSB periodicity is extended to 160ms. However, for Set 1-2, 160ms SSB periodicity is insufficient to achieve full coverage, so it is necessary to combine the extended periodicity with the use of wide beams. Specially, in the case of Set 1-2, SSB periodicity is reduced to 80ms with the wide beam.

RAN1#117 Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs. Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint: - 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%. - 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively. - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams. - 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%. - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth. Note: RAN1 will further investigate the impact of SSB periodicity extension Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...)

DL common channels are transmitted using a wider beam to enhance coverage. However, the reduced per beam transmit power cannot support high data rate transmission. To transmit traffic channels with high rate, narrow beams are primarily used to focus energy on certain direction and provide higher capacity. Compared with wider beam for DL common channels, narrower beam may be adopted for DL/UL dedicated channels (incl. PRACH). It is essential to set a threshold to determine whether a UE should use a wide or narrow beam, and indicate the decision to the UE accordingly. Besides, to minimize the impact on specs, the approaches for narrow beam selection can be left to network implementation.

Proposal 8: If wide beams are used, minimizing the impact on specs should be carefully considered.

  - 2.2 Link level coverage enhancements

In RAN1#120-bis meeting, the following agreement about enabling/disabling SIB1 PDSCH repetition was proposed [5].

Agreement For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

It is obvious that option 1 is the direct way for SIB1 PDSCH repetition enabling/disabling. However, as agreed in RAN1#120-bis meeting, the PDCCH repetition for Type0-PDCCH CSS is enabled/disabled by reserved bit(s) in PBCH payload. Given that there are only two reserved bits allocated for FR1 in the PBCH payload, adopting option 1 would fully consume these reserved bits. For option 2, although it leverages dynamically scheduled transmissions to enhance flexibility, it necessitates introducing a new DCI field to indicated the repetition, which would increase signaling overhead.

In RAN1# 117 meeting, LLS results and observations about PDCCH coverage and PDSCH SIB1 coverage were obtained as following.

RAN1#117 Observation Based on LLS results on PDCCH coverage evaluation collected from different sources: - It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1. - The coverage margin is around 4 dB compared to CNR of -1.9 dB. - With parameter LEO600km Set1-3 FR1: - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average compared to CNR of -9.9 dB. - Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDCCH. RAN1#117 Observation Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources: - For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources) - For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources) - With parameter LEO600km Set1-1 FR1 and 1-2FR1: - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1: - The coverage margin is around 3.9 dB on average compared to CNR of -1.9 dB - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2: - The coverage margin is around 1.5 dB on average compared to CNR of -1.9 dB - With parameter LEO600km Set1-3 FR1: - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1: - The coverage gap is around 4.1 dB on average compared to CNR of -9.9 dB - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1: - The coverage margin is 3.4 dB compared to CNR of -9.9 dB - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2: - The coverage gap is around 6.5 dB on average compared to CNR of -9.9 dB - Note: some results assumed SIB1 combination (where SIB1 is repeated within 160 ms) and some results assumed no SIB1 combination - Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1.

Evaluation results from the majority of companies indicate that the SIB1 PDSCH coverage gap is larger than that of the PDCCH. Therefore, SIB1 PDSCH repetition should be activated when PDCCH repetition is enabled. To avoid additional signaling overhead, option 3 can be supported, which implicitly links the SIB1 PDSCH repetition indication to Type-0 CSS PDCCH repetition indication.

Proposal 9: Option 3 should be supported which implicitly links SIB1 PDSCH repetition indication to Type-0 CSS PDCCH repetition indication, without additional signaling overhead.

- Conclusions

In this contribution, we provide some views on downlink coverage enhancement for NR NTN scenarios. The proposals are summarized as following.

Proposal 1: SSB index should be associated with the ground beam footprint to ensure consistent synchronization and simplify mobility management for UEs.

Proposal 2: RAN1 to consider the beam hopping pattern, including dwell time and revisit time, and the UL beam hopping pattern should be considered in conjunction with the DL beam hopping pattern.

Proposal 3: The imbalanced user distribution of different beam footprints and DL/UL inter-beam interference should be considered for the beam hopping pattern design.

Proposal 4: Sufficiently sparser synchronization grids could be employed to decrease the cell search complexity.

Proposal 5: The existing Type0-PDCCH CSS MO needs to be adjusted to adapt the extended SSB periodicity and beam hopping pattern, i.e., dwell time and revisit time.

Observation 1: If the validity window mechanism is adopted:

- If the same window length is configured for all cells, the complexity of on-board processing can be reduced.

- Only continuous ROs can be configured for one cell in one validity window, which highly restricts the flexibility of RO configuration and beam hopping.

Observation 2: If the bitmap indication mechanism is adopted:

- This mechanism needs different bitmap indications for different cells, which brings some complexity to on-board processing.

- The continuous or discontinuous ROs can be configured by bitmap indication mechanism with greater flexibility, and the signaling overhead is related to the segmentation granularity.

Proposal 6: One of the following mechanisms should be introduced to configure ROs for different cells within the coverage of the same active beam:

- Option 1: A validity window configured by time offset and window length,

- Option 2: Time-segmented bitmap indication.

Proposal 7: The existing PO needs to be adjusted to adapt the extended SSB periodicity and beam hopping pattern.

Proposal 8: If wide beams are used, minimizing the impact on specs should be carefully considered.

Proposal 9: Option 3 should be supported which implicitly links SIB1 PDSCH repetition indication to Type-0 CSS PDCCH repetition indication, without additional signaling overhead.

- References

- Chairman's Notes RAN1#120-bis, Apr. 17th-21st, 2025.

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Dec. 9th-12nd, 2024.

- 3GPP TR38.811, Study on New Radio (NR) to support non-terrestrial networks.

- 3GPP TR38.821, Solutions for NR to support non-terrestrial networks (NTN).

- R1-2501728, FL Summary #4: NR-NTN downlink coverage enhancements, Apr. 7th-11th, 2025.

- R1-250xxxx Chair notes RAN1#120bis (9.11 R19 NTN) v05, Apr. 7th-11th, 2025.