3GPP TSG RAN WG1 #121                                                  R1- 2504591

St Julian’s, Malta, May 19th – 23th, 2025

Source: Google

Title:	On CLI handling for SBFD operation

Document for: Discussion and Decision

# Introduction

The Rel-19 Work item for the evolution of NR duplex operation has been approved in RAN#102 [1] and updated in RAN#104 [2]. One of the main constraints in effective SBFD operation is the gNB-to-gNB CLI and the UE-to-UE CLI.

Conventional TDD operation mitigate interference between neighboring UEs and gNBs by segregating UL and DL transmissions into distinct time slots/symbols, hence splitting the time domain between UL and DL. In contrast, SBFD allows simultaneous UL and DL transmissions within the same time slots/symbols at the gNB. This introduces the potential for:

- Inter-UE CLI: the UL transmission of one aggressor UE can degrade the DL signal reception quality at a neighboring victim UE.

- Inter-gNB CLI: the DL transmission of one aggressor gNB can interfere with the UL signal reception at a neighboring victim gNB.

Effective CLI mitigation techniques are needed to be specified in Rel-19.  The following WI objectives for the CLI mitigation have been captured as below:

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - Information exchange among gNBs, including [RAN3]

      - Semi-static cell-specific SBFD time and frequency location configuration

      - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

      - Strongest DL beam information

      - CLI-mitigation request

    - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

      - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

      - PUSCH resource mapping, i.e., rate-matching around the muted REs

      - UCI resource determination in symbols with muted REs

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

      - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

      - At least aperiodic reporting

      - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - UCI bits generation

      - Priority rules for multiple CSI reporting

      - CLI measurement accuracy requirement

    - Note: Without dedicated optimization for dynamic/flexible TDD.

In this Tdoc, we share our views and propose techniques to specify the above enhancements.

# Remaining CLI handling issues

#### QCL type-D configuration for Aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting

During the RAN1#120 meeting, an agreement was made for scenarios where the scheduling offset between the triggering PDCCH last symbol (for aperiodic CLI reporting) and the first symbol of aperiodic SRS-RSRP or CLI-RSSI measurement resources is less than the UE reported beamSwitchTiming threshold.

Agreement

If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, down-select from the following alternatives:

- Alt.1: UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

- Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

FFS: whether Alt.1 or Alt.2 has any specification impact.

We prefer to re-use the established AP CSI-RS default beam rules to ensure the UE has a defined procedure rather than considering it as an error case. Furthermore, re-using the existing mechanisms minimize specification effort.

- Support Alt-2 for scenarios where the scheduling offset between the triggering PDCCH last symbol (for aperiodic CLI reporting) and the first symbol of aperiodic SRS-RSRP or CLI-RSSI measurement resources is less than the UE reported beamSwitchTiming threshold:

- Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

#### Measurement and Reporting L1-RSRP/L1-RSSI for SBFD UEs with multiple antenna ports

In RAN1#120bis, the following proposal was made:

#### Proposal

For SRS-RSRP measurement resource configuration, the number of antenna ports is 1.

We don’t have such restriction for L3 reporting. If a multiple ports SRS resource is configured for L1-SRS-RSRP measurement, the UE can measure and report the L1-SRS-RSRP for each SRS port or can be configured with the antenna port to measure.

- If a multiple ports SRS resource is configured for L1-SRS-RSRP measurement, the UE can measure/report the L1-SRS-RSRP for each SRS port or can be configured with the antenna port to measure.

#### Collision handling between L1-CLI measurement and other UL transmission(s) or DL reception(s)

The SRS transmission from the aggressor UE may not be always guaranteed, e.g., due to collision with other uplink signals, and the aggressor UE may drop the SRS transmission. Then the victim UE may not be able to measure the SRS.

- Study the behaviour of the victim UE when the SRS transmission is dropped by the aggressor UE, e.g. due to collision with other UL signals.

UE can be semi-statically or dynamically scheduled to receive a DL signal and is also expecting an SRS transmission from a another UE for UE-to-UE CLI measurements based on different QCL -TypeD, the victim UE cannot receive both the SRS and the DL signal simultaneously.

Some solutions to explore include:

- Dropping SRS: when the SRS and the DL signal are configured with different QCL- Type D, the first UE is configured to prioritize the DL reception and drop the SRS reception (e.g., the L1-SRS-RSRP/SRS-RSSI) if the two transmissions are overlapping in time.

- Drop the DL signal: when the SRS  and the DL signal are configured with different QCL- Type D, the first UE can be configured to prioritize the SRS reception and drop the DL reception if the two transmissions are overlapping in time

- Scheduling restriction: scheduling restrictions could be specified to avoid such collision scenarios. For example, the UE is not expected to receive a DCI scheduling DL transmission overlapping with the reception of an SRS, where the DL transmission and the SRS may be configured with different QCL-TypeD properties. The gNB should ensure the SRS and the DL signal are configured with the same QCL-TypeD property.

- Priority rules: priority rules could be established to determine which signal – a DL data transmission or an SRS transmission should be prioritized for reception by the UE in the event of a scheduling conflict.

- If a vicitm UE is to receive a DL signal and is also expecting an SRS transmission from  agressor UE for UE-to-UE CLI measurements based on different QCL -TypeD, it cannot receive both the SRS and the DL signal simultaneously. Some solutions could be specified:

- Dropping SRS

- Dropping DL signal

- Follow priority rules

- If not discussed in RAN1, send LS to RAN4 to address the collision handling between L1 SRS-RSRP measurement or L1 CLI-RSSI and other UL transmission(s) or DL reception(s) and if any scheduling restrictions needed.

- pi/2 BPSK modulation for PUSCH/PT-RS symbols with UL muting

PUSCH supports π/2-BPSK modulation, particularly with the DFT-S-OFDM waveform, to enhance uplink coverage. A key advantage of π/2-BPSK is its ability to offer a lower Peak-to-Average Power Ratio (PAPR) compared to BPSK. Standard BPSK modulation can suffer from high PAPR due to the zero crossings in the signal envelope. These zero crossings happen when the bit stream changes from 0 to 1, or 1 to 0 leading to a 180-degree phase shift as the signal transitions between constellation points. Such phase change causes the signal envelope to momentarily pass through or near zero, contributing to an increased PAPR.

A π/2-BPSK modulated symbol is generated as

from a BPSK symbol

where i is the symbol index.

The incremental π/2 phase rotation applied to successive symbols ensures that direct 180-degree phase transitions between adjacent symbols are avoided, thereby maintaining a more constant signal envelope and achieving the desired PAPR reduction.

However, specific handling of Uplink (UL) resource muting, as discussed in 3GPP RAN1, may inadvertently compromise these benefits of π/2-BPSK when transform precoding is enabled for PUSCH.

In RAN1#120bis, an agreement was reached that, under such conditions, the DFT processing effectively operates on MscPUSCH2 modulated symbols, where MscPUSCH​ is the total number of scheduled subcarriers for the PUSCH. This reduction is achieved either by modifying the DFT size to MscPUSCH2  (Option 1) or by employing symbol repetition or zero-padding strategies with a DFT of size MscPUSCH​ ​, followed by selection of specific subcarriers (Options 2A and 2B).

If the complex symbols are selected with even indices, for example like complex symbols with even indices 0,2,4,…, MscPUSCH-1 or with odd indices like for example 1,3,…, MscPUSCH then the π/2-BPSK complex valued symbol

Would change into a BPSK complex valued symbol

And this is because the index i is even and is equal to 0, 2, 4, …, MscPUSCH (hence i mod 2 is always 0) and there will be no π/2 phase rotation.

To address this issue, the indices i for PUSCH and the positions m for PT-RS should be based on the DFT samples before muting. For example taking the first MscPUSCH2 complex symbols (e.g., 0,1,2,3,… MscPUSCH2-1).

- To avoid impact on pi/2 BPSK modulation for PUSCH/PT-RS symbols with UL muting, the first MscPUSCH2 complex symbols are selected for the frequency mapping

- PUSCH with UL muting and OCC

OCC are applied to multiplex signals in the same radio resources from different sources, such as signals from different layers, UEs, …OCC can be applied with UL repetitions in the coverage enhancement scenario. For example, multiple UEs may be scheduled or configured to send UL repetitions in the same radio resources, where each UE applies a distinct OCC sequence in an OCC group. In accordance with the OCC sequence, the base station can distinguish individual signals from the multiplexed signals.

If the gNB configures the UE to transmit PUSCH with UL muting (e.g., comb-2 muting) and OCC, clarification is needed on whether the UE needs to apply the same UL muting pattern for the PUSCH repetitions within an OCC group or if PUSCH with OCC repetitions is not expected to overlap with UL muting symbols.

- Clarify whether PUSCH repetitions with OCC can overlap with UL muting symbols and if they need to have the same muting patterns.

- TBS determination with UL muting

Clarification is needed for the determination of TBS when UL muting is used especially when the number of scheduled PUSCH symbols is small, e.g. 2-symbol PUSCH and when the ratio the muted REs is large. In this scenario, the number of REs allocated for PUSCH within a PRB  in the TBS formula can be adjusted to account for the muted REs.

- At least when when the ratio the muted REs in a PUSCH is large, the number of REs allocated for PUSCH within a PRB  in the TBS formula has to be adjusted to account for the muted REs.

For the case of initial transmission with no muted REs and the HARQ retransmission(s) with muted REs and vice versa (and also for PUSCH repetitions), the TBS determination can be based on the initial transmission or no muted RSs are always assumed in the TBS calculation.

- For the case of initial transmission with no muted REs and the HARQ retransmission(s) with muted REs and vice versa (and also for PUSCH repetitions), the TBS determination can be based on the initial transmission.

- UL muting symbol overlaps a symbol containing PT-RS

In the agreement below, Alt-4 was proposed so that UL resource muting is not applied on PUSCH symbols containing PT-RS. And the motivation is to reduce the impact on phase error estimation and tracking.

Agreement

When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, down-select from the following alternatives in RAN1#121

- Alt-3

  - For PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding

    - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1

    - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups

    - Note: After PT-RS insertion, all of the mathematically equivalent DFT Options 1, 2A, and 2B from the RAN1#120 agreement are applicable, where the choice is up to UE implementation

- Alt-4

  - UL resource muting is not applied in the PUSCH symbol containing PT-RS

However, in DFT-s-OFDM based PUSCH, the employment of PTRS offers limited benefits for the UEs with compromised coverage. Such UEs typically utilize low-order modulations, which are inherently less susceptible to phase noise, thereby mitigating the performance gains achievable through PTRS.

Therefore, we propose that PT-RS are not transmitted on symbols with DFT-s-OFDM when UL resource muting is applied on the OFDM symbol.

- PT-RS are not transmitted on symbols with DFT-s-OFDM when UL resource muting is applied on the OFDM symbol

# Conclusion

In this contribution, we have presented several proposals to specify the support of the gNB-to-gNB CLI and the UE-to-UE CLI handling. We made the following proposals:

- Support Alt-2 for scenarios where the scheduling offset between the triggering PDCCH last symbol (for aperiodic CLI reporting) and the first symbol of aperiodic SRS-RSRP or CLI-RSSI measurement resources is less than the UE reported beamSwitchTiming threshold:

- Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

- If a multiple ports SRS resource is configured for L1-SRS-RSRP measurement, the UE can measure/report the L1-SRS-RSRP for each SRS port or can be configured with the antenna port to measure.

- Study the behaviour of the victim UE when the SRS transmission is dropped by the aggressor UE, e.g. due to collision with other UL signals.

- If a vicitm UE is to receive a DL signal and is also expecting an SRS transmission from  agressor UE for UE-to-UE CLI measurements based on different QCL -TypeD, it cannot receive both the SRS and the DL signal simultaneously. Some solutions could be specified:

- Dropping SRS

- Dropping DL signal

- Follow priority rules

- If not discussed in RAN1, send LS to RAN4 to address the collision handling between L1 SRS-RSRP measurement or L1 CLI-RSSI and other UL transmission(s) or DL reception(s) and if any scheduling restrictions needed.

- To avoid impact on pi/2 BPSK modulation for PUSCH/PT-RS symbols with UL muting, the first MscPUSCH2 complex symbols are selected for the frequency mapping

- Clarify whether PUSCH repetitions with OCC can overlap with UL muting symbols and if they need to have the same muting patterns.

- At least when when the ratio the muted REs in a PUSCH is large, the number of REs allocated for PUSCH within a PRB  in the TBS formula has to be adjusted to account for the muted REs.

- For the case of initial transmission with no muted REs and the HARQ retransmission(s) with muted REs and vice versa (and also for PUSCH repetitions), the TBS determination can be based on the initial transmission.

- PT-RS are not transmitted on symbols with DFT-s-OFDM when UL resource muting is applied on the OFDM symbol

# References

- RP-234035 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, CMCC (Moderator, RAN1 VC), RAN#102, Edinburgh, Scotland, December 11-15, 2023.

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) ”, Huawei, RAN#104,  Shanghai, China, June 17-20, 2024

- 3GPP TR 38.858 V18.0.0 “Study on Evolution of NR Duplex Operation”

- R1-2308336, Discussion on evaluation and methodologies on evolution of NR duplex operation, Huawei, HiSilicon

- R1-2307922, On Deployment scenarios and evaluation Methodology for NR duplex evolution, Qualcomm Incorporated

- R1-2306874, On the evaluation methodology for NR duplexing enhancements, Nokia, Nokia Shanghai Bell

- R1-2307922, On Deployment scenarios and evaluation Methodology for NR duplex evolution, Qualcomm Incorporated