3GPP TSG RAN WG1 #121			R1-2504667

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.10

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary of discussion on Rel-19 NR UE features for IoT-NTN TDD mode

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 9.15.10. regarding Rel-19 NR UE features for IoT-NTN TDD mode.

[121-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

- Companies’ inputs

The following inputs have been provided in companies’ contributions in this meeting:

| Company | Proposal/observation |
| --- | --- |
| Huawei, HiSilicon | In Release-17/18, a set of NB-IoT feature groups were introduced for IoT NTN operation with FDD mode. A basic FG (FG2-1b) introduced in Release-17 is referred to as prerequisite for the rest of feature groups. In our perspective, Release-19 IoT NTN TDD UE can work independently of IoT NTN FDD features. Considering that FDD is not explicitly defined as a component of FG2-1b, we can accept FG2-1b as prerequisite with a note to clarify that it does not imply to support FDD. In such case the components in Release-17 feature group 2-1b which are common for TDD and FDD operation are not required to be copied in the FG for TDD. Proposal 1: Release-17 feature group 2-1b can be the prerequisite of FG2-1 with a note to clarify that it does not imply a TDD UE should always support FDD operation. Proposal 2: The following components are added for the basic feature groups: - SIB1-NB is dropped in non-D NB-IoT subframes; - NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode, instead, periodicities of 90ms and 180ms are supported. For the rest of FGs introduced in previous releases for IoT NTN FDD, it should be discussed one by one whether to inherit by IoT NTN TDD. Release-17 FG 2-2a for enhancing timing relationships and Release-17 FG 2-3a for TA compensation reporting can be introduced as the optional capabilities to reduce the delay of scheduling. Release-17 FG 2-1c (Segmented UL transmission for NB-IoT) and FG 2-1e (Gaps for segmented UL transmission for NB-IoT) are not needed because UE had sufficient time to adjust its TA and CFO during non-U subframes. Among Release-18 UE features, we think the FG for HARQ disabling are necessary considering the short duration available for DL reception and UL transmission. These feature groups include Release-18 FG2-1e-1 (Semi-static HARQ feedback disabling for NB-IoT in single TB case), FG2-1f-1 (Dynamic HARQ feedback disabling by DCI-based direct indication for NB-IoT in single TB case), FG2-1g-1 (Dynamic HARQ feedback disabling by DCI-based overridden indication for NB-IoT in single TB case), FG2-1e-2 (Semi-static HARQ feedback disabling for NB-IoT in multi TB case), FG2-1f-2 (Dynamic HARQ feedback disabling by DCI-based direct indication for NB-IoT in multi TB case) and FG2-1g-2 (Dynamic HARQ feedback disabling by DCI-based overridden indication for NB-IoT in multi TB case). Considering the constellation of iridium is NGSO, the Release-18 FG2-2b (NGSO for HARQ disabling for NB-IoT) should be inherited. How to capture the inherited FGs from previous releases (e.g. copy as new FGs or modify the existing FGs) can be further discussed. Proposal 3: Release-17 feature groups 2-2a and 2-3a can be inherited in Release-19 IoT-NTN TDD. Proposal 4: Release-18 feature groups 2-1e-1, 2-1f-1, 2-1g-1, 2-1e-2, 2-1f-2, 2-1g-2 and 2-2b can be inherited in Release-19 IoT-NTN TDD mode. For the granularity of feature groups, per UE is preferred. Because this feature only applies to a single band, per band granularity is not essential. Proposal 5: For the granularity of feature groups, per UE is preferred. Proposal 6: The basic feature group should be optional with capability signaling, and a UE must indicate the support of this FG if it supports IoT NTN TDD in 1616-1625.5 MHz MSS band. Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Support NPRACH periodicity of ms90, ms180, ms160, ms240, ms320, ms640, ms1280, ms2560. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)]; [Rel-17 2-1b]; Yes; N/A; IoT-NTN TDD mode is not supported; [Per UE or Per band]; [TDD only]; [N/A]; Applicable only for 1616-1626.5 MHz MSS allocated band A UE report this FG does not imply it always support FDD operation.; FFS FG 2-1 is optional with capability signaling, and a UE must indicate the support of this FG if it supports IoT NTN TDD in 1616-1625.5 MHz MSS band.. |
| Vivo | - Where/how to capture the FG of IoT TDD mode and prerequisite One approach is that a UE supporting IoT NTN TDD mode should also support the common components introduced in FG 2-1b of Rel-17, which are not differentiated between TDD and FDD operation. From the below details of FG 2-1b, all the components are applicable regardless of duplex mode. While K_offset may not be necessary due to the presence of a guard period between DL burst and UL burst, supporting component 13 is harmless. Besides, NW can decide to not configure K_offset if the guard period is sufficient to cover RTT time. Furthermore, if a UE supports any additional FG introduced in previous releases when operating in IoT NTN TDD mode, and if extra behaviors are required for UEs, a separate capability should be introduced accordingly. For R17 FG: FG; FG function; content; Whether should be supported in IoT NTN TDD mode; Any enhancements if supported in IoT NTN TDD mode. 2-1c; Segmented UL transmission for NB-IoT; UE applies segmented UL transmission according to duration configuration by the network; No; N/A. 2-1e; Gaps for segmented UL transmission for NB-IoT; Support of gaps for segmented UL transmission for NB-IoT; No; N/A. 2-2a; Enhancing timing relationships using a time offset for NB-IoT; UE receives and applies UE specific K_offset/ cell specific K_mac in timing relationship enhancements; Yes; RAN2 agreed to support K_mac with a value range up to 1023 ms, whether to have additional FG for extended K_mac should be up to RAN2. 2-3a; TA pre-compensation reporting for NB-IoT; Support reporting of information about the UE specific TA pre-compensation; Yes; N/A. For R18 FG: FG; FG function; content; Whether should be supported in IoT NTN TDD mode; Any enhancements if supported in IoT NTN TDD mode. 2-1e-1; Semi-static HARQ feedback disabling for NB-IoT in single TB case; 1. UE gets RRC configuration for disabling HARQ feedback per UE per process; Yes; N/A. 2-1f-1; Dynamic HARQ feedback disabling by DCI-based direct indication for NB-IoT in single TB case; 1. UE receives DCI indication to directly indicate disabling HARQ feedback; Yes; N/A. 2-1g-1; Dynamic HARQ feedback disabling by DCI-based overridden indication for NB-IoT in single TB case; 1. UE receives DCI indication to override RRC configuration for disabling HARQ feedback 2. For single TB scheduled by single DCI, UE follows NPDCCH monitoring behavior for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI; Yes; N/A. 2-1e-2; Semi-static HARQ feedback disabling for NB-IoT in multi TB case; 1. UE gets RRC configuration for disabling HARQ feedback per UE per process; Yes; N/A. 2-1f-2; Dynamic HARQ feedback disabling by DCI-based direct indication for NB-IoT in multi TB case; 1. UE receives DCI indication to directly indicate for disabling HARQ feedback; Yes; N/A. 2-1g-2; Dynamic HARQ feedback disabling by DCI-based overridden indication for NB-IoT in multi TB case; 1. UE receives DCI indication to override RRC configuration for disabling HARQ feedback 2. For multi TB scheduling a single transport block by single DCI, UE follows NPDCCH monitoring behavior for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI; Yes; N/A. 2-3b; GNSS position fix in RRC Connected state for NB-IoT—triggered; 1. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode via RRCConnectionReestablishmentComplete-NB 2. UE receives eNB GNSS measurement trigger 4. UE re-acquires GNSS position fix within a configured gap 5. UE reports the remaining GNSS validity duration with MAC CE in connected mode; Depends on RAN1 discussion. Our preference is that no enhancement is needed if supported, as discussed in [2]. 2-4b; GNSS position fix in RRC Connected state for NB-IoT—autonomous; 1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode via RRCConnectionReestablishmentComplete-NB 3. UE reports the remaining GNSS validity duration with MAC CE in connected mode; Depends on RAN1 discussion. Our preference is that no enhancement is needed if supported, as discussed in [2]. 2-5b; UL transmission extension after original validity duration expires for NB-IoT; 1. UE is allowed to transmit UL in a duration X after original GNSS validity duration expires without GNSS re-acquisition when enabled by the network; Depends on RAN1 discussion. Our preference is that no enhancement is needed if supported. Based on the above analysis, for optional features beyond Rel-17 FG 2-1b introduced in previous releases, no behavioural modification is needed for their operation under IoT TDD mode. Therefore, there is no need to introduce a new FG; the UE can simply indicate support for both IoT TDD mode and the respective features. - Whether the FG should be defined as per UE or per band: - If defined per band, there is no need to differentiate between TDD and FDD as the duplex mode for a band is pre-defined in the spec, but it should be clearly specified as TDD-only in the note as this FG is for MSS band only. - If defined per UE, there is a need to differentiate between TDD and FDD, as this feature is for TDD only. It is preferable to define the FG as per band to ensure better flexibility and forward compatibility. If additional TDD bands are introduced for IoT NTN in the future with different features, defining the FG per UE could lead to implementation challenges. - Mandatory or Optional with signalling This should be a mandatory feature for UEs operating on the MSS band. However, from a signaling design perspective, it would be preferable to also introduce a capability signaling. This would help ensure consistency between the UE’s declared capability and the subsequent scheduling behavior by the network. Therefore, the feature should be conditionally mandatory with capability signaling for UEs that indicate support for the 1616–1626.5 MHz MSS band. - Update or deletion of components based on agreements In the previous RAN1 meeting, agreements were reached regarding SIB1 dropping and the introduction of new PRACH periodicities. There are two possible approaches to handling these changes: one way is to update the relevant components and include the new agreements once made. Another way is to delete the components and refer directly to the corresponding RAN1 agreements for details. Proposal 1. Agree on the FG in table1 with the following clarifications - Prerequisite feature group is Rel-17 2-1b - Capture SIB1-NB dropping and new periodicities of PRACH resources in the components - Type is set to per band - No need of FDD/TDD differentiation - Add ‘TDD only’ in the note Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1 transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. Support of new periodicities of PRACH resources. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)]; [Rel-17 2-1b]; Yes; N/A; IoT-NTN TDD mode is not supported; [Per UE or Per band] Per band; [TDD only]; [N/A]; Applicable only for 1616-1626.5 MHz MSS allocated band; FFS mandatory with capability signalling for UEs that indicate support of the 1616-1626.5MHz MSS band TDD only. |
| Nokia | Proposal 1: Adopt the proposed FGs and components in Table 1 for Rel-19 UE features for NTN for Internet of Things (IoT) TDD mode. Proposal 2: Adopt the proposed FGs as per band as it is special for band 1616-1626.5 MHz MSS band. Index; Feature group; Components; Prerequisite feature groups; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support NPRACH periodicities of 90 ms and 180 ms. Do not support NPRACH periodicities of 40 ms and 80 ms. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)]; [Rel-17 2-1b]; Per band. |
| Samsung | - Components: Rel-17 2-1b should be captured as a prerequisite, as this feature is for NTN operation and requires support for Rel-17 2-1b as a basic feature. Otherwise, further details may need to be explained in the components. - Need for FDD/TDD differentiation: It should be TDD-only, according to the WID. There is no motivation to consider this feature in FDD. - Mandatory/Optional: As this feature requires new changes on NPSS/NSSS/NPBCH, it should be optional without capability signaling. Proposal 1: Consider the above updates for IoT-NTN TDD mode. |
| ZTE | As lists in Table 1, following changes are suggested: - Regarding pre-requisite, Rel-17 2-1b is preferred to be pre-requisite of the FG 2-1. For Rel-18 feature, this is taken as the pre-requisite already and same way is expected for Rel-19 feature. Additionally, it’s also beneficial to let the UE to report whether some Rel-18 features are supported or not since the same pre-requisite is shared. For example. if beneficial, the UE can additionally report to support the GNSS position fix in connected state (Rel-18 2-3b) as combined UE feature with Rel-19. Otherwise, additional FG will need to be specified to support the legacy IoT-NTN UE features in IoT-NTN TDD mode, which complicates the specification and design. - Regarding the column of type, “per band” is preferred since IoT-NTN TDD mode will only be supported for 1616-1626.5 MHz MSS allocated band. UE does not need to support this feature in other bands. - The FG is only applicable for TDD band. Hence, the column of FDD/TDD differentiation can be “TDD only”, and no need to consider the mixture of TDD and FDD. - Regarding optional/mandatory, the basic feature for IoT-NTN (i.e., Rel-17 2-1b) is written as “optional with capability signaling” with an additional sentence “For UEs supporting NB-IoT NTN, it must indicate this FG is supported”. Similar wording can be used as shown in Table 1. - Considering that following agreement is achieved, SIB1-NB can be added in the component 5. An additional component 7 can be added to specify the support of new NPRACH periodicities considering that postponement related enhancement has been captured in component 6. And an additional component 8 can be added to specify the support of adjusting pre-compensation before the beginning of UL burst. Proposal 1: Adopt the feature groups in Table 1 for Rel-19 IoT-NTN TDD mode. Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. NPRACH periodicities of 90ms and 180ms are supported instead of 40ms and 80ms in NB-IoT NTN TDD mode 8. UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)]; [Rel-17 2-1b]; Yes; N/A; IoT-NTN TDD mode is not supported; [Per UE or Per band]; [TDD only]; [N/A]; Applicable only for 1616-1626.5 MHz MSS allocated band; FFS Optional with capability signalling A UE supports access to satellite in 1616-1626.5 MHz MSS allocated band must support this FG. |
| Xiaomi | Proposal 1: Add in FG 2-1 component 7) 90ms and 180ms NPRACH periodicities are supported in NB-IoT NTN TDD mode instead of 40ms and 80ms. However, there should be some conditions attached to component 13 and component 16 to make sure that the transmission occasion after the application of the aforementioned Koffset/K_mac is indeed UL subframes rather than non-U NB-IoT subframes as defined in component 4. Even considering this point, we are convinced the Rel-17 2-1b FG can be used as a prerequisite FG and add some clarification note to address this potential confusion in the Rel-19 FG 2-1 description. We don’t think there is any explicit linkage that a UE supporting FG 2-1 can be used to derive the support for FG 2-1b, as the enhancements in FG 2-1b are orthogonal to those in FG 2-1. Therefore, a UE compliant with FG 2-1 may not inherently support FG 2-1b. In summary, we propose putting Rel-17 FG 2-1b as prerequisite FG to the Rel-19 FG 2-1 and if necessary add a clarification note as in the proposal below. Proposal 2: Set Rel-17 FG 2-1b as a prerequisite FG for Rel-19 FG 2-1. If needed, capture the following sentence to the Rel-19 FG 2-1 Note column "The application of timeline in component 13 and component 16 in Rel-17 FG 2-1b shall be subject to the fulfillment of the components in FG 2-1" From our perspective, this FG only works for IoT NTN UE operating over 1616-1626.5 MHz MSS allocated band. Since a Rel-19 IoT NTN UE only supports this band, there is not much difference in terms of indicating supporting this FG regardless of per UE or per band granularity. However, when we consider the indication of not supporting this FG, per UE granularity seems more efficient compared with the per band granularity indication since the UE can indicate not supporting this FG directly, avoiding unnecessary complexity in band-specific signaling for indication of not supporting this FG. Moreover all the IoT NTN FDD FGs are indicated per UE, thus a unified signaling structure can be preserved from both UE and NW perspective. In summary, we propose the following Proposal 3: The granularity for capability indication is "per UE". Proposal 4: The FG 2-1 only operates in TDD only mode and the relevant columns shall be filled accordingly. As previously mentioned, this FG is tailored for the Iridium band, ensuring seamless integration with existing IoT NTN infrastructure. Any Rel-19 IoT NTN TDD mode UE operating on this band shall mandatorily support this FG, to ensure the UE operates interference-free towards the Iridium system. Proposal 5: The FG type is mandatory if a UE indicates support of the 1616-1626.5 MHz MSS allocated band. |
| OPPO | Proposal 1: Capture dropping SIB1-NB in non-D NB-IoT subframes as the component of the FG for IoT-NTN TDD mode. |
| Apple | Proposal 1: Adopt the IoT-NTN TDD mode UE features in above table. Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU.; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; Per band; TDD only; No; Applicable only for 1616-1626.5 MHz MSS allocated band; Optional with capability signalling For UEs supporting NB-IoT NTN TDD mode, it must indicate this FG is supported. |
| Qualcomm | This contribution includes the following proposed changes to UE features for IOT NTN TDD mode: - We do not see the need to capture under “Components” all the agremenets made under the RAN1 work item. - 2-1b should be a prerequisite of 2-1. Removing this prerequisite would break the “chain” of features (e.g. HARQ-ACK disabling depends on 2-1b). - For the rest of yellow parts, we think there is no need to introduce an explicit capability signaling for this: support of 2-1 would depend on signalng of the TDD band (band 249). Proposal: Endorse the following changes to IOT NTN TDD mode UE features Features; Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 2. IoT_NTN_TDD; 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU.; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; Only applicable for band 249; TDD only; N/A; Applicable only for 1616-1626.5 MHz MSS allocated band; Mandatory without capability signaling for UEs supporting band 249. |
| NTT DOCOMO | For prerequisite, although it was proposed to include FG Rel-17 2-1b as prerequisite of this R19 FG, it was objected as using FG Rel-17 2-1b of signaling per UE may mean that UE supporting IoT-NTN TDD must support IoT NTN FDD. However, we do not think this concern is valid. Even when UE reports FG Rel-17 2-1b as support, the UE reports also which band is supported via supportedBandListEUTRA. In other words, this FG Rel-17 2-1b is applicable only to the supported band. If it is desirable for a UE to support IoT NTN TDD only, then the UE can report support of the TDD band only. Thus, the FG Rel-17 2-1b can be adopted as prerequisite features group. Regarding whether the FG of support of IoT-NTN TDD mode is mandatory or optional, it can be optional with capability signalling, and for UEs supporting 1616-1626.5MHz MSS allocated band, it must indicate this FG is supported. Dropping of SIB1-NB in non-D NB-IoT subframes was agreed in RAN1#120bis meeting, which can be captured in component 5. Proposal 1: Update FG 2-1 as follows. Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)]; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; Per UE; TDD only; N/A; Applicable only for 1616-1626.5 MHz MSS allocated band; Optional with capability signalling For UEs supporting 1616-1626.5MHz MSS allocated band, it must indicate this FG is supported. |
| Nordic | - [Rel-17 2-1b] A basic NTN feature should be a pre-requisite of NTN IoT TDD. All components from Rel-17 2-1b are applicable also to NTN IoT TDD. - [Per UE or Per band] For this feature there is no difference, i.e. Per UE = Per band, because there is only single band. On the other hand, if some other MNO would like to use this TDD protocol on a different band, it is better to have this feature “Per band”. - [Mandatory/Optional] Feature should be mandatory without capability signaling, it is enough that UE indicates support for the b249, then it automatically supports NTN IoT TDD protocol. - [TDD only] This one is tricky, because protocol follows FDD, even if band is TDD. In addition, this discontinuous TX/RX protocol can equally well run on FDD band. Whether someone would like to do so is of course questionable. |

- Discussion

Summary of companies’ views

- How to capture components in Rel-17 2-1b - Rel-17 2-1b as prerequisite FG: Huawei, vivo, Samsung, ZTE, Xiaomi, Apple, Qualcomm, Docomo, Nordic - New components to capture latest agreements - SIB1-NB drop: Huawei, vivo, Nokia, ZTE, OPPO, Docomo - New NPRACH periodicity: Huawei, Nokia, ZTE, Xiaomi - Pre-compensation before the beginning of UL burst: ZTE - Type - Per band: vivo, Nokia, ZTE, Apple, Nordic - Per UE: HW, Xiaomi, Docomo - TDD/FDD differentiation - TDD only: Huawei, Samsung, ZTE, Xiaomi, Apple, Qualcomm, Docomo, Nordic - Not needed: vivo - Capability interpretation for mixture of FDD/TDD - Not needed: Huawei, ZTE, Apple, Qualcomm, Docomo - Optional/Mandatory - Mandatory without capability signalling: Qualcomm, Nordic, vivo, Xiaomi, - (Conditionally) Mandatory with capability signalling: vivo, Xiaomi - Optional with capability signalling, UE must indicate support this FG if support 1616-1626.5 MHz MSS allocated band: Huawei, ZTE, Apple, Docomo - Optional without capability signalling: Samsung - Whether/How to apply other optional features of IoT-NTN FDD in previous releases - Huawei: Rel-17 FGs 2-2a and 2-3a, Rel-18 FGs 2-1e-1, 2-1f-1, 2-1g-1, 2-1e-2, 2-1f-2, 2-1g-2 and 2-2b can be inherited in Release-19 IoT-NTN TDD mode. - vivo: For optional features beyond Rel-17 FG 2-1b introduced in previous releases, no behavioural modification is needed for their operation under IoT TDD mode. Therefore, there is no need to introduce a new FG; the UE can simply indicate support for both IoT TDD mode and the respective features.

Question 1:

Companies are encouraged to share views on how to define “Mandatory/optional” column in FG 2-1:

- Alt-1: Mandatory without capability signalling for UEs supporting band 249

- Alt-2: Optional with capability signalling, and add a note that “UE supporting band 249 must indicate the support of this FG”.

- Alt-3: Optional without capability signalling, and add a note that “UE supporting band 249 must support this FG”.

| Company | Comment |
| --- | --- |
| Moderator | It seems there is a common understanding that this FG is essential for the operation in band 249, while not required at all for the other operations. In that sense I believe the question that RAN1 has here should be almost editorial. While all the alternatives seem to be workable, here are some views from moderator: - Alt-1: It is a bit unclear if we can use “mandatory” here. Typically, for features added in later releases for a RAT, we will see it as “optional”. Although Alt-1 has a good clarification that the “mandatory” is subject to the support of a particular band, I am not very confident that usage of “mandatory” is correct or not from RAN2 36.306 implementation point of view. - Alt-2: This might be the safest approach with a cons that is unclear need for having signalling. If 3GPP may support more TDD bands in later releases, I think Alt-2 should be the choice. However, it is unclear how likely such other TDD bands will be specified in NR. - Alt-3: Seems workable as well. Not sure if there is a clear disadvantage. No practical difference with the other two though. Since there may be something I am not aware yet, I would like to invite more inputs from companies. |
| Vivo1 | We support Alt2. As moderator explained, this is the safer solution compared with option1. Regarding the signalling, there is no harm to have signalling for double confirmation. |
| Nordic | Alt-1 or Alt-3 OK for us. Alt-2 we cannot support. We do not think there is a need to have extra capability bit. Indicating support of b249 along with R17 NTN is clear indication that UE supports this feature. |
| Nokia, NSB | Alt-1 and Alt-3 are both fine. If UE access into the cell in band 249, then UE does not need to indicate the basic UE feature for IoT NTN TDD mode again. |
| Xiaomi | Alt -1 |
| NTT DOCOMO | We prefer Alt-2. |
| Qualcomm | Either Alt-1 or Alt-3 |
| ZTE | In Rel-17 2-1b (Basic IoT over NTN support for NB-IoT) the solution similar to Alt-2 was applied, i.e., optional with capability signaling and specifying “For UEs supporting NB-IoT NTN, it must indicate this FG is supported”. Hence, Alt-2 is preferred as what we do previously. |
| Huawei, HiSilicon | Alt 2. Similar happens to R17 FG2-1b |
| Iridium | Alt-1 is preferred, Alt-3 is acceptable. These options minimize info sent OTA |
| Apple | Alt-2 is preferred. |

Proposal 2:

Update Components for FG 2-1 as follows.

- Add “SIB-NB” in component 5

- Add a new component “Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms”

- Remove “[FFS other component(s)/FG(s)]”

| 2-1 | Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)] |
| --- | --- | --- |

| Company | Comment |
| --- | --- |
| Vivo1 | Support to remove “[FFS other component(s)/FG(s)]”, there is no need to capture every detail of TDD mode. |
| Nordic | OK to update 5. Component 7 is unnecessary. | Component 7 is unnecessary. |
| Nokia, NSB | We support these updates as aligned with RAN1 agreements. |
| Xiaomi | Support |
| NTT DOCOMO | Support |
| Huawei, HiSilicon | support |
| Iridium | Support |
| Apple | OK |

Proposal 3:

- Confirm “Rel-17 2-1b” as a perquisite FG for 2-1.

- Remove “[FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.]” from FG2-1 component

| Company | Comment |
| --- | --- |
| Vivo1 | Support |
| Nordic | Support |
| Nokia, NSB | Support. |
| Xiaomi | Support |
| NTT DOCOMO | Support |
| Qualcomm | Support |
| ZTE | Support |
| Huawei, HiSilicon | We hope to add a note to clarify it does not imply TDD UE always support FDD operation. |
| Iridium | Support |
| Apple | Support |

Question 4:

Companies are encouraged to share views on how to define “Type”:

- Alt-1: Per UE

- Alt-2: Per band

| Company | Comment |
| --- | --- |
| Vivo1 | We prefer per band because we are not sure if in the future additional TDD bands will be introduced for other purposes. Option2 has better compatibility |
| Nordic | It is not applicable in our view, if there is no capability bit. |
| Nokia, NSB | Alt 2 as it is clear mentioned in WID that the WI is specially for the band but not discuss/define for any other band. “Per UE” will make things confused. |
| Xiaomi | Alt 1 |
| NTT DOCOMO | As this FG only applies for band 249, Alt-1 is slightly preferred. |
| Qualcomm | We would like to just add a not “This FG applies only to B249” |
| ZTE | Per band is preferred since the TDD feature will be apply to band 249. UE does not need to support this FG for other supported bands. |
| Huawei, HiSilicon | Alt 1 |
| Apple | Per band. |

Proposal 5:

- Put “TDD only” in “Need of FDD/TDD differentiation” column in FG 2-1.

- Put “n/a” in “Capability interpretation for mixture of FDD/TDD” column in FG 2-1.

| Company | Comment |
| --- | --- |
| Vivo1 | If option2(per band) is adopted in proposal5, there is no need to put TDD only, because the half duplex mode for a band is clearly specified in RAN4 spec. If option1(per UE) is adopted, there is a need to differentiate between TDD and FDD, as this feature is for TDD only. |
| Nordic | Not applicable to this feature |
| Nokia, NSB | Support. |
| Xiaomi | Support. |
| NTT DOCOMO | Support. |
| Qualcomm | Support |
| ZTE | Support |
| Huawei, HiSilicon | support |
| Iridium | Support |
| Apple | Support |

- Conclusions

TBD

References

- R1-2503287	UE features for IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2503394	Discussion on UE features for IoT-NTN TDD mode	vivo

- R1-2503407	NTN for Internet of Things (IoT) TDD mode UE features	Nokia

- R1-2503600	UE features for IoT-NTN TDD mode	Samsung

- R1-2503642	Discussion on the UE feature for IoT-NTN TDD	ZTE Corporation, Sanechips

- R1-2503908	Discussion on UE features for IoT-NTN TDD mode	Xiaomi

- R1-2504198	Discussion on UE features for IoT-NTN TDD mode	OPPO

- R1-2504359	On Rel-19 IoT-NTN TDD mode UE Features	Apple

- R1-2504428	UE features for IOT-NTN TDD Mode	Qualcomm Incorporated

- R1-2504531	Discussion on UE features for for IoT-NTN TDD mode	NTT DOCOMO, INC.

- R1-2504577	On R19 TDD IoT-NTN features	Nordic Semiconductor ASA

Appendix: IoT-NTN TDD mode UE features after RAN1#120bis

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_TDD | 2-1 | Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)] | [Rel-17 2-1b] | Yes | N/A | IoT-NTN TDD mode is not supported | [Per UE or Per band] | [TDD only] | [N/A] | Applicable only for 1616-1626.5 MHz MSS allocated band | FFS |