3GPP TSG RAN WG1 #121                                                            R1-2504686

St Julian, Malta, May 19th – 23rd, 2025

Source:	vivo, BUTP

Title:	Views on channel model adaptation/extension of TR38.901 for 7-24GHz

Agenda Item:	9.8.2

Document for:	Discussion and Decision

Introduction

A new study item of channel modeling for new spectrum (7-24GHz) has been approved in RAN #102 [1], and accordingly, in RAN1 #120bis [2], the agreements are made as follows:

Agreement-1 For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS) is determined as: - For the nth UE, the Prn is generated using a truncated normal distribution within [0,1] Prn=Nμ, σ2; where 0≤Prn≤1. - FFS: the value of μ, σ. - Note: It should be fixed in RAN1#120-bis; - For kth cluster of the nth user, if the xk,n is smaller than Prn, the cluster is impacted by the SNS, where the xk,n is defined as xk,n~U(0,1)) Agreement-2 For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, generate the visibility region as a rectangle randomly located at a corner of the antenna array with dimension a*b, where a~U(Vn*W,W), b=Vn*H*W/a, H is the antenna array height in vertical dimension and W is the antenna array width in horizontal dimension, Vn is calculated by Vn=Α∙e-maxPn-Pnλ+Β+δ. where A=0.15, B=0.45, λ=33, δ~N(0,0.0015) for UMa scenario, and n is a cluster index - FFS: The value for other scenarios. - Note: It should be fixed in RAN1#120-bis; - FFS: the details of randomly located VR Agreement-3 For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, - For clusters without SNS, the power attenuation factor αs,n is set to 1. - For clusters with SNS, the power attenuation factor αs,n is generated by αs,n=1 ,xs-x0,n≤a,ys-y0,n≤be-xs-xa,nDn∙C ,xs-x0,n>a,ys-y0,n≤b e-ys-yb,nDn∙C ,xs-x0,n≤a,ys-y0,n>b e-xs-xa,n2+ys-yb,n2Dn∙C ,others where, Dn=xA,n-xa,n2+yB,n-yb,n2 , xA,n, yB,n is the coordinate of the other corner of the antenna array on the diagonal with reference corner x0,n, y0,n, and xa,n, yb,n is the coordinate of the other corner of the rectangular VR on the diagonal with reference corner x0,n, y0,n. The xs, ys denotes the coordinate of the antenna element out of VR region. C is the roll-off factor between the visible and invisible regions. - FFS: The value of C. - Note: It should be fixed in RAN1#120-bis; Agreement-4 For the modelling of spatial non-stationarity at UE side, introduce 3 sets of fixed attenuation values, each set corresponding to a range of frequency, Antenna index; Power attenuation (dB). One hand grip; Dual hand grip; Head and one hand grip. Below 1 GHz; (1-8.4) GHz; 14.5-15.5 GHz; Below 1 GHz; (1-8.4) GHz; 14.5-15.5 GHz; Below 1 GHz; (1-8.4) GHz; 14.5-15.5 GHz. 1; -; 0.7; 1.0; -; 11.0; 3.8; -; 3.7; 4.0. 2; -; 4.1; 2.4; -; 1.1; 3.8; -; 4.1; 3.7. 3; -; 3.3; 3.8; -; 10.5; 3.8; -; 4.3; 3.3. 4; 13.6; 7.2; 3.6; 5.8; 5.6; 2.6; 15.1; 7.8; 3.3. 5; -; 10.8; 3.8; -; 1.5; 1.0; -; 11.7; 3.3. 6; -; 9.1; 2.4; -; 1.4; 1.0; -; 10.1; 3.7. 7; -; 0.7; 1.0; -; 1.3; 1.0; -; 2.9; 4.0. 8; 2.4; 0.6; 1.0; 5.6; 6.2; 2.6; 4.9; 4.2; 4.0. Note: For below 1GHz, the model is only applicable for 2-antenna system with antenna location index 4 and 8 Note: The selection of number of antenna and antenna index will be determined as part of the assumption for simulation. Agreement-5 For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS): - In UMa, the distribution of Prn is determined as: μ=0.56, σ=0.20; Agreement-6 For near-field channel, the scaling factor (sUE,n) used to generate the d2 is: - For number of k1 non-direct paths, sUE,n=1. - For number of k2 non-direct paths: - Option-1: sUE,n=1-sTRP,n; - Note 1: It’s assumed that only single non-specular interaction along with specular reflection(s) is considered in the propagation channel - Note 2: At least most of the sources of non-specular interaction are assumed to be a point scatterer or diffractions. Agreement-7 For near-field channel, to generate the d1 for the k2 non-direct paths, the distribution of scaling factor sTRP,n is: - For the UMi scenarios, a Beta distribution with α=1.53, β=1.42 - For the UMa scenarios, a Beta distribution with α=1.93, β=1.33 - For the InH scenarios, a Beta distribution with α=1.25, β=1.27 Conclusion For near-field channel, to generate the d1 for k2 non-direct paths: - No additional lower bound (i.e., dmin) of d1 is defined. Agreement-8 In the Indoor office scenario for near-field channel, k1= 4. Agreement-9 In the Indoor factory scenarios, to generate the d1 non-direct path for the near-field channel model: - k1= 4; - For the k2 non-direct paths, the distribution of scaling factor sTRP,n is a Beta distribution with α=1.38, β=1.26 Agreement-10 For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, to generate the power attenuation factor αs,n for clusters with SNS, the value of C (i.e., the roll-off factor between the visible and invisible regions) is: - C= 13 for UMa. Agreement-11 For the modelling of spatial non-stationarity, the visibility region, i.e., a rectangle, is randomly located at a corner of the antenna array by following approach: - The location of the VR is jointly determined by following conditions: - The VR is located at either left or right part in horizontal domain of antenna array with equal probability of 1/2. - The probability of VR at the upper part of antenna array is [0.8] and the probability of VR at the lower part of antenna array is [0.2];

This contribution is organized as follows:

- First of all, we are discussing the remaining issue regarding the absolute delay towards the sub-cluster.

- Then, the issue of spatial non-stationarity is addressed, two models for spatial non-stationarity modeling at BS side are compared.

Near-field Channel Modeling

Remaining Issue on Indirect Path

In the previous meeting, it has been agreed that the  k1 clusters are the strongest of the total number of clusters in the channel. In the channel model in TR 38.901, the two strongest clusters are mapped to sub-clusters, which is described in the table as follows:

For the two strongest clusters, say n = 1 and 2, rays are spread in delay to three sub-clusters (per cluster), with fixed delay offset. The delays of the sub-clusters are (7.5-26) where is cluster delay spread specified in Table 7.5-6. When intra-cluster delay spread is unspecified (i.e., N/A) the value 3.91 ns is used; it is noted that this value results in the legacy behaviour with 5 and 10 ns sub-cluster delays. Twenty rays of a cluster are mapped to sub-clusters as presented in Table 7.5-5 below. The corresponding offset angles are taken from Table 7.5-3 with mapping of Table 7.5-5. Table 7.5-5: Sub-cluster information for intra cluster delay spread clusters sub-cluster #; mapping to rays; Power; delay offset. 10/20; 0. 6/20; 1.28. 4/20; 2.56.

As listed in the table, the delays between different sub-clusters are different. Whether to calculate the absolute delay independently or use the same absolute delay for different sub-clusters in the same cluster, and how to calculate the distance d1 in this case, need further clarification.

For the  k1 clusters, the distance d1 is equal to the speed of light times the absolute delay of the cluster. According to formula 7.6-43 in section 7.6.9 of TR38.901, the absolute delay of the sub-cluster i in cluster n is defined as τn,i+d3Dc+Δτ. It can be observed that the absolute delay is different between different sub-clusters in the same cluster. Hence, the distance d1,n should be generated independently for the sub-cluster i in cluster n, and defined as

d1,n,i=sTRP,n∙d3D+cτn,i+cΔτ.

where i=0,1,2.

- The distance d1,n is generated independently for different sub-clusters in the same cluster, using the formula for the sub-cluster i as d1,n,i=sTRP,n∙d3D+cτn,i+cΔτ.

Spatial Non-Stationarity Modeling

Blocker-based Model vs. Stochastic-based Model

For spatial non-stationarity modeling at BS side, two models of blocker-based and stochastic-based were agreed. In the following, we compared two models from the perspectives of compatibility, comprehensiveness and calibration.

In the compatibility, the main consideration is whether it can be easily applied to link-level simulation. For link-level simulation, the performance of a single link is evaluated, and the relevant coordinates are typically not modeled. For stochastic-based model, it belongs to statistical channel modeling. The agreed stochastic-based model could be easily applied to the link-level simulation without any modification. However, for blocker-based model, it belongs to deterministic channel modeling and requires explicitly modeling the coordinates of the blockers. Therefore, the agreed blocker-based model is not suitable for link-level simulation.

In the comprehensiveness, the main consideration is the causes of spatial non-stationary characteristics. There are two main reasons; one is that the object with limited size might not completely block the entire antenna array with a large antenna aperture, denoted as blockage. The other is that the object with limited size in the propagation environment might no longer serve as the complete scatterer for the entire antenna array with a large antenna aperture, denoted incomplete scatterer. For stochastic-based model, the abstract concept of VR can reflect both reasons of spatial non-stationarity characteristic. However, for blocker-based model, it could only reflect or model one reasons of blockage.

In the calibration, the main consideration is the alignment of the calibration results. For stochastic-based model, the steps of modeling are relatively clear and fixed, and company could easily align the simulation implementation based on the agreed steps, thus it is relatively easy to achieve alignment of the calibration results. However, for blocker-based model, many of the steps as the procedure are up to proponent implementations. For example, for a specific scenario, the type of blockers and the number of blockers are uncertain, and these uncertainties are likely to cause deviations in the calibration results of each proponent.

According to the above discussions, in our opinion, the stochastic-based model is the more appropriate model, it's easier to scale, more comprehensive, and easier to implement and calibrate.

- The stochastic-based model is the more appropriate to model the spatial non-stationarity phenomenon at BS side.

Conclusions

In this contribution, we have elaborated the realization of near-filed channel modeling in consideration of the antenna element-wise channel parameters associated with indirect paths. Then, two models for spatial non-stationarity modeling at BS side are compared. The observations and proposals are summarized as follows.

Proposal 1: The distance d1,n is generated independently for different sub-clusters in the same cluster, using the formula for the sub-cluster i as d1,n,i=sTRP,n∙d3D+cτn,i+cΔτ.

Proposal 2: The stochastic-based model is the more appropriate to model the spatial non-stationarity phenomenon at BS side.

References

- RP-234018, “New SID: Study on channel modelling enhancements for 7-24GHz for NR”, Nokia, Nokia Shanghai Bell.

- 3GPP RAN1 #120bis, Chair notes RAN1#120bis, Wuhan, China, April 7th – 11th, 2025.