3GPP TSG RAN WG1 #121			R1-2504745

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.5.1

Source: 	Moderator (LG Electronics)

Title: 	Summary #2 of on-demand SSB for NES

Document for:	Discussion and decision

# Introduction

This is the summary document for agenda item 9.5.1 on-demand SSB for NES based on the contributions listed in reference section.

# L1/L3 measurement based on on-demand SSB

| Company | Views |
| --- | --- |
| [3] ZTE | Proposal 6: Introduce the on-demand SSB resource configuration information in existing CSI resource Configurations. Proposal 7: It is unnecessary to associate different CSI report types with the signaling method that indicating the on-demand SSB transmission Proposal 8: Beam failure detection based on OD-SSB is not supported. Proposal 9: Beam failure recovery based on OD-SSB is supported. |
| [4] vivo | Proposal 15: If the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, UE should measure only the most recent, no later than the CSI reference resource, occasion between OD-SSB occasion and AO-SSB occasion. Proposal 16: To distinguish whether AO-SSB or OD-SSB is associated with a CSI report configuration, indicate the SSB type in the resource config associated with the CSI report configuration. Proposal 17: Semi-persistent and aperiodic CSI report configuration can be associated with any kind of SSB. Proposal 18: Periodic CSI report configuration can only be associated with on-demand SSB that is activated through RRC signalling and will be deactivated along with the deactivation of SCell. Proposal 19: Do not support on-demand SSB for LTM. Observation 1: SSB is not used for BFD on Scell in legacy. Proposal 21: Do not support on-demand SSB for BFD on Scell. |
| [5] Nokia | Observation 2: The CSI resources are associated with a DL BWP. As per the current specification, the csi-SSB-ResourceList is the list of SSB-Index values which identify an SS-Block within the SSB burst. We do not see the need to modify the Resource setting. Proposal 10: RAN1 to confirm that no additional signaling is required to indicate that CSI-report is associated with OD-SSB only. Proposal 11: CSI report configuration can be associated with on-demand SSB regardless of signaling mechanism for indicating OD-SSB transmission Proposal 12: For OD-SSB where od-ssb-nrOfBurst parameter is configured, only aperiodic CSI-reporting shall be configured. Otherwise, the reportConfigType shall be periodic, semi-persistent or aperiodic. Proposal 13: On-demand SSB work is intended to enhance SCell operation, and we propose to keep mobility measurements and LTM out of scope of on-demand SSB. |
| [6] NEC | Proposal 9: UE expects that on-demand SSB indication is received and UE can measure on-demand SSB before UE transmits a CSI report corresponding to a CSI reporting configuration associated with on-demand SSB. Proposal 10: Before SCell activation, UE expects that gNB configures one of on-demand SSB or always-on SSB for all configured periodic SSB measurements. Proposal 11: For Case #2, after SCell activation, down-select one of the following alternatives regarding OD-SSB measurements and link recovery: - Alternative 1: UE relies on periodic measurements of always-on SSB for SCell link monitoring/recovery procedures. UE measurements based on OD-SSB (when configured) are restricted to aperiodic reporting types. - Alternative 2: Support using on-demand SSB for SCell link recovery procedures. Proposal 12: For aperiodic CSI reporting based on on-demand SSB consider one of the following options: - Option-1: Support group-common based DCI indication for on-demand SSB indication - Option-2: Support indication of on-demand SSB within the CSI report trigger indication |
| [7] Spreadtrum | Proposal 3 It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “notConfigured”. |
| [8] TCL | Proposal 5: Discussion on the impact of OD-SSB transmission including case #1 and case #2 on BFD reporting periodicity or detection timer. |
| [9] Samsung | Proposal 6: For Case 2 (i.e., with always-on SSB), only when always-on SSB and on-demand SSB have different center frequencies, a CSI report configuration is associated with on-demand SSB only. Proposal 7: For L1 measurement based on on-demand SSB: - Before on-demand SSB is indicated to be transmitted, the UE is not required to perform L1 measurement or CSI reporting even though it receives the configuration on L1 measurement and reporting; - When on-demand SSB is indicated to be transmitted in Scenario #2, the UE can be required to perform CSI reporting in Scenario #2. |
| [11] Ofinno | Proposal 12: Regarding “FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”., we propose not to agree. Proposal 13: CSI measurement associated only with the OD-SSB can be configured for both same and different frequencies of AO-SSB and OD-SS in Case #2. Proposal 14: Periodic, aperiodic and semi-persistent reporting of the L1 measurements on OD-SSB are supported. Proposal 15: For a CSI report configuration configured with reportConfigType set to periodic, Alt 2 is supported: - Alt 2: The CSI report configuration can be associated with on-demand SSB regardless of signaling mechanism for indicating OD-SSB transmission. Proposal 16: Do not define UE behavior on L1 measurement for on-demand SSB before on-demand SSB is indicated to be transmitted or after on-demand SSB transmission is deactivated. Proposal 17: The beam failure detection (BFD) based on OD-SSB is supported in Case # 1 and Case # 2. Proposal 18: The candidate beam detection (CBD) based on OD-SSB is supported in Case # 1 and Case # 2. |
| [12] CATT | Proposal 3: Deprioritize the discussion of LTM based on on-demand SSB in Rel-19. Proposal 4: Consider two candidate solutions to add on-demand SSB resource configuration to existing CSI resource configuration. - Alt-1: The existing IE CSI-ResourceConfig should include the on-demand SSB resource configuration information. - Alt-2: A new dedicated resource configuration IE for on-demand SSB resource configuration should be introduced, e.g. CSI-ResourceConfig-NES. Proposal 5: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., always-on SSB is periodically transmitted on the cell), when a CSI report configuration is associated with both AO-SSB and OD-SSB: - It should apply for the case where AO-SSB and OD-SSB have the same center frequency and should not apply for the case where AO-SSB and OD-SSB have the different center frequencies. - In this case, the UE shall derive the channel measurements for computing L1-RSRP/L1-SINR reported in uplink slot n based on only the most recent occasion of SS/PBCH associated with the CSI resource setting, and the occasion is no later than the CSI reference resource, if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. Proposal 5: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., always-on SSB is periodically transmitted on the cell), when a CSI report configuration is associated with both AO-SSB and OD-SSB: - It should apply for the case where AO-SSB and OD-SSB have the same center frequency and should not apply for the case where AO-SSB and OD-SSB have the different center frequencies. - In this case, the UE shall derive the channel measurements for computing L1-RSRP/L1-SINR reported in uplink slot n based on only the most recent occasion of SS/PBCH associated with the CSI resource setting, and the occasion is no later than the CSI reference resource, if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. Proposal 6: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., always-on SSB is periodically transmitted on the cell), for a CSI report configuration is associated with OD-SSB only, in addition to the case where AO-SSB and OD-SSB have the different center frequencies, it should apply for the case where AO-SSB and OD-SSB have the same center frequency as well. Proposal 7: In order to support a CSI report configuration being associated with both of AO-SSB and OD-SSB or OD-SSB only using a single CSI-ReportConfig IE for Case #2 (i.e., always-on SSB is periodically transmitted on the cell), the reporting configuration for AO-SSB in the CSI-ReportConfig IE should be optional. Proposal 8: It is up to gNB implementation whether OD-SSB and always-on SSB have same beam or not. Proposal 9: Consider two candidate solutions to add on-demand SSB reporting configuration to existing CSI reporting configuration. - Alt-1: The existing IE CSI-ReportConfig should include the on-demand SSB reporting configuration information. - Alt-2: A new dedicated reporting configuration IE for on-demand SSB reporting configuration should be introduced, e.g. CSI-ReportConfig-NES. Proposal 10: Consider two candidate solutions to activate and deactivate semi-persistent L1 measurement reporting on PUCCH for on-demand SSB. - Alt-1: The existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE should include the activation and deactivation of SP CSI reporting on PUCCH for on-demand SSB, e.g., one of the reserved bits can be used to indicate whether the MAC CE applies to SP CSI reporting on PUCCH Activation/Deactivation for on-demand SSB or not. - Alt-2: A new dedicated MAC CE should be introduced for activation and deactivation of semi-persistent L1 measurement reporting on PUCCH for on-demand SSB. Proposal 11: Consider two candidate solutions to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB. - Alt-1: The existing DCI field CSI request is reused to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB, and the existing DCI field Transform precoding indicator is used to indicate the DCI is used to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB, or for legacy MIMO/LTM. - Alt-2: A new dedicated RNTI (e.g., OD-SSB-SP-Reporting-RNTI) for DCI format 0_1 and 0_2 should be introduced for triggering of semi-persistent L1 measurement reporting on PUSCH for on-demand SSB. Proposal 12: Consider two candidate solutions to support the semi-persistent L1 measurement reporting on PUSCH for multiple on-demand SSBs from multiple SCells. - Alt-1: The existing IE CSI-SemiPersistentOnPUSCH-TriggerState should include multiple CSI-ReportConfigIds. Each CSI-ReportConfigId is associated with one on-demand SSB resource configuration information. - Alt-2: A new dedicated trigger state IE for on-demand SSB should be introduced, e.g. CSI-SemiPersistentOnPUSCH-TriggerState-NES. Proposal 13: Consider the following solution to trigger aperiodic L1 measurement reporting on PUSCH for on-demand SSB. - A new dedicated RNTI (e.g., OD-SSB-Aperiodic-Reporting-RNTI) for DCI format 0_1 and 0_2 should be introduced for triggering of aperiodic L1 measurement reporting on PUSCH for on-demand SSB. |
| [13] CMCC | Proposal 7: Update the previous RAN1 agreement as follows. Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, and aperiodic L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point |
| [14] Xiaomi | Proposal 8: UE should preclude measurement result associated to a SSB which is turned off by gNB. Proposal 9: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - both of AO-SSB and OD-SSB - Applies for the case where AO-SSB and OD-SSB have the same/different center frequencies and the same positions in the same SSB burst - It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. - OD-SSB only - Applies for the case where AO-SSB and OD-SSB have the same/different center frequencies Proposal 10: For a cell supporting on-demand SSB SCell operation and for L1 measurement based on on-demand SSB - For Scenario #2 (Case #1 & Case #2), Scenario #2A (Case #1 & Case #2), Scenario #3B (Case #1 & Case #2), semi-persistent and aperiodic CSI report are supported. Proposal 11: Beam failure detection based on OD-SSB is not supported for Case #1 and Case #2. Proposal 12: Beam failure recovery based on OD-SSB is supported for both Case #1 and Case #2. |
| [15] InterDigital | Proposal 3: If the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, the UE derives L1 measurement result based on one of AO-SSB or OD-SSB that is the most recent Proposal 4: Support CSI report configuration associated with only OD-SSB at least when the center frequency locations of AO-SSB and OD-SSB are different |
| [16] Ericsson | Proposal 7 If OD-SSB positions in burst is a subset of AO-SSB positions in burst, NW can associate a CSI report only with OD-SSB by configuring in the CSI report only the SSB indices that are in the OD-SSB burst. |
| [17] Google | Proposal 4: For CSI report configuration with OD-SSB only, UE reports the SSBRI based on the activated OD-SSB only. Proposal 5: For CSI report configuration with OD-SSB only, when all the OD-SSBs are deactivated, UE assumes the CSI report configuration is deactivated. Proposal 6: Support an LTM CSI report configuration can be associated with - Both AO-SSB and OD-SSB - OD-SSB only |
| [18] China Telecom | Proposal 8: If a CSI report configuration is associated with both of AO-SSB and OD-SSB, the AO-SSB and OD-SSB can have same/different center frequency, and it is up to UE to measure both SSBs or only OD-SSB. Proposal 9: The case that CSI report configuration is only associated with OD-SSB can only applied for the case where AO-SSB and OD-SSB can have different center frequency. |
| [21] Fujitsu | Proposal 5: For a CSI report configuration associated with both AO-SSB and OD-SSB, if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, the UE shall measure the most recent SSB between OD-SSB and AO-SSB. Proposal 6: Support that a CSI report configuration associated with only OD-SSB is applied at least for the case where AO-SSB and OD-SSB have different center frequency location. Proposal 7: Support a CSI report configuration associated with only AO-SSB, particularly when AO-SSB and OD-SSB have different center frequency locations (i.e., AO-SSB is CD-SSB while OD-SSB is NCD-SSB). - A CSI report configuration associated with OD-SSB for a short time to enable fast SCell activation, and another CSI report configuration associated with AO-SSB for long-term beam management and measurement. Observation 2: In the scenario where AO-SSB and OD-SSB have different center frequency locations (i.e., AO-SSB is CD-SSB while OD-SSB is NCD-SSB), if a CSI report configuration associated with only AO-SSB is supported, it is essential for a UE to determine whether a configured CSI report configuration is associated with AO-SSB or OD-SSB. Proposal 8: In the scenario where AO-SSB and OD-SSB have different center frequency locations (i.e., AO-SSB is CD-SSB while OD-SSB is NCD-SSB), if a CSI report configuration associated with only AO-SSB is supported: - The CSI resource configuration enhancement can be introduced to differentiate on-demand SSB and always-on SSB. Proposal 9: For CSI resource configuration enhancement for on-demand SSB associated with CSI report configuration, the following two options can be considered. - Option A: CSI resource enhancement at the level of the resource set list - To specify a dedicated resource set list for on-demand SSB. - Option B: CSI resource enhancement at the level of CSI-SSB resource set - To introduce an on-demand SSB indicator within the CSI-SSB-ResoureSet. Proposal 10: If a newly defined resource set parameter, e.g., CSI-OD-SSB-ResourceSet, is supported, the corresponding enhancement in the aperiodic trigger state list should be considered as well. - E.g., CSI-OD-SSB-ResourceSet should be included in CSI-AperiodicTriggerStateList. Proposal 11: For a CSI report configuration configured with reportConfigType set to periodic, support Alt 1 which aligns with the legacy CSI configuration mechanism. - The CSI report configuration can be associated with on-demand SSB indicated by RRC and cannot be associated with on-demand SSB indicated by MAC CE. Proposal 12: None of the following configurations are considered as on-demand SSB indicated by RRC: - Option 1: an on-demand SSB configuration comprising od-ssb-nrofBurst. - Option 2: an on-demand SSB configuration activated by RRC (via an ‘active’ state) while deactivated by MAC CE. - Option 3: an on-demand SSB configuration activated by MAC CE while deactivated by RRC. Observation 3: In the existing spec on SCell, only periodic CSI-RS can be configured for beam failure detection purposes, while either SSB or periodic CSI-RS can be configured for beam failure recovery. Proposal 13: Support candidate beam selection based on on-demand SSB in Case#1. |
| [26] Panasonic | Proposal 1: L3 measurement based on on-demand SSB should be supported in general for all Scenarios. Proposal 2: L1 measurement when on-demand SSB is ON should be supported at least for Scenario 2A, 3A and 3B i.e., after the SCell activation command is received. On Scenario 2, on-demand SSB based L1 measurement for LTM is not supported. Proposal 3: For RRC triggered/indicated on-demand SSB, the periodic, semi-persistent and aperiodic CSI report should be supported with current CSI mechanism. Proposal 4: For MAC CE triggered/indicated on-demand SSB, semi-persistent and aperiodic CSI report should be supported with enhancement that on-demand SSB is expected to be available when CSI report is active. On periodic CSI report, although our first preference is not to support, we can be flexible if the specification impact can be minimized. Proposal 5: For group common DCI triggered/indicated on-demand SSB, if supported, only semi-persistent and aperiodic CSI report should be supported. Proposal 10: O_cpu for L1-RSRP measurement should be counted as 1, despite whether both or one of AO-SSB and OD-SSB are/is measured. Proposal 11: When a CSI report is associated with both AO-SSB and OD-SSB, the CPU occupied OFDM symbols should be from the first symbol of either AO-SSB or OD-SSB resource, respective latest SSB occasion no later than the corresponding CSI reference resource, despite whether both or one of AO-SSB and OD-SSB are/is measured. Proposal 12: When both AO-SSB and OD-SSB are configured and associated with a CSI report, and if the SSB beams in a burst are different, and if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, legacy UE behaviour is reused to determine which set of SSB to measure and select the reported SSBRI for the best beam. Proposal 13: When both AO-SSB and OD-SSB are configured and associated with a CSI report, and if the SSB beams in a burst are different, to consider below two alternatives: - Alt 1 (preferable): Both sets of SSBs are measured. The best beam SSBRI is selected from the beams of the union of AO-SSB and OD-SSB. For the measurement of the beams with the same SSB indices (i.e., QCLed) between two sets of SSB, it is allowed and up to UE implementation to measure one of or both QCLed SSB beams. - Alt 2: Only OD-SSB is measured. The best beam SSBRI is selected from the beams of OD-SSB. |
| [27] LG Electronics | Proposal #1: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), if AO-SSB and OD-SSB have the different center frequencies, - A CSI report configuration can be associated with OD-SSB only, - In addition, considering the case where OD-SSB is deactivated during Scenario #3B, - Alt 1: A CSI report configuration can be associated with both of AO-SSB and OD-SSB. - Alt 2: A CSI report configuration can be associated with AO-SSB only. Proposal #2: Introduce new higher layer parameters ‘od-ssb-Index-RSRP’, ‘od-ssb-Index-SINR’, ‘od-ssb-Index-RSRP-Index’ and ‘od-ssb-Index-SINR-Index’ as components of reportQuantity-r19 for a CSI report configuration. - If reportQuantity for a CSI report configuration is set to ‘od-ssb-Index-RSRP’, ‘od-ssb-Index-SINR’, ‘od-ssb-Index-RSRP-Index’ or ‘od-ssb-Index-SINR-Index’, the CSI report configuration is associated with OD-SSB only. - Otherwise, if reportQuantity for a CSI report configuration is set to ‘ssb-Index-RSRP’, ‘ssb-Index-SINR’, ‘ssb-Index-RSRP-Index’ or ‘ssb-Index-SINR-Index’, the CSI report configuration is associated with both of AO-SSB and OD-SSB. Proposal #3: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), if the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", - If OD-SSB is activated for the SCell, - The UE shall derive the channel measurements for computing L1-RSRP or L1 SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of “OD-SSB” associated with the CSI resource setting. - Otherwise, if OD-SSB is deactivated for the SCell, - The UE shall derive the channel measurements for computing L1-RSRP or L1 SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of “AO-SSB” associated with the CSI resource setting. Proposal #4: For a CSI report configuration configured with reportConfigType set to periodic, support Alt 2 (i.e., The CSI report configuration can be associated with OD-SSB regardless of signaling mechanism for indicating OD-SSB transmission). Proposal #5: Before OD-SSB is indicated to be transmitted or after on-demand SSB transmission is deactivated, the UE is not required to perform L1 measurement or CSI reporting even though it has received the CSI report configuration for L1 measurement and reporting based on OD-SSB. Proposal #15: For Case #1, and for Case #2 where AO-SSB and OD-SSB have the same center frequency, OD-SSB can be used for the following purposes. - Beam failure recovery - Path-loss estimation reference signal - QCL reference signal |
| [29] Apple | Proposal 11: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - If a CSI report configuration is associated with both of AO-SSB and OD-SSB, - if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, - which SSB to measure between the most recent AO-SSB no later than the CSI reference resource and the most recent OD-SSB no later than the CSI reference resource is up to UE implementation as long as RAN4 requirements are met. Proposal 12: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - OD-SSB only - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies |
| [30] Qualcomm | Proposal 6: When a CSI report configuration is associated with both AO-SSB and OD-SSB, it is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. |
| [31] NTT DOCOMO | Observation 3: It is important for NW to know which RS to be used for the measurement when the timeRestrictionForChannelMeasurement is set to “configured”. Proposal 10: For a CSI report configuration that can be associated with both AO-SSB and OD-SSB, - We support not to let UE implementation for which SSBs to be used when timeRestrictionForChannelMeasurement is set to “configured”. Proposal 11: For a CSI report configuration that can be associated with OD-SSB only, - Support this regardless of the frequency location between AO-SSB and OD-SSB are the same or different. Observation 4: In order for NW to know which configured SCell and beam is best to be activated with minimizing NW TX, L1 measurement based on OD-SSB is beneficial. Proposal 12: Support L1 measurement based on OD-SSB in scenario #2/case #1 by reusing the existing LTM mechanism. |
| [32] ITRI | Observation 1: For the conventional CSI report behaviour, if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of NZP CSI-RS associated with the CSI resource setting. Proposal 1: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with both of AO-SSB and OD-SSB - If the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of NZP CSI-RS associated with the CSI resource setting. - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst Observation 2: If UE selects AO-SSB for CSI measurement and report (e.g. CSI report configuration associated with AO-SSB only), CSI report may be updated in a long periodicity related to the periodicity of AO-SSB; and thus, UE may not report the most recent CSI measurement result to gNB. Proposal 2: For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - At least for the case where AO-SSB and OD-SSB have the different center frequencies, CSI report configuration can be associated with OD-SSB only |
| [33] KDDI | Proposal 1: If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements based on only the most recent, no later than the CSI reference resource, occasion of AO-SSB or OD-SSB. Proposal 2: A CSI report configuration associated with OD-SSB only applies at least for the case where AO-SSB and OD-SSB have the different center frequencies. Proposal 3: CSI report configuration configured with reportConfigType set to periodic can be associated with on-demand SSB indicated by RRC. |

Agreement (RAN1#118) - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement (RAN1#120bis) For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - both of AO-SSB and OD-SSB - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. - OD-SSB only - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

## [Moderator’s note] Following issues are identified on L1/L3 measurement based on on-demand SSB.

- Issue #1: CSI report configuration associated with AO-SSB and/or OD-SSB

- Issue #2: How to differentiate whether a CSI report configuration is associated with both AO-SSB and OD-SSB or associated with OD-SSB only

- Issue #3: Whether/how to associate OD-SSB with a CSI report configuration configured with reportConfigType set to periodic

- Issue #4: UE behavior if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”

- Issue #5: Whether to support LTM

- Others

  - Panasonic: O_cpu = 1 as in legacy

  - Panasonic: When a CSI report is associated with both AO-SSB and OD-SSB, the CPU occupied OFDM symbols should be from the first symbol of either AO-SSB or OD-SSB resource, respective latest SSB occasion no later than the corresponding CSI reference resource, despite whether both or one of AO-SSB and OD-SSB are/is measured.

  - NEC: GC-DCI based A-CSI triggering

## [Moderator’s note #1] For Issue #1 (i.e., CSI report configuration associated with AO-SSB and/or OD-SSB):

- When can a CSI report configuration be associated with both of AO-SSB and OD-SSB?

  - Only for the same center frequency case: CATT

  - For the same and different center frequency cases: Xiaomi, China Telecom, LG Electronics

- When can a CSI report configuration be associated with OD-SSB only?

  - Only for the different center frequency case: Samsung, InterDigital, China Telecom, Fujitsu, LG Electronics, Apple, ITRI, KDDI

  - For the same and different center frequency cases: Ofinno, CATT, Xiaomi, NTT DOCOMO

- Additionally support AO-SSB only: Fujitsu, LG Electronics

- Google: For CSI report configuration with OD-SSB only, UE reports the SSBRI based on the activated OD-SSB only.

- Panasonic: When both AO-SSB and OD-SSB are configured and associated with a CSI report, and if the SSB beams in a burst are different

  - Both sets of SSBs are measured. The best beam SSBRI is selected from the beams of the union of AO-SSB and OD-SSB. For the measurement of the beams with the same SSB indices (i.e., QCLed) between two sets of SSB, it is allowed and up to UE implementation to measure one of or both QCLed SSB beams.

### [Closed] Proposal #2-1 (CSI report configuration):

- For a cell supporting on-demand SSB SCell operation,

  - For Case #1 (i.e., No always-on SSB on the cell),

    - A CSI report configuration is associated with OD-SSB only,

      - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

    - A CSI report configuration can be associated with both of AO-SSB and OD-SSB,

      - Alt 1-1: Only for the case where AO-SSB and OD-SSB have the same center frequency

        - Supported by Samsung, Google, Futurewei, Fujitsu, CMCC, Apple, TCL, Qualcomm, Spreadtrum, CATT, China Telecom

      - Alt 1-2: For the case where AO-SSB and OD-SSB have the same center frequency and the case where AO-SSB and OD-SSB have the different center frequencies

        - Supported by Google, LG Electronics, Xiaomi, vivo, Nokia, Panasonic, KDDI, NTT DOCOMO, Ofinno, Lenovo

      - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

    - A CSI report configuration can be associated with OD-SSB only,

      - Alt 2-1: Only for the case where AO-SSB and OD-SSB have the different center frequencies

        - Supported by Samsung, Google, Futurewei, Fujitsu, Apple, Nokia, TCL, Qualcomm, Panasonic, KDDI, Spreadtrum, China Telecom, Ofinno

        - Qualcomm: When AO-SSB and OD-SSB have the different center frequencies, UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated

      - Alt 2-2: For the case where AO-SSB and OD-SSB have the same center frequency and the case where AO-SSB and OD-SSB have the different center frequencies

        - Supported by Xiaomi, CMCC, NTT DOCOMO

      - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

    - Alt 3: A CSI report configuration can be associated with AO-SSB only,

      - Only for the case where AO-SSB and OD-SSB have the different center frequencies

      - UE reports SSBRI based on SSB-index corresponding to AO-SSB.

      - Supported by Fujitsu, vivo

  - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

Companies are encouraged to provide views on Proposal #2-1 and preference between alternatives.

| Company | Views |
| --- | --- |
| Moderator | From the moderator’s understanding, - Alt 1-2 can be needed to allow UE to measure AO-SSB after OD-SSB is deactivated, when two SSBs have the different center frequencies. That is, without Alt 1-2, a CSI report configuration can be associated with OD-SSB only for the case of different center frequencies of AO-SSB an OD-SSB, so UE cannot measure AO-SSB after OD-SSB is deactivated. - Alt 2-2 can be needed if gNB wants UE to measure only OD-SSB, even if both SSBs have the same center frequency. - If at least one of Alt 1-2 and Alt 2-2 is taken, the method to differentiate whether a CSI report configuration is associated with both AO-SSB and OD-SSB or associated with OD-SSB only, needs to be introduced as in Proposal #2-2. |
| Samsung | We support Alt 1-1 and Alt 2-1 to simplify the design. Didn’t see a use case for configurability. |
| Google | OK with Case 1. For Case 2, we think the first sub-bullet is sufficient. We failed to see the necessity for others. |
| ZTE, Sanechips | For the use case, we think at least Alt 1-2 should be supported. Moreover, we suggest Proposal #2-1 and #2-2 can be discussed together, since the same issue (i.e., how to differentiate the OD-SSB and AO-SSB in RAN1) is considered under both two solutions. |
| LGE | Especially, Alt 1-2 is needed for the case where there is no OD-SSB in Scenario 3B. If A CSI report configuration can be also associated with both of AO-SSB and OD-SSB for the case where AO-SSB and OD-SSB have the different center frequencies. |
| Futurewei | OK with the proposal. OK for Case #1, Alt 1-1mand Alt 2-1 seem to be sufficient. |
| Xiaomi | From functionality perspective, there is no difference between OD-SSB and AO-SSB. Especially RAN1 agreed that AO-SSB and OD-SSB are contained in the same BWP, there may be no reason to limit the case only AO-SSB and OD-SSB have different frequency center. All in all, we agree with FL’s assessment and support alt 1-2/2-2. |
| Fujitsu | For SSBRI reporting in Case #1, we think that it should follow the legacy way. As specified in TS 38.214 shown below, SSBRI refers to the entry number in the associated csi-SSB-ResourceList. We suggest to capture this in the proposal. TS 38.214 clause 5.2.1.4.2 … If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-RSRP' or 'ssb-Index-RSRP- Index', the UE shall report SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet. …. For Case #2, we support Alt 1-1 and Alt 2-1. In addition, when AO-SSB and OD-SSB have different center frequencies, A CSI report configuration associated with AO-SSB only should also be supported. Otherwise, CSI reporting based on SSB cannot be performed when OD-SSB is deactivated. Moreover, for the case where a CSI report configuration is associated with either AO-SSB or OD-SSB, as discussed above, it also should be clarified that the SSBRI indicates the entry number in the associated csi-SSB-ResourceList. To sum up, we suggest the following updates of the proposal: ### [HIGH] Proposal #2-1 (CSI report configuration): - For a cell supporting on-demand SSB SCell operation, - For Case #1 (i.e., No always-on SSB on the cell), - A CSI report configuration is associated with OD-SSB only, - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB. - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with both of AO-SSB and OD-SSB, - Alt 1-1: Only for the case where AO-SSB and OD-SSB have the same center frequency - Alt 1-2: For the case where AO-SSB and OD-SSB have the same center frequency and the case where AO-SSB and OD-SSB have the different center frequencies - UE reports SSBRI based on SSB-index corresponding to AO-SSB or based on SSB-index corresponding to the currently being transmitted OD-SSB. - A CSI report configuration can be associated with OD-SSB only, - Alt 2-1: Only for the case where AO-SSB and OD-SSB have the different center frequencies - Alt 2-2: For the case where AO-SSB and OD-SSB have the same center frequency and the case where AO-SSB and OD-SSB have the different center frequencies - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB. - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet. - A CSI report configuration can be associated with AO-SSB only, - Only for the case where AO-SSB and OD-SSB have the different center frequencies - UE reports SSBRI based on SSB-index corresponding to the AO-SSB. - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet. |
| vivo | When OD-SSB and AO-SSB are in different frequency, if OD-SSB is deactivated, the spec. should allow UE to continue the SSB measurement based on AO-SSB, i.e., alt.1-2. We think at least for periodic and semi-persistent CSI report, such operation is reasonable. By the way, for case 2, whether to support measurement based on only AO-SSB should be clarifed. There are two understandings, 1) measurement based on only OA-SSB is not supported, and 2) measurement based on AO-SSB is supported by default since it is legacy behavior. Our preference is 2). |
| NEC | For Case #1 (No always-on SSB): We agree that a CSI report configuration is associated with OD-SSB only, and the UE reports SSBRI based on the SSB-index corresponding to the currently transmitted OD-SSB. For Case #2 (Always-on SSB is periodically transmitted): CSI report configuration associated with both AO-SSB and OD-SSB: We support that a CSI report configuration can be associated with both AO-SSB and OD-SSB, and it applies at least for the case where they have the same center frequency and the same positions in the same SSB burst. We are open to discussing whether this should extend to different center frequencies as well. CSI report configuration associated with OD-SSB only: We support that a CSI report configuration can be associated with OD-SSB only at least for the case where AO-SSB and OD-SSB have different center frequencies. We are open to discussing extending this to the same center frequency case. |
| CMCC | Alt1-1 is preferred than Alt1-2 to avoid UE to perform RF retuning between different SSB when AO-SSB and OD-SSB have different center frequency. Alt2-2 is slightly preferred than Alt2-1 since it can give more flexibility from gNB side. |
| Apple | Alt 1-1 and Alt 2-1 are preferred with the following modification. - For a cell supporting on-demand SSB SCell operation, - For Case #1 (i.e., No always-on SSB on the cell), - A CSI report configuration is associated with OD-SSB only, - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with both of AO-SSB and OD-SSB, - Alt 1-1: Only for the case where AO-SSB and OD-SSB have the same center frequency - Alt 1-2: For the case where AO-SSB and OD-SSB have the same center frequency and the case where AO-SSB and OD-SSB have the different center frequencies - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used for reporting is up to UE implementation. - A CSI report configuration can be associated with OD-SSB only, - Alt 2-1: Only for the case where AO-SSB and OD-SSB have the different center frequencies - Alt 2-2: For the case where AO-SSB and OD-SSB have the same center frequency and the case where AO-SSB and OD-SSB have the different center frequencies - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB. |
| Nokia, NSB | For Case#2 our view is that Alt 1-2 and Alt 2-1 should be selected. |
| TCL | We think Alt 1-1 and 2-1 is enough to consider CSI report configuration. |
| Qualcomm | For Case #2, when a CSI report configuration can be associated with both of AO-SSB and OD-SSB, Alt 1-1 should be sufficient. When a CSI report configuration can be associated with OD-SSB only, we prefer Alt 2-1 with clarification that UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated. This clarification should also be needed for Alt 2-1 when OD-SSB and AO-SSB on different center frequencies. Hence, a new bullet under “A CSI report configuration can be associated with OD-SSB only,”: - When AO-SSB and OD-SSB have the different center frequencies, UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated |
| Panasonic | We support Alt 1-2 and Alt 2-1 for flexible network operation for NES. |
| KDDI | For Case #1, we are fine with the FL’s proposal. For Case #2, if a CSI report configuration is associated with both of AO-SSB and OD-SSB, it can apply for both of the case where AO-SSB and OD-SSB have the same center frequency and the case where AO-SSB and OD-SSB have the different center frequencies, while if a CSI report configuration is associated only with OD-SSB, then it can apply at least for the case where AO-SSB and OD-SSB have the different center frequencies. So, we prefer Alt 1-2 and Alt 2-1. |
| Spreadtrum | Support Alt 1-1 and 2-1 to simplify the design |
| DCM | We support Alt. 1-2 and Alt. 2-2. For Alt. 1-2, when the center frequencies are the same between AO-SSB and OD-SSB, the SSB properties between AO-SSB and OD-SSB are the same for the same SSB index. Thus, a CSI report configuration can be associated with both Ao-SSB and OD-SSB. Similarly, in the current agreements, at least the case where both AO-SSB and OD-SSB with different frequency have QCL assumption is supported, so SSB properties are the same for both AO-SSB and OD-SSB. For Alt. 2-2, it is not necessary to make restriction regardless of the center frequency between AO-SSB and OD-SSB are same or different. One of the usecase for the same center frequency is when the union of the SSBs are non-periodic. |
| CATT | Alt1-1 is preferred |
| China Telecom | Prefer Alt 1-1 and Alt 2-1. In our contribution we prefer Alt 1-2, but now we think Alt 1-1 should be adopted, reporting the SSB measurement with different center frequency in one CSI report makes no sense. With such configuration, even if OD-SSB and AO-SSB are QCLed, associate the CSI report with OD-SSB is though. |
| Ofinno | We support the proposal on Case # 1. For Case # 2, we support Alt 1-2 (when CSI report configuration is associated with both AO-SSB and OD-SSB) and Alt 2 (when CSI report configuration is associated with only OD-SSB ). |
| Lenovo | Prefer Alt.1-2 to allow UE measure AO-SSB when OD-SSB is deactivated. |

### [Closed] Proposal #2-1a (CSI report configuration):

- For a cell supporting on-demand SSB SCell operation,

  - For Case #1 (i.e., No always-on SSB on the cell),

    - A CSI report configuration is associated with OD-SSB only,

      - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

    - A CSI report configuration is associated with both of AO-SSB and OD-SSB,

      - For the case where AO-SSB and OD-SSB have the same center frequency and the case where AO-SSB and OD-SSB have the different center frequencies

      - For the case where AO-SSB and OD-SSB have the same center frequency,

        - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

      - For the case where AO-SSB and OD-SSB have the different center frequencies,

        - Option 1: UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB if present, otherwise, UE reports SSBRI based on SSB-index corresponding to AO-SSB.

        - Option 2: UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

    - A CSI report configuration can NOT be associated with OD-SSB only

      - This reverts previous RAN1 agreement.

  - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

Companies are encouraged to provide views on Proposal #2-1a and preference between alternatives.

| Company | Views |
| --- | --- |

### [HIGH] Proposal #2-1b (CSI report configuration):

- For a cell supporting on-demand SSB SCell operation,

  - For Case #1 (i.e., No always-on SSB on the cell),

    - A CSI report configuration is associated with OD-SSB only,

      - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

    - For the case where AO-SSB and OD-SSB have the same center frequency,

      - A CSI report configuration is associated with both of AO-SSB and OD-SSB,

      - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

    - For the case where AO-SSB and OD-SSB have the different center frequencies,

      - Option 1’: A CSI report configuration can be associated with AO-SSB only or associated with OD-SSB only. NEW RRC parameter is introduced to differentiate CSI report configuration associated with AO-SSB only and CSI report configuration associated with OD-SSB only.

        - If a CSI report configuration is associated with OD-SSB only, UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

          - Note: UE is not expected to be configured with SSB-index that is not present in the currently transmitted OD-SSB

          - Huawei, ZTE, IDC, Ericsson, Nokia, FW, DOCOMO, Fujitsu,

        - If a CSI report configuration is associated with AO-SSB only, UE reports SSBRI based on SSB-index corresponding to AO-SSB.

      - Option 2: A CSI report configuration is associated with both of AO-SSB and OD-SSB.

        - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

          - Qualcomm, Xiaomi, Panasonic, Samsung, OPPO, China Telecom, Ofinno

    - This reverts previous RAN1 agreement.

  - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

Companies are encouraged to provide views on Proposal #2-1b and preference between alternatives.

| Company | Views |
| --- | --- |
| Qualcomm | For “NEW RRC parameter is introduced to differentiate CSI report configuration associated with AO-SSB only and CSI report configuration associated with OD-SSB only.” in Option 1’, we may not need to introduce new RRC parameter. The SSB frequency (e.g., od-ssb-absoluteFrequency) can be part of CSI-SSB-ResourceSet to inform UE on which SSBs are configured. For example, if od-ssb-absoluteFrequency is not configured, it is AO-SSB. Otherwise, it is OD-SSB. |
| LGE | Opton1 is supported |

## [Moderator’s note #2] For Issue #2 (i.e., How to differentiate whether a CSI report configuration is associated with both AO-SSB and OD-SSB or associated with OD-SSB only):

- By enhancing CSI resource configuration

  - Supported by ZTE, vivo, CATT, Fujitsu

    - Fujitsu: If a newly defined resource set parameter, e.g., CSI-OD-SSB-ResourceSet, is supported, the corresponding enhancement in the aperiodic trigger state list should be considered as well. (e.g., CSI-OD-SSB-ResourceSet should be included in CSI-AperiodicTriggerStateList.)

- By introducing new higher layer parameters ‘od-ssb-Index-RSRP’, ‘od-ssb-Index-SINR’, ‘od-ssb-Index-RSRP-Index’ and ‘od-ssb-Index-SINR-Index’ as components of reportQuantity-r19 for a CSI report configuration

  - Supported by LG Electronics

- Differentiation is not needed

  - Supported by Nokia, Ericsson

<Captured from [3] ZTE>

<Captured from [21] Fujitsu>

Figure 4 CSI resource enhancement in the level of resource set list

Figure 5 CSI resource enhancement in the level of resource set

### [MID] Proposal #2-2 (CSI resource configuration):

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), [if Alt 1-2 and/or Alt 2-2 in Proposal #2-1 is taken,]

  - Introduce a NEW higher layer parameter XYZ within the IE CSI-SSB-ResourceSet, as an indicator to differentiate whether both AO-SSB and OD-SSB are associated or OD-SSB only is associated.

Companies are encouraged to provide views on Proposal #2-2.

| Company | Views |
| --- | --- |
| Moderator | Proposal #2-2 can be discussed after making a decision for Proposal #2-1. Nevertheless, please provide your views if you have another alternative as way forward. It is noted that Proposal #2-2 is made based on the majority view and for minimizing specification impact as described in [21] Fujitsu. |
| Google | This depends on the outcome of proposal #2-1. If we need to differentiate it, an RRC parameter is the straight-forward way, but we failed to see the motivation for this. |
| InterDigital | Same understanding with Google |
| ZTE, Sanechips | OK with FL’s proposal. We suggest that Proposal #2-1 and #2-2 can be discussed together, since the same issue (i.e., how to differentiate the OD-SSB and AO-SSB in RAN1) is considered under both two solutions. |
| LGE | If Alt 1-2 and/or Alt 2-2 in Proposal #2-1 is taken, an indicator which can inform whether both AO-SSB and OD-SSB are associated or OD-SSB only is associated, is needed. Without the indicator, NW can’t know which SSB the measurement is based on, when L1-RSRP or L1 SINR is reported in uplink slot n. |
| Futurewei | Same understanding as Google. |
| Xiaomi | Our feeling is that we may not need this proposal as it is RAN2 work. RAN1’s design is crystal clear that a CSI report can be associated with either OD-SSB only or AO-SSB and OD-SSB. We agree from configuration perspective, UE needs to have the capability to differentiate which SSB(s) are associated with a CSI report so as to avoid ambiguous UE behavior. But for the signaling design, it can be up to RAN2. |
| Fujitsu | We agree that it depends on the outcome of proposal #2-1. For case #2, if a CSI report configuration can be associated with either OD-SSB only or AO-SSB only, even if neither Alt 1-2 nor Alt 2-2 is supported, differentiation between OD-SSB and AO-SSB is needed. |
| vivo | We support to introduce such indicator to distinguish the associated SSB. However, we have a concern to drop the legacy behavior, we think CSI report based on AO-SSB only should be allowed as well, which is legacy behavior. Regarding the highlighted part, we think even if alt 2-1 are supported, we also need to discuss the indicator. In our understanding, if we support the measurement based on AO-SSB only as we commented in proposal 2-1 (i.e., understanding 2 in previous comment), we need the indicator to indicate whether the measurement is based on AO-SSB only or OD-SSB only. |
| NEC | Support |
| CMCC | Same understanding as Google. |
| DCM | We are fine with the proposal |
| China Telecom | Agree with Google. |
| Ofinno | We generally support the proposal. But it is also fine to first conclude Proposal # 2-1 |

## [Moderator’s note #3] For Issue #3 (i.e., Whether/how to associate OD-SSB with a CSI report configuration configured with reportConfigType set to periodic):

- Alt 1: The CSI report configuration can be associated with on-demand SSB indicated by RRC and cannot be associated with on-demand SSB indicated by MAC CE.

  - Supported by vivo, Fujitsu, Panasonic

    - vivo: Periodic CSI report configuration can only be associated with on-demand SSB that is activated through RRC signalling and will be deactivated along with the deactivation of SCell

    - Fujitsu: None of the following configurations are considered as on-demand SSB indicated by RRC:

      - Option 1: an on-demand SSB configuration comprising od-ssb-nrofBurst.

      - Option 2: an on-demand SSB configuration activated by RRC (via an ‘active’ state) while deactivated by MAC CE.

      - Option 3: an on-demand SSB configuration activated by MAC CE while deactivated by RRC.

- Alt 2: The CSI report configuration can be associated with on-demand SSB regardless of signaling mechanism for indicating OD-SSB transmission.

  - Supported by ZTE, Ofinno, Panasonic (acceptable), LG Electronics, ITRI

- Alt 3: No support of periodic CSI reporting for OD-SSB

  - Supported by CMCC, Xiaomi

- Nokia: For OD-SSB where od-ssb-nrOfBurst parameter is configured, only aperiodic CSI-reporting shall be configured. Otherwise, the reportConfigType shall be periodic, semi-persistent or aperiodic.

- Before on-demand SSB is indicated to be transmitted, the UE is not required to perform L1 measurement or CSI reporting even though it receives the configuration on L1 measurement and reporting

  - Supported by Samsung, Google, LG Electronics

  - No need: Ofinno

### [HIGH] Proposal #2-3 (P-CSI reporting):

- For a cell supporting on-demand SSB SCell operation and for L1 measurement based on on-demand SSB,

  - For a CSI report configuration configured with reportConfigType set to periodic,

    - The CSI report configuration can be associated with OD-SSB regardless of signaling mechanism for indicating OD-SSB transmission.

  - Before OD-SSB is indicated to be transmitted or after OD-SSB transmission is deactivated, the UE is not required to perform L1 measurement or CSI reporting based on OD-SSB even though the UE has been provided with the CSI report configuration for L1 measurement based on OD-SSB.

Companies are encouraged to provide views on Proposal #2-3.

| Company | Views |
| --- | --- |
| Moderator | Proposal #2-3 may not be aligned with the principle of current NR specification, in that OD-SSB which can be (de)activated by MAC CE can be associated with periodic CSI reporting. However, if we prevent OD-SSB from being associated with periodic CSI reporting, further discussion on how to handle the following OD-SSB seems to be needed, as several companies already pointed out. - OD-SSB configured with od-ssb-nrofBurst - OD-SSB activated by RRC while deactivated by MAC CE - OD-SSB activated by MAC CE while deactivated by RRC Rather, to simplify the solution, any OD-SSB can be allowed to be associated with P-CSI and the last bullet of Proposal #2-3 can handle the case where P-CSI reporting is configured while OD-SSB has not been activated yet. |
| Samsung | Support. |
| Google | We suggest the second sub-bullet be changed into the follows: For CSI report configuration based on OD-SSB with none of the OD-SSBs being transmitted, UE assumed the CSI report configuration is deactivated. |
| InterDigital | We support the proposal, agree with moderator that the 2nd bullet can address the issue related to before OD-SSB is transmitted or after OD-SSB is deactivated. |
| ZTE, Sanechips | OK with FL proposal. In our view, there is no difference between MAC CE indicated OD-SSB and RRC indicated OD-SSB. So, there is no need to associate different CSI report types with the signaling method that indicating the on-demand SSB transmission. |
| LGE | Support it. The last bullet of Proposal #2-3 should be captured for handling the cases which are listed above. |
| Futurewei | Support |
| Xiaomi | Agree with the second sub-bullet. For the first sub bullet, more clarification is needed. From our understanding, periodic CSI reporting can only be associated with periodic reference signal. However, MAC CE based OD-SSB transmission doesn’t have a periodic character, it is more like semi-persistent transmission. From this perspective, we think the limit from vivo is valid, i.e., periodic CSI report can be only associated with OD-SSB triggered by RRC signaling. |
| Fujitsu | We are fine with this proposal in principle. Regarding the second sub-bullet, to the best our knowledge, the current specification only specifies that UE can drop a CSI report if some conditions are met. However, there is no existing specification on whether UE can skip L1 measurement. We suggest to leave the decision on whether to perform L1 measurement to UE implementation. Therefore, we propose the following updates on the second sub-bullet: - Before OD-SSB is indicated to be transmitted or after OD-SSB transmission is deactivated, the UE is not required to perform L1 measurement or CSI reporting based on OD-SSB even though the UE has been provided with the CSI report configuration for L1 measurement based on OD-SSB. - Before OD-SSB is indicated to be transmitted or after OD-SSB transmission is deactivated, it is up to UE implementation on whether to perform L1 measurement |
| Vivo | We also think the current proposal is not aligned with the principle of current NR specification. The current proposal seems a kind of semi-persistent CSI report, in this case, we may not further enhance the periodic CSI report to repeat the functionality of semi-persistent CSI report. Therefore, we propose 1) specify a real periodic CSI report based on RRC activated periodic OD-SSB, or 2) do not support periodic CSI report based on OD-SSB. |
| ETRI | Support |
| NEC | Support |
| CMCC | Three OD-SSB activation/deactivation cases mentioned by FL are all SP SSB transmission regardless of signaling mechanism for indicating OD-SSB transmission, and they can be associated with SP report and AP report. We prefer not to allow P report for SP RS as legacy. |
| Apple | We think application time after OD-SSB activation needs to be considered. We suggest the following: - For a cell supporting on-demand SSB SCell operation and for L1 measurement based on on-demand SSB, - For a CSI report configuration configured with reportConfigType set to periodic, - The CSI report configuration can be associated with OD-SSB regardless of signaling mechanism for indicating OD-SSB transmission. - Before m+3 Nslotsubframe,μ+1 (note: currently working assumption) slots after slot n, where n is a slot when a PDSCH reception for OD-SSB activation and adaptation providing the MAC CE ends, where n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211], before UE receives OD-SSB activation by RRC, OD-SSB is indicated to be transmitted or after UE receives OD-SSB deactivation, OD-SSB transmission is deactivated, the UE is not required to perform L1 measurement or CSI reporting based on OD-SSB even though the UE has been provided with the CSI report configuration for L1 measurement based on OD-SSB. |
| Nokia, NSB | Support |
| TCL | Support |
| Qualcomm | For the 1st bullet, we share the same view as Xiaomi. For the 2nd bullet, it is important to first understand when L1 measurement is needed before we agree on this bullet – since we see the risk that UE will have to measure OD-SSB in the cases not supported in legacy specs e.g., in Scenario 2 for deployments without LTM. Hence, discussion in the direction of 2nd bullet in Proposal #2-5 should be done first. |
| Panasonic | This is different from the current specification principle on the CSI resource for periodic CSI report. In our view, only RRC-based OD-SSB activation should support periodic CSI report. |
| KDDI | We share the same view as vivo. The limitation on OD-SSB with which a periodic CSI report configuration can be associated is needed. |
| DCM | Support |
| CATT | For the 1st bullet, we share the same view as Xiaomi. |
| China Telecom | Agree with Xiaomi. P CSI report can only be supported Periodic reference signal in current specs, while the MAC CE indicated OD-SSB is more like SP procedure. |
| Ofinno | We support |

## [Moderator’s note #4] For Issue #4 (i.e., UE behavior if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”):

- Based on recent SSB between AO-SSB and OD-SSB: vivo, CATT, InterDigital, Fujitsu, Panasonic, KDDI

- Up to UE implementation as long as RAN4 requirement is met: Xiaomi, Apple, Qualcomm

  - Objected by NTT DOCOMO

- Based on OD-SSB if activated: LG Electronics

- Not needed: Ofinno

- If the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “notConfigured”

  - Up to UE implementation: Spreadtrum, Apple

### [MID] Proposal #2-4 (CSI channel measurement):

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

  - For CSI-ReportConfig associated with both of AO-SSB and OD-SSB, if the higher layer timeRestrictionForChannelMeasurements is set to “Configured”,

    - The UE shall derive the channel measurements for computing L1-RSRP or L1 SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion between OD-SSB occasion and AO-SSB occasion associated with the CSI resource setting.

Companies are encouraged to provide views on Proposal #2-4.

| Company | Views |
| --- | --- |
| Samsung | If the intention of the proposal is up to UE implementation to choose the latest SSB for measurement, then we are supportive. |
| Google | We think this can be up to UE implementation. |
| InterDigital | Support |
| ZTE, Sanechips | We have agreed that when CSI report is associated with both AO-SSB and OD-SSB, which SSB to measure is up to UE implementation as long as RAN4 requirements are met. To this regard, for the case raised in proposal #2-4, the most recent SSB should also be up to UE implementation. |
| LGE | Agree it |
| Futurewei | We think that should be left for the implementation. |
| Xiaomi | We are not clear why this kind of restriction is needed. It seems it complicates UE behavior as UE is configured to prepare CSI report based on both OD-SSB and OA-SSB while only either OD-SSB or AO-SSB is allowed to be used in the end. More clarification is appreciated. |
| Fujitsu | Support |
| vivo | We think the intention is choose the latest SSB from AO and OD-SSB. This is aligned with legacy principle, so we are fine with this proposal. Especially, the computing time for the UE to perform measurement can be estimated by gNB side based on the latest SSB, gNB can properly schedule the UE based on the computing time. |
| NEC | Support |
| CMCC | Ok. |
| TCL | Support |
| Qualcomm | We prefer leaving it to UE implementation. The benefit of the proposal is unclear while it unnecessarily complicates UE implementation. |
| Panasonic | We are okay. |
| KDDI | Support. |
| DCM | We are fine with the proposal. If we cannot make agreements on this, we should make slight modification to the following. The UE shall derive the channel measurements for computing L1-RSRP or L1 SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion between OD-SSB occasion and AO-SSB occasion associated with the CSI resource setting if OD-SSB is indicated and only AO-SSB occasion associated with the CSI resource setting otherwise. |
| China Telecom | Support. |
| Ofinno | This behaviour should be left for UE implementation |

## [Moderator’s note #5] For Issue #5 (i.e., Whether to support LTM):

- Deprioritize LTM: vivo, Nokia, CATT, Panasonic (not in Scenario #2)

- Support LTM: Google, NTT DOCOMO

- Samsung: When on-demand SSB is indicated to be transmitted in Scenario #2, the UE can be required to perform CSI reporting in Scenario #2

### [Closed] Proposal #2-5 (LTM):

- For a cell supporting on-demand SSB SCell operation,

  - LTM based on OD-SSB is NOT supported in Rel-19.

  - L1 measurement based on OD-SSB for a serving/non-serving/neighbor cell during Scenario #2 is NOT supported in Rel-19.

    - Note: As per previous RAN1 agreement, Scenario #2 is when SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321).

Companies are encouraged to provide views on Proposal #2-5.

| Company | Views |
| --- | --- |
| Samsung | Support |
| Google | We think LTM based on OD-SSB does not require additional effort. It can be supported |
| InterDigital | Support |
| ZTE, Sanechips | Fine with FL’s proposal. |
| LGE | LTM should be deprioritized in Rel-19. We don’t have enough time to discuss LTM in Rel-19 NES Enhancement |
| Futurewei | OK |
| Fujitsu | Support |
| Vivo | Support the proposal |
| ETRI | Support |
| NEC | Support |
| CMCC | Support |
| Nokia, NSB | Support |
| TCL | Agree with this proposal |
| Qualcomm | We have some related discussion in #2-3. For the 2nd bullet, we should also add that L1 measurement based on OD-SSB on a non-serving cell or neighbor cell is not supported. |
| Panasonic | Support |
| DCM | We support at least L1 measurement based on OD-SSB in scenario #2/case #1. It is important that NW can know which configured SCell/beam is better and then determine which SCell/beam is to be activated in scenario #2/case #1 with minimizing NW TX (which means maximizing NES gain). Although the existing L1 measurement is not supported on deactivated SCell, the expected specification impact and UE implementation difficulty would not be large given that L1 measurement on non-serving cell is already supported in Rel-18 LTM. |
| China Telecom | Support. |
| Ofinno | Support |

### [HIGH] Proposal #2-5a (LTM):

Conclusion

For a cell supporting on-demand SSB SCell operation,

- LTM based on OD-SSB is NOT supported in Rel-19.

- L1 measurement based on OD-SSB for a serving cell in Scenario #2 is NOT supported in Rel-19.

  - Note: As per previous RAN1 agreement, Scenario #2 is when SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321).

- L1 measurement based on OD-SSB for a non-serving/neighbor cell is NOT supported in Rel-19.

Companies are encouraged to provide views on Proposal #2-5a.

| Company | Views |
| --- | --- |
| Moderator | The alternative of the yellow-highlighted part would be: - L1 measurement based on OD-SSB for a serving cell before completion of SCell activation is NOT supported in Rel-19. Please express your preference between yellow-color part and cyan-color part. |
| Qualcomm | We think it is more accurate to write that - L1 measurement based on OD-SSB for a serving cell in Scenarios #2, #2A and #3A is NOT supported in Rel-19. The reason is that according to the legacy specs, the UE only performs L1 measurement & reporting in Scenario #3B. Furthermore, we think that this should be agreement (to be captured in spec) instead of conclusion. Similar comment is applied to the agreed conclusion “L1 measurement based on OD-SSB for a non-serving/neighbor cell is NOT supported in Rel-19.” |
| LGE | We think that the yellow-color part is correct. According to our RAN4 delegators, when Sell is unknown, L1 measurement & Report is needed in Scenario #3A. 38.133-8.3.2 … - 6 ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3 ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. … TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1 and TReport=0. And the wording “L1 measurment” in the yellow-part is recommended to be modified. My understanding is that L3 measurement can be activated in Scenario #2 and L3 measurement is performed by L3 filtering with L1 measurement. So, without L1 measurement, L3 measurement cannot be performed. Actually, whether to measure any RS in Layer 1 may be up to UE implementation, without configuration on reporting. Our proposal: - L1 measurement reporting based on OD-SSB for a serving cell in Scenario #2 is NOT supported in Rel-19. |
| Fujitsu | We prefer the yellow highlighted part. As quoted by LG, the legacy specification allows L1 measurement based on SSB during the SCell activation procedure. Additionally, the same section contains further content that supports this, as shown below. 38.133 clause 8.3.2 Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.. Therefore, L1 measurement based on OD-SSB during the SCell activation procedure can also be permitted. This approach has the advantage of enabling fast acquisition of L1 RSRP reports, which ultimately facilitates a quicker SCell activation procedure. |
| Moderator | In addition, the following agreements were made in Athens. Agreement(RAN4#114): - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. - Based on OD-SSB periodicity |

# SSB collision and multiplexing

## RO validation and SSB-to-RO mapping

| Company | Views |
| --- | --- |
| [2] Huawei | Proposal 1: For Case#1, the SSBs for SSB-RO mapping should be the union of all the SSB indices configured by multiple od-ssb-PositionsInBurst. Proposal 2: For Case#2, the SSBs for SSB-RO mapping should be the union of all the SSB indices configured by multiple od-ssb-PositionsInBurst and by ssb-PositionsInBurst. Proposal 3: The RO validation should be dynamic, i.e., based on the actual transmission of OD-SSB. |
| [5] Nokia | Observation 3: For Case#1, RACH configuration may not be provided to the UE as AO-SSB is not configured. Proposal 16: For Case#2, the SSB to RO mapping is based on the ssb-PositionsInBurst defined for always-on SSB. For Case#1, if necessary, ssb-perRACH-Occasion kind of parameter needs to be configured for the OD-SSB. RAN1 should discuss the necessity of providing such configuration. |
| [9] Samsung | Proposal 8: - For UEs knowing the time and frequency resources of on-demand SSB, the same procedures for resource allocation (e.g., PDCCH, and PDSCH) and validation (e.g., PRACH, and PUSCH) based on always-on SSB can be reused for on-demand SSB. - For UEs not knowing the time and frequency resources of on-demand SSB, it is up to gNB implementation to handle resource allocation and validation based on always-on SSB. - RateMatchPattern needs enhancement to cover the time and frequency resources of on-demand SSB. |
| [11] Ofinno | Proposal 21: PRACH occasion and/or PUSCH occasion may be determined based on OD-SSB in Case #1. PRACH occasion and/or PUSCH occasion are determined only based on AO-SSB in Case #2. |
| [18] KT | Proposal 6: OD-SSB has less priority of transmission than the previously configured periodic signals, like RACH and CSI-RS. |
| [24] OPPO | Proposal 1: If no AO-SSB on the cell, the SSB indexes for RO mapping should be the union of all the configured candidate SSB indexes for OD-SSB bursts. Proposal 2: If AO-SSB is periodically transmitted on the cell, OD-SSB indexes should be same or a subset of AO-SSB indexes, and the legacy SSB-RO mapping relationship is reused. |
| [27] LG Electronics | Proposal #16: For Case #1, RO validation before SSB-to-RO mapping is performed by assuming that no OD-SSB is activated. Proposal #17: For Case #2, RO validation before SSB-to-RO mapping is performed based on only AO-SSB, as in legacy. Proposal #18: For Case #1, SSB-to-RO mapping is performed based on the union of od-ssb-PositionsInBurst parameters configured by od-ssb-config. Proposal #19: For Case #2, SSB-to-RO mapping is performed based on only AO-SSB, as in legacy. |
| [30] Qualcomm | Proposal 7: For OD-SSB Case #1, the following is considered for PRACH occasion validation rules and SSB-RO mapping rules: - For RRC based indication of OD-SSB transmission, the PRACH occasion validation rules and SSB-RO mapping rules are determined based on OD-SSB configuration in the RRC. - For MAC-CE based indication of OD-SSB transmission, the PRACH occasion validation rules and SSB-RO mapping rules are determined based on OD-SSB configuration in RRC and the latest MAC-CE indicating OD-SSB transmission at or before the time UE receives Scell activation command. Proposal 8: For OD-SSB Case #2, PRACH occasion validation rules and SSB-RO mapping rules are not impacted by OD-SSB transmission. |
| [35] CEWiT | Observation 4: On-demand SSB impacts the need and validity of periodic RACH occasions and their association with SSBs, requiring adjustments to ensure efficient RRC connection establishment. Proposal 4: Support handling the impacts of on-demand SSB on RACH occasions for RRC connection establishment. |

## [Moderator’s note] Company views on RO validation and SSB-to-RO mapping are as follows.

- RO validation before SSB-to-RO mapping

  - For Case #1,

    - Based on OD-SSB

      - Ofinno

    - Based on OD-SSB indicated by RRC or latest OD-SSB indicated by MAC-CE

      - Qualcomm

    - Without considering OD-SSB

      - LG Electronics

  - For Case #2,

    - Based on AO-SSB

      - Ofinno, LG Electronics, Qualcomm

  - Based on the actual transmission of OD-SSB

    - Huawei

- SSB-to-RO mapping

  - For Case #1,

    - Based on the union of all the SSB indices configured by multiple od-ssb-PositionsInBurst

      - Huawei, OPPO, LG Electronics

    - if necessary, ssb-perRACH-Occasion kind of parameter needs to be configured for the OD-SSB.

      - Nokia

    - Based on OD-SSB indicated by RRC or latest OD-SSB indicated by MAC-CE

      - Qualcomm

  - For Case #2,

    - Based on the union of all the SSB indices configured by multiple od-ssb-PositionsInBurst and by ssb-PositionsInBurst

      - Huawei

    - Based on the ssb-PositionsInBurst defined for always-on SSB

      - Nokia, OPPO, LG Electronics, Qualcomm

### [Closed] Proposal #3-1-1 (Case #1 RO):

- For a cell supporting on-demand SSB SCell operation and for Case #1 (i.e., No always-on SSB on the cell),

  - For RO validation before SSB-to-RO mapping,

    - Alt 1

      - For RRC based indication of OD-SSB transmission, RO validation rule is determined based on OD-SSB configuration in the RRC.

      - For MAC-CE based indication of OD-SSB transmission, RO validation rule is determined based on OD-SSB configuration in RRC and the latest MAC-CE indicating OD-SSB transmission at or before the time UE receives Scell activation command.

    - Alt 2

      - RO validation rule is determined without considering transmission occasions for OD-SSB.

    - Alt 3

      - RO validation rule is determined based on transmission occasions for the currently being transmitted OD-SSB.

  - For SSB-to-RO mapping,

    - Alt A

      - For RRC based indication of OD-SSB transmission, SSB-to-RO mapping rule is determined based on OD-SSB configuration in the RRC.

      - For MAC-CE based indication of OD-SSB transmission, SSB-to-RO mapping rule is determined based on OD-SSB configuration in RRC and the latest MAC-CE indicating OD-SSB transmission at or before the time UE receives Scell activation command.

    - Alt B

      - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by one or multiple od-ssb-PositionsInBurst.

Companies are encouraged to provide views on Proposal #3-1-1.

| Company | Views |
| --- | --- |
| Samsung | For RO validation before SSB-to-RO mapping, we support Alt 3. For SSB-to-RO mapping, we support one alternative similar to Alt 3 – the ssb-PositionsInBurst corresponding to currently being transmitted OD-SSB |
| Google | Support the proposal |
| InterDigital | For RO validation before SSB-RO mapping, we prefer not to complicate the design based on type of signaling and as such we prefer Alt 3. For SSB-to-RO mapping, we are fine with Alt B. |
| ZTE, Sanechips | - For RO validation before SSB-to-RO mapping, we think Alt 3 is preferred. - We think the Alt B, i.e., union of ssb-positionInburst should be considered for SSB-RO mapping. - In our understanding, OD-SSB would be the common signal for all the UEs in SCell, when one UE receives their MAC CE for updating the ssb-positionInburst in OD-SSB configuration, if another UE does not receive corresponding MAC CE yet or another UE is in the duration of processing the corresponding MAC CE, the SSB-RO mapping results would be different between two UEs. To this end, in order to ensure the SSB-RO-mapping results are same as all UEs, it is better to considered the union of all the SSB indices configured in OD-SSB configuration. |
| LGE | Alt B and Alt 2 are supported. |
| Futurewei | OK. Alt 3 and Alt B are preferred |
| Xiaomi | From our understanding, the most important thing is to make sure the SSB-to-RO mapping is fixed. In the other words, SSB-to-RO mapping should not be changed with OD-SSB ON and OFF. On the other hand, RO validation has to base on actual SSB transmission. Accordingly, we have the follow opinions: - For RO validation before SSB-to-RO mapping, - Support alt 3 - For SSB-to-RO mapping, - Support alt B |
| Fujitsu | We support Alt 3 and Alt A. |
| vivo | Regarding RO validation, we think case 1 and case 2 can have the same rule, our preference is Alt2 which is more semi-static and easy for implementation. If the RO validation is determined based on OD-SSB transmission, the PRACH resource may be per-UE validated, this is not preferred from system perspective. Regarding SSB RO mapping, altB is preferred. |
| ETRI | We support Alt 3 and Alt B. |
| NEC | Support to discuss the given options. |
| CMCC | For case 1, without legacy UEs on SCell, Alt 3 is supported for RO validation. Alt B is supported so that different R19 UEs with different activated OD-SSB index have the same SSB-to-RO mapping. |
| Apple | For both RO validation before SSB-to-RO mapping and SSB-to-RO mapping, we prefer the rules are determined based on activated OD-SSB rather than (multiple) configured OD-SSB. We suggest the following based on Alt 1 and Alt A. - For a cell supporting on-demand SSB Scell operation and for Case #1 (i.e., No always-on SSB on the cell), - For RO validation before SSB-to-RO mapping, - Alt 1 - For RRC based indication of OD-SSB transmission, RO validation rule is determined based on the latest activated OD-SSB transmission before deactivating OD-SSB transmission configuration in the RRC. - For MAC-CE based indication of OD-SSB transmission, RO validation rule is determined based on OD-SSB configuration in RRC and the latest MAC-CE indicating OD-SSB transmission at or before the time UE receives Scell activation command. - Alt 2 - RO validation rule is determined without considering transmission occasions for OD-SSB. - Alt 3 - RO validation rule is determined based on transmission occasions for the currently being transmitted OD-SSB. - For SSB-to-RO mapping, - Alt A - For RRC based indication of OD-SSB transmission, SSB-to-RO mapping rule is determined based on the latest OD-SSB transmission before deactivating OD-SSB transmission configuration in the RRC. - For MAC-CE based indication of OD-SSB transmission, SSB-to-RO mapping rule is determined based on OD-SSB configuration in RRC and the latest MAC-CE indicating OD-SSB transmission at or before the time UE receives Scell activation command. - Alt B - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by one or multiple od-ssb-PositionsInBurst. |
| Nokia, NSB | Our view is that in Case#1 support for RACH is not needed in the OD-SSB SCell. If RACH is needed e.g. for TA setting, network should transmit AO-SSB in the SCell |
| TCL | Prefer Alt 3 and Alt B |
| Qualcomm | Per legacy specs, SSB-to-RO mapping is fixed so that PRACH association period and association pattern period are fixed for efficient UE implementation. From our perspective, this principle should be maintained for OD-SSB operation. In other words, it should not depend on the SSB that can be turned on/off by MAC-CE or its parameters such as periodicity and/or SSB position in burst can be updated by MAC-CE. It should be noted that SSB-to-RO mapping is for valid RO only. The determination whether a RO is valid or not depends at least on SSB transmission (in addition to other aspects such as symbol direction). Hence, to have fixed SSB-to-RO mapping, parameters (like periodicity and transmitted SSBs in burst) of the SSB should be fixed. Due to on/off SSB, at the time of PRACH transmission, the UE would need to check whether it can transmit PRACH in a valid RO – This is where impact of on/off SSB is considered. In other word, the valid RO that is used for SSB-to-RO mapping may not be valid for PRACH transmission. Therefore, under OD-SSB operation, we should have following steps: - Step 1: Determination of valid RO for SSB-to-RO mapping - This requires UE to know both SSB periodicity and SSB positions in burst for the mapping purpose. - For the mapping purpose, the SSB periodicity and SSB positions in burst should be fixed. - Step 2: Map SSB indices of the SSB the UE has used to determine valid ROs in Step 1 to valid ROs - Step 3: When UE transmits PRACH, it would determine whether it can transmit in a valid RO used for SSB-to-RO mapping according to the being transmitted SSB. - This requires UE to know both SSB periodicity and SSB positions of the being transmitted SSB. Step 1 and Step 2 should be tightly coupled. Otherwise, it is not clear how SSB-to-RO mapping is done. We would like to understand whether companies have common understandings on steps first. |
| Panasonic | We are okay with the proposal. But some clarification is needed: - for Alt 1 and Alt 2, what is the UE behaviour? UE just transmits PRACH even when OD-SSB is transmitted or up to implementation? |
| Spreadtrum | Support Alt3 and Alt B. |
| DCM | We support Alt. 3 and not support either Alt. A or B. For SSB-to-RO mapping, SSB-to-Ro mapping rule is determined based on the SSB indices currently being transmitted OD-SSB. For SSB-to-RO mapping, in the current spec, it is captured as following, which means that every time the periodicity changed, SSB-RO mapping rule is applied. Therefore, Alt. C should be supported. Similarly we support RO validation should be based on the currently being transmitted OD-SSB. - SS/PBCH block indexes provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or in SSB-MTC-AdditionalPCI are mapped to valid PRACH occasions in the following order where the parameters are described in [4, TS 38.211]. |
| CATT | Alt3. Regarding RO mapping, we agree with QC to clarify the understanding first. |
| China Telecom | We support Alt 3 and Alt B. |
| Ofinno | We support Alt 3 and Alt A |
| Lenovo | Similar view with Samsung. For RO validation, support Alt.3. For SSB to RO mapping, support an Alt.: SSB-to-RO mapping rule is determined based on od-ssb-PositionsInBurst of current OD-SSBs. |

### [HIGH] Proposal #3-1-1a (Case #1 RO):

- For a cell supporting on-demand SSB SCell operation and for Case #1 (i.e., No always-on SSB on the cell),

  - For RO validation before SSB-to-RO mapping,

    - Alt 1

      - For RRC based indication of OD-SSB transmission, RO validation rule is determined based on OD-SSB configuration in the RRC.

      - For MAC-CE based indication of OD-SSB transmission, RO validation rule is determined based on OD-SSB configuration in RRC and the latest MAC-CE indicating OD-SSB transmission at or before the time UE receives Scell activation command.

    - Alt 2

      - RO validation rule is determined without considering transmission occasions for OD-SSB consideration(s).

    - Alt 3

      - RO validation rule is determined based on union of all transmission occasions of all configured transmission occasions of OD-SSB configuration(s) in RRC.

      - RO validation rule is determined based on transmission occasions for the currently being transmitted OD-SSB.

  - For SSB-to-RO mapping,

    - Alt A

      - For RRC based indication of OD-SSB transmission, SSB-to-RO mapping rule is determined based on OD-SSB configuration in the RRC.

      - For MAC-CE based indication of OD-SSB transmission, SSB-to-RO mapping rule is determined based on OD-SSB configuration in RRC and the latest MAC-CE indicating OD-SSB transmission at or before the time UE receives Scell activation command.

    - Alt B

      - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst in RRC.

Companies are encouraged to provide views on Proposal #3-1-1a.

| Company | Views |
| --- | --- |
| Moderator | As Qualcomm emphasized, “RO validation before SSB-to-RO mapping” and “SSB-to-RO mapping” should NOT be dynamically changed depending on ON/OFF status of OD-SSB transmission. In that sense, Alt 3 or SSB-to-RO mapping based on the currently being transmitted OD-SSB is now removed. |
| OPPO | Comment#1: For Alt 1, why there are two bullets talking about two different options? To our understanding the RO validation rule should result in common-understanding valid/invalid ROs, therefore, taking into account the latest MAC-CE will cause different valid/invalid understanding for different UEs. Comment#2: it is not clear the difference between Alt2 and Alt3. Comment#3: for Alt A: similar feeling ! the mapping should be commonly understood. If MAC-CE is involved, the mapping for a given RO will become chaos. |
| LGE | Alt 2 and Alt B are still supported. But we can live with Alt 3 in RO validation before SSB-to-RO mapping for progress. |

### [Closed] Proposal #3-1-2 (Case #2 RO):

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

  - For RO validation before SSB-to-RO mapping,

    - Alt 1

      - RO validation rule is determined based on AO-SSB, as in legacy.

    - Alt 2

      - ?

  - For SSB-to-RO mapping,

    - Alt A

      - SSB-to-RO mapping rule is determined based on AO-SSB, as in legacy.

    - Alt B

      - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by one or multiple od-ssb-PositionsInBurst and by ssb-PositionsInBurst.

Companies are encouraged to provide views on Proposal #3-1-2.

| Company | Views |
| --- | --- |
| Moderator | This proposal seems to be linked with Proposal #6-4-1. If Proposal #6-4-1 is agreed, Alt 1 & Alt B would be a straight-forward solution. |
| Samsung | For RO validation before SSB-to-RO mapping, we support being based on both AO-SSB and OD-SSB. For SSB-to-RO mapping, we support Alt B (actually we support OD-SSB beams as a subset of AO-SSB, then the bitmap in Alt B is same as the one for AO-SSB) |
| Google | We think the solution for Case #1 and Case #2 should be unified. We can discuss this after reaching consensus for Case #1 |
| InterDigital | Same understanding with moderator |
| ZTE, Sanechips | For RO validation before SSB to RO mapping, Alt 1 would be preferred. For SSB-to-RO mapping, Alt A is preferred, since we have agreed that the beam of OD-SSB is QCLed with the beam of AO-SSB and od-ssb-PositionsInBurst of OD-SSB is same as ssb-PositionsInBurst of AO-SSB. Moreover, there is also an FFS for the od-ssb-PositionsInBurst of OD-SSB is a subset of ssb-PositionsInBurst of AO-SSB. To this end, no matter which case is considered (e.g., same or subset), SSB-to-RO mapping rule is based on AO-SSB can map all actually transmitted SSBs to RO. |
| LGE | Alt 1 and Alt A are supported. Both do not require big spec change, based on the legacy behavior. |
| Futurewei | OK, Alt 1 and Alt A preferred |
| Xiaomi | Same as Proposal #3-1-1 (Case #1 RO):, we believe fixed SSB-to-RO mapping is the most important thing while RO validation should be determined according to actual SSB transmission. - For RO validation before SSB-to-RO mapping, - Support alt 2: RO validation rule is determined based on transmission occasions for the currently being transmitted OD-SSB and AO-SSB. - For SSB-to-RO mapping, - Support alt B |
| Fujitsu | We support FL’s suggestion. A unified solution for both case #1 and case#2 would be preferred, |
| vivo | Regarding SSB RO mapping, altA and altB seems the same. Since we agreed that OD SSB should be the same or subset of AO SSB. |
| ETRI | We support Alt 1 and Alt B. |
| NEC | Support to further discuss |
| CMCC | For case 2, there can be legacy UE consider the corresponding cell as SCell. Thus, Alt 1 is supported to avoid negative impact on RO validation of legacy UEs, meanwhile R19 UEs shall not transmit PRACH in RO overlapping with OD-SSB or in RO preceding a SS/PBCH block in the PRACH slot and not starting at least Ngap symbols after last OD-SSB reception symbol. For Alt A/B, our view is to first discuss whether the SSB index within OD-SSB burst can be a subset of AO-SSB burst. If so, Alt A shall be adopted. |
| Apple | As in Proposal #3-1-1 (Case #1 RO), we prefer to apply the rules are applied for union of AO-SSB and the latest OD-SSB transmission. We proposed the following: - For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - For RO validation before SSB-to-RO mapping, - Alt 1 - RO validation rule is determined based on AO-SSB, as in legacy. - Alt 2 - ? RO validation rule is determined based on AO-SSB, as in legacy, and the latest OD-SSB transmission before deactivating OD-SSB transmission - For SSB-to-RO mapping, - Alt A - SSB-to-RO mapping rule is determined based on AO-SSB, as in legacy. - Alt B - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by one or multiple od-ssb-PositionsInBurst and by ssb-PositionsInBurst. - Alt C - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by ssb-PositionsInBurst and the most recently indicated by od-ssb-PositionsInBurst before deactivating OD-SSB transmission. |
| Nokia, NSB | We think that Alt 1 and Alt A should be selected. |
| TCL | Prefer Alt 1 and Alt B |
| Qualcomm | Same comments as being discussed in 3-1-1. |
| Panasonic | Same clarification on Alt 1 as the reply to the previous proposal. |
| DCM | For RO validation, we support Alt. 1. Here, we understand that RO validation in this context means that RO is invalid when the SSB and RO is within Ngap (as in the current spec.). For SSB-to-RO mapping, we are fine with Alt. A |
| CATT | Prefer Alt 1 and Alt A |
| China Telecom | Support Alt1 and Alt A. |
| Ofinno | We support Alt 1 and Alt A. |
| Lenovo | For RO validation, support an Alt: RO validation based on AO-SSB and current activated OD-SSBs. For SSB to RO mapping, support based on AO-SSB (OD-SSBs are subset of AO-SSBs). |

### [HIGH] Proposal #3-1-2a (Case #2 RO):

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

  - Alt 1)

    - For the case when the center frequency locations of AO-SSB and OD-SSB are different,

      - It is ONLY supported that SSB indices within OD-SSB burst is subset of SSB indices within AO-SSB burst or equal to SSB indices within AO-SSB burst.

    - For RO validation before SSB-to-RO mapping,

      - RO validation rule is determined based on AO-SSB, as in legacy.

    - For SSB-to-RO mapping,

      - SSB-to-RO mapping rule is determined based on AO-SSB, as in legacy.

  - Alt 2)

    - For the case when the center frequency locations of AO-SSB and OD-SSB are different,

      - It is supported that at least one of SSB indices within OD-SSB burst iscan be different from any of SSB indices within AO-SSB burst.

    - For RO validation before SSB-to-RO mapping,

      - RO validation rule is determined based on AO-SSB and union of all transmission occasions of OD-SSB configuration.

    - For SSB-to-RO mapping,

      - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by one or multiple od-ssb-PositionsInBurst and by ssb-PositionsInBurst.

  - Alt 3)

    - For the case when the center frequency locations of AO-SSB and OD-SSB are different,

      - It is supported that at least one of SSB indices within OD-SSB burst iscan be different from any of SSB indices within AO-SSB burst.

    - For RO validation before SSB-to-RO mapping,

      - RO validation rule is determined based on AO-SSB, as in legacy

    - For SSB-to-RO mapping,

      - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by one or multiple od-ssb-PositionsInBurst and by ssb-PositionsInBurst.

Companies are encouraged to provide views on Proposal #3-1-2a.

| Company | Views |
| --- | --- |
| Moderator | Merged with Proposal #6-4-1. As Qualcomm emphasized, “RO validation before SSB-to-RO mapping” and “SSB-to-RO mapping” should NOT be dynamically changed depending on ON/OFF status of OD-SSB transmission. |
| Qualcomm | We don’t see the beam linkage between AO-SSB and OD-SSB impact discussion on RO validation and SSB-RO mapping. This linkage may impact discussion in Proposal #3-5-1. For example, if we agree that SSB indices within OD-SSB burst is subset of SSB indices within AO-SSB burst or equal to SSB indices within AO-SSB burst, the PRACH/SSB collision is not needed as long as OD-SSB remains unchanged. Hence, Proposal #6-4-1 should be a separate discussion or part of PRACH tx/OD-SSB Rx prioritization in #3-5-1. As we discussed earlier, RO validation and SSB-RO mapping should be tightly coupled. Hence, for Alt.2, “RO validation rule is determined based on AO-SSB, as in legacy, OR” text should be removed. So, below is our suggestion: - Alt 1) - For the case when the center frequency locations of AO-SSB and OD-SSB are different, - It is ONLY supported that SSB indices within OD-SSB burst is subset of SSB indices within AO-SSB burst or equal to SSB indices within AO-SSB burst. - For RO validation before SSB-to-RO mapping, - RO validation rule is determined based on AO-SSB, as in legacy. - For SSB-to-RO mapping, - SSB-to-RO mapping rule is determined based on AO-SSB, as in legacy. - Alt 2) - For the case when the center frequency locations of AO-SSB and OD-SSB are different, - It is supported that at least one of SSB indices within OD-SSB burst is different from any of SSB indices within AO-SSB burst. - For RO validation before SSB-to-RO mapping, - RO validation rule is determined based on AO-SSB, as in legacy, OR - RO validation rule is determined based on AO-SSB and union of all transmission occasions of OD-SSB configuration. - For SSB-to-RO mapping, - SSB-to-RO mapping rule is determined based on the union of all the SSB indices configured by one or multiple od-ssb-PositionsInBurst and by ssb-PositionsInBurst. |
| OPPO | We think for the case 2, the restriction of SSB index of OD-SSB is subset of AO-SSB is necessary. Then, the RO validation rule just follows AO-SSB as legacy and SSB-RO mapping is also following AO-SSB as legacy. Otherwise, for Alt 2) the mapping for R19 UE and legacy UE will be different. Thus, for a given RO, it may be resulted from different mapping. This is problematic. |
| LGE | Alt 1 is preferred. |

## PDCCH monitoring

| Company | Views |
| --- | --- |
| [5] Nokia | Proposal 15: Consider the text proposal to update requirements in Chapter 10 of TS38.213 for interaction with OD-SSB and PDCCH monitoring. Text proposal for Chapter 10 of 38.213 For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell or has received [od-ssb-PositionsInBurst] for a SS/PBCH block in [od-ssb-config] and received an activation of the on-demand SS/PBCH block transmission, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon or by [od-ssb-PositionsInBurst] if provided for the on-demand SS/PBCH block, the UE is not required to monitor the PDCCH candidate.. |
| [9] Samsung | Proposal 8: - For UEs knowing the time and frequency resources of on-demand SSB, the same procedures for resource allocation (e.g., PDCCH, and PDSCH) and validation (e.g., PRACH, and PUSCH) based on always-on SSB can be reused for on-demand SSB. - For UEs not knowing the time and frequency resources of on-demand SSB, it is up to gNB implementation to handle resource allocation and validation based on always-on SSB. - RateMatchPattern needs enhancement to cover the time and frequency resources of on-demand SSB. |
| [13] CMCC | Proposal 8: RAN1 to further discuss the UE behavior with OD-SSB operation under the following procedures: - UE behavior on rate matching of PDSCH - UE behavior on PDCCH monitoring - UE behavior on UL (e.g. PUSCH, PUCCH, PRACH) transmission Proposal 10: Adopt the following update to Clause 10 of TS 38.213 for PDCCH monitoring. 10 UE procedure for receiving control information < Unchanged parts are omitted > For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE on a secondary cell, if the UE - is indicated the activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300] by DCI format 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by od-ssb-PositionsInBurst corresponding to the transmission for SS/PBCH blocks, if any, otherwise by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. < Unchanged parts are omitted >. |
| [20] Sharp | Proposal 1 If a PDCCH candidate and a transmitted OD-SSB are overlapped in at least one RE, the UE is not required to monitor the PDCCH candidate. |
| [27] LG Electronics | Proposal #21: If at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate activated OD-SSB corresponding to parameters indicated in od-ssb-config, the UE is not required to monitor the PDCCH candidate. |
| [30] Qualcomm | Proposal 10: Adopt the following update to Clause 10 of TS 38.213 for PDCCH monitoring. 10 UE procedure for receiving control information <omitted text> For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE on a secondary cell, if the UE - has received [od-ssb-PositionsInBurst] in [od-ssb-Config] for the secondary cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by [od-ssb-PositionsInBurst] in [od-ssb-Config] and associated with a SS/PBCH block that is transmitted according to an on-demand SSB transmission operation in the cell, the UE is not required to monitor the PDCCH candidate. A UE is not required to monitor PDCCH candidates for a Type0/0A/0B/1/1A /2/2A -PDCCH CSS set when the active TCI state for a corresponding CORESET is not associated with physCellId in ServingCellConfigCommon. If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. <omitted text>. |

### [HIGH] Proposal #3-2 (PDCCH):

- For a cell supporting on-demand SSB SCell operation,

  - If a PDCCH candidate and a currently being transmitted OD-SSB are overlapped in at least one RE, the UE is not required to monitor the PDCCH candidate.

- Adopt the following TP to Clause 10 of TS 38.213 for PDCCH monitoring.

10 UE procedure for receiving control information <omitted text> For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE on a secondary cell, if the UE - has received [od-ssb-PositionsInBurst] in [od-ssb-Config] for the secondary cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by the latest [od-ssb-PositionsInBurst] in [od-ssb-Config] before deactivating on-demand SSB transmission and associated with a SS/PBCH block that is transmitted according to an on-demand SSB transmission operation in the cell, the UE is not required to monitor the PDCCH candidate. A UE is not required to monitor PDCCH candidates for a Type0/0A/0B/1/1A /2/2A -PDCCH CSS set when the active TCI state for a corresponding CORESET is not associated with physCellId in ServingCellConfigCommon. If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. <omitted text>

Companies are encouraged to provide views on Proposal #3-2.

| Company | Views |
| --- | --- |
| Moderator | Qualcomm’s TP can be a starting point. If the TP is not agreeable, we can try to agree only on the first sub-bullet. |
| Samsung | We support the proposal, with the understanding that the UE receives the configuration/indication of ssb-PositionsInBurst for the OD-SSB. |
| Google | For USS, it seems to be fine, but for CSS we are not sure whether this is feasible, since the UE information on the OD-SSB status could be different. |
| ZTE, Sanechips | OK with FL’s suggestion |
| LGE | OK |
| Futurewei | OK with sub-bullet, we could discuss the TP |
| xiaomi | We are fine with Qualcomm’s TP. |
| Fujitsu | OK with the proposal and we share the same view as Samsung that it applies for the UE(s) receives the activation indication of OD-SSB. |
| Vivo | Fine with the proposal |
| ETRI | OK |
| NEC | We agree with the principle. If a PDCCH candidate and a currently transmitted OD-SSB overlap in at least one RE, the UE should not be required to monitor that PDCCH candidate. This is consistent with general procedures for handling SSB and PDCCH overlap. The proposed TP from Qualcomm appears to be a good starting point for discussion. |
| CMCC | Support. |
| Apple | The assumption based on configuration level seems wasteful. We suggest the following: For monitoring of a PDCCH candidate by a UE on a secondary cell, if the UE - has received [od-ssb-PositionsInBurst] in [od-ssb-Config] for the secondary cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by the latest activated or adapted [od-ssb-PositionsInBurst] in [od-ssb-Config] before deactivating on-demand SSB transmission and associated with a SS/PBCH block that is transmitted according to an on-demand SSB transmission operation in the cell, the UE is not required to monitor the PDCCH candidate. |
| Nokia, NSB | Support |
| TCL | Support |
| Qualcomm | The change from Apple is fine for us in general. We suggest changing “the latest activated or adapted” to “the latest” since the spec/endorsed CR does not write a parameter is activated or adapted. Instead, it is writing SSB transmission is activated or updated instead. |
| Panasonic | We are okay with the direction and intention. TP can be discussed further. |
| Spreadtrum | Support |
| DCM | We support the FL proposal. Our understanding is that every time the OD-SSB configuration changes, UE’s monitoring behavior for PDCCH monitoring will be updated based on the currently configured OD-SSB configuration. |
| China Telecom | Support. |
| Ofinno | OK |
| Moderator | Apple/Qualcomm’s comments are reflected. |

## PDSCH rate-matching

| Company | Views |
| --- | --- |
| [2] Huawei | Proposal 4: To address the inter-cell interference issue, the rate-matching pattern on a Rel-19 NES cell is based on all of the possible candidate OD-SSBs, i.e., the union of all the SSB indices configured by multiple od-ssb-PositionsInBurst with a periodicity set to the minimum value among the configured periodicities. |
| [5] Nokia | Proposal 14: PDCSH rate matching around OD-SSB is supported. |
| [6] NEC | Proposal 14: Address PDSCH rate matching around on-demand SSB indicated to other UE of the NES cell as well as on-demand SSB indicated to the UE, and discuss whether common or dedicated signalling is used |
| [8] TCL | Proposal 6: Discuss how to handle collision between OD-SSB resource and NR resource and how to handle rate-matching. |
| [9] Samsung | Proposal 8: - For UEs knowing the time and frequency resources of on-demand SSB, the same procedures for resource allocation (e.g., PDCCH, and PDSCH) and validation (e.g., PRACH, and PUSCH) based on always-on SSB can be reused for on-demand SSB. - For UEs not knowing the time and frequency resources of on-demand SSB, it is up to gNB implementation to handle resource allocation and validation based on always-on SSB. - RateMatchPattern needs enhancement to cover the time and frequency resources of on-demand SSB. |
| [11] Ofinno | Proposal 20: Rate matching on OD-SSB should consider active OD-SSB transmission for the UE, potential OD-SSB transmission to other UE(s), and optionally non-serving cell. |
| [12] CATT | Proposal 26: Rate-matching issue needs to consider on-demand SSB transmission of both serving cell and neighboring cell. |
| [13] CMCC | Proposal 8: RAN1 to further discuss the UE behavior with OD-SSB operation under the following procedures: - UE behavior on rate matching of PDSCH - UE behavior on PDCCH monitoring - UE behavior on UL (e.g. PUSCH, PUCCH, PRACH) transmission Proposal 11: Adopt the following update to Clause 5.1.4 of TS 38.214 for PDSCH resource mapping. ### 5.1.4 PDSCH resource mapping < Unchanged parts are omitted > When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and according to the activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300] indicated by DCI format 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI, if provided, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and according to the activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300] indicated by DCI format 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI, if provided, if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. < Unchanged parts are omitted >. |
| [14] Xiaomi | Proposal 13: PDSCH is rate-matched around OD-SSB when OD-SSB is transmitted. |
| [17] Google | Proposal 2: For non-UE dedicated signals, the rate matching pattern should be based on both always-on SSB and OD-SSB. Proposal 3: For UE-dedicated signals, the rate matching pattern should be based on always-on SSB only |
| [18] KT | Proposal 5: Proposal #9-1 in [2] is supported, i.e., for a cell supporting on-demand SSB SCell operation, support the following rate-matching behavior. - Rate-matching for PDSCH around on-demand SSB is applied from time instance A when UE receives on-demand SSB activation signaling. - Rate-matching for PDSCH around on-demand SSB is not applied from time instance B when UE receives on-demand SSB deactivation signaling, or not applied after N on-demand SSB burst transmissions if configured/indicated. |
| [20] Sharp | Proposal 2 If a PDSCH resource allocation overlaps with PRBs containing OD-SSB transmission resources, the UE shall assume that the PRBs containing the OD-SSB are not available for the PDSCH reception. |
| [27] LG Electronics | Proposal #22: The UE shall assume that the PRBs containing activated OD-SSB transmission resources are not available for PDSCH in the OFDM symbols where activated OD-SSB is transmitted. |
| [29] Apple | Observation 13: Rate-matching behavior around OD-SSB needs to be specified. Proposal 13: Support the following rate-matching behavior: - Rate-matching for PDSCH around OD-SSB is applied from time instance A when UE receives OD-SSB activation. - Rate-matching for PDSCH around OD-SSB is not applied from time instance B when UE receives OD-SSB deactivation, or not applied after N OD-SSB bursts if configured/indicated. |
| [30] Qualcomm | Proposal 11: Adopt the following update to Clause 5.1.4 of TS 38.214 for PDSCH resource mapping. ### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block, after puncturing if applicable, is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. For an UE supporting on-demand SSB Scell operation, the UE assumes SS/PBCH block transmission according to od-ssb-PositionsInBurst according to an on-demand SSB transmission operation if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. <omitted text>. |
| [33] KDDI | Proposal 5: A UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH from time instance A when the UE receives on-demand SSB activation signaling until time instance B when the UE receives on-demand SSB deactivation signaling or after the number N of on-demand SSB bursts if configured/indicated. Proposal 6: PDSCH for which rate-matching is performed by UE for on-demand SSB is the PDSCH scheduled UE-dedicatedly. |

## [Moderator’s note] Most companies (at least including Huawei, CATT, Ofinno, Google, Nokia, NEC, Samsung, Ofinno, Xiaomi, KT, Sharp, LG Electronics, Apple, KDDI and ITRI) agree on handling of rate-matching for PDCCH overlapped with OD-SSB. In addition,

- Huawei suggested to perform PDSCH rate-matching based on all of the possible candidate OD-SSBs, i.e., the union of all the SSB indices configured by multiple od-ssb-PositionsInBurst with a periodicity set to the minimum value among the configured periodicities.

- Samsung suggested an Enhancement of RateMatchPattern for UEs not knowing the time and frequency resources of on-demand SSB

- CATT and Ofinno suggested to consider neighbor cell’s OD-SSB pattern

- Google suggested differentiation between non-UE dedicated signal and UE-dedicated signal

### [HIGH] Proposal #3-3 (PDSCH):

- For a cell supporting on-demand SSB SCell operation,

  - The UE shall assume that the PRBs containing currently being transmitted OD-SSB resources are not available for PDSCH in the OFDM symbols where OD-SSB is currently being transmitted on the cell.

- Adopt the following TP to Clause 5.1.4 of TS 38.214 for PDSCH resource mapping.

### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block, after puncturing if applicable, is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. For a UE supporting on-demand SSB Scell operation, the UE assumes SS/PBCH block transmission according to od-ssb-PositionsInBurst according to the latest an on-demand SSB transmission operation before deactivating on-demand SSB transmission if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. <omitted text>

Companies are encouraged to provide views on Proposal #3-3.

| Company | Views |
| --- | --- |
| Moderator | Qualcomm’s TP can be a starting point. If the TP is not agreeable, we can try to agree only on the first sub-bullet. |
| Samsung | OK in general. |
| Google | For PDSCH scheduled by DCI format other than DCI format 4_x, this should be fine. For MBS PDSCH, it is hard to perform the rate matching in UE-specific manner. |
| ZTE, Sanechips | OK with Moderator’s suggestion. |
| LGE | OK |
| Futurewei | OK |
| Xiaomi | We are fine with Qualcomm’s TP. |
| Fujitsu | Same comment as in proposal #3-2. It applies for the UE(s) receives the activation indication of OD-SSB. |
| ETRI | OK |
| NEC | We support that the UE shall assume PRBs containing currently transmitted OD-SSB resources are not available for PDSCH in the OFDM symbols where OD-SSB is transmitted. Furthermore, we propose to address PDSCH rate matching around on-demand SSB indicated to other UEs on the NES cell, in addition to the OD-SSB indicated to the UE itself. We also suggest discussing whether common or dedicated signaling is used for this. The proposed TP from Qualcomm for Clause 5.1.4 of TS 38.214 can be a basis for discussion, and we should ensure it covers rate-matching for OD-SSB intended for the specific UE as well as potentially for other UEs if that information is available or necessary for robust operation. |
| CMCC | Support |
| Apple | Similar to the comments in Proposal #3-2, the assumption based on configuration level seems wasteful. We suggest the following: For a UE supporting on-demand SSB Scell operation, the UE assumes SS/PBCH block transmission according to od-ssb-PositionsInBurst according to the latest activated or adapted an on-demand SSB transmission operation before deactivating on-demand SSB transmission if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. |
| Nokia, NSB | Support |
| Qualcomm | Apple’s update is fine in general. We suggest changing “the latest activated or adapted” to “the latest” since the spec/endorsed CR does not write a parameter is activated or adapted. Instead, it is writing SSB transmission is activated or updated instead. |
| Panasonic | We support the intention of the proposal. TP can be discussed further. |
| KDDI | OK. |
| Spreadtrum | Support |
| DCM | The wording on-demand SSB SCell operation needs to be clarified. We think the on-demand SSB SCell operation can be understood as on-demand SSB which is now being activated. |
| China Telecom | Support. |
| Ofinno | OK |
| Moderator | Apple/Qualcomm’s comments are reflected. |
| Qualcomm | We should update “to the latest an on-demand SSB transmission operation” to “to the latest an on-demand SSB transmission operation” |
| Moderator | The latest Qualcomm’s comment is also reflected. |

## SSB multiplexing

| Company | Views |
| --- | --- |
| [6] NEC | Proposal 6: For Case#2, when an on-demand SSB transmission occasion overlaps in the time domain with an always-on SSB transmission occasion, the UE may expect the transmission to follow the parameters (including transmitted SSB indices defined by ssb-PositionsInBurst) of the always-on SSB. |
| [8] TCL | Observation 3: One restriction is whether overlapping happened between two different OD-SSB transmission if same center frequency is assumed for these two OD-SSB. Proposal 2: Consider how to avoid overlap of time domain or frequency domain between multiple OD-SSB transmissions. Proposal 4: Consider how to handle collision between OD-SSB and AO-SSB, including priority definition or following previous type of SSB to receive SSB when collision happened. |
| [10] Tejas | Proposal 4: Supporting Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB. |
| [11] Ofinno | Proposal 19: The UE does not expect that the OD-SSB is transmitted when OD-SSB overlaps with AO-SSB in time and frequency domains. |
| [12] CATT | Proposal 25: Both Mux-Case #1 and Mux-Case #2 in the agreement in RAN1#118-bis should be supported in Rel-19. - If always-on SSB and on-demand SSB overlap in both time domain and frequency domain, always-on SSB should be prioritized over on-demand SSB to avoid the collision between always-on SSB and on-demand SSB. |
| [19] China Telecom | Observation 2: OD-SSB and always-on SSB transmitted close to each other in time domain should also be avoided to save energy. Proposal 2: There should be no on-demand SSB and always-on SSB transmitted within a specific time period so that the largest NES gain can be acquired. - The AO-SSB should be terminated to transmit when such situation happens. |
| [23] ETRI | Proposal 7: It is proposed that when a conflict occurs due to overlapping of always-on SSB and on-demand SSB in both time and frequency domains, the network should prioritize the transmission of the always-on SSB and drop the on-demand SSB. |
| [29] Apple | Observation 3: For the same center frequency case, in case of overlapping, AO-SSB and OD-SSB are the identical. Proposal 3: There is no special handling needed for overlapping case between AO-SSB and OD-SSB for the same center frequency case. Observation 4: For the different center frequency case, in case of overlapping in time, AO-SSB and OD-SSB are still distinguishable in frequency domain. Proposal 4: There is no special handling needed for overlapping case between AO-SSB and OD-SSB for the different center frequency case. |

Agreement (RAN1#118bis) For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

## [Moderator’s note] Company views on SSB multiplexing are as follows.

- China Telecom: No on-demand SSB and always-on SSB transmitted within a specific time period

- Apple: No additional handling

- If overlapped,

  - NEC, Ofinno, CATT, ETRI, Ericsson: Prioritize always-on SSB

According to previous RAN1 agreements, it seems that both of Mux-Case #1 and Mux-Case #2 can be supported. The remaining issue would be how to handle the case where AO-SSB and OD-SSB are fully overlapped in time and frequency domains. However, unless it is agreed that SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst, no additional handling is needed for Mux-Case #2.

| Company | Views |
| --- | --- |
| NEC | This section of the email discussion aligns with our Proposal 6: "For Case#2, when an on-demand SSB transmission occasion overlaps in the time domain with an always-on SSB transmission occasion, the UE may expect the transmission to follow the parameters (including transmitted SSB indices defined by ssb-PositionsInBurst) of the always-on SSB". Hence, we suggest prioritizing AO-SSB parameters in case of full overlap in time and frequency. This is needed for clarity. |
| China Telecom | We agree with moderator’s comment that the remaining issue would be how to handle the case where AO-SSB and OD-SSB are fully overlapped in time and frequency domain, but if so, we think the Mux-Case #2 should be: Always-on SSB and on-demand SSB overlap at least only in time or frequency domain. Otherwise, we still need to discuss if there are overlap in both time and frequency domain. |

## Others

| Company | Views |
| --- | --- |
| [4] vivo | Proposal 20: To support on-demand SSB operation, prioritize the on-demand SSB transmission if there is collision between on-demand SSB and other transmission. |
| [6] NEC | Proposal 13: Upon failure to receive or detect the on-demand SSB, the UE may indicate on-demand SSB failure indication to the network. Following options can be considered for the indication - Option-1: RRC signalling - Option-2: MAC CE - Option-3: UCI transmitted over PCell |
| [9] Samsung | Proposal 8: - For UEs knowing the time and frequency resources of on-demand SSB, the same procedures for resource allocation (e.g., PDCCH, and PDSCH) and validation (e.g., PRACH, and PUSCH) based on always-on SSB can be reused for on-demand SSB. - For UEs not knowing the time and frequency resources of on-demand SSB, it is up to gNB implementation to handle resource allocation and validation based on always-on SSB. - RateMatchPattern needs enhancement to cover the time and frequency resources of on-demand SSB. Proposal 9: Support uplink power control based on on-demand SSB, and similar approach to L1 measurement based on on-demand SSB can be used. |
| [10] Tejas | Proposal 5: Collision handling of OD-SSB with other channels should be discussed. |
| [11] Ofinno | Proposal 21: PRACH occasion and/or PUSCH occasion may be determined based on OD-SSB in Case #1. PRACH occasion and/or PUSCH occasion are determined only based on AO-SSB in Case #2. Proposal 22: TCI state/PL-RS: in case TCI state has OD-SSB as RS source in Case # 2 and OD-SSB is deactivated, then UE fallbacks to QCL-ed AO-SSB. Proposal 23: Support OD-SSB as a PL fallback in Case #1. |
| [13] CMCC | Proposal 8: RAN1 to further discuss the UE behavior with OD-SSB operation under the following procedures: - UE behavior on rate matching of PDSCH - UE behavior on PDCCH monitoring - UE behavior on UL (e.g. PUSCH, PUCCH, PRACH) transmission Proposal 12: Adopt the following update to Clause 11.1 of TS 38.213 for UL transmission. 11.1 Slot configuration < Unchanged parts are omitted > For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, and according to the activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300] indicated by DCI format 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI, if provided, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and does not transmit PRACH if the occasion precedes a SS/PBCH block in the PRACH slot and does not start at least Ngap symbols after a last SS/PBCH block reception symbol where Ngap is provided in Table 8.1-2, according to the activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300] indicated by DCI format 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI, if provided, and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. If a UE - is configured with multiple serving cells and is provided half-duplex-behavior = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, and according to the activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300] indicated by DCI format 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI, if provided, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and does not transmit PRACH if the occasion precedes a SS/PBCH block in the PRACH slot and does not start at least Ngap symbols after a last SS/PBCH block reception symbol where Ngap is provided in Table 8.1-2, according to the activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300] indicated by DCI format 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI, if provided, and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells. < Unchanged parts are omitted > For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indices corresponding to the SS/PBCH block indexes indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, as described in Clause 4.1, and according to the activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300] indicated by DCI format 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI, if provided, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink. < Unchanged parts are omitted >. |
| [18] KT | Proposal 8: Using on-demand SSB as a PL-RS/QCL RS and associated with a TCI state is not supported in Rel. 19 NES. |
| [20] Sharp | Proposal 3 Support that OD-SSB can be used as a QCL source of a TCI state at least for Case #1. |
| [21] Fujitsu | Proposal 14: In case#1, when on-demand SSB is activated, - The downlink SS/PBCH SSS EPRE can be derived from the on-demand SSB downlink transmit power given by the parameter od-ss-PBCH-BlockPower. - The downlink CSI-RS EPRE can be derived from the on-demand SSB downlink transmit power given by the parameter od-ss-PBCH-BlockPower and CSI-RS power offset given by the parameter powerControlOffsetSS. |
| [26] Panasonic | Proposal 6: On-demand SSB for time/frequency synchronization and TCI state reference resource should be supported in Scenario #3B and Case#1. |
| [27] LG Electronics | Proposal #15: For Case #1, and for Case #2 where AO-SSB and OD-SSB have the same center frequency, OD-SSB can be used for the following purposes. - Beam failure recovery - Path-loss estimation reference signal - QCL reference signal Proposal #20: After SSB-to-RO mapping, if an RO is collided with activated OD-SSB or located within N_gap symbols after a last symbol of activated OD-SSB, UE does not transmit PRACH within the RO. |
| [30] Qualcomm | Proposal 9: For OD-SSB Scell operation, the UE is not expected to transmit PRACH in a valid PRACH occasion if the occasion precedes a SS/PBCH block in the PRACH slot and does not start at least N_gap symbols after a last SS/PBCH block reception symbol where - N_gap is provided in Table 8.1-2 of TS 38.213, and - SS/PBCH block is the SS/PBCH block that is transmitted in response to RRC/MAC-CE based indication of OD-SSB transmission. |
| [33] KDDI | Proposal 7: Study whether UE transmits PUSCH, PUCCH or PRACH in cells other than the cell transmitting OD-SSB in the slot if the transmission would overlap with any symbol from the set of symbols corresponding to the OD-SSB in case of directional collision handling on half-duplex operation in TDD CA with same SCS. |
| [35] CEWiT | Observation 5: UE not receiving the SSB after on-demand SSB operation leads to unnecessary monitoring by the UE and negative impacts on performance. Proposal 5: Support handling of the case where the UE cannot receive SSB after the on-demand SSB operation. |

## [Moderator’s note] Following issues are identified.

- Issue #1: Prioritize OD-SSB than PUSCH/PUCCH/PRACH if there is a collision, as described in Clause 11.1 of TS 38.213 for legacy SSB

  - Supported by vivo, Ofinno, CMCC, LG Electronics, Qualcomm, KDDI

- Issue #2: Whether/how to support the following functionalities for OD-SSB

  - PL-RS

    - Supported by Samsung, Ofinno, LG Electronics

    - Objected by KT

  - Reference signal for QCL/TCI configuration

    - Supported by Sharp (for Case #1), Panasonic (for Case #1 and Scenario #3B), LG Electronics

    - Objected by KT

  - Beam failure detection

    - Supported by Ofinno

    - Objected by ZTE, vivo, Xiaomi, Fujitsu

  - Beam failure recovery

    - Supported by ZTE, NEC, Ofinno, Xiaomi, Fujitsu, LG Electronics

- Other issues

  - Clarify that CSI-RS power is derived based on OD-SSB power

    - Supported by Fujitsu

  - OD-SSB reception failure handling

    - Supported by NEC, CEWiT

  - OD-SSB has less priority of transmission than the previously configured periodic signals, like RACH and CSI-RS

    - Supported by KT

### [MID] Proposal #3-5-1 (UL TX):

- For a cell supporting on-demand SSB SCell operation,

  - OD-SSB transmission by gNB is prioritized over PUSCH/PUCCH/PRACH transmission by UE.

  - The SS/PBCH block in TS 38.213 Clause 11.1 can be replaced by the SS/PBCH block that is transmitted according to an on-demand SSB transmission operation in the cell.

Companies are encouraged to provide views on Proposal #3-5-1.

| Company | Views |
| --- | --- |
| Samsung | OK with the proposal |
| Google | We suggest discussing this based on the TP |
| ZTE, Sanechips | OK to consider the modification is on the Clause 11.1 of TS 38.213 to avoid multiple modifications in the specifications. |
| LGE | Without big spec change, based on AO-SSB which is legacy SSB, we think that OD-SSB transmission by gNB should is prioritized over any channels. |
| Futurewei | OK |
| Xiaomi | Support |
| Fujitsu | We are fine with the proposal. |
| vivo | OK with the proposal |
| ETRI | OK |
| NEC | Support |
| CMCC | Ok. |
| TCL | Support |
| KDDI | We support the FL’s proposal. |
| Ofinno | Support |

### [MID] Proposal #3-5-2 (PL/QCL/TCI/BFR):

- For a cell supporting on-demand SSB SCell operation,

  - It is supported to use OD-SSB as

    - PL-RS,

    - Reference signal for QCL/TCI configuration, or

    - Candidate beam detection for beam failure recovery.

Companies are encouraged to provide views on Proposal #3-5-2.

| Company | Views |
| --- | --- |
| Samsung | We support the proposal. |
| Google | Support |
| ZTE, Sanechips | We think at least the 3rd sub-bullet can be supported, since on-demand SSB can also be used to select the appropriate beams for recovery. For the first two bullet, we are open to consider if the motivation is clear. |
| LGE | OK. But, OD-SSB should not be considered for BFD |
| Futurewei | OK |
| Xiaomi | Support |
| Fujitsu | Support for case #1. For case#2, it can rely on the AO-SSB. |
| vivo | OK with the proposal |
| ETRI | OK |
| NEC | Support |
| CMCC | OD-SSB is used at least for QCL/TCI configuration. |
| Nokia, NSB | We think that further discussion is needed. Because of the late phase of the release, we prefer not to introduce any new features that require specification changes. |
| Ofinno | Support |

# Remaining issues for signalling methods and contents

| Company | Views |
| --- | --- |
| [1] Futurewei | Proposal 3: From RAN1 perspective all OD-SSB RRC parameters—frequency, SSB position, subcarrier spacing, physical cell ID, transmit power, and time domain position—may be configured as lists to enable fast and flexible selection. Proposal 4: For Case #1, od-ssb-nrofBurst should not be configured, as deactivation would impair UE operation. Proposal 5: For Case #2, when od-ssb-nrofBurst is configured, gNB should have the option to deactivate OD-SSB prior to completing the full N-burst transmission by issuing a MAC-CE. Proposal 6: No additional deactivation mechanisms beyond those already agreed (MAC-CE, N bursts) are necessary. Proposal 7: Do not support indication of time offset between always-on SSB and on-demand SSB. |
| [3] ZTE | Observation 1: After the SCell is deactivated, the on-demand SSB triggered before the SCell is deactivated is not necessary and will cause additional power consumption. Proposal 2: Option 4 and option 4a can be supported. Proposal 3: When the od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB can be deactivated via MAC CE. Observation 2: Using timer to implicitly indicate the value of N performs the same function as configuring N directly but introduces a large workload. Proposal 4: Introducing timer to implicitly determine the value of N is not supported. |
| [4] vivo | Proposal 1: When MAC CE is used for updating the transmission parameter of a transmitted OD-SSB for the cell, only the parameter of periodicity could be updated. Proposal 2: For updating the transmission parameter of a transmitted OD-SSB for the cell, SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity. Proposal 3: MAC CE could be used for updating the transmission parameter of a transmitted OD-SSB in Scenario #2 and Scenario #2A. Proposal 8: The number N of on-demand SSB bursts to be transmitted could only be indicated via MAC-CE. Proposal 9: Support Option 4 and 4a for Case #1 in addition to Option 2. Proposal 10: A nonnumerical indication of N indicates that on-demand SSB transmission is deactivated along with the deactivation of SCell. Proposal 11: Do not support that the value of N can be implicitly determined using a timer. Proposal 12: MAC CE could be used to deactivate the on-demand SSB if od-ssb-nrofBurst for an on-demand SSB is configured. Proposal 13: The time offset between always-on SSB and on-demand SSB is not introduced. Proposal 14: The parameter list includes the following parameters: - SFN offset - Half frame index |
| [5] Nokia | Proposal 5: The od-ssb-config is cell-specific configuration. Proposal 6: RAN1 to remove the od-ssb-physCellId from the OD-SSB configuration parameter list to avoid ambiguity. This parameter is already provided for every SCell during SCell addition. Proposal 7: Besides MAC CE based and configured number of bursts-based deactivation, no other options are specified for OD-SSB deactivation. Proposal 8: If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB can be deactivated via MAC CE. Proposal 9: MAC CE indication of OD-SSB transmission overrides potentially ongoing OD-SSB SSB transmission on the SCell. |
| [6] NEC | Proposal 3: Support on-demand SSB indication via group-common DCI for Scenario#2 and Scenario#3. Proposal 4: On-demand SSB for SCell may be enabled via DCI format 1_x on PCell with a carrier indication field to indicate the applicable carrier. |
| [7] Spreadtrum | Proposal 2 If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst or via MAC CE. |
| [8] TCL | Observation 1: Even if most cases consider semi static-based MAC CE to indicate OD-SSB transmission, but specific DCI could be used for reducing signaling overhead if multiple UE configured with same SCell, e.g., group common DCI. Observation 2: In RAN 1#120 meeting, DCI format 2_9 as group common DCI has been agreed for the indication of periodicity adaptation of SSB. Proposal 1: Support at least group common DCI used for indicating OD-SSB transmission. Proposal 3: Support define reference point of OD-SSB for case #2 based on periodicity of AO-SSB. |
| [9] Samsung | Proposal 1: For on-demand SSB: - Support a list of configurations by RRC, wherein each configuration includes the following parameters: - od-ssb-Periodicity; - od-ssb-sfn-Offset; - od-ssb-halfFrameIndex; - od-ssb-PositionsInBurst; - od-ssb-nrofBurst. - Note: some parameter(s) can be optional subject to RAN1 agreements. - The maximum number of configurations in the list is 16. Proposal 2: When on-demand SSB transmission is indicated to be activated by the RRC, the UE applies the parameters associated with the first configuration within the list of configurations included the RRC for the on-demand SSB transmission. - Need to specify the timeline at least when the number of transmitted on-demand SSB bursts is provided. Proposal 3: For adaptation of parameter(s) for on-demand SSB, support all the following cases: - Case A1: activation by a RRC and N is not provided + adaptation by a MAC CE; - Case A2: activation by a RRC and N is provided + adaptation by a MAC CE; - Case B1: activation by a MAC CE and N is not provided + adaptation by a MAC CE; - Case B2: activation by a MAC CE and N is provided + adaptation by a MAC CE. Proposal 4: For deactivation for on-demand SSB, support all the following cases: - Case X1: activation by a RRC and N is not provided + deactivation by a MAC CE; - Case X2: activation by a RRC and N is provided + deactivation after N transmission bursts; - Case X3: activation by a RRC and N is provided + deactivation by a MAC CE; - Case Y1: activation by a MAC CE and N is not provided + deactivation by another MAC CE; - Case Y2: activation by a MAC CE and N is provided + deactivation after N transmission bursts; - Case Y3: activation by a MAC CE and N is provided + deactivation by another MAC CE; - Case Z1: adaptation by a MAC CE and N is not provided + deactivation by another MAC CE; - Case Z2: adaptation by a MAC CE and N is provided + deactivation after N transmission bursts; - Case Z3: adaptation by a MAC CE and N is provided + deactivation by another MAC CE. |
| [11] Ofinno | Proposal 7: DCI based signaling to indicate on-demand SSB transmission on the cell is not supported. Proposal 8: The time offset between OD-SSB and AO-SSB is indicated by using similar parameter (e.g., ssb-TimeOffset) used for indicating the time offset between CD-SSB and the NCD-SSB. Proposal 9: No additional restriction is needed when multiple candidate values of the OD-SSB burst (i.e., od-ssb-PositionsInBurst) and the number of OD-SSB bursts (i.e., od-ssb- nrofBurst) are configured via RRC signaling. Proposal 10: We support that the configuration of the OD-SSB is cell specific. Proposal 11: If od-ssb-nrofBurst for an on-demand SSB is configured and the SCell is deactivated (e.g., via MAC-CE) then the UE shall assume that the on-demand SSB is deactivated upon the deactivation of the SCell. |
| [12] CATT | Proposal 14: A unified group-common DCI could be designed to indicate on-demand SSB transmission. Proposal 15: Support the following options for OD-SSB deactivation: - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell. - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired. Proposal 16: If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst or via MAC CE. Proposal 17: On-demand SSB configuration is cell-specific. |
| [12] CATT | Proposal 27: For parameter change, an on-demand SSB transmission indication for an SCell could override a potentially ongoing on-demand SSB transmission for that SCell. For the other cases, the ongoing on-demand SSB should not be impacted by an on-demand SSB transmission indication. |
| [13] CMCC | Proposal 1: For the remaining issues in deactivation mechanism, support: - Up to RAN4 to determine the specific value of N2. - If od-ssb-nrofBurst is configured, the on-demand SSB deactivation based on value for od-ssb-nrofBurst is a baseline and the MAC CE based deactivation can be the enhancement on top of it. Proposal 2: For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) Proposal 3: For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), one candidate value is configured by RRC, and is applicable for multiple OD-SSB configuration, if any. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) for Case #1 - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) for Case #1 - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) for Case #1 Proposal 9: Use DCI 2_9 with CRC scrambled by ssbPeriodicityIndication-RNTI to indicate the transmission of OD-SSB for collision handling. - A new indication for the start position of OD-SSB indication in each information block can be introduced. |
| [14] Xiaomi | Proposal 2: For other cases other than the following case, support RRC based signaling to indicate on-demand SSB transmission. - This RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. Proposal 3: For a SCell supporting on-demand SSB operation, Option 1 can be used to deactivate on-demand SSB transmission in the following scenarios - scenario #2 and scenario #2A with Case #1 and Case #2 - scenario #3B and Case #2 Proposal 4: For a SCell supporting on-demand SSB operation, Option 4, 4A are not supported. - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired |
| [15] InterDigital | Proposal 1: If od-ssb-nrofBurst for an OD-SSB is configured, support deactivation of OD-SSB based on the configured value for od-ssb-nrofBurst or via MAC CE Proposal 2: UE follows the last received MAC CE on OD-SSB (i.e. MAC CE containing the OD-SSB configuration index) to identify the number N of OD-SSB bursts Proposal 6: For Case #2, a time offset parameter for indicating an offset between AO-SSB and OD-SSB bursts is not supported Proposal 7: Support explicit indication of deactivation for OD-SSB via MAC-CE for all supported scenarios (i.e. Scenarios #2, #2A and #3B) Proposal 8: Deactivation of OD-SSB when receiving SCell deactivation (Option 4) or expiry of SCell deactivation timer (Option 4A) are not supported Proposal 9: An additional timer for implicitly determining the value of N for the number of OD-SSB bursts is not introduced |
| [16] Ericsson | Proposal 1 Up to M OD-SSB configurations can be configured per SCell, wherein each OD-SSB configuration includes a single value (if configured) of OD-SSB periodicity (i.e., od-ssb-periodicity), OD-SSB positions in burst (i.e., od-ssb-PositionsInBurst), and number of OD-SSB burst (i.e., od-ssb-nrofBurst), and other parameters. Observation 1 If a single value of number of OD-SSB bursts is configured per OD-SSB configuration, the maximum number N2 of values of number of OD-SSB bursts is equal to the maximum number M of OD-SSB configurations. Proposal 2 Support up to 8 OD-SSB configurations per SCell. Observation 2 The functionality of Option 4 can be realized by Option 1 if a MAC CE that deactivates the OD-SSB can also deactivate the SCell. Proposal 3 Support restarting or prolonging an OD-SSB transmission at the same time as the SCell deactivation timer is restarted. Observation 3 The functionality of Option 4A can be realized by Option 2 if the on-demand-SSB transmission is restarted when the SCell deactivation timer is restarted. Proposal 4 Support MAC CE deactivation also for OD-SSB transmission configured with a value of number of OD-SSB bursts. Proposal 5 Default OD-SSB periodicity (if not configured) is 20 ms. |
| [18] KT | Proposal 1: Remove the bracket of the previous agreement, i.e., for a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),if od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst or via MAC CE. Proposal 2: Support DCI based signalling to indicate on-demand SSB transmission on the cell with group-common DCI. Proposal 7: An on-demand SSB transmission indication for an SCell (e.g., (de)activation or parameter change) overrides a potentially ongoing on-demand SSB transmission for that SCell. |
| [19] China Telecom | Observation 1: There is no benefit to support DCI based initial indication for on-demand compared with RRC/MAC CE based signalling for scenario #2 and #2A. Proposal 1: Support DCI based signalling for on-demand SSB indication for scenario #3B. Proposal 3: The value range of od-ssb-nrofBusrt N2 should be 2/4. Proposal 4: On-demand SSB should be supported to be deactivated via MAC CE if od-ssb-nrofBurst of an on-demand SSB is configured. Proposal 5: For the deactivation of on-demand SSB, Option 1, i.e., Explicit indication of deactivation for on-demand SSB via MAC-CE, can be applied to all the scenarios. Proposal 6: For the deactivation of on-demand SSB, Option 4 and 4A, i.e., the deactivation of SCell, can be sued as the deactivation signal in addition to Option 2. Proposal 7: For the deactivation of on-demand SSB, there is no need to further support to indicate value of N can be implicitly using a timer for Option 2. |
| [21] Fujitsu | Proposal 3: If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB can be deactivated via MAC CE. Observation 1: Option 3 (simultaneous CD-SSB and NCD-SSB configuration as shown in Figure 3) requires BWP-specific on-demand SSB configuration to enable on-demand SSB operation for Redcap UEs. However, whether to support this option requires further discussion. Proposal 4: RAN1 to clarify whether to support Option 3 (simultaneous CD-SSB and NCD-SSB transmission as shown in Figure 3) to enable on-demand SSB SCell operation for both non-RedCap UEs and RedCap UEs. |
| [22] Lenovo | Proposal 2: If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated only based on the configured value for od-ssb-nrofBurst. Proposal 3: Do not support Option 4 and Option 4a on top of Option 2 for deactivation of OD-SSB. |
| [23] ETRI | Proposal 2: It is proposed not to support DCI-based indication for indicating on-demand SSB transmission. Proposal 3: It is proposed to specify that on-demand SSB configuration is cell-specific, rather than BWP-specific. Proposal 4: It is proposed to discuss whether the UE is required to support BWP switching on the SCell with on-demand SSB for Case #1. - If BWP switching is supported, the switched BWP shall include on-demand SSB. - Alternatively, BWP switching is not supported with on-demand SSB operation. |
| [24] Transsion | Proposal 3 DCI based signaling to indicate on-demand SSB transmission can be supported. Proposal 4 If DCI based signaling support on-demand SSB transmission, DCI is UE-specific. Proposal 5 Option 4, 4A can be used to deactivate on-demand SSB transmission. Proposal 6 For case #2, using time offset between always-on SSB and on-demand SSB to define time domain location of on-demand SSB should be supported. |
| [26] Panasonic | Proposal 7: For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), if od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst or via MAC CE. Proposal 8: In addition to RRC and MAC CE based SSB trigging, DCI-based on-demand SSB triggering and deactivation indication is supported by the DCI format 2_9 framework defined in 9.5.3. Proposal 9: On whether/how to indicate time offset between always-on SSB and on-demand SSB, we can see the on-demand SSB feature can work without supporting new pattern for sake of progress. If a new pattern is to be supported, the specification efforts and impact need to be minimized. |
| [27] LG Electronics | Proposal #7: When on-demand SSB transmission is indicated by a RRC, UE expects that the value of N is NOT provided by the RRC. - The on-demand SSB activated by RRC signaling can be deactivated by applying Option 1 (Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication) and cannot be deactivated by applying Option 2 (Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated). Proposal #8: RAN1 to discuss the following values for od-ssb-nrofBurst as the starting point and send an LS to RAN4 to confirm whether those values are feasible or not. - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}. - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}. Proposal #9: For two options to deactivate on-demand SSB transmission from a UE perspective, Option 1 (Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication) and Option 2 (Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated) should be used/adopted exclusively. Update the previous agreement as follows. Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].. Proposal #10: Support on-demand SSB configuration per BWP similar to NCD-SSB configured with the higher layer parameter nonCellDefiningSSB. Proposal #12: For the following parameter, multiple candidate values can be configured (including the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) Proposal #13: For Case #2, the periodicity of OD-SSB (i.e., od-ssb-Periodicity) can be absent within od-ssb-config. - If absent, the periodicity of OD-SSB is the same as that of AO-SSB (i.e., ssb-Periodicity). |
| [28] MediaTek | Proposal 1: When there is always-on SSB transmitted in the cell, use "SFN offset" and "half frame index" to indicate the time domain position of on-demand SSB. No need to indicate time offset between always-on SSB and on-demand SSB. |
| [29] Apple | Observation 6: RAN2 agreed to support RRC based deactivation of OD-SSB transmission. Only {RRC based activation} + {RRC based deactivation} is supported in conjunction with RRC based deactivation. In consequence, RRC based adaptation is not supported and MAC CE based adaptation is not supported with RRC based activation/deactivation. Proposal 6: - RRC based adaptation is NOT supported. - MAC CE based adaptation is not supported with RRC based activation/deactivation. Observation 10-1: The agreement of configuring time domain location of OD-SSB per OD-SSB periodicity does not necessarily mean multiple values of time domain location of OD-SSB are supported. Rather, the agreed RRC parameters (R1-2503155) shows it is an open issue. Observation 10-2: The benefit to allow OD-SSB adaptation for different time location of SFN offset and half frame index is not proven. Observation 10-3: Allowing more parameters in addition to periodicity, SSB positions in burst, and number N can increase the complication of not only network but UE implementations without clear benefit. Proposal 10: No more parameters are allowed for OD-SSB adaptation than periodicity, SSB positions in burst and number N for implicit OD-SSB deactivation. |
| [30] Qualcomm | Observation 2: At least for Case #2, the adaptation of SSB burst periodicity via DCI 2_9 in AI 9.5.3 can already achieve the same purpose of DCI based signaling to indicate on-demand SSB transmission. Proposal 2: DCI based signaling to indicate on-demand SSB transmission on the cell is not supported. Proposal 12: The maximum number of SSB burst periodicities to be configured as part of OD-SSB configuration is 2. |
| [31] NTT DOCOMO | Proposal 8: N2 integer values for the value range of od-ssb-nrofBurst should be 8 Proposal 9: For deactivation mechanism, we support either one from following directions. - 1. if od-ssb-nrofBurst for an OD-SSB is configured, the OD-SSB can be deactivated via MAC CE - 2. deactivation mechanism of option 4/4a should be supported. |
| [33] KDDI | Proposal 4: DCI based signaling to indicate on-demand SSB transmission on the cell is not supported. |
| [34] ASUSTeK | Observation 1: Fixing on-demand SSB transmission with a single periodicity for an SCell whenever there is at least one UE consider the SCell as activated is harmful to the network energy saving gain of on-demand SSB. Observation 2: there are be two cases where a group common DCI could be utilized to indicate on-demand SSB transmission: 1. when the on-demand SSB on a SCell is triggered due to activation of the SCell to other UEs 2. when the periodicity of on-demand SSB is adjusted Proposal 1: RAN1 further discuss whether using group common DCI to indicate on-demand SSB transmission for the case of: 1. when the on-demand SSB on a SCell is triggered due to activation of the SCell to other UEs 2. when the periodicity of on-demand SSB is adjusted Proposal 2: If group common DCI indicating on-demand SSB transmission is supported, RAN1 further investigates whether there is any misalignment issue between UE and gNB. |
| [35] CEWiT | Proposal 3: Support MAC CE-based deactivation as an optional enhancement on top of od-ssb-nrofBurst when configured. |

Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement (RAN1#118bis) For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement (RAN1#119) - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement (RAN1#119) For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer Agreement (RAN1#120) - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Agreement (RAN1#120) - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list Agreement (RAN1#120bis) For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE]. Agreement (RAN1#120bis) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions Agreement (RAN1#120bis) For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

## [Moderator’s note] Regarding signaling methods for on-demand SSB transmission indication, company views are summarized as follows.

- For Option 1

  - Apply to all scenarios: InterDigital, China Telecom

  - Apply to Scenarios #2/2A: Xiaomi (for Case #1 and Case #2)

  - Apply to Scenario 3B: Xiaomi (for Case #2)

- For Option 2

  - Implicitly determine N by using timer

    - NO: ZTE, vivo, InterDigital, China Telecom

- Further support

  - Option 4: ZTE, vivo, Ofinno, CATT, China Telecom, Transsion, NTT DOCOMO

    - NO: Futurewei, Nokia, Xiaomi, InterDigital, Ericsson, Lenovo

  - Option 4A: ZTE, vivo, CATT, China Telecom, Transsion, NTT DOCOMO

    - NO: Futurewei, Nokia, Xiaomi, InterDigital, Ericsson, Lenovo

- Ericsson: Support restarting or prolonging an OD-SSB transmission at the same time as the SCell deactivation timer is restarted.

- For DCI based signaling

  - Supported by NEC (via DCI format 1_x), TCL, CATT, CMCC, KT, China Telecom (for Scenario #3B), Transsion (UE-specific), Panasonic, ASUSTeK

  - Negative: Ofinno, ETRI, Qualcomm, KDDI

- For RRC

  - Support other cases: Xiaomi

- When the od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB can be deactivated via MAC CE.

  - YES: Futurewei, ZTE, vivo, Nokia, Spreadtrum, Samsung, CATT, InterDigital, Ericsson, KT, China Telecom, Fujitsu, Panasonic, NTT DOCOMO

  - NO: Lenovo, LG Electronics

- Nokia, vivo, CATT, InterDigital, KT: MAC CE indication of OD-SSB transmission overrides potentially ongoing OD-SSB SSB transmission on the SCell.

  - InterDigital: UE follows the last received MAC CE on OD-SSB (i.e. MAC CE containing the OD-SSB configuration index) to identify the number N of OD-SSB bursts

- Combinations of OD-SSB operation

  - Samsung: Support all combinations

    - When on-demand SSB transmission is indicated to be activated by the RRC, the UE applies the parameters associated with the first configuration within the list of configurations included the RRC for the on-demand SSB transmission.

  - Apple: No support for Case A1/A2/X1/X3,

  - For activation

    - Case A1: activation by a RRC and N is not provided + adaptation by a MAC CE;

    - Case A2: activation by a RRC and N is provided + adaptation by a MAC CE;

    - Case B1: activation by a MAC CE and N is not provided + adaptation by a MAC CE;

    - Case B2: activation by a MAC CE and N is provided + adaptation by a MAC CE.

  - For deactivation

    - Case X1: activation by a RRC and N is not provided + deactivation by a MAC CE;

    - Case X2: activation by a RRC and N is provided + deactivation after N transmission bursts;

    - Case X3: activation by a RRC and N is provided + deactivation by a MAC CE;

    - Case Y1: activation by a MAC CE and N is not provided + deactivation by another MAC CE;

    - Case Y2: activation by a MAC CE and N is provided + deactivation after N transmission bursts;

    - Case Y3: activation by a MAC CE and N is provided + deactivation by another MAC CE;

    - Case Z1: adaptation by a MAC CE and N is not provided + deactivation by another MAC CE;

    - Case Z2: adaptation by a MAC CE and N is provided + deactivation after N transmission bursts;

    - Case Z3: adaptation by a MAC CE and N is provided + deactivation by another MAC CE.

## [Moderator’s note] Regarding contents of on-demand SSB configuration/indication, company views are summarized as follows.

- od-ssb-config signaling structure

  - Multiple od-ssb-config configurations: Ericsson

  - All related parameters as list: Futurewei, Samsung

    - Samsung: Up to 16 candidate values for each of parameters

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

  - Single candidate: Samsung, CMCC, Apple

  - Multiple candidates: LG Electronics

- Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Multiple candidate values: Samsung, CMCC, vivo

  - NO: Apple

- Periodicity

  - Ericsson: Default value as 20 ms

  - LG Electronics: Default value as the same one for AO-SSB

  - Qualcomm: Maximum number = 2

- N

  - Value range

    - CMCC: Up to RAN4

    - Ericsson, LG Electronics, NTT DOCOMO: N2=8

    - China Telecom: N2=2 or 4

    - LG Electronics

      - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}.

      - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}.

  - For Case #1, NOT configured: Futurewei

  - vivo: Only in MAC CE

  - Non-numerical indication of N: vivo (on-demand SSB is deactivated when SCell is deactivated)

- Cell-specific vs. BWP-specific configuration for on-demand SSB

  - Cell-specific: Nokia, Ofinno, CATT, ETRI

  - BWP-specific: LG Electronics

  - Fujitsu: RAN1 to clarify whether to support Option 3 (simultaneous CD-SSB and NCD-SSB transmission as shown in Figure 3) to enable on-demand SSB SCell operation for both non-RedCap UEs and RedCap UEs.

<Captured from [21] Fujitsu>

- FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

  - YES: TCL, Ofinno, Transsion

  - NO: Futurewei, vivo, InterDigital, MediaTek

- vivo: For parameter adaptation during Scenario #3B, only periodicity can be changed, by satisfying subset relationship

- Nokia: Remove od-ssb-physCellId from the list

### [HIGH] Proposal #4-1 (od-ssb-nrofBurst):

For a cell supporting on-demand SSB SCell operation, for configuring od-ssb-nrofBurst of which the value range is {N2 integer values},

- N2= [8]

  - Note: This is updated from the previous RAN1 agreement.

- The following values for od-ssb-nrofBurst are taken as the starting point and send an LS to RAN4 to confirm whether those values are feasible or not.

  - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}.

  - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}.

Companies are encouraged to provide views on Proposal #4-1.

| Company | Views |
| --- | --- |
| Samsung | What is the reason for choosing the values? |
| Google | OK |
| InterDigital | Fine |
| ZTE, Sanechips | Fine with FL’s proposal. |
| LGE | We think that the proposal can be sent to RAN4 as LS without change. If the big change of N2 and the value ranges for FR1 and FR2 are needed, the decision can be made by RAN4 later. |
| Futurewei | OK |
| Fujitsu | We are fine with the value of N2. Regarding the value ranges of od-ssb-nrofBurst, they can be decided by RAN4. |
| Vivo | Ok |
| ETRI | OK |
| NEC | Support |
| CMCC | The value of od-ssb-nrofBurst depends on purpose and scenario that OD-SSB is activated, which shall be decided by RAN4. |
| Nokia, NSB | We are fine to remove square brackets around the value “8”. |
| Qualcomm | For Case #1, it was agreed that UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated. Since the time when the activated cell becomes deactivated is uncertain from both UE and NW, how do we know whether the above values of od-ssb-nrofBurst are good or not? If the value is not able to cover (uncertain) duration of Scell activation and activated cell, the gNB may need to send multiple MAC-CEs to update the value if UE supports MAC-CE based OD-SSB tx indication, which may not be efficient from NES perspective. For the case where the OD-SSB tx indication by RRC and deactivation by RRC, the MAC-CE based OD-SSB update may not be supported. For this case, the improper configuration of the value may not make “UE does not expect the OD-SSB transmission indicated by RRC to be deactivated while the SCell is activated.” achievable. If RRC reconfiguration is needed to update this value, it is not efficient. Therefore, we think the following should be captured in the proposal: - If the RRC/MAC-CE based indication provides a value of od-ssb-nrofBurst that triggers the deactivation of OD-SSB transmission while the Scell is activated, the UE expects the OD-SSB according to the RRC/MAC-CE based indication is transmitted until the time Scell deactivation is completed. |
| Spreadtrum | Support |
| DCM | For N2, we are fine with the FL proposal. For the values for od-ssb-nrofBurst, we should let RAN4 decide the values without starting point. Providing values as starting point gives RAN4 assumption that RAN1 has come to conclude to take the fast measurement window based values, which has not agreed yet. |
| CATT | Fine with the value of N2. Regarding the value ranges of od-ssb-nrofBurst, they can be decided by RAN4. |
| China Telecom | We don’t understand the motivation to define the N2 with so many values, if so many candidate values are needed, why not use the explicit method. |
| Ofinno | Support. Agree with above comments that we should ask RAN4 to also confirm the value range of N2. |
| Moderator | Some background of how value ranges of od-ssb-nrofBurst are derived: - For a detected cell, Section 9.2.5.2 in TS 38.133 Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; TSSB_measurement_period_intra. No DRX; max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320 ms; max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. <FR1> Minimum od-ssb-nrofBurst = 5, assuming Kp = 1, SMTC period = OD-SSB periodicity, and CSSFintra = 1 Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps (FR2) DRX cycle; TSSB_measurement_period_intra. No DRX; max(400 ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320 ms; max(400 ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. <FR2> Minimum od-ssb-nrofBurst = 24, assuming Mmeas_period_w/o_gaps = 24 (for a UE supporting FR2-1 power class 3), Kp = 1, SMTC period = OD-SSB periodicity, and CSSFintra = 1 - For a newly detectable cell, Section 9.2.5.1 in TS 38.133 More number of od-ssb-nrofBurst is required. |

### [Closed] Proposal #4-2 (RRC+MAC-CE):

For a cell supporting on-demand SSB SCell operation,

- The following combinations are supported.

  - For OD-SSB transmission indication (OD-TI) and OD-SSB transmission adaptation (OD-TA),

    - Case A1: RRC-based OD-TI without N configured + MAC CE-based OD-TA;

      - Objected by Apple, Qualcomm

    - Case A2: RRC-based OD-TI with N configured + MAC CE-based OD-TA;

      - Objected by Apple, Qualcomm

    - Case B1: MAC CE-based OD-TI without N configured + MAC CE-based OD-TA;

      - Apple: MAC CE-based OD-TA “without N configured”

    - Case B2: MAC CE-based OD-TI with N configured + MAC CE-based OD-TA.

      - Apple: MAC CE-based OD-TA “with N configured”

  - For OD-SSB transmission deactivation (OD-TD),

    - Case X1: RRC-based OD-TI without N configured + MAC CE-based OD-TD;

      - Objected by Apple, Qualcomm

    - Case X2: RRC-based OD-TI with N configured + implicit OD-TD;

    - Case X3: RRC-based OD-TI with N configured + MAC CE-based OD-TD;

      - Objected by Apple, Qualcomm

    - Case Y1: MAC CE-based OD-TI or OD-TA without N configured + MAC CE-based OD-TD;

      - Apple: MAC CE-based OD-TA “without N configured”

    - Case Y2: MAC CE-based OD-TI or OD-TA with N configured + implicit OD-TD;

    - Case Y3: MAC CE-based OD-TI or OD-TA with N configured + MAC CE-based OD-TD.

      - Apple: MAC CE-based OD-TA “with N configured”

  - Note: Some of the above can be subject to UE capability, based on discussion under 9.15.4.

  - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

- [MAC CE-based OD-TI overrides potentially ongoing OD-SSB transmission on the SCell.]

- [For the case of RRC-based OD-TI, the UE applies the parameters associated with the first configuration within the list of configurations included in the RRC for the on-demand SSB transmission.]

Companies are encouraged to provide views on Proposal #4-2.

| Company | Views |
| --- | --- |
| Samsung | We support the proposal. |
| Google | OK |
| ZTE, Sanechips | OK. For the last bullet, we think it should be firstly clarified that RRC would configure only one OD-SSB configuration in RRC-based OD-SSB indication or RRC would configure multiple OD-SSB configurations and only indicate one of OD-SSB configurations. |
| LGE | It is preferred |
| Futurewei | OK |
| Xiaomi | We think any combination between agreed OD-SSB activation and OD-SSB deactivation is feasible. We are not sure what does the following mechanism mean: - Another MAC CE-based OD-TI - Implicit OD-TD - Another MAC CE-based OD-TD |
| Vivo | Ok |
| ETRI | OK |
| NEC | Support |
| CMCC | Ok. |
| Apple | 1/ For the following proposal, we don’t think it is needed as the design is up to RAN2. In addition, the current RAN2 design is already accommodating any RRC parameter for the case when RRC based OD-SSB activation and configuration arrives at the same time. 2/ We don’t think it is a good idea to mix RRC with MAC CE operation. 3/ We think ‘adaptation’ needs to be clarified rather than using ‘activation’ implying both activation and adaptation. Having said that, we proposed the following update. For a cell supporting on-demand SSB SCell operation, - The following combinations are supported. - For OD-SSB transmission indication (OD-TI) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-TI without N configured + MAC CE-based OD-TI; - Case A2: RRC-based OD-TI with N configured + MAC CE-based OD-TI; - Case B1: MAC CE-based OD-TI without N configured + another MAC CE-based OD-TAI without N configured; - Case B2: MAC CE-based OD-TI with N configured + another MAC CE-based OD- TAI with N configured. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-TI without N configured + MAC CE-based OD-TD; - Case X2: RRC-based OD-TI or OD-TA with N configured + implicit OD-TD; - Case X3: RRC-based OD-TI with N configured + MAC CE-based OD-TD; - Case Y1: MAC CE-based OD-TI or OD-TA without N configured + another MAC CE-based OD-TD without N configured; - Case Y2: MAC CE-based OD-TI or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-TI with N configured + another MAC CE-based OD-TD. (note: this example may cause ambiguity from RAN2’s common signaling design between implicit and explicit deactivation) - Note: Some of the above can be subject to UE capability, based on discussion under 9.15.4. - MAC CE-based OD-TI overrides potentially ongoing OD-SSB transmission on the SCell.(note: this can be addressed by adding OD-TA as above) - For the case of RRC-based OD-TI, the UE applies the parameters associated with the first configuration within the list of configurations included in the RRC for the on-demand SSB transmission. |
| TCL | Support |
| Qualcomm | We do not support this proposal. For the discussion under the first bullet, which combination is supported is under UE feature discussion. For the 2nd bullet, it is unclear – it really depends on whether the UE supports RRC based OD-TI + MAC-CE based OD-TI or not. For the case where OD-TI is by MAC-CE, RAN1 agreed that MAC-CE could be sent in scenario 3B to update some OD-SSB parameters. For the 3rd bullet, is a combination of RRC-based OD-TI + MAC-CE based OD-TI assumed? If this is the case, let’s discuss in UE feature on whether this combination can be supported. |
| DCM | For N2, we are fine with the FL proposal. For the values for od-ssb-nrofBurst, we should let RAN4 decide the values without starting point. Providing values as starting point gives RAN4 assumption that RAN1 has come to conclude to take the fast measurement window based values, which has not agreed yet. |
| China Telecom | Support. |
| Ofinno | Support |

### [HIGH] Proposal #4-2 (RRC+MAC-CE):

For a cell supporting on-demand SSB SCell operation,

- The following combinations are supported.

  - For OD-SSB transmission indication (OD-TI) and OD-SSB transmission adaptation (OD-TA),

    - Case A1: RRC-based OD-TI without N configured + MAC CE-based OD-TA;

      - Objected by Apple, Qualcomm

    - Case A2: RRC-based OD-TI with N configured + MAC CE-based OD-TA;

      - Objected by Apple, Qualcomm

    - Case B1: MAC CE-based OD-TI without N configured + MAC CE-based OD-TA;

      - Apple: MAC CE-based OD-TA “without N configured”

    - Case B2: MAC CE-based OD-TI with N configured + MAC CE-based OD-TA.

      - Apple: MAC CE-based OD-TA “with N configured”

  - For OD-SSB transmission deactivation (OD-TD),

    - Case X1: RRC-based OD-TI without N configured + MAC CE-based OD-TD;

      - Objected by Apple, Qualcomm

    - Case X2: RRC-based OD-TI with N configured + implicit OD-TD;

    - Case X3: RRC-based OD-TI with N configured + MAC CE-based OD-TD;

      - Objected by Apple, Qualcomm

    - Case Y1: MAC CE-based OD-TI or OD-TA without N configured + MAC CE-based OD-TD;

      - Apple: MAC CE-based OD-TA “without N configured”

    - Case Y2: MAC CE-based OD-TI or OD-TA with N configured + implicit OD-TD;

    - Case Y3: MAC CE-based OD-TI or OD-TA with N configured + MAC CE-based OD-TD.

      - Apple: MAC CE-based OD-TA “with N configured”

  - Note: Some of the above can be subject to UE capability, based on discussion under 9.15.4.

  - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

- [MAC CE-based OD-TI overrides potentially ongoing OD-SSB transmission on the SCell.]

- [For the case of RRC-based OD-TI, the UE applies the parameters associated with the first configuration within the list of configurations included in the RRC for the on-demand SSB transmission.]

Companies are encouraged to provide views on Proposal #4-2a.

| Company | Views |
| --- | --- |

### [MID] Proposal #4-3 (DCI):

- For a cell supporting on-demand SSB SCell operation,

  - DCI based signaling to indicate or deactivate on-demand SSB transmission is NOT supported in Rel-19.

Companies are encouraged to provide views on Proposal #4-3.

| Company | Views |
| --- | --- |
| Google | OK |
| InterDigital | Support |
| LGE | We don’t have enough time to discuss this proposal. So, we think that the DCI-based signaling should not be supported in Rel-19. |
| Futurewei | OK |
| Fujitsu | OK |
| Vivo | Ok |
| ETRI | OK |
| NEC | We see benefits for DCI, especially for reducing signaling overhead for L1 measurements or dynamic indications. |
| CMCC | DCI 2_9 does not indicate or activate OD-SSB transmission for some specific UEs, but can inform all UEs about on-demand SSB transmission for UL/DL collision handling. Based on RAN1#120b meeting agreement, Scenario #3B is supported and MAC-CE is used for updating the transmission parameter of a transmitted OD-SSB. Thus, even if on-demand SSB is triggered for some UEs, the transmission of on-demand SSB should be awarded by all the CONNECTED UEs under Scenario #3B in this SCell. One way to solve this issue is to indicate each UE with MAC CE signaling, which is a UE-specific behavior and results in a significantly overhead. The other way is to introduce a group-common signaling, i.e., reuse the DCI 2_9 design in SSB adaptation to inform transmission of OD-SSB to UEs with feasible signaling overhead. Through informing transmission of OD-SSB to all UEs, the UE behavior on rate matching of PDSCH, PDCCH monitoring, UL (e.g. PUSCH, PUCCH, PRACH) transmission will be impacted by OD-SSB. |
| Nokia, NSB | Agree |
| TCL | Do not support this proposal. In RAN1#120 meeting, DCI format 2_9 has been agreed for the indication of periodicity adaptation of SSB. Same comments like CMCC, we think DCI 2_9 as a group-common signaling could be introduced and reused to inform OD-SSB transmission, even for activation or deactivation of multiple candidate values agreed in RAN 1 #120 b. |
| Qualcomm | We support this proposal |
| Panasonic | As DCI format 2_9 framework is already there, we proposal to use it also for this agenda. |
| KDDI | OK. |
| Spreadtrum | Fine with proposal |
| DCM | We are fine with the proposal. |
| China Telecom | DCI based indication can be beneficial for Scenario #3B. And considering the limited time left, we agree with CMCC the DCI format 2_9 can be reused for OD-SSB. |
| Ofinno | Support |

### [MID] Proposal #4-4 (Cell-specific vs. BWP-specific):

- For a cell supporting on-demand SSB SCell operation,

  - On-demand SSB configuration is cell-specific.

  - The case where CD-SSB in a BWP and NCD-SSB in another BWP are simultaneously configured for the cell is NOT supported in Rel-19.

Companies are encouraged to provide views on Proposal #4-4.

| Company | Views |
| --- | --- |
| Samsung | We are ok with the first sub-bullet, and wonder what is the motivation for the second sub-bullet. |
| Google | OK |
| ZTE, Sanechips | At least fine with the first sub-bullet, and open to discuss the second sub-bullet. |
| LGE | Whether On-demand SSB configuration is cell-specific or BWP-specific is up to gNB implementation. When OD-SSB is only configured in BWP 1 and Active BWP is BWP 2 from BWP 1, MG is needed to measure OD-SSB. But, When OD-SSBs are configured in Dedicated BWP 1 and BWP 2, MG is not needed to measure any OD-SSB. But, If On-demand SSB configuration is confined to cell-specific, we think that the frequency location should be also confined within firstActiveDownlinkBWP for Scenario 3A where MG is not required. |
| Futurewei | OK |
| Xiaomi | We think cell-specific configuration is sufficient. |
| Fujitsu | OK |
| vivo | OK, the second bullet seems redundant, since we have agreed that the CD and NCD SSB should in the same BWP |
| ETRI | OK |
| NEC | We are open to further discuss this issue |
| CMCC | Ok. |
| Nokia, NSB | Support the first bullet, The second bullet can be discussed. |
| TCL | Ok |
| Qualcomm | The first bullet is fine. We don’t understand the motivation and scenarios leading to the 2nd bullet. |
| DCM | Currently, indication mechanisms are based on UE dedicated signals, so we have concern if OD-SSB is based on cell specific works. It might need further study whether OD-SSB is configured per BWP. |
| China Telecom | OK. |
| Ofinno | Support. The second bullet is good to have to clarify the scenario |

# Remaining issues of TX behavior of on-demand SSB burst

| Company | Views |
| --- | --- |
| [17] Google | Proposal 1: Support the MAC-CE for OD-SSB activation to indicate value of the action delay T for the alignment of the updated configuration for the OD-SSB for multiple UEs. |
| [27] LG Electronics | Proposal #14: Adopt the following TP in TS 38.213 Section 4.4. <omitted text> When the activation of transmission for the SS/PBCH blocks is by od-ssb-config, the UE expects that the transmission of the SS/PBCH blocks are transmitted according to parameters indicated in od-ssb-config after receiving od-ssb-config [starts at the next half frame with transmission of the SS/PBCH blocks]. <omitted text>. |
| [29] Apple | Proposal 7: On the following agreement in RAN1 #120, it is to proposed to clarify: - For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission - Interpretation 2: The first on-demand SSB burst indicates the first actually transmitted SSB index of on-demand SSB burst according to SSB positions in burst. Proposal 8: For RRC based OD-SSB deactivation, UE expects that on-demand SSB is NOT transmitted after receiving RRC carrying deactivation of OD-SSB transmission. Proposal 9: Time instance C for MAC CE based OD-SSB adaptation is the same as time instance A for OD-SSB activation as agreed. |
| [30] Qualcomm | Proposal 13: The reference for determining the MAC-CE based indication/deactivation timeline of OD-SSB transmission is slots for the PUCCH transmission (similar to legacy Scell activation/deactivation timeline). In particular, adopt the following update to TS 38.213. 4.4 Activation/deactivation of SS/PBCH block transmissions on a secondary cell <ommited text> When the activation or adaptation of the SS/PBCH blocks transmission is by the [first or the third] MAC CE, respectively, [and with reference to slots of a configured DL BWP for the SS/PBCH blocks transmission for the PUCCH transmission,] the UE expects that the transmission of the SS/PBCH blocks according to the indicated parameters starts from a first slot including the candidate SS/PBCH block occasion corresponding to the first transmitted SS/PBCH block index and located in a first half frame within the half frames for the transmissions of the SS/PBCH blocks, that is at least m+3 Nslotsubframe,μ+1 slots after slot n, where n is a slot when a PDSCH reception providing the [first or the third] MAC CE ends, respectively, n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211]. When the activation of transmission for the SS/PBCH blocks is by od-ssb-config, the UE expects that the transmission of the SS/PBCH blocks [starts at the next half frame with transmission of the SS/PBCH blocks]. When the deactivation of the SS/PBCH blocks transmission is by the [second] MAC CE, [and with reference to slots of the configured DL BWP for the SS/PBCH blocks transmission for the PUCCH transmission,] the UE expects that the transmission of the SS/PBCH blocks according to the indicated parameters terminates from <ommited text>. |

Agreement (RAN1#120) For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

### [MID] Proposal #5-1 (TP#1 from LG Electronics):

Adopt the following TP in TS 38.213 Section 4.4.

<omitted text> When the activation of transmission for the SS/PBCH blocks is by od-ssb-config, the UE expects that the transmission of the SS/PBCH blocks are transmitted according to parameters indicated in od-ssb-config after receiving od-ssb-config [starts at the next half frame with transmission of the SS/PBCH blocks]. <omitted text>

Companies are encouraged to provide views on Proposal #5-1.

| Company | Views |
| --- | --- |
| Samsung | OK in general. |
| Google | OK |
| InterDigital | OK |
| LGE | [starts at the next half frame with transmission of the SS/PBCH blocks] can means that OD-SSB starts to be transmitted just at the next half frame after UE receives RRC carrying indication of OD-SSB transmission. According to the agreement in RAN1#120, OD-SSB should be transmitted on the parameters indicated by an RRC carrying indication of OD-SSB transmission, irrespective of the timing when UE receives the RRC. |
| Futurewei | OK |
| Xiaomi | We are fine with the proposal. |
| Fujitsu | OK |
| Vivo | OK |
| ETRI | OK |
| NEC | Support |
| CMCC | Ok. |
| Nokia, NSB | Support |
| TCL | OK |
| Qualcomm | We don’t see the issue of the current spec text. Hence, we don’t see the proposed text is needed. |
| DCM | support |
| China Telecom | OK. |
| Ofinno | OK |

### [MID] Proposal #5-2 (TP#2 from Qualcomm):

The reference for determining the MAC-CE based indication/deactivation timeline of OD-SSB transmission is slots for the PUCCH transmission (similar to legacy Scell activation/deactivation timeline).

- Adopt the following TP to TS 38.213 Clause 4.4.

4.4 Activation/deactivation of SS/PBCH block transmissions on a secondary cell <ommited text> When the activation or adaptation of the SS/PBCH blocks transmission is by the [first or the third] MAC CE, respectively, [and with reference to slots of a configured DL BWP for the SS/PBCH blocks transmission for the PUCCH transmission,] the UE expects that the transmission of the SS/PBCH blocks according to the indicated parameters starts from a first slot including the candidate SS/PBCH block occasion corresponding to the first transmitted SS/PBCH block index and located in a first half frame within the half frames for the transmissions of the SS/PBCH blocks, that is at least m+3 Nslotsubframe,μ+1 slots after slot n, where n is a slot when a PDSCH reception providing the [first or the third] MAC CE ends, respectively, n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211]. When the activation of transmission for the SS/PBCH blocks is by od-ssb-config, the UE expects that the transmission of the SS/PBCH blocks [starts at the next half frame with transmission of the SS/PBCH blocks]. When the deactivation of the SS/PBCH blocks transmission is by the [second] MAC CE, [and with reference to slots of the configured DL BWP for the SS/PBCH blocks transmission for the PUCCH transmission,] the UE expects that the transmission of the SS/PBCH blocks according to the indicated parameters terminates from <ommited text>

Companies are encouraged to provide views on Proposal #5-2.

| Company | Views |
| --- | --- |
| Sansung | We don’t agree – following similar behavior as SCell activation, the SCS shall be DL SCS. |
| Google | OK |
| LGE | OK |
| Futurewei | OK |
| Xiaomi | We are fine with the proposal. |
| Fujitsu | OK |
| Vivo | OK |
| ETRI | OK |
| CMCC | Ok. |
| TCL | OK |
| Qualcomm | We support this proposal. @Samsung: The proposed text is to follow the current Scell activation timeline. TS 38.213 4.3 Timing for secondary cell activation / deactivation With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot n+k, <ommited text> With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot n+k. <ommited text> |
| DCM | We support. |
| China Telecom | OK. |
| Ofinno | OK |

# Remaining issues on OD-SSB properties

## OD-SSB periodicity

| Company | Views |
| --- | --- |
| [5] Nokia | Proposal 3: The periodicity of OD-SSB cannot be configured to be larger than the periodicity of always-on SSB. |
| [10] Tejas | Proposal 3: Periodicity of OD-SSB burst periodicity can be independent of AO-SSB periodicity. |
| [11] Ofinno | Proposal 1: RAN1 to agree “The periodicity of OD-SSB is equal to or smaller than that of AO-SSB in Case #2”. |
| [12] CATT | Proposal 18: Regarding the relation in terms of periodicity between always-on SSB and OD-SSB, the periodicity of on-demand SSB should be equal to or smaller than that of always-on SSB. |
| [14] Xiaomi | Proposal 5: For Case#2, the periodicity of on-demand SSB should be equal to or smaller than that of always-on SSB. |
| [18] KT | Proposal 9: Consider the inter-operatable RRC configuration for the periodicity and positions in burst for on-demand-SSB with always-on-SSB. Proposal 10: Adaptation of period is supported for on-demand SSB. |
| [23] ETRI | Proposal 8: It is proposed to specify a configuration limitation that the periodicity of on-demand SSB shall be equal to or smaller than that of always-on SSB. |
| [29] Apple | Observation 5: There is no network energy saving gain in Case #2 when the periodicity of AO-SSB is smaller than that of OD-SSB. Proposal 5: The case when the periodicity of AO-SSB is smaller than that of OD-SSB is deprioritized. |

Agreement (RAN1#119) Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress.

## [Moderator’s note] Company views on OD-SSB periodicity:

- Is the configuration limitation that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB needed?

  - YES: Nokia, Ofinno, CATT, Xiaomi, KT?, ETRI, Apple?

  - Up to gNB: Tejas

### [LOW] Proposal #6-1 (Periodicity):

- For a cell supporting on-demand SSB SCell operation,

  - The periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Companies are encouraged to provide views on Proposal #6-1.

| Company | Views |
| --- | --- |
| Samsung | No need for restriction. Up to gNB implementation. |
| Google | We failed to see the necessity |
| ZTE, Sanechips | Support FL’s proposal. The motivation of configuring periodicity of OD-SSB larger than that of AO-SSB is not clear. |
| LGE | We think the proposal should be specified. If gNB configures the lager periodicity of on-demand than that of always-on SSB, there is no reason to indicate and configure OD-SSB. Such configuration/indication can be against Network Energy Saving and UE power consumption. |
| Futurewei | We prefer to leave it to the UE implementation |
| Xiaomi | Support. |
| Fujitsu | We are fine with the proposal. |
| ETRI | Support |
| NEC | Support |
| CMCC | Support but this can be achieved via gNB implementation. |
| Nokia, NSB | Support |
| TCL | Support |
| China Telecom | OK. |
| Ofinno | Support |

## Time domain relation of OD-SSB

| Company | Views |
| --- | --- |
| [2] Huawei | Proposal 5: For the case when the centre frequency locations of AO-SSB and OD-SSB are different, PBCH payload of OD-SSB can be known to the UE by configuration. |
| [3] ZTE | Observation 3: Time-C2 allows the half frame index of OD-SSB and AO-SSB to be different. Proposal 5: When the center frequency locations of AO-SSB and OD-SSB are different, there should be no restriction requiring that PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB. |
| [4] vivo | Proposal 6: For the case when the center frequency locations of always-on SSB and on-demand SSB are different, other than SFN index, half frame index, k_SSB and pdcch-ConfigSIB1 could also be different between always-on SSB and on-demand SSB in PBCH payload for the same SSB index. Proposal 7: For the case when the center frequency locations of always-on SSB and on-demand SSB are same，the half frame index of AO-SSB and OD-SSB should be same in Alt Time C1 and could be different in Alt Time-C2. |
| [5] Nokia | Proposal 4: For Case#2, when AO-SSB and OD-SSB are on different frequencies, PBCH payload for same SSB index shall be different due to the k_SSB contents. |
| [6] NEC | Proposal 7: For Case #2, when AO-SSB and OD-SSB share the same centre frequency, support half frame index for OD-SSB to be the same as or different from the half-frame index associated with AO-SSB. Proposal 8: For Case #2, when AO-SSB and OD-SSB have different centre frequencies: - Confirm that the MIB parameters k_SSB (ssb-SubcarrierOffset), pdcch-ConfigSIB1, and halfFrameIndex for OD-SSB can be different than those of AO-SSB. - Agree that other MIB parameters (dmrs-TypeA-Position, subCarrierSpacingCommon, cellBarred, intraFreqReselection) for OD-SSB are the same as those broadcast in the PBCH of the AO-SSB. |
| [9] Samsung | Proposal 5: For the case when the center frequency locations of always-on SSB and on-demand SSB are different: - On-demand SSB is a NCD-SSB; - Transmitted SSB indexes for on-demand SSB are same as or a subset of the ones for always-on SSB. |
| [10] Tejas | Proposal 1: For the case when the centre frequency locations of always-on SSB(AO-SSB) and on-demand SSB(OD-SSB) are same, half frame index can be same or different for AO-SSB and OD-SSB. Proposal 2: For the case when the center frequency locations of always-on SSB and on-demand SSB are different, the PBCH payload for AO-SSB and OD-SSB can be same or different. |
| [11] Ofinno | Proposal 2: For the case when frequency locations of AO-SSB and OD-SSB are the same, the half frame index can be the same or different for AO-SSB and OD-SSB. Proposal 3: For the case when frequency locations of AO-SSB and OD-SSB are different, PBCH payload for the same SSB index (other than SFN index, half frame index) does not have to be the same for AO-SSB and OD-SSB. Proposal 4: We propose the following changes to the previous agreement as follows: Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - Active AO-SSB and OD-SSB are located in an active the same BWP - OD-SSB is NCD-SSB - FFS: Additional conditions - Subject to separate UE capability Note: UE is not required to measure both AO-SSB and OD-SSB Proposal 5: Other potential conditions are left for RAN4 to decide, e.g., intra-frequency measurement. |
| [12] CATT | Proposal 19: For the case when the center frequency locations of always-on SSB and on-demand SSB are the same, half frame index should be the same for AO-SSB and OD-SSB. Proposal 20: For the case when the center frequency locations of always-on SSB and on-demand SSB are different, it is up to the gNB implementation whether PBCH payload for the same SSB index (other than SFN index, half frame index) is the same or different for AO-SSB and OD-SSB. |
| [13] CMCC | Proposal 5: When the center frequency locations of AO-SSB and OD-SSB are same, there is no need to restrict the half frame index to be same or different. Proposal 6: When the center frequency locations of AO-SSB and OD-SSB are different, there is no need to restrict the PBCH payload for the same SSB index (other than SFN index, half frame index) to be same. |
| [14] Xiaomi | Proposal 6: For AO-SSB and OD-SSB with same SSB index, SFN index and half frame index can be same or different. In addition, the ssb-SubcarrierOffset should be different for OD-SSB and AO-SSB if their centre frequency is different. |
| [15] InterDigital | Proposal 5: For the case when the center frequency locations of AO-SSB and OD-SSB are different, no restriction for the PBCH payload to be the same for both AO-SSB and OD-SSB for the same SSB index |
| [18] KT | Proposal 3: Proposal #7-2 in [2] is supported, i.e., It is up to gNB implementation for the followings: - Whether half frame index is the same or different for AO-SSB and OD-SSB for the case when the center frequency locations of AO-SSB and OD-SSB are same - Whether PBCH payload for the same SSB index (other than SFN index, half frame index) is same or different for AO-SSB and OD-SSB for the case when the center frequency locations of AO-SSB and OD-SSB are different |
| [21] Fujitsu | Proposal 1: For the case where the center frequency locations of AO-SSB and OD-SSB are same, half frame index can be the same or different for AO-SSB and OD-SSB. Proposal 2: When the center frequency locations of AO-SSB and OD-SSB are different, the PBCH payload for the same SSB index may be different for AO-SSB and OD-SSB. |
| [23] ETRI | Proposal 5: For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - It is up to gNB implementation whether half frame index is the same or different for always-on SSB and on-demand SSB Proposal 6: For the case where the center frequency locations of the always-on SSB and the on-demand SSB are different: - It is up to gNB implementation whether on-demand SSB is configured as a CD-SSB. - It is up to gNB implementation whether PBCH payload for the same SSB index (excluding SFN index and half frame index) is the same or different between always-on SSB and on-demand SSB. |
| [25] OPPO | Proposal 3: Support the same half frame index for AO-SSB and OD-SSB for the case when the center frequency locations of AO-SSB and OD-SSB are same. Proposal 4: PBCH payload for the same SSB index (other than SFN index, half frame index) can be different for AO-SSB and OD-SSB for the case when the center frequency locations of AO-SSB and OD-SSB are different. |
| [28] MediaTek | Proposal 2: At least ssb-SubcarrierOffset (k_ssb, see Fig. 1) inside MIB should be different for AO-SSB and OD-SSB when center frequency locations of always-on SSB and on-demand SSB are different. |
| [29] Apple | Observation 2: To make PBCH payload for the same index being the same for AO-SSB and OD-SSB (other than SFN index, half frame index) for different frequency case can reduce the number of combinations to 8. Proposal 2: Confirm the following FFS to agreement: - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB |
| [30] Qualcomm | Proposal 3: For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - For Alt Time C-1, the half-frame index is different for AO-SSB and OD-SSB if the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern of 5ms periodicity. Otherwise, the half-frame index is identical for AO-SSB and OD-SSB. - For Alt Time C-2, the half-frame index may be different for AO-SSB and OD-SSB. Proposal 4: For the case when the center frequency locations of always-on SSB and on-demand SSB are different, PBCH payload (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB for the same SSB index. |
| [31] NTT DOCOMO | Proposal 2: Support, half frame index can be the same or different for AO-SSB and OD-SSB. It is up to NW implementation. Proposal 3: In terms of the content of PBCH payload for both OD-SSB and AO-SSB with same or different center fre-quency, - We do not support the restriction that the PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - E.g., the value of PDCCH-ConfigSIB1, k_ssb and cell barring information can be different. |
| [35] CEWiT | Proposal 2: Support allowing the half-frame index to be either the same or different, without restriction. |

Agreement (RAN1#119) Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1: - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB Agreement (RAN1#120) Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following. - Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst. - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB. - Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB. - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB. - Alt Time-C: The specification has no restriction with regards to overlapping Agreement (RAN1#120) Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

## [Moderator’s note] Company views on FFS points:

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

    - No limitation: CMCC, KT, ETRI, NTT DOCOMO, CEWiT

    - Can be same or different: NEC, Tejas, Ofinno, Xiaomi, Fujitsu

    - Same: CATT, OPPO, Apple

    - vivo: Same in Alt Time-C1 but different in Alt Time-C2

    - Qualcomm: Same or different depending on alt-Time C1 or alt-Time C2 or SSB periodicity

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different

  - Huawei: PBCH payload provided by configuration

  - Samsung, Ofinno: OD-SSB is an NCD-SSB

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

    - No limitation: ZTE, CATT, CMCC, InterDigital, KT, ETRI, OPPO, NTT DOCOMO

    - Different: vivo/NEC (due to k_ssb and pdcch-ConfigSIB1), Nokia (due to k_ssb), Ofinno, Xiaomi, Fujitsu, MediaTek (due to k_ssb),

    - Tejas: Same or different

    - Same: Apple, Qualcomm

### [LOW] Proposal #6-2 (Time):

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - It is up to gNB implementation whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - It is up to gNB implementation whether PBCH payload for the same SSB index (other than SFN index, half frame index) is same or different for AO-SSB and OD-SSB

Companies are encouraged to provide views on Proposal #6-2.

| Company | Views |
| --- | --- |
| Samsung | OK with the proposal, or without any conclusion (which is same as the proposal). |
| Google | OK |
| ZTE, Sanechips | Fine with FL’s proposal. |
| LGE | Support it |
| Futurewei | OK |
| Fujitsu | Support |
| Vivo | OK |
| ETRI | OK |
| NEC | Support |
| CMCC | Ok. |
| Nokia, NSB | Support |
| TCL | Support |
| DCM | For half frame index, we are fine with the proposal. We think there is no need to restrict to the same center frequency case. For PBCH payload, we support the FL proposal. |
| Ofinno | Support |

## Frequency domain relation of OD-SSB

| Company | Views |
| --- | --- |
| [1] Futurewei | Proposal 1: If AO SSB is CD-SSB not located on the synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB. Observation 1: There is no strong motivation to support OD CD-SSB not on sync raster and AO NCD-SSB scenario. Proposal 2: If OD CD-SSB not on sync raster is supported, OD CD-SSB and AO NCD-SSB should have the same frequency location. |
| [4] vivo | Proposal 4: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster. Proposal 5: Always-on SSB and on-demand SSB are located in the same BWP with the smallest bandwidth, and the center frequency gap between always-on SSB and on-demand SSB are smaller than a pre-defined threshold. |
| [5] Nokia | Observation 1: For simplification of the options and progress towards a feasible solution, we can deprioritize the case when always-on SSB is CD-SSB not in the synchronization raster. Proposal 1: OD-SSB transmitted as CD-SSB located on sync-raster is supported in Case#1. |
| [6] NEC | Observation 1: For Case#1, gNB does not need to newly trigger on-demand SSB for a UE if the SCell is already active for another UE (FFS: indication). Observation 2: For Case#1, on-demand SSB is not expected to be deactivated as long as at least one UE is active on the Cell even when the SCell is deactivated for a UE for which the network triggered the on-demand SSB. Observation 3: On-demand SSB would be transmitted periodically for a while as long as at least one UE is active on the cell, as SCell is a capacity cell and traffic on the capacity cell would not be low. Proposal 1: When on-demand SSB is transmitting periodically, NES cell can be used as an SCell for non-NES UEs irrespective of whether on-demand SSB is CD-SSB. Proposal 2: At least for case#1, on-demand SSB can be CD-SSB and transmitted on synch-raster with assumption the network ensures periodic CD-SSB transmission when NES cell is activated for any UE. |
| [7] Spreadtrum | Proposal 1 For the case when AO-SSB is CD-SSB and not on a synchronization raster, the frequency location of OD-SSB and AO-SSB subject to network configuration. |
| [11] Ofinno | Proposal 2: For the case when frequency locations of AO-SSB and OD-SSB are the same, the half frame index can be the same or different for AO-SSB and OD-SSB. Proposal 3: For the case when frequency locations of AO-SSB and OD-SSB are different, PBCH payload for the same SSB index (other than SFN index, half frame index) does not have to be the same for AO-SSB and OD-SSB. Proposal 4: We propose the following changes to the previous agreement as follows: Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - Active AO-SSB and OD-SSB are located in an active the same BWP - OD-SSB is NCD-SSB - FFS: Additional conditions - Subject to separate UE capability Note: UE is not required to measure both AO-SSB and OD-SSB Proposal 5: Other potential conditions are left for RAN4 to decide, e.g., intra-frequency measurement. Proposal 6: There is no strong motivation to support OD-SSB for CD-SSB is located on sync-raster. If supported, then proper restriction should be studied to minimize the impact on the UE initial cell search. |
| [12] CATT | Observation 3: The motivation of supporting OD-SSB for CD-SSB located on sync-raster is not clear. Proposal 2: Deprioritize the discussion of additional support of OD-SSB for CD-SSB located on sync-raster in Rel-19. Proposal 21: There is no need to introduce additional conditions for the following Alt 1 in the agreement in RAN1#120: - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB Proposal 22: RAN1 should clarify the meaning of the note: UE is not required to measure both AO-SSB and OD-SSB in the agreement in RAN1#120. - Possible meaning Alt-1: the UE is not required to measure both AO-SSB and OD-SSB after UE received RRC or MAC CE based signaling indicating OD-SSB transmission. - Possible meaning Alt-2: the UE is not required to measure both AO-SSB and OD-SSB during the actual transmission of OD-SSB. Proposal 23: The following Alt A in the agreement in RAN1#119 should be supported. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. |
| [13] CMCC | Proposal 4: The case where always-on SSB is CD-SSB and not on a synchronization raster is not supported. |
| [23] ETRI | Proposal 1: Regarding whether on-demand SSB is cell-defining or not, additional support for configuring the on-demand SSB as a CD-SSB located on the synchronization raster is not necessary. Proposal 11: It is proposed to reflect the configuration limitation (i.e., whether on-demand SSB can be configured on the same frequency location as always-on SSB depending on CD-SSB status) in TS 38.331 rather than TS 38.213. |
| [24] Transsion | Proposal 1 On-demand SSB for CD-SSB located on synchronization raster cannot be supported. Proposal 2 It is recommended to support Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster. |
| [25] OPPO | Proposal 5: Send an LS to RAN2 to request it to capture the following restrictions in TS 38.331, and remove the following draft CR from TS 28.213. When a first SS/PBCH block in a configured DL BWP can be used to obtain SIB1 and a frequency location of the first SS/PBCH block, provided by absoluteFrequencySSB, corresponds to the GSCN of a synchronization raster entry, the UE expects: - a frequency location of a second SS/PBCH block, provided by od-absoluteFrequencySSB, to be different from the frequency location of the first SS/PBCH block and not to correspond to the GSCN of a synchronization raster entry - frequency resources of the second SS/PBCH block not to overlap with frequency resources of the first SS/PBCH block - the second SS/PBCH block to be within the configured DL BWP as the first SS/PBCH block When a first SS/PBCH in a configured DL BWP cannot be used to obtain SIB1, the UE expects - a same frequency location for a second SS/PBCH block, provided by od-ssb-config, and for the first SS/PBCH block, provided by absoluteFrequencySSB - a same PBCH payload, other than the SFN index and the half frame index, for the first SS/PBCH block and for the second SS/PBCH block with same SS/PBCH block index as the first SS/PBCH block. |
| [27] LG Electronics | Proposal #11: Adopt Alt A. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. |
| [29] Apple | Observation 1-1: CD-SSB on sync-raster should be precluded as OD-SSB SCell operation to avoid impact on initial cell selection / cell reselection for both legacy and Rel-19 UE. Observation 1-2: Towards implementation/test/spec simplicity, minimizing combo of AO-SSB and OD-SSB is highly desirable unless more combos bring obvious benefits. Precluding CD-SSB on sync raster for OD-SSB can help to reduce unnecessary combos significantly. Proposal 1: CD-SSB on sync-raster is not supported for OD-SSB. |
| [30] Qualcomm | Observation 1: Having on-demand SSB configured as cell-defining SSB has negative impact to both legacy idle/inactive UEs and R19 idle/inactive UEs. Proposal 1: On-demand SSB for cell defining SSB located on synchronization raster is not supported Proposal 5: The case where always-on SSB is CD-SSB that is not on sync raster (Alt. C) is not supported. |
| [31] NTT DOCOMO | Proposal 1: Support OD-SSB for CD-SSB located on sync-raster. Proposal 4: When AO-SSB is CD-SSB, - No additional condition should be added to the Alt. 1 - We support Alt. A for the case when - OD-SSB and AO-SSB are located on the same frequency. - OD-SSB and AO-SSB are not located on the same frequency with the restriction that UE does not require to do RF tuning. Observation 1: It seems unclear which combinations b/w AO and OD-SSB are supported when AO-SSB is NCD-SSB in case #2. Proposal 5: Support the following combinations when AO-SSB is NCD-SSB in case #2. Table. 1 combination for the case when AO-SSB is NCD-SSB AO-SSB; OD-SSB; Support?. NCD-SSB located on a sync raster; CD-SSB located on a sync raster; NO. NCD-SSB located on a sync raster; CD-SSB located not on a sync raster; YES. NCD-SSB located on a sync raster; NCD-SSB located on a sync raster; YES. NCD-SSB located on a sync raster; NCD-SSB located not on a sync raster; YES. NCD-SSB located not on a sync raster; CD-SSB located on a sync raster; Not agreed. NCD-SSB located not on a sync raster; CD-SSB located not on a sync raster; YES. NCD-SSB located not on a sync raster; NCD-SSB located on a sync raster; YES. NCD-SSB located not on a sync raster; NCD-SSB located not on a sync raster; YES. Same restriction that was already agreed for the case of Alt. 1 should be applied to the supported combinations. |

Agreement (RAN1#116bis) - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster Agreement (RAN1#119) Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Agreement (RAN1#119) Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster. Agreement (RAN1#120) Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

## [Moderator’s note] Company views on remaining issues for frequency domain relation:

- Alt A (for always-on CD-SSB not on sync raster, same or different frequencies for always-on SSB and on-demand SSB)

  - Supported by Spreadtrum, CATT, LG Electronics, NTT DOCOMO

- Alt B (for always-on CD-SSB on sync raster, same frequency for always-on SSB and on-demand SSB)

  - Supported by Futurewei

- Alt C (Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster)

  - Supported by vivo, Nokia?, CMCC, Transsion, Qualcomm

- If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB

  - vivo: Define minimum frequency gap between AO-SSB and OD-SSB during same BWP

  - Further clarify that the restriction ‘AO-SSB and OD-SSB are located in the same BWP’ means that

    - vivo: located in the same BWP with the smallest bandwidth

    - Ofinno: Active BWP

  - NTT DOCOMO: No additional conditions

- CATT: RAN1 should clarify the meaning of the note: UE is not required to measure both AO-SSB and OD-SSB in the agreement in RAN1#120.

  - Possible meaning Alt-1: the UE is not required to measure both AO-SSB and OD-SSB after UE received RRC or MAC CE based signaling indicating OD-SSB transmission.

  - Possible meaning Alt-2: the UE is not required to measure both AO-SSB and OD-SSB during the actual transmission of OD-SSB.

## [Moderator’s note] Regarding FFS point for additional support of OD-SSB for CD-SSB located on sync-raster, company views are summarized as follows.

- Deprioritize: CATT

- Supported by

  - Nokia, NEC, Xiaomi, NTT DOCOMO

- Objected by

  - Ofinno, ETRI, Transsion, Apple, Qualcomm

- ETRI, OPPO: RAN2 to reflect the configuration limitation (i.e., whether on-demand SSB can be configured on the same frequency location as always-on SSB depending on CD-SSB status) in TS 38.331 rather than TS 38.213.

### [Closed] Proposal #6-3-1 (Frequency):

- For a cell supporting on-demand SSB SCell operation,

  - The following cases are NOT supported in Rel-19.

    - AO-SSB is CD-SSB and not on a synchronization raster.

    - OD-SSB is CD-SSB and on sync-raster.

    - When AO-SSB is not CD-SSB, the center frequency of AO-SSB is different from the center frequency of OD-SSB.

  - When the center frequency locations of always-on SSB and on-demand SSB are different, OD-SSB is not CD-SSB.

Companies are encouraged to provide views on Proposal #6-3-1.

| Company | Views |
| --- | --- |
| Moderator | Here is the summary of what we agreed so far and what is being proposed by Proposal #6-3-1. AO-SSB; OD-SSB; Support?. CD-SSB located on a sync raster; CD-SSB or NCD-SSB located on a sync raster; NO (Agreed). CD-SSB located on a sync raster; CD-SSB located not on a sync raster; NO (Proposed). CD-SSB located on a sync raster; NCD-SSB located not on a sync raster; YES (Proposed). CD-SSB located not on a sync raster; NO (Proposed). NCD-SSB located on a sync raster; CD-SSB located on a sync raster; NO (Agreed). NCD-SSB located on a sync raster; CD-SSB located not on a sync raster; NO (Proposed). NCD-SSB located on a sync raster; NCD-SSB located on a sync raster; YES (Agreed). NCD-SSB located on a sync raster; NCD-SSB located not on a sync raster; NO (Proposed). NCD-SSB located not on a sync raster; CD-SSB located on a sync raster; NO (Proposed). NCD-SSB located not on a sync raster; CD-SSB located not on a sync raster; NO (Agreed). NCD-SSB located not on a sync raster; NCD-SSB located on a sync raster; NO (Proposed). NCD-SSB located not on a sync raster; NCD-SSB located not on a sync raster; YES (Agreed). |
| Samsung | OK with the proposal. |
| Google | OK |
| InterDigital | Fine |
| LGE | Fine |
| Futurewei | OK |
| Xiaomi | For the third sub-sub-bullet, we are not sure why this restriction is needed. For the second sub-bullet, it seems it is not needed as the proposal already mentions OD-SSB cannot be CD-SSB. |
| Fujitsu | We can accept the proposal for the sake of progress. |
| Vivo | OK |
| ETRI | OK |
| CMCC | Ok. |
| Apple | We suggest to have the same type of SSB between AO-SSB and OD-SSB (i.e. {CD-SSB – CD-SSB}, or {NCD-SSB – NCD-SSB} for {AO-SSB – OD-SSB}. We would like to ask why the following proposals are made. - When AO-SSB is not CD-SSB, the center frequency of AO-SSB is different from the center frequency of OD-SSB. - When the center frequency locations of always-on SSB and on-demand SSB are different, OD-SSB is not CD-SSB. |
| Nokia, NSB | We think that it should be possible to configure OD-SSB as CD-SSB located on sync raster. |
| TCL | Support |
| DCM | We would like to clarify that NCD-SSB in this context just means a SSB which is not associated with RMSI (I.E., Not CD-SSB defined in TS38.300). If the above understanding is aligned with companies, we can accept this proposal. |
| CATT | Agree with Nokia that that case should be supported. |
| Ofinno | Support |
| Moderator | The following agreement was made so this issue can be closed. Agreement For a cell supporting on-demand SSB SCell operation, - The following cases are NOT supported in Rel-19. - AO-SSB is CD-SSB and not on a synchronization raster. - OD-SSB is CD-SSB and on sync-raster. - When AO-SSB is not CD-SSB, the center frequency of AO-SSB is different from the center frequency of OD-SSB. - When the center frequency locations of always-on SSB and on-demand SSB are different, OD-SSB is not CD-SSB. |

### [MID] Proposal #6-3-2 (331 spec):

Send an LS to RAN2 to request capturing the following restrictions and restrictions in Proposal #6-3-1 (if agreed) in TS 38.331, and remove the following draft CR from TS 28.213.

When a first SS/PBCH block in a configured DL BWP can be used to obtain SIB1 and a frequency location of the first SS/PBCH block, provided by absoluteFrequencySSB, corresponds to the GSCN of a synchronization raster entry, the UE expects: - a frequency location of a second SS/PBCH block, provided by od-absoluteFrequencySSB, to be different from the frequency location of the first SS/PBCH block and not to correspond to the GSCN of a synchronization raster entry - frequency resources of the second SS/PBCH block not to overlap with frequency resources of the first SS/PBCH block - the second SS/PBCH block to be within the configured DL BWP as the first SS/PBCH block When a first SS/PBCH in a configured DL BWP cannot be used to obtain SIB1, the UE expects - a same frequency location for a second SS/PBCH block, provided by od-ssb-config, and for the first SS/PBCH block, provided by absoluteFrequencySSB - a same PBCH payload, other than the SFN index and the half frame index, for the first SS/PBCH block and for the second SS/PBCH block with same SS/PBCH block index as the first SS/PBCH block

Companies are encouraged to provide views on Proposal #6-3-2.

| Company | Views |
| --- | --- |
| Samsung | OK other than the second bullet in the second paragraph. PBCH payload also includes PHY bits, which should not be captured in RAN2 spec. |
| Google | OK. It can be included in the RRC parameter list |
| LGE | OK |
| Futurewei | OK |
| ETRI | OK |
| CMCC | Ok. |
| Ofinno | OK |

## Spatial domain relation of OD-SSB

| Company | Views |
| --- | --- |
| [2] Huawei | Observation 1: The presence of od-ssb-PositionsInBurst in the RRC configuration indicates that the SSB indices within OD-SSB burst are independently/separately configured from the SSB indices within AO-SSB burst. Proposal 6: When a signal/channel is configured to be QCLed with an SSB index that is only present in OD-SSB, the signal/channel is QCLed with OD-SSB only. - UE expects that the QCLed RS can be transmitted after OD-SSB is applied. |
| [3] ZTE | Proposal 1: Support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst when the center frequency locations of always-on SSB and on-demand SSB are the same. |
| [5] Nokia | Proposal 2: In Case#2, SSB indices of on-demand SSB burst are subset of SSB indices of always-on SSB burst. |
| [6] NEC | Proposal 5: The UE may assume that the set of SSB indices configured within od-ssb-PositionsInBurst for on-demand SSB is a subset of, or identical to, the set of SSB indices configured within ssb-PositionsInBurst for always-on SSB on the same cell. |
| [9] Samsung | Proposal 5: For the case when the center frequency locations of always-on SSB and on-demand SSB are different: - On-demand SSB is a NCD-SSB; - Transmitted SSB indexes for on-demand SSB are same as or a subset of the ones for always-on SSB. |
| [12] CATT | Proposal 24: Regarding the spatial relation between the always-on SSB and OD-SSB, it is up to the gNB implementation whether SSB indices within the on-demand SSB burst are a subset of or not a subset of the SSB indices within always-on SSB burst. |
| [14] Xiaomi | Proposal 7: SSB indices within on-demand SSB burst can be subset or same as that set of SSB indices within always-on SSB burst, which is subject to its configuration. |
| [16] Ericsson | Proposal 6 OD-SSB positions in burst can be the same or a subset of AO-SSB positions in burst both for the case of same center frequency and different center frequencies. |
| [18] KT | Proposal 4: Regarding the spatial relation between the always-on SSB and on-demand SSB, Alt 1 is supported, i.e., it is supported that SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst. |
| [23] ETRI | Proposal 9: It is proposed not to support the case where the SSB indices within on-demand SSB burst are a subset of those within always-on SSB burst. Proposal 10: It is proposed not to support the case where the SSB indices within on-demand SSB burst can be separately configured from SSB indices within always-on SSB burst. |
| [27] LG Electronics | Proposal #6: For Case #1, following is supported for QCL between the first OD-SSB and the second OD-SSB. - SS/PBCH blocks with the same SSB indexes for the first OD-SSB and the second OD-SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. |
| [31] NTT DOCOMO | Proposal 6: For the applicable SSB-index arrangements for OD-SSB in case #2, we support at least following options. - Option 1: The set of SSB indices of AO-SSB and those of OD-SSB are identical. - Option 2: The set of SSB indices of OD-SSB becomes a subset of the set of SSB indices of AO-SSB. Proposal 7: For the spatial relationship between AO-SSB and OD-SSB, - When AO-SSB and OD-SSB have the same center frequency, support the QCL assumption for the same SSB-index regardless of whether the SSB indices within OD-SSB burst are identical to those within AO-SSB or not. - When AO-SSB and OD-SSB do not have the same center frequency, QCL assumption should not always be assumed. - Study whether/how to use OD-SSB as QCL source RS on behalf of/in addition to AO-SSB in case #1 and case #2. |
| [35] CEWiT | Observation 1: SSB indices within on-demand SSB equal to SSB indices within always-on SSB burst cause unnecessary SSB transmissions in areas with non uniform UE density. Observation 2: If both AO-SSB and on-demand SSB are configured for L1 measurement, the UE generates a single tracking process to determine SSB locations based on legacy starting symbol indices. Proposal 1: Support “Alt 1: It is supported that SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst” for spatial relation between the always-on SSB and on-demand SSB. |

Agreement (RAN1#119) Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst. Agreement (RAN1#120bis) For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications Agreement (RAN1#120bis) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

## [Moderator’s note] Company views on spatial domain relationship of OD-SSB:

- For the same center frequency case, support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst

  - YES: ZTE, NEC, Ericsson, KT, OPPO, CEWiT

  - NO: ETRI

- Subset relation for different center frequency case

  - YES: Nokia, NEC, Samsung, Xiaomi, Ericsson, KT, OPPO, NTT DOCOMO, CEWiT

  - NO: Huawei

  - Up to gNB implementation: CATT

- Huawei: When a signal/channel is configured to be QCLed with an SSB index that is only present in OD-SSB, the signal/channel is QCLed with OD-SSB only.

- LG Electronics: QCL between OD-SSB indexes

### [Closed] Proposal #6-4-1 (QCL for different frequency locations):

- For the case when the center frequency locations of AO-SSB and OD-SSB are different,

  - It is ONLY supported that SSB indices within OD-SSB burst is subset of SSB indices within AO-SSB burst or equal to SSB indices within AO-SSB burst.

Companies are encouraged to provide views on Proposal #6-4-1.

| Company | Views |
| --- | --- |
| Samsung | Support |
| Google | We failed to see the necessity |
| InterDigital | Support |
| ZTE, Sanechips | Fine with FL’s proposal. |
| LGE | Support it |
| Xiaomi | Support. |
| Fujitsu | Support |
| vivo | support |
| ETRI | Support |
| NEC | Support |
| CMCC | Support |
| Nokia, NSB | Support |
| TCL | Agree with this proposal |
| DCM | We are fine with the proposal. We are also fine that subset and identical cases can be applied for the same center frequency. We also support that QCL assumption cannot always be assumed, so it should be configurable. Especially for the case when the AO-SSB and OD-SSB use different beams, QCL between AO-SSB and OD-SSB should not be assumed. |
| CATT | OK |
| Ofinno | Support |
| Moderator | As discussed in online session, this issue is merged into Proposal #3-1-2a. |

### [Closed] Proposal #6-4-2 (QCL for Case #1):

For a cell supporting on-demand SSB SCell operation and for Case #1 (i.e., No always-on SSB on the cell), the following is supported for QCL between different OD-SSB configurations

- SS/PBCH blocks with the same SSB indexes for different OD-SSB configurations are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of different OD-SSB configurations with the same QCL parameters according to existing specifications.

Companies are encouraged to provide views on Proposal #6-4-2.

| Company | Views |
| --- | --- |
| Samsung | We understand the intention and support the proposal, but wonder whether there will be additional spec impact of the proposal. |
| Google | We failed to see the necessity |
| ZTE, Sanechips | Fine with FL’s proposal. |
| LGE | The QCL between AO-SSB and OD-SSB is specified last meeting, even if SSB positions are changed by every OD-SSB indication, UE can assume that the same index of AO-SSB and OD-SSB is QCLed, And, for the case that There are only OD-SSB in a Cell, As the Cell has no AO-SSB, when the SSB position is changed on the OD-SSB already transmitted by OD-SSB indication, we are wondering whether the same SSB indexes of two different OD-SSB configurations before and after the SSB position is changed, have the same beam characteristic or not. Without the same assumption for Case 2, UE measurement/reporting on each OD-SSB index can be complicated and require additional considerations/handling, whenever the SSB position is changed by OD-SSB indication. So, we think that the assumption of QCL between AO-SSB and OD-SSB should be similarly considered between different OD-SSB configurations for Case 1 and UE can assume that the same SSB index of multiple different OD-SSB configurations is QCLed. So, the UE measures each OD-SSB index with the assumption that the same SSB index of different OD-SSB configurations has the same beam characteristic, although the SSB position is changed by OD-SSB indication. The proposal should be captured to avoid additional UE effort/complexity for the case where the same SSB index of multiple different OD-SSB configurations is not QCLed. |
| Xiaomi | Support. |
| Fujitsu | Support |
| vivo | support |
| ETRI | Support |
| NEC | Support |
| CMCC | Support |
| Nokia, NSB | Support |
| TCL | Support |
| DCM | We are fine with the proposal. |
| CATT | OK |
| Ofinno | Support |
| Moderator | The following agreement was made and follow-up discussion is needed to resolve FFS point. Agreement For a cell supporting on-demand SSB SCell operation, the following is supported for QCL between different OD-SSB configurations - SS/PBCH blocks with the same SSB indexes according to OD-SSB configuration and indication are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index according to OD-SSB configuration and indication with the same QCL parameters according to existing specifications. - FFS: Case where there is different SSB indexes before and after OD-SSB transmission adaptation |

### [HIGH] Proposal #6-4-2a (QCL for different SSB index):

For a cell supporting on-demand SSB SCell operation,

- UE shall not assume quasi co-location between an SSB index before OD-SSB adaptation and different SSB index after OD-SSB adaptation.

Companies are encouraged to provide views on Proposal #6-4-2a.

| Company | Views |
| --- | --- |
| Moderator | To address FFS point (i.e., Case where there is different SSB indexes before and after OD-SSB transmission adaptation), this proposal is suggested. |

# Remaining issues for scenarios

| Company | Views |
| --- | --- |
| [2] Huawei | Proposal 7: RAN1 shall clarify whether an SSB-less SCell is interpreted as Case#1 or Case#2. |
| [12] CATT | Observation 1: Since the new MAC CE for OD-SSB transmission indication and the legacy MAC CE for SCell activation/deactivation are two independent MAC CEs, the gNB can trigger OD-SSB when needed via the new MAC CE, irrespective of the timing relationship with SCell activation/deactivation. Proposal 1: It is up to gNB implementation when OD-SSB is triggered, irrespective of the timing relationship with SCell activation/deactivation. Observation 2: In order to support scenario 2A, the new MAC CE for OD-SSB transmission indication and the legacy MAC CE for SCell activation/deactivation can be sent together in one PDSCH. |
| [29] Apple | Observation 14: The conclusion to preclude Scenario #3A should be agreement instead of conclusion. Proposal 14: The following conclusion made in RAN1 #120 is escalated to agreement. - ConclusionAgreement - The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 - Proposed wording: UE shall not assume OD-SSB indication is received after receiving SCell activation command and before SCell activation is completed. - Above does not impact discussion on SSB periodicity adaptation in time domain |

Agreement (RAN1#116) For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement (RAN1#116bis) For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Conclusion (RAN1#120) The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain Agreement (RAN1#120bis) For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

## [Moderator’s note] Company views for scenarios/cases are as follows.

- Huawei: RAN1 shall clarify whether an SSB-less SCell is interpreted as Case#1 or Case#2.

- Apple: UE shall not assume OD-SSB indication is received after receiving SCell activation command and before SCell activation is completed.

### [MID] Proposal #7-1 (Scenario):

- Huawei’s proposal: RAN1 shall clarify whether an SSB-less SCell is interpreted as Case#1 or Case#2.

- Apple’s proposal: UE shall not assume OD-SSB indication is received after receiving SCell activation command and before SCell activation is completed.

Companies are encouraged to provide views on above two proposals.

| Company | Views |
| --- | --- |
| LGE | Apple’s proposal can be discussed. |
| NEC | We think it is good to discuss (at least for Case#1) how SSB-less SCell is interpreted in presence of on-demand SSB and if the cell is considered as SSB-less SCell then how to rephase the specification to allow UE synchronization based on OD-SSB on the SCell. |
| Nokia, NSB | Regarding SSB-less SCell our view is that RAN1 should revisit the agreement from the previous meeting so that if Scell is SSB-less (reference cell configured) then it is possible to deactivate OD-SSB while SCell is activated. |

# Text proposals

[5] Nokia

Proposal 17: Consider the text proposal to improve the RAN1 requirements in Chapter 4.4 of TS38.213 - Reason for changes Current text is not aligned with the RAN2 agreements on MAC-CE design. As per RAN2 agreement, only the field of configuration-index is provided by the MAC-CE. The transmission parameters (i.e. od-ssb-nrOfBursts, od-ssb-periodicity and od-ssb-positionsInBursts, etc) of an OD-SSB signal is provided to UE that is associated with a configuration-index. There can be a list of configuration indexes corresponding to configurations of different OD-SSB signal transmissions. - Summary of the changes A od-ssb-config is associated with a configuration-index. The transmission parameters (i.e. od-ssb-nrOfBursts, od-ssb-periodicity and od-ssb-positionsInBursts, etc) of an OD-SSB signal is provided to UE via od-ssb-config that is associated with a configuration index. There can be multiple od-ssb-config configurations corresponding to different OD-SSB signal transmissions that is associated with different configuration index. The configuration-index can be indicated to the UE by the higher layer parameter or by MAC-CE for activation/deactivation and adaptation of OD-SSB signal transmission. Consequence if not approved RAN1 and RAN2 spec are not in alignment, leads to ambiguity for both UE and network implementation. ------------------------------------------------- Start of the TP for TS38.213---------------------------------------- A UE can be configured with [one or more] [od-ssb-config] [12, TS 38.331] for transmission of SS/PBCH blocks in a configured DL BWP of an SCell and a [od-ssb-config] is associated with a [configuration-index]. A UE can be indicated, by [od-ssb-config] [12, TS 38.331] by a higher layer parameter or by a [first] MAC CE [11, TS 38.321], activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300]. The UE can also be indicated, by a [configuration-index] provided via [third] MAC CE activation of transmission for SS/PBCH blocks or adaptation of parameters for the activated transmission of SS/PBCH blocks when the SCell is activated. A number of half frames with transmission of SS/PBCH blocks is indicated by the [first or third] MAC CE from values provided by od-ssb-nrofBurst, if provided; otherwise, the transmission of the SS/PBCH blocks occurs until it is deactivated by a [second] MAC CE [11, TS 38.321], The parameters for transmission of SS/PBCH blocks are provided within od-ssb-config where - the physical cell identity of the SS/PBCH blocks is indicated by od-ssb-physCellId, if provided; otherwise, by ssb-physCellId in ServingCellConfigCommon - the indexes of transmitted SS/PBCH blocks are indicated by the [first or the third] MAC CE from candidate values provided by od-ssb-PositionsInBurst, if provided; otherwise, by ssb-PositionsInBurst - the frequency location of the SS/PBCH blocks is indicated by od-absoluteFrequencySSB, if provided; otherwise, by absoluteFrequencySSB - the SCS configuration of the SS/PBCH blocks is indicated by od-ssbSubcarrierSpacing, if provided; otherwise, by ssbSubcarrierSpacing - the power of the SS/PBCH blocks is indicated by od-ss-PBCH-BlockPower, if provided; otherwise, by ss-PBCH-BlockPower - the periodicity of the transmission of the SS/PBCH blocks are indicated by the [first or third] MAC CE from candidate values by od-ssb-Periodicity - the half frames for the transmission of the SS/PBCH blocks are determined based on an indication by the [first or third] MAC CE - the transmission of the SS/PBCH blocks is in frames with SFN determined from SFN+SFN_offset⋅10 mod P=0, where P is the periodicity for the transmission of the SS/PBCH blocks, and SFN_offset is the indicated SFN offset by the [first or third] MAC CE from candidate values by od-ssb-sfn-Offset, if provided; else, SFNoffset=0. An index of a half frame with transmission of the SS/PBCH blocks in a corresponding frame is indicated by the [first or third] MAC CE from candidate values by od-ssb- halfFrameIndex, if provided; else the index is 0 When the activation or adaptation of the SS/PBCH blocks transmission is by the [first or the third] MAC CE, respectively, [and with reference to slots of a configured DL BWP for the SS/PBCH blocks transmission,] the UE expects that the transmission of the SS/PBCH blocks according to the indicated parameters starts from a first slot including the candidate SS/PBCH block occasion corresponding to the first transmitted SS/PBCH block index and located in a first half frame within the half frames for the transmissions of the SS/PBCH blocks, that is at least m+3 Nslotsubframe,μ+1 slots after slot n, where n is a slot when a PDSCH reception providing the [first or the third] MAC CE ends, respectively, n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211]. When the activation of transmission for the SS/PBCH blocks is by od-ssb-config, the UE expects that the transmission of the SS/PBCH blocks [starts at the next half frame with transmission of the SS/PBCH blocks]. When the deactivation of the SS/PBCH blocks transmission is indicated by the [second] MAC CE, [and with reference to slots of the configured DL BWP for the SS/PBCH blocks transmission,] the UE expects that the transmission of the SS/PBCH blocks according to the indicated parameters terminates from - a slot n+T, if the slot n+T is not within a first slot to a last slot with activated transmission of SS/PBCH blocks in a half frame, where T=m+3 Nslotsubframe,μ+1, n is a slot when a PDSCH reception providing the [second] MAC CE ends, n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211] - the first slot including the candidate SS/PBCH block corresponding to the last transmitted SS/PBCH block index that is not earlier than the slot n+T, if the slot n+T is within a first slot to a last slot with activated transmission of SS/PBCH blocks in a half frame --------------------------------------------------- End of the TP for TS38.213---------------------------------------

[22] Lenovo

Proposal 1: Adopt the following TP for TS38.213.

<Unrelated part omitted> A UE can be indicated, by [od-ssb-config] [12, TS 38.331] or by a [first] MAC CE [11, TS 38.321], activation of transmission for SS/PBCH blocks in a configured DL BWP of an SCell [19, TS 38.300]. The UE can be indicated, by a [third] MAC CE, adaptation of parameters for the activated transmission of SS/PBCH blocks when the SCell is activated. <Unrelated part omitted> When the UE is not provided absoluteFrequencySSB for the SCell, the UE does not expect the transmission of the SS/PBCH blocks activated by od-ssb-config or a [first/third] MAC CE to be deactivated while the SCell is activated. <Unrelated part omitted>

## [Moderator’s note] Companies can provide views on above two TPs, if any.

| Company | Views |
| --- | --- |

# Proposals for Wednesday online session

### [HIGH] Proposal #2-5a (LTM):

Conclusion

For a cell supporting on-demand SSB SCell operation,

- LTM based on OD-SSB is NOT supported in Rel-19.

- L1 measurement based on OD-SSB for a serving cell in Scenario #2 is NOT supported in Rel-19.

  - Note: As per previous RAN1 agreement, Scenario #2 is when SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321).

- L1 measurement based on OD-SSB for a non-serving/neighbor cell is NOT supported in Rel-19.

### [HIGH] Proposal #3-2 (PDCCH):

- For a cell supporting on-demand SSB SCell operation,

  - If a PDCCH candidate and a currently being transmitted OD-SSB are overlapped in at least one RE, the UE is not required to monitor the PDCCH candidate.

- Adopt the following TP to Clause 10 of TS 38.213 for PDCCH monitoring.

10 UE procedure for receiving control information <omitted text> For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE on a secondary cell, if the UE - has received [od-ssb-PositionsInBurst] in [od-ssb-Config] for the secondary cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by the latest [od-ssb-PositionsInBurst] in [od-ssb-Config] before deactivating on-demand SSB transmission and associated with a SS/PBCH block that is transmitted according to an on-demand SSB transmission operation in the cell, the UE is not required to monitor the PDCCH candidate. A UE is not required to monitor PDCCH candidates for a Type0/0A/0B/1/1A /2/2A -PDCCH CSS set when the active TCI state for a corresponding CORESET is not associated with physCellId in ServingCellConfigCommon. If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. <omitted text>

### [HIGH] Proposal #3-3 (PDSCH):

- For a cell supporting on-demand SSB SCell operation,

  - The UE shall assume that the PRBs containing currently being transmitted OD-SSB resources are not available for PDSCH in the OFDM symbols where OD-SSB is currently being transmitted on the cell.

- Adopt the following TP to Clause 5.1.4 of TS 38.214 for PDSCH resource mapping.

### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block, after puncturing if applicable, is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. For a UE supporting on-demand SSB Scell operation, the UE assumes SS/PBCH block transmission according to od-ssb-PositionsInBurst according to the latest an on-demand SSB transmission operation before deactivating on-demand SSB transmission if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. <omitted text>

### [HIGH] Proposal #3-1-1b (Case #1 RO):

- For a cell supporting on-demand SSB SCell operation and for Case #1 (i.e., No always-on SSB on the cell),

  - For RO validation before SSB-to-RO mapping,

    - Alt 2

      - RO validation rule is determined without considering OD-SSB consideration(s).

    - Alt 3

      - RO validation rule is determined based on all configured OD-SSB configuration(s) in RRC.

  - For SSB-to-RO mapping,

    - SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst in RRC.

### [HIGH] Proposal #3-1-2b (Case #2 RO):

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

  - Alt 1A)

    - For the case when the center frequency locations of AO-SSB and OD-SSB are different,

      - It is ONLY supported that SSB indices within OD-SSB burst is subset of SSB indices within AO-SSB burst or equal to SSB indices within AO-SSB burst.

    - For RO validation before SSB-to-RO mapping,

      - RO validation rule is determined based on AO-SSB, as in legacy.

    - For SSB-to-RO mapping,

      - SSB-to-RO mapping rule is determined based on AO-SSB, as in legacy.

  - Alt 1B)

    - For the case when the center frequency locations of AO-SSB and OD-SSB are different,

      - It is ONLY supported that SSB indices within OD-SSB burst is subset of SSB indices within AO-SSB burst or equal to SSB indices within AO-SSB burst.

    - For RO validation before SSB-to-RO mapping,

      - RO validation rule is determined based on AO-SSB and all configured OD-SSB configuration(s) in RRC.

    - For SSB-to-RO mapping,

      - SSB-to-RO mapping rule is determined based on AO-SSB, as in legacy.

  - Alt 2A)

    - For the case when the center frequency locations of AO-SSB and OD-SSB are different,

      - It is supported that at least one of SSB indices within OD-SSB burst can be different from any of SSB indices within AO-SSB burst.

    - For RO validation before SSB-to-RO mapping,

      - RO validation rule is determined based on AO-SSB and all configured OD-SSB configuration(s) in RRC.

    - For SSB-to-RO mapping,

      - SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst in RRC and SSB indices configured for AO-SSB.

  - Alt 2B)

    - For the case when the center frequency locations of AO-SSB and OD-SSB are different,

      - It is supported that at least one of SSB indices within OD-SSB burst can be different from any of SSB indices within AO-SSB burst.

    - For RO validation before SSB-to-RO mapping,

      - RO validation rule is determined based on AO-SSB, as in legacy

    - For SSB-to-RO mapping,

      - SS SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst in RRC and SSB indices configured for AO-SSB.

### [HIGH] Proposal #2-1b (CSI report configuration):

- For a cell supporting on-demand SSB SCell operation,

  - For Case #1 (i.e., No always-on SSB on the cell),

    - A CSI report configuration is associated with OD-SSB only,

      - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

    - For the case where AO-SSB and OD-SSB have the same center frequency,

      - A CSI report configuration is associated with both of AO-SSB and OD-SSB,

      - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

    - For the case where AO-SSB and OD-SSB have the different center frequencies,

      - Option 1’: A CSI report configuration can be associated with AO-SSB only or associated with OD-SSB only. NEW RRC parameter is introduced to differentiate CSI report configuration associated with AO-SSB only and CSI report configuration associated with OD-SSB only.

        - If a CSI report configuration is associated with OD-SSB only, UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

          - Note: UE is not expected to be configured with SSB-index that is not present in the currently transmitted OD-SSB

          - Huawei, ZTE, IDC, Ericsson, Nokia, FW, DOCOMO, Fujitsu,

        - If a CSI report configuration is associated with AO-SSB only, UE reports SSBRI based on SSB-index corresponding to AO-SSB.

      - Option 2: A CSI report configuration is associated with both of AO-SSB and OD-SSB.

        - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

          - Qualcomm, Xiaomi, Panasonic, Samsung, OPPO, China Telecom, Ofinno

    - This reverts previous RAN1 agreement.

  - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

### [HIGH] Proposal #6-4-2a (QCL for different SSB index):

For a cell supporting on-demand SSB SCell operation,

- UE shall not assume quasi co-location between an SSB index before OD-SSB adaptation and different SSB index after OD-SSB adaptation.

# Reference

- R1-2503233	Discussion of on-demand SSB Scell operation	FUTUREWEI

- R1-2503267	On-demand SSB SCell operation for eNES	Huawei, HiSilicon

- R1-2503315	Discussion on on-demond SSB for NES	ZTE Corporation, Sanechips

- R1-2503365	Remaining issues on-demand SSB Scell operation	vivo

- R1-2503412	On-demand SSB SCell Operation	Nokia, Nokia Shanghai Bell

- R1-2503416	Discussion on on-demand SSB for SCell operation	NEC

- R1-2503519	Discussion on on-demand SSB SCell operation	Spreadtrum, UNISOC

- R1-2503539	On-demand SSB Scell operation for NES	TCL

- R1-2503570	On-demand SSB SCell operation	Samsung

- R1-2503718	OD-SSB for SCELL operation	Tejas Network Limited

- R1-2503737	Discussion on on-demand SSB SCell operation	Ofinno

- R1-2503797	Discussion on on-demand SSB SCell operation	CATT

- R1-2503835	Discussion on on-demand SSB SCell operation	CMCC

- R1-2503886	Discussion on on-demand SSB SCell operation	Xiaomi

- R1-2503972	Discussion on on-demand SSB SCell operation	InterDigital, Inc.

- R1-2504015	On-demand SSB SCell operation	Ericsson

- R1-2504030	On-demand SSB SCell Operation	Google

- R1-2504037	Discussion on on-demand SSB SCell operation	KT Corp.

- R1-2504050	Discussion on on-demand SSB operation for SCell	China Telecom

- R1-2504075	Discussion on remaining details of on-demand SSB operation on SCell	Sharp

- R1-2504092	Discussion on on-demand SSB SCell operation	Fujitsu

- R1-2504107	On-demand SSB SCell operation	Lenovo

- R1-2504141	Discussion on On-demand SSB SCell operation	ETRI

- R1-2504176	Discussion on On-Demand SSB SCell operation	Transsion Holdings

- R1-2504192	Discussion on the enhancement to support on demand SSB SCell operation	OPPO

- R1-2504235	Discussion on on-demand SSB SCell operation	Panasonic

- R1-2504247	On-demand SSB SCell operation	LG Electronics

- R1-2504262	On-demand SSB SCell operation	MediaTek Inc.

- R1-2504325	On-demand SSB SCell Operation	Apple

- R1-2504398	On-demand SSB operation for Scell	Qualcomm Incorporated

- R1-2504505	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.

- R1-2504538	Discussion on on-demand SSB SCell operation	ITRI

- R1-2504554	Discussion on on-demand SSB SCell operation	KDDI Corporation

- R1-2504558	DCI based signaling for on-demand SSB	ASUSTeK

- R1-2504602	Discussion on on-demand SSB Scell operation	CEWiT

# Appendix: Previous agreements

## RAN1#116

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

## RAN1#116bis

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

## RAN1#117

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

- FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

## RAN1#118

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Agreement

LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - (Working assumption) T=T_min

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.

## RAN1#118bis

Agreement

For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

- Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

- Option 3: Configuration/indication of the duration of on-demand SSB transmission window

- Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

- Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

- Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

- Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

- FFS: Each option is applicable to which Cases or Scenarios

- FFS: Details related to each of the above options

Agreement

For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1.

- Sub-carrier spacing of the on-demand SSB

  - FFS if this can be absent

- Physical Cell ID of the on-demand SSB

- FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index

- Downlink transmit power of on-demand SSB

- FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

- FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB.

- Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB

- Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB

- FFS: Whether OD-SSB and always on SSB have same beam or not

Conclusion

No consensus on the support of on-demand SSB SCell operation triggered by UE.

Agreement

The previous RAN1 agreement is partly confirmed and further revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

      - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - (Working assumption) T=T_min

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

- Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

- Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

## RAN1#119

Agreement

Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1:

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Further update to be made based on RAN1#119 progress.

Agreement

Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1:

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Agreement

Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1:

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Agreement

- For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

  - For Case #1 (i.e., No always-on SSB on the cell),

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

    - Alt A: Same as for Case #1

    - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

Agreement

- New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Agreement

Down-select at least one of the following alternatives.

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

- Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster.

Down-select at least one of the following alternatives.

- Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration.

- Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Agreement

Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1:

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

## RAN1#120

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following.

- Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-C: The specification has no restriction with regards to overlapping

Agreement

- For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC,

  - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e.,

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

    - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

      - By defining reference point as SFN where AO-SSB burst exists

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Agreement

For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

Conclusion

The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19

- Scenario #3A and Case #1

- Scenario #3A and Case #2

Above does not impact discussion on SSB periodicity adaptation in time domain

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633.

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, when UE determines time instance A, the SCS to determine the value of T is down selected among the following options

- Option 1: the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication

- Option 2: the minimum of “the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication” and “the SCS of the active DL BWP where UE receives on-demand SSB”

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

## RAN1#120bis

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, adopt Option 3 to determine SCS for Nslotsubframe,μ for determining T

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),

- Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}

  - N2= [8]

  - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE.

  - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Agreement

Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

- Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

  - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

    - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - (Working assumption) T=T_min

  - RAN4 to confirm the above two working assumptions

  - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Agreement

LS to RAN4 on time instance B for on-demand SSB SCell operation is agreed. Final LS in R1-2503108.

Agreement

For a cell supporting on-demand SSB SCell operation,

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2

  - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB.

- od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap.

- od-ssb-PositionsInBurst can be absent for Case #2

  - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell)

- UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- A CSI report configuration can be associated with

  - both of AO-SSB and OD-SSB

    - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met

    - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst

    - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”.

  - OD-SSB only

    - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies