3GPP TSG-RAN WG1 Meeting #121	R1-2504875

St Julian's, Malta, 19 - 23 May, 2025

Agenda item :		9.11.5

Source:			Moderator (Qualcomm Incorporated)

Title:			Summary#1 of discussions on RRC parameters for Rel-19 IOT NTN TDD mode

Document for:		Discussion and decision

- Moderator input

Proposal 1: The following RRC parameters are agreed:

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | Required for initial access or IDLE/INACTIVE | Specification | Comment | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_TDD | - | TS 36.211 | 10.1.6.1 | nprach-Parameters-r14 and NPRACH-Parameters-NB-r13 | nprach-Periodicity | NPRACH resource periodicity | Existing | Periodicity of NPRACH | {ms90, ms180, ms160, ms240, ms320, ms640, ms1280,s2560} |  | Per cell | Yes | TS 36.331 | Agreement NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported NOTE: It is up to RAN2 how to implement this agreement (e.g. keeping the same codepoints as in legacy but with a NOTE that for NBIOT NTN TDD 40/80ms are interpreted as 90/180) | Stable |
| IoT_NTN_TDD | - | TS 36.213 | 16.6 | SC-MTCH-Info-NB-r14, NPDCCH-SC-MCCH-Config-NB-r14, NPDCCH-ConfigDedicated-NB-r13, NPRACH-Parameters-NB-r13, NPRACH-Parameters-NB-r14, NPRACH-ParametersFmt2-NB-r15 | npdcch-StartSF | G | Existing | Scaling factor to determine NPDCCH search space periodicity | {v1dot5, v2, v4*11.25, v8*11.25, v16, v32, v48, v64} |  | Per cell / per UE | Yes | TS 36.331 | Agreement Confirm the following WA with the following modification For NPDCCH monitoring, new periodicities are introduced for NPDCCH monitoring by supporting values of G={11.25*4, 11.25*8} instead of G={4,8} NOTE: It is up to RAN2 how to implement this agreement (e.g. keeping the same codepoints as in legacy but with a NOTE that for NBIOT NTN TDD 4/8 are interpreted as 4*11.25/8*11.25) NOTE: The new value of G should be applied to all defined search spaces (npdcch-StartSF-SC-MCCH-r14, npdcch-StartSF-USS-r13, npdcch-StartSF-CSS-RA-r13, npdcch-StartSF-CSS-RA-r14, npdcch-StartSF-CSS-RA-r15, | Stable |
| IoT_NTN_TDD | - | TS 36.211 | 10.2.3.4 | DL-GapConfig-NB-r13 | dl-GapPeriodicity-r13 | N_{gap,period} | Existing | Perodicity of DL gaps | {sf1024, sf128, sf256, sf512} |  | Per UE / per cell | Yes | TS 36.331 | Agreement - For the issue of handling DL gaps in NB-IoT NTN TDD mode: - DL gaps apply based on current specifications, with the following modification: - dl-GapPeriodicity with a value of “sf1024” is supported instead of the value of “sf64” NOTE: It is up to RAN2 how to implement this agreement (e.g. keeping the same codepoints as in legacy but with a NOTE that for NBIOT NTN TDD sf64 is interpreted as sf1024) | Stable |