3GPP TSG RAN WG1 #122	R1-2505145

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	11.5

Source:	Futurewei

Title:	Discussion on 6G energy efficiency techniques

Document for:	Discussion and decision

# Introduction

In RAN # 108 meeting (RP-251881) the Study on 6G Radio SID was agreed.

“The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914].

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. “

Among the 9 detailed objectives of this study the first objective provides a general framework:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

The energy efficiency item from the above objective is planned to be discussed in RAN1 #122 agenda item 11.5:

“Including discussion of proposal for NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving, targeting to categorize proposals by RAN1#123. From RAN1#124, proposals will be distributed to respective related agenda.”

In this contribution, we discuss energy saving solutions and their categorization.

# Background

In 5G NR several energy saving solutions were adopted. These solutions aim to decrease energy consumption at UE, gNB or both. While several studies show their efficiency, since they were introduced in later releases of 5G, backward compatibility with prior designs limited their potential gains. Furthermore, some of the solutions for UE power saving and Network energy saving were developed independently without thorough consideration of the mutual impact and the energy efficiency of the overall system; see Figure 1.

Figure 1: Timeline of UE Power Saving and Network Energy Saving Work in 3GPP NR

Based on this observation, prior discussions in RAN workshop [6GWS-250002], and the 5G Study document [RP-251881], we could identify the following design principles for improved energy efficiency in 6G:

Native Energy Efficiency: Energy efficiency should be a fundamental requirement for all 6G design aspects from Day 1, rather than an add-on feature. This includes considering energy efficiency during the design of the 6G carrier, antenna architectures, and AI/ML-based operations.

Joint Network and UE Power Saving: Proposals emphasize a unified and harmonized design for energy saving that benefits both the network and the UE simultaneously, including Idle, Inactive, and Connected RRC states. This aims to minimize unnecessary power consuming transmissions/receptions and synchronize sleep times at both the UE and gNB.

Simplified System Design: 6G should adopt a simpler design with minimized complexity and optionality to ease implementation, reduce costs, and inherently lower energy consumption.

Performance vs. Energy Trade-off: The design should allow for a balanced trade-off between energy saving gains and performance, ensuring that QoS/QoE is not compromised. If it is feasible for a feature to improve both energy efficiency and throughput/latency performance, such a feature should be prioritized.

Mandatory Feature Support: Energy saving features should be mandatory for all 6G devices and networks from the first release to ensure widespread adoption and impact.

KPI Definition: New KPIs should be defined to meaningfully measure energy consumption, especially on the network side.

Proposal 1: For 6G energy efficiency consider the following design principles:

- Native Energy Efficiency

- Joint Network and UE Power Saving

- Simplification of the Design

- Balanced Trade-off between Energy Saving and Performance

- Energy Efficiency as Mandatory Features

# A taxonomy of energy efficiency techniques

To achieve higher energy efficiency for wireless communication, the nodes involved (UE and gNB) should minimize transmissions and RF link monitoring as much as possible, while still satisfying a set of basic quality requirements. This translates into minimizing the active time and/or transmit or receive power of the transceivers. More specifically, energy saving solutions may be devised into these large categories:

- The time domain techniques enable component-level shutdown during inactive periods, with no transmission or reception by the UE and /or BS.

- The frequency domain techniques explore the adaptation of frequency resources in general, including the shutdown of secondary cells/carriers (SCell) in multi-carrier operation and adjustments in bandwidth segments based on BWP adaptation as well as limiting the maximum bandwidth (RedCap). Such techniques may include a sparser Sync Raster, and simpler carrier designs.

- The spatial domain techniques involve dynamically deactivating spatial elements, i.e., antenna ports/elements and/or TX/RX points (TRPs) in multi-TRP operation. Another approach was the introduction of low power wakeup signal (LP-WUS) of the higher power transceiver at UE.

- The power domain techniques delve into dynamic adaptation of downlink (DL) transmit power and energy-efficient DL transmission through approaches such as new antenna architectures.

The above mentioned categories for energy efficiency techniques can also be grouped differently (and used individually or jointly) according to association to gNB/UE transmission activities (periodic or aperiodic), monitoring/reception activities, carrier design (single/multi-carrier), and antenna architecture design as follows:

- Transmission duty cycle based solutions

  - Periodic transmission activities

Periodic transmissions may be generally needed for essential functionalities such as SSB delivery at initial access, RS delivery for RRM measurement, RS delivery for beam / CSI acquisition, etc. However, periodic transmissions lead to persistent energy consumption and can reduce energy efficiency, and hence, it is desirable to minimize periodic transmission activities while still fulfilling the essential functionalities.

    - Frequent periodic transmissions may be replaced with infrequent (i.e., long-periodicity) periodic transmissions plus on-demand transmissions.

For example, frequent periodic transmissions of SSB (e.g., with 5 ms periodicity) and SIB1 (e.g., with 20 ms periodicity) may be replaced with infrequent periodic transmissions of SSB and SIB1 (e.g., with 160 ms periodicity) plus on-demand SSB and on-demand SIB1, in which the on-demand SSB and on-demand SIB1 may be determined by the network via monitoring the traffic arrivals, UE UL wakeup signals (WUS), cell activation procedures, and so on. During the on-demand transmissions, the transmissions may be repeated, or the RS may have higher densities in time-frequency domains (higher than those for the associated periodic transmissions) to ensure satisfactory performance. Overall, since the infrequent periodic transmissions plus on-demand transmissions may be less often than the (original) frequent periodic transmissions, energy saving can be achieved.

    - Frequent periodic transmissions with high per-transmission overhead may be replaced with frequent periodic lightweight transmissions and/or on-demand / infrequent periodic transmissions.

For example, CSI-RS transmissions with 256 ports and 40 ms periodicity may be replaced with CSI-RS transmissions with selected 16 ports and 40 ms periodicity plus CSI-RS transmissions with all 256 ports and 160 ms periodicity. Alternatively, the CSI-RS with all 256 ports may be on-demand transmissions (potentially with repetitions and/or higher density) when high-speed data is to be sent. The reduced per-transmission overhead can help reduce energy consumption.

For another example, CSI-RS transmissions on a carrier of 400 MHz bandwidth and with 40 ms periodicity may be replaced with CSI-RS transmissions on 40 MHz bandwidth hopping within the carrier and 40 ms periodicity.

  - Other transmission activities

For other transmission activities, it is also desirable to examine their behaviors in time/frequency/spatial/power domain and apply one or more of the above techniques to reduce energy consumption.

- Monitoring/reception duty cycle based activities

Periodic/continuous monitoring, at either the network or the UE, may be generally needed for essential functionalities such as SSB acquisition at the UE, paging reception at the UE, receiving scheduling PDCCH at the UE, contention-based PRACH monitoring at the network, scheduling request monitoring at the network, etc. However, periodic/continuous monitoring leads to continuous energy consumption and can reduce energy efficiency, especially if the monitoring requires the main radio of the receiver to perform complicated operations such as blind detection, PDCCH decoding, etc. Hence, it is desirable to minimize/reduce/offload periodic/continuous monitoring activities while still fulfilling the essential functionalities.

    - Periodic/continuous monitoring via the main radio may be replaced by periodic/continuous monitoring via low-power radio.

LP-WUS has been introduced for this purpose, and the same concept may be applied/extended for more general situations. Advanced design of the sequences/preambles, such as multiplexed OOK sequence with OFDM data, and careful design of the sequence/preamble-based procedures, such as WUS procedures or early indication procedures, may be needed to improve the energy efficiency of both UE and network.

    - Periodic/continuous monitoring that requires complicated operations may be replaced by periodic/continuous monitoring that requires simpler operations, such as sequence detection, (WUS) preamble detection, etc.

Periodic/continuous PDCCH monitoring can cause high energy consumption at the UE due to the complicated baseband processing operations. On the other hand, sequence/preamble detection can be much simpler, though it may only be able to carry a very small amount of information.

- Carrier design based solutions

  - When the above techniques and general principles for energy-efficient transmissions/receptions/monitoring are applied to a carrier in carrier aggregation cases, minimum configurations of overhead such as always-on transmissions and cell-level transmissions can be considered. For example, on a secondary component carrier (SCC), the always-on transmissions of SCell SSB may be replaced/reduced to one lightweight synchronization sequence without PBCH transmission, since all the configuration information for the SCC may be delivered to the UE on another carrier. This may further lead to a design on a SCC without SCell configuration at all, further reducing energy consumption.

Moreover, on a component carrier, it may not be necessary to have all transmission/reception/monitoring activities on the entire carrier bandwidth (especially if the carrier bandwidth is very large) but restricted on a smaller bandwidth (such as on a narrower BWP) until heavy data load arrives. For the multiple BWPs on the same carrier, their energy efficiency may be improved if they share some common configurations/properties/parameters, such as their numerology, QCL properties, etc.

- Antennas architecture based solutions

  - Energy-efficient Extreme MIMO: Design Extreme MIMO with up to a few thousands of antenna ports for energy-efficient operation, including dynamically controlling the minimum number of active antenna ports based on service quality.

For example, operating a base station antenna with up to 1024 ports or even more all the time on all carriers leads to high energy consumption, overhead, and complexity that may be prohibitive in a practical network. It may be useful to have the capability of turning on/off some of the antenna elements, subarrays, (sub-)panels, and/or transceiver chains to save energy.

  - Hybrid Antenna Architecture: Use hybrid antenna architecture to reduce the number of RF chains and ADC/DAC converters, lowering energy consumption.

For instance, in FR3, though fully digital antenna architecture can have obvious advantages, high energy consumption due to a large number of TRx chains coupled with hundreds of megahertz of bandwidth and the associated baseband processing should be taken into consideration. For example, the total ADC/DAC power consumption scales roughly linearly with the product of the number of TRx chains and the bandwidth, so if from Massive MIMO on FR1 to Extreme MIMO on upper midband, the number of TRx chains grows by 4x to 8x, and the bandwidth grows by 2x to 8x, the total ADC/DAC power consumption grows by up to 8x to 64x. If, however, the number of TRx chains for upper midband remains the same as that for FR1, i.e. adopting a hybrid antenna architecture, the total ADC/DAC power consumption will be more acceptable in a practical network operating on upper midband.

Proposal 2: Consider the following general energy saving techniques:

- Common Signaling and Channels Optimization

  - Reduce/minimize the energy consumption due to periodic transmission activities via prolonged periodicities, on-demand transmissions, lightweight transmissions, etc.

  - Reduce/minimize the energy consumption due to periodic/continuous reception/monitoring activities via prolonged periodicities, lower-power radio based reception/monitoring, sequence/preamble detection based reception/monitoring, etc.

  - Reduce/minimize the energy consumption on a carrier by reducing/minimizing always-on and cell-level transmissions and cell-level/UE-level configurations.

- Carrier design based solutions

  - Reduce carrier-level energy consumption and overhead via, e.g., removing SCell configurations on a secondary component carrier, on-demand bandwidth adaptation within a carrier.

- Antennas architectures

  - Energy efficient Extreme MIMO based on subarray adaptation

  - Hybrid antennas for upper midband

# Baseline UE Energy Saving Solutions

Duty-cycled operations in the form of Discontinuous Reception (DRX) and extended Discontinuous Reception (eDRX) are crucial time domain techniques that have always been considered for power consumption reduction in RRC_IDLE and RRC_INACTIVE states through the reduction of the number of Paging Occasions (POs) monitored by the UE. Further power consumption reduction was achieved through Paging Early Indication (PEI) in Rel-17 NR RRC_IDLE and RRC_INACTIVE states, which was still subject to the duty-cycled operations. Similar power saving techniques were also considered for RRC_CONNECTED state in the form of connected mode DRX (C-DRX) and Rel-16 Wake-Up Signal/DCI with CRC scrambled by PS-RNTI (DCP). In this case when DRX is configured, the UE does not have to continuously monitor PDCCH. the UE may be indicated, when configured accordingly, whether it is required to monitor or not the PDCCH during the next occurrence of the on-duration by a DCP monitored on the active BWP (TS 38.300, Clause 11.1).

Both Rel-17 PEI and Rel-16 DCP were received by UEs as DCIs in the PDCCH.

The DRX, eDRX, and C-DRX could provide higher power saving gain by increasing the duty cycle duration at the expense of higher latency to be expected by the UE. On top of DRX/eDRX or C-DRX, Rel-17 PEI and Rel-16 DCP could provide more power saving gain without an impact on latency, but the gain was limited by the power consumption required to decode a DCI in the PDCCH.

In Rel-19, a Low Power Wake-Up Signal (LP-WUS) was introduced, which can be received by a Low Power Wake-Up Radio/Receiver (LP-WUR) at considerably lower power consumption than existing Rel-17 PEI and Rel-16 DCP designs. The evaluations of LP-WUS in its Rel-18 study showed that a significant power saving gain was achievable. The Rel-19 LP-WUS could therefore enable new trade-off regions of latency versus power using dedicated LP-WURs with simple architectures. The Rel-19 LP-WUS design addressed two classes of LP-WUR architectures: (i) an envelope detection (ED)-based LP-WUR employing an On-Off Keying (OOK) waveform, and (ii) an OFDM-based LP-WUR utilizing OFDM sequences overlaid on the OOK waveform.

Considering the simplicity of duty-cycled operations and the significant power-saving benefits of LP-WUS, existing DRX/eDRX, C-DRX, and LP-WUS techniques can be adopted as baseline mechanisms for 6G. It is noted that LP-WUS may lower the latency imposed by duty-cycled operation by considering the configuration of dynamic POs [6]. Furthermore, due to functional overlap with Rel-17 PEI and Rel-16 DCP, an enhanced LP-WUS mechanism could serve as a unified solution, replacing the Rel-17 PEI and Rel-16 DCP. This enhancement may include support for standalone OFDM-based LP-WUS (i.e., not reliant on the OOK waveform) and integration of dormancy indication functionality as defined in Rel-16 DCP.

Observation 1: Duty-cycled operations can continue to offer simple and reliable solutions for UE power saving while LP-WUS can offer significant power saving benefits and lower latency associated with duty-cycled operations.

Observation 2: There is a functional overlap of Rel-19 LP-WUS with Rel-17 PEI and Rel-16 DCP. An enhanced LP-WUS mechanism can serve as a unified energy efficiency solution, i.e., taking over the Rel-17 PEI and Rel-16 DCP functionalities.

The Rel-19 LP-WUS and Low Power Synchronization Signal (LP-SS) designs employing the OOK waveform are expected to achieve greater power saving gain compared to the OFDM-based LP-WUS design, owing to the simpler ED-based LP-WUR architecture. However, the Rel-19 OOK-based LP-WUS/LP-SS designs exhibit considerably lower resource utilization efficiency. Therefore, enhancements aimed at improving overall resource utilization, e.g., through integration with other signals or channels with efficient resource utilization, may be necessary.

Observation 3: Rel-19 OOK-based LP-WUS/LP-SS designs exhibit considerably low resource utilization efficiency. However, enhancements aimed at improving overall resource utilization, e.g., through integration with other resource efficient signals or channels, may be considered.

Relaxed RRM measurements for UEs based on low mobility and not-at-cell-edge criteria were introduced in NR Rel-16 with considerable power saving gain shown in TR38.840 whereas relaxed RRM measurements for (e)RedCap UEs based on stationarity and not-at-cell-edge criteria were introduced in NR Rel-17 and Rel-18. Additionally, further RRM measurement relaxation of intra-/inter-frequency neighboring cells as well as serving cell measurement relaxation and/or offloading to LP-WUR were highlighted as critical elements to fully benefit from the Rel-19 LP-WUS and minimize the main radio (MR)’s power consumption by keeping it in a sleep state as much as possible. Therefore, relaxed RRM measurements and offloading of at least serving cell measurements to a LP-WUR should, as critical elements for the significant UE power saving benefits of LP-WUS, also be adopted as baseline mechanisms in 6G.

Observation 4: RRM measurement relaxation for neighboring cells and relaxation/offloading to LP-WUR for serving cell based on stationarity, low mobility, and/or not-at-cell-edge criteria are critical elements for the significant UE power saving benefits in 6G.

Proposal 3: Adopt from day one 5G UE power saving techniques as baseline mechanisms in 6G, such as:

- Duty-cycled based operations (iDRX, eDRX, cDRX),

- LP-WUS with at least PEI and DCP functionality replacement,

- Relaxed RRM measurements of neighboring cells, and

- Relaxed/Offloading to LP-WUR of serving cell measurements.

# Baseline NW Energy Saving Solutions

Network Energy Saving (NES) solutions were considered in 5G NR but were standardized relatively late in 5G (Figure 1).

The basic principles of energy savings presented in Section 3 above were applied in 5G for gNB solutions as well. For instance, reduced periodic transmission of common signaling and control optimization techniques or replacing the periodic transmissions with on-demand transmissions was defined for SSB and SIB1 in Release 19.

SSB transmission time adaptation allows adaptation of SSB burst periodicity based on the traffic load.

In other scenarios, the SSB transmission was eliminated. SSB-less SCell is extended from the intra-band contiguous FR1 co-located scenario (supported in Release-15) to inter-band FR1 co-located scenario. By omitting SSB transmission, the energy cost in SCell can be reduced. The scenarios and working conditions for inter-band SSB-less SCell, and how to measure and get essential information for a SCell are all specified.

On-demand SSB was introduced for SCell operation for UE in connected mode with CA, for both intra-/inter-band CA where gNB using MAC-CE indicates the start of on-demand SSB (which may be temporary). The on-demand SSB solution applies for two distinct cases, Case #1(no always-on periodic SSB in SCell) and Case #2 (there is always-on SSB transmission, with large periods).

On-demand SIB1 was defined for idle and inactive UE, where (Case #2) UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell.

Another category of solutions for saving energy at gNB was based on principle of reducing monitoring occasions.  Such a solution was applied for time adaptation of PRACH resources where resources can be dynamically assigned only to R19 NES capable UEs, while these UEs may also access the legacy PRACH resources.

Cell Discontinuous Transmission and Discontinuous Reception (Cell DTX/DRX), involves a technique where the gNB can enter a sleep state during periods of inactivity. However, specific signals, such as Synchronization Signal Block (SSB), System Information Block (SIB)-1, paging, and others, need to be periodically transmitted regardless of the UEs presence in the cell, which limits the ability to enter sleep modes.

For energy saving during cell selection and initial access procedure a new barring bit, optionally present in SIB1, was introduced in Release 18 to allow only UEs capable of cell DTX/DRX to access a cell that has enabled this technique, thus preventing legacy UEs camping on cells adopting the Release-18 NES techniques.

In our opinion, all gNB energy saving techniques adopted in 5G should be considered from day one for energy efficiency solutions in 6G and further enhanced free from the constraints of backward compatibility. For example, if SCell configurations can be eliminated on a secondary component carrier, 5G features such as SCell with on-demand SSB or no SSB may be enhanced to be secondary component carriers with on-demand synchronization signals or cross-carrier synchronization signals, which may further reduce energy consumption. Moreover, these solutions should be extended to cover scenarios not covered in 5G. For instance, consider on-demand SIB1 for connected UEs or extending on-demand SIB1 for standalone cells.

Proposal 4: Adopt and further enhance from day one the gNB power saving solutions developed in 5G (e.g., 5G NES features SCell with on-demand SSB or no SSB, on-demand SIB1, Cell DTX/DRX, etc.).

As noted, the 5G energy saving solutions should be extended for consistency and compatibility reasons.

Proposal 5: Develop consistent energy efficiency solutions among all UE states. For instance, consistent on-demand control SIB1 signaling for all IDLE, INACTIVE and ACTIVE UEs.

Proposal 6: Increased energy efficiency through compatibility between different energy savings solutions. For instance, DRX/DTX type solutions must operate simultaneously with time adaptive control solutions (adaptive SSB, PRACH), or multiplexing NCD-SSB burst in LP-WUS transmissions.

# AI/ML and Energy Efficiency

5G-A started introducing/exploring AI/ML based approaches for enhancing physical layer functions. With the advancements in AI/ML technologies, it is expected that AI/ML can be leveraged to improve energy efficiency at both NW-side and UE-side. However, at the same time, powerful AI/ML models typically require more complex architectures to achieve better performance when the models are trained with huge number of samples, at least based on the experiences from the studies of AI/ML use cases in 5G-A, e.g., models with millions of parameters or FLOPs. These complex models incur extensive computations to generate the prediction outcome during inference, which consume significant amounts of power/energy.

In this section, we discuss potential areas that AI/ML-based approach can help in improving energy efficiency and what aspects must be considered when introducing AI/ML-based approaches in general moving forward in 6G.

## AI/ML based solutions to improve energy efficiency

In 6G networks, AI/ML technology is envisioned to become one of the fundamental pillars for energy saving across multiple operational aspects. From areas like more intelligent and automated deep-sleep prediction which can adapt to the dynamic environment/traffic behavior changes in a timely manner to areas like optimizing resource allocation policies based on UE behaviors in near real-time, we expect AI/ML technology can be leveraged in minimizing unnecessary energy consumption and optimizing network transmission activities to enable overall solutions to achieve the sustainability goals for 6G networks.

- AI/ML-based dynamic traffic load/UE distribution prediction for efficient network inactivity

Using AI/ML to dynamically predict future network traffic and UE distribution based on historical UE characteristics and behavior to reasonably plan parameters like NW power settings, sleep schedules, etc. will likely provide more accurate information for NW inactivity configurations, e.g., adaptive coverage by adjusting antenna configurations based on dynamically learned traffic patterns for reaching NW energy saving expectations.

- AI/ML-based parameter optimization for resource management

When some base stations are in power-saving mode, AI/ML-based approaches, e.g., via reinforcement learning can be employed to jointly optimize the antenna tilts and power adaptation for neighboring base stations while still satisfying the UEs to be served, e.g., throughput.

A variant case is to jointly optimize power allocation together with spectrum efficiency using AI/ML techniques.

Proposal 7: Consider studying the following AI/ML-based approaches for energy efficiency enhancements:

- Dynamic traffic prediction for efficient adaptation of network inactivity at various granularities, e.g., active antenna elements.

- Joint optimization of energy efficiency/power allocation and throughput.

## Solutions to improve energy efficiency of AI/ML based solutions

With the advancement in machine learning technologies, large deep learning models are employed widely, e.g., adopting Transformer based model structure. In 5G NR AI/ML use cases, many companies leveraged large language model-based solutions, e.g., for beam management and CSI prediction (both are 1-sided models) and CSI feedback (2-sided model). Based on evaluations, those models outperform conventional non-AI/ML based solutions. However, those large deep learning models have high complexity, typically with hundreds of millions of parameters and/or FLOPs. During the inference phase, these large deep learning models involve extensive matrix operations, especially when processing high-dimensional data, which significantly increase computational load and energy consumption.

For 6G, envision more AI/ML-based approaches will be studied and realized across releases. To have sustainable solutions, energy efficiency should be considered as an entrance criterion for any approach, including AI/ML-based solutions. In 5G NR, the focus for AI/ML-based approaches was majorly on performance gain and energy consumption was not considered in the performance evaluation. It’s our strong belief that 6G should study and adopt energy efficiency evaluation as part of the KPIs in evaluating AI/ML-based approaches.

Proposal 8: Consider studying energy consumption evaluation methodologies for AI/ML-based approaches in 6G.

Proposal 9: Include energy efficiency as part of the evaluation KPIs for all 6G AI/ML-based use cases based on studied and agreed methodologies.

# Conclusions

In this contribution, we discussed energy efficiency solutions for 6GR. The following are proposed:

Proposal 1: For 6G energy efficiency consider the following design principles:

- Native Energy Efficiency

- Joint Network and UE Power Saving

- Simplification of the Design

- Balanced Trade-off between Energy Saving and Performance

- Energy Efficiency as Mandatory Features

Proposal 2: Consider the following general energy saving techniques:

- Common Signaling and Channels Optimization

  - Reduce/minimize the energy consumption due to periodic transmission activities via prolonged periodicities, on-demand transmissions, lightweight transmissions, etc.

  - Reduce/minimize the energy consumption due to periodic/continuous reception/monitoring activities via prolonged periodicities, lower-power radio based reception/monitoring, sequence/preamble detection based reception/monitoring, etc.

  - Reduce/minimize the energy consumption on a carrier by reducing/minimizing always-on and cell-level transmissions and cell-level/UE-level configurations.

- Carrier design based solutions

  - Reduce carrier-level energy consumption and overhead via, e.g., removing SCell configurations on a secondary component carrier, on-demand bandwidth adaptation within a carrier.

- Antennas architectures

  - Energy efficient Extreme MIMO based on subarray adaptation

  - Hybrid antennas for upper midband

Observation 1: Duty-cycled operations can continue to offer simple and reliable solutions for UE power saving while LP-WUS can offer significant power saving benefits and lower latency associated with duty-cycled operations.

Observation 2: There is a functional overlap of Rel-19 LP-WUS with Rel-17 PEI and Rel-16 DCP. An enhanced LP-WUS mechanism can serve as a unified energy efficiency solution, i.e., taking over the Rel-17 PEI and Rel-16 DCP functionalities.

Observation 3: Rel-19 OOK-based LP-WUS/LP-SS designs exhibit considerably low resource utilization efficiency. However, enhancements aimed at improving overall resource utilization, e.g., through integration with other resource efficient signals or channels, may be considered.

Observation 4: RRM measurement relaxation for neighboring cells and relaxation/offloading to LP-WUR for serving cell based on stationarity, low mobility, and/or not-at-cell-edge criteria are critical elements for the significant UE power saving benefits in 6G.

Proposal 3: Adopt from day one 5G UE power saving techniques as baseline mechanisms in 6G, such as:

- Duty-cycled based operations (iDRX, eDRX, cDRX),

- LP-WUS with at least PEI and DCP functionality replacement,

- Relaxed RRM measurements of neighboring cells, and

- Relaxed/Offloading to LP-WUR of serving cell measurements.

Proposal 4: Adopt and further enhance from day one the gNB power saving solutions developed in 5G (e.g., 5G NES features SCell with on-demand SSB or no SSB, on-demand SIB1, Cell DTX/DRX, etc.).

Proposal 5: Develop consistent energy efficiency solutions among all UE states. For instance, consistent on-demand control SIB1 signaling for all IDLE, INACTIVE and ACTIVE UEs.

Proposal 6: Increased energy efficiency through compatibility between different energy savings solutions. For instance, DRX/DTX type solutions must operate simultaneously with time adaptive control solutions (adaptive SSB, PRACH), or multiplexing NCD-SSB burst in LP-WUS transmissions.

Proposal 7: Consider studying the following AI/ML-based approaches for energy efficiency enhancements:

- Dynamic traffic prediction for efficient adaptation of network inactivity at various granularities, e.g., active antenna elements.

- Joint optimization of energy efficiency/power allocation and throughput.

Proposal 8: Consider studying energy consumption evaluation methodologies for AI/ML-based approaches in 6G.

Proposal 9: Include energy efficiency as part of the evaluation KPIs for all 6G AI/ML-based use cases based on studied and agreed methodologies.

# References

RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RAN #108

6GWS-250002, Revised agenda for 3GPP workshop on 6G

R1-2505100, Draft Agenda, 3GPP TSG RAN WG1 #122

TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

TS 38.304, NR; User equipment (UE) procedures in Idle mode and in RRC Inactive state

TR-38.864, Study on network energy savings for NR

TR-38.869, Study on-low-power Wake-up Signal and Receiver for NR

TR-38.875, Study on support of reduced capabilities NR devices