3GPP TSG-RAN WG1 Meeting #122	R1-2505232

Bengaluru, India, August 25 – 29, 2025

Agenda Item:	8.13.1

Source:	Huawei, HiSilicon

Title:	Maintenance for LTE based 5G broadcast

Document for:	Discussion and Decision

Introduction

The LTE based 5G broadcast work item was completed in the last meeting. One agreement made in RAN1 in the last meeting regarding MBMS Interest Indication (MII) is as follows:

Agreement Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s): - Frequency - New values of sub-carrier spacing = {2.5 kHz, 1.25 kHz} - New values for bandwidth = {n30, n35, n40} - A new UE baseband resource Scaling factor (e.g., utilized soft buffer at the UE) used for receiving the time interleaved broadcast

Towards providing the RRC parameters to the other working groups, the MII related parameters were not finalized so are missing in the endorsed version [1]. This contribution will provide the related discussions.

Discussion

On the contents for MII

MBMSInterestIndication as defined is used to inform E-UTRAN that the UE is receiving/ interested to receive or no longer receiving/ interested to receive MBMS via an MRB or SC-MRB including MBMS service(s) in receive only mode.

The contents included in MBMSInterestIndication for ROM are expected to indicate the network that UE is able to receive the MBMS from the non-serving cell with the certain capabilities. For instance, the supported frequencies, subcarrier spacings, and bandwidth as in the previous releases.

Now with the time or frequency interleaved MBMS in Rel-19, the new capabilities to be included in the MII should directly indicate whether UE supports time and/or frequency interleaved MBMS, instead of a new UE baseband resource Scaling factor as in the agreement. This is because baseband utilization is up to eNB and the scaling factor should be reported to eNB if any, as the specified scaling factor due to supporting different subcarrier spacing as in previous releases.

Proposal 1: Update the following agreement as in red:

Agreement

Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s):

- Frequency

- New values of sub-carrier spacing = {2.5 kHz, 1.25 kHz}

- New values for bandwidth = {n30, n35, n40}

- A new UE baseband resource Scaling factor (e.g., utilized soft buffer at the UE) used for receiving the time interleaved broadcast

- UE supports time-interleaved MBMS, or frequency-interleaved MBMS, or both.

On scaling the soft buffer

As discussed in the last meeting, for UE capable of receiving time-interleaved broadcast, the soft buffer is shared with unicast reception. The question which was not discussed thoroughly is how to make the soft buffer shared or shared more properly. Here are several candidates:

- Alt1: Similar to legacy broadcast reception, in CA operation, if the eNB scheduling for DL-SCH and an MCH in one serving cell at a given TTI is larger than the defined processing capability, the prioritization between DL-SCH and MCH is left up to UE implementation.

- Alt2: Scaling down the soft buffer size used for unicast reception considering the soft buffer may be used for the broadcast reception.

For the time interleaved broadcast reception, the difference from the legacy operation is that it affects at least M*N subframes continuously, in which time frame either unicast or broadcast is dropped by UE implementation for Alt1. If UE keeps dropping unicast reception because of prioritized broadcast that is transmitted over M*N subframes, the unicast transmission performance degradation is probably not tolerable. Instead, if network is aware of UE expecting or prioritizing the broadcast transmission, it would rather be preferred for network to make UE receive a smaller soft buffer rate-matched unicast than UE drop the unicast at all. Therefore, scaling down the soft buffer for unicast transmission should be considered for such purpose.

It should be noted that this scaling down factor for the soft buffer should be reported to eNB via EUTRA-Capability instead of MII. Moreover, this soft buffer scaling factor should be different from the baseband scaling factor that was introduced in the previous release as excerpted as follows from TS36.213 [2], since the former is performed on the soft buffer size but the latter is former on the bandwidth.

In other words, the soft buffer scaling factor and the baseband scaling factor should be used together because the time-interleaved MBMS also supports the already specified subcarrier spacings except 0.37kHz.

A UE may optionally report parameters mbms-MaxBW (T), mbms-ScalingFactor1dot25 (A(1.25) , mbms-ScalingFactor7dot5 (A(7.5), mbms-ScalingFactor2dot5 (A(2.5), and mbms-ScalingFactor0dot37 (A(0.37) to indicate a limitation on baseband capability provided by the following inequality T≥c=1C(RcBcIc(15kHz +A(7.5BcIc(7.5kHz +A(2.5 BcIc(2.5kHz +A(1.25 BcIc(1.25kHz +A(0.37 BcIc(0.37kHz ) where - T is the maximum bandwidth capability of the UE, indicated by mbms-MaxBW: - if mbms-MaxBW is set to implicitValue, then T=roundMaximum number of DL-SCH transport block bits received within a TTIMaximum TBS for 2 layers for the maximum supported modulation scheme× 20MHz where the numerator and the denominator correspond to the corresponding columns of the maximum indicated ue-CategoryDL in [12]. - if mbms-MaxBW is set to explicitValue, then T=explicitValue× 40MHz - C is the number of serving cells the UE is configured with, or receiving PMCH from. - Rc is the number of spatial layers the UE can receive in the c-th serving cell according to MIMO-CapabilityDL. - Bc is the bandwidth (in MHz) of the c-th serving cell. - Bc=10 if the UE is receiving PMCH belonging to an MBSFN area with higher layer parameter pmch-Bandwidth configured for the c-th serving cell. - If the UE is receiving PMCH with 1.25kHz numerology in serving cell c, then - Ic(0.37kHz =0,Ic(1.25kHz =1, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =0 - else, if the UE is receiving PMCH with 7.5kHz numerology in serving cell c, then - Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =1,Ic(15kHz =0 - else, if the UE is receiving PMCH with 2.5kHz numerology in serving cell c, then - Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =1,Ic(7.5kHz =0,Ic(15kHz =0 - else, if the UE is receiving PMCH with 0.37kHz numerology in serving cell c, then - Ic(0.37kHz =1,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =0 - else, - Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =1

Since both unicast and MBMS reception share the same UE soft buffer size and the rate matching for each being performed based on a soft buffer size individually can already be supported by the current specification, the only additional change needed is to replace the individual soft buffer size number by a ‘portioned soft buffer size’. With that, for simplicity, the scaling factor can be as 1/2, 1/3, 1/4, 2/3, 3/4, 1.

Proposal 2: To avoid UE keeps dropping unicast reception within the time frame for the time-interleaved broadcast reception, scaling down the soft buffer for unicast reception should be supported.

- The value is taken from {1/2, 1/3, 1/4, 2/3, 3/4, 1}.

- The value is configured to UE receiving both unicast and time-interleaved MBMS.

- UE capability of supporting the soft buffer scaling factor should be included in EUTRA-Capability.

RRC parameter discussion

The MII contents related RRC parameters are missing. In line with the contents included in MII in the earlier releases as follows:

Based on the discussion in section 2.1, the following parameters should be included in the MBMS-ROM-Info: frequency, subcarrier spacing, and the numerated capability of time-interleaved, frequency-interleaved, or both.

Proposal 3: The contents to be included in the MBMS-ROM-Info are as follows:

MBMS-ROM-Info-r19 ::= SEQUENCE { mbms-ROM-Freq-r19 ARFCN-ValueEUTRA-r9, mbms-ROM-SubcarrierSpacing-r19 ENUMERATED {kHz15, kHz7dot, kHz2dot5, kHz1dot25}, mbms-Bandwidth-r19 ENUMERATED {n6, n15, n25, n30, n35, n40, n50, n75, n100} mbms-Time-Freq-Interleaving-19 ENUMERATED {Time-interleaving only, Freq-interleaving only, both}

Conclusions

This contribution discusses the remaining issues about MII and the scaling soft buffer, which leads to the following proposals:

Proposal 1: Update the following agreement as in red:

Agreement

Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s):

- Frequency

- New values of sub-carrier spacing = {2.5 kHz, 1.25 kHz}

- New values for bandwidth = {n30, n35, n40}

- A new UE baseband resource Scaling factor (e.g., utilized soft buffer at the UE) used for receiving the time interleaved broadcast

- UE supports time-interleaved MBMS, or frequency-interleaved MBMS, or both.

Proposal 2: To avoid UE keeps dropping unicast reception within the time frame for the time-interleaved broadcast reception, scaling down the soft buffer for unicast reception should be supported.

- The value is taken from {1/2, 1/3, 1/4, 2/3, 3/4, 1}.

- The value is configured to UE receiving both unicast and time-interleaved MBMS.

- UE capability of supporting the soft buffer scaling factor should be included in EUTRA-Capability.

Proposal 3: The contents to be included in the MBMS-ROM-Info are as follows:

MBMS-ROM-Info-r19 ::= SEQUENCE { mbms-ROM-Freq-r19 ARFCN-ValueEUTRA-r9, mbms-ROM-SubcarrierSpacing-r19 ENUMERATED {kHz15, kHz7dot, kHz2dot5, kHz1dot25}, mbms-Bandwidth-r19 ENUMERATED {n6, n15, n25, n30, n35, n40, n50, n75, n100} mbms-Time-Freq-Interleaving-19 ENUMERATED {Time-interleaving only, Freq-interleaving only, both}

- References

R1-2503243 (Consolidated Rel-19 higher layers parameters list Post RAN1#121), RAN1#121, St Julian’s, Malta, May 19th – 23rd, 2025.

TS36.213, Physical layer procedures, v19.0.0.