3GPP TSG RAN WG1 #122		R1-2505259

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.5.3

Source:	Google

Title:	Adaptation of Common Signals

Document for:	Discussion/Decision

# Introduction

In RAN1 #121 meeting, the following agreements on adaptation of common signal have been achieved.

Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell for the case when cell DTX/DRX is not configured, reuse existing search space configuration parameter for DCI 2_9-based monitoring and existing DCI 2_9 size configuration parameter and update in specification that these are also applicable to SSB burst periodicity adaptation (when configured) Agreement Value d (in the WA from RAN1#120bis) is the number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213. Agreement Update the agreement from RAN1#120bis as shown below (i.e. updates in red). Agreement (from RAN1#120bis) For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon Note: Whether the additional PRACH configuration can be from RRC other than SIB1 is up to RAN2. Agreement The fourth candidate value for K (for PRACH subset mask) is 16. K=1 is default value (if parameter is not configured). Agreement Supported values for validity duration configured by higher layers are - {2,4,8,16} x SI modification period. Agreement Value range for PRACH-Config Index parameter for additional RACH configuration is same as legacy, i.e. INTEGER (0...255). - Note: Final decision on the value range is up to RAN2 Conclusion Using DCI 1_0 with P-RNTI to explicitly deactivate the additional PRACH resources is NOT supported. Conclusion There is no consensus to support adaptation of RACH in time domain for 2-step RA in Rel-19. Agreement ‘PRACH resource indicator’ field is present in DCI 1_0 with C-RNTI for PDCCH order when the configuration of the additional RACH resources is provided in SIB1(i.e. addl-RACH-Config-Adaptation). - Above applies for PCell Agreement When the SSB burst periodicity is switched from periodicity value P1 to periodicity value P2 based on DCI format 2_9 indication, - Alt 1: SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value. - the first SSB burst according to the periodicity value P2 is determined as the first SSB burst according to the SSB burst periodicity value P2 and associated SFN offset and half-frame index that is no earlier than slot m+d. SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity. Agreement Both CBRA and CFRA based on additional PRACH resources is supported for PDCCH order via DCI 1_0 with C-RNTI. - For CFRA, - The indicated SSB index and PRACH mask index are applied to both legacy PRACH resources and additional PRACH resources. - Note: The PRACH mask applies to either additional resources and/or legacy resources, depending on which one satisfies the conditions for the mask to be applicable. Agreement Adopt the below TP for Clause 8.1 of TS 38.213, as per Editor CR available in R1-2503167. *** Unchanged text omitted *** For valid PRACH occasions associated with addl-RACH-Config-Adaptation [in RACH-ConfigCommon], the UE can be additionally provided a PRACH mask index, by prach-SubsetMask-Index-Adaptation that, if provided, indicates one or more association periods per K_mask association pattern periods according to Table 8.1-0, where K_mask is provided by KforAPPForPRACHsubsetMask. Table 8.1-0: Mapping of mask index to association periods per Kmask association pattern periods Mask Index; Association periods (APs) perKmask association pattern periods (APPs). 0; First half of APs in Kmask APPs. 1; First quarter of APs in Kmask APPs. 2; First eighth of APs in Kmask APPs. 3; First sixteenth of APs in Kmask APPs. Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally associated with in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are activated indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI [or a C-RNTI] [5, TS 38.212]. For activation indication by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI. *** Unchanged text omitted *** Agreement Additional PRACH availability indication can be carried by a DCI 1_0 with P-RNTI with Short Messages Indicator set to 00, 01,10,11. - Note: Above is already reflected in the endorsed editor CR 38.212

In this contribution, we provide some discussion on adaptation of common signal/channel transmissions.

# Adaptation of SSB in time domain

## Monitoring of the DCI for SSB adaptation

For SSB adaptation, it is specified that the DCI based on ssbPeriodicityIndication-RNTI can be used to indicate the periodicity for the SSB. It is unclear whether the UE should monitor such DCI during the inactive period for the cell DTX or not. One possible way is to reuse the same mechanism as other types of group-cast DCI, that the UE should not monitor the DCI based on ssbPeriodicityIndication-RNTI during the inactive period of cell DTX.

Proposal 1: Clarify that the UE should not monitor the DCI based on ssbPeriodicityIndication-RNTI during the inactive period for the cell DTX.

# Adaptation of PRACH

## Interaction with SBFD

For SBFD, the NW can also configure additional PRACH resources for SBFD symbols/slots. Then for PRACH adaptation for NES, it should be clarified that whether the additional PRACH resources can be applicable to SBFD symbols/slots. One possible way is to configure two additional PRACH resource sets for NES, where one is not applicable to the SBFD symbols/slots and the other one is applicable to the SBFD symbols/slots, with regard to UEs that support NES only and UEs that support both SBFD and NES.

Proposal 2: With regard to UEs that support NES only and UEs that support both SBFD and NES, support to configure two additional PRACH resource sets for NES

- The first set is applicable to the non-SBFD symbols/slots and the second set is applicable to the SBFD symbols/slots

# Conclusion

In this contribution, we provided discussion on adaptation of common signals. Based on the discussion, the following proposals are provided.

SSB time-domain adaptation

Proposal 1: Clarify that the UE should not monitor the DCI based on ssbPeriodicityIndication-RNTI during the inactive period for the cell DTX.

PRACH time-domain adaptation

Proposal 2: With regard to UEs that support NES only and UEs that support both SBFD and NES, support to configure two additional PRACH resource sets for NES

- The first set is applicable to the non-SBFD symbols/slots and the second set is applicable to the SBFD symbols/slots